dctv.library - September 23, 1992

Copyright (C) 1991-1992, Digital Creations, Inc.

TABLE OF CONTENTS

dctv.library/AllocDCTVCvt

dctv.library/AllocDCTVCvtTagList

dctv.library/AllocDCTVCvtTags

dctv.library/CvtDCTVLine

dctv.library/DCTVCvtLine

dctv.library/DCTVFormat

dctv.library/FormatDCTV

dctv.library/FreeDCTVCvt

dctv.library/InitDCTVCvt

dctv.library/ReadDCTVPixel

dctv.library/SetDCTVColorTable

dctv.library/TestDCTVSignature

dctv.library/AllocDCTVCvt                           dctv.library/AllocDCTVCvt

NAME

AllocDCTVCvt -- Old form of AllocDCTVCvtTagList().

SYNOPSIS

struct DCTVCvtHandle *AllocDCTVCvt (struct BitMap *BitMap,

a0

ULONG Width, ULONG Height, ULONG Flags)

d0           d1            d2

FUNCTION

This is the V1 compatible form of AllocDCTVCvtTagList().  It offers

only a subset of the features of AllocDCTVCvtTagList():

. RGB->DCTV conversion only.

. No extended error code.

This function is equivalent to:

AllocDCTVCvtTags ( BitMap,

DCTVCVTA_Width,  Width,

DCTVCVTA_Height, Height,

DCTVCVTA_Flags,  Flags,

TAG_END );

This function is considered obsolete, but is still present in the

library for backwards compatibility.

INPUTS

BitMap - BitMap arg for AllocDCTVCvtTagList().

Width,

Height,

Flags - Values for DCTVCVTA_Width, DCTVCVTA_Height, and DCTVCVTA_Flags

respectively.

RESULTS

Pointer to initialized DCTVCvtHandle for use with other conversion

functions.  NULL on failure.

SEE ALSO

AllocDCTVCvtTagList(), FreeDCTVCvt(), libraries/dctv.h

dctv.library/AllocDCTVCvtTagList             dctv.library/AllocDCTVCvtTagList

NAME

AllocDCTVCvtTagList -- Allocate a conversion environment. (V3)

SYNOPSIS

struct DCTVCvtHandle *AllocDCTVCvtTagList

(struct BitMap *BitMap, struct TagItem *TagList)

a0                     a1

FUNCTION

Allocates and initializes a DCTV conversion environment.  The following

types of conversions are supported:

. 24-bit RGB to DCTV BitMap

. DCTV BitMap to 24-bit RGB

In both cases the 24-bit RGB data is expected to be in 70ns (HIRES)

pixels, interlace or non-interlace.  The RGB and DCTV pixels correspond

1:1.  No image scaling is performed.

The supplied taglist desribes the conversion type, image dimensions,

and a variety of other parameters.

There are two methods of handling DCTV BitMaps:  full-screen (default)

and 1-line.  A full-screen BitMap contains the entire DCTV display

image (either as destination or source depending on conversion type).

Successive calls to CvtDCTVLine() process successive lines in the

BitMap.

A 1-line BitMap (DCTVCVTF_1LineBM set) contains only 1 line of the

DCTV BitMap instead of the full image.  This sort of BitMap is not

displayable, but is useful as a file buffer.  It also requires less

memory than a full-screen BitMap.  In this case, each call to

CvtDCTVLine() uses the same line in the BitMap.

RGB data is handled in the form of a 24-bit RGB line buffer in the

form of 3 UBYTE arrays in the conversion environment: one for each

component (Red, Green, and Blue).  The RGB data is in chunk pixel

format instead of a BitMap.  These arrays are used to buffer 1 line

of RGB data and must be read or written to (depending on conversion

type) between calls to CvtDCTVLine().

Normally AllocDCTVCvtTagList() allocates the 24-bit RGB line buffer.

When DCTVCVTF_CustomRGBBuf is set, the client is allowed to supply

the line buffer instead.  This is useful if your RGB buffer is

taller than 1 line since this mode allows changing the RGB buffer

pointers between calls to CvtDCTVLine().  This also eliminates copying

to and from the conversion 24-bit line buffer.

Note that the buffer pointers in the DCTVCvtHandle may only be changed

if DCTVCVTF_CustomRGBBuf is set.  These pointers are read-only when

this bit isn't set.

A special case exists in which DCTVCVTF_CustomRGBBuf may be specified

and the buffers left NULL.  This is legal only when using

DCTVCVTT_DCTVtoRGB conversion mode and only calling ReadDCTVPixel()

(not CvtDCTVLine()) because ReadDCTVPixel() does not access the RGB

line buffers.

The actual conversion process consists of calling CvtDCTVLine()

for each line to be converted.

The line number values SrcLineNum, DstLineNum and NDelayLines are used

to determine which line is to be input and output by the next call to

CvtDCTVLine().  NDelayLines indicates how many lines of source data

are required to be buffered in order to generate the first line of

output data.  The actual value of NDelayLines depends on the

conversion type and flags and may change from one library version to

another.

The following relationship is always true:

SrcLineNum - DstLineNum == NDelayLines.

Usually CvtDCTVLine() must be called a total of Height + NDelayLines

times to completely generate the output image.

A conversion environment can only be used for the type of conversion

specified in DCTVCVTA_Type.  A single conversion environment can be

used to process multiple frames sequentially by calling InitDCTVCvt()

between frames.  The only restriction is that each frame must use the

same BitMap and be the same dimensions.

Call FreeDCTVCvt() to dispose of conversion environment.

INPUTS

BitMap - Pointer to an initialized BitMap structure with the following

dimension limits:

Width >= 256

Height >= 2 for non-lace, 3 for lace

Depth >= 3

The Planes[] data need not be in chip ram for conversion (but

obviously do if you expect to display the BitMap).

Both interleaved and non-interleaved bitmaps are supported.

For full-screen mode, the BitMap must be large enough to hold the

entire conversion as specified by DCTVCVTA_Width and

DCTVCVTA_Height.  For 1-line mode, the BitMap needs to have at

least one line of data wide enough to hold an entire line as

specified by DCTVCVTA_Width.

The supplied BitMap structure and Planes[] data must remain valid

until FreeDCTVCvt() is called.

TagList - Pointer to TagItem array with Tags described below.

TAGS

DCTVCVTA_Type       (UWORD)

DCTVCVTT_RGBtoDCTV - Convert 24-bit RGB chunky pixels to DCTV a

display BitMap.  This is the default conversion type.

DCTVCVTT_DCTVtoRGB - Convert a DCTV display BitMap to 24-bit

RGB chunky pixels.

DCTVCVTA_Width      (WORD)

Pixel width of image to convert in 70ns (HIRES) pixels.  Must be

multiple of 16 in the range of 256 .. BitMap width.  The practical

maximum width is 736 (the maximum display width).  Non-multiples of

16 are truncated.

The default value is derived from the BitMap.

It's safest to always include this tag when using a 1-line BitMap

since there is some ambiguity as to what BitMap->BytesPerRow

actually means for a 1-line BitMap.

DCTVCVTA_Height     (WORD)

Pixel height of image to convert.

For full-screen mode, must be in the range of 2 for non-lace (3

for lace) .. BitMap height.  For 1-line mode, must be at least 2

for non-lace, 3 for lace.

The default value is derived from the BitMap.

It's safest to always include this tag when using a 1-line BitMap

since there is some ambiguity as to what BitMap->Rows actually

means for a 1-line BitMap.

DCTVCVTA_Flags      (UWORD)

DCTVCVTF_Lace - Interlaced DCTV display BitMap.

DCTVCVTF_Filter - Apply RGB filtering.  Only observed for

DCTVCTVT_RGBtoDCTV.

DCTVCVTF_1LineBM - Treat BitMap as 1-line instead of full-screen.

DCTVCVTF_CustomRGBBuf (V3) - Allow client to supply a custom RGB

line buffer.  Normally this function allocates the RGB line

buffer.

Unknown flags are ignored.

DCTVCVTA_ColorTable (UWORD *)

Initial palette associated with BitMap for DCTVCVTT_DCTVtoRGB

conversion.  Must contain 1 << BitMap->Depth entries.  This array

must remain valid until it is replaced with SetDCTVColorTAble()

or the conversion environment is freed.

Mid-BitMap palette changes, if any, can be set with

SetDCTVColorTable().

This tag is required for DCTVCVTT_DCTVtoRGB in order to be able to

decode the bits in the BitMap.  Ignored for DCTVCVTT_RGBtoDCTV.

DCTVCVTA_ErrorCode  (ULONG *)

Pointer to a ULONG where an extended error code is placed if

AllocDCTVCvtTagList() fails.  The buffer pointed to by this

tag is modified only on an error.

Errors codes:

DCTVCVTERR_NoMem - Not enough memory.

DCTVCVTERR_BadBitMap - Supplied BitMap is not valid.  Could be

invalid dimensions or depth.

DCTVCVTERR_UnknownType - Unknown conversion type passed to

DCTVCVTA_Type.

DCTVCVTERR_BadParams - Invalid parameters provided for

conversion.

RESULTS

Pointer to initialized DCTVCvtHandle for use with other conversion

functions.  NULL on failure.  Can fail if out of memory or supplied

parameters are not legal.

On success the DCTVCvtHandle structure is initialized as follows:

(see libraries/dctv.h for a more complete explanation of each field)

Red,

Green,

Blue - If DCTVCVTF_CustomRGBBuf isn't set, these point to allocated

24-bit RGB line buffer, Width bytes wide.  The buffers are

initialized to zero.

If DCTVCVTF_CustomRGBBuf is set, these are set to NULL and

the client is expected to set these pointers prior to calling

CvtDCTVLine().

BitMap - BitMap pointer passed to AllocDCTVCvtTagList().

Width,

Height - Values from DCTVCVTA_Width and DCTVCVTA_Height or derived

from BitMap.

ImageBounds - Computed from Width, Height and Modes.

ColorTable,

NColors - Palette information for BitMap Depth.  For

DCTVCVTT_RGBtoDCTV this is the palette supplied by dctv.library

for the output BitMap.  For DCTVCVTT_DCTVtoRGB, this is the

array supplied by DCTVCVTA_ColorTable.

NDelayLines - Initialized depending on type and flags.

SrcLineNum - Initialized to 0 (as set by InitDCTVCvt()).

DstLineNum - Initialized to -NDelayLines (as set by InitDCTVCvt()).

NOTE

V1 AllocDCTVCvt() was documented as tolerating a BitMap with the

BitMap->Planes[] array not initialized.  This practice is not

recommended and will probably break in the future.

SEE ALSO

AllocDCTVCvtTags(), FreeDCTVCvt(), CvtDCTVLine(), libraries/dctv.h

dctv.library/AllocDCTVCvtTags                   dctv.library/AllocDCTVCvtTags

NAME

AllocDCTVCvtTags -- Varargs version of AllocDCTVCvtTagList(). (V3)

SYNOPSIS

struct DCTVCvtHandle *AllocDCTVCvtTagList

(struct BitMap *BitMap, Tag tag1, ...)

FUNCTION

Varargs version of AllocCvtTagList().  See AllocDCTVCvtTagList() for

complete description.

INPUTS

BitMap - BitMap arg for AllocDCTVCvtTagList().

Tag1... - TagItem array on the stack to be passed to

AllocDCTVCvtTagList().

RESULTS

Pointer to initialized DCTVCvtHandle for use with other conversion

functions.  NULL on failure.

NOTE

16-bit integer users:  be careful to cast integer tag data to 32-bits.

For example:

AllocDCTVCvtTags ( bm,

DCTVCVTA_Type,   (ULONG)DCVTCVTT_RGBtoDCTV,

DCTVCVTA_Width,  (ULONG)width,

DCTVCVTA_Height, (ULONG)height,

DCTVCVTA_Flags,  (ULONG)DCTVCVTF_Filter | DCTVCVTF_Lace,

TAG_END );

SEE ALSO

AllocDCTVCvtTagList(), FreeDCTVCvt(), CvtDCTVLine(), libraries/dctv.h

dctv.library/CvtDCTVLine                             dctv.library/CvtDCTVLine

NAME

CvtDCTVLine -- Convert a line.

SYNOPSIS

void CvtDCTVLine (struct DCTVCvtHandle *Handle)

a0

FUNCTION

Converts a line as described below.

NOTE: All references to SrcLineNum and DstLineNum values below are

the values before calling CvtDCTVLine().

DCTVCVTT_RGBtoDCTV - Converts the RGB data in the Handle's RGB line

buffers to DCTV.  Assumes that the buffers contain data for

SrcLineNum and will generate the BitMap line indexed by DstLineNum.

The source RGB line buffer is only read when SrcLineNum is in the

vertical range of ImageBounds.  Otherwise it is ignored.  Pixels

outside the range of ImageBounds are ignored.

Destination BitMap data is only emitted for values of DstLineNum

in the range of 0..Height-1.  The BitMap is not affected for

DstLineNum values outside this range.

The contents of the RGB line buffer are not affected by this

function.

DCTVCVTT_DCTVtoRGB - Converts the DCTV display BitMap line indexed by

SrcLineNum to the RGB line buffer.  The resulting RGB data is for

DstLineNum.

Source BitMap data is only read for SrcLineNum values in the

vertical range of ImageBounds.

Destination RGB data is only emitted for DstLineNum values in the

range of 0..Height-1.  Pixels outside the ImageBounds rectangle

are emitted as black.

The BitMap is not affected by this function.

SrcLineNum and DstLineNum are incremented after processing the line.

CvtDCTVLine() should be called until DstLineNum >= Height in order

to convert the entire image (see example below) when NDelayLines

is non-zero.  Be careful to avoid overindexing your buffers.

INPUTS

Handle - DCTVCvtHandle returned by AllocDCTVCvtTagList().  Note to

DCTVCVTF_CustomRGBBuf users:  the RGB line buffer pointers must

point to a valid buffer by the time of this call (i.e. non-NULL)

for line number ranges that would access the RGB line buffer

(see above for when this happens).

RESULTS

Outputs BitMap or RGB data (depending on conversion type) for line

DstLineNum when DstLineNum is in the range of 0 <= DstLineNum < Height

prior to call.

Increments SrcLineNum and DstLineNum.

EXAMPLE

while (cvt->DstLineNum < cvt->Height) {

renderline (cvt);           /* Render a line of the gradient to

DCTVCvtHandle RGB line buffer.

This function must be smart enough

to deal w/ SrcLineNum >= Height. */

CvtDCTVLine (cvt);          /* Convert the line in the line buffer

to DCTV display data. Results in

rendering a line of the BitMap. */

}

SEE ALSO

AllocDCTVCvtTagList(), InitDCTVCvt(), libraries/dctv.h

dctv.library/DCTVCvtLine                             dctv.library/DCTVCvtLine

NAME

DCTVCvtLine -- Old name for CvtDCTVLine().

SYNOPSIS

void DCTVCvtLine (struct DCTVCvtHandle *Handle)

a0

FUNCTION

V1 name for CvtDCTVLine().  This is the same library vector with a new

name so it is completely run-time compatible with old software and V1

of dctv.library.

There is a #define in clib/dctv_protos.h to translate the old name to

the new name.  Also dctv.lib contains glue functions and LVOs for both

names.

Use CvtDCTVLine() from now on.

SEE ALSO

CvtDCTVLine()

dctv.library/DCTVFormat                               dctv.library/DCTVFormat

NAME

DCTVFormat -- Old name for FormatDCTV().

SYNOPSIS

void DCTVFormat (struct DCTVCvtHandle *Handle)

a0

FUNCTION

V1 name for FormatDCTV().  This is the same library vector with a new

name so it is completely run-time compatible with old software and V1

of dctv.library.

There is a #define in clib/dctv_protos.h to translate the old name to

the new name.  Also dctv.lib contains glue functions and LVOs for both

names.

Use FormatDCTV() from now on.

SEE ALSO

FormatDCTV()

dctv.library/FormatDCTV                               dctv.library/FormatDCTV

NAME

FormatDCTV -- Format BitMap for DCTV display.

SYNOPSIS

void FormatDCTV (struct DCTVCvtHandle *Handle)

a0

FUNCTION

Formats the Handle's BitMap for a black DCTV display.  This is useful

if you wish to display the conversion process while in progress.

Only use with a full-screen BitMap in a DCTVCVTT_RGBtoDCTV conversion

environment.

INPUTS

Handle - DCTVCvtHandle returned by AllocDCTVCvtTagList().

RESULTS

Formatted BitMap.

SEE ALSO

AllocDCTVCvtTagList()

dctv.library/FreeDCTVCvt                             dctv.library/FreeDCTVCvt

NAME

FreeDCTVCvt -- Free conversion environment.

SYNOPSIS

void FreeDCTVCvt (struct DCTVCvtHandle *Handle)

a0

FUNCTION

Frees a DCTVCvtHandle allocated by AllocDCTVCvtTagList().

INPUTS

Handle - DCTVCvtHandle returned by AllocDCTVCvtTagList().

RESULTS

None.

SEE ALSO

AllocDCTVCvtTagList()

dctv.library/InitDCTVCvt                             dctv.library/InitDCTVCvt

NAME

InitDCTVCvt -- Init conversion environment for frame.

SYNOPSIS

void InitDCTVCvt (struct DCTVCvtHandle *Handle)

a0

FUNCTION

Initializes line numbers for a new frame (image).  This is done

automatically by AllocDCTVCvtTagList().  Use it if you need to process

multiple frames using the same DCTVCvtHandle.

This function does not initialize the RGB line buffer data or BitMap

data in any way.

Also, there is no provision to change image dimensions, BitMap, or any

other parameter of a conversion environment.

INPUTS

Handle - DCTVCvtHandle returned by AllocDCTVCvtTagList().

RESULTS

DCTVCvtHandle fields are initialized as follows:

Handle->SrcLineNum = 0

Handle->DstLineNum = -Handle->NDelayLines

SEE ALSO

AllocDCTVCvtTagList(), FormatDCTV()

dctv.library/ReadDCTVPixel                         dctv.library/ReadDCTVPixel

NAME

ReadDCTVPixel -- Get 24-bit RGB value of a pixel from a DCTV display

BitMap. (V3)

SYNOPSIS

ULONG ReadDCTVPixel (struct DCTVCvtHandle *Handle, LONG x, LONG y)

a0                 d0      d1

FUNCTION

Returns 24-bit RGB value for the specified pixel in a

DCTVCVTT_DCTVtoRGB environment in the following format 32-bit format:

xxxxxxxx rrrrrrrr gggggggg bbbbbbbb

where the most significant byte value is undefined.

Returns 0 for pixels outside of ImageBounds.

INPUTS

Handle - DCTVCvtHandle returned by AllocDCTVCvtTagList().  The RGB

line buffers are not used by this function and may be NULL

if DCTVCVTF_CustomRGBBuf was specified.

x, y - Pixel to examine using the coordinate system of the BitMap.

RESULTS

24-bit right-justifed RGB value or 0.

NOTE

For the sake of speed, this function does less horizontal filtering

than CvtDCTVLine() and therefore yields slightly different results.

The most visible difference is that sharp vertical edges in the DCTV

image get a bit "zippered" when converted to RGB using

ReadDCTVPixel().  CvtDCTVLine() typically makes smoother RGB images.

SEE ALSO

AllocDCTVCvtTagList(), CvtDCTVLine(), libraries/dctv.h

dctv.library/SetDCTVColorTable                 dctv.library/SetDCTVColorTable

NAME

SetDCTVColorTable -- Set mid-BitMap palette change for DCTV->RGB

conversion. (V3)

SYNOPSIS

void SetDCTVColorTable (struct DCTVCvtHandle *Handle,

a0

UWORD *ColorTable)

a1

FUNCTION

Replaces the Handle's ColorTable for DCTVCVTT_DCTVtoRGB conversions.

This can be used to supply a mid-screen palette change for the source

DCTV display BitMap.  This function is of limited usefulness, but is

essential for a couple of things:

. converting the DCTVPaint control panel to 24-bit RGB.

. handling PCHG chunks in a DCTV display ILBM.

INPUTS

Handle - DCTVCvtHandle returned by AllocDCTVCvtTagList().

ColorTable - RGB4 UWORD array containing NColors (1<<BitMap->Depth)

entries.  This array must remain valid until it is replaced with

another call to SetDCTVColorTable() or the conversion environment

is freed.

RESULTS

Updates Handle->ColorTable.

NOTE

This function is ignored for DCTVCVTT_RGBtoDCTV.

SEE ALSO

AllocDCTVCvtTagList(), libraries/dctv.h

dctv.library/TestDCTVSignature                 dctv.library/TestDCTVSignature

NAME

TestDCTVSignature -- Check a BitMap for DCTV signature.

SYNOPSIS

BOOL TestDCTVSignature (struct BitMap *BitMap)

a0

FUNCTION

Checks the top line of the supplied BitMap for the DCTV signature.

This signature must appear at the top of all DCTV pictures.  This

function provides a way of determining if a picture, possibly loaded

from an ILBM, is a DCTV display picture.

INPUTS

BitMap - BitMap to check.  Planes[] data does not need to be in

chip memory for this function.

RESULTS

TRUE (non-zero) if the top line of the BitMap contains a DCTV

signature.  FALSE otherwise.

SEE ALSO