VERSION 1.0Beta-19950416

(Note: this readme now includes information about getting AmiTCP ARP

to publish your "hardware" address.  It should have been here before, and

my explanation could probably use work, but it should be sufficient for

you to get this driver working with AmiTCP, or other packages that use

an ARP type facility).

This is currently a BETA product.  There are no known enforcer hits during

intended operation.

The driver currently assumes that once started, it will never be stopped

(except by a reboot).  It does not currently shut down gracefully if

told to, or if it fails to start (due to problems with either the config file

or the underlying serial port.)

This is a SANA2 device driver that encapsulates a higher-level protocol

(such as TCP/IP) in AX25 UI (unnumbered-information) frames.

AX25 is an amateur (HAM) radio protocol that allows digital communications

via radiowaves.

This driver does not currently support direct AX25 activity.

This driver has been successfully tested with amitcp-2.2 (special modifications

were made to the 2.2 source to support AX25, this support should be

directly available in the 3.0 AMITCP.) and has interacted successfully with

various versions of the KA9Q NOS.  With some versions of NOS, telnet does

not properly negotiate linemode-with-local-echo.  I do not yet know if this

is a fault of NOS, or telnet (or both).  I have found one version, so far,

of NOS that operates properly in linemode with the amitcp telnet.

A note about the TNC:  Your TNC should already be in kiss mode

and properly configured before starting this driver, the driver will

not attempt to configure the TNC in any way.  You must set your

TXDelay and other parameters before hand, and place your TNC in KISS

mode.  Also the TNC needs to be in hardware flowcontrol mode, and NOT using

XON/XOFF for flow control.  Most modern TNCs will remember their configuration

between power ups/downs, including being in KISS mode. (I hope, I haven't

actually verified this, but in this day and age non-volatile

configuration memory should be very common.)

(I personally am currently using the Kantronics dataengine, it remembers

its configuration, and remains in kiss mode through power cycles.)

(Also note:  It is not sufficient to only tell your TNC what your callsign

is, you must also tell this driver, see below.)

INSTALLATION:

copy the devs/networks/ax25.device to devs:networks/ax25.device

copy the envarc/sana2/ax25.config to envarc:sana2/ax25.config

edit envarc:sana2/ax25.config to suit your needs, instructions are included

in that file.

NOTE: IT IS *VERY* IMPORTANT that you change the callsign in the config

file to a callsign you are authorized to use.

edit whatever network software you're using to use this driver.

::: AmiTCP 3.0beta example:

::: ifconfig axdm1 inet aa0aaa.ampr.org netmask 255.255.255.0

^^^^^^ your callsign here

(This assumes you have defined your nodename in AmiTCP:db/hosts,

you can substitute your IP address directly instead if you want.)

depending on local conventions, you may need to change the netmask.

::: arp -s aa0aaa.ampr.org 82:82:60:82:82:82:60 pub

The arp line needs some more explaining:  Your network software

may need to be told to publish your "Hardware" address, which is your callsign

for this driver.  If you don't do this you may not be able to talk to

anyone, and they probably won't be able to talk to you.

This information is specific to amitcp, but probably applies

to other network software which uses arp as well.  Your "hardware" address

is specified as 6 hexadecimal numbers, colon separated.  Due to the nature

of AX25 and AmiTCP's ARP, it is necessary to left-shift each byte (the values

between the colons) left one BIT.  In addition, the last byte contains

an SSID, or Secondary Identifier.  Your TNC manual should explain the SSID,

or find a book on amateur packet radio.  It is a value from 0 to 7, if you don't

know what the SSID is, then just use 0.

Example:  Your callsign is AB2IJ and you decide to use 3 as your SSID

AB2IJ is 41:42:32:49:4a in hexadecimal.  Use uppercase letters for your callsign.

To convert this into the proper string for arp, we must shift each value 1 bit

left.

The value for 41 in binary is 01000001. Shift it left one: 10000010, convert

it back to hex: 82.  After doing this for each character:

41:42:32:49:4a   becomes   82:84:64:92:94 (note: instead of shifting left one,

you can multiply by 2, either multiply by two before converting to hex in the

first place, or convert to hex and multiply by 2, but use HEX math.)

Now, it must be padded out to six characters, as all ax25 hardware addresses

are six characters in the callsign part.  So, add spaces on the end (space is

hex 20, but becomes 40 after shifting left (doubling).)

(***NOTE: I have included a table below so you won't actually need to

calculate this; don't forget to space pad though if your callsign

is less than 6 characters, SSID must be the last byte.

***)

82:84:64:92:94:40

Now convert your SSID (3 in this example) to HEX ascii: 33

Shift left one: 66

And append: 82:84:64:92:94:40:66

If you don't know what SSID to use, just use 0, which

will be :60, 82:84:64:92:94:40:60

So, the arp command for AmiTCP and a Callsign of ab2ij-3 (the -3 is the SSID)

is:

arp -s AB2IJ.AMPR.ORG 82:84:64:92:94:40:60 pub

This will work if you have specified your IP address for AB2IJ.AMPR.ORG in

your AmiTCP:db/hosts file, or you can just put your IP address directly:

arp -s 44.x.y.z 82:84:64:92:94:40:60 pub

Where 44.x.y.z is whatever your IP address is.

Here is a reference table to simplify this process, so you won't have to

calculate it:

SPACE:40

A:82   B:84   C:86   D:88   E:8A   F:8C   G:8E   H:90

I:92   J:94   K:96   L:98   M:9A   N:9C   O:9E   P:A0

Q:A2   R:A4   S:A6   T:A8   U:AA:  V:AC   W:AE   X:B0

Y:B2   Z:B4

0:60   1:62   2:64   3:66   4:68   5:6A   6:6C   7:6E

8:70   9:72

Donations of money or other items are accepted and appreciated, but are

not required.  Please make sure monetary donations are in U.S. funds, or can

be converted to such without costing more than the value of the donation.

(exception, if you want to send me actual money of some form, I collect it,

but apparently money tends to disappear in various postal systems.)

I will attempt to place updated versions on AmiNET sites and/or with the

AmiTCP distribution and possibly via other routes as well (such as Fred Fish).

At least please send me a note (mail or EMAIL) with your comments and

suggestions about this driver, or at least let me know your using it.

***

You should let me know especially if you are having a problem, as I can't

fix what I don't know about.

***

martind@hillwpos.hill.af.mil

dave@cs.weber.edu

Dave Martin

843 Clover Circle

Ogden, Utah  84404

USA

This driver is Copyright 1994,1995 by David E Martin KB7VBD, it may be distributed

without profit.  Organizations such as magazines or shareware collections

or other people that are involved commercially should contact me first.

Permission for appearance in the Fred Fish collection (disk or CDROM) is

hereby granted.  Permission for appearance in the aminet CDROM is hereby

granted.  Permission for other CDROM collections is granted if they are

comparably priced to other shareware type CDROMs from other manufacturers.

The Author reserves the right to change these terms in future

versions of this driver.  I would appreciate at least one edition of a

cdrom of a given collection that contains this package.

HISTORY:

V1.0Beta-19940320	First external release.

V1.0Beta-1994????       Minor memory problem fix.

V1.0Beta-19940704	Documentation fixes and some debugging kprintfs removed.

Documentation revised on 19940714

V1.0Beta-19941224	Occasional lockup bug hopefully exterminated.

Documentation revisions and CDROM distribution rights

updated.

V1.0Beta-19950416	Major memory overwrite problem fixed.

---------------------------------------------------------------------------

See ax25.config for additional info