PLIP.DEVICE(1)                                     PLIP.DEVICE(1)

NAME

PLIP - A Parallel Line IP SANA2 Device Driver

OVERVIEW

PLIP  is  a  network device driver conforming to the SANA2

standard.  It allows linking of  two  Amigas  using  their

parallel  ports.   Transfer  speed is around 40 kbyte/sec.

The complete low level transfer protocol is integrated  in

the  device driver, including port arbitration handshaking

and interrupt driven reads.  Although the transfer process

itself is cpu intensive, the device needs no cpu resources

when no transfer is in progress.

PLIP is intended as a low-cost networking solution,  since

the  capabilities  of  the  parallel hardware are somewhat

limited.  However, it gives much better results than  SLIP

and leaves the serial port free.

REQUIREMENTS

You  will  need   at   least kickstart and workbench V2.04

(37.175)  to use PLIP. Also some SANA2 compliant  network-

ing  protocol (like Envoy, AS225R2 or AmiTCP) will come in

handy :-).

INSTALLATION

Just copy plip.device to devs:networks.  See the  documen-

tation of your networking package for further installation

steps.  If you are going to use both Envoy and AS225  over

PLIP  at the same time, be sure to first start Envoy, then

AS225.  If you've  got  problems  with  multiple  protocol

stacks  you  might  try making the packet and host IDs for

each protocol unique.

Since PLIP only allows two hosts to  be  linked  together,

addressing   is   limited   to   1  Bit.   One  host  uses

plip.device, Unit 0, the other one  plip.device,  Unit  1.

The  device  will  not  work if both machines use the same

unit number.

PLIP   looks   for   an   environment   variable    called

ENV:SANA2/PLIP.config. This should contain one line termi-

nated with a newline, which will  be  parsed  by  ReadArgs

according to the template

TIMEOUT/K/N,PRIORITY=PRI/K/N,MTU/K/N,BPS/K/N,RETRIES/K/N

where

TIMEOUT=num

Packet receive timeout, default is 330000.  Set to

a  higher  value  if  you  have problems with slow

machines.

PRIORITY=num

PLIP task priority, default is 15. Do  not  change

Version 38.11               Release 4                           1

PLIP.DEVICE(1)                                     PLIP.DEVICE(1)

this.

MTU=num Maximum  packet  data size, default and maximum is

8192. If you change this, be sure to change it  on

both machines to the same value!

BPS=num The  nominal  bits per second rate returned from a

device query. Default is 25000 which is  much  too

low but works with beta Envoy.

RETRIES=num

The  maximum  number of times, the driver tries to

resend a packet on  error.  Default  is  63  which

means a total of 64 tries.

An example PLIP.config is included. All recognized parame-

ters will be printed into the startup window of  PLIP.  If

PLIP  does  not  run  or  shows problems transmitting data

first try deleting the PLIP.config file on  all  connected

machines,  so  PLIP  uses  the internal default values. If

that doesn't help, please contact us (see end of  documen-

tation).

CABLE

You  will  need  male  (A1000: female) DB25 connectors and

some (up to 4 meters) parallel cable. Connect:

D0-D7/BUSY/POUT   to   D0-D7/BUSY/POUT

ACK               to   STROBE

STROBE            to   ACK

GND               to   GND

You can look up the pin assignment in your Amiga User Man-

ual, appendix A.  Please note that this cable differs from

ParNet! Double check before using!

There is a potential problem with the RI (ring  indicator)

line  on  the  serial  port.  If you have problems running

PLIP, first try to remove  any  device  connected  to  the

serial  port.  If that helps, you might try cutting the RI

line in the serial connector.

COPYRIGHT

Under no circumstances may PLIP be  used  on  any  machine

which is used for the research, development, construction,

testing or production of weapons or other military  appli-

cations.  This also includes any machine which is used for

training persons for any of the above mentioned  purposes.

This  is  the  only restriction, in all other respects you

may do with PLIP what you want. Please  keep  us  informed

about any enhancements you make.

Use PLIP at your own risk.  Please send bug reports to:

Version 38.11               Release 4                           2

PLIP.DEVICE(1)                                     PLIP.DEVICE(1)

o.wagner@aworld-2.aworld.de

or

m.balzer@aworld.aworld.de

Please  exactly  state  the  machine  configuration  which

causes PLIP to fail.

HISTORY

V37.11

- small speedups in transmission handshaking

- now uses utility library for parsing the buffer

management taglist

V37.14

- optimized BPS and MTU

- cleaned up the code

V38.5

- fixed CRC check

- added information window

- some protocol fixes

V38.7

- changed hardware type to 13 (official PLIP ID)

- added configuration mechanism

V38.9

- some race conditions eliminated

- packet sequencing corrected

- added write retry mechanism (configurable)

- fixed possible AbortIO bug

- seems to work with Envoy 37.131, at least using

the default config...

V38.10

- fixed bug with configuration during startup

- default MTU reduced to 1024 bytes

V38.11

- added tracktype support (beta) in order to make

PLIP work in conjunction with AmiTCP

- removed autoinit for dos.library

Version 38.11               Release 4                           3