ASYNCHRONOUS FUNCTIONS FOR MICROSOFT, TURBO, OR ZORTECH C

The LIB(s) on the included disk are libraries of serial

communications functions designed specifically for use with

Microsoft C, Turbo C / C++, and Zortech C / C++.  The regis-

tered version has LIBs for all memory models.  The shareware

version is small model only.

--- Features of the library functions ---

o Support for 16550 UART's FIFO mode of operation

o Fully interrupt driven

o Baud rates up to 115,200 baud

o User defined transmit and receive ring buffer sizes.

o Ring buffers may be in placed in FAR memory even in small

and medium model programs.

o User defined port addresses, IRQ use, and interrupt vector

numbers.

o Support for simultaneous operation of 2 ports.

o Built in support for XON/XOFF and hardware flow control

o Block transmit and receive functions

o Receive buffer look ahead function

o 100% assembler code for maximum speed

o Totally compatible with Microsoft C, Turbo C, Zortech C,

and other C compilers that use standard Microsoft high

level language calling conventions.

---------------------------------------------------------------

Mike Dumdei,  6 Holly Lane,  Texarkana TX  75503

MCOMM5 (c) 1989, 1990, 1991  All Rights Reserved

---------------------------------------------------------------

REGISTRATION AND DISTRIBUTION POLICY

MCOMM5, the various versions of Smalterm, the ANSI video

routines, and other miscellaneous functions contained in the

COMM_x libraries are distributed as shareware.  If you use

these functions regularly please register them.

Two types of registration are available for MCOMM5.  The

first is $25 and includes assembled libraries for all memory

models.  The $25 registration does not include source code.

The second type of registration is $45 and includes libraries

and source code.  Previous versions were $25 for source code

and libraries.  If you send $25 and want source code based on

previous offers, you will receive a previous version of the

source.  Registered users of versions prior to version 5 may

upgrade their libraries for $3 or upgrade source code version

for $10.

Considerable time and effort has gone into developing and

debugging these routines.  If you are not a registered user and

have obtained a copy of the registered version, please take

time now to register the software or delete it.  The copy you

have is a registered version if it contains libraries for any

memory model other than small model or if any of the ASM source

code is included.

This software is provided as is without warranty as to its

fitness for a particular use or being free of errors.  I assume

no liability for any loss or damages you may incur through its

use.  I will, however, make every effort to correct any

software errors that may exist, and provide you with a working

version.

I will continue to assist registered and non-registered

users whenever possible.  If you have any problems using these

routines or questions, I may be contacted at:

North East Texas DataLink --  903 838-6713 (modem)

FIDO net:  1:19/128

Mike Dumdei               --  903 838-8307 (voice)

GETTING STARTED

All functions included in this library are listed on the fol-

lowing pages in alphabetical order.  The basic functions needed

to begin are: async_open, async_tx, async_rx, and async_close.

Functions are also available to support XON/XOFF and hardware

flow control, the 16550 UART, block transmit and receive,

receive buffer look ahead, and several other features.

All functions take a pointer to an ASYNC structure for one of

its arguments.  This structure is defined in COMM.H.  It

may be helpful to think of this structure as being similar to

the standard C library 'FILE *'.

If you have been using versions of these routines prior to

version 5.00, there have been some changes made that will

require you to make some modifications to any existing code

before it can be used with the new libraries.  The main

differences are:

> Async_open takes different parameters.  Also, instead

of the open function automatically allocating the ring

buffers for you, you provide the ring buffers.

> All functions now take a pointer to an ASYNC structure

as the first argument rather than 0 (COM1) or 1 (COM2).

> LITES is no longer supported.

NOTES ON 16550 FIFO MODE

The 16550 UART is automatically detected and enabled when

async_open is called.  If, for some reason, you want to operate

a 16550 UART in non-FIFO mode, automatic detection and use can

be disabled.  See async_open for more information.

--- Use of receiver FIFOs ---

The receiver trigger level may be programmed for 1, 4, 8, or

14 bytes by the async_FIFOrxlvl function.  When a port is first

opened the trigger is set to 1 byte.  There are 3 reasons why I

chose to default to a 1 byte receive trigger level.

1) While a higher trigger level results in less CPU

overhead, it also provides less overrun protection if the

CPU is slow to respond.  For example, with a 1 byte

trigger level the 16550 -- when it generates an interrupt

-- can hold 15 more characters in its FIFOs before the

CPU responds.  With a 14 byte trigger level it will only

have room for 2 more characters.

2) The MCOMM interrupt functions completely empty the

receiver FIFOs every time an interrupt occurs.  If the

CPU is slow to respond, the interrupt handler will

process 2,3,4.. characters (whatever is in the FIFOs)

whenever it does get control so the data still gets

buffered if the CPU isn't able to keep up.

3) Setting the trigger level at higher trigger levels when

displaying incoming data to the screen will result in a

jerky looking display, especially at low baud rates.

Again, this is because MCOMM empties the buffer

completely when it processes the interrupt.  With a

trigger level other than 1, data stacks up in the FIFOs

until the trigger level is reached.  When the interrupt

finally occurs, all the data in the FIFOs is pulled out

and made available to the higher level functions at

one time.  The result is your code sits for 4, 8, or 14

bytes (whatever the trigger is) and then puts those bytes

on the screen in a burst.  It looks rough at low baud

rates.  At 9600 baud and above you could probably get by

with a slightly higher trigger level.

For best use of the receive FIFOs, stick with 1 byte trigger

level when displaying data to the screen.  If your application

does not display the data or uses a high baud rate, you may

want to experiment with setting it to either 4 or 8 bytes.

--- Use of transmit FIFOs ---

The MCOMM transmit interrupt handler takes a parameter (set

by async_FIFOtxlvl) that is the maximum number of bytes to load

into the transmit FIFOs at one time.  The default value for

NOTES ON 16550 FIFO MODE

this parameter is 3 bytes.  The reason for not loading up the

FIFOs to maximum capacity (16 bytes) is flow control.  The

16550 UART will continue to transmit characters in its FIFOs

regardless of the state of CTS, DSR, or CD, or if an XOFF is

received.  It is totally up to the software to respond to

requests for flow control.  If the FIFOs are loaded with more

bytes than the receiver can handle when it invokes flow

control, the receiver gets overwritten.  There is a bit that

can be sent to the UART to flush the transmit FIFOs but then it

is impossible to tell what was flushed and what was sent.  The

solution is to not the load the FIFOs up so full.  If the

receiving device can handle more than 3 characters of overrun,

use a larger value.  If the application does not require flow

control, you can use a 16 byte level with no problems.

Another factor to consider concerning the transmit FIFO byte

level is that (from the NS Data Communications/LAN/UART

Handbook) the transmit FIFO empty indication will be delayed by

1 character time minus the last stop bit until there are at

least 2 bytes in the transmit FIFOs simultaneously for the

current transmit operation.  After the 2 byte requirement is

met the transmit interrupt occurs immediately when the transmit

FIFOs empty.  Using a value of 3 or above (1 byte for the

transmit shift register and 2 for the FIFO registers)

guarantees the interrupt delay is deactivated.  Once the '2

bytes in the FIFOs' requirement is met, the transmit byte level

can be reduced if necessary.

--- Problems with Western Digital 16550s ---

The following problems DO NOT apply to the National Semicon-

ductor version of the 16550.  Neither are they unique to MCOMM.

Western Digital 16550 UARTs have two problems you should be

aware of if your application will be ran on system that uses

this chip.

The first of these only shows up under the following

conditions:

1) The UART is in FIFO mode.

2) A low baud rate is being used.

3) The transmitter is initially empty.

4) A block of data longer than 16 to 18 bytes (depth of the

FIFOs plus 1 or 2 for what gets out while the FIFOs are

being filled up) is sent to the chip.

Under these conditions the WD16550 UART will momentarily fail

to recognize the FIFOs are full and continue to generate

transmit interrupts.  It also fails to reset the 'Transmit Hold

Register Empty' bit making it impossible for the software to

detect a full FIFO condition.  The result is the software keeps

sending data to the chip, overrunning the FIFOs.  What you will

NOTES ON 16550 FIFO MODE

see is, the first 16 to 18 bytes transmitted correctly, a

missing block of data, and then the remainder of the block also

correct.  This can be verified by trying to send blocks of data

through this chip (WD16550) at a low baud rate with any

software that enables the FIFOs.  How low of a baud rate is low

enough to cause the problem depends on the particular chip and

how fast the CPU and software can supply data to the chip.

With MCOMM, a 20 MHZ 386, and the TxByteCnt at the default

level of 3, the chip I was testing would not work in FIFO mode

below 9600 baud.  On the same system using different software,

it worked as low as 4800 baud.  On a 12 MHZ 286, MCOMM would

work at 4800 and above while the other software worked at 2400

and above.

For publicly distributed applications, I recommend you do one

or more of the following:

1) Provide an option in your program to allow the end user

to operate a 16550 with the FIFOs disabled.  When the

FIFOs are disabled, all baud rates work.  ORing the

ComBase parameter of async_open with 0x4000 will force

FIFO mode off.

2) Provide an option in your program to allow setting the

TxByteCnt level (async_FIFOtxlvl) to a lower value.  This

will help slightly.

3) Put a flag in your program the end user can set to let

you know you are dealing with a WD16550.

4) State your application does not support the Western

Digital version of the 16550 at low baud rates.

The second problem with the WD16550 is that if the 'Clear to

Send' signal is not connected it can, under some conditions,

cause the 'Delta Clear to Send' bit in the Modem Status

Register to become set and it will not reset.  I did not

research the problem enough to know if the 'locked bit' only

occurs when MSR interrupts are enabled, whether other floating

MSR input signals can cause the same problem, if all WD16550's

have the problem, or that is unique to the Western Digital chip

(considering the FIFO overrun problem, I assume it is unique).

With a properly functioning UART, reading the MSR register will

reset all the 'delta bits' and clear any interrupts.  With the

chip I had, I tried everything I could think of to clear the

interrupt and nothing worked except the ON/OFF switch.  I tried

clearing interrupts and reading the MSR multiple times checking

to see if the bit reset with interrupts off (it didn't),

disabling Modem Status interrupts via the Interrupt Enable

Register and then reading the MSR register (didn't work), FIFO

mode on / FIFO mode off (no difference), keyboard reboot of the

computer (didn't fix it), shut the computer off and turn it

back on (that fixed it).

NOTES ON 16550 FIFO MODE

After encountering this problem and having the system lock up,

I added code to MCOMM's interrupt driver so that it would quit

trying to clear an interrupt after 1000 attempts.  It then sets

a new port structure member 'IERVal' to zero as an indicator

the port is locked up.  Normally 'IERVal' will be set to the

current value of the Interrupt Enable Register.  If you try to

open a port that is in a stuck interrupt condition, async_open

now returns R_UARTERR.  In addition, options were added to open

a port without enabling Modem Status interrupts or Line Status

interrupts or both.  ORing the ComBase parameter of async_open

with 0x2000 disables MSR interrupts.  ORing with 0x1000

disables LSR interrupts.  So far I have encountered no reason

for disabling LSR interrupts.  Disabling MSR interrupts

prevents the lock up problem but has the side effect of causing

all functions and variables that use Modem Status signals to be

invalid.  This includes hardware handshake and carrier detect.

I have probably over emphasized the Modem Status lock up

problem and my personal opinion is that it is not that serious.

I considered totally disregarding it since it has a simple

solution and that is to connect all Modem Status input signals

to some driving signal (wire the RS232 cable right).  Unused

signals can be connected to a driving signal on the computer

end if necessary.  Also it may be isolated to one defective

chip.  I tested two WD16550s and the problem only showed up on

one of them during my testing.  Because the computer totally

locked up when this happened though, I decided to go ahead and

program for the possibility.  At least control of the computer

is regained and you can tell the user he needs a properly wired

RS232 cable or his UART is broke and then exit gracefully.

This will probably show up very very rarely so don't be too

concerned about it.  In closing, I would like to point out

again that these problems are a fault of the WD16550 and are

not unique to MCOMM.

async_16550

---------------------------------------------------------------

Purpose: Check if 16550 UART was detected in system.

Format:  int async_16550(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int is_16550;

is_16550 = async_16550(port);

Returns: Returns zero if a 16550 is not in the system or the

'ignore 16550 bit' was set when async_open was called.

Returns NZ if a 16550 was found.  Async_open must be

called before this function is valid.  If a 16550 is

found, it will be placed in the FIFO mode of operation

by async_open unless '16550 detect' was overridden or

the force FIFOs off option was selected.

Remarks: This function is implemented as a macro.

See async_open.

async_breakrxd

---------------------------------------------------------------

Purpose: Checks if a break signal has been received.

Format:  int async_breakrxd(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int break_status;

break_status = async_breakrxd(port);

Returns: If a break signal has been received the function

returns a non-zero value.  If a break has not been

detected, zero is returned.

Remarks: Once a break signal has been detected, this function

continually returns non-zero values until async_reset

is called.  If a break is received, a line status

register interrupt is generated.  The interrupt

handler sets a bit in the Stat1 port structure member.

The async_breakrxd function is a macro that checks the

status of this bit.

See also async_reset, async_rx, async_stat,

async_sndbrk.

async_carrier

---------------------------------------------------------------

Purpose: Check for the presence of a carrier.

Format:  int async_carrier(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int carrier_status;

carrier_status = async_carrier(port);

Returns: A non-zero value is returned if a carrier is present.

If the carrier detect line is LOW, zero is returned.

Remarks: This function is implemented as a macro.  See also

async_rx and async_stat.

async_close

---------------------------------------------------------------

Purpose: Closes a port opened by async_open.

Format:  int async_close(port);

ASYNC *port;    Pointer to ASYNC structure

Example:

ASYNC port;

async_close(&port);

Returns: No return value

Remarks: Do not attempt to close an unopened port.  Some

minimal checking is done internally by async_close in

an attempt to detect whether or not the specified port

is actually opened.  If an unopened port makes it past

those checks and the main close routine is entered,

the system may lock up (immediately, 2 hours later, or

never).  When a port is closed the UART registers,

interrupt controller mask, and interrupt vectors are

restored to the value they had when async_open was

called.  It is possible to close the port and leave

various signals (DTR for one) in the state you desire

rather than in their initial state.  See async_dtr and

async_rts for some examples on how to do this.

The B_ORGFIFO bit in the ASYNC 'Stat3' structure

member is used by async_close to return the 16550 --

if present -- to the mode it was in when async_open

was called.  FIFOs may be 'forced' to remain enabled

or 'forced' to be disabled when closing the port by

first setting or resetting this bit.  Ex:

Force off:    port->Stat3 &= ~B_ORGFIFO;

async_close(port);

Force on:     port->Stat3 |= B_ORGFIFO;

async_close(port);

async_cts

---------------------------------------------------------------

Purpose: Check status of the Clear to Send signal.

Format:  int async_cts(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int cts_status;

cts_status = async_cts(port);

Returns: Returns zero if Clear To Send is LOW.  If CTS is HIGH,

a non-zero value is returned.

Remarks: This function is implemented as a macro.

async_dsr

---------------------------------------------------------------

Purpose: Check status of the Data Set Ready signal.

Format:  int async_dsr(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int dsr_status;

dsr_status = async_dsr(port);

Returns: Returns zero if Data Set Ready is LOW.  If DSR is

HIGH, a non-zero value is returned.

Remarks: This function is implemented as a macro.

async_dtr

---------------------------------------------------------------

Purpose: Set or reset the Data Terminal Ready signal.

Format:  void async_dtr(port, DTRflag);

ASYNC *port;       Pointer to ASYNC structure

int DTRflag;       Set/Reset DTR flag

Example:

ASYNC *port;

if (WantToDropDTR)

async_dtr(port, 0);

else if (WantDTRhigh)

async_dtr(port, 1);

Returns: No return value

Remarks: This signal is set HIGH when a port is first opened.

When the port is closed, it is restored to the cond-

ition it was in when the port was opened.  To force

DTR low when a port is closed, regardless of its state

when the port was opened, AND the port member, OldMCR,

with NOT B_DTR.  To force DTR to remain high when the

port is closed, OR OldMCR with B_DTR.

Force low:    port->OldMCR &= ~B_DTR;

async_close(port);

Force high:   port->OldMCR |= B_DTR;

async_close(port);

async_FIFOrxlvl  <5.20 addition>

---------------------------------------------------------------

Purpose: Set the 16550 UART's receive trigger level.

Format:  void async_FIFOrxlvl(port, trigger_lvl);

ASYNC *port;       Pointer to ASYNC structure

int trigger_lvl;   Rx FIFO interrupt trigger level

Example:

ASYNC *port;

/* set the receiver trigger level to 8 bytes */

async_FIFOrxlvl(port, 8);

Returns: No return value

Remarks: Valid values for 'trigger_lvl' are 1, 4, 8, and 14

bytes.  When a port with a 16550 UART is first opened,

the trigger level is set to 1 byte.  See the

discussion on 'Use of receiver FIFOs' at the beginning

of this document.  This function has no effect if the

FIFOs were not enabled when async_open was called.

async_FIFOtxlvl  <5.20 addition>

---------------------------------------------------------------

Purpose: Set the number of bytes loaded into the 16550 UART's

transmit FIFOs when a transmit interrupt occurs.

Format:  void async_FIFOtxlvl(port, bytes);

ASYNC *port;       Pointer to ASYNC structure

int bytes;         # of bytes to load into tx FIFOs

Example:

ASYNC *port;

/* set the transmit FIFO byte level to 16 bytes */

async_FIFOrxlvl(port, 16);

Returns: No return value

Remarks: Valid values for 'bytes' are 1 through 16 bytes.  When

a port with a 16550 UART is first opened, the transmit

level byte count is set to 3 bytes.  See the

discussion on 'Use of transmit FIFOs' at the beginning

of this document.  This function has no effect if the

FIFOs were not enabled when async_open was called.

async_ignerr

---------------------------------------------------------------

Purpose: Set or reset 'ignore characters with errors' bit

Format:  void async_ignerr(port, flag);

ASYNC *port;       Pointer to ASYNC structure

int flag;          Set/Reset flag

Example:

ASYNC *port;

if (WantToDiscardErrChars)

async_ignerr(port, 1);

else if (WantToKeepErrChars)

async_ignerr(port, 0);

Returns: No return value

Remarks: This function sets the bit that determines what hap-

pens to received characters that have parity or fram-

ing errors.  If the function is called with the flag

set to one, incoming characters that have framing or

parity errors are discarded -- they do not get put

into the receive ring buffer.  If the flag is 0, char-

acters with errors are not ignored, they are put into

the ring buffer.  The error bits in Stat1 that reflect

these error conditions are set regardless of the

setting of the discard flag.  When a port is first

opened, the flag is set to 0.  This function is imple-

mented as a macro.

See also async_stat.

async_msr

---------------------------------------------------------------

Purpose: Get the contents of the Modem Status Register.

Format:  int async_msr(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int MSRcontents;

MSRcontents = async_msr(port);

Returns: The contents of the Modem Status Register.

Remarks: This function is a macro that returns the port struc-

ture member, MSRVal.  This value is initialized when a

port is first opened and then updated any time a modem

status interrupt occurs.  The reason the MSR is not

read directly is to avoid inadvertently intercepting a

pending MSR interrupt.  All bits returned except for

the delta bits reflect the true current status.

async_msrflow  <5.50 modified>

---------------------------------------------------------------

Purpose: Enables or disables hardware flow control.

Format:  void async_msrflow(port, flowmask);

ASYNC *port;       Pointer to ASYNC structure

int flowmask;      Bit mapped flow control mask

Example:

ASYNC *port;

if (enable_RTS_CTS_handshake)

async_msrflow(port, B_CTS);

else if (disable_handshake)

async_msrflow(port, 0);

Returns: No return value

Remarks: The flow mask is a bit mapped value of the signal that

is monitored by the receiver when performing flow

control.  Bits that may be monitored are B_CTS, B_DSR,

and B_CD.  This provides capability for RTS/CTS hand-

shake, DTR/DSR handshake, and handshaking with the

carrier detect line.  To monitor multiple signals, OR

the bits together.  Both hardware flow control and

XON/XOFF flow control may be enabled at the same time.

Beginning with version 5.50, ORing B_RTS into the

'flowmask' parameter, will cause the RTS signal to be

driven low whenever the receive buffer exceeds the

XoffTrip point.  RTS will automatically return to its

normal condition when the receive buffer empties to a

point below the XonTrip level.  Both XOFF/XON and RTS

flow control share the same trip point variable but

may be activated separately or together.  Do not get

RTS and CTS confused.  Use CTS to keep the computer

from overrunning a device (e.g. sending to a modem

with a locked baud rate).  Use RTS to keep from

getting ran over by a device (receiving from a high

speed modem using a slow computer).

See async_xflow.

async_open  <5.40 modified -- see Remarks at end of function>

---------------------------------------------------------------

Purpose: Initializes the hardware, interrupt vector, and ASYNC

port structure for interrupt driven operation.

Format:

int async_open(port, base, irqmask, vctrnbr, params);

ASYNC *port;   Pointer to ASYNC structure

int base;      Base port address of UART chip

int irqmask;   8259 mask value for selected IRQ line

int vctrnbr;   Number of selected interrupt vector

char *params;  Baud, parity, data, & stop bits in

string format

Entry:   Before calling this function you must allocate ring

buffer memory and pre-initialize the following ASYNC

structure members:

RxSize : the size that you want the receive ring

buffer to be.  The memory allocated or set aside

for the receive ring buffer must be contiguous

with the memory used for the transmit  ring

buffer.  In most cases (RxSize + TxSize) bytes

will be allocated using some form of malloc.

A function to do this is included in the examples.

The combined size of RxSize + TxSize can not

exceed 32K - 1.

TxSize : the size of the transmit buffer in bytes.

As stated above, the transmit and receive buffers

for each individual port must be allocated as one

contiguous block.

RingSeg: the segment address of the memory allocated

for the receive and transmit buffers.  FAR memory

can be used for the receive and transmit buffers

by using your compiler library's version of FAR

malloc and putting the segment portion (high 16

bits) of the returned memory pointer in RingSeg.

If you are using a NEAR data model program and

want NEAR ring buffers, set RingSeg equal to 0.

With RingSeg set to zero, the open function will

use the DS segment register for RingSeg.  Using

FAR ring buffers causes no loss of performance.

RingOfst: the offset address of the memory allocated

for the receive and transmit buffers.  This value,

in conjunction with the RxSize and TxSize vari-

ables, is used to set up the receive and transmit

ring buffers.

async_open

---------------------------------------------------------------

THE FOLLOWING FUNCTION IS INCLUDED WITH MCOMM5 AND IS

RECOMMENDED FOR ALLOCATING RING BUFFER MEMORY AND

INITIALIZING THE ABOVE VARIABLES.

#if defined (__TURBOC__)         /* Turbo C */

#include <alloc.h>

#define _fmalloc farmalloc

#elif defined (__ZTC__)          /* Zortech C */

#include <dos.h>

#include <stdlib.h>

#define _fmalloc farmalloc

#else                            /* Microsoft C */

#include <malloc.h>

#endif

int AllocRingBuffer(

ASYNC *port,                  /* pointer to port structure */

int rxsize,      /* number bytes to use for receive buffer */

int txsize,     /* number bytes to use for transmit buffer */

int useFARmem)    /* flag set if using FAR mem for buffers */

{

unsigned long memptr;

int memsize;

memsize = rxsize + txsize;

if (useFARmem || sizeof(char *) == 4)/* if FAR Ring bufs */

memptr = (unsigned long)_fmalloc(memsize);

else                  /* if Ring buffers use NEAR memory */

memptr = (unsigned long)(unsigned int)malloc(memsize);

/* pre-initialize 4 required structure members */

port->RxSize = rxsize;            /* receive buffer size */

port->TxSize = txsize;           /* transmit buffer size */

port->RingSeg = (int)(memptr >> 16);          /* SEG adr */

port->RingOfst = (int)memptr;            /* OFST address */

if (memptr == 0L)

return 0;          /* return 0 if no memory available */

return 1;                   /* return 1, had some memory */

}

async_open

---------------------------------------------------------------

Example 1:

/*

Open COM1 at 2400,N,8,1 using FAR ring buffers. COM1 is

defined in COMM.H as:  0x3f8, 0x10, 12.  It is not

necessary to check the return value of AllocRingBuffer

since async_open will return R_NOMEM if the memory

allocation function fails.

*/

#include "comm.h"

ASYNC port;

int rcode;

AllocRingBuffer(&port, 24000, 8000, 1);

rcode = async_open(&port, COM1, "2400N81");

/**  or

rcode = async_open(&port, 0x3f8, 0x10, 12, "2400N81");

**  or

rcode = async_open(&port, 0x3f8, IRQ4, VCTR4, "2400N81");

**/

if (rcode != R_OK)

{

printf("\nAsync_open failed, exit code = %d", rcode);

exit(rcode);

}

async_open

---------------------------------------------------------------

Example 2:

/*

Open a port located at 0x2e0 that uses IRQ3.  In this

case NEAR ring buffers are specified and the memory for

the port structure is obtained using malloc.  See the

AllocRingBuffer function described previously.

*/

#include "comm.h"

ASYNC *port;

int rcode;

if ((port = (ASYNC *)malloc(sizeof ASYNC)) == NULL)

{

printf("\nNot enough memory");

exit(R_NOMEM);

}

AllocRingBuffer(port, 4096, 1200, 0);

rcode = async_open(port, 0x2e0, IRQ3, VCTR3, "115200E71");

if (rcode != R_OK)

{

printf("\nAsync_open failed, exit code = %d", rcode);

exit(rcode);

}

async_open

---------------------------------------------------------------

Example 3:

/*

Open both COM1 and COM2 at the same time.  You can open

two ports at once as long as they don't share the same

IRQ line or interrupt vector.

*/

#include "comm.h"

ASYNC ports[2], *port[2];

int rcode;

port[0] = &ports[0], port[1] = &ports[1];

AllocRingBuffer(port[0], 10240, 10240, 1);

rcode = async_open(port[0], COM1, "9600N81");

if (rcode != R_OK)

{

printf("\nAsync_open failed, exit code = %d", rcode);

exit(rcode);

}

AllocRingBuffer(port[1], 10240, 10240, 1);

rcode = async_open(port[1], COM2, "1200E71");

if (rcode != R_OK)

{

printf("\nAsync_open failed, exit code = %d", rcode);

exit(rcode);

}

async_open

---------------------------------------------------------------

Example 4:

/*

Open COM1 using its current baud, parity, data, and

stop bit settings.  The port structure member, BPDSstr,

will reflect those settings after calling async_open.

*/

#include "comm.h"

ASYNC *port;

int rcode;

/* alloc port structure & ring buffer memory */

port = malloc(sizeof(ASYNC));

AllocRingBuffer(port, 4096, 2048, 1);

/* empty parameter string causes port to be opened at

its current settings  */

rcode = async_open(port, COM1, "");

if (rcode != R_OK)

{

printf("\nAsync_open failed, exit code = %d", rcode);

exit(rcode);

}

printf(

"\nBaud, parity, data, stop bits = %s\n", port->BPDSstr);

async_open

---------------------------------------------------------------

Returns:

R_OK (0) - the port was opened with no errors.

R_NOMEM - a NULL pointer was passed as the address

of the ring buffers.  Probable causes are either

failing to initialize the port structure ring

buffer variables or assigning the RingSeg and

RingOfst structure a NULL pointer.  This could

occur if you use the AllocRingBuffer function or

something similar and the call to malloc fails.

R_BAUDERR - the passed baud rate in the BPDS string

was invalid.  Some possible reasons for this error

are leading spaces in the parameter string or use

of commas.  "115200N81" is correct. " 115200N81"

and "115,200N81" are both incorrect.

R_PARITYERR - an invalid character was found in the

parameter string where the parity should have

been.  Commas or spaces in the parameter string

are possible causes.  Valid parity settings are N

- none, E - even, or O - odd.  See R_BAUDERR.

R_DTABITERR - an invalid number of data bits was spec-

ified.  Only settings of 7 or 8 are supported.

R_STPBITERR - an invalid number of stop bits was spec-

ified.  Stop bits must be set to either 1 or 2.

R_IRQUSED - an attempt was made to open two ports that

use the same IRQ line.

R_VCTRUSED - an attempt was made to open two ports

that use the same interrupt vector.

R_NOPORT - an attempt was made to open more than 2

simultaneous ports.

R_PORTUSED - an attempt was made to open two ports

that have the same I/O address.

R_UARTERR - an attempt was made to open a port that is

stuck in an interrupt condition (see the notes on

problems with the WD16550 at the beginning of this

documentation ).

async_open

---------------------------------------------------------------

Remarks (16550 support):

This function now automatically detects the presence

of a 16550 UART and enables FIFO operation if one is

detected.  By ORing either 0x8000 or 0x4000 with the

ComBase argument, you can change the default of auto-

enabling FIFO mode.  If 0x8000 is ORed with the

ComBase argument (ex: 0x8000|0x3f8), no attempt is

made to detect a 16550 and no changes are made to the

mode of operation of the 16550 if one is present.  If

this bit is set, none of the functions or bits

relating to the 16550 UART will be valid.  If 0x4000

is ORed with the ComBase, the 16550 will be forced

into non-FIFO mode.

ORing 0x2000 with the ComBase parameter will open the

port with MSR interrupts disabled.  ORing the ComBase

parameter with 0x1000 will open the port with LSR

interrupts disabled.  See the notes on the WD16550 at

the beginning of this documentation for why this may

be desired and the side effects of opening a port with

MSR interrupts disabled.

When async_open is called, the FIFOs are momentarily

disabled and then re-enabled with the receive trigger

level set to 1 byte.  In versions of MCOMM prior to

5.40, the FIFOs were not reset if they were already

on.  Resetting the FIFOs was added to correct

occasional rare problems with the port not opening.

/* detect 16550, enable FIFOs if present */

async_open(PortPtr, 0x3f8, IRQ, Vector, Params);

/* do not attempt to detect 16550 */

async_open(PortPtr, 0x8000|0x3f8, IRQ, Vctr, Params);

/* detect 16550, disable FIFOs if present */

async_open(PortPtr, 0x4000|0x3f8, IRQ, Vctr, Params);

/* open port, disable MSR interrupts */

async_open(PortPtr, 0x2000|0x3f8, IRQ, Vctr, Params);

/* open port, disable MSR interrupts and FIFOs */

async_open(PortPtr, 0x6000|0x3f8, IRQ, Vctr, Params);

async_open

---------------------------------------------------------------

Remarks (versions previous to 5.00):

There are some major differences in the async_open

function in MCOMM5 and previous versions of the rou-

tines.  The two most major ones are that the parame-

ters passed to the function are different and it is

now necessary to allocate the memory for the ring

buffers before calling the function.  To make things

simpler, COM1 and COM2 are defined in COMM.H and the

function, AllocRingBuffer, is provided.  The COM1 and

COM2 definitions satisfy the requirements for the 2nd,

3rd, and 4th parameters required by the function.  For

ports other than the standard COM1 and COM2, it is

necessary to list the individual arguments.  The func-

tion, AllocRingBuffer, is provided in C source form on

the disk and included in the COMM_x libraries.  This

function allocates the ring buffer memory and initial-

izes the required port structure members.

The return values for async_open are different and may

have a different meaning than in earlier versions.

If a port is opened with an empty parameter string

(using "" for the parameters), then the port is opened

at its current settings and the port member BPDSstr is

set to the string representation of those values.

Async_open no longer checks the BIOS ram area at 40:0

to verify that a port actually exists.  There were too

many compatibles that did not store the proper values

there.

The maximum combined size of the receive and transmit

buffers for each port is 32K - 1.

FAR ring buffers in small and medium memory model

programs is now supported.  Use of FAR buffers does

not in any way affect the speed of the routines.

async_peek

---------------------------------------------------------------

Purpose: Indexed receive ring buffer look ahead function.

Format:  int async_peek(port, index);

ASYNC *port;       Pointer to ASYNC structure

int index;         Index value

Examples:

ASYNC *port;

char ch, buf[10];

int i;

/* peek next char in receive ring buffer */

ch = async_peek(port, 0);

/* peek next 10 chars in receive ring buffer */

for (i = 0; i < 10; i++)

buf[i] = async_peek(port, i);

Returns: This function returns the character in the receive

ring buffer at the indexed position.  If an attempt is

made to peek deeper in the ring buffer than there are

characters in the ring buffer, -1 (0xffff) is

returned.

Remarks: No characters are removed from the ring buffer by this

function.

The index value is 0 based.

To avoid confusing the character, '\0xff', from the

error return value, 0xffff, use an integer size

variable for the return value.  The character '\0xff'

is returned as 0x00ff and the error condition is

returned as 0xffff.

async_regs

---------------------------------------------------------------

Purpose: Provides direct access to the UART registers.

Format:  int async_regs(port, reg_ofst, value);

ASYNC *port;       Pointer to ASYNC structure

int reg_ofst;      Offset of register from base adrs

int value;         Value to write or -1 if reading

Examples:

ASYNC *port;

int LCRvalue;

/* read the line control register */

LCRvalue = async_regs(port, LCRreg, RDreg);

/* write a 0 to the modem control register */

async_regs(port, MCRreg, 0);

Returns: When reading a register, the return value is the value

of the UART register that was read.  When writing a

register, the return value is undefined.

Remarks: The following values for register offsets are defined

in COMM.H for use with async_regs:

RXreg, TXreg - receive & transmit regs, value = 0

IERreg - interrupt enable register,     value = 1

IIDreg - interrupt identification reg,  value = 2

FCRreg - FIFO control register          value = 2

LCRreg - line control register,         value = 3

MCRreg - modem control register,        value = 4

LSRreg - line status register,          value = 5

MSRreg - modem status register,         value = 6

LObaud - offset of low baud divisor,    value = 0

HIbaud - offset of high baud divisor,   value = 1

Also RDreg is defined as -1 for use when reading one

of the UART registers.  See the examples.  See

cautions on the following page before using this

function.

async_regs

---------------------------------------------------------------

Remarks (continued):

This function is provided primarily as a debugging

tool.  Reading or writing to the UART registers while

operating in an interrupt driven mode can cause

several types of problems.  Do not use this function

unless you know what you are doing.  Do not read the

RXreg, IIDreg, LSRreg, or MSRreg on an open port

except for debugging purposes.  Do not write any of

the registers.

The function may be used on a port that has not yet

been opened if the ComBase member of the port struc-

ture has been set.  When async_regs is used with an

unopened port the UART registers may be read or writ-

ten without restriction.

async_reset

---------------------------------------------------------------

Purpose: Clear parity error bit, framing error bit, character

overrun bit, receive buffer overflow bit, and break

signal received bit in the the Stat1 byte.

Format:  void async_reset(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

if (error)

async_reset(port);

Returns: No return value

Remarks: This function is implemented as a macro.  See also

async_rx and async_stat.

async_restart

---------------------------------------------------------------

Purpose: Re-initialize an already opened serial port

Format:  void async_restart(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

async_restart(port);

Returns: No return value

Remarks: Async_restart resets the interrupt vector, UART regis-

ters, and 8259 interrupt controller mask to the values

necessary for interrupt driven operation.  It also

flushes the receive and transmit buffers and resets

the Stat1 byte.  Use this function to restart a comm

port after spawning a program that may have altered

the interrupt vector, UART regs, or interrupt mask.

See also async_stop.

async_rts

---------------------------------------------------------------

Purpose: Sets or resets the Request to Send signal.

Format:  void async_rts(port, flag);

ASYNC *port;       Pointer to ASYNC structure

int flag;          Set/Reset RTS flag

Example:

ASYNC *port;

if (WantToDropRTS)

async_rts(port, 0);

else if (WantRTShigh)

async_rts(port, 1);

Returns: No return value

Remarks: The RTS signal is primarily used for hardware hand-

shaking.  When a port is first opened RTS is set HIGH

(enabled).  When the port is closed, it is restored to

the condition it was in when the port was opened.  To

force RTS low when closing a port, regardless of its

state when the port was opened, AND the port member,

OldMCR with NOT B_RTS.  To force RTS to remain high

when the port is closed, OR OldMCR with B_RTS.

Force low:    port->OldMCR &= ~B_RTS;

async_close(port);

Force high:   port->OldMCR |= B_RTS;

async_close(port);

async_rx

---------------------------------------------------------------

Purpose: Gets one character from the receive ring buffer.

Format:  int async_rx(port);

ASYNC *port;       Pointer to ASYNC structure

Examples:

ASYNC *port;

int rxch;

rxch = async_rx(port);       /* rxch == Stat1/char */

rxch = async_rx(port) & 0xff;      /* rxch == char */

/* chk if char available & process only if it was */

if (!((rxch = async_rx(port)) & B_RXEMPTY)

procRxdChar(rxch & 0xff);

Returns: Async_rx returns the character received as the low

byte and the Stat1 status byte as the high byte.  If

the receive ring buffer was empty when async_rx was

called the low byte will be 0 and the high byte will

have the B_RXEMPTY bit set.  The Stat1 byte is a bit

mapped value that reflects parity errors, framing

errors, character overrun errors, receive buffer

overflow errors, receive buffer empty, break signal

received, and carrier lost.  If no receive errors

have occurred, a break signal has not been received,

and a carrier is present, Stat1 will be 0.

async_rx

---------------------------------------------------------------

Remarks: Error bits set in the Stat1 byte are not necessarily

associated with the character just taken out of the

receive ring buffer.  In fact, they probably are not.

The error bits are set when the error is detected and

reset when async_reset is called.  For example:

1) 20 bytes of data are received with no errors.

2) Async_rx has been called 10 times so 10 characters

have been taken out of the ring buffer and 10 of

the bytes with no errors are still in the buffer.

3) A parity error occurs on the next byte received

from the UART -- the 11th byte in the receive ring

buffer.

4) The next byte taken out of the buffer, which is

the first of the 10 remaining good bytes will have

the parity error bit in the Stat1 byte.  It will

remain set until async_reset is called.

If the 'ignore characters with errors' bit is set (see

async_ignerr), then characters that have either parity

or framing errors are never placed in the receive ring

buffer.  They are read from the UART and discarded.

The error bits in Stat1 are set regardless of the

setting of the 'ignore characters with errors' bit.

async_rxblk

---------------------------------------------------------------

Purpose: Retrieves a block of characters from the receive ring

buffer.

Format:  int async_rxblk(port, buf, maxchars);

ASYNC *port;       Pointer to ASYNC structure

char *buf;         Buffer for received data

int maxchars;      Maximum number of chars to read

Example:

ASYNC *port;

char buf[256];

int bytesRead;

bytesRead = async_rxblk(port, buf, sizeof(buf));

Returns: Async_rxblk returns the number of bytes read.  This

value will be 'maxchars' if there are that many bytes

available in the receive buffer.  If there are not

'maxchars' in the receive buffer, the entire contents

of the receive buffer will be moved to 'buf' and the

return value will be the number of bytes that were in

the buffer.

Remarks: Very fast way to read data.  See also async_rx and

async_rxblkch.

async_rxblkch

---------------------------------------------------------------

Purpose: Retrieve bytes from the receive ring buffer until the

specified character is found, 'maxchars' are read, or

the receive ring buffer is emptied.

Format:

int async_rxblkch(port, buf, maxchars, keychar, includekey);

ASYNC *port;      Pointer to ASYNC structure

char *buf;        Destination for bytes read

int maxchars;     Maximum number of chars to read

char keychar;     Character to scan for

int includekey;   Flag set if 'keychar' is to be read also

Example:

ASYNC *port;

char buf[256];

int bytesRead;

int foundkey;

bytesRead = async_rxblkch(port, buf, sizeof(buf), '\n', 1);

if (bytesRead < 0)

bytesRead = -bytesRead, foundkey = TRUE;

else

foundkey = FALSE;

Returns: This function returns the number of bytes read if the

'keychar' is not found.  If the 'keychar' is found the

2's complement of the number of bytes read is

returned.

async_rxblkch

---------------------------------------------------------------

Remarks: The 'includekey' flag, which is the last argument of

the function, determines whether or not the 'keychar'

is to be included if it is found.  If the flag is 1

and the 'keychar' is found it will be the last char-

acter in the user buffer.  If this flag is 0 and the

'keychar' is found, the 'keychar' is left in the re-

ceive ring buffer and will be the first character read

when you next call one of the async_rx type functions.

This can cause a problem if you set this flag to 0 and

the 'keychar' is the first character in the receive

buffer.  The function will return 0, which would seem

to indicate that there were no bytes in the buffer.

Actually the character was found but since you are not

taking it out of the ring buffer zero bytes are read.

There are two ways to check for this condition.  You

can first call async_rxcnt to verify there are bytes

in the receive buffer and then call async_rxblkch.  If

async_rxblkch returns 0, the 'keychar' is there and is

the first byte in the buffer.  The other method is to

use async_peek to see what the next character in the

receive buffer is before calling async_rxblkch.

A 2's complement value of the bytes read is returned

by async_rxblkch when the 'keychar' is found.  For

actual bytes use:

bytes = async_rxblkch( ....);

if (bytes < 0)

bytes = -bytes;

A byte count less than zero is the signal the

'keychar' was found.  (Except for the circumstance

noted in the previous paragraph).

See async_rx, async_rxblk, async_rxcnt, async_peek.

async_rxcnt

---------------------------------------------------------------

Purpose: Returns the number of bytes in the receive ring

buffer.

Format:  int async_rxcnt(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

BytesInRxBuf = async_rxcnt(port);

Returns: Number of bytes in the receive ring buffer

Remarks: This function is implemented as a macro.

async_rxerr

---------------------------------------------------------------

Purpose: Checks if a receive error has been detected.

Format:  int async_rxerr(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int rxerr_status;

rxerr_status = async_rxerr(port);

Returns: ZR if no receive errors have been detected, NZ if

receive error has been detected.

Remarks: Types of receive errors that are detected by this

function are parity errors, framing errors, UART

receive register overflow, and receive ring buffer

overflow.  This information is also returned as the

high byte of the async_rx and async_stat functions.

This function is implemented as a macro.

See also async_rx, async_stat.

async_rxflush

---------------------------------------------------------------

Purpose: Empties out the receive ring buffer.

Format:  void async_rxflush(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

async_rxflush(port);

Returns: This function does not have a return value

Remarks: Clears the receive ring buffer and -- if a 16550 is

present in the system with its FIFOs activated --

resets the receiver FIFOs.  All data that was in the

buffer is lost.  If XON/XOFF flow control is being

used and an XOFF has been sent to the remote, this

function automatically sends an XON.  An XON is not

ALWAYS sent -- only if XFLOW control is in use and the

async interrupt routine has sent an XOFF.  It will not

clear false XOFFs the remote may have received.

async_rxfree (5.40)

---------------------------------------------------------------

Purpose: Returns the number of bytes left available in the

receive ring buffer for incoming characters.

Format:  int async_rxfree(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

RxBufFreeSpace = async_rxfree(port);

Returns: Number of bytes that can be received without taking

any out before a receive buffer overflow will occur.

Remarks: This function is implemented as a macro.

async_setbpds

---------------------------------------------------------------

Purpose: Change the baud rate, parity, number of data bits or

number of stop bits on an already opened port.

Format:  int async_setbpds(port, params);

ASYNC *port;       Pointer to ASYNC structure

char *params;      Pointer to parameter string

Example:

ASYNC *port;

int rcode;

rcode = async_setbpds(port, "2400E71");

Returns: This function returns 0 if successful and the new

parameter string is copied to the port structure

member, BPDSstr.  Errors that can be returned are

R_BAUDERR, R_PARITYERR, R_DTABITERR, or R_STPBITERR.

If one of these error codes are returned, the port

settings are unchanged and the port member BPDSstr is

left unchanged.

See async_open for a more complete description of

these errors.

Remarks: If 7 data bits is selected the StripMask is set to

strip the high bit off incoming data.  If 8 data bits

are selected the StripMask is disabled.

Valid baud rates are from 50 to 115200 baud.  Parity

may be 'E'ven, 'N'one, or 'O'dd.  The number of data

bits must be 7 or 8 and the number of stop bits must

be 1 or 2.  Do not use spaces, commas, or any other

type of punctuation in the parameter string.

See also async_open, async_strip.

async_sndbrk

---------------------------------------------------------------

Purpose: Enable or disable a break signal

Format:  void async_sndbrk(port, Brkflag);

ASYNC *port;       Pointer to ASYNC structure

int Brkflag;       Set/Reset break signal flag

Example:

ASYNC *port;

/* send break for 50 msec */

async_sndbrk(port, 1); /* enable break signal */

delay(50MSEC);         /* your delay function */

async_sndbrk(port, 0); /* disable break signal */

Returns: No return value

Remarks: This function causes the UART to send a continuous

break signal if it is called with the flag set to 1.

It must be called again with the flag set to 0 to

disable the break signal.

async_stat

---------------------------------------------------------------

Purpose: Get the current status of a port

Format:  int async_stat(port, mask);

ASYNC *port;       Pointer to ASYNC structure

int mask;          Mask applied to return value

Examples:

ASYNC *port;

int status;

/* status = all status bits */

status = async_stat(port, 0xffff);

/* check for XOFF received condition */

if (async_stat(port, B_XRXD))

do_something();

Returns: Returns Stat1/Stat2 status bits after masking them

with the passed mask value.  Stat1 is the high byte

and Stat2 is the low byte of the return value.

Stat1 bit map:

Bit 7 = set if no carrier present

Bit 6 = receive ring buffer empty

Bit 5 = used by interrupt routines (always 0)

Bit 4 = break signal was detected

Bit 3 = framing error was detected

Bit 2 = parity error was detected

Bit 1 = character overrun error was detected

Bit 0 = receive ring buffer has overflowed

Stat2 bit map:

Bit 7 = carrier detect monitored for flow control

Bit 6 = XOFF rx'd or hardware flow control active

Bit 5 = data set ready monitored for flow control

Bit 4 = clear to send monitored for flow control

Bit 3 = transmit ring buffer is empty

Bit 2 = an XOFF has been sent by interrupt handler

Bit 1 = an XOFF has been received

Bit 0 = using XON/XOFF flow control

async_stat

---------------------------------------------------------------

Remarks: For some situations, it is more efficient to use one

of the other library functions targeted specifically

for the information you want.  For instance, instead

of calling:

async_stat(port, B_XRXD);

use:

async_xrxd(port);

The information in the Stat1 byte is also returned as

the high byte of the async_rx function.  Bits 0-4 of

Stat1 remain set until cleared by the async_reset

function.

Not all bits in the Stat2 and Stat1 port structure

members are kept updated on a constant basis so do not

try to check a condition by testing the Stat1 or Stat2

values in the ASYNC structure.  Use one of the docu-

mented functions or macros to be sure of getting the

correct result.

async_stop

---------------------------------------------------------------

Purpose: Temporarily disables interrupt driven operation of an

already opened port.

Format:  void async_stop(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int x, inhdl, outhdl, errhdl;

async_stop(port);               /* release port */

/* redirect console to async port */

x = open("COM1", O_RDWR|O_BINARY);

inhdl = dup(0);  outhdl = dup(1);  errhdl = dup(2);

dup2(x, 0);  dup2(x, 1);  dup2(x, 2);

/* run a redirected DOS shell */

spawnlp(P_WAIT, "COMMAND.COM", "COMMAND.COM", NULL);

/* restore stdin, stdout, stderr */

close(x);

dup2(inhdl, 0);   close(inhdl);

dup2(outhdl, 1);  close(outhdl);

dup2(errhdl, 2);  close(errhdl);

async_restart(port);   /* regain control of port */

Returns: No return value

Remarks: Call this function before spawning a program that will

be doing its own port I/O, such as an external commun-

ications program or a redirected DOS shell.  After the

other application has completed, call async_restart to

re-enable interrupt operation.

This function disables interrupts for the named port

by clearing UART's interrupt enable register and the

OUT2 bit of its modem control register.

See also async_restart.

async_strip

---------------------------------------------------------------

Purpose: Set the strip mask for received characters.

Format:  void async_strip(port, mask);

ASYNC *port;       Pointer to ASYNC structure

char mask;         Strip mask

Example:

ASYNC *port;

async_strip(port, '\x7f');

Returns: No return value

Remarks: The strip mask is automatically set to 0x7f when the

parameters are set for 7 data bits and to 0xff when 8

data bits are used.  All incoming data is ANDed with

the strip mask before it is placed in the receive ring

buffer.  This function is implemented as a macro.

async_tx

---------------------------------------------------------------

Purpose: Send 1 character to a port.

Format:  int async_tx(port, ch);

ASYNC *port;       Pointer to ASYNC structure

char ch;           Character to transmit

Example:

ASYNC *port;

/* send a CR/LF to a port */

async_tx(port, '\r');

async_tx(port, '\n');

Returns: Returns the number of bytes left available in the

transmit ring buffer unless the ring buffer was full

when the function was called.  R_TXERR is returned if

the transmit ring buffer was full and the character

that was to be transmitted is not placed in the ring

buffer.

Remarks: The error return value, R_TXERR, is a negative number.

The bytes free return value can never be negative.

See also async_txblk, async_txfree.

async_txblk

---------------------------------------------------------------

Purpose: Send a block of characters to a port.

Format:  int async_txblk(port, buf, count);

ASYNC *port;       Pointer to ASYNC structure

char *buf;         Pointer to block to transmit

int count;         Size of block in bytes

Examples:

ASYNC *port;

char buf[128];

static char *msg = "Send this message\r\n");

put_data_in_buf();

async_txblk(port, buf, sizeof(buf));

async_txblk(port, msg, strlen(msg));

Returns: Returns the number of bytes left available in the

transmit ring buffer unless the ring buffer would not

hold the entire block when the function was called.

R_TXERR is returned if there was no room for the

block and none of the bytes in the block are sent.

Remarks: This function is much faster than using repeated calls

to async_tx when multiple bytes are being transmitted.

The error return value, R_TXERR, is a negative number.

The bytes free return value can never be negative.

See also async_tx, async_txfree.

async_txcnt (5.40)

---------------------------------------------------------------

Purpose: Returns the number of bytes in the transmit ring

buffer.

Format:  int async_txcnt(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

BytesInTxBuf = async_txcnt(port);

Returns: Number of untransmitted bytes in the transmit ring

buffer.

Remarks: This function is implemented as a macro.  If a 16550

is in the system with its FIFOs active, there may be

up to 16 (depends on what async_FIFOtxlvl is set at --

the default is 3 bytes) more bytes of untransmitted

data in the 16550 FIFOs plus 1 byte in the UART

transmit hold register.  With no 16550 there may still

be up to 2 untransmitted bytes -- one in the UART's

transmit hold register and one in the UART's transmit

shift register.  To monitor for a transmit buffer

empty condition use async_txempty.

async_txempty

---------------------------------------------------------------

Purpose: Checks if transmit ring buffer is empty.

Format:  int async_txempty(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

/* wait for transmit ring buffer to empty out */

while (!async_txempty(port))

;

Returns: NZ if the transmit ring buffer is empty, ZR if the

buffer still has characters to be transmitted.

Remarks: This function is implemented as a macro.  If a 16550

is in the system with its FIFOs active, there may be

up to 16 (depends on what async_FIFOtxlvl is set at --

the default is 3 bytes) more bytes of untransmitted

data in the 16550 FIFOs plus 1 byte in the UART

transmit hold register.  With no 16550 there may still

be up to 2 untransmitted bytes -- one in the UART's

transmit hold register and one in the UART's transmit

shift register.

async_txflush

---------------------------------------------------------------

Purpose: Flushes the transmit ring buffer.

Format:  void async_txflush(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

async_txflush(port);

Returns: No return value

Remarks: Clears the transmit ring buffer and -- if a 16550

operating in FIFO mode is in the system -- resets the

transmit FIFOs.  All characters in the buffer that

have not been transmitted are lost.

async_txfree

---------------------------------------------------------------

Purpose: Returns the number of bytes left available in the

transmit ring buffer.

Format:  int async_txfree(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int tx_bytes_available;

tx_bytes_available = async_txfree(port);

Returns: Number of bytes of free space left in the transmit

buffer.

Remarks: The number of free bytes left in the transmit ring

buffer is also the return value of async_tx and

async_txblk.  This function is implemented as a macro.

async_xany

---------------------------------------------------------------

Purpose: Checks if accepting the first character after an XOFF

is received as the XON character.  DOES NOT enable any

character XON (use async_xflow).

Format:  int async_xany(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int rcode;

rcode = async_xany(port);

Returns: Not zero if first character after an XOFF is to be

treated as an XON.  Zero if a real XON is required to

clear an XOFF.

Remarks: This function is implemented as a macro.  See also

async_xflow.

async_xflow

---------------------------------------------------------------

Purpose: Enables/disables use of XON/XOFF flow control.

Format:  void async_xflow(port, flag);

ASYNC *port;       Pointer to ASYNC structure

int flag;          Enable/Disable & any char XON flag

Examples:

ASYNC *port;

async_xflow(port, B_XUSE); /* use flow control */

async_xflow(port, 0);      /* disable flow control */

/* use XON/XOFF flow control with any char being

accepted as an XON */

async_xflow(port, B_XUSE | B_XONANY);

/* enable XON/XOFF on transmit side only */

async_xflow(port, B_XUSET);

/* enable XON/XOFF on receive side only */

async_xflow(port, B_XUSER);

Returns: No return value

Remarks: Passing B_XUSET enables the transmission of XON/XOFF

characters when the receive buffer fills to the 'trip'

levels discussed in the following paragraph.  Incoming

XON/XOFFs are not regarded as flow control characters

and are stored in the ring buffer.  Passing B_XUSER,

enables recognition of received XON/XOFFs as flow

characters but does not enable transmission of

XON/XOFF at the trip levels.  If B_XUSE is passed,

both transmission and recognition of XON/XOFF as flow

characters is enabled (the same as B_XUSET | B_XUSER).

ORing B_XONANY with either B_XUSE or B_XUSER causes

the first character received after an XOFF to be

treated as the XON unless that character is another

XOFF.  Passing zero as the flag disables XON/XOFF flow

control.  The default is no flow control.

async_xflow

---------------------------------------------------------------

Remarks (continued):

The port structure member, XoffTrip, determines when

an XOFF will be sent.  Whenever the number of bytes

left available in receive buffer falls below XoffTrip,

an XOFF is sent.  The default value of XoffTrip is 25%

of the size of the receive buffer so an XOFF is sent

whenever the receive buffer is 75% full.

The port structure member, XonTrip, determines when an

XON will be sent.  Whenever the number of bytes left

available in the receive buffer rises above XonTrip,

an XON will be sent.  Its default value is 50% of the

size of the receive buffer.

The port structure member, XTxRptInit, determines how

many bytes may be received after an XOFF is sent

before another XOFF is sent.  Its default value is

6.25% of the size of the receive buffer or 160 bytes,

whichever is less.

XoffTrip, XonTrip, and XTxRptInit may all be changed

from their default values by directly accessing the

port structure.  To do this:

ASYNC *port;

/* send XOFF when 300 bytes left in rx ring buf */

port->XoffTrip = 300;

/* send XON when 350 bytes left in rx ring buf */

port->XonTrip = 350;

Be sure to keep the XonTrip level higher than the

XoffTrip level.  The values are for bytes left

available not bytes used.

The XTxRptInit value is used to automatically send

another XOFF in the event the first one was ignored.

Instead of sending an XOFF character everytime another

character is received, the interrupt routines wait for

'XTxRptInit' characters to be received and then repeat

the XOFF.  If this value is set to low some systems

that allow any character to be an XON will treat the

repeated XOFF character as an XON.

async_xoffclr

---------------------------------------------------------------

Purpose: Manually clears an XOFF received condition

Format:  void async_xoffclr(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

async_xoffclr(port);

Returns: No return value

Remarks: Async_xoffclr has the same effect as if the remote

system had sent an XON.  This will unstop the system

in the event a false XOFF is received or the remote's

XON was lost.  Use this function with caution.  Unless

something goes wrong, the remote will clear XOFF cond-

itions automatically by sending an XON.  If an XOFF is

manually cleared, the remote site may be overran and

lose data.  For use only when XON/XOFF flow control

has been activated.

See also async_xoffset.

async_xoffset

---------------------------------------------------------------

Purpose: Manually sets an XOFF received condition

Format:  void async_xoffset(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

async_xoffset(port);

Returns: No return value

Remarks: Forces the local site to act as though an XOFF had

been received.  Transmit interrupts are disabled until

the remote sends an XON or you call async_xoffclr.

For use only when XON/XOFF flow control has been

activated.

See also async_xoffclr.

async_xrxd

---------------------------------------------------------------

Purpose: Check if an XOFF has been received and has activated a

flow control halt.

Format:  int async_xrxd(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int rcode;

rcode = async_xrxd(port);

Returns: Not zero if an XOFF has been received and an XON has

not been received to clear it.  Returns zero if XOFF

flow halt is not currently active.

Remarks: This function is implemented as a macro.  See also

async_xoffclr, async_xtxd.

async_xtxd

---------------------------------------------------------------

Purpose: Check if an XOFF has been sent

Format:  int async_xtxd(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

int rcode;

rcode = async_xtxd(port);

Returns: Not zero if an XOFF has not been sent by the receive

interrupt routine and an XON has not been sent to

clear it.  Returns zero if an XOFF has not been sent.

Remarks: This function is implemented as a macro.  See also

async_xoffclr, async_xrxd.

async_xuse

---------------------------------------------------------------

Purpose: Check if XON/XOFF protocol is currently being used

Format:  int async_xuse(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

rcode = async_xuse(port);

Returns: Not zero if XON/XOFF protocol is being used on either

the transmit or receive sides, zero if XON/XOFF flow

is not being used at all.

Remarks: This function is implemented as a macro.  See

async_xflow, async_xuset, async_xuser.

async_xuser

---------------------------------------------------------------

Purpose: Check if XON/XOFF protocol is enabled on the receive

side (receiver recognizes XON/XOFF as flow control).

Format:  int async_xuser(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

rcode = async_xuser(port);

Returns: Not zero if XON/XOFF protocol is being used on the

receive side, zero if it is not.

Remarks: This function is implemented as a macro.  See

async_xflow, async_xuset, async_xuse.

async_xuset

---------------------------------------------------------------

Purpose: Check if XON/XOFF protocol is enabled on the transmit

side (transmitter sends XOFF/XON as receive buffer

bytes available crosses trip levels).

Format:  int async_xuset(port);

ASYNC *port;       Pointer to ASYNC structure

Example:

ASYNC *port;

rcode = async_xuset(port);

Returns: Not zero if XON/XOFF protocol is being used on the

transmit side, zero if it is not.

Remarks: This function is implemented as a macro.  See

async_xflow, async_xuser, async_xuse.