VOX-Mail v0.99 (evaluation)

---------------------------

(C)1994 written by Michael Fung

Email:     jf4y+@andrew.cmu.edu

Valid up till May 1994

Address:   1060 Morewood Ave, Box 1334

Pittsburgh, PA 15213-3827

U.S.A.

Telephone: 1-(412)8623692

In the summer (May-August)

Address:   Blk 14, #10-03

Ghim Moh Road

S(1027)

Republic of Singapore

Telephone: (65)4670761

What it is!

-----------

VOX-Mail is a voice mail system designed for the Atari Falcon 030

computer. It utilizes the sound hardware available in the Falcon for

playing as well as recording sound directly to hard disk in real time.

A hardware board interfaces the computer to the telephone line.

At present, VOX-Mail detects incoming calls, picks up the line, plays

customized announcements/tones and records messages to the hard disk

with line monitoring. In other words, it acts exactly like a good

answering machine, with messages stored digitally. These messages can

be loaded into a sample editing program, e.g. the System Audio

Manager.

Requirements

------------

VOX-Mail runs on an Atari Falcon 030 system in 640*480 (16 colors) or

higher video modes. The hardware plugs into the mic/speaker jacks, the

joystick port as well as the printer port. Power to the hardware is

supplied by the computer; a separate power supply unit is not

required.

VOX-Mail requires about 300K of free RAM and a hard disk is highly

recommended for recording samples. There should be at least 5Mb of

free disk space for recorded messages, since sound samples are

notoriously large. If you are expecting many messages, more free space

should be allocated.

Evaluation notice

-----------------

This is an evaluation version of the VOX-Mail system. You can launch

the program and tinker around. Try out the recording functions as well

as the playback of messages. For full operation, the hardware

extension is required. To have a listen to what VOX-Mail sounds like,

call my voice number listed above. (I have VOX-Mail running as my

answering machine!)

Some notes:-

- In the answering machine mode, you can simulate an incoming ring by

pressing the space bar. During playback/recording, pressing the left

mouse button will stop the playback/recording

- To record messages, connect a microphone to the MIC jack

- VOX-Mail utilises 8195 Hz sound samples (it doesn't make sense to

use higher speed samples as the phone lines are not high quality).

There is a convert sample function in VOX-Mail to allow you to convert

samples for use with VOX-Mail.

- the sound sample data files used for announcing the time should be

in the same directory as the announcement file

- in converting stereo sound samples, VOX-Mail takes only the right

channel since the telephone line is only monophonic

Limitations

-----------

Currently, VOX-Mail does not run perfectly under Multi-TOS. Recorded

messages are also not compressed. These will be improved in future

releases.

Shareware Notice

----------------

You are free to distribute the VOX-Mail program provided that it is

not modified in any way and the documentation files are intact.

VOX-Mail is shareware. If you like what you see (which I think you

would), fill out the registration form and mail it to the above

address. The registered version of the software, complete

documentation as well as the hardware will be mailed out to you upon

receipt of the registration.

If you have any questions or suggestions, please feel free to call me

at my voice number or send an email to me.

Ideas for the future

--------------------

- implementation of touch-tone detection

- scripting language for customizing of voice-mail system, including

features that allow for an information system with multiple menus,

password controlled access to pre-recorded information (e.g.

individual messages for specific people)

- remote recording of announcement/messages and remote recall of

recorded messages

- automatic call transfer by calling you up at your office (or any

number that you designate) and putting the caller on a 3-way

conversation. Automatic paging when you have a message on the machine

Registered users will receive free updates of the software. Depending

on the nature of the touch-tone detection implementation, a hardware

upgrade of reasonable cost might be necessary to utilize the features

that require touch-tone recognition.

Disclaimer

----------

In no event shall the author be responsible for any damage caused

directly or indirectly by the use of VOX-Mail and its hardware. By

using VOX-Mail, the user is deemed to have expressed agreement to this

disclaimer.

VOX-Mail's hardware extension has not been submitted for FCC approval.

However, the hardware has been designed to meet FCC requirements by

complete electrical isolation between the phone lines and the

computer, with interference kept to a minimum. Clamping circuitry is

also present to diffuse spikes of reasonable magnitude, preventing

damage to the computer and phone line. Its has been tested out

extensively and its use should not have any adverse effect on the

telephone lines.

My thang

--------

The idea of a voice-mail system has been in my head ever since I was

using the Atari Mega ST. However, the hardware on the older Atari

could not handle a voice-mail system at reasonable cost. With the

Falcon, the built-in sound hardware is excellent for the job and I set

out to build the system.

I have now developed a fully working answering machine out of the

Falcon and besides just being cosmetic, it has some real potential.

Imagine a low cost voice-mail system that allows full scripting and

information access with multiple menus. With the Falcon and VOX-Mail,

a simple answering machine or a full featured voice-mail system is

within the reach of all Falcon users. Voice-mail is no longer a dream

for me now, it is here!

The development of the system did take quite a bit of time, but I have

my sights set on developing an much more powerful voice-mail system. I

hope to get good response on this as it will be a strong motivation

for me to carry on developing the system.

The Falcon is indeed a great computer, and I am truly amazed by the

power of this small little computer.

- End of documentation March 4, 1994 -