.HM A 1 1 1 1 1 1

.H 1 "Appendix for asz80 Frankenstein Assembler"

.H 2 "Pseudo Operations"

.H 3 "Standard Pseudo Operation Mnemonics"

.VL 40 5 1

.LI "End"

END

.LI "File Inclusion"

INCL

INCLUDE

.LI "If"

IF

.LI "Else"

ELSE

.LI "End If"

ENDI

.LI "Equate"

EQU

.LI "Set"

SETEQU

.LI "Org"

ORG

.LI "Reserve Memory"

RESERVE

RMB

.LI "Define Byte Data"

BYTE

DB

FCB

.LI "Define Word Data"

DW

FDB

WORD

.LI "Define String Data"

FCC

STRING

.LI "Define Character Set Translation"

CHARSET

.LI "Define Character Value"

CHARDEF

CHD

.LI "Use Character Translation"

CHARUSE

.LE

.H 3 "Machine Dependent Pseudo Operations"

.H 4 "Instruction Set Selection"

.DS I N

CPU string

.DE

The instruction set can be specified in the source file with the CPU

pseudooperation.

The string, delimited by quotes or apostrophes, is scanned for a

substring which selects which instruction set is used.

When the program is invoked, this operation is performed on the name of

the program, then the -p optional arguement, if any, and then any CPU

statements.

The last one selects the which subset of the instructions the assembler will

accept.

The instruction set can be changed at any place in the source file.

.VL 30 5 1

.LI "Instruction Set"

Substrings

.LI 64180

180

.LI z80

z80

Z80

.LI 8085

85

.LI 8080

80

.LE

.H 2 "Instructions"

.H 3 "Instruction List"

.TS H

;

l l l.

Opcode	Syntax	Selection Criteria

.sp

.TH

.sp

ADC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRBC

ADC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRDE

ADC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRHL

ADC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRSP

ADC	 reg8 ',' '(' DREGHL ')' 	REGISA

ADC	 reg8 ',' index 	TSZ80PLUS|DRIX|REGISA

ADC	 reg8 ',' index 	TSZ80PLUS|DRIY|REGISA

ADC	 reg8 ',' reg8 	0|REGISA

ADC	 reg8 ',' topexpr 	REGISA

.sp

ADD	 dreg ',' dreg 	DRDESTHL|DRBC

ADD	 dreg ',' dreg 	DRDESTHL|DRDE

ADD	 dreg ',' dreg 	DRDESTHL|DRHL

ADD	 dreg ',' dreg 	DRDESTHL|DRSP

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIX|DRBC

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIX|DRDE

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIX|DRIX

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIX|DRSP

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIY|DRBC

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIY|DRDE

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIY|DRIY

ADD	 dreg ',' dreg 	TSZ80PLUS|DRDESTIY|DRSP

ADD	 reg8 ',' '(' DREGHL ')' 	REGISA

ADD	 reg8 ',' index 	TSZ80PLUS|DRIX|REGISA

ADD	 reg8 ',' index 	TSZ80PLUS|DRIY|REGISA

ADD	 reg8 ',' reg8 	0|REGISA

ADD	 reg8 ',' topexpr 	REGISA

.sp

AND	 '(' DREGHL ')'

AND	 index 	TSZ80PLUS|DRIX

AND	 index 	TSZ80PLUS|DRIY

AND	 reg8

AND	 topexpr

.sp

BIT	 expr ',' '(' DREGHL ')' 	TSZ80PLUS

BIT	 expr ',' index 	TSZ80PLUS|DRIX

BIT	 expr ',' index 	TSZ80PLUS|DRIY

BIT	 expr ',' reg8 	TSZ80PLUS

.sp

CALL	 condition ',' expr 	CCSELC

CALL	 condition ',' expr 	CCSELM

CALL	 condition ',' expr 	CCSELNC

CALL	 condition ',' expr 	CCSELNZ

CALL	 condition ',' expr 	CCSELP

CALL	 condition ',' expr 	CCSELPE

CALL	 condition ',' expr 	CCSELPO

CALL	 condition ',' expr 	CCSELZ

CALL	 expr

.sp

CCF

.sp

CP	 '(' DREGHL ')'

CP	 index 	TSZ80PLUS|DRIX

CP	 index 	TSZ80PLUS|DRIY

CP	 reg8

CP	 topexpr

.sp

CPD		TSZ80PLUS

.sp

CPDR		TSZ80PLUS

.sp

CPI		TSZ80PLUS

.sp

CPIR		TSZ80PLUS

.sp

CPL

.sp

DAA

.sp

DEC	 '(' DREGHL ')'

DEC	 dreg 	DRBC

DEC	 dreg 	DRDE

DEC	 dreg 	DRHL

DEC	 dreg 	DRSP

DEC	 dreg 	TSZ80PLUS|DRIX

DEC	 dreg 	TSZ80PLUS|DRIY

DEC	 index 	TSZ80PLUS|DRIX

DEC	 index 	TSZ80PLUS|DRIY

DEC	 reg8

.sp

DI

.sp

DJNZ	 topexpr

.sp

EI

.sp

EX	 '(' DREGSP ')' ',' dreg 	DRHL

EX	 '(' DREGSP ')' ',' dreg 	TSZ80PLUS|DRIX

EX	 '(' DREGSP ')' ',' dreg 	TSZ80PLUS|DRIY

EX	 dreg ',' dreg 	EX1DE|EX2HL

EX	 dreg ',' dreg 	TSZ80PLUS|EX1AF|EX2AF

.sp

EXX		TSZ80PLUS

.sp

HALT

.sp

IM	 expr 	TSZ80PLUS|INTSETMODE0

IM	 expr 	TSZ80PLUS|INTSETMODE1

IM	 expr 	TSZ80PLUS|INTSETMODE2

.sp

IN0	 reg8 ',' '(' topexpr ')' 	TS64180

.sp

IN	 reg8 ',' '(' REGC ')' 	TSZ80PLUS

IN	 reg8 ',' '(' topexpr ')' 	REGISA

.sp

INC	 '(' DREGHL ')'

INC	 dreg 	DRBC

INC	 dreg 	DRDE

INC	 dreg 	DRHL

INC	 dreg 	DRSP

INC	 dreg 	TSZ80PLUS|DRIX

INC	 dreg 	TSZ80PLUS|DRIY

INC	 index 	TSZ80PLUS|DRIX

INC	 index 	TSZ80PLUS|DRIY

INC	 reg8

.sp

IND		TSZ80PLUS

.sp

INDR		TSZ80PLUS

.sp

INI		TSZ80PLUS

.sp

INIR		TSZ80PLUS

.sp

JP	 '(' dreg ')' 	DRHL

JP	 '(' dreg ')' 	TSZ80PLUS|DRIX

JP	 '(' dreg ')' 	TSZ80PLUS|DRIY

JP	 condition ',' expr 	CCSELC

JP	 condition ',' expr 	CCSELM

JP	 condition ',' expr 	CCSELNC

JP	 condition ',' expr 	CCSELNZ

JP	 condition ',' expr 	CCSELP

JP	 condition ',' expr 	CCSELPE

JP	 condition ',' expr 	CCSELPO

JP	 condition ',' expr 	CCSELZ

JP	 expr

.sp

JR	 condition ',' expr 	CCSELC|TSZ80PLUS

JR	 condition ',' expr 	CCSELNC|TSZ80PLUS

JR	 condition ',' expr 	CCSELNZ|TSZ80PLUS

JR	 condition ',' expr 	CCSELZ|TSZ80PLUS

JR	 expr 	TSZ80PLUS

.sp

LD	 '(' dreg ')' ',' reg8 	DRBC|REGISA

LD	 '(' dreg ')' ',' reg8 	DRDE|REGISA

LD	 '(' dreg ')' ',' reg8 	DRHL

LD	 '(' dreg ')' ',' topexpr 	DRHL

LD	 '(' topexpr ')' ',' REGA

LD	 '(' topexpr ')' ',' dreg 	DRHL

LD	 '(' topexpr ')' ',' dreg 	TSZ80PLUS|DRBC

LD	 '(' topexpr ')' ',' dreg 	TSZ80PLUS|DRDE

LD	 '(' topexpr ')' ',' dreg 	TSZ80PLUS|DRIX

LD	 '(' topexpr ')' ',' dreg 	TSZ80PLUS|DRIY

LD	 '(' topexpr ')' ',' dreg 	TSZ80PLUS|DRSP

LD	 dreg ',' '(' topexpr ')' 	DRHL

LD	 dreg ',' '(' topexpr ')' 	TSZ80PLUS|DRBC

LD	 dreg ',' '(' topexpr ')' 	TSZ80PLUS|DRDE

LD	 dreg ',' '(' topexpr ')' 	TSZ80PLUS|DRIX

LD	 dreg ',' '(' topexpr ')' 	TSZ80PLUS|DRIY

LD	 dreg ',' '(' topexpr ')' 	TSZ80PLUS|DRSP

LD	 dreg ',' dreg 	TSZ80PLUS|DRHL|DRDESTSP

LD	 dreg ',' dreg 	TSZ80PLUS|DRIX|DRDESTSP

LD	 dreg ',' dreg 	TSZ80PLUS|DRIY|DRDESTSP

LD	 dreg ',' topexpr 	DRBC

LD	 dreg ',' topexpr 	DRDE

LD	 dreg ',' topexpr 	DRHL

LD	 dreg ',' topexpr 	DRSP

LD	 dreg ',' topexpr 	TSZ80PLUS|DRIX

LD	 dreg ',' topexpr 	TSZ80PLUS|DRIY

LD	 index ',' expr 	TSZ80PLUS|DRIX

LD	 index ',' expr 	TSZ80PLUS|DRIY

LD	 index ',' reg8 	TSZ80PLUS|DRIX

LD	 index ',' reg8 	TSZ80PLUS|DRIY

LD	 reg8 ',' '(' dreg ')' 	DRBC|REGISA

LD	 reg8 ',' '(' dreg ')' 	DRDE|REGISA

LD	 reg8 ',' '(' dreg ')' 	DRHL

LD	 reg8 ',' '(' topexpr ')' 	REGISA

LD	 reg8 ',' index 	TSZ80PLUS|DRIX

LD	 reg8 ',' index 	TSZ80PLUS|DRIY

LD	 reg8 ',' reg8

LD	 reg8 ',' specialr 	TSZ80PLUS|SPECIALIR|REGISA

LD	 reg8 ',' specialr 	TSZ80PLUS|SPECIALRR|REGISA

LD	 reg8 ',' topexpr

LD	 specialr ',' REGA 	TSZ80PLUS|SPECIALIR

LD	 specialr ',' REGA 	TSZ80PLUS|SPECIALRR

.sp

LDD		TSZ80PLUS

.sp

LDDR		TSZ80PLUS

.sp

LDI		TSZ80PLUS

.sp

LDIR		TSZ80PLUS

.sp

MULT	 dreg 	TS64180|DRBC

MULT	 dreg 	TS64180|DRDE

MULT	 dreg 	TS64180|DRHL

MULT	 dreg 	TS64180|DRSP

.sp

NEG		TSZ80PLUS

.sp

NOP

.sp

OR	 '(' DREGHL ')'

OR	 index 	TSZ80PLUS|DRIX

OR	 index 	TSZ80PLUS|DRIY

OR	 reg8

OR	 topexpr

.sp

OTDM		TS64180

.sp

OTDMR		TS64180

.sp

OTDR		TSZ80PLUS

.sp

OTIM		TS64180

.sp

OTIMR		TS64180

.sp

OTIR		TSZ80PLUS

.sp

OUT0	 '(' topexpr ')' ',' reg8 	TS64180

.sp

OUT	 '(' REGC ')' ',' reg8 	TSZ80PLUS

OUT	 '(' topexpr ')' ',' reg8 	REGISA

.sp

OUTD		TSZ80PLUS

.sp

OUTI		TSZ80PLUS

.sp

POP	 dreg 	DRAF

POP	 dreg 	DRBC

POP	 dreg 	DRDE

POP	 dreg 	DRHL

POP	 dreg 	TSZ80PLUS|DRIX

POP	 dreg 	TSZ80PLUS|DRIY

.sp

PUSH	 dreg 	DRAF

PUSH	 dreg 	DRBC

PUSH	 dreg 	DRDE

PUSH	 dreg 	DRHL

PUSH	 dreg 	TSZ80PLUS|DRIX

PUSH	 dreg 	TSZ80PLUS|DRIY

.sp

RES	 expr ',' '(' DREGHL ')' 	TSZ80PLUS

RES	 expr ',' index 	TSZ80PLUS|DRIX

RES	 expr ',' index 	TSZ80PLUS|DRIY

RES	 expr ',' reg8 	TSZ80PLUS

.sp

RET

RET	 condition  	CCSELC

RET	 condition  	CCSELM

RET	 condition  	CCSELNC

RET	 condition  	CCSELNZ

RET	 condition  	CCSELP

RET	 condition  	CCSELPE

RET	 condition  	CCSELPO

RET	 condition  	CCSELZ

.sp

RETI		TSZ80PLUS

.sp

RETN		TSZ80PLUS

.sp

RIM		CPU8085

.sp

RL	 '(' DREGHL ')' 	TSZ80PLUS

RL	 index 	TSZ80PLUS|DRIX

RL	 index 	TSZ80PLUS|DRIY

RL	 reg8 	TSZ80PLUS

.sp

RLA

.sp

RLC	 '(' DREGHL ')' 	TSZ80PLUS

RLC	 index 	TSZ80PLUS|DRIX

RLC	 index 	TSZ80PLUS|DRIY

RLC	 reg8 	TSZ80PLUS

.sp

RLCA

.sp

RLD		TSZ80PLUS

.sp

RR	 '(' DREGHL ')' 	TSZ80PLUS

RR	 index 	TSZ80PLUS|DRIX

RR	 index 	TSZ80PLUS|DRIY

RR	 reg8 	TSZ80PLUS

.sp

RRA

.sp

RRC	 '(' DREGHL ')' 	TSZ80PLUS

RRC	 index 	TSZ80PLUS|DRIX

RRC	 index 	TSZ80PLUS|DRIY

RRC	 reg8 	TSZ80PLUS

.sp

RRCA

.sp

RRD		TSZ80PLUS

.sp

RST	 expr

.sp

SBC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRBC

SBC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRDE

SBC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRHL

SBC	 dreg ',' dreg 	TSZ80PLUS|DRDESTHL|DRSP

SBC	 reg8 ',' '(' DREGHL ')' 	REGISA

SBC	 reg8 ',' index 	TSZ80PLUS|DRIX|REGISA

SBC	 reg8 ',' index 	TSZ80PLUS|DRIY|REGISA

SBC	 reg8 ',' reg8 	0|REGISA

SBC	 reg8 ',' topexpr 	REGISA

.sp

SCF

.sp

SET	 expr ',' '(' DREGHL ')' 	TSZ80PLUS

SET	 expr ',' index 	TSZ80PLUS|DRIX

SET	 expr ',' index 	TSZ80PLUS|DRIY

SET	 expr ',' reg8 	TSZ80PLUS

.sp

SIM		CPU8085

.sp

SLA	 '(' DREGHL ')' 	TSZ80PLUS

SLA	 index 	TSZ80PLUS|DRIX

SLA	 index 	TSZ80PLUS|DRIY

SLA	 reg8 	TSZ80PLUS

.sp

SLP		TS64180

.sp

SRA	 '(' DREGHL ')' 	TSZ80PLUS

SRA	 index 	TSZ80PLUS|DRIX

SRA	 index 	TSZ80PLUS|DRIY

SRA	 reg8 	TSZ80PLUS

.sp

SRL	 '(' DREGHL ')' 	TSZ80PLUS

SRL	 index 	TSZ80PLUS|DRIX

SRL	 index 	TSZ80PLUS|DRIY

SRL	 reg8 	TSZ80PLUS

.sp

SUB	 '(' DREGHL ')'

SUB	 index 	TSZ80PLUS|DRIX

SUB	 index 	TSZ80PLUS|DRIY

SUB	 reg8

SUB	 topexpr

.sp

TST	 '(' DREGHL ')' 	TS64180

TST	 reg8 	TS64180

TST	 topexpr 	TS64180

.sp

TSTIO	 topexpr 	TS64180

.sp

XOR	 '(' DREGHL ')'

XOR	 index 	TSZ80PLUS|DRIX

XOR	 index 	TSZ80PLUS|DRIY

XOR	 reg8

XOR	 topexpr

.TE

.H 3 "Selection Criteria Keywords"

.VL 25 5

.LI CPU8085

Instruction is only implemented for the 8085.

.LI TSZ80PLUS

Instruction is implemented in the z80 and 64180 instruction sets.

.LI TS64180

Instruction is only implemented in the 64180

.LI DRIX

Instruction refers to the IX index register

.LI DRIY

Instruction refers to the IY index register

.LI DRSP

Instruction refers to the Stack Pointer

.LI DRHL

Instruction refers to the HL register

.LI DRDE

Instruction refers to the DE register

.LI DRBC

Instruction refers to the BC register

.LI DRAF

Instruction refers to the AF concatenated register

.LI DRDESTSP

Instruction which refers to two double register operands, uses the Stack

Pointer as the destination.

.LI DRDESTHL

Instruction which refers to two double register operands, uses the HL double

register as the destination.

.LI DRDESTIX

Instruction which refers to two double register operands, uses the IX index

register as the destination.

.LI DRDESTIY

Instruction which refers to two double register operands, uses the IY index

register as the destination.

.LI REGISA

Instruction is restricted to using the A register for an 8 bit register

operand.

.LI CCSELNZ

Instruction uses NonZero condition.

.LI CCSELZ

Instruction uses Zero condition.

.LI CCSELNC

Instruction uses No Carry condition.

.LI CCSELC

Instruction uses Carry condition.

.LI CCSELPO

Instruction uses Parity Odd condition.

.LI CCSELPE

Instruction uses Parity Even condition.

.LI CCSELP

Instruction uses Plus condition.

.LI CCSELM

Instruction uses Minus condition.

.LI EX1AF

First operand of an Ex instruction is the AF register.

.LI EX1DE

First operand of an Ex instruction is the DE register.

.LI EX2AF

Second operand of an Ex instruction is the AF register.

.LI EX2HL

Second operand of an Ex instruction is the HL register.

.LI SPECIALIR

Instruction uses the I special register.

.LI SPECIALRR

Instruction uses the R special register.

.LE

.H 3 "Apostrophes"

The apostrophes in the syntax field are a notation used for the

parser generator and are not put in the assembler source statement.

.H 2 "Notes"

.H 3 "Conditions"

Conditions are represented by the reserved symbols z, nz, nc, pe, po, p, m,

and c, and their uppercase versions.

.H 3 "Indexed Addressing"

Index addressing uses the format "( index register + expression )" where

index register is IX or IY.

.H 3 "Top Expressions"

The syntax of some of the instructions combined with the standard expression

syntax resulted in confusion whether an operand was an expression surrounded

by parenthesis, or a memory reference.

To get around this, the expressions in these ambiguous cases were restricted

to those forms of expression that don't have surrounding parenthesis at the

top level.

Subexpressions may be parenthesized, but only if an operator seperates or

precedes the subexpression.

.DS I N

Example

.SP

ld a, (47h)	; load from memory address 0x47

ld a, +(47h)	; load immediate value 0x47

ld a, ((47h))	; error

.DE

.H 3 "dreg, reg8, specialr"

Double registers (dreg) are the set of symbols af, bc, de, hl, ix, iy, and sp

and their uppercase versions.

.P

Eight bit registers (reg8) are the set of symbols a, b, c, d, e, h, l, and

their uppercase versions.

.P

Special registers are i, r, I, R.

.H 3 "Bit Numbers"

The bit number expression in the BIT, RES, and SET operations has

to have value defined when the instruction is read in the first pass.

The value must be between 0 and 7.

.H 3 "Reserved Symbols"

.H 4 "Machine Dependent Reserved Symbols"

A

AF

B

BC

C

D

DE

E

H

HL

I

IX

IY

L

M

NC

NZ

P

PE

PO

R

SP

Z

a

af

b

bc

c

d

de

e

h

hl

i

ix

iy

l

m

nc

nz

p

pe

po

r

sp

z

.H 4 "Standard Reserved Symbols"

AND

DEFINED

EQ

GE

GT

HIGH

LE

LOW

LT

MOD

NE

NOT

OR

SHL

SHR

XOR

and

defined

eq

ge

gt

high

le

low

lt

mod

ne

not

or

shl

shr

xor

.TC 1 1 7