(sort of) VESA-Compliant TSR for the IBM XGA Adapter

version 1.3

Another freeware utility by Bert Tyler

Compuserve ID:  73477,433

BIX ID:  btyler

WHAT'S NEW IN THIS RELEASE?

Release 1.3 adds the VESA 640x400x256 mode to the list of supported

VESA modes.  LINKS386, for example, uses this mode if it is available

(and 640x480x256 mode and the top 5.6ths of the screen otherwise).

Note that LINKS386 version 1.03 or later displays its colors correctly

with this VESA driver if the "/p" option is used to force its palette

updates to use the BIOS.

I've recently purchased a 15" CrystalScan monitor from Gateway, and the

various 800x600 modes have also been adjusted so as to look reasonably

good with it.  This is the first "real" 800x600-capable monitor I've owned.

Release 1.2 added support for the VESA 1.2 spec and its "true-color"

modes - in particular, mode 111h (640x480x65536-color) that seems to

have been designed especially for XGA-like adapters.

WHAT DOES THIS PROGRAM DO?

XGAVESA is a TSR that (sort of) makes your XGA adapter VESA-compliant

(the "sort of" clause is explained in detail below, and is related

to the fact that the XGA adapter has some basic incompatibilities with

the VGA Video DAC and vertical retrace signals when it is in its

XGA-specific video modes).

The reason for this TSR's existence is simple:  many software packages

out there in the MS-DOS world currently have drivers for VESA-compliant

video adapters but not for IBM's XGA adapter.  This TSR is an attempt

to address that situation, by translating standard VESA calls into

routines that IBM's XGA adapter understands.

WHAT VIDEO MODES AND VESA CALLS DOES THIS PROGRAM SUPPORT?

This TSR supports the following standard VESA modes:

VESA mode           Description

109h                132-column by 25-rows text mode

100h                640x400x256 graphics mode

101h                640x480x256 graphics mode

103h *              800x600x256 graphics mode (requires a multisynching

video monitor)

105h **             1024x768x256 graphics mode (requires a high-resolution

video monitor and 1024K of memory on the adapter)

111h **             640x480x65536 graphics mode (requires 1024K of

memory on the adapter)

*  (The program reports support for this video mode only if you

have fired it up with the '800' option.  Note that this is

different from earlier versions, which supported this mode unless

you specifically told it not to using the '-800' option.  There

is no way for the program to auto-detect the presence of a

multisynching monitor.)

** (The program reports support for this video mode only if it detects

that the appropriate hardware exists for it.)

XGAVESA supports all of the VESA calls defined in version 1.2 of that

spec.  Given that VESA versions are downward-compatible, that means

that XGAVESA also supports all of the VESA calls defined by earlier

versions of the VESA specs.

This TSR does not currently support any of the XGA adapter's 16-color

SuperVGA modes.  Adding support for the XGA's 800x600x16 and 1024x768x16

modes would have been trivial, but the XGA does *not* use the standard

EGA/VGA style of pixel addressing when it is in those modes (the XGA's

"packed pixel" method of handling 16-color modes is better, IMHO, but in

this case the word "different" has more impact than the word "better").

Furthur, the responses to a query I posted on various VESA forums indicated

that there is no general consensus as to how most VESA drivers would

attempt to handle a "16-color packed-pixel" VESA mode, so I simply

left those modes out of this release.

When it is first fired up, XGAVESA attempts to detect the presence of

your XGA adapter and determine the amount of memory on your adapter and

the type of monitor to which it is connected.  If it can't locate any

XGA adapter, the program errors out - otherwise, it displays a sign-on

banner, sets itself up to support the VESA modes supported by your

particular hardware and goes TSR.

WAIT A MINUTE - THE XGA SUPPORTS 800x600?

Yup - piece of cake.  Maybe your IBM manuals don't mention it, but

that's probably only because IBM doesn't make a monitor that supports

that particular resolution.  This particular resolution *does* require

the presence of a monitor that supports 800x600 resolution, though, such

as a multisynching monitor like the NEC 2A/3A/4A/5A.

One limitation that the program currently has is that there is no way

to automatically detect the presence or absence of a multisynching

monitor attached to an XGA adapter.  For now, the program just assumes

that you do not have a multisynching monitor that supports the 800x600x256

mode unless you fire it up with the '800' option ("xgavesa 800").

Earlier versions of this program supported that mode unless you specifically

disabled it with the '-800' option, and in retrospect that was not a good

approach - A few of the VESA programs I tried this program out on

automatically use the highest resolution the VESA driver supports, and

if your hardware can't handle 1024x768x256 mode, that's going to be

800x600 unless that particular mode is disabled.  The current version

cheerfully ignores any '-800' option, as that's what it was going to

do anyway.

WADDYA MEAN "SORT OF" VESA-COMPLIANT?

Well, actually, the XGA adapter with this TSR is totally VESA-compliant.

The problem is that the current version of the XGA adapter in its

XGA-specific video modes isn't totally VGA-compliant, and many programs

make assumptions about VGA-compliance that don't work with the XGA

adapter.  (The XGA adapter is totally VGA-compatible when it is in

VGA mode, but not even close to it when in its XGA-specific modes.)

The worst-case scenario is total lockup and Big Red Switch time.

The two problems I have run into are both related to programs which

attempt to update the VGA's Video DAC via direct hardware writes

(and yes, my own Fractint program is one of them).

1) The XGA video DAC is completely different from the VGA video DAC,

and any attempt to update the "VGA's" Video DAC via direct hardware

while the XGA is in an XGA-specific mode has no effect.  If a

program goes through the BIOS to update the Video DAC, all is well

(the XGAVESA program traps the INT 10H, AH=10h interrupts and either

translates those BIOS calls and data into information that the XGA

adapter can understand or "eats" them).

2) The XGA does not support the VGA's vertical retrace hardware

signals when it is in its XGA-specific modes, and any program that

attempts to wait for a VGA-style vertical retrace ends up locking

up the system.

Here is a short list of VESA-compliant programs that have been tested

with XGAVESA, and the results:

LINKS386        works (but colors are only displayed correctly if

you're using version 1.03 or later and are using

its "/P" option to force it to use the BIOS to

update its palettes.

VPIC            works (version 4.5 of VPIC added VESA drivers)

MVGAVU          works

VUIMG           works

PV 0.5          works ("Persistence of Vision" is the successor to the

DKB raytracer program)

CSHOW           works *if* invoked with the "+V" option that forces it

to avoid the vertical retrace wait and go through the BIOS

(if you have a registered copy, the CONFIG program supplied

with CSHOW lets you hard-wire this option into the program)

((On the other hand, CSHOW 8.3 offers direct XGA support

that is far better than using this XGAVESA driver))

VGAKIT          works (once I fixed the VESA driver in release 3.5 <grin>.

Uncomment out the 'jmp fini' statement just prior to

the 'novesa:' label in BANKS.ASM)

FRACTINT        fails  (looks for the vertical retrace)

((On the other hand, Fractint offers direct XGA support

that is far better than using this XGAVESA driver))