Personal Communications Library

For the C Language

(PCL4C)

USERS MANUAL

Version 6.0

October 21, 1996

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1995

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice : 205-881-4630

FAX : 205|880|0925

BBS : 205-880-9748

email : info@marshallsoft.com

Anon FTP : ftp.marshallsoft.com /marshallsoft

web : www.marshallsoft.com

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCL4C Users Manual                                        Page 1

C O N T E N T S

Chapter                                                   Page

1.0 Introduction................................................3

1.1 User Support............................................4

1.2 ASP Ombudsman...........................................4

1.3 A Typical Application...................................5

1.4 Installation............................................6

2.0 Library Organization........................................7

2.1 Configuration...........................................7

2.2 Initialization & Termination............................7

2.3 Modem Control & Status..................................8

2.4 Serial I/O..............................................8

2.5 Error Detection.........................................9

2.6 General Support.........................................9

3.0 Library Overview...........................................10

3.1 Memory Models..........................................10

3.2 16-bit Protected Mode..................................11

3.3 32-bit Protected Mode..................................12

3.4 Compilers Supported....................................13

3.5 Using the Library......................................13

3.6 Application Notes......................................14

3.7 Compiling & Linking....................................15

4.0 Talking to Your Modem......................................16

4.1 Modem Standards........................................16

4.2 Flow Control...........................................17

4.3 MODEM_IO functions.....................................17

4.4 Modem Initialization...................................18

5.0 Problems...................................................19

6.0 Serial Communications......................................20

6.1 Communications Basics..................................20

6.2 Standard Port Addresses................................21

6.3 Running 3 or 4 Ports Concurrently......................22

6.4 Using Multiport Cards..................................23

6.5 Transmitter Interrupts.................................24

6.6 RS232 Signals..........................................25

6.7 National INS8250, INS16450, and INS16550 UARTs.........26

6.8 Register Summary.......................................27

7.0 Example Programs...........................................29

8.0 Legal Issues...............................................30

8.1 Registration...........................................30

8.2 License................................................30

8.3 Warranty...............................................31

9.0 Summary....................................................32

9.1 Revision History.......................................32

9.2 Function Summary.......................................34

9.3 Further Reading........................................34

10.0 Other MarshallSoft Computing products for C................35

10.1 The Personal Protocol Library for C...................35

10.2 The EMS Expanded Memory Library for C.................35

10.3 Other Products........................................35

PCL4C Users Manual                                        Page 2

1.0 Introduction

The Personal Communications Library for the C Language (PCL4C) is an

asynchronous communications library designed for experienced software

developers programming in C/C++.  Six compilers are supported:

Microsoft C, Quick C, Borland C, Turbo C, Watcom C, and MIX Power C.

An IBM PC/XT/AT or compatible is required.  The PCL features:

o Supports 16-bit & 32-bit PROTECTED MODE.

o SMALL, COMPACT, MEDIUM, LARGE, & FLAT memory models.

o 38 communications and support functions.

o Supports the high performance 16550 UART.

o Supports the PC/4 and PC/8 DigiBoard.

o Supports the BOCA BB1004, BB1008, and BB2016 boards.

o Supports hardware (RTS/CTS) flow control.

o Interrupt driven transmitter & receiver.

o Supports 300 baud to 115,200 baud.

o Use IRQ2 through IRQ15 with any UART address.

o Supports COM1 thru COM4 (COM20 in registered version).

o Adjustable receive queues from 8 bytes to 32 KB.

o Control-BREAK error exit.

o 18 communications error conditions trapped.

o Allows 4 ports to run concurrently (more with multiport).

o Complete modem control & status.

o Written in assembly language for small size & high speed.

o Terminal program featuring ASCII (with XON/XOFF), XMODEM,

YMODEM, & YMODEM-G.

Why should you buy PCL4C ?  Several good reasons are:

COMPLETE - PCL4C is complete since it provides absolute control of

the serial ports (including the 16550 UART).

COMPACT - PCL4C is very compact at less than 10 KB.  Your

application doesn't carry a lot of excess code.

FAST - PCL4C is fast since it will run at 38400 baud on even

slow 8088 PCs (4.77 MHZ) and at 115200 baud on most

everything else.

SUPPORT - If you get stuck, you talk to the programmer that wrote

the code, not a person hired to answer the phone.

BBS - A BBS is available (to 14400 baud, N81) in order to

provide immediate support as necessary.

NEWSLETTER - We publish a quarterly newsletter "Comm Talk", which

discusses serial communications problems and solutions.

PRICE - You get PCL4C for a very reasonable price!

UPGRADES - Once you buy PCL4C, you can always update to the most

recent version very inexpensively ($25 plus shipping).

PCL4C Users Manual                                        Page 3

1.1 User Support

We want you to be successful in developing your applications using

PCL4C! We depend on our customers to let us know what they need in a

communications library.  This means we are committed to providing the

best communications library that we can. If you have any suggestions

or comments, please let us know!

We provide customer support for registered customers by voice, BBS,

email, and US mail.

If you are having a problem using PCL4C, call us at 205-881-4630

between 1:30 PM and 9:30 PM (CST) Monday through Friday. You can also

call at other times and leave a message, and call back later for a

reply. We cannot return phone calls.

However, we can only answer questions with respect to using the

PCL4C library. We cannot help you program your application, but we'll

be glad to discuss it with you.

You may also call our User Support BBS (2400 to 14400 baud, no

parity, 8 data bits, 1 stop bit) at 205-880-9748 and leave a message

(address it to the SYSOP).  We will usually have a reply ready for

you within 24 hours.

The BBS is available 24 hours per day. All files are in standard ZIP

format. The BBS will contain the latest shareware version of all

MarshallSoft Computing products as well as related files such as:

BUGS.ZIP     -  Bug report.

PRODUCTS.ZIP -  List of all shareware products.

The MarshallSoft Computing, Inc.  newsletter "Comm Talk" is published

quarterly.  It discusses various communications problems and

solutions using PCL4C as well as related information.

The latest copy of our newsletter can be found on our support BBS,

anonymous FTP site, or on our Web site.

BBS      : 205-880-9748            (file area "Newsletters")

Anon FTP : ftp.marshallsoft.com    (directory  /marshallsoft)

Web site : www.marshallsoft.com

1.2 ASP Ombudsman

MarshallSoft Computing, Inc.  is a member of the Association of

Shareware Professionals (ASP).  ASP wants to make sure that the

shareware principle works for you.  If you are unable to resolve a

shareware-related problem with an ASP member by contacting the member

directly, ASP may be able to help. The ASP Ombudsman can help you

resolve a dispute or problem with an ASP member, but does not provide

technical support for members' products. Please write to the ASP

Ombudsman at 545 Grover Road, Muskegon, MI USA 49442-9427, Fax

616-788-2765, or send a CompuServe message via CompuServe Mail to

ASP Ombudsman 70007,3536.

PCL4C Users Manual                                        Page 4

1.3 A Typical Application

In general, there are two classes of applications that use a

communications library like PCL4C - those that use a modem to connect

to the outside world and those that connect directly to a peripheral

device. In either case, a typical (real mode) application program

using PCL4C might look like the following code outline:

+---------------------------------------------------+

| #include  "pcl4c.h"                               |

|                                                   |

| void main(void)                                   |

| {                                                 |

|  ...                                              |

|  /* initialize serial comm  system  */            |

|  SioRxBuf(Port,AllocSeg(1024),Size1024);          |

|  SioTxBuf(Port,AllocSeg(128),Size128);            |

|  SioParms(Port,NoParity,OneStopBit,WordLength8);  |

|  SioReset(Port,Baud2400);                         |

|                                                   |

|  ...application code...                           |

|                                                   |

|  /* terminate serial comm system  */              |

|  SioDone(Port);                                   |

| }                                                 |

+---------------------------------------------------+

In the above example, SioRxBuf is called to set up the a 1024 byte

receive buffer; SioTxBuf is called to set up a 128 byte transmit

buffer; SioParms is called to set up the parity, stop bit count, and

word length; SioReset is called to set the baud rate to 2400 and

reset the UART (Univeral Asynchronous Receiver / Transmitter).

The function AllocSeg allocates a buffer of specified size on the

far heap and returns the segment SEG such that SEG:0 points to the

buffer.  The function AllocSeg is part of the example code and can be

found in the file ALLOCSEG.C.

Before leaving your application, SioDone is called to restore the

prior state of the serial communications system.

If you are using a modem, you also need to be concerned about

initializing your modem correctly and handling any required flow

control. Refer to the "Talking to Your Modem" chapter for detailed

information.

The communication library has been pre-assembled to library files:

PCL4C_S.LIB : Small memory model.

PCL4C_C.LIB : Compact memory model.

PCL4C_M.LIB : Medium memory model.

PCL4C_L.LIB : Large memory model.

PCL4C_P.LIB : 16-bit Protected mode [Large model].

PCL4C32.OBJ : 32-bit Protected Mode [Flat model].

PCL4C_S.MIX : MIX PowerC small memory model.

PCL4C_M.MIX : MIX PowerC medium memory model.

PCL4C_L.MIX : MIX PowerC large memory model.

PCL4C Users Manual                                        Page 5

1.4 Installation

(1) Before installation of PCL4C, your C compiler should already be

installed on your system and tested. If you are not familiar with

makefiles, refer to your compiler manual. If you are using the

interactive environment for Quick C or Turbo C, be sure to compile

with the memory model corresponding to the PCL4C library used, and

include the correct library in the project file.  Examine the file

"FILES.LST" for a list of all the distribution files.

(2) Make a backup copy of your distribution disk.  Put your original

distribution disk in a safe place.

(3) Create a work directory on your work disk (normally your

harddisk).  For example, to create a work directory named PCL4C, we

first log onto the work disk and then type:

MKDIR PCL4C

(4) Copy all the files from your backup copy of the distribution

disk to your work directory.  For example, to copy from the A: drive

to your work directory, we type:

CD PCL4C

COPY A:*.*

(5) [OPTIONAL] Delete the makefiles that you won't need.  For

example, if you use the Microsoft C compiler, then you want to keep

all makefiles ending *._M_ but can delete those for Turbo C (*._T_),

Quick C (*._Q_), and MIX Power C (*.PRJ).  You may also delete any

libraries that you won't need.

(6) Compile SIMPLE.C and link with the appropriate PCL4C library

(PCL4C_S.LIB for all but Power C which must use PCL4C_S.MIX).

Makefiles (or project files) are provided for each of the supported

compilers.

a) Microsoft C/C++:  MAKE SIMPLE._M_

b) Microsoft QuickC: MAKE SIMPLE._Q_

c) Borland C/C++:    MAKER -fSIMPLE._B_

(32-bit)          MAKER -fSIMPLE32._B_

d) Turbo C/C++:      MAKE -fSIMPLE._T_

e) MIX Power C:      PC/E SIMPLE.PRJ

f) WATCOM C/C++:     WMAKE -f SIMPLE._W_

(32-bit)          WMAKE -f SIMPLE32._W_

SIMPLE.C [or SIMPLE32.C] should compile without any problems as all

example code has been tested with each of the supported compilers.

(7) The recommended way to test SIMPLE is to run it on two computers

connected by a null modem cable.  Whatever is typed on one computer

should be displayed on the other.

SIMPLE can also be tested by connecting your port to a modem.

Transmitting a "AT" to the modem should result in an "OK" being

received.

PCL4C Users Manual                                        Page 6

2.0 Library Organization

The PCL4C library is organized into six categories of functions.

Refer to the PCL Reference Manual (PCL4C.REF) for details on

individual functions.

2.1 Configuration

There are three functions in the configuration category.  SioPorts

sets the number of PC and DigiBoard (or BOCA board) ports. SioUART is

used to change the UART base address for a communications port to a

non-standard address, while SioIRQ is used to assign a nonstandard

IRQ line to a port.  (See chapter 6, Serial Communications for more

details on standard UART addresses and IRQ lines).

The configuration functions SioPorts, SioUART and SioIRQ must be

called before calling any other library functions.  Be very careful

in using these functions.  Remember that your serial hardware must

support the UART and IRQ that you specify.  Always test any new

configuration immediately.

SioPorts -  Sets number of PC and DigiBoard (or BOCA board) ports.

SioUART  |  Sets the UART base address.

SioIRQ   -  Assigns an IRQ line to a port.

THE IRQ GOLDEN RULE: You may open (via SioReset) only one port per

IRQ (except for the DigiBoard and BOCA board).

2.2 Initialization & Termination

There are eight functions in the initialization and termination

category. Together, SioParms, SioFIFO, SioRxBuf, SioTxBuf, and

SioReset initialize your serial communications system.  Your

application must call SioRxBuf and SioTxBuf before calling SioReset,

and SioReset must be called before any serial I/O processing can be

done.

After initialization, SioParms and SioBaud can be called to change

the communications parameters without resetting the serial port.

SioFlow can be called to enable hardware flow control.  SioFIFO can

be called to set the interrupt level for the 16550 FIFO.

Before exiting from your application, SioDone must be called.

Failure to call SioDone can crash your system later.

SioRxBuf   : Sets up receive buffer.

SioTxBuf   : Sets up transmitter buffer.

SioFIFO    : Sets the interrupt level for the INS16550.

SioParms   : Sets parity, stop bits, and word length.

SioReset   : Initialize a serial port for processing.

SioDone    : Terminates further serial processing.

SioBaud    : Sets the baud rate of the selected port.

SioFlow    : Enables / disables hardware flow control.

PCL4C Users Manual                                        Page 7

2.3 Modem Control & Status

There are nine functions in the modem control and status category

which provide your application with complete control over the status

and control bits of your modem.

There are two modem control bits, "Data Terminal Ready" (DTR) and

"Request To Send" (RTS).  These bits can be read, set, or cleared by

SioDTR and SioRTS.

There are four modem status bits, "Data Set Ready" (DSR), "Clear To

Send" (CTS), "Ring Indicator" (RI), and "Data Carrier Detect" (DCD).

SioModem can read any of the modem status bits.  SioDSR, SioCTS,

SioRI, and SioDCD can only read their respective modem status bit.

SioGetDiv reads the baud rate divisor register so the baud rate can

be determined.

Refer to the chapter entitled "RS232 Signals" for a discussion of

each of the control and status bits.

SioDTR    : Set, clear, or read the Data Terminal Ready (DTR) bit.

SioRTS    : Sets, clears, or reads the Request to Send (RTS) line.

SioModem  : Reads the modem status register.

SioDSR    : Reads the Data Set Ready (DSR) modem status bit.

SioCTS    : Reads the Clear to Send (CTS) modem status bit

SioDCD    : Reads the Data Carrier Detect (DCD) modem status bit.

SioRI     : Reads the Ring Indicator (RI) modem status bit.

SioRead   : Reads the contents of the 7 UART registers.

SioGetDiv : Reads the baud rate divisor registers.

2.4 Serial I/O

There are eleven library functions in the serial I/O category.

Together, these functions give the programmer complete control over

serial I/O.  Higher level functions such as protocols and smart modem

communications can be completely implemented in terms of these

functions.  Refer to the example code.

SioGetc, SioGets, SioPutc, and SioPuts perform all the actual serial

I/O. SioUnGetc "ungets" the last serial byte read.  SioRxClear clears

the receive queue while SioTxClear clears the transmit queue.

SioTxFlush flushes the transmit queue.  SioLine can be used to test

for UART errors. SioRxQue returns the number of bytes in the receive

queue while SioTxQue returns the number of bytes in the transmit

queue.

SioGetc    : Reads the next character from the serial line.

SioGets    : Reads a buffer of characters from the serial line.

SioLine    : Reads the line status register.

SioPutc    : Transmit a character over a serial line.

SioPuts    : Transmit a buffer of characters over a serial line.

SioRxClear : Clears the RX buffer.

SioRxQue   : Returns the number of characters in the RX queue.

SioTxClear : Clears the TX buffer.

SioTxFlush : Flushes the TX buffer by restarting TX interrupts.

SioTxQue   : Returns the number of characters in the TX queue.

SioUnGetc  : "Un:gets" (puts back) a specified character.

PCL4C Users Manual                                        Page 8

2.5 Error Detection

There are four functions in the error detection category.  They are

concerned with detecting or reporting communications errors.  Use of

these functions can make your application significantly more robust.

SioBrkKey can be used as an "emergency" exit from your application.

SioBrkSig can read or modify the UART break bit.  This is useful for

signalling the remote system that a fatal condition has occurred.

SioLoopBack can be used to test the integrity of your UART.  SioError

displays a error message corresponding to an error code returned from

a PCL4C function (every PCL4C function returns a code).

SioBrkKey   : Returns !0 if CTL-BREAK was pressed [16-bit lib only].

SioBrkSig   : Asserts, cancels, or detects the RS232 BREAK  signal.

SioError    : Displays error in text [32-bit lib uses SioErr32.C].

SioLoopBack : Performs a UART loopback test. [16-bit lib only].

2.6 General Support

There are three functions in the general support category.  They are

as follows:

SioInfo     : Returns library information such as version number.

SioStats    : Return port specific statistics [32-bit only].

SioDelay    : Delays one or more tics (18.2 tics per second).

SioTimer    : Returns the number of system clock tics.

PCL4C Users Manual                                        Page 9

3.0 Library Organization

3.1 Memory Models

Because of the segmented architecture of the INTEL CPU, there are

four real mode memory organizations possible for computer programs.

These are named the SMALL, COMPACT, MEDIUM, and LARGE memory models,

which correspond to the four combinations of "near" and "far"

addresses for code and data.

Each (real mode) executable is composed of one or more segments,

where each segment can occupy from one byte to 64 KB of memory.  A

"near" address is a 16 bit offset in a segment, whereas a "far"

address consists of both a 16 bit segment value and a 16 bit offset.

In the small memory model, code and data each occupy one segment.

Thus, near addresses are allocated for both code and data.

In the compact memory model, code occupies one segment while data

may occupy multiple segments.  Near addresses are allocated for code

but far addresses are allocated for data.

In the medium memory model, data occupies one segment while code may

occupy multiple segments.  Near addresses are allocated for data but

far addresses are allocated for code.

In the large memory model, data and code each occupy multiple

segments.  Far addesses are allocated for both code and data. Thus,

both code and data can use as many segments as required.

Refer to your compiler manual for a discussion of the memory models

supported by your compiler.

PCL4C is organized as four separate libraries (PCL4C_S.LIB,

PCL4C_C.LIB, PCL4C_M.LIB and PCL4C_L.LIB) corresponding to the four

standard memory models. For the MIX Power C compiler, the small,

medium, and large models are provided. Lastly, there is the 16-bit

and 32-bit protected mode model which is linked into protected mode

applications.

MODEL      CODE      DATA     Library

Small      Near      Near     PCL4C_S.LIB & PCL4C_S.MIX

Compact    Near      Far      PCL4C_C.LIB

Medium     Far       Near     PCL4C_M.LIB & PCL4C_M.MIX

Large      Far       Far      PCL4C_L.LIB & PCL4C_L.MIX

Protected  Far       Far      PCL4C_P.LIB [16-bit PM only].

Flat       Near      Near     PCL4C32.OBJ [32-bit PM only].

However, one can always use the large memory model library

PCL4C_L.LIB with any memory model application code by explicitly

declaring the PCL4C procedures to be FAR (by prefixing "far" before

the name of each function in the PCL4C.H file) and declaring your

receive buffer to be FAR.  If you are compiling with the HUGE memory

mode (real mode), link with PCL4C_L.LIB.

PCL4C Users Manual                                        Page 10

3.2 16-Bit Protected Mode

16-Bit Protected Mode programming takes advantage of the segmented

architecture of 286 and up Intel processors.  In contrast to normal

real mode programming which limits programs to an address space of 1

MB (1024 KB), 16-bit protected mode allows up to 16 MB. Protected

mode also offers some measure of protection of one program from

another.

A program that can execute in protected mode under DOS requires three

modules:

(1) A DPMI (DOS Protected Mode Interface) server loaded in memory.

Execute the real mode program DPMI.EXE to determine if a DPMI server

is loaded.

(2) A protected mode runtime C library linked to your program.

(3) A 16-bit DOS extender which provides protected mode equivalents

for:

a) A subset of DOS interrupt 21H calls.

b) A subset of BIOS calls.

The Personal Communications Library (PCL4C_P.LIB) supports 16-bit

protected mode. The application program must be compiled with a C/C++

compiler that supports protected mode.  The Borland PowerPack for DOS

provides all the required modules, although the library can be used

by any DOS extender that provides the three required components as

outlined above.

Examine the example program PMSIMPL.C, which is a (16-bit) protected

mode version of the normal DOS based SIMPLE.C. Compare it to

SIMPLE.C to see the differences necessary to run in protected mode.

In particular, note the manner in which memory is allocated for use

in PCL. The transmit and receive buffers must be allocated in

conventional DOS memory (the first 1MB) since serial interrupts can

occur during both protected mode and real mode.

Also notice that memory should be free after calling SioDone but

before exiting:

Examine the makefile (PMSIMPLE._B_) used to compile PMSIMPLE.C using

the Borland compiler and Borland's PowerPack for DOS.

The program SELFTEST.C can also be compiled for protected mode by

using the makefile PM_SELF._B_.

For more information on protected mode, refer to your DOS extender

such as the Borland Power Pack.

PCL4C Users Manual                                        Page 11

3.3 32-Bit Protected Mode

32-bit Protected Mode programming takes advantage of the 32-bit flat

memory architecture of 386 and up Intel processors.  In contrast to

real mode programming which limits programs to an address space of

one MB, a 32-bit Protected Mode program can address four GB.

The 32-bit flat memory model uses 48-bit addesses: a 16-bit selector

plus a 32-bit offset. The selectors are not usually modified by the

application code, although the DOS extender and PCL4C32 do manipulate

them.

32-bit Protected Mode programming requires a 386 (or higher) CPU, a

C/C++ compiler that can generate 32-bit code, and a 32-bit DOS

extender which supports the DPMI standard. PCL4C32 has been tested

with two 32-bit compilers: Borland 4.5 (using the Borland Power Pack

extender 32RTM) and WATCOM 10.5 & 10.6 (using the DOS4GW extender

which comes with the WATCOM compiler). Other 32-bit DOS extenders may

also work provided that DPMI services are available.

If you are shopping for a 32-bit compiler, we recommend WATCOM. Their

DOS extender (DOS4GW) is very stable and WATCOM even has technical

support (no credit card or 900 number required!).

You must have a DPMI server loaded in order to use the 32-bit PCL4C

library. There are several options. If you are using Windows 3.X or

Windows 95, simply drop into a DOS box since Windows itself requires

DPMI. A better solution is to configure your memory manager (386Max,

etc.) to provide DPMI support, typically by disabling EMS memory.

Refer to your memory manager's manual.

Execute the real mode program DPMI.EXE to determine if a DPMI server

is loaded.

Examine the two 32-bit example programs SIMPLE32.C and SELF32.C. The

corresponding makefiles are SIMPLE32._B_, SIMPLE32._W_, SELF32._B_,

and SELF32._W_. Note that a 0 can be passed as the second argument

to SioRxBuf and SioTxBuf which will cause PCL4C32 to allocate all

required memory itself.

The shareware version of PCL4C is limited to 4 ports and can not

execute for more than 5 minutes at any one time without having to

restart the application. The registered version supports up to 20

ports and has no shareware reminder screen or any time limits. The

assembly language source code for the PCL4C32 library is not

provided.

PCL4C Users Manual                                        Page 12

3.4 Compilers Supported

At this time, six C compilers are supported by PCL4C.

(1)  Microsoft C Compiler.

(2)  Quick C Compiler.

(3)  Turbo C Compiler.

(4)  Borland C Compiler [also 32-bit with Power Pack].

(5)  MIX Power C Compiler

(6)  Watcom C compiler [also 32-bit].

The Microsoft C Compiler supports all memory models.  Just be careful

to link with the PCL4C library that corresponds to the memory model

used.  Recall that the small memory model is the default.  Examine

the (small model) makefiles *._M_ for the Microsoft compiler.

The Microsoft Quick C Compiler supports all memory models, but be

careful to link with the PCL4C library that corresponds with the

memory model used. Recall that the small memory model is the default

for the command line compiler (QCL) while the medium memory model is

the default for the interactive compiler environment.  Examine the

(small model) makefiles *._Q_ for the Microsoft Quick C compiler.

The Borland and Turbo C Compilers also support all memory models. Be

sure to link with the correct PCL4C library corresponding to the

memory model used. Examine the (small model) makefiles *._T_ for the

Turbo C compiler and *._B_ for the Borland compiler.

The MIX Power C Compiler supports the small, medium, and large

memory models. However, older versions of Power C only support the

small model.  Examine the (small model) project batch files *.PRJ for

the Power C Compiler.

Other compilers may also work with one or more of the PCL4C libraries

but have not been tested. Give us a call if you have any difficulty.

3.5 Using the Library

Please examine the PCL4C.H file (PCL4C32.H for 32-bit). Note that

COM1 is defined as port zero, not port one.  The user must assume the

responsibilty for passing the correct information when calling PCL4C

functions.

If there are any conflicts between PCL4C definitions and those in

other libraries, the PCL4C definitions can be changed in the PCL4C.H

file and any file that uses the definition. There is no change

necessary for the library code itself.

The PCL4C libraries (16 and 32 bit) contain no references to any

runtime libraries. Only BIOS and MSDOS functions are called.

PCL4C Users Manual                                        Page 13

3.6 Application Notes

3.6.1 Terminal Programs

The "terminal program" is the most common class of communications

program.  It is used to call up a BBS or on-line service such as

CompuServe, America On-Line, etc.  Refer to the programs SIMPLE and

LOGIN in section 7 for examples of simple terminal programs.

A more sophisticated terminal program featuring ASCII, XMODEM,

YMODEM, and ZMODEM protocol file transfers can be found in our sister

product -- The Personal Protocol Libray (PPL4C).  Source code is

included in the shareware distribution for everything except ZMODEM

and the script intepreter.

3.6.2 Door Programs

In order to write a door program which "takes over" a serial port

without resetting the port or changing the baud rate, call SioReset

with NORESET as the second argument rather than the baud rate. Call

SioGetDiv to get the baud rate divisor if the baud rate must be

determined. Be sure to call SioDone before returning to the invoking

program.  Refer to the SPAWN.C and DOOR.C example programs.  Also be

sure to free any memory that you may have allocated in your door

program before exiting.

3.6.3 BBS Programs

If you are designing a BBS program (also known as HOST programs),

consider using 16550 UARTS.  You should also choose a multiport card

such as the DigiBoard or BOCA board if you wish to run more than 4

ports simultaneously.

If you are using an error correcting modem, then you should be sure

to set flow control and fix your baud rate at the highest possible

transfer rates.  For 14,400 modems, this means 19200 or 38400. You

may need a 16550 UART in order to run at the higher speed.

If you are using an older multi-speed modem (say 1200, 2400, 4800,

9600) that doesn't use flow control, you should change your baud rate

to match the CONNECT message baud rate.

3.6.4 Communicating with Devices

If you will be communicating with a serial device (motor control,

digitizing tablet, etc.) be sure to (1) not enable flow control

include unless you are certain that flow control is used by the

serial device, (2) include a short delay after sending each character

to the serial device (start with 1/4 second), and (3) set DTR and RTS

since many devices require DTR to be set before they will respond.

As a first step, try to communicate with the serial device using a

terminal emulator such as SIMPLE.

PCL4C Users Manual                                        Page 14

3.7 Compiling and Linking

Registered users may wish to assemble (16-bit) PCL4C.ASM.  Use the

/MX switch in order to disable automatic conversion from lower case

to upper case.  If the /MX switch is not used, then all PCL4C

function references in C code must be in upper case.

Model    Command

Small    MASM PCL4C /DSMALL_MODEL   /MX;

Compact  MASM PCL4C /DCOMPACT_MODEL /MX;

Medium   MASM PCL4C /DMEDIUM_MODEL  /MX;

Large    MASM PCL4C /DLARGE_MODEL   /MX;

To disable transmitter interrupts, add "/DNO_TBE" to each MASM

command line above.

For example, to make the (small) model PCL4C.OBJ into a library file:

DEL PCL4C_S.LIB

LIB PCL4C_S.LIB+PCL4C,PCL4C.MAP;

If you are using the MIX Power C Compiler, create the MIX object

file (you will need version 1.3 of MIX which has the /_ switch):

MIX /_ PCL4C_S

Similarly with the other memory model libaries.  See the batch files

MAKE_S.BAT, MAKE_C.BAT, MAKE_M.BAT, and MAKE_L.BAT.  Similiarly for

MAKE_ST.BAT, MAKE_CT.BAT, MAKE_MT.BAT, and MAKE_LT.BAT.

To compile and link (small model) using:

Microsoft C:   NMAKE SIMPLE._M_

Quick C:   QCL /AS SIMPLE.C /LINK PCL4C_S.LIB

Borland C:   MAKER -fSIMPLE._B_

MAKE -fSIMPLE32._B_    [32-bit]

Turbo:   MAKE -fSIMPLE._T_

MIX Power C:   PC/E /ms SIMPLE.C PCL4C_S.MIX

WATCOM C:   WMAKE -f SIMPLE._W_

WMAKE -f SIMPLE32._W_  [32-bit]

Makefiles or project files are provided for all example code. Borland

makefiles end with the extension '._B_', Turbo C makefiles with

'._T_', Microsoft C makefiles end with '._M_', Microsoft Quick C

makefiles files end with '._Q_', WATCOM makefiles end with '._W_',

and Power C project batch files end with '.PRJ'.

The PCL4C libraries may also be used with integrated development

environments. Place all required files along with the library

corresponding to the memory model being used into the project file.

Be sure to select the appropriate memory model, and turn off all case

sensitivity.

PCL4C Users Manual                                        Page 15

4.0 Talking to Your Modem

A modem is used to extend the distance over which you may

communicate. Without a modem, your RS232 cable is limited to a

maximum of approximately 50 feet. But with a modem, you can

communicate literally around the world.

Also refer to Section 4.3 for details on MODEM_IO functions.  These

functions faciliate communications with modems.

4.1 Modem Standards

Two modems can communicate over a telephone line only if they are

both using the same signaling frequencies and modulation, which are

determined by the the modem standards used.  Modem standards can be

divided into three sets: (1) speed, (2) data compression used, and

(3) error control.

The Bell standards (103 & 212A) are those of AT&T.  The CCITT (The

International Consultative Committee for Telephone and Telegraph)

standards are designated as "V. ".

Speed

Bell 103  :   300 baud

Bell 212A :  1200 baud

V.21      :   300 baud

V.22bis   :  1200 & 2400 baud

V.32      :  4800 & 9600 baud

V.32bis   :  4800, 7200, 9600, 12000, and 14400 baud

V.34      :  through 28800 baud

Data Compression

MNP 5     :  Microcom Networking Protocol (proprietary).

V.42bis   :  International data compression standard.

Error Control

MNP 2,3,4 :  Three level error correction (public domain).

V.42      :  International error correction standard.

Most of the newer high speed modems use several of the above

standards. However, not all combinations of modem makes communicate

easily with each other, especially at high speeds (9600 and up).

PCL4C Users Manual                                        Page 16

4.2 Flow Control

With modems using data compression, the modem to modem connection

will run at various speeds depending on the quality of the line.  The

computer to modem connection will be at a fixed baud rate. Therefore,

a protocol (flow control) is necessary to synchronize the data flow

between a modem and the computer to which it is connected.

Two flow control protocols are used by modems which require flow

control. Software flow control is called "XON/XOFF" and hardware flow

control is called "RTS/CTS".

4.2.1 Software Flow Control (XON/XOFF)

In XON/XOFF (software) flow control, the computer suspends

transmitting data if it receives a XOFF character (13 hex) from the

modem, and continues transmitting when it receives a XON character

(11 hex).  Similiarly, the computer can signal the modem not to send

any more data by transmitting a XOFF to it, and can tell the modem to

continue transmission be sending a XON.

Software flow control cannot be used to transfer binary data since

binary data may have hex 11 or hex 13 bytes embedded in it. The ASCII

protocol typically uses XON/XOFF to control display flow while the

modem may be using RTS/CTS. Refer to the module XONOFF.C.

4.2.2 Hardware Flow Control (RTS/CTS)

In RTS/CTS (hardware) flow control, the RTS line is used by the

computer to signal the modem , while the CTS line is used by the

modem to signal the computer.  The RTS line is set OFF by the

computer to tell the modem to suspend transmission, and set to ON to

tell the modem to continue transmission.  The CTS line is set to OFF

by the modem to tell the computer to stop transmitting, and set to ON

to tell the computer to continue transmitting.

Given the choice, always choose hardware flow control over software

flow control so that all data transmission is transparent.  If

hardware flow control is not the default (which it almost always is),

you should modify your modem initialization string to turn hardware

flow control on.

Hardware flow control is internal to the PCL4C library and is

selected by the SioFlow function.

4.3 MODEM_IO Functions

The file MODEM_IO.C has several functions that ease communicating

with your modem.  Look in LOGIN.C for examples of use.

ModemSendTo   : Sends string (including control chars) to the modem.

ModemWaitFor  : Waits for a string from the modem, passing all else through.

ModemQuiet    : Waits for continuous quiet of specified duration.

ModemHangup   : Hangs up the modem.

ModemCmdState : Goes into the modem's command state.

ModemEcho     : Echos all serial incoming bytes to the display.

PCL4C Users Manual                                        Page 17

4.4 Modem Initialization

If your application uses a modem (as opposed to using a null modem

cable), then you should always send an initialization string to your

modem if it is a programmable modem such as those made by Hayes.

Communication programs such as PROCOMM and TELIX always send such a

string automatically as soon as they start up.

The particular initialization string depends on the make of your

modem. Virtually all modems implement the basic Hayes AT command set,

and add their own "extended" AT commands.  These extended AT commands

vary among modem manufacturers.

For AT command set modems the following string (followed by a

carriage return) may work:

AT E1 S7=60 S11=60 V1 X1 Q0 S0=0

Refer to your Modem User's Guide for a full discussion of these

commands.  A brief description is as follows:

AT     : Modem attention command.

E1     : Modem will echo what you send to it.

S7=60  : Wait 60 seconds for carrier and/or dial tone.

S11=60 : Use 60 milliseconds for tone dialing duration & spacing.

V1     : Display result code as words (not numbers).

X1     : Use the extended result message (CONNECT XXXX) set.

Q0     : Modem returns result codes.

S0=0   : Do not answer RING.

If your application will answer incoming calls, then set the S0

register to the ring on which to automatically answer.

Alternatively, use the ATA command to explicitly answer the modem

after detecting the RING.

If you send the above codes by using SioPutc (as opposed to typing

them from the keyboard), then follow these guidelines:

(1) Send an initial carriage return before the initialization string.

(2) Pause at least two tics (18 tics to the second) after each

character sent as your modem needs the time to perform its own

internal processing. Pause a little longer if your modem is not

accepting your initialization string.

(3) Pause two seconds after sending any initialization command such

as ATZ or AT&F since your modem must do quite a bit of processing.

If you experience any problems in initializing your modem, you

should first reset it to factory settings by sending:

AT&F

Your modem may require more initialization than presented above.

Refer to your modem manual for details. If you have a communications

program such as ProComm or Telix that is known to initialize your

modem correctly, then use the same initialization string.

PCL4C Users Manual                                        Page 18

5.0 Problems

Before developing your application, you should run the example

programs. They should all execute correctly.

If you have two ports on your computer, connect them together with a

null modem cable and run SELFTEST.C. If SELFTEST does not execute

properly then there is something wrong with either (1) your serial

port hardware, (2) your null modem cable, or (3) you selected ports

that are not connected by the null modem cable.

Another easy test is to connect the serial port to a modem, and use

SIMPLE to send a "AT" to the modem, which should respond with a

"OK". If you cannot get your application to run properly, first

compile and run the terminal emulator program SIMPLE provided on your

distribution disk.

If your application does not run but the example programs run

correctly, then you have most likely made a programming mistake in

your application. MarshallSoft Computing cannot debug your

application, especially over the telephone!  However, consider each

of the following when searching for an error in your application.

1.  Have you included the file PCL4C.H in your application ?

2.  Did you link with the correct PCL4C library ?  This is the most

probable cause if your application 'hangs' as soon as it starts and

you must reboot. The function SioInfo('M') returns the model ID under

which the library was assembled.

3.  Is your receive buffer large enough ? If you are using 1K data

blocks in YMODEM, then your receive buffer should be at least 1K (2K

if baud rates above 19200 are to be used).

4.  Have you selected too high a baud rate (if you are using a slow

PC) ?  If only one COM port is being run, you should be able to run

at 38400 baud on 8088 machines and 115200 on most 286s and above.

5.  Are you attempting to run another application in the background ?

Try running without any other programs running in the background.

6.  If you are running two COM ports simultaneously, are you using

separate receive and transmit buffers ? (you should).

7.  Did SioReset return a zero value ?  If not, then you must call

SioReset again. See SIMPLE.C for an example.

8.  Did you send the proper initialization string to your modem ?

Did you set DTR and RTS ? (you should).

9.  Do you have more than one COM1 port, etc.  For example, if you

have a COM1 port on your motherboard, you cannot add another COM1

port or modem board that uses COM1 without first disabling the COM1

on the motherboard.

10.  Are you passing the proper segment of the receive (and

transmit) buffer? See SIMPLE.C for an example.

PCL4C Users Manual                                        Page 19

6.0 Serial Communications

6.1 Communications Basics

The heart of serial communications is the UART (Universal

Asynchronous Receiver Transmitter).  The IBM PC/XT/AT and compatibles

use the INS8250, INS16450, or the INS16550 UART.  The purpose of the

UART is:

(1) To convert bytes from the CPU (Central Processing Unit), into a

serial format by adding the necessary start, stop, and parity bits to

each byte before transmission, and to then transmit each bit at the

correct baud rate.

(2) To convert the incoming stream (at a specified baud rate) of

serial bits into bytes by removing the start, stop, and parity bits

before being made available to the CPU.

The UART is part of the serial interface circuitry which allows the

CPU to send and receive signals over the RS232 lines. This can be

diagrammed as follows:

Serial Interface

+-------------------+

|                   |

+-----+  Data Bus  |     +------+      |    RS232 Signals

| CPU +------------+     | UART |      +----------------*

+-----+            |     +------+      |

|                   |

+-------------------+

The INS8250/16450/16550 UART is capable of operating in one of two

modes, "polled" and "interrupt driven".  The serial communications

functions in the BIOS uses the polled method.  In this approach, the

CPU is typically in a loop asking the UART over and over again if it

has a byte ready.  If it does, the polling code returns the byte.

But, if the next byte comes in before the polling code is executing

again, then that byte is lost.

In the interrupt driven approach, when a byte is received by the

UART, an interrupt is generated and the "Interrupt Service Routine"

(ISR) is executed immediately, suspending temporarily whatever else

is executing.  The ISR then moves the byte to the receive buffer so

that your application program can later read it.

When a byte is to be transmitted, it is moved to the transmit queue

and transmitter interrupts are "kickstarted", which causes an

interrupt to be generated each time the transmitter buffer becomes

empty. The ISR is invoked, which removes the byte from the

transmitter queue and writes it to the UART transmit buffer.

Refer to section 6.5 "Transmitter Interrupts", section 6.6 "RS232

Signals", and section 6.7 "National INS8250, INS16450 and INS16550

UARTs" for further information on these topics.

PCL4C Users Manual                                        Page 20

6.2 Standard Port Addresses

There are a few things to know about how serial communications ports

are used by IBM PC/XT/AT and compatible computers.  The standard IBM

PC/XT/AT configuration values are as follows:

Port    Reg.   IRQ  Vector

COM1    3F8H    4     12

COM2    2F8H    3     11

COM3    3E8H    4     12

COM4    2E8H    3     11

(Refer to your DigiBoard manual for DigiBoard addresses, or your

BOCA board manual for BOCA port addresses).

PCL4C assumes the above values.  If necessary, the UART base address

can be changed by SioUART, and IRQ lines can be re-assigned by

SioIRQ. Remember that each port to be used concurrently must have a

unique IRQ line.  Refer to the PCL4C Reference Manual for specific

details.

When installing new communications cards, the following guidelines

are recommended:

(1) Be sure to read the documentation for the hardware you are

installing.  Pay special attention to UART base addresses and IRQ

lines, particularly if trying to set up a non-standard configuration.

(2) If you have a choice in base addresses and IRQ lines, always

choose standard values as defined above.

(3) The first port should be COM1, the second COM2, etc.  Do not

skip over any port number if possible.

(4) Use SioUART to zero all unused ports (for example, call

SioUART(COM4,0) if there is no COM4 port installed).

(5) Be carefull not to configure two ports for the same address.

This is easier to do than you may believe.

(6) Choose an external modem over an internal one.  It is much

easier to debug problems with an external modem than an internal one.

(7) Select hardware flow control (RTS/CTS) if flow control is

required and hardware flow control is not the default.

(8) Always test your port as soon as it is installed. Try several

programs that use the communications ports.

(9) If your serial card has two or more ports, run SELFTEST after

configuring it for your new card.

PCL4C Users Manual                                        Page 21

6.3 Running 3 or 4 Ports Concurrently

PCL4C supports up to 4 serial ports running concurrently (more if

you have a DigiBoard or BOCA board).  One free interrupt for each

port is required.  Refer to the next section if you have a DigiBoard

or BOCA board.

Interrupts IRQ4 and IRQ3 are dedicated to the communications ports

in a standard IBM PC/XT/AT configuration.  IRQ4 is shared between

COM1 and COM3 while IRQ3 is shared between COM2 and COM4.  This means

that you can run two ports simultaneously provided that they don't

share an interrupt.

Suppose that you wish to run 3 ports simultaneously. To begin, you

must have 3 UARTs (serial ports) installed on your computer.  Assume,

for purposes of this discussion, that COM1 is installed on your

motherboard, and that you have purchased a new 2 port serial

communications board.

You should be able to configure the first serial board port as COM2,

which uses IRQ3.  Refer to the manual that came with your serial

board.

In order to run the third serial port concurrently with the first

two, an unused interrupt must be found.  If your serial card can use

only IRQ3 and IRQ4, then there is no way to run a third line since

IRQ4 and IRQ3 are used for COM1 and COM2.

However, many serial cards can use other IRQs, typically IRQ2

through IRQ7. Many of the newer serial cards can use IRQs through

IRQ15.  Since IRQ5 is normally used for a second printer port, it is

a good candidate for COM3.  To use IRQ5 for the third serial port,

first set your serial card to use IRQ5 for COM3 (refer to your serial

card manual) and then add the following line to your applications

code before calling SioReset:

SioIRQ(COM3,IRQ5);

Don't forget to disable any device that might use IRQ5, such as a

second printer port or a music card.  Unfortunately, there is no easy

way to determine that you have no conflicts until you actually

attempt to use the IRQ.  If there are conflicts, your system will

probably hang and you will have to reboot.

To run a fourth serial port, another free IRQ must be found.  On

some systems, IRQ7 can be used. To use IRQ7 for the fourth serial

port, first set your serial card to use IRQ7 for COM4 and then add:

SioIRQ(COM4,IRQ7);

Also note that conflicts are possible with port addresses. You cannot

have two cards using the same port address. This problem most often

occurs when adding a multiport board since each UART uses 8

consecutive port addresses.

Of course, you can select any IRQ and UART address that your serial

card can generate. But be carefull about IRQ and address conflicts.

PCL4C Users Manual                                        Page 22

6.4 Using Multiport Cards

6.4.1 The DigiBoard

PCL4C supports the DigiBoard PC/4 and PC/8. In order to use the

DigiBoard, you must configure PCL4C using the SioPorts, SioUART, and

SioIRQ functions.

Your PC's ports must be partitioned into "standard" PC ports and

dumb card ports.  Remember that standard PC ports cannot share IRQs

like the DigiBoard (or BOCA board) can. If you are using IRQ4 and

IRQ3 for standard PC ports COM1 and COM2, then you cannot use either

for DigiBoard ports (try IRQ5 or IRQ7).

Suppose that COM1 through COM2 are standard PC ports (using IRQ4 and

IRQ3) and you have installed a PC/8 DigiBoard that you wish to use

for COM3 through COM10 using interrupt line IRQ5.  You choose to use

the recommended DigiBoard UART addresses starting at 0x100:

+-------------------------------------------------------------------+

| SioPorts(10,COM3,0x140,DIGIBOARD);/* COM3 = 1st DigiBoard port */ |

| Address = 0x100;                  /* 1st DigiBoard UART address */|

| for(Port=COM3;Port<=COM10;Port++) /* look at each port */         |

|   {SioUART(Port,Address);         /* set the UART address */      |

|    Address += 8;                  /* compute next address */      |

|    SioIRQ(Port,IRQ5);             /* set the DigiBoard IRQ */     |

|   }                                                               |

+-------------------------------------------------------------------+

The DigiBoard uses 0x140 for the status address for odd interrupts

and 0x141 for even interrupts.

Digiboard may be contacted at 6400 Flying Cloud Drive, Eden Prairie,

MN 55344. Telephone 612-943-9020 or FAX 612-943-5398.

6.4.2 The BOCA Board

PCL4C supports the dumb BOCA board.  As with the DigiBoard, you must

configure PCL4C before using the BOCA board.

For example, to configure the BOCA BB2016 to use COM1 to COM16, with

base addresses starting at 0x100 and IRQ5:

+-------------------------------------------------------------------+

| SioPorts(16,COM1,0x107,BOCABOARD);/* COM3 = 1st BOCA board port */|

| Address = 0x100;                  /* 1st BOCA UART address */     |

| for(Port=COM1;Port<=COM16;Port++) /* look at each port    */      |

|   {SioUART(Port,Address);         /* set the UART address */      |

|    Address += 8;                  /* compute next address */      |

|    SioIRQ(Port,IRQ15);            /* set the BOCA IRQ */          |

|   }                                                               |

+-------------------------------------------------------------------+

BOCA may be contacted at BOCA Research, Inc., 6413 Congress Avenue,

Suite 130, Boca Raton, FL 33487.  Phone 407-241-8088, FAX

407-997-0918.

PCL4C Users Manual                                        Page 23

6.5 Transmitter Interrupts

PCL4C comes assembled for all memory models with transmitter

interrupts enabled. PCL4C can also be assembled without transmitter

interrupts enabled by including "/DNO_TBE" on the MASM line.

When transmitter interrupts are NOT enabled, the following logic

occurs everytime SioPutc or SioPuts is called:

1.  Wait for the UART transmit buffer to become empty.  The UART

transmit buffer may not be empty if the previous transmit is not

completed (the UART breaks down the byte & sends 1 bit at a time).

2.  When the UART transmit buffer is empty, the byte from the

SioPutc or SioPuts call is loaded into the UART transmit buffer and

control is returned to the caller.

Transmitter interrupts provide a way for your application program to

write to the serial port faster than the baud rate, but transmitter

interrupts do have several caveats.  To begin, they are slower than

polled transmission, although not significantly.  Secondly, they are

more complex and thus result in a slightly larger library.

The increased complexity of transmitter interrupts is due to the

fact that even though transmitter interrupts are enabled, a

transmitter interrupt can occur only when the UART transmitter

register transitions from not-empty to empty (a byte has just been

transmitted). To start the transmitter interrupt process, the first

byte in the PCL4C transmitter queue must be transmitted by SioPutc

(or SioPuts) rather than by the Interrupt Service Routine (ISR).

The remainder of the bytes in the PCL4C transmit queue are

transmitted by the ISR when the transmitter interrupt occurs.  This

process is called "kickstarting" the transmitter interrupts.

When transmitter interrupts are enabled, the following logic occurs

everytime SioPutc or SioPuts is called:

1.  The byte from SioPutc or bytes from SioPuts are put into the

PCL4C transmitter queue.

2.  If there is no byte presently being transmitted, then the

transmitter interrupt process is kickstarted.

3.  Each time a transmitter interrupt occurs, the ISR gains control,

removes the next byte from the PCL4C transmitter queue and puts it in

the UART transmit register.

While you can now call SioPutc and SioPuts faster than the baud

rate, bytes are still transmitted at the given baud rate. The

advantage of using transmitter interrupts is that an application

program can write a string of bytes to the serial transmit queue

without having to wait for those bytes to be transmitted before

regaining control.

When running a streaming protocol (one that doesn't wait for any

acknowledgement from the other side), be sure to check for

transmitter buffer overflow (eg, SioPutc returns -1).

PCL4C Users Manual                                        Page 24

6.6 RS-232 Signals

RS-232 is the name of the serial data interface standard used to

connect computers to modems.  Most IBM compatible computers are built

with at least one serial port and use either DB9 (9 pin) or DB25 (25

pin) connectors.

A summary of these pins and their function follows.  For more

detailed information, refer to one of the many books dealing with

RS-232 interfacing.

Signal Ground Pin 7 (DB25), Pin 5 (DB9)

The SG line is used as the common signal ground, and must always be

connected.

Transmit Data Pin 2 (DB25), Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data Pin 3 (DB25), Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready Pin 20 (DB25), Pin 4 (DB9)

The DTR line is used by the computer to signal the modem that it is

ready. DTR should be set high when talking to a modem.

Data Set Ready Pin 6 (DB25), Pin 6 (DB9)

The DSR line is used by the modem to signal the computer that it is

ready.

Request to Send Pin 4 (DB25), Pin 7 (DB9)

The RTS line is used to "turn the line around" in half duplex

modems, and for hardware flow control in most modems that require

flow control.

Clear to Send Pin 5 (DB25), Pin 8 (DB9)

The CTS line is used to "turn the line around" in half duplex

modems, and for hardware flow control in most modems that require

flow control.

Data Carrier Detect Pin 8 (DB25), Pin 1 (DB9)

The DCD line is used by the modem to signal the computer that a data

carrier signal is present.

Ring Indicator Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

PCL4C Users Manual                                        Page 25

6.7 National INS8250, INS16450, and INS16550 UARTs

The Personal Communications Library is based on the standard

National INS8250, INS16450, and INS16550 UARTs. The 8250 was the

original UART used in the IBM PC, whereas the 16450 is a faster

version found on most 286 & up machines.  The 16550 contains a 16

byte FIFO to further reduce communications overhead.  These UARTs

consists of 8 register ports as follows:

Offset    R/W   Register

0       R/W   Receiver (read) / Transmitter (write)

1       R/W   Interrupt Enable (read)

2       R     Interrupt Identification

2       W     FIFO control (INS16550 only)

3       R/W   Data Format (Line Control)

4       R/W   RS-232 (Modem) Control

5       R/W   Line Status

6       R/W   RS-232 (Modem) Status

7       R/W   Not used.

For the standard PC ports (not DigiBoard or BOCA ports), the UART

registers are based at 3F8h (COM1), 2F8h (COM2), 3E8h (COM3), and

2E8h (COM4).  COM1 and COM3 share interrupt request line IRQ4 while

COM2 and COM4 share request line IRQ3. This means that COM1 and COM3

can't be used concurrently.  Similarly for COM2 and COM4.

If you have a DigiBoard (or BOCA board) installed, you will have 4

or more additional ports using INS16450 or INS16550 UARTS.  The

default DigiBoard and BOCA board ports are located at 100h, 108h,

110h, etc.  Refer to your DigiBoard (or BOCA board) manual.

Four sources of interrupts are possible with the 8250 and 16550: (1)

receiver error or BREAK, (2) receiver data ready, (3) ready to

transmit, and (4) RS232 input.  These four sources of interrupts are

summarized as follows:

Source of Interrupt        Action Required to Clear

Receiver error or BREAK.   Read Line Status register.

Receiver data.             Read data from data register.

Transmitter Buffer Empty.  Write to data register or read IID reg.

RS232 input.               Read Modem Status register.

However, PCL4C only enables transmitter and receiver interrupts.

If you are not familiar with the INS8250, several good books are

available. Refer to the Serial Communications chapter for

recommendations.  Although a knowledge of the 8250 is not necessary

to use PCL4C, a general knowledge of the theory of asynchronous

serial communications is recommended.

PCL4C Users Manual                                        Page 26

6.8 Register Summary

REG 0 : Data Register

Reading from the data register fetches the next input byte, once it

is ready. Writing to the data register transmits the byte written to

it over the serial line.

REG 1 : Interrupt Enable

The Interrupt Enable register enables each of four types of

interrupts when the appropriate bit is set to a one.

bit 3 : Enable interrupt on RS232 input.

bit 2 : Enable interrupt on receiver error or break.

bit 1 : Enable interrupt on transmitter buffer empty (TBE).

bit 0 : Enable interrupt on received data (RxRDY).

REG 2 : Interrupt Identification (IID)

Reading the Interrupt Identification (read only) register once an

interrupt has occurred identifies the interrupt as follows:

Bit 2  Bit 1  Bit 0  Priority    Interrupt

0      0      1      none      none

1      1      0      0 (high)  Serialization or break.

1      0      0      1         Received data.

0      1      0      2         Transmitter Buffer Empty.

0      0      0      3 (low)   RS232 Input.

In the INS16650, REG 2 (write only) is also the FIFO control

register.  Writing bits 6 & 7 will set the FIFO trigger level

(number of bytes received before an interrupt is generated).

Bit 7  Bit 6   Trigger             Bit 7  Bit 6   Trigger

0      0      1 byte               1      0      8 bytes

0      1      4 bytes              1      1      14 bytes

REG 3 : Line Control

RS232 line parameters are selected by writing to this register.

bit 7    : DLAB = 0

bit 6    : BREAK on(1), off(0).

bits 5-3 : Parity None(000),ODD(001),EVEN(011),MARK(101),SPACE(111)

bit 2    : One stop bit(0), two stop bits(1).

bits 1-0 : Data bits = 5 (00), 6(01), 7(10), 8(11).

When the Divisor Latch Access Bit (DLAB) is 1, registers 0 and 1

become the LS and MS bytes of the Baud Rate Divisor registers.

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

PCL4C Users Manual                                        Page 27

REG 4 : Modem Control

RTS, DTR, loopback testing, and General Purpose Outputs #1 and #2 are

controlled by the Modem Control register as follows:

bit 4 : Enable local loopback.

bit 3 : Enable GP02. Necessary for 8250 interrupts.

bit 2 : Enable GP01.

bit 1 : Set / clear RTS.

bit 0 : Set / clear DTR.

REG 5 : Line Status

Reading the Line Status register provides status information as

follows (1 for TRUE, 0 for FALSE) :

bit 6 : Transmitter Empty.

bit 5 : Transmitter Buffer Empty (TBE).

bit 4 : BREAK detect.

bit 3 : Framing error.

bit 2 : Parity error.

bit 1 : Overrun error.

bit 0 : Data Ready.

REG 6 : Modem Status

Reading the Modem Status register provides the following status

information (1 for TRUE, 0 for FALSE) :

bit 7 : DCD status.

bit 6 : RI status.

bit 5 : DSR status.

bit 4 : CTS status.

bit 3 : Delta DCD status.

bit 2 : Delta RI status.

bit 1 : Delta DSR status.

bit 0 : Delta CTS status.

The delta bits (bits 0 through 3) are set whenever one of the status

bits (bits 4 through 7) changes (from 0 to 1 or from 1 to 0) since

the last time that the Modem Status register was read. Reading the

Modem Status register clear the delta bits.

REG 7 : Scratch Register

There is no function associated with register 7.  It does not exist

in early versions of the 8250.

PCL4C Users Manual                                        Page 28

7.0 Example Programs

Six (real mode) example programs are include with PCL4C.  In

addition, the Personal Protocol Library for C (PPL4C) includes the

terminal program TERM which features ASCII, XMODEM, YMODEM, and

ZMODEM protocol transfers.  Complete source code is included in the

shareware product for all of the protocols above except ZMODEM.  In

order to get ZMODEM source, PPL4C must be registered.

7.1 MINIMAL

MINIMAL is the simpliest possible communications program. It

displays on the screen what it reads from the serial port and

transmits over the serial line what is typed at the keyboard.  COM1

and 9600 baud are hard coded for simplicity.

7.2 SIMPLE

SIMPLE is a simple terminal program. It operates like MINIMAL,

except that both a port and a baud rate can be specified. For

example. SIMPLE32.C is the 32-bit equivalent.

SIMPLE 1 9600

7.3 LOGIN

LOGIN is programmed to dial our support BBS (205-880-9748) and log

on as GUEST. Start LOGIN like SIMPLE by providing a COM port and a

baud rate. For example,

LOGIN 1 38400

7.4 SPAWN and DOOR

The SPAWN program is the same as the SIMPLE program except that it

spawns DOOR when the user types a '$'. For example, to start SPAWN on

COM3, type

SPAWN 3

The DOOR program is also the same as SIMPLE except that it can "take

over" a serial port without changing any of the port parameter.

Exiting DOOR returns to the invoking program (SPAWN).

7.5 SELFTEST

The SELFTEST.C [or SELF32.C] program is designed to test serial

ports provided that one port (with a loopback adapter) or two ports

(connected together with a null-modem adapter) are avaiable. SELFTEST

can also be used to test your multiport board.

For example, to test PC port COM1 against COM2, type:

SELFTEST PC 1 2

SELFTEST may need to be configured for non-standard PC ports or your

multiport board. Refer to the SELFTEST source code for more info.

PCL4C Users Manual                                        Page 29

8.0 Legal Issues

8.1 Registration

If you wish to register the PCL4C library (which also includes the

32-bit protected mode version), please send $75 plus $5 S&H ($10

outside of North America) to:

MarshallSoft Computing, Inc.

Post Office Box  4543

Huntsville AL 35815

The $75 price is good for one year from the date on the title page of

the manual. Call for the current price after one year.

Multiple copies are available: $55 for 3 to 9, $45 for 10 to 19, and

$35 for 20 or more. A site license is also available for $675

(includes 20 sets of printed documentation). We pay shipping.

We accept American Express, MasterCard, and VISA (account number,

expiration date, exact name on your card, and complete card billing

address required), checks in US dollars drawn on a US bank, purchase

orders (POs) from recognized US schools and companies listed in Dun &

Bradstreet, and COD (street address and phone number required) within

the USA (plus an additional $4.50 COD charge).

You can also order PCL4C from The Public Software Library (PSL) in

Houston Texas with your American Express, MasterCard, VISA, or

Discover card by calling 800-242-4PSL (from overseas: 713-524-6394)

or by FAX at 713-524-6398 or by CompuServe at [71355,470].  THESE

NUMBERS ARE FOR ORDERING ONLY. The product number for PCL4C is 10908.

Please have your credit card billing address ready.

If you wish to update from an older version of PCL4C, send $25 plus

$5 S&H ($10 outside of North America).  Updates must be ordered

directly from MarshallSoft Computing.

The registered package includes:

o  Small, Compact, Medium, Large, 16-bit PM  & 32-bit PM libs.

o  COM1 thru COM20 supported for all libs.

o  Assembler source code for the library [16-bit only].

o  Printed Users and Reference Manuals.

o  Telephone, FAX, and BBS support for one year.

Print the file INVOICE.DOC if an invoice is needed. The registered

user will receive the latest version of PCL4C shipped by two day

priority mail (packet airmail overseas).  A 3.5" diskette is provided

unless a 5.25" diskette is requested.

PCL4C Users Manual                                        Page 30

8.2 License

MarshallSoft Computing, Inc. grants the registered user of PCL4C the

right to use one copy of the PCL4C library (in object form) on a

single computer in the development of any software product (other

than libraries such as PCL4C).  The user may not use the library on

more than one computer at the same time.  The source code for the

library (PCL4C.ASM) is copyrighted by MarshallSoft Computing and may

not be released in whole or in part.

Products developed using the registered version of PCL4C or PCL4C32

may include the object form of the library and may be distributed

royalty free.

Under no circumstances can PCL4C or PCL4C32 be used for any

commercial purpose without registration.

8.3 Warranty

MARSHALLSOFT COMPUTING, INC.  DISCLAIMS ALL WARRANTIES RELATING TO

THIS SOFTWARE, WHETHER EXPRESSED OR IMPLIED, INCLUDING BUT NOT

LIMITED TO ANY IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR

A PARTICULAR PURPOSE, AND ALL SUCH WARRANTIES ARE EXPRESSLY AND

SPECIFICALLY DISCLAIMED.  NEITHER MARSHALLSOFT COMPUTING, INC. NOR

ANYONE ELSE WHO HAS BEEN INVOLVED IN THE CREATION, PRODUCTION, OR

DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR ANY INDIRECT,

CONSEQUENTIAL, OR INCIDENTAL DAMAGES ARISING OUT OF THE USE OR

INABILITY TO USE SUCH SOFTWARE EVEN IF MARSHALLSOFT COMPUTING, INC.

HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR CLAIMS.  IN NO

EVENT SHALL MARSHALLSOFT COMPUTING, INC.'S LIABILITY FOR ANY SUCH

DAMAGES EVER EXCEED THE PRICE PAID FOR THE LICENSE TO USE THE

SOFTWARE, REGARDLESS OF THE FORM OF THE CLAIM.  THE PERSON USING THE

SOFTWARE BEARS ALL RISK AS TO THE QUALITY AND PERFORMANCE OF THE

SOFTWARE.

Some states do not allow the exclusion of the limit of liability for

consequential or incidental damages, so the above limitation may not

apply to you.

This agreement shall be governed by the laws of the State of Alabama

and shall inure to the benefit of Marshallsoft Computing, Inc.  and

any successors, administrators, heirs and assigns.  Any action or

proceeding brought by either party against the other arising out of

or related to this agreement shall be brought only in a STATE or

FEDERAL COURT of competent jurisdiction located in Madison County,

Alabama. The parties hereby consent to in personam jurisdiction of

said courts.

PCL4C Users Manual                                        Page 31

9.0 Summary

9.1 Revision History

Version 1.0 - 14 January 1991 - original release.

Version 1.1 - 11 March 1991

o  Added SioUnGetc function to library.

Version 1.2 - 1 June 1991

o  SioParms bug - could not call before SioReset.

o  SioReset bug - was not saving & restoring all regs.

Version 1.3 - 1 July 1991

o  Added NORESET option to SioReset.

o  Added SioDSR, SioCTS, SioDCD, SioLoopBack, and SioRI.

Version 2.0 - 1 Nov 1991

o  Reorganized as four memory model libraries.

o  Added SioModel function to library.

o  Added support for Quick C and Power C.

Version 2.1 - 1 Dec 1991

o  Fixed bug due to Microsoft Assembler (MASM 5.0,5.1) error.

Version 3.0 - 15 Jan 1992

o  Added SioUART function.

o  Added "UART undefined" error code.

o  Added "Bad or missing UART" error code.

o  Added "Port already enabled" error code.

o  Added "Cannot enable both COM1 & COM3 ..." error code.

o  Fixed several minor bugs (using new automated testing).

Version 3.1 - 1 March 1992

o  Added SioFIFO (INS16550 support).

o  Added SioIRQ function.

o  Increased maximum receive buffer size to 32K bytes.

Version 3.2 - 1 May 1992

o  Modified SioReset so that it no longer clears DTR & RTS.

o  Modified SioModel & renamed to SioInfo.

o  Fixed bug in SioDone when using 2 ports simultaneously.

o  Added SioFlow to library.

o  Added YMODEM-G protocol to TERM program.

Version 3.3 - 3 August 1992

o  Fixed bug in SioUnGet when using 2 ports simultaneously.

o  Add SioRead function.

Version 3.4 - 4 Jan 1993

o  Library modified to use up to four ports simultaneously.

o  SioIRQ was modified to include a third argument.

o  EXAMPORT utility distributed to registered users.

PCL4C Users Manual                                        Page 32

9.1 Revision History (continued)

Version 3.5 - 15 May 1993

o  Supports dumb DigiBoards ( PC/4 and PC/8).

o  Two new error traps added ("No such IRQ" & "No such ISR").

o  ASCII file transfer protocol added to TERM (with XON/XOFF).

Version 4.0 - 18 Oct 1993

o  The library supports transmitter interrupts.

o  Corrects bug in Ver 3.5 requiring calling SioIRQ for COM3/4.

o  All example code compiles with supported C++ compilers.

o  The SioIRQ function has been simplified.

Version 4.1 - 1 May 1994

o  Transmitter FIFO enabled.

o  Minor internal modifications.

o  Supports dumb BOCA boards (BB1004, BB1008, & BB2016).

o  Port definition extented to COM16.

Version 4.2 - 1 Sept 1994

o  A flow control bug was fixed.

o  SioGetDiv function added.

o  SioRxBuf & SioTxBuf function modified.

Version 4.3 - 15 March 1995

o  BREAK detection bug fixed,

o  Port definitions extended to COM20.

o  Support for IRQ8 through IRQ15.

o  Line status bits preserved.

Version 5.0 - 20 November 1995

o  Supports Protected Mode

o  Renamed SioRxFlush to SioRxClear.

o  Renamed SioTxFlush to SioTxClear.

o  Added SioTxFlush

o  Added more choices in SioInfo.

o  Removed SioCrtWrite (use putch instead).

o  Removed SioKeyPress (use kbhit instead).

o  Removed SioKeyRead  (use getch instead).

o  Added SioGets & SioPuts.

Version 6.0 - 21 October 1996

o Only force 15 bytes (not 16) into TX side FIFO.

o Handles simultaneous BOCA interrupts under all conditions.

o Added 32-bit library!

PCL4C Users Manual                                        Page 33

9.2 Function Summary

Refer to the PCL4C Reference Manual (PCL4C.REF) for detailed

information on the communications and support functions.  A one line

summary of each function follows:

SioBaud     Sets the baud rate of the selected port.

SioBrkKey   Returns non-zero if the Control-BREAK key was pressed.

SioBrkSig   Asserts, cancels, or detects BREAK signal.

SioCTS      Reads the Clear to Send (CTS) modem status bit.

SioDCD      Reads the Data Carrier Detect (DCD) modem status bit.

SioDelay    Delays one or more tics (18 tics per second).

SioDone     Terminates further serial processing.

SioDSR      Reads the Data Set Ready (DSR) modem status bit.

SioDTR      Set, clear, or read the Data Terminal Ready (DTR) bit.

SioError    Displays error in text.

SioFIFO     Sets the interrupt level for the INS16550.

SioFlow     Enables / disables hardware flow control.

SioGetc     Reads the next character from the serial line.

SioGets     Reads a buffer of characters from the serial line.

SioGetDiv   Reads the baud rate divisor registers.

SioInfo     Returns library information (version number, etc.)

SioIRQ      Assigns an IRQ line to a port.

SioLine     Reads the line status register.

SioLoopBack Performs a UART loopback test.

SioModem    Reads the modem status register.

SioParms    Sets parity, stop bits, and word length.

SioPorts    Sets # ports, 1st DigiBoard / BOCA port & status reg.

SioPutc     Transmits a character over a serial line.

SioPuts     Transmits a buffer of characters over a serial line.

SioRead     Reads any of 7 UART ports.

SioReset    Initialize a serial port for processing.

SioRI       Reads the Ring Indicator (RI) modem status bit.

SioRTS      Sets, clears, or reads the Request to Send (RTS) line.

SioRxClear  Clears the receive buffer.

SioRxBuf    Sets up receive buffer.

SioRxQue    Returns the number of characters in the receive queue.

SioTimer    Returns the number of system clock tics.

SioTxBuf    Sets up transmit buffer.

SioTxClear  Clears the transmit buffer.

SioTxFlush  Restarts transmitter interrupts thus flushing the TX queue.

SioTxQue    Returns the number of characters in the transmit queue.

SioUART     Sets the UART base address.

SioUnGetc   "Un-gets" (puts back) a specified character.

9.3 Further Reading

The best way to learn about serial communications is to read a good

book on the subject. Several good texts are available.  Two that I

like are:

(1) C Programmers's Guide to Serial Communications by Joe Campbell

(SAMS)

(2) Mastering Serial Communications by Peter Gofton (SYBEX).

PCL4C Users Manual                                        Page 34

10.0 Other MarshallSoft Computing Products

10.1 The Personal Protocol Library for C/C++

The Personal Protocol Library for C (PPL4C) consists of a C/C++

language library which implements XMODEM, XMODEM-CRC, XMODEM-1K,

XMODEM-G, YMODEM, YMODEM-G, and ZMODEM file transfer protocols.  A

script compiler and interpreter is also included which is capable of

such tasks as automatically logging onto a BBS and downloading a file

or retrieving mail.  Three example script programs are included.

The protocol library (PPL4C) requires the Personal Communications

Library for C (PCL4C).

The Personal Protocol Library for C is available for $40 plus $5 S&H

($10 S&H overseas).

10.2 The EMS Expanded Memory Library

The EMS4C library implements version 3.2 of the LIM

(Lotus-Intel-Microsoft) specification for expanded memory. It will

run with either version 3.2 or 4.0 of the LIM specification.

The EMM4C library (included with the EMS4C library) is an expanded

memory manager which allows C programmers to allocate and free EMS

(expanded) memory similiar to malloc and free in the standard C

runtime library.  Both EMM4C and EMS4C require that your system be

configured with expanded (EMS) memory. But, 386 & up systems can use

extended memory as expanded memory.

The EMS Expanded Memory Library for C is available for $35 plus $5

S&H ($10 S&H overseas).

10.3 Other Products

Marshallsoft Computing, Inc. makes serial communications libraries

for Windows (C/C++, Visual Basic, and DELPHI), Visual Basic (DOS),

Power Basic (DOS), and Turbo/Borland Pascal.

PCL4C Users Manual                                        Page 35