------------------------------------------------------------------------

From    : Pierre Cornelis, ON7PC    rue Ballings, 88    B-1140 Brussels

Phone : + 32 2 241 5805

To      : All

Date    : March 30, 1993

Subject : TheNetNode version 1.49 : "Quick documentation"

------------------------------------------------------------------------

1. SYSOP COMMANDS :

===================

AUTO <port_number> :  computes automatically IRTT and T2  [version ON5ZS]

BCnodes <+|-> <port> <call> <digipeater>

BCnodes !          :  broadcast immediately

Beacon <port> = <text>  : set or change the beacon text (+ param 18)

Beacon <port> <time> <tlm> <name>

<time> is the time interval in minutes (recommended value = 10 min)

TLM 0 = No telemetry, 1 = telemetry, 2 = telemetry + link statistics

<name> is the name of the unnumbered info frame (recommended = QST)

CLEAR     :  clears the token ring.

CLEAR *   :  clears all the statistics.

CVshost <+|-> <port> <call> <digipeater>

DOS <command> :  Performs the internal DOS commands, for example :

DOS DIR,  DOS d:,  DOC CD d:\path,  DOS DEL d:\path\filename.ext

DOS COPY d1:path1\filename1.ext1 d2:path2\filename2.ext2 , etc ...

EDIT <text>

ERASE <text>

IO <port_bit> <1|0>

LINKS <+|-> <port> <call>

LOAD <filename>

LOcal <+|-> <Call> <Alias> <Quality> <Port>

MHEARD = <value>

NODE <nodecall> <+|-> <ident> <quality> <count> <port> <neighbor> [<digicall>]

PARAMETER <number> <value>

PATH <+|-> <call1> <call2>

PORT <rx_port_number> <tx_port_number> <mode> <port_name>

<rx_port_number>, this refers to a logical number in TheNetNode

<tx_port_number>, this refers to a logical number in TheNetNode

<mode> represents the following bits :

bit 0 : value = 1   : 1 = TNC OK          0 = TNC locked

bit 1 : value = 2   : 1 = user port       0 = link port

bit 2 : value = 4   : 1 = CTEXT on        0 = CTEXT off

bit 3 : value = 8   : 1 = DAMA on         0 = DAMA off

bit 4 : value = 16  : 1 = digipeater on   0 = digipeater off

bit 5 : value = 32  : 1 = RMNC

bit 6 : value = 64  : 1 = No link via digipeater

bit 7 : value = 128 : 1 = T2 dynamic

<port_name> is a name for each port.

Bit 5, 6 and 7 are implemented in the version of ON5ZS :

PROMPT <string>

%a  for the nodes identifier

%c  for the user call

%C  for the user call with SSID

%d  for the node call

%D  for the node call with SSID

%n  for the users name

%r  for a carriage return

%t  for the time

%0  to clear the prompt

READ <filename>

RESET SYSTEM

RESET <port>

ROUTES <port> <nodecall>[digicall] <+|-> <quality>

STAT <+|-> <call>

SUSPEND <+|-> <port> <callsign> <number_of_connections>

Two pseudo port number may be used : 255 to lockout on ALL ports,

and 254 to allow only L2 connections.

SYSOP

TEST <port>

2. THE ESCAPE COMMANDS :

========================

ALT X        Stops the program

ESC C        Connect the keyboard as host.

ESC D        Disconnect.

ESC F <path> Modify the path for CONFIG.NET and DOS commands.

ESC I   Shows the identification of the node, the four parameters can be

modified separately with

ESC I0 <node_callsign>        e.g. ESC I0 ON7RC

ESC I1 <node_identifier>      e.g. ESC I1 BRU

ESC I2 <login>                e.g. ESC I2 this_is_my_node

ESC I3 <password>             e.g. ESC I3 TOP_SECRET

By default, the login is "thenet" and the password "Geheim".

ESC K   Gives date and time

ESC K0  Monitoring without date and time

ESC K1  Time and date will be added in monitoring

ESC L       Shows the status

ESC Ln      Shows the status of channel n

ESC LOGOUT  Opposite of the login ...

ESC M x Controls the monitoring :

ESC M N    No monitor

ESC M I    Monitors only information frames

ESC M U    Monitors only unnumbered information frames

ESC M S    Monitors only supervisory frames

ESC MUSIC  Monitor all kind of frames

ESC <+|-> MUSIC + ON5ZS ON6NR  add or remove monitoring of

all frames with ON5ZS or ON6NR in the address field.

ESC P       Gives the password

ESC QUIT     Saves all the configuration parameters and quits TheNetNode.

ESC R0       No trace of token recoveries

ESC R1       Trace the token recoveries.

ESC S        Shows the used channel

ESC S n      Switch to channel n

ESC T        Shows the token ring speed

ESC T 19200  Sets the token ring speed at 19200 bit/s

ESC Y0       No connection to the host

ESC Y1 - Y4

3. THE USER COMMANDS :

======================

Aktuell

BCnodes

BEacon

CONNECT

CONvers

/?, /HELP            Returns the help information.

/BYE, /EXIT, /QUIT   Exits from CONVERS mode.

/CHANNEL n           Changes the CONVERS channel to n.

/INVITE user         To ask another user to oin you on the channel.

/LINKS [LONG]        list all connections on the OTHER TNN nodes.

/MSG user text...

/WRITE user text...  Sends a message containing 'text' to 'user'.

/WHO [QUICK] [LONG]  Lists out all the users and the channel they

are listening on.

CQ

HArdware

Help

Help INDEX

Help <file_name>

Info

Links

LOcal

Map

MHeard

Nodes

Node <call> or Node <partial_Call>*

NODES #

NODES *

Parms

Path

POrt

Quit

Routes

SOftware

Statistics

S *

Time

User

User +

Version