THENET @EU       de:ON5ZS  16.12.91 14:03 101   2285 Bytes

TNN + 16550 , faster

*** Bulletin-ID: 16C105DB0GE ***

911216/1316Z ON1BWP, 911216/1310Z ON6AR , 911216/1313Z ON1AEO

911216/1207z ON4UBO, 911216/1155z DB0IZ , 911216/1141z DB0GE

de ON5ZS @ DB0GE

Hello everybody,

Some time ago, I writted RS232_PC.C who could support the 16550. As you could

see a few intermediate versions were written in order to detect the origin

of some errors. These releases worked more than 3 weeks on ON5PL and ON7RC.

Now that we have transferred more than 300 Mbytes over these nodes we can

be sure that there is

NO framing error,

NO fifo error, and,

NO overrun error.

We can assert that the Maximum_rx_buffer_used is related to the speed of the

computer and that an average value is 1000 bytes, and that the

Maximum_tx_buffer_used is 4000 bytes (occurs only with a "TEST" command).

The two circular buffers are properly sized with their 12288 bytes.

All these test will not be performed anymore in the next releases and that

will speed the things a little bit ...

Results after one week of use with the 16550 :

Node   nbr of TNC  computer type  token-speed   minimun   maximun

-----------------------------------------------------------------

ON5PL       5        80286/12     57600 Baud      33     134 token/sec

ON5PL       5        80286/12    115200 Baud      89     201 token/sec

ON7RC       8        80386/33     57600 Baud      80     174 token/sec

Note: In TNN version 1.4 the number of token is measured for a 60 second

period, and the result is then divided by 60.

In my version the measuring period is 8 sec and the result is of

cause divided by 8 ! This give a beter idea of the limit values.

In a following message, you will find the source of the present version.

73 from Daniel (ON5ZS @ LX0PAC)