MSYS

(C) Copyright 1993 By HUB COMPUTERS, INC.

All Rights Reserved

INTRODUCTION

MSYS is a Copyrighted program that I am making available to the

amateur radio community free of charge.  Source code is not generally

available (if you want it you will have to pay dearly for it!).

KA-NODE is a trademark of Kantronics Inc.  The KANODe component of

MSYS is compatible with the Kantronics KA-NODe but is completely

different code from that used in the Kantronics tncs.

NET/ROM is a trademark of SOFTWARE 2000.  The network NODe component

of MSYS is compatible with NET/ROMs but is completely different code

from that sold by SOFTWARE 2000.

The network NODe component of MSYS is completely different from that

in the KA9Q-NOS program and G8BPQ Node software.

IBM is a trademark of International Business Machines Corp.

SAM is a trademark of RT Systems

j-comm is a trademark of j-comm Inc.

MSYS is available from the Cleveland Hamnet phone BBS. The number is

216-942-6382.  It's speed is 300 to 14,400 baud.  This is a free BBS

(you pay for the call).  MSYS may also appear on other phone BBS

systems, but this is the only one I put it on directly.  MSYS may

also be obtained by anonymous ftp on Internet from

coffman.csuohio.edu  [137.148.21.6].

You may also obtain MSYS from me directly on disk.  Distribution is

on 5.25 inch 360K formatted disks. Costs per disk which includes the

disk, mailer and postage are as follows:

U.S.                $5.00 per disk

Canada & Mexico     US$7.50 per disk

All Others          US$10.00 per disk

The number of disks may vary from release to release.  Recent

releases have been one disk to three disks.  Four disks may be needed

in the future.  You may order several versions at a time, I will hold

your request until the versions are released.  This way you get to be

the first kid on the block with the new toy!  When ordering from me,

please specify which version you already have or which one you want..

I keep records with the following information for people who order

MSYS, so you may want to give me this info.  Also please indicate if

you want me to call you (collect) if it would be helpful to answer

questions you send me about MSYS.

CALL:          Your callsign.  This is the key I use to record all

your                information.

NAME:          Where to send the disk to!

ADDR1:

ADDR2:

CITY:

STATE:

ZIP:

COUNTRY:

TELEPHONE:     If you want me to call you back (collect)

VERSION:       The version you have now

OWED:          How many you have prepaid

COMPUTER:      Speed, memory, disk space, CPU chip etc.

TNC:           Model & ROM versions

DOS:           Which version?

BBS:           BBS callsign to send you msgs .

IP:            Your IP address, if you have one

Mail requests for MSYS on disk to:

Michael Pechura

10809 Beechwood Drive

Chesterland, OH 44026

You may call me with questions/comments at   216-256-1588.

A stripped down version of MSYS that supports only the BBS functions

is also available.  There is no network NODe, no amtor, no tcp/ip in

this version.  You can still forward using external network nodes,

but the bbs is NOT a netrom network node!  The KANODe is still there.

All of the BBS functions remain, along with modem support.  Expanded

memory is also still supported.  This version is called MSYSB and is

available on a separate disk or on Cleveland Hamnet.  If want to get

this disk from me, it is the same price as the others: $5 to US

addresses, $7.50 to Canada and Mexico, and $10 to other countries.

MSYS 1.16                                                   Page 2

A Multi-User, Multi-Port, Multi-Function

Amateur Packet Radio BBS

By Mike Pechura, WA8BXN

(C) Copyright 1993 By HUB COMPUTERS, INC.

All rights reserved.

Version 1.16  DECEMBER 1993

MSYS Documentation Table of Contents

INTRODUCTION........................................................1

MSYS DOCUMENTATION TABLE OF CONTENTS................................3

MSYS INITIAL INSTALLATION...........................................4

MSYS UPDATE INSTALLATION INSTRUCTIONS...............................6

SETUP CHANGES.....................................................6

ADDITIONS & CHANGES...............................................7

COMMENTS ABOUT CHANGES............................................7

MSYS COMMAND LINE.................................................8

ABBREVIATIONS/CONVENTIONS.........................................9

MSYS DIRECTORY LAYOUT DESCRIPTION..................................10

MSYS.OPT.........................................................11

MASK INFORMATION...................................................21

BIT PATTERN CHART..................................................21

DRSI CARDS.........................................................23

MSYS COMMAND SUMMARY...............................................24

MSYS SYSOP COMMANDS..............................................32

MSYS BBS USER COMMANDS...........................................72

REQXXX SERVERS.....................................................84

CALLSIGN SERVER....................................................86

THINGS NEEDED TO MAKE THE CD-ROM DATA BASE CALLSIGN SERVERS WORK:86

THINGS NEEDED TO MAKE THE SAM (C) DATA BASE CALLSIGN SERVER WORK:87

MODEM SUPPORT......................................................88

MSYS MODEM FORWARDING PROTOCOL...................................88

INCOMING CALL PROTOCOL...........................................88

MSYS.OPT FILES...................................................90

MSYS.DO..........................................................90

MSYSTNC.X FILES..................................................91

MODEM USAGE AT 4.8, 9.6 AND 14.4K................................93

NETNODE USAGE....................................................93

FORWARDING ON THE MODEM PORT.....................................94

FBB ON THE PHONE.................................................95

FORWARDING TO AA4RE SYSTEMS......................................95

ANONYMOUS CHECK-INS..............................................96

ACOUSTIC COUPLED MODEMS..........................................98

MISCELLANEOUS NOTES..............................................98

DATABASE HELP......................................................99

FORWARD FILE DESCRIPTION..........................................101

NODE SCRIPTS....................................................104

MSYSBBSB.DAT....................................................106

(CALLSIGN).FWD/.REV FILES.......................................107

MSYS 1.16                                                   Page 3

MERGE: (MUST DO THIS IN ORDER TO FORWARD MAIL)..................108

CALLSIGN @ BBS REPLACEMENT......................................109

REVERSE FORWARDING..............................................110

SELECTIVE FORWARDING:...........................................111

TCP/IP FORWARDING:..............................................112

SMTP FORWARDING.................................................113

FORWARDING NOTES..................................................114

BID:............................................................114

FORWARDING DISPLAY..............................................115

HF FORWARDING...................................................116

MSYS NODE FORWARDING............................................116

PACTOR FORWARDING...............................................117

MISSING TEXT TO MESSAGE.........................................117

FBB COMPATIBLE FORWARDING.......................................118

FBB COMPATIBLE COMPRESSED FORWARDING............................119

W0RLI STANDARD FOR DATA COMPRESSION DURING THE FORWARDING OF

MESSAGES IN THE AMATEUR RADIO BBS NETWORK.......................121

HIERARCHICAL FORWARDING AND MSYS..................................123

CREATING THE BBSTONTS.BIN FILE..................................125

HOUSECLEANING (HCL COMMAND).......................................126

MSYS.HCL........................................................126

MSYS NETWORK NODE INTRODUCTION....................................128

ROUTING.........................................................128

BASIC DECISIONS ABOUT THE NETWORK NODE..........................129

NETWORK NODE QUICKSTART.........................................129

MANUAL OPERATIONS ON THE NETWORK NODE DATABASE..................130

ONE POSSIBLE NETWORK NODE SETUP.................................131

NODE RELATED PARMS IN MSYS.DO...................................132

MISCELLANEOUS COMMENTS ON NETWORK NODE..........................134

NET NODE PARAMETERS SUMMARY.....................................135

PACTOR............................................................137

OVERVIEW........................................................137

HARDWARE........................................................137

PACTOR SET UP...................................................137

PACTOR TNC PARAMETERS...........................................138

PACTOR MANUAL CONNECTS..........................................138

PACTOR FORWARDING...............................................138

USER'S VIEW WHEN CONNECTED......................................139

PACTOR LIMITATIONS..............................................139

PACTOR TROUBLESHOOTING..........................................140

MSYS PACTOR SCANNING..............................................140

IOBUS.DOC.......................................................140

INTERRUPTS......................................................140

ADDRESSING......................................................141

DSR LINE........................................................141

TTL TO RS-232 INTERFACE (1488/1489 VERSION).....................142

TTL TO RS-232 INTERFACE (MAX232)................................143

SCANDEF UTILITY PROGRAM.........................................145

TURNING IT ON AND OFF...........................................146

ADDING ANTENNAS.................................................147

THE LAST STEP, THE TSR..........................................148

USING OTHER RADIOS..............................................148

MSYS 1.16                                                   Page 4

MSYS GENERAL INFORMATION..........................................149

BBS ONLY ACCESS.................................................149

CONNECTS FROM NON-AMATEUR CALLSIGNS.............................149

#  COMMAND......................................................149

CHANNEL NUMBERS.................................................149

DEVICE ERRORS...................................................149

DISK FULL MESSAGE...............................................149

DOS.............................................................150

END OF LINE BELLS:..............................................150

HEADERS.........................................................150

KEYBOARD........................................................150

***LINKED.......................................................150

LOCKUPS.........................................................151

LOGGING.........................................................151

MAIL BEACON.....................................................151

MAIL.BAK........................................................151

MAX CHANNELS AND PORTS..........................................151

MEMORY MANAGEMENT...............................................152

EXPANDED MEMORY USAGE...........................................153

AUTOMATIC MESSAGE HOLDING.......................................153

MSYS.DO.........................................................156

MSYSTODO.DAT....................................................156

MSYSTNC.#.......................................................157

SPECIAL KISS PARAMETERS.........................................157

PACKET SIZE.....................................................158

PASSWORDS.......................................................158

REMOTE SYSOP....................................................158

AUTOMATIC PASSWORD REPLY........................................159

USER PASSWORDS..................................................159

THENET & NETROM RS-232 CONNECTIONS..............................160

SMTP............................................................160

STACK REQUIREMENT...............................................161

TERM............................................................161

TESTING.........................................................162

TITLES TRUNCATED................................................162

TYPE OF MESSAGE.................................................162

USER BIT SETTINGS:..............................................165

WP SERVER:......................................................166

KANTRONICS 3.0 EPROM UPGRADE USERS:.............................166

FREQUENT PROBLEMS AND NOTES FROM THE AUTHOR.......................167

OVERRUNS, FRAMING ERRORS........................................167

ABRUPT STOPS IN FORWARDING......................................168

MESSAGES DON'T FORWARD..........................................168

THE SYSTEM FORWARDS MESSAGES TO IMPROPER LOCATIONS..............168

THE SYSTEM APPEARS TO WORK BUT:.................................168

THE SYSTEM COMES UP AND APPEARS TO HANG.........................169

SYSTEM REBOOTS AFTER FORWARDING.................................169

PERFORMANCE HINTS...............................................169

MFJ KISS MODE...................................................170

ADDITIONAL NOTES................................................170

OS2 REPORTED PROBLEMS...........................................171

FINAL NOTE......................................................171

MSYS 1.16                                                   Page 5

MSYS UTILITY SUPPORT PROGRAMS.....................................172

MUTIL.EXE.......................................................172

MUTIL FUNCTION 1 - CHANGE MAX NUMBER OF MESSAGES................173

MUTIL FUNCTION 2 - RESTORE MESSAGES.............................174

MUTIL FUNCTION 3 - PRINT REMOTE SYSOP PHRASE....................175

MUTIL FUNCTION 4 - CHECK MAIL INDEX FILE INTEGRITY..............176

MUTIL FUNCTION 5 - TEST SERIAL PORTS............................177

MUTIL FUNCTION 6 - PRINT FORWARDING TREE........................178

MUTIL FUNCTION 8 - RUN LOG FILE ANALYZER........................179

MUTIL FUNCTION 9 - RUN SETUP PROGRAM............................181

MUTIL FUNCTION 10 - SCAN MESSAGE HEADERS........................182

MUTIL FUNCTION 11 - DELETES OLD ENTRIES.........................183

MUTIL FUNCTION 12 - FIX MAIL INDEX FILE.........................183

RENUMBERING MESSAGES............................................184

MUTIL FUNCTION 13 - BBSLIST.DAT TO BBSTONTS.DAT.................185

MUTIL FUNCTION 14 - BBSTONTS.DAT TO BBSTONTS.BIN................187

MUTIL FUNCTION 15 - BUILD LIST OF ARCHIVED MESSAGES.............187

MUTIL FUNCTION 16 - PRINTER STATUS..............................187

MUTIL FUNCTION 17 - MERGE BBSLIST.DAT FILES.....................188

MUTIL FUNCTION 18   CHECK MSYSNODE.DAT INTEGRITY................188

MUTIL FUNCTION 19 - CONVERT USER FILES..........................188

MUTIL FUNCTION 20 - SORT BBSLIST.DAT FILE.......................189

MUTIL FUNCTION 21 - DELETE OLD BIDS (AND MIDS)..................189

MUTIL FUNCTION 22 - SPECIFY PRINTER FILE NAME...................189

SAMPLES OF MSYS FILES.............................................190

MSYS.OPT:.......................................................190

MSYS.DO:........................................................191

MSYS.OTD:.......................................................194

MSYSK.OTD.......................................................194

MSYS.REP:.......................................................194

MSYS.RMT:.......................................................195

MSYSBBSB.DAT:...................................................195

MSYSFWD.DEF:....................................................196

MSYSHOST.NET:...................................................197

MSYSPASS.DAT:...................................................197

MSYSPASS.IP.....................................................197

TYPICAL MSYSTNC.1...............................................198

(AEA PK-232)....................................................198

INFORMATION FOR ADVANCED USERS....................................199

REPLACEABLE DIRECTORY NAMES.....................................199

REPLACEABLE FILE NAMES..........................................200

MSYS MSG FILE FORMAT:(FOR THE ADVENTURESOME ONLY!)..............202

ROUTES.DAT FILE FORMAT:.........................................203

MSYS UNDER WINDOWS 3.1..........................................204

MSYS UNDER DESQVIEW.............................................205

SYSOP COMMANDS RELATED TO TCP/IP................................207

FILES RELATED TO TCP/IP.........................................208

USER ACCESS LEVEL CHART.........................................210

FTP COMMANDS:...................................................210

IP ROUTING:.....................................................211

EDITVIEW (ED COMMAND)...........................................212

EDITFILE (EF COMMAND)...........................................214

MSYS 1.16                                                   Page 6

EPILOG..........................................................217

INDEX.............................................................218

MSYS 1.16                                                   Page 7

MSYS INITIAL INSTALLATION

Copyright 1993 by HUB COMPUTERS, INC.

1. Installation of MSYS is not particularly difficult but you should

follow the following steps in order.

2. If you are upgrading from MSYS 1.15, please see the UPDATE

Instructions.

3. Make a backup copy of the distribution disk.  You can copy the

entire disk with Diskcopy.

4. Make a default directory on your hard disk.  I call mine MSYS.

Use the command:

md c:\msys

5. NOTE: You may use any drive letter.  All references to the C:

drive apply to your selected drive.

6. Go to the default directory: cd c:\msys.

7. Make required sub directories:

md c:\msys\mail

md c:\msys\files

md c:\msys\help

md c:\msys\mail.bak

md c:\msys\mail.bin             ***NEW VERSION 1.14

8. Move the file MSYS116.exe into the c:\MSYS directory

9. Un-arc the distribution disk by running the program MSYS116.EXE.

10.To move the help files to the help sub-directory.  Type MOVEFILE

11.NOTE:  REQUIRED FILE:  C:\MSYS\HELP\MSYSMSGS.DAT

This file contains a number of the messages used by MSYS.

12.Change Directory to SAMPLES.  Edit the file

C:\MSYS\SAMPLES\MSYS.OPT with your favorite program editor.  See

documentation on MSYS.OPT for details.  After editing move

MSYS.OPT file to the default directory.

13.RUN MUTIL.EXE Function 9 to create C:\MSYS\MSYS.DEF from your

edited MSYS.OPT

MSYS 1.16                                                   Page 8

14.Make sure you have at least:

files=40              ***CHANGE VERSION 1.14

buffers=20

in your config.sys file in the root directory of your boot disk.

Don't forget to reboot after changing this!

15.Using any ASCII editor, create MSYSFWD.DEF.  If you don't know

what to put it in now, put the following (to be replaced later):

F0 WA8BXN

WA8BXN

-----

16.Type MSYS and it should begin to run!

17.Type MERge. You can ignore any messages about MSYSBBSB.DAT at

this time.

SPECIAL NOTE

If you have a color monitor or any kind of a color card, the

default is OK.  If you have a monochrome monitor that uses and

RCA phono pin plug then you are using a Color Graphics Adapter

interface card and the default should be OK.  If you can do the

command MODE MONO (this is a DOS command) and not get any error

messages and the computer continues to work, you probably have a

monochrome (TTL) interface card.  You will need to specify WINDOW

DISPLAY MONOCHROME in the MSYS.OPT file.

MSYS 1.16                                                   Page 9

MSYS UPDATE INSTALLATION INSTRUCTIONS

SETUP CHANGES

There are two new directories that are required to update MSYS Ver.

1.13 to Ver. 1.16  One is the MSYS\MAIL.BIN directory that is used to

store the compressed messages as they are received and sent, and the

other is MSYS\YAPP which is used to store files that have been

transferred via YAPP compressed protocol.  Please see the MSYS

DIRECTORY LAYOUT DESCRIPTION for further details.  Use DOS commands

to make the new sub-directories.  If you are already running MSYS

1.15, no further changes need to be made in the directory structure.

Increase your FILES statement in CONFIG.SYS to files=40.  BUFFERS can

remain at 20.

It is suggested that a program similar to the Quarterdeck Systems:

VIDRAM ON be run prior to running MSYS to create as much working

memory as possible.  MSYS does not require graphics capability.  Of

course, if other background applications require graphics, this

option is not available.  VIDRAM works with EGA or VGA video

adapters, and should not be loaded into upper memory.

Run MUTIL 9 (setup) and of course, MUTIL 4 before restarting the new

version of MSYS.

MSYS 1.16                                                   Page 10

ADDITIONS & CHANGES

There have been many changes to MSYS, some major and many minor.

Here are some of the "major" changes:

1. Support of PACTOR on KAM Plus tncs (ONLY!) has been added.

2. "Attended" Pactor forwarding on HF can be initiated from the

keyboard at the same time VHF forwarding is in progress.

3. Scanning of HF radios is supported in the Pactor mode.

4. A new TERM command has been added to permit direct keyboard

access to a port.  It is expected this will be used to perhaps

modify certain TNC or modem parameters.

5. Modem connection to FBB systems for forwarding has been improved.

6. The ability to read messages while listing them has been added.

7. The SR (Send Reply) command has been added in addition to the

REPLy command.

8. The BITs of personal messages are updated upon forwarding so if

the message is not killed on forwarding, the sysop may determine

which bbs the message was forwarded to.

9. The memory leak caused by an incomplete modem forwarding session

has been fixed.

10.MUTIL 15 has been updated to properly search R: lines.

11.If a message disappears during forwarding (sysop kill), the

connection will be terminated.

12.User passwords have been fixed.

13.A number of changes have been made in the Listing and Reading and

SYSOP killing of messages.

14.A SHUTdown command added to "encourage" current users to

disconnect prior to a QUIT.

15.A ListCatagories (LC) command has been added to the BBS.

16.A RBid command has been added to insert the received BID/MID

number in the R: line.

17.W0RLI compatable message forwarding compression has been added.

COMMENTS ABOUT CHANGES

One of the major changes in Ver. 1.16 is the addition of PACTOR.

MSYS properly interfaces a KAM Plus that has the proper firmware

installed.  At this point, the firmware provided by MFJ and AEA does

not permit positive control of direction of transmission, a most

undesireable conditon to inflict on the HF bands.

The second major change is the addition of the W0RLI compressed

message forwarding protocol.  MSYS is currently the only BBS system

that is compatable with both the F6FBB and the W0RLI compression

protocols.

MSYS 1.16                                                   Page 11

MSYS COMMAND LINE

The MSYS command typed at the DOS prompt may have several switches or

modifiers.

NOHEARD   Disables JK,JM,JD,JG,JB and JT recording.  Saves

26,880 bytes

NONODE    Disables network node, saves 28,104 bytes

NORAMHDRS Stores message headers on disk only, saves 136 bytes

per possible message.  This can be quite slow if you

have lots of msgs and no disk cache. Could be useful

on 286 systems with cache in EXTENDED memory.

NOEMM     Disables Expanded Memory.

NOEXMEM   Disables Extended Memory.

Examples:

MSYS NOHEARD      Starts MSYS but will not log any stations it

hears.

MSYS NONODE       Starts MSYS without the NETNODe feature.

MSYS NORAMHDRS    Starts MSYS and stores all headers on disk

not in RAM

MSYS NOEMM        Starts MSYS and forces MSYS to use EXTENDED

memory (if any).

MSYS              Starts MSYS, permitting it to use EXPANDED

memory, if any) and all other features that

have been optioned.

MSYS has the ability to determine if there is an EXTENDED or EXPANDED

memory manager in use.  It will attempt to use the EXPANDED memory

unless the NOEMM option is specified, in which case it will attempt

to use the EXTENDED memory.

While MSYS may be run on an 8088 or 8086 system, much better

performance will be achieved by using a 286 or higher system.

MSYS 1.16                                                   Page 12

ABBREVIATIONS/CONVENTIONS

Just a word about this manual.  Most commands are not case sensitive,

with the notable exception of the \Sysop command.  Many commands may

be abbreviated.  In this manual, we have attempted to show the

minimum abbreviation with the UPPER CASE letters with the balance of

the command word in lower case.  For example, the command ADDUser may

be typed as:  addu  or  ADDU  or  adduser  or  ADDUSER  with the same

results.  Where there is an argument shown as [on/OFF], the UPPER

CASE will be the default.  For example, in the case of the command

PC [on/OFF],  the call server lookup defaults to OFF.

MSYS 1.16                                                   Page 13

MSYS DIRECTORY LAYOUT DESCRIPTION

The following chart is the recommended organization for your

directories and files.  NOTE: Some of these files are created at time

of installation and others are operating files that MSYS creates

while operating.

C:\MSYS  Contains all of the MSYS operating programs and

configuration  |         files

|  MSYS.EXE $     MSYS.MSG #      BIDLIST.DAT #  MSYSPASS.IP @

|  MUTIL.EXE $    MSYS.BBS #      MSYSPASS.MOD @ MSYSPASS.DAT @

|  ROUTES.DAT $   MSYS.DEF #      MSYSBBSB.DAT @ MSYS.HCL @

|  STATES.DAT $   MSYS.FWD #      MSYSTODO.DAT @ MSYSHCL.DO @

|  ZIPCODES.DAT $ MSYS.HRD #      MSYSHOLD.DAT @ MSYS.OPT @

|  CALLSRV3.EXE $ MSYS.LOG #      MSYSFWD.DEF @  MSYS.OTD @

|  MSYS.BBS #     MSYS.USR #      MSYSHOST.NET @ MSYS.REP @

|  BBSTONTS.BIN # MSYSNODE.DAT #  BADWORDS.DAT @ MSYS.RMT @

|  BBSTONTS.DAT # MSGLIST.DAT #   MSYSTNC.1 @    MSYS.DO @

|       Files marked with a: $ are Distribution files.

|                            # are created by MUTIL or MSYS

|                            @ are SYSOP created files

|

|--------FWD

|   Directory which may contain forwarding files (.FWD)

|   the reverse forwarding  files (.REV) and the

|   PASSWORD.RMT file and is used by MERge command

|

|--------HELP

|   Directory which contains HELP messages including

|   MSYSMSGS.DAT

|

|--------MAIL

|   Directory which contains ACTIVE messages

|

|--------MAIL.BAK ***Optional

|   Directory which may contain BACKUP messages  See ADVANCED

|   USER section for suggestion!

|

|--------MAIL.BIN

|   Directory which contains COMPRESSED messages

|

|--------PASSWORD ***Optional

|   Directory which may contain user PASSWORD files

|

|--------PUBLIC   ***Optional

|   Directory which may contain uploaded FTP/SMTP messages

|

|--------YAPP     ***Optional

|   Directory which may contain YAPP files

MSYS 1.16                                                   Page 14

MSYS.OPT

Before MSYS can be used a configuration file (MSYS.OPT) must be

created and the program SETUP (MUTIL.EXE Function 9) must be run to

create the file MSYS.DEF. The file MSYS.OPT is a  standard ASCII file

and may be created with any text editor.  The basic format of the

commands in the file are:

PORT number AT number INT number SPEED number [NAME description]

PORT number SUBPORT number OF [PORT] number [NAME description]

DIGI callsign FROM [PORT] number TO [PORT] number [AS callsign]

NUMCHANS number

BBS CALL callsign

BBS ALIAS callsign

KANODe CALL callsign

ANSWER CALL callsign

FORWARD CALL callsign

ID CALL callsign

ID EVERY number

AX25 FRACK number

AX25 MAXFRAMES number

AX25 RETRIES number

AX25 CHECK number

MONITOR IFRAMES on/off

MONITOR UFRAMES on/off

MONITOR SFRAMES on/off

MONITOR BFRAMES on/off

MONITOR OUTGOING on/off

MONITOR PORTS [MASK]

WINDOW number SIZE number number number number

WINDOW number BACKGROUND number

WINDOW number FOREGROUND number

WINDOW number PAGE number

WINDOW UNUSED BACKGROUND number

WINDOW UNUSED FOREGROUND number

WINDOW DISPLAY color/monochrome

PROCESS processname [WINDOW] number

STACK processname number ***NOT USED IF USING EXT/EXP MEMORY             (SEE

SYSOP NAME name

SYSOP QTH location

SYSOP ZIP zip code

MAKE

MSYS 1.16                                                   Page 15

Notes:

1.Items enclosed in square brackets ([...]) are optional

2.Numbers assumed in decimal unless prefixed by $ then hex is

assumed

3.on/off means the word ON or OFF

4.Where more than 1 space is shown, 1 or more space are OK

5.Callsign can include SSID (-number)

6.Ordering of the statements is generally not important

7.If there is an * at the beginning of a line it is ignored

8.The BBS alias, KANODe, Answer and Forward calls must be

different calls or SSID's.

9.MASK is described on page 24

Output from the execution of SETUP is the file MSYS.DEF (a binary

file) and a listing to the display console.  The listing consists of

two parts: the original input statements followed by a complete list

of all options generated (including defaults).

If any errors are detected, the MSYS.DEF file will not be created.

A sample of a MSYS.OPT file is found in MSYS.OPT on the distribution

disk.

A minimal MSYS.OPT file will have similar statements to these:

port 0 at $3F8 int 4 speed 1200 name 145.01

bbs call wa8bxn-13

answer  call wa8bxn-14

sysop name Mike

sysop qth Kirtland, OH

sysop zip 44026

id call wa8bxn

make

EXPLANATION OF EACH STATEMENT TYPE

PORT number at number INT number SPEED number [NAME Description]

[RADIOSPEED #][TNCtype#]   (ALL ON ONE LINE!)

PORT number SUBPORT number OF [PORT] number [NAME description]

The PORT statement has the above two variations.  The number

following the word PORT is the port number.  It must be between 0 and

6.  The version containing the word 'at' defines an actual serial

interface, while the SUBPORT version defines a second logical port

for multiport TNCs such as the KPC-4.  There must  be one PORT

statement for each RADIO port to be used AND each MODEM port to be

used.

Assign port numbers beginning with 0 in increments of 1.

MSYS 1.16                                                   Page 16

Following the word AT is the base I/O address of the 8250 in the

serial port, such as $3F8. Following the word  INT is the interrupt

number for the 8250, such as 4.

After SPEED is the baud rate to be used, such as 4800.  This is the

speed between the computer and the TNC or the MODEM.  It is STRONGLY

suggested that with faster modems that a value of 19200 be used to

prevent buffer overruns.  Some adjustment of this speed may be

necessary to avoid overruns on a per port basis.

Optionally the word NAME followed by some description of the port can

be given. A good description would be the corresponding radio

frequency.

If MSYS is interfacing a NET/ROM matrix, or a TNC using the NetRom

Serial protocol (NRS), precede the NAME with a ">" as in: >MATRIX

If MSYS is interfacing a modem, the NAME must be: MODEM

If MSYS is interfacing a PACTOR tnc, the NAME must contain: PACTOR.

You may choose to name it PACTOR_20M for better description.  The

name must not exceed 10 Characters total.

For the other form of the PORT statement, the number following the

word SUBPORT is the logical channel number, typically 1 for the KPC-

4.  Next comes the word  OF  in the statement optionally followed by

the word PORT.  Next comes the port number of a port statement

defining an actual port (i.e., contains  the word AT in its

definition).  The optional NAME and description is the same as above.

PORT description lines can also have two additional parameters:

RADIOSPEED bitrate# TNCTYPE #

RADIOSPEED specifies the bitrate used by the TNC to the radio (300 or

1200 probably).  If 300 bps is selected, shortened prompts are used

in the BBS and perhaps elsewhere.

TNCTYPE  is a number from 0 to 999.  TNC type 0 is a KPC-x or a KAM.

Any other number causes a file MSYSTNC.# to be read for

initialization statements to be sent to the TNC.  These parameters

would be used for a TNC other than a KPC-X or a KAM, or for the file

to initialize a modem..

NOTE: You must specify RADIOSPEED if you use TNCTYPE#

For example:

PORT 2 AT $2F8 INT3 SPEED 4800 NAME 145.01 RADIOSPEED 1200 TNCTYPE 1

MSYS 1.16                                                   Page 17

IN THE MSYSTNC.1 FILE YOU WOULD HAVE THE COMMANDS THAT  WOULD

PUT YOUR TNC INTO KISS MODE. (see note in general info section)

DIGI callsign FROM [PORT] number TO [PORT] number [AS callsign]

The DIGI statement defines a digipeater callsign.  Repeat this

statement type up to a total of 25 times as desired.  The first

callsign is the callsign that would be used in the VIA list by a

station trying to use the digipeater.  The port number following FROM

indicates the port that will be using this call.  The TO port number

indicated the port on which packets having this the first callsign as

digipeater should be output on.  For a normal digipeater the two port

numbers will be the same;  for a gateway they will be different.  The

optional AS callsign specifies a replacement for the first callsign

in the digipeater list when the packet is sent out the TO port.  If

omitted the first callsign is assumed (i.e., no change).  Consider

the following examples:

DIGI MYDIGI FROM 0 TO 0

DIGI MYDIGI FROM 1 TO 1

DIGI W1XX-3 FROM 0 to 0

This would allow the callsign MYDIGI to be used in the via list on

either port 0 or 1.  The packet is retransmitted on the same port was

heard on.  The call W1XX-3 can also be used to digipeat on port 0.

DIGI MYGATE FROM 0 TO 1

DIGI MYGATE FROM 1 TO 0

This pair would define a gateway between ports 0 and 1 with the

callsign MYGATE used in both directions.

DIGI TOF1 FROM 0 TO 1 AS TOF2

DIGI TOF2 FROM 1 TO 0 AS TOF1

This interesting pair allows TOF1 to be used as a gateway digipeater

call on port 0 (no effect if used on port 1).  When the packet is

repeated it will have TOF2 in the digipeater list where TOF1

originally was.  The opposite is true on port 1: When TOF2 is used as

a digipeater call the packet is repeated on port 0 with

TOF1 being substituted for TOF2.  The substitution is transparent to

the users.

MSYS 1.16                                                   Page 18

NUMCHANS number

NUMCHANS is used to specify the number of logical channels that will

be allocated.  Each connection (while connected) uses one logical

channel.  When the KANODe is being used a second logical channel is

used for outgoing connects.  Each logical channel requires about 7K

of memory when in use.  They are available for reuse when a station

disconnects.

BBS CALL callsign

This statement gives the callsign that can be connected to for the

BBS.  The callsign can contain an SSID.  This statement is also

optional; if omitted stations will not be able to connect to the BBS

unless a BBS alias is given.

BBS ALIAS callsign

This statement gives the callsign that can be connected to for the

BBS.  The callsign can contain an SSID.  This statement is also

optional; if omitted stations will not be able to connect to the BBS

unless a BBS CALL is given.

KANODE CALL callsign

This statement gives the callsign of the KANODe which is similar to

the KANODe provided in the Kantronics tncs.  The callsign can (and

probably should) contain an SSID.  This statement is optional; if

omitted stations will not be able to connect to the KANODe.

ANSWER CALL callsign

This statement gives the callsign that can be connected to by

stations wanting to talk to the SYSOP.  It is also used as the

callsign when the Connect command is used.  The callsign can (and

maybe should) contain an SSID.  This statement should not be omitted.

FORWARD CALL callsign

This statement gives the callsign used when the BBS is forwarding.

It can (and probably should) contain and SSID.  This statement should

not be omitted if the BBS is to do forwarding.

MSYS 1.16                                                   Page 19

ID CALL callsign

This statement gives the callsign that will be used for

identification purposes.  It must be your actual amateur callsign and

should NOT have an SSID specified.  This statement MUST be used.

ID EVERY number

This statement specifies the number of minutes between IDs.  The list

of stations for which there is mail is generated with every ID.

AX25 FRACK number

Number is the number of seconds to wait before retransmitting a

packet.  All of the AX25 statement forms are

optional.

AX25 MAXFRAMES number

Number specifies the maximum number of frames that can be outstanding

before waiting for an ACK.  Must be between 1 and 7.

AX25 RETRIES number

This is the number of retries that will be made before abandoning a

connection.

AX25 CHECK number

If there is no activity on a logical channel for this number of

seconds there will be an automatic disconnect.

MONITOR IFRAMES on/off

Turns on or off the displaying of Information frames (SABM, DISC,

UA).  All forms of the MONITOR statement are optional.

MONITOR SFRAMES on/off

Turns on or off the displaying of System (RR, RNR, REJ) frames.

MSYS 1.16                                                   Page 20

MONITOR BFRAMES on/off

Turns on or off the displaying of Beacon frames (Un-numbered

information).

MONITOR OUTGOING on/off

Turns on or off the displaying of all outgoing packets.

MONITOR PORTS  [MASK]

Determines which ports will be monitored.  The rightmost bit of this

number corresponds to port 0, the next bit to the left port 1 and so

on.  Ports with 1 bits are displayed (as selected above).  A value of

13 (decimal, $D in hex) would enable ports 0, 2 and 3, for example.

SEE MASK on page 24 for further details.

WINDOW number SIZE number number number number

This form of the window statement gives the bounds of the window

whose number is given after the word WINDOW.  The four numbers after

the word SIZE are the upper left column (1 to 80), the upper left row

(1 to 25), the lower right column, and the lower right row

respectively.  All of the WINDOW statement forms are optional and

should be used with great caution.

WINDOW number BACKGROUND number

WINDOW number FOREGROUND number

These two forms of the WINDOW statement give the background and

foreground colors of the window specified after the word WINDOW.  The

rightmost number is a color number from 0 to 15 for foregrounds and 0

to 7 for backgrounds.  The corresponding colors are:

Black - 0               Magenta - 5              Lt. Cyan - 11

Blue - 1                Brown - 6                Lt. Red - 12

Green - 2               Lt. Gray - 7             Lt. Mag - 13

Cyan - 3                Gray - 8                 Yellow - 14

Red - 4                 Lt. Blue - 9             White - 15

Lt. Green - 10

MSYS 1.16                                                   Page 21

WINDOW number PAGE number

This statement assigns a window to the specified page (0-3) which

corresponds to the F!, F2, F3, and F4 keys.

WINDOW UNUSED BACKGROUND number

WINDOW UNUSED FOREGROUND number

These two statements give background and foreground colors to the

places on the screen not assigned to any window.

WINDOW DISPLAY  color/monochrome

SEE SPECIAL NOTE on PAGE 8.

This statement selects the kind of display adapter you have, COLOR or

MONOCHROME.  If you have a CGA card, select COLOR.

PROCESS processname [WINDOW] number

This statement specifies the window that is to be used for output

from the allowable process names .  Be extremely careful if you use

this statement.

STACK processname number

This statement specifies the number of bytes to be allocated for the

stack for the allowable process names .  You probably shouldn't mess

with the defaults!  ***THIS OPTION IS IGNORED ON A 286/386 equipped

with EMM or EMS.

DISCUSSION:

First, if using extended or expanded memory, you have no control over

stack size, they are all 8K. Only when just 640K (or less if that is

possible) regular memory alone is used can adjustments be made to

stack sizes.  Normally these should not be necessary.  The default

values set by MUTIL 9 should be correct.  This means that normally

there should be no process stack statements in MSYS.OPT.

How to tell when you need to adjust stack sizes --- If you get

unexpected crashes that can't be solved by other means, do somewhat

frequent process commands when conditions are similar to when crashes

occur (like when lots of channels are in use, when forwarding is

happening, etc.). If the SPmin value for any process gets below a few

hundred an increase in the stack allocation for that kind of process

may be needed.  In the event SPmin goes to 0 or negative, an

adjustment is required.

MSYS 1.16                                                   Page 22

How to adjust stack sizes --- Get a printed copy of the output from

MUTIL 9. You will see there a list of the process type names and

current stack sizes for each.  Add a process stack line in MSYS.OPT

for the one you need to increase. Any increases should probably be

done in 1K increments.

Stack sizes should probably not be adjust downward greatly from the

default values even though it appears on most process command lists

that the SPmin is always very high. How much stack space is needed is

dependent on the particular activities that are occurring.  Values

that appear excessive may be required for infrequently performed

operations that if done with small stacks would result in

unpredictable results. (Can you say "CRASH"?  How about "HANG"?)

SYSOP NAME name

You must use this statement to specify your first name.

SYSOP QTH location

You must use this statement to specify your location.

SYSOP ZIP postal code

You must use this statement to specify your zip or postal code.

MAKE

Use this statement if you want to produce the new MSYS.DEF file if

there are no errors.

MSYS 1.16                                                   Page 23

Here is a sample MSYS.OPT file for you to figure out!

port 1 at $3e8 int 4 speed 1200 name 223.70

port 0 at $2f8 int 3 speed 1200 name 145.01

digi bxng from 0 to 0 as bxng

digi bxng from 1 to 1 as bxng

digi bxn501 from 0 to 0 as bxn501

digi bxn501 from 1 to 0 as bxn370

digi bxn370 from 0 to 1 as bxn501

digi bxn370 from port 1 to port 1 as bxn370

digi test-4 from port 0 to port 1 as test-5

digi wa8bxn-3 from 0 to 0

bbs call wa8bxn-13

kanode call wa8bxn-1

answer  call wa8bxn-14

forward call wa8bxn-12

sysop name Mike

sysop qth Kirtland, OH

sysop zip 44026

id call wa8bxn

window display color

make

MSYS 1.16                                                   Page 24

MASK INFORMATION

Several commands require the use of a MASK to determine the exact

ports you wish to perform the function.  For example, you.  You may

wish to only permit NetRom connects on port 2, or you may wish to

have bbs connects only on port 3, or you may wish to monitor only

ports 2 , 3 and 8 on the F2 screen.  The MASK is merely a hexadecimal

representation of the ports.  The BIT PATTERN CHART is the way to

determine what the MASK value will be.

BIT PATTERN CHART

To be used for DPorts, KPorts, MONPorts, NPorts and TPorts

MASK VALUE      PORT NUMBER                 CONDITION

6  5   4  3  2  1  0          1=ON, 0=OFF

0       0  0   0  0  0  0  0            All Off

1       0  0   0  0  0  0  1            0 - ON

2       0  0   0  0  0  1  0            1 - ON

3       0  0   0  0  0  1  1            1,0 - ON

4       0  0   0  0  1  0  0            2 - ON

5       0  0   0  0  1  0  1            0,2 - ON

6       0  0   0  0  1  1  0            2,1 - ON

7       0  0   0  0  1  1  1            0,1,2 - ON

8       0  0   0  1  0  0  0            3 - ON

9       0  0   0  1  0  0  1            3,0 - ON

A       0  0   0  1  0  1  0            3,1 - ON

B       0  0   0  1  0  1  1            0,1,3 - ON

C       0  0   0  1  1  0  0            3,2 - ON

D       0  0   0  1  1  0  1            0,2,3 - ON

E       0  0   0  1  1  1  0            1,2,3 - ON

F       0  0   0  1  1  1  1            0,1,2,3 - ON

--------------------------------------------------------------

MSYS 1.16                                                   Page 25

When you have more than 4 ports, a 2 digit hexadecimal value is

required.  The following table represents the left-hand digit for

ports 4,5, and 6.

MASK VALUE   PORT NUMBER      CONDITION

6   5   4      1=ON, 0=OFF

0         0   0   0      4,5,6 OFF

1         0   0   1      4 - ON

2         0   1   0      5 - ON

3         0   1   1      4,5 - ON

4         1   0   0      6 - ON

5         1   0   1      4,6 - ON

6         1   1   0      5,6 - ON

7         1   1   1      4,5,6 - ON

For ALL ports to be turned on, the MASK value would be 7F, and is

written as $7F.

For Ports 0 and 1 on, the MASK value would be 3, written as $3.

For Ports 2 and 4 on, the MASK value would be 14, written as $14.

For Ports 3 and 5 on, the MASK value would be 28, written as $28.

MSYS 1.16                                                   Page 26

DRSI CARDS

Minimum things that must be done to use DRSI cards:

Put HAPNDRSI.SYS in root (C:\) directory

Put HAPNKISS.COM where it can be found to be executed.

Put DEVICE=HAPNDRSI.SYS IRQ=7 (or whatever interrupt you have the

board set up for) in CONFIG.SYS.

Reboot system.

Put:

PORT 1 AT $0 INT 7 SPEED 1200 NAME *DRSI-0

PORT 2 AT $1 INT 7 SPEED 1200 NAME *DRSI-1

in MSYS.OPT and run MUTIL function 9.

Notes: Change the port numbers to what is appropriate to your system

.  The ports on the

DRSI card are referenced by the numbers after the AT $ .  The INT 7

is taken as comments only (but INT something is required).

SPEED  of the DRSI ports is really set on the DEVICE=HAPNDRSI.SYS

statement in CONFIG.SYS .  SPEED value given on port statement is

more or less only comments.

NAME must begin with *, can be something like *145.01 if you prefer .

Before you start MSYS, you must run HAPNKISS.

That should get you started.  For other configurations and options,

read the appropriate documentation.

MSYS 1.16                                                   Page 27

MSYS COMMAND SUMMARY

The following is a SUMMARY of the commands available at the LOCAL

keyboard for MSYS.  The commands may be abbreviated to the capital

letters.  This list does NOT include the BBS commands that are

available only from the BBS.  Most "LOCAL" commands may be executed

from the BBS by prefixing them with a "/".  Please refer to the MSYS

SYSOP section for more detailed explanation of the commands.

*comment  Enables comment line from user to SYSOP

#msgs/use Displays total number of active msgs and known user on BBS

@home     Designates @ home bbs option

ABortfwd  Aborts forwarding as soon as possible

ADDUser   Allows sysop to add a user to bbs

APpend    Used to append a file to a message and other general uses

ARChive   Puts given @BBS in specified sub directory

ARP       Lists the known callsign/IP address

ARP add   Adds digipeaters to TCP/IP stations

ARP del   Removes TCP/IP stations from arp list

ATtended  Turn HF forwarding on and off

AUTODest  Add new DESts for known neighbors

AUTOHold  Hold messages that contain own call in R: lines

AUTOKill  Kill all messages after forwarding

AUTONode  Add new neighbors heard

AX25Backoff    Provides exponential delay to RNR messages.

AX25L2V2  Turns on Version 2 of the AX.25 protocol BY PORT #

BADuser   Enables the bad user list

BBs       Enter the BBS as a local user

BBSNode   If on and the node is enabled, the bbs will appear as a

node to the network

BBSTimeout     Number of seconds of inactivity on bbs to disconnect

user

BDigi     Specifies digipeaters for beacons

BElloff   Turns bells on or

BID       Manipulates bids in the BIDLIST.DAT file.

BINterval Nodes Broadcast interval

BMaxcon   Maximum BBS connects

BOOT      Reboots computer. (Remote sysop only)

BPorts    Used to indicate BBS ports only

BText     Specifies one line of beacon text

BUDCalls  Set or display the list of calls with SSID to be monitored

MSYS 1.16                                                   Page 28

C#        Attempt to connect to station via digi list given on port #

CHAnstat  Displays status of a logical channel whose number is given

CHEck     Number of 1 second intervals of no activity to disconnect

CL B #    Change border color

CLrscrn   Clear window number given or * for clear all windows

CMdwindow Specifies the window # to be used for commands (CM 4)

COPy sourcepathname destpathname   (include drive: if needed)

CONference Used to enable/disable conference function in BBS

CTS       Allows specification for each port if hardware hand shaking

should be used

CRetries  Sets number of retries for forward connect attempt

Cscript   Another form of the Connect command:

DEADletter     Used as default for unknown @BBS

DEBug          Used to display or modify memory

DELEfile       Specifies file name to delete

DELUDays       Specifies the number of days after which a user that

has not connected will automatically be deleted from

the system.

DELUSer        Deletes a user from the user list

DESt ADD, REP, DEL   Adds replaces or deletes destinations in

nodelist.

DFree          Displays amount of disk space free

DIGipeats      Displays digipeater/gateway list

DIRectory      Displays directory for path specified Example:

DIR mail\*.*

DISconnec      Forces a disconnect on logical channel specified

DOcmdfile      To execute a command file

DPorts         Allows sysop to selectively disable digipeater

DRoute         Callsign port# [digi1...digi8]

DTimedate      Displays current date and time

DUmpbbs        Disconnect any bbs that has connected and uses a

command other than S or F.

EDit      Screen editor for files

EFile     Edits the specified file

EMsghdr   Edits the header of the specified message number in BBS

EUser     Edits specified user

MSYS 1.16                                                   Page 29

F2 time   Number of seconds to continue displaying monitored packets

after F2 (function key) was last pressed.

FBBC      Sets the FBB Compatibility mode

FBBG      Sets the FBB forwarding group size

FBBStimes Allows specification of hours bbs is full function

FCall     Finds the logical channel(s) in use by given callsign

FFile     (find file) FF *.DAT C:\

FIle      Converts specified message number into file name given.

FMsg      Searches messages in MAIL.BAK for given pattern string.

FOrward   Initiates forwarding immediately

FPactor   Initiates PACTOR forwarding

FRAck     Displays or sets time in seconds to retransmit a packet

FREE      Deallocates the logical channel specified

FSize     Displays the file size for the pathname specified

FTIme     Forward time

FTPgm     File Transfer Protocol

FUlldup   Used on the specified ports if set to ON.

GEtmsgotd Reads and displays Message of the day file

GMtime    Sets GMT offset hours from local time for BBS use

GRep      Will search the given file for the specified pattern

HApnstat  Displays HAPN/DRSI statistics.

HCAll     Allows you to properly specify your bbs hierarchical

callsign.

HCLean    This command automatically deletes specified messages.

HEard     The J heard list recording is turned off if # > Dispatches

Help      Lists the names of the commands

HOLdlevel Parameter is a number which if the sum of word weights

matched in message exceeds the value the message is held.

HOStname  Hosts id name TCP/IP

HReplace  Selects adding to end of existing hierarchical @BBS field

or replace.

ID *      Forces transmission of an ID immediately (also mail for

list)

IGnore    Calls in budcalls list will not be monitored

IMport    Imports messages from given filename.

INitpr    Allows you to specify if printer should be initialized

before each file is printed.

IPNCheck  Inactivity timeout in seconds.

IPNFrack  Retry timer in seconds.

IPNRetry  Retries, default 5.

IPTtl     TCP/IP Time to live

MSYS 1.16                                                   Page 30

JBbs      Lists (P)BBSs heard and their paths

JDigipeat Lists Digipeaters heard and their paths

JGateways Lists Gateways heard and their paths

JHeard    Lists stations heard and those connected to BBS

JKanodes  Lists KA Nodes heard and their paths

JMsys     List other MSYS systems heard

JNetrom   Lists NET/ROM nodes heard and their paths

JTcp/ip   Lists TCP/IP stations and their addresses

KEyboard  Assign keyboard to channel number specified

KILLproc  Abort specified process number (Can be hazardous!)

KPorts    Allows sysop to selectively disable k NODe

LCount    Counts number of lines, characters, and words in file.

LOCKkbd   Locks the keyboard.  OR unLOCKs keyboard if locked.

LOGclose  This command closes the log file, renames it to form of

yymmdd.LOG and then opens a new log file.

LUsers    List known user calls that have connected to BBS

LU xxx    Selects: BADusers, BBSystems, EXPert, LIMited, NEW, LOCked,

RESet, XFAst_set

NOTE: Use only the first 3 letters of the item from the

list for xxx!

LZhuf     Enables Huffman coding on PACTOR connects.

MAKEPrivate    Sets/resets making messages private by default

MAStersrv      Specifies the master White Pages server (default is

AD8I)

MAXBullage     Used to discard bulletins older than a specified

amount

MAXFOrward     Maximum # bytes to forward on specific port

MAxFrames      Display or change number of frames sent before ACK

rcvd

MAXRead        Specifies maximum size message that can be read or

file that can be downloaded

MCForward      Ports where forwarding is counted as bbs connects.

MCOn           Number of connects allowed to sysop keyboard

MDir           Creates given directory

MEMory         Display amount of memory free

MERge          Reads MSYSFWD.DEF and copies it to MSYS.FWD

MEXpert        If on, display the message of the day

MHclear        Clears various J heard lists

MIDchar        Message ID indicator (max 4 chars)

MINmem         Allows specification of the minimum free memory

MISsing        Reports number of times forwarding aborted due to

missing text file

MONBframe      Enable/disable monitoring of Beacon (UI) frames

MONCC          Enables/disables monitoring of TCP/IP packets

MONCF          Enables/disables monitoring of NetRom packets

MONIFrame      Enable/disable monitoring of Information frames

MONITor        Enable/disable monitoring of incoming frames

MSYS 1.16                                                   Page 31

MONOutgoing    Enable/disable display of outgoing frames

MONPorts       Select which ports to monitor [MASK]

MONSFrame      Enable/disable monitoring of Supervisory frames

MONUFrame      Enable/disable monitoring of Un-numbered frames

MONxx          Enable/disable monitoring of PIDs on screen F2

monitoring

MSgtrailer     Enable/disable message trailer on messages read from

bbs.

MYAlias        Display or change bbs alias callsign

MYBbs          Display or change bbs primary callsign

MYCall         Display or change Call/Answer callsign

MYIpaddr       Sets IP address that the system will respond to.

MYKnode        Display or change K NODe callsign

MYTcpcall      Callsign used when sending arp and TCP/IP frames

NBdcast        Enables Node Broadcasts on a per port basis

NDwindow #     Net default window size (#=frames 1-7)

NETBbs         Enable BBS cmd in net NODe

NETCon         Enable connect cmd in net NODe

NETFbbstime    Allows specification of hours NET is full function

NETFBBC #      NET/ROM FBB Compatibility

0=No FBB Compatibility (Default)

1=FBB Non-Compressed Compatibility

2=FBB Compression Compatibility

NETFBBG        NetRom FBB forwarding group size (0-5)

NETMaxfwd      Maximum message size to forward through network

NETNode        Enable the Net Node

NETOnly        If on, only BBSs can connect to bbs via network

NETRead        Specifies maximum size message that can be read or

file that can be downloaded through the Netnode

NETTalk        Enable Talk cmd in net NODe

NETXBatchsize  Sets Max batch size for RLI compression on network

NETXOption     If on, enables RLI compressed forwarding on network

NETYapp        If on, allows YAPP transfers via network

NEWUser #      Allows you to set the initial user flags

NInfo          Gives net NODe status

NNode          Takes you into the NODe as if you were connected to it

NOBEacons      If a given port is set to 1 then there will be no ID,

mail for, or BText beacons

NOBUlls        When on, no bulletins will be forwarded during normal

forwarding.

NODe SEnd, SAve, REStore, ADD, REP, DEL

NODETimeout #     Number of seconds of inactivity on network node

NPorts            Ports to which nodes broadcast

NRCall callsign   Sets net NODe callsign

NRId mnemonic     Sets net NODe mnemonic identifier

NTtl              Network time to live (max hops)

OCInit            Initial obsolescence count

OCMin             Minimum obsolescence to broadcast

MSYS 1.16                                                   Page 32

PAClen    Specifies maximum number of bytes that are put in a packet

PATh      Display information about given user callsign

PC        Enables/disables callsign serverPForward Display

information about given BBS callsign

PIng      Determines Round Trip Time to destination

PMessage  Prints specified message

POrts     Display information about ports

PPersist   "Agressiveness" value (255 to 0)

PQuality  Quality for nodes heard on port

PRInt     Prints a file (on printer)

PROcesses Display status of processes

PRTmsgs   Enables/disables printing of messages

PScan     Enables Pactor Scan

PWindow   Sets window to be used by a process

Qmsg      Copies msg into editor for reply.

QUIT      Terminates execution of MSYS gracefully (more or less)

Also done with Ctrl F4

RBid      Adds received BID/MID to R: Line

RDir      Removes the given directory

REadbackup     Allows you to read a message from the /MAIL.BAK

directory

REMove DESt callsign     Deletes DESt through all neighbors

REName    Oldpathname newpathname (must both be on same drive)

REPlaces  Replaces incoming @BBS calls

RESPonstime    Number of seconds to wait before sending out a

response

RESTore   Restores a message from the mail.bak directory

RETries   Displays or sets number of retries before disconnecting

REQDIR    Enables/Disables Request directory server

REQFIL    Enables/Disables Request File server

REQMaxfile     Used to set a limit on the size of files that may be

requested from the REQFIL server

REQQTH    Enables/Disables Request QTH server

RUn       Allows you a DOS program with many limitations

SCreensave     Enables/Disables screen saver

SEnd      Sends a line of text to specified channel: SE 1 <text>

SHUtdown  If on, advises users of impending bbs shutdown

SLottime  Value (0 to 255)

SMTP      Simple Mail Transfer Protocol

SNow      Enables/disables changing screen only during retrace

SOrt      Will sort up to 200 eighty byte records

STatus    Displays some status information

SUspendtrace   Number of free bytes in input buffers below which

monitoring is turned off

MSYS 1.16                                                   Page 33

TABs      Enables/Disables tabs in the bbs

TACk      Transport ACK delay (RESPonsetime)

TBdelay   Transport busy delay

TElnet    Used to establish keyboard to keyboard connection

TERminal  A command which permits direct keyboard access to port

TFrames   Send test frames

TImezone  Allows entry of your time zone

TNatime   Transport no activity timeout time

TOdo      Causes the MSYSTODO.DAT file to be scanned immediately

TPorts    Enables/disables ports for TCP/IP

Trace25   Enables/disables trace of AX.25 state changes

TraceIP   Enables/disables trace of TCP/IP state changes

TREtries  Transport retries

TRUncate  filename Adds CR/LF to files as needed

TS (hhmmss)    TimeSet command - sets DOS time to given value (must

be exactly 6 digits)

TTimeout  Transport timeout (Net FRACK)

TXDelay   Transmit Delay time value (0 to 255)

TXTail    Transmit Tail value (0 to 255)

TYpe      Allows you to type a file

Users <file_name> Writes out all users to file <file_name>

VAlidmsgtype   Permits control over second character in message type

VErifysysop Enables/Disables Sysop verification when connected to BBS

VIdeo     Displays or changes video monitor in use

WAtch     Toggles watching of specified channel number

WBackgrnd Sets background color for specified window

WColors   Gives list of colors and their numbers

WForegrnd Sets foreground color for specified window

WHO       Displays current users/messages

WHY       Displays why message was held

Window    Displays current window definitions

WPAge     Sets page number to display specified window

WPEvery   Specifies the number of days between sending out WP

information

WQupdate  Worst quality DESt route to record

WRecord   Window record which sends all output from specified window

to a disk file.  Useful for debugging

WSize     Specifies coordinates for specified window

W?        Displays window number in upper left of each window

XBatchsize     Sets max batch size in bytes for RLI compression

XOption   If on, enables RLI compressed forwarding on port

Yapp      If on, enables YAPP transfers on port

MSYS 1.16                                                   Page 34

<Esc>     Cancel partially typed command

Function key usage:

F1 through F4   Select page to display (0 to 3 respectively)

F6        Causes the following characters to the next return to be

treated as a command even if keyboard currently connected

to some other process (such as local bbs, etc.).

F8 key    Used to blank the screen.

Alt-F1 Alt-F2 Alt-F3 keys work like DOS F1 F2 and F3 commands to

retrieve/edit last sysop command typed.

Ctrl/F2   Suspends activity on the monitor screen (F2).

Ctrl/F4   Terminates MSYS as if QUIT command was used

Ctrl/F5   Same as the PRocess command

Ctrl/F6   Assigns keyboard to command processor

MOST COMMANDS GIVE INFORMATION ON THEIR USAGE IF

YOU TYPE THE COMMAND NAME FOLLOWED BY A QUESTION

MARK.

Example: WS ?

MSYS 1.16                                                   Page 35

MSYS SYSOP Commands

*comment

Not really a command as such, * may be used at the beginning of

a line making anything else you type on the line a comment.

Useful in MSYS.DO to temporarily remove commands.

#msgs/use

Displays total number of active msgs and known users on BBS .

The number of messages that can be on the bbs is set by the

Mutil Function 1 program.  The maximum number of users is 500.

@home (ON/off)

If On, causes @bbs field of messages for registered users of the

system to be replaced with the home BBS they gave with the NH

command (the way it has been done in the past) . If Off, the

@BBS field is not modified.

ABortfwd

Aborts forwarding as soon as possible (not always immediately).

You may want to DISC the channel forwarding is using

(particularly if its trying to connect) to speed things up.

ADDUser callsign

Adds a user callsign to the user file without that station

having to connect to the BBS . This can be useful in conjunction

with the automatic @BBS generation . After the new user has been

added you automatically will EUsers for the callsign at which

time you can fill in the desired fields.

APpend Sourcefile Destfile

This command causes the source file to be appended to the

destination file.  This command may be used to append a file to

a message file, among other more general uses.

ARChive @BBS filename

This command will concatenate all current msgs with @BBS given

to FILES/filename . No compression is done . Example: ARC RLIBBS

RLINOTES/MSGS   (This assumes you have a RLINOTES subdirectory

in the FILES directory)

ARP

Lists known callsign/IP address correspondence used for TCP/IP

connects.  ARP stands for Address Resolution Protocol.

ARP CLEAR

Erase all of the ARP entries.

MSYS 1.16                                                   Page 36

ARP ADD callsign port# ip-address[/#] [digi1 [digi2 ...[digi8]]]

Allows you to manually add (or put it in MSYS.DO) a station (and

up to eight digipeaters) to the ARP list.  Periods are optional

in the IP address.

Examples:

arp add WA8BXN 0 44 70 4 8 NO8M

arp add N8HSP 2 44.70.4.10 WB8CQR-5 WB8APD

You can put a / and a number after the IP address . For example,

you can say:

arp add W3AAA 1 44 80 1 1/24 K8AAA

This says that any IP frames going to an IP address with 44 80 1

in the left 24 bits should go to W3AAA (through K8AAA as a

digi).

Thus the number after the / is the number of significant bits to

keep in comparisons.

ARP DELete callsign

Removes a single entry from the ARP table.

Example: ARP DEL K8EIW

ARP REP callsign port# ip-addr[/#] [digi1 [digi2 ... [digi8]]]

Replaces existing arp entry for given callsign with new

information specified.

ATtended [ON/off]

When set to ON all forwarding is allowed.  If off, ports with

radio speed of 300 will forward only those messages in which the

from call is the idcall (messages that you originate) or the to

station in the message is the station you would directly connect

to for forwarding.  HF reverse forwarding is also inhibited if

ATtended is OFF.  Note that the regulations appear to allow you

to receive messages unattended .  So if you don't have a STA for

HF turn AT OFF when you aren't there and remember to turn it ON

when you are.  To be on the safe side, put ATtended OFF in your

MSYS.DO file should the system reboot when you aren't present.

AUTODESt (on/OFF)

When on and network node is enabled allows automatic updating of

network node destination tables when node broadcasts are heard

from neighbor nodes.

AUTOHold (ON/off)

This command specifies what should be done with messages that

contain in them R: lines with your Hierarchical BBS callsign

(see HCA command).  This normally indicates that the message has

passed through your system before and probably is in a routing

loop.  Currently such messages are held .  AUTO- Hold ON means

to do this.  If AUTOHold is OFF the messages will not be held.

MSYS 1.16                                                   Page 37

AUTOKill [on/OFF]

When on, non-bulletin messages are automatically killed after

they have been forwarded.

AUTONode [on/OFF]

If on and network node is enabled, node broadcasts heard from

new neighbor nodes are automatically added to the list of

neighbor nodes.

AX25Backoff [MASK]

A 1 bit in a given port position in the mask causes the FRACK

value for the port to be multiplied by the retry number to

determine how long to wait before retransmitting packets that

haven't been acked.

AX25L2v2 [port#] [on/OFF]

AX.25 version 2 protocol: Version 2 can be enabled for all ports

or specified ports.  Version 1, the default, may be better for

HF operation.

BADuser [on/OFF]

When ON, the calls in the BUDCall list will be totally ignored

by the system.  Such stations will not be able  to connect to

the system in any way (AX.25 or TCP/IP) and will not be

digipeated.  Do NOT use SSIDs in  this list so that the listed

calls can be used to match any incoming SSID the station may

use.  Try to avoid using this facility unless really necessary

as it does add some overhead in the processing of every packet

received.  Hopefully stations that misbehave will realize that

they can be turned off and not persist in their activity.

The BADuser command is more inclusive than the Bad User Bit

which can be set for a given user. Setting the bad user bit only

prevents that user from using the BBS.  There is relatively

little overhead in supporting the bad user bit since it is only

checked when connects attempts to the BBS are made.

Putting calls in the BUDCall list and turning on BADuser has a

lot of overhead since every incoming packet has to be compared

with each call in the BUDCall list.

BBs [window#]

Enter the BBS as a local user this command gets you into the BBS

as if you had connected to the BBS, with a few minor

differences.  Of course while you are using the BBS other

stations can still connect as usual. When in the BBS you use the

BBS command set (A,B,C,D,G,H,I,J,K,L,M,N,P,R,S,U,V,W,X,Y,?,*).

If you use the L command you will see all messages including the

private ones.  Likewise you can Read and Kill all private

messages.

MSYS 1.16                                                   Page 38

You can include the contents of a file in a message you are

sending by typing:

\+\ filename

at the point you wish to include the file. If you want to

include the contents of a message file from the MAIL.BAK

directory, type:

\+\#msgnumber

at the point you want to include the file. Example:

\+\#1538

You can specify a window number as an argument to the BB

command.  BB 0 will use the top half of the F1 screen for

example.  If you were in the BBS locally and leave it for some

reason with it still active and use the BB command you will

return to your previous session rather than starting a new one.

BBSNode [ON/off]

If on and the node is enabled, the BBS will appear as a node to

the network (using the alias as the identifier and the BBS call

as the call).  This way connects may be made to the BBS directly

from the network.

BBSTimeout #

Number of seconds of inactivity on BBS to disconnect user,

default 300 seconds.  Note that when version 2 of the AX.25

protocol is enabled, no activity for CHEck time does not causes

a disconnect.

BDigi [digi1 [digi2 ... [digi8]]]

Lets you specify up to 8 digipeaters for beacons (id, mail,

btext). The list of digipeaters can be different for each port.

If only a port number is given, the digipeater list for that

port is cleared.

Examples:

bd 4 digi1 digi2  sets digi1 and digi2 as digipeaters for

port 4 only

bd digia digib    sets digia and digib for all ports

bd 5              clears digipeaters for port 5

BElloff (ON/off)

On   All bells are turned off.

OFF  All bells are sounded.

-1   Keyboard connects sound but not other connects.

BID

Manipulates bids in the BIDLIST.DAT file.  The following

operands may be specified:

+ bidstring    Adds given bidstring to bid file

- bidstring    Deletes given bidstring

= bidstring    Tells if given bidstring is present

#                   Tells number of active bids/mids

MSYS 1.16                                                   Page 39

BINterval number

Network node broadcast of known destination nodes is made every

number seconds.

BMaxcon [port#] #

This command limits the maximum number of connects to the BBS on

each port.  BBS connects are exempted from the limit but count

for other users.  Normal users get a BUSY response when over

limit.  BM 1 allows 1 user  per port. If a port is marked BBS

ONLY (see BPorts) the BMax value  specified limits the number of

BBS connects.  For non-BBS only ports, the BMax value does not

effect number of BBS connects. See also the MCF command.

BOOT

Boots computer. You should have an autoexec.bat file set up that

runs MUTIL 4 and then MSYS so that after you reboot the computer

MSYS will be restarted. Note that this is an abrupt termination

of MSYS. Files are not closed for example.  The correct reply to

the question asked by the boot command is Yes (CASE IS

IMPORTANT).

BPorts [MASK]

This command is used to indicate which ports are BBS ONLY

connects.  A One in the corresponding bit  position makes a port

BBS ONLY.  For example, BP 5 would only allow BBS connects on

ports 0 and 2.   The default is no BBS-only ports.  If you do

use this command to make BBS only ports, you better be sure to

mark all the BBS stations that you want to forward to you on

BBS-only ports with user flag of 2!

BText [port#] text

Lets you specify 1 line of up to 80 characters of your own

beacon text.  Can be different for each port. Use with great

restraint!

MSYS 1.16                                                   Page 40

BUDCalls [callsign1 [callsign2 ... [callsign10]]]

Set or display the list of calls with SSID to be monitored or

ignored. This command is used in conjunction with the  BUDLIST

or BADuser commands.  As operands you give a list of callsigns,

with SSIDs, that are to be monitored or ignored.  The calls

should be separated by blanks.  To list the current setting of

BUDCalls, type the command with no operands.

Examples:

budc w1xx w2xx-1 w2xx

budcalls K9XXX

budc

Note: there can be up to 10 calls.

BUDList [on/OFF]

To enable only those packets with calls in the BUDCalls list to

be monitored use the command BUDL ON.   To monitor packets for

all calls, use the command BUDL off.  OFF is the default

setting.

CHAnstat channel#

Displays status of a logical channel whose number is given.

This command requires an active channel  number as a parameter.

Information about the channel will be displayed in response to

this command.  The  information includes the Port number used,

the call of the station connected to and a status byte in hex.

CHEck [number]

Number of 1 second intervals of no activity to disconnect if

Version 1 of the AX.25 protocol is in use, or to send an RR Poll

if Version 2 is enables (see AX25L command). To see what value

is currently set, give the command with no parameters.

CL B number

Allows you to set the border color (used on all screens).  Use

the WC command to see the color list.   Default color is black

for the border.  Any of the 16 color numbers (0-15) may be used.

CLrscrn [number/*]

Clear window number given or use * to clear all windows. If no

parameter is given, then the command processor window is

cleared.

MSYS 1.16                                                   Page 41

CMdwindow number

This command is used to change which window is used by the

command processor.  The default command  window is number 0

which by default is the upper half of display page 0.  This

command is useful when you  want a larger window (to type a file

for example) to be used by some command.  The new window is

cleared  automatically by this command.  You will then get the

cmd: prompt.  (page 0 =F1 page 1 = F2 etc.) A good windowk to

use is 4 (CM 4) which is the one you see when you press F4.

COPy

Sourcepathname Destpathname (include drive: if needed) The copy

command copies the specified files and will return an error

message if it isn't able to copy all the records.  Directories

and wildcards can be used.

CRetries number

Sets number of retries that there will be for forward connect

attempt. This value should probably be less than RETries.

C script-filename

In addition to the usual C# form (like C0) there is now a

connect command in which you do not specify a port number with

the C, nor do you give the call of the station to which you want

to connect, but rather give the name of a file that contains a

connect script to reach that station.  The files must be in the

/FWD directory and can have any name.  The connect script is the

same format used in the forward file except that the first line

begins with C rather the other letters used in the forward file.

You can thus use the command C HOME in which case in the FWD

directory you would have a file called HOME.  An example of the

contents of this file is

c0 wa8bxn

@olvgn

#120

+to

.c akron

#180

+to

.c cle220

#300

+to

MSYS 1.16                                                   Page 42

This example can be interpreted as follows.  In the first line

the C means this is a connect script, port 0 is to be used for

the initial connect, and the ultimate destination is WA8BXN.

The second line says to make a connect to OLVGN (which is a

network node).  The following lines are commands sent to the

node, the # lines are times to wait for responses, and the +to

lines indicate the responses to look for that indicate success.

See the descriptions for forwarding scripts for more information

on these various options.  Another simple example of what you

could put in a connect script file is

C0 WA8BXN

@WA8BXN V TOMSYS

That's the entire file; it is a simple way to put the digipeater

(TOMSYS) needed to reach the destination station (WA8BXN).

CTS [port#] [on/OFF]

Allows specification for each port if hardware handshaking

should be used with the TNC or Modem.  If on, MSYS will suspend

sending data to the TNC or Modem when Clear to Send is not true.

This will be quite useful with AEA TNCs which have problems when

MSYS sends packets too rapidly to them. Also useful with most

Modems.  Make sure you have the proper wires in the cable

between the computer and TNC or Modem, and that you have your

TNC or Modem configured properly if you try to use this option.

C# callsign [Via digi1 [digi2  ... [digi8]]]

This command is used to connect to another station.  The letter

C must be followed by a digit indicating the port number to be

used for the connection.  This is followed by a space.  Next

comes an optional window number to be used for the conversation.

Next comes the callsign of the station you want to connect to,

with SSID if needed.  Following this can come Via and a list of

digipeaters to use, separated by commas.

Examples:

c1 w1xxx

c0 w2xxx v k2xxx

c2 k3xxx-2 v n4xxx-3,a5xxx

MSYS 1.16                                                   Page 43

When you issue this command the screen to be used for the

conversation will be cleared and you will see a line "Enter

converse for channel # port #".  When connection is made you

will get a message "*** Connection established to callsign".  If

the attempt retries out (determined by the RETries value), you

will get the message "Quit converse for channel # because no

connect!".  In this case the default CALL process window may

show "*** callsign busy" if this is why the connection could not

be made.  Pressing any key before getting one of these messages

will abort the connect attempt and return you to the current

command processor window.  When you are connected to a station,

you type lines to be sent to the other station and will see

lines typed by the other station.  Once you begin typing a line

it will not be interrupted by lines from the other station until

you press return.  To terminate the connection, type ^C, the

letter d (^Cd) and press return.

If you want to save what you are getting on disk, press the \

key three times and then press return.  You will get the message

"*** Capture file opened ***".  All lines you receive will be

displayed as usual as well as being written to the file

CAPTURE.DAT.  When you turn capture on with the \\\ at the

beginning of a line, you can also specify a filename to be used

in place of the default CAPTURE.DAT.  For example you can type

\\\ junk.abc to start capturing your conversation into the file

JUNK.ABC.  When you have already turned capture on during a

connection, if you type something like \\\morejunk.xyz then the

current capture file will be closed and now your conversation

will be saved in the file MOREJUNK.XYZ.  Three \followed by

return will alternately suspend and resume the capturing of

lines to disk.  When you disconnect (or get disconnected) the

capture file will be closed.  Note: That the default capture

file name is always CAPTURE.DAT.  These files can be viewed by

using the TYpe command or edited with the EFile command You

could for example connect to some other BBS, read a message

found there after turning on capture and then include the

capture file in one of your own messages.  This procedure is

explained in more detail for the BBs command above.

You can also send any file on your system as if you typed it on

your keyboard to the station you are connected to by typing

\+\filename

at the beginning of a line. Example:

\+\ files\stuff

would send the file called stuff from the files subdirectory.

DEADletter (bbscall)

This command specifies a BBS call that will be used for private

messages entered on your system without @BBS and whose home bbs

is not known.  This permits you to forward the messages to a WP

server or another station that enjoys tracking down unknown

souls.

MSYS 1.16                                                   Page 44

DEBug

This command is primarily used during development of MSYS.  Its

results will change from release to release and are not further

documented.  Although it should not do any damage it will

probably be of little use to the normal Sysop.

DELEfile pathname

This command is used to delete a file.  You must give as a

parameter a single file path name with or without wildcards.

The parameter can contain a drive specification and/or directory

names for the given file.

Example: DELE mail.bin\*.*

DELUDays number

Specifies the number of days after which a user that has not

connected will automatically be deleted from the system.

Default is 100 days.  Locked users (see bit list in EUser cmd),

BBS call signs and bad users are not automatically deleted.

DELUSer callsign

This command removes a user call from the BBS user list.  The

callsign given as a parameter must match the a call seen on the

list produced by LUsers exactly (including case).  Use this

command to REMove users that haven't been active lately when you

are getting near the capacity of the list (currently 500 users).

The last time a given station connected to the BBS may be

obtained by using the P command while in the BBS.  If you give *

for the callsign (as in DELUS *) then most user entries that

contain garbage callsigns or ? for the user's name.  This option

is pretty safe to use, but if in doubt make a copy of the

MSYS.USR file if you wish.

DESt ADD [id:]destcall neighborcall neighborport obs# quality

Adds new network destination node which is routed through given

neighborcall using specified port. The obsolescence count and

quality values are stored as well.

DESt REP [id:]destcall neighborcall neighborport obs# quality

Changes obs# and quality for specified destination node

DESt DEL [id:]destcall neighborcall neighborport

Deletes route to specified destination node through given

neighbor.  When all routes have been deleted, the destination

itself is also removed from the network node tables.  See also

the REMove command.

MSYS 1.16                                                   Page 45

DFree [driveletter:]

Displays the number of bytes free on a disk. If no operands are

given the space is for the default disk (normally C:).  If a

letter is given as a parameter, the number of free bytes will be

given for that drive.  Be sure the drive is ready before using

this command. If its not and you get the DOS error message, type

I for ignore.

DIGipeaters

This command displays the list of digipeater definitions and

related information. The output of this command is the same as

using the ID command in the BBS.

DIRectory [path]

Displays directory for path specified Example DIR mail\*.* This

command displays a sorted directory listing.  Only the names of

the files (or directories) are given, not the sizes.  To get the

size of a file, use the FSize command described below.  Names of

directories are enclosed in <>.  If no argument is given, you

will get a directory for the current default directory (normally

C:\MSYS).  To list some other directory, give the path to that

directory and a file name of *.*.  Other file

specifications can be given for a selective directory list.  If

you include a drive specification in the pathname, be sure the

drive is ready before using this command.  Examples:

dir mail\*.*

dir a:*.*

dir files\*.c

DISconnect channel#

This command can be used to force a disconnect on the specified

active channel. If the channel you specify was in the process of

disconnecting when you use this command, then the channel will

be immediately disconnected. Otherwise the channel will begin

disconnecting when you use this command. Use it a second time

for that channel if you want to speed up the disconnect.

The disconnect command can be useful when you want to shut down

the system. Another good use is force a disconnect from the bbs

you are forwarding to after you have used the ABort command to

abort forwarding.  The disconnect command also will properly

terminate a modem or a pactor connection.

DOcmdfile filename

This command is used to execute the sysop commands found in the

specified file.  A command file will contain the same things you

would normally type as commands.  To execute a command file,

type DO followed by the command file name. Example:  DO cmds.dat

MSYS 1.16                                                   Page 46

DPorts [MASK)

A one in the mask for a given port permits digipeat on that

port.

DRoute callsign port# [digi1 ... digi8]

DRoute DELete

The first form of this command allows you to specify the TCP/IP

station to which IP frames are routed by default (Default

Route).  You specify as parameters the callsign of that station

(with SSID) and the port number used to reach that station.

Optionally up to 8 digipeaters can also be specified to be used

to reach that station.   The second form of this command allows

you to turn off default routing of IP frames.  If you do have a

default route specified, then all incoming IP frames that are

not otherwise handled by existing ARP table entries will be sent

to the default station.  This station will also be used for any

server functions you initiate (telnet for example) to IP

addresses that are not currently in your ARP table.  Care must

be taken in deciding if this command is appropriate to use in

your particular situation.

DTimedate

This command displays what the computer thinks the current date

and time is. The time can be changed with the TS command.  To

change the date you must exit MSYS and use the DOS date command.

DUmpbbs [ON/off]

Disconnect any BBS that has connected and uses a command other

than S or F.  A BBS is defined as a station that sends a SID

([...]) line.  This is useful in cases where the two BBB's get

out of sync and message titles/text are being interpreted as

commands.

EDit filename

Screen editor for files.  Escape gets to command mode, do help

for command list. See section on EDITVIEW later in this document

for more information.

EFile filename

This command is used to edit an ASCII file with re- cords <= 80

bytes in length.  At least one operand is required which is the

pathname to the file you want to edit.  A second operand may

optionally be given that gives the size of the work area in

lines to be allocated (default is 200).  See the separate

documentation for EDITFILE for more information on the editor.

MSYS 1.16                                                   Page 47

EMsghdr msg#

This command allows you to edit the fields of a message header.

One operand is required which is the message number to edit.

This command also automatically sets the message length to the

actual size of the associated message file.  This is useful when

you have replaced a message file with some other file as

described above.  This command is also useful to change the

status of messages to ALL (or others) that have not been marked

as being read (but actually have been) so that they don't appear

in the MAIL FOR list sent out with each ID.  If you press Esc

when editing you will skip the current and remaining fields and

see the header again.  This allows you to bypass having to press

return for all the fields you don't want to change.

If you place a string of characters that begins with a period

after the message number, it will be appended to the existing

@BBS field of the given message.  Thus if the @BBS of message

123 was WA8BXN then:

EM 123 .NTSOH

would cause the @BBS field of message 123 to become

WA8BXN.NTSOH.

If you type a blank for the BID field the Bid is cleared

entirely (you still need to get rid of the $ in the type).

MSYS 1.16                                                   Page 48

EUser callsign

Edits user information. Also displays the meanings of the

possible user flags bits. The EUser command will give a display

similar to the following:

cmd: eu k8eiw

K8EIW is Don located at Cuyahoga Falls, OH  Last new message 36457

Last connected Sun Nov 21 23:29:36 1993 on port -1 Flags=$1A53

Lines=19

Postal code:44223 Home BBS: WB8BII Last WP Update: Jul 15 01:00:03

1990

Path to K8EIW is <Local Console>

User flags:

$8000 New user

$2000 No auto LC in bbs          $1000 Autoreply

$800 YAPP upload authorized      $400 User password required

$200 File upload authorized      $100 Lock user

$80 Can't use S cmds             $40 Multiline packets (XF)

$20 Need to send WP Update       $10 Remote sysop

$08 Limited function BBS         $04 Bad user

$02 BBS                          $01 Expert user

The flags add hexidecimally per character position.  For

example, if a user has both File upload and YAPP upload

privileges, the flag for that position would be "A".

To prevent WP messages from being issued on a particular call,

such as V1SIT, set the flag to $100 at a minimum.

The No Auto LC, Autoreply, Multiline Packets, and Expert User

flags can be set by users with the appropriate XC, XR, XF, or X

commands.

In the above example, the flags 1A53 mean this user has

Autoreply turned on, YAPP and File upload permitted, Multiline

packets will be sent, is a remote sysop, and is a bbs and an

expert user.

F2time number

The number of seconds to continue displaying monitored packets

after F2 (function key) was last pressed.  The default is 600

seconds (10 minutes).  Thus 10 minutes after you last press F2

monitoring will be turned off until you press F2 again.  If you

set F2 to 0 this function will be turned off. There is a fair

amount of overhead in formatting all the information displayed

on the F2 screen so on slower systems it is a good idea to allow

the display to be suspended when you aren't actively looking at

it!

MSYS 1.16                                                   Page 49

FBBCompat [port#] [0, 1 or 2]

This command determines the level of compatibility with FBB

style forwarding.  FBBC 0 means MSYS will forward non-compressed

to all stations as before.  FBBC 1 means that MSYS detects the

"F" in the SID, it will forward non-compressed, but in FBB style

groups.  FBBC 2 Means that if a station identifies itself with a

"B" and "F" in the SID, it will forward compressed in groups per

the FBB style format.  It should NEVER be specified for the

modem port due to the high probability of buffer overflows.

FBBGroupsize #  [1-5]

This command determines the maximum number of messages that may

be forwarded out in one group using FBB compliant protocol.  The

maximum group size is 5.  In FBB systems, the maximum group size

is 10K bytes, and the number of messages proposed in the group

for forwarding will be limited by this value.  MSYS does not

have this limitation.

FBBStimes [Port#] [time-range]

Allows specification of hours BBS is full function for each

port. Default is 0-23 which is all the time. The format of the

time range is the same as used on the ! lines in the forwarding

file.

FCall callsign

This command finds the channel associated with a callsign.  The

single required operand is a callsign without SSID.  All

channels in use by that callsign (with any SSID) will be

displayed giving the Port, process number and name and complete

callsign (with SSID).  This can be useful when you were in the

middle of a conversation with some station (and still connected)

but switched the keyboard to some other process (maybe the

command processor) and now want to get back to that

conversation.  Use the KEyboard command with the appropriate

process number to do so.

FFile filenamepattern [starting-path]

When you give a file name pattern and optionally a starting path

location this command will list all files that match the

pattern.  The following would find all files that have the

extension .DAT on your C: disk

FF *.DAT C:/

The output from this command gives the date, time, size and

complete path names for all files that match the search

specification.

FIle msg# filename

Converts specified message into a file with name given. This

command removes the R: lines at the beginning of the file it

creates.  If you really want to keep them, use an appropriate

rename command instead.

MSYS 1.16                                                   Page 50

FMsg pattern-string

Searches all messages in MAIL.BAK directory for given pattern

string.  For example, to find messages that refer to hurricanes

you might type:

FM hurricane

FOrward [callsign]

This command causes forwarding to begin immediately if it is not

currently going on.  You might want to use this command when you

have just entered a new message with an @BBS specification that

you want sent out now.  FO with a callsign of a single station

begins forwarding to that station.  This must match the callsign

(including SSID) found on the F/R/P line in the forward file.

FRAck number

This commands allows you to set or display the number of seconds

to wait for an acknowledgment to arrive for a packet that is

sent out.  If it doesn't arrive within the number of seconds

specified the packet will be retransmitted.  The retry count for

the channel will also be incremented.  Making this parameter to

small will congest the channel and cause unwarranted

disconnects.  The default is 6 seconds.  If in doubt, error on

the high side! (port number optional)  All IP (and IP through

netrom) frame retry timing uses a dynamic FRACK value.

FREE chan#

This command allows you to immediately release an allocated

channel.  The process using the channel will see this as a

disconnect.  This command will not be needed very often.

Disconnecting which frees the channel will automatically occur

after the CHEck interval has expired with no activity.

Sometimes unpredictable results can occur with this command.  It

is best to permit the DISC command do it's thing.

FSize filename

This command can be used to determine the size of a disk file.

One operand is required which is the pathname to the file you

want the size of.  It may include a drive specification and

directory paths in addition to the file name.  Wild cards are

not allowed.  Be sure the drive containing the file is ready

before using this command.

FTIme number

This command sets the time in minutes after the hour that

forwarding will start.  If time is negative number such as -5

then you will forward every 5 minutes.

MSYS 1.16                                                   Page 51

FTPgm host-id

Takes a host-id as an argument (it can be either an IP address

in the square brackets or a symbolic IP address).  It

establishes a connection to the FTP server of the specified

system.  FTP is the File Transfer Protocol used in TCP/IP.

FUllduplex [port#] [on/OFF]

Enables full duplex operation in the TNC on selected ports. This

may be useful for satellite operation or with full duplex

repeaters.  Examples:

FU ON     turns full duplex on for all ports

FU 2 OFF  turns full duplex off for port 2

GEtmsgotd

This command causes the Message of the day file (MSYS.OTD) to be

read and displayed.  The Message of the day is sent to all

stations connecting to the BBS.  You can create and/or change

this file using the EFile command.  Keep the contents short! See

also MEXpert command.

GMtime hours

This sets the GMT offset from the local time.  The computer can

be set for local time and, if the TImezone GMT option is set,

MSYS will insert the proper GMT in R: lines.

GRep filename search-string

Will search the given file for the specified pattern, displaying

lines that contain the given string. Case is ignored in the

search. Wild cards are NOT allowed.

HApnstat

Provides HAPN/DRSI interface statistics.

HCAll string

This command allows you to properly specify your BBS

hierarchical callsign.  This is the callsign put in the R: lines

of messages as they pass through your system.  If you don't give

this a value, your ID callsign will be used.  There is a maximum

of 49 characters for this entry.  Hint: if you run more than one

copy of MSYS for some reason and pass messages between the two

systems, make your HCAll slightly different on each.  I might

use for example WA8BXN.OH.USA on one and WA8BXN.OH.USA.NA on the

other.  This will prevent unnecessary holding (or deleting, see

OLdbids) of messages validly passed from one system to the

other.

MSYS 1.16                                                   Page 52

HCLean [number]

This command allows you to automatically delete specified

messages every day at specified time (see MSYS.HCL in index). If

no operand is given HCL shows time in minutes after 00:00

(midnight) that housecleaning happens. If a number is given as

an operand it sets the time for housecleaning in minutes after

00:00 (midnight).

HCLean NOW

Manually initiates housecleaning.

HEard number

The J heard list recording is turned off if the number of

dispatches per second in the last minute is less than the number

given.  Default value is 3100 which j heard recording.  J Heard

list recording takes quite a bit of computation for every packet

heard.  This command was added to speed things up on slower

computers.  (you might want to use a value of 10) Help  This

command lists the names of the available commands with the

required part of the command name in capital letters.

HOLdlevel number

Parameter is a number which if the sum of word weights matched

in message exceeds the value, the message is held.  Default is

0, which will hold if any words are matched.  See Automatic

Message Holding

HOStname string

Sets TCP/IP hostname.  Maximum of 49 characters.

HReplace [ON/off]

This command selects adding to end of existing hierarchical @BBS

field info from BBSTONTS.DAT file (if HRep is off) or replacing

anything past the first part with the information from the

BBSTONTS.DAT file (if HRep is ON).  Note that if some MSYS knows

how to forward from some existing part of the @BBS field, no

changes are made to it.

ID [*]

This command will send out an ID on all the ports immediately.

You might use it to see if all your radios go into transmit as

they should.  It also will send out the MAIL FOR list.  The text

of the ID is not displayed at this time, but is shown once

during initialization.  Nothing is displayed by the automatic

IDs sent out (on the F1 screen).  To see what mail is

outstanding for local users, type ID.  To really force an ID to

be sent, type:  ID *

MSYS 1.16                                                   Page 53

IGNORE [on/OFF]

When on calls in the budcalls list will be ignored.

IMport filename

Imports messages from the given file.  The file must follow the

form of the files in the mail.bak directory.

INitpr [on/OFF]

Command allows you to specify if printer should be initialized

before each file is printed.  Set it to ON if you need the

initialization.  If you need to press buttons (to select draft

or letter quality mode for example) that you don't want cleared

before each file, set it to OFF.

IPNCheck number

Inactivity timeout in seconds for TCP/IP connections, default

600.

IPNFrack number

Retry timer in seconds for TCP/IP connections, default 30.

IPNRetry number

Retries for TCP/IP connections, default 5.

IPTtl number

Sets TCP/IP time to live value. This is the number of times a

TCP/IP frame will get relayed before it gets discarded. Default

is 16.

JBbs [port#]

Lists BBSs heard and their paths for given port or on all ports

if no port number is given.

JDigipeat [port#]

Lists Digipeaters heard and their paths for given port or on all

ports if no port number is given. JD command screens out invalid

callsigns.

JGateways [port#]

Lists Gateways heard and their paths for given port or on all

ports if no port number is given.

JHeard [port#]

Lists stations heard and those connected to BBS or PBBS  This

command gives a list of the last 10 stations that were heard on

the ports or connected to the BBS or PBBS. If a port number is

given only stations heard on that port are listed. An asterisk

after a callsign indicates that the station was heard through a

digipeater.

MSYS 1.16                                                   Page 54

JKanodes [port#]

Lists KANode beacons heard and their paths for given port or on

all ports if no port number is given.

JMsys [port#]

Lists other MSYS systems' beacons heard for given port or on all

ports if no port number is given.

JNetrom [port#]

Lists NET/ROM node beacons heard and their paths for given port

or on all ports if no port number is given.

NOTE for all the J commands: These commands give the PBBS

systems, KANodes and other beacons that have been heard on the

ports.  A BBS is a system that sends out a beacon that contains

/B after what looks something like a callsign.  A KANODe is

identified by /N in its beacon to ID.  For both of these

commands the date, time, callsign and path are given.  This pair

of commands is useful to identify the PBBS that goes with a

given KANODe.  The Net nodes are also included.  The real call

for such nodes are given in [ ].  Real NetRoms get the real call

in { }.

JTcp/ip [port#]

List TCP/IP stations heard and their addresses for given port or

on all ports if no port number is given.

KEyboard chan#

The KEyboard command allows you to manually reassign the

keyboard.  As a parameter you give the channel number of the

process you want to have the keyboard.  You can get the channel

number from the lower right part of the F1 screen.  To get the

keyboard back to the command processor so you can issue this

command either press ^F6 to assign the keyboard to the command

processor or press F6 to temporarily send keyboard input to the

command process up to and including the next return key.  When

you press a key, it is placed in the input buffer of the process

to which the keyboard is logically connected.  Initially this is

the command process.  When you give certain commands (such as

BB, PB, C) the keyboard will be assigned to the new process

created for the command.  When the process is terminated, the

keyboard is automatically assigned back to the command

scheduler.

KILLproc process#

Abort specified process number (Can be hazardous!). Never use

this command to kill a permanent process.  Better yet, just

don't use this command! Process numbers can be viewed using the

PROcess command.

MSYS 1.16                                                   Page 55

KPorts [MASK]

This command allows you to selectively disable the K-node on any

ports.

A one bit for a given port number enables the K-node on that

port.

LCount filename

This command counts the number of lines, characters and words in

a given file.  Note: This is NOT the LC command found in the

bbs.

LOCKkbd

This command "locks" the keyboard so that any keys pressed are

ignored.  I put it in for two reasons.  First, I am getting RF

into my keyboard from HF radios and it generates random

characters.  Second, my 3 year old son sometimes likes to press

the keys.  To "unlock" the keyboard, simply type "lock" again

(no CR needed this time).  When unlocking the keyboard, "lock"

must be all LOWER case.

LOGclose

This command closes the log file, renames it to form of

yymmdd.LOG and then opens a new log file.  Caution:  Do NOT use

more than once per day!

LUsers

Use this command to see who has connected and more importantly

how many users are in the user file since it can hold only 500

users currently.  LU takes an optional argument to list users

flagged as BAD, EXPert, LIMited, LOCked, XFAst, NEW and BBS

system.  Example: LU EXP

LU RESET clears all new user flags.

MAKEPrivate [ON/OFF]

Sets/resets making personal messages private by default.

MAStersrv hierarchical-address

Specifies the master WP server (default is AD8I.OH.USA.NA and

probably shouldn't be changed).  If MAS is set to NONE, no WP

messages will be sent.

MAXBullage number

This command may be used to discard bulletins older than a

specified amount as soon as they are received.  Default age is

32000.  A reasonable value might be 21 days.  Bulletin age is

determined by the date on the last R: line found in the

bulletin.  If MAXBullage is set to a negative number, bulletins

older than the number given (in days) will be held rather than

discarded.

MSYS 1.16                                                   Page 56

MAXFOrward [Port#] # bytes

This command permits you to set the largest message size that

will be forwarded on a port.  This may be useful for HF

forwarding.

MAXFrames number

Display or change number of frames (1 to 7) sent before waiting

for and ACK (RR) to be received. A value of 4 is generally good

for VHF ports. Use 1 or 2 on HF ports. For very good vhf paths

and RS-232 wire links, 7 may be a good value.

MAXRead [port#] number

Specifies maximum size message that can be read or file that can

be downloaded. If no port number is specified value applies to

all ports.

MCForward [MASK]

Allows you to control Multiple Connections during Forwarding.

The operand for this command is a port bit mask.  Default is all

1's in the mask which allows forwarding to go on as in previous

versions.  If a particular port has its bit set to 0, then

forwarding on that port is counted as a BBS connect on that

port.  Thus if you have BMaxcon set to 1 for that port and you

are forwarding on that port, a BBS connect will not be allowed

(since you are already using the port).  Also, if there are

already BMaxcon BBS connections for that port forwarding will be

bypassed for that port.  Careful use of these commands should

reduce the congestion on HF forwarding as you now have complete

control over the level of multiconnection for both BBS and

forwarding activities combined.  In the [MASK] argument you put

1's in the ports on which you want forwarding to be counted as a

BBS connect.  Using MCF along with BM (and BP if you have marked

any stations as BBSs), allows you to limit a given port to a

single BBS connect or forward at a time.  If you want this on

port 0, use the following:

MCF $FE

BM 0 1

BP $01

MCOn [number]

Specifies number of connects allowed to the SYSOP keyboard at

one time if MCon 1, only allows an incoming call to the key

board to be answered if there are no other keyboard connections.

MCOn 0 prevents all incoming keyboard requests.

MDir directoryname

Makes specified directory.

MSYS 1.16                                                   Page 57

MEMory

Displays amount of memory free.  This number may not include the

amount of memory available in the "holes" created by stations

disconnecting.

MERge

This command reads MSYSFWD.DEF and copies it to MSYS.FWD,

expanding any include files ($filename).  A new file is also

produced, MSYS.BBS that is a sorted list of all possible BBSs

that can be forwarded to (including things like ARRL, 44*,

etc.).  This command was added for 2 reasons: previously, if

there were any include files in the MSYS.FWD files they did not

get read when a PF command was used and secondly, the new file

is needed for hierarchical forwarding.  The leftmost part of the

hierarchical route only is used for forwarding and the contents

of the new file were needed for efficient processing.  So, the

bottom line is rename your existing MSYS.FWD file to MSYSFWD.DEF

and then use the MERge command.  Any future changes you make to

your forward file must be done to MSYSFWD.DEF followed by use of

the MERge command.  Its called MERge because it merges in the

include files.

MEXpert [on/OFF]

If on, the message of the day will be displayed to all non-BBS

marked stations.  If off, it will only be displayed to non-

expert, non-BBS stations.

MHclear [letter [port or B]]

Clears the various J heard lists. Letters are HBKDN or G

(Heard, BBSs, K Nodes, Digi's, NetRoms).  Examples:

MH        clears everything.

MH D      clears all Digi's heard

MH D 1    clears Digi's heard on port 1

MIDchar string

Message ID indicator (max 4 chars) to be sent in SID. If non-

blank, MIDS will be sent for private messages when forwarding to

systems that have the same string in their SID (System

IDentifier, the stuff in [ ] when a connect is made to a BBS).

MINmem number

Allows specification of the minimum free memory as reported by

the MEM for which a new connect request will be accepted.

Default is 32000.  Values less than 16000 probably will be

disastrous. A value of 32000 or more is strongly recommended,

particularly if you are using compressed forwarding.

MISsing

Reports number of times forwarding aborted due to missing text

file.

MSYS 1.16                                                   Page 58

MONBframe [ON/off]

Enable/disable monitoring of Beacon (UI) frames

MONCC [ON/off] - enables/disables monitoring of TCP/IP packets

MONCF [ON/off] - enables/disables monitoring of NetRom packets

MONIFrame [ON/off]

Enable/disable monitoring of Information frames

MONITor [ON/off]

This is the master command for monitoring packets heard.  If

this is set to OFF then there will be no monitoring.  If set to

ON then the types of packets selected will be monitored for

calls as selected by BUDCalls on the ports set by MONPorts.

MONOutgoing [ON/off]

Enable/disable display of outgoing frames

MONPorts [MASK]

This command is used to enable or disable monitoring of the

various ports on the F2 screen.  See the section on MASK on how

to set the mask for the desired ports.

MONSframe [ON/off]

Enable/disable monitoring of Supervisory frames (RR, RNR, REJ).

MONUframe [ON/off]

Enable/disable monitoring of unnumbered frames (SABM, UA, DISC,

DM)

MONxx

Specifies any hex value for PIDs to ignore on screen 2

monitoring. Example: MONCC ON  would enable monitoring of frames

with PID of CC

Note: You must specify exactly two hex digits

MSgtrailer [ON/off]

If on when a message is read on the BBS (except with RN or RE) a

line is sent at the end of the message indicating its number and

who it was from.  (Default is ON)

MSYS 1.16                                                   Page 59

MYAlias string

Display or change BBS alias callsign. Must be 1 to 6 characters,

no SSID.

MYBbs callsign

Display or change BBS primary callsign. Can have SSID.

MYCall callsign

This command displays your keyboard callsign if there is no

argument, or sets the keyboard callsign (used to connect and

respond to connects) to the call given as an argument (including

SSID).

MYIpaddr ip-addr

Takes four integers each in the range 0 to 255.  It sets the IP

address that the system will respond to.  Example: MYI 44 70 4 6

If MYIpaddress is not specified, or set to all zeros, the TCP/IP

support will be inactive.

MYKnode callsign

Display or change K-node callsign. Should have SSID.

MYTcpcall callsign

Display or change a callsign (with SSID) that will be used when

sending ARP and TCP/IP frames.  It should be the same as the

network node call in most cases.

NBdcast [port#] [ON/off]

If OFF, node broadcasts will not go out on port, but the node

will otherwise operate on the port (assuming it is enabled using

NP and other commands).

NDwindow number

Network default window size in frames. Default is 4. Note that

his has nothing to do with windows on the screen.

NETBbs (ON/off)

Enable/Disable BBS command in network node.

NETCon (ON/off)

Enable/Disable Connect command in network node.

NETFBBC number  NET/ROM FBB Compatibility Flag

0 = No FBB style forwarding (Default)

1 = Uncompressed FBB forwarding protocol

2 = Compressed FBB forwarding protocol

This option only has meaning when connected to FBB compatible

systems through the network node.

MSYS 1.16                                                   Page 60

NETFBBG number

Sets maximum number of messages included in a group when

forwarded using network  (1-5 messages per forward group)

This option only has meaning when connected to FBB compatible

systems through the network node.

NETFBBStime [hourlist]

Allows specification of hours BBS is full function when accessed

through the network.  Default is 0-23 which is all the time.

Hour list has same format as that used in the ! lines in the

forwarding file.

NETNode [ON/OFF]

Enables the Network Node

NETOnly [on/OFF]

This command is used to indicate if BBSs only or any stations

can connect to the BBS via the network.  If ON, only BBSs are

allowed to connect via the network.  If OFF, then any station

can connect to BBS via network.

NETRead number

Specifies maximum size message that can be read or file that can

be downloaded when connected to the BBS through the network.

NETTalk (ON/off)

Enable Talk command in the network node.

NEWUser number

Allows you to set the initial user flags for all new users (see

EUuser command for the bit position meanings).  Thus you can

choose to make new users limited (or I suppose for a closed

board, "bad users") until they identify themselves somehow.

NETXBatchsize

Sets the number of uncompressed bytes available for RLI

compressed forwarding via the network port.

NETYapp [on/OFF]

Enables YAPP (Yet Another Packet Protocol used for binary file

transfer between user and BBS) for connections made to the BBS

through the network.  Users must be enabled with user bit $800

to be able to upload files.

NInfo

Gives network node status display. Same output as U * command

from in the network node.

MSYS 1.16                                                   Page 61

NNode [window#]

This command takes you into the NODe as if you had connected to

it.  When in the NODe you can issue SYSOP commands if you need

to by putting \ or / in front of them as you can in the BBS.

Particularly useful are the NODe, DEStination and REMove

commands when in the NODe to modify neighbor nodes and

DEStination nodes. You can optionally specify a window number to

use (4 might be a good one).

NODETimeout number

Number of seconds of inactivity on network node or K node at

command prompt to disconnect user.  Default is 300 seconds (5

minutes).

NOBEacons [mask]

If a given port is set to 1 then there will be no ID, mail for,

or BText beacons.

NOBUlls (on/OFF)

When on, no bulletins will be forwarded during normal

forwarding.  Useful in clearing backlog of messages,

particularly during earthquakes and hurricanes!

NODe ADD [nodeid:]call port quality [digi1 [digi2 ... [digi8]]]

Adds neighbor network node using given port and quality

NODe DEL call port

Deletes neighbor nodes & its DESts

NODe REP [nodeid:]call port quality [digi1 [digi2 ... [digi8]]]

Changes quality of neighbor network node

NODe REStore

Read net NODe Database from disk. If you are using the network

node, you would probably put this command in your MSYS.DO file.

NODe SAve

Write network node Database to disk.

NODe SEnd

Force known network node destination broadcast.

NPorts [mask]

Ports to which network node broadcasts are made. Default is 0.

NRCall [callsign]

Sets net work mode callsign. It should have an SSID.  Example:

NRC WA8BXN-2

MSYS 1.16                                                   Page 62

NRId [mnemonic]

Sets net NODe mnemonic identifier for network node.  Example:

NRI 460406

NTtl [number]

Network node time to live (max hops). Default is 64.

OCInit [number]

Initial obsolescence count for network node. Default is 6.

OCMin [port#] [number]

Minimum obsolescence to broadcast a destination node in network

node broadcast.  OCM is settable for each port. Default 10

PAClen [port#] [number]

Specifies maximum number of bytes that are put in a packet

before it is sent.  Possible range of values is 1 to 255. If

packets will go through a network node on their way to their

destination 236 should be used. On HF a value of 40 is generally

good.

PATh callsign

Displays information about given user callsign (same as P on

BBS)

PC [on/OFF]

This command enables or disables the on-line Callsign Server.

PForward callsign

Displays information about given BBS callsign (same as PF on

BBS)

PIng destination

Where destination is either an IP address like [44.70.4.6.] or a

mnemonic defined in MSYSHOST.NET.  The reply comes in window 2

(where the connect and disconnect messages appear)and gives the

round trip time in seconds.

PMessage message#

This will print the given message.  If you have MAIL.BAK as a

directory then the file MAIL.BAK/MSG###.DAT will be printed.

This will look nice.  If you don't, then MAIL/MSG###.DAT will be

printed.  This will be only the message text.  This command just

makes it easier to print a message than using the PRInt command.

MSYS 1.16                                                   Page 63

POrts [port#] [ON/OFF]

Various information is given by this command if no operands are

specified.  Interesting to look at but may not be very useful.

POrt can have operand on/off.  When off a port will not respond

to the id callsign (with any SSID) and will no longer digipeat

or be usable from the KANODe.  BBS will not id unless you use

it.  Id will be very short if it has to id (no text for the UI

frame).If you call a station with the port off this will still

work because incoming packets from that station don't include a

connect packet.If you give a PO [port#] with no operand, the

port will be turned off.

The POrts command will take a single parameter (ON or OFF) in

addition to an optional port number.  With just the single

parameter, all ports are turned on or off.  Useful when you want

to shut down the system.  PO OFF allows the current users to

finish while preventing further connects.

PPersist [port#] value

Sends number given to TNC 255 to 0 as persistence value.

Basically the higher the number the more likely the TNC will

actually transmit a frame (if it has one to transmit) when it

hears the channel is clear. Better overall channel throughput

will result if all stations use a moderate value of persistence

(50 to 100). A value of 255 says to transmit as soon as the

channel is clear.

PQuality [port#] number

Default is 192. Sets quality for network nodes heard on port.

PRInt [filename]

Prints a file (on printer) The argument is the pathname to the

file to print.  Make sure the drive is ready before using this

command.  Make sure the printer is ready (and exists) before

using this command.  Printing is done to LPT1: via BIOS calls.

This command will queue up to 25 print requests.  PRInt with no

operands lists the files in the queue.

PROcesses

This command displays for each active process its number,

program running for that process, an integer parameter passed to

the process (usually a port or channel number), the least amount

of memory left in the stack for the process (labeled SPmin), and

the window used for output by the process.  Of particular

importance is SPmin if the system does bizarre things.  If this

number is less than say 100 the stack is too small.  If its a

negative number, you have problems for sure!

PRTmsgs (on/OFF)

Enables printing of all incoming messages as they are received.

MSYS 1.16                                                   Page 64

PScan [port#] |[freq]

Enables the PACTOR scanning routines

PWindow number number

If given one parameter this command displays the name and window

used by the given process number.  If two numbers are given as

parameters, the first is the process and the second number is

the window number to assign to that process.

Qmsg <msg#>

This command copies the given message file to a file called

QMSG, stripping off the R: lines, and inserting > at the

beginning of each line.  It then takes you into the editor to

edit the file QMSG.  You may now respond to the quoted message

text adding and deleting lines as desired.  Editor help is

available with Esc HELP.  When you are done editing, save the

file (Esc SAVE).  Next go into the bbs and use the REP command

to begin replying to the message.  Instead of typing your reply,

use the \+\QMSG to include the edited quoted message.  When it

has been read, type the ^Z or /EX to signal the end of your

message.  If the command is done from within the bbs by using

the form /QM, the bbs will prompt your entries for sending the

message.

QUIT

After you use QUIT command (or Ctrl/F4) the screen is cleared.

JK etc. information is saved when you do a QUIT and reloaded

when BBS is started again.

RBid [on/OFF]

When on, will take the received BID/MID ($:xxxxxx) and insert it

in the current R: line.

RDir directoryname

REMove specified directory

Readbackup number

This command allows you to read a message in the MAIL.BAK

directory (if it exists).     Example: RE 12345

REMove DESt callsign

Deletes specified destination network node through all

neighbors.

REName Oldpathname newpathname

Renames a file. Both name must be on same drive.

REPlaces

Reads BBS call/@BBS replace file and stores it in memory. Use

this command every time you modify the MSYS.REP file while MSYS

is running.

MSYS 1.16                                                   Page 65

REQDIR [on/OFF]

Enables Request directory server in the BBS.

REQFIL [on/OFF]

Enables Request File server in the BBS.

REQMaxfile number

Used to set a limit on the size of files that may be requested

from the REQFIL server and also the files that will be displayed

in REQDIR requests.  The default is 5000 which is probably too

high.

REQQTH [on/OFF]

Enables Request QTH server in the BBS.

RESPonsetime #  Where # is the number of seconds to wait before

sending out a response.  This is the T2 timer value in the AX.25

spec.  If you set RESP to 0 it will work the old way, always

sending out a response to each I frame received as soon as

possible.  With non-zero values for RESP, the responses are

held, the  latest one replacing any one being held, until no  I

frames have been received for the time given.  It then  sends

out its response.  RESPonstime Can be set for individual ports.

RESTore <arguement>  Restores a message from the MAIL.BAK dir to the

active mail directory specified by the arguement.  IT DOES NOT

maintain the old number, but assigns a new MESSAGE number.  It

WILL maintain the original BID (if any).

Possible arguements:

msg#      restores a given message number

msg# msg# restores all messges in given range of numbers

Missing   restores messages that exist in MAIL directory

but don't have headers.

NOTE: This command may now be done as a SYSOP command OR from

within the BBS.  BBS Example:      /REST 12345

RETries number

Displays or sets number of retries before disconnecting. A value

of 5 is good for VHF, 10 can be used on HF.

MSYS 1.16                                                   Page 66

RUn <programname>

Allows you a DOS program with many limitations.  For those of

you who have asked for some sort of DOS shell capability you may

recall I have said it can't be done.  But some of you said all

you needed to do was output 8 bits to some port or something

like that.  Well if you have something VERY SIMPLE and VERY

SMALL and very QUICK here is something that should work for you.

Note: You must follow all of the rules given here or very

strange things might happen for which I claim no responsibility.

If you use the RUn command and it doesn't work it is clearly you

who have made the error!  Here are the rules for programs you

might consider using with the run command (if you don't

understand these rules or aren't sure your program complies with

them, don't run it!):

1. Program must be small (I'd say under 10K).  If you get a

message that says not enough memory, the program is too

big.

2. Program must be in default MSYS directory.

3. Program must be fast.  If runs much over 1 minute, the system

will reboot.  While it is running no packets of any kind

will be transmitted.

4. Programs should not use any of the "standard" pre-opened

files.  Console I/O must be done using ROM BIOS only.  If

you open any files, you must close them.  If you use any

memory, you must free it.  If you change anything in the

computer you must put it back for the most part.

5. Any screen output will be put in a file called $POOL.  The

TYpe command may be used to view the file.

6. You should write your programs in either assembly language or

C.  Here is an example of a C program that is suitable for

framing (I mean execution using the RUn command):

#include <stdio.h>

#include <stdlib.h>

main(int argc, char *argv[])

{

int I;

char *path;

path = getenv("PATH");

for (i=0;i<argc;i++)

cprintf("argv[%d] is '%s'\n",i,argv[i]);

if (path) cprintf("Path = '%s'\n",path);

exit(1);/* You didn't expect this to do anything useful I

hope*/

}

MSYS 1.16                                                   Page 67

SCreensave number

If the number given is zero (which is the default) the screen

will not be blanked automatically.  The method used to blank the

screen will work with all display cards (the screen content is

saved and then blanks are written to the screen).  When you

press a key to unblank the screen you will always get to screen

1 (the one you get with the F1 key).

SEnd <chan#> <message>

Parameters are an active channel number and the text of the

message to send.  The text is prefixed with "MESSAGE FROM SYSOP

AT id-callsign".  Use this to give helpful hints to users you

see that are in trouble with the commands they are trying to

use.  If you wish to NOT send the "MESSAGE.FROM..." verbage, use

the form:   SE- <chan#> <message>

SHUTdown [on/OFF]

When on, will send shutdown in porgress message to users.  Does

NOT affect bbs connects or forwarding.  Forwarding should be

ABorted and incoming bbs connects and network connects should be

DISConnected on a per channel basis.

SLottime [port#] value

Sends the given value (0 to 255) to the TNC. This value is used

in conjunction with PPersist. For more information consult your

TNC manual.

SMTP

Allows local access to the SMTP server (mainly for test

purposes).  The data channel port number for SMTP transfers is

not correctly displayed.  It does not affect operation and may

be fixed in a future release.

SNow  [on/OFF]

If you have a CGA adapter and there is a lot of snow during

screen changes (particularly when changing display pages with

F1-F4) and you don't like it, set SNow to ON.  Otherwise leave

it off and the display will run faster.

SOrt filename

Useful to sort your forward include files in the /fwd directory

so you can find things there more easily.  Will sort up to 200

eighty byte records.

MSYS 1.16                                                   Page 68

STatus <option>

With no option, it displays the status of active channels,

buffer space/errors on serial ports and dispatches per second.

OPTIONS:

RESET     Clears counters, sets dispatch values to last

minute value.

Channel   Displays only information on active channels

Ints      Displays information about interrupts received

Stats     Displays only buffer space/errors on serial ports

and dispatches per second info.

I put this one in for my own debugging (like some of the other

information displays).  FREE is what is currently free in the

buffer used to get characters from the TNC (an interrupt driven

routine is used).  LEAST is  the smallest amount  of space that

was available.  If least is small (say less than 100) you've got

some real problems.  I don't think I've ever seen it go below

3000.  If it is small it means that characters are coming from

the TNC faster than they are being processed.  You might try a

lower baud rate between the computer and TNC but this probably

won't really help the situation much.  Dispatches per second is

how many times per second each process was given control of the

CPU.  If this goes to zero I'd worry a lot!  Note: will only

display information for real ports.

SUspendtrace number

Where number is the number of free bytes in the input buffers

below which the monitoring is turned off.  There is 1 buffer of

4096 bytes for each serial port (tnc).  The amount free at any

time is given by the STatus command.  When the free space gets

less than the SU value, F2 monitoring is turned off to allow

things to get processed more quickly.  Default value is 2000

bytes.  Again, F2 displays take a lot of time and when the

system gets behind it is a good idea to turn off the updating of

the display.  When the free space in all the buffers goes above

the SU value tracing is turned back on (unless otherwise

suspended).  This function can be effectively disabled by

setting SU to 0 and then it will work as before.

TABs [ON/off]

when on tabs are expanded on local display; when off a tab shows

up as ^I.  Default is ON.

TACk [number]

Transport ack delay in seconds for network node. Default is 3.

Similar in function to RESPonsetime but for network node.

MSYS 1.16                                                   Page 69

TBdelay [number]

Transport busy delay in seconds for network node. Default is

180.

TElnet host-id [server#]

Takes an IP address (or host mnemonic) and optional server

number as parameters.

Examples:

TE [44.70.4.10]

te eiw    (eiw would have to be defined in MSYSHOST.NET)

NOTE: To allow Telnet access to your BBS, you must modify the

entries in your MSYSHOST.NET file.  Basically you must add the

call (without SSID) in square brackets of those stations that

are to be allowed BBS access.  Example:

44.70.4.5 eiw K8EIW      #this IP address would not have access

to the BBS

44.70.4.32 [k8eiw] don   #this IP address would have access to

the BBS

Note: You can use any case within the [ ], do NOT use any

SSID's!

TERM [port#]

This command connects the keyboard directly to the port so that

you may access a tnc or modem.   It may be useful for resetting

parameters in the modem or tnc.  There are two F-keys that may

be used while in this command.  The F9 key will send a sequence

which should take a tnc out of the KISS mode.  The F10 key exits

the term mode.  When leaving the term mode, make sure that tnc's

are in the KISS mode, or <PACTOR STANDBY> mode.  Modems should

be left in the auto answer mode.  While in the TERM command, the

keyboard can not be used for other functions within MSYS (F6

key), nor will input from the port be visible on the F2 screen.

There is no recording or sending of included files (\+\)and

there are no plans to add them.

TFrames Port#

Send test frames on a given port each time enter is pressed. Use

Esc key to terminate.

TImezone [string]

Allows entry of your time zone.  Set timezone GMT to all caps to

get lower case z on message time R: lines.

TNatime [number]

Network node Transport no activity timeout time in seconds.

Default is 600.

MSYS 1.16                                                   Page 70

TOdo

Causes the MSYSTODO.DAT file to be scanned immediately, doing

anything it is time to do and also setting the time of the next

thing to do.

NOTE: Be sure to use this command every time you change

MSYSTODO.DAT while MSYS is running!

TPorts [MASK]

This number is used the same way as other similar commands:

When the bit is set to 1 for a given port, that port is

available for TCP/IP use (particularly ARP broadcasts).

Trace25 [ON/OFF]

Enables/disables traces of AX.25 packets.  If you set it on you

will get more stuff that you can't understand faster than you

ever wanted to see it!  Mainly used for debugging of AX.25

protocol problems.

TraceIP [ON/OFF]

Enables/disables trace of TCP/IP problems. Again mainly for

debugging purposes!

TREtries number

Network node Transport retries. Default is 3.

TRUncate filename

Adds CR/LF to files as needed to make lines no longer than 80

characters so it can be edited with ED command.

TS   HHMMSS

TimeSet command - sets DOS time to given value (must be exactly

6 digits)

TTimeout number

Network node Transport timeout in seconds (FRACK for the

network). Default is 60 seconds.

TXDelay [port#] number

Keyup delay (usually in 10 mS increments) number given can be

between 0 and 255 (its best to err on the high side). This

number is sent to the TNC. For more information, check your TNC

book.

MSYS 1.16                                                   Page 71

TXTail  [port#] number

Keydown delay - how long to wait after sending a packet to unkey

the transmitter. The number can be between 0 and 255. This

command should be used with TNC-2 clone TNCs (any tnc that uses

hardware switches to set baud rate going to radio) when on HF.

Without this parameter the TNC keys up on the transmitter before

the complete packet is sent.  Try a value of 20 as a starting

point.

TYpe filename

Allows you to type a file.  Use spacebar to stop/start and a

control Z or A to abort.

Users filename

Writes out all users info to formatted ASCII file

VErifysysop [ON/OFF]

If on then whenever any variation of the id call is used to

connect to the BBS (except from the local keyboard) then 4

characters from the REMOTE SYSOP phrase must be entered.)  NOTE:

THIS MAY NEED TO BE SET TO PREVENT UNWANTED ACCESS BY LESS-THAN-

HONEST (AB)USERS.  The REMOTE SYSOP phrase is in a file called

PASSWORD.RMT in your FWD directory that is the same as the

MSYS.RMT file on the other system.

VIdeo     [COLOR/MONOCHROME]

You can change the monitor you want to use while the system is

running (using COLOR or MONOCHROME as a parameter).  I've got

both connected to my systems here (at the same time) so it is

meaningful to switch back and forth on rare occasions (like to

see if it works).  You probably ought to specify the one you

want to use in your MSYS.OPT file and not otherwise use this

command.

WAtch chan#

This command allows you to see all that occurs  in both

directions for the channel you specify.  Watching is turned off

when the channel is closed or you use the WAtch command a second

time  for that same channel.  The lines that go to the screen

are not labeled in any way, they are just sent to the screen.

If you WAtch more than one channel at a time it will work but

you may wonder which line came from which channel.  Using watch

may have some side effects like resetting the window being used

by the process using the channel you have selected.  You should

probably not "watch" a forwarding channel. That's a "feature" of

this program and will not be corrected!

MSYS 1.16                                                   Page 72

WBackgrnd number

This command requires two parameters to set back- ground color:

Window# and Color#.  The color number should be between 0 and 7.

WColors

Gives list of color names and their numbers

WForegrnd number

This command requires two parameters to set fore- ground color:

Window# and Color#.  The color number should be between 0 and

15.

WHO

Displays current users and messages.

Window

Use of this command is a good way to see what numbers mean in

the various window definition commands.

WHY msg#

This command will search for the words in wordfile or hold file

in the given message#.  Useful when you do a LH and want to see

why the message was held.

WPAge number number

This command requires two parameters to set page number: Window#

and Page#.  The page number must currently be between 0 and 3.

Basically this command defines what page a window will appear

in. Example: WPA 5 3

would assign window 5 to page 3 (the one you see when you press

the F4 key). This example assignment, by the way, will not be

particularly useful!

WPEvery number

Specifies the number of days between sending out WP information

for users who haven't changed their  information.  Default is 60

days.

WQupdate number

Worst quality network node destination node route to record.

Default is one.

MSYS 1.16                                                   Page 73

WRecord [number]

Window record sends all output to a disk file. The first time

use the command for a given window it begins recording

everything sent to that window in a file called window.dat where

N is the window number.  The second time you use the command for

the same window it turns off recording for that window.  Using

the command with no operands displays the status of recording.

NOTE: This function produces a lot of disk output quickly.  It

can fill up your disk.  It also slows down the system a fair

amount due to all the disk writes.  This command was added for

debugging purposes and should not be used as a general purpose

capture command.  You can determine the window numbers by using

the W? command.

WSize number number number number number

Specifies coordinates for specified window This command requires

5 parameters.  The first is the window# being defined.  The

remaining parameters are two pairs of numbers giving the upper

left and lower right corners of the window.  The pair is column

number (1,80) then row number (1 to 25).  Overlapping windows

may produce amusing results.  To specify window 3 as being the

entire screen the following command may be used: WS 3 1 1 80 25

W?

Use of this command is the easiest way to find  out which window

is which! It displays the window number of each window in the

upper left hand corner of that window.

XBatchsize [port] #

Sets number of bytes of uncompressed data per batch for RLI

compression on port.  The value defaults to 0, which means only

one message will be forwarded per batch.

XOption [port#] [on/OFF] If on, enables RLI compressed forwarding

on port

Yapp [port#] [on/OFF]

Enables use of Yapp (binary file transfer between users and the

BBS) on selected ports.

<Esc>

Cancel partially typed command

MSYS 1.16                                                   Page 74

Function key usage:

F1 through F4 select page to display (0 to 3 respectively)

F6 causes the following characters to the next return to be treated

as a command even if keyboard currently connected to some other

process (such as local BBS, etc.).

F8 key can be used to blank the screen.  Pressing any other key will

unblank the screen.  A new SYSOP command is also added that

allows you to specify the number of minutes after last keystroke

to blank the screen automatically:

Alt-F1 Alt-F2 Alt-F3 keys work like DOS F1 F2 and F3 commands to

retrieve/edit last SYSOP command typed.

Ctrl/F2  suspends  activity on the monitor  screen.   ^F2 restores

normal operation.  This is probably as close as  being able to

scroll the F2 screen that I will be implementing.

Ctrl/F4 terminates MSYS as if QUIT command was used

Ctrl/F5 is the same as the PROcesses command

Ctrl/F6 assigns keyboard to command processor

MOST COMMANDS GIVE INFORMATION ON THEIR

USAGE IF YOU TYPE COMMAND NAME FOLLOWED BY A

QUESTION MARK.

Example: WS ?

MOST COMMANDS GIVE THE CURRENT VALUE SET IF

YOU TYPE JUST THE COMMAND NAME.

Example: FRAck

MSYS 1.16                                                   Page 75

MSYS BBS USER COMMANDS

A - Abort                P - Path to station

B - Bye                  PC - Call Sign Server

C - Conference           R - Read message

D - Download             S - Send Message

G - Search file          T -Talk to SYSOP

H - Help                 U - Current users

I - Information          V - Version

J - Calls heard          W - What files

K - Kill message         X - Expert-Mode

L - List Messages        Y - YAPP Command

LC - List Catagories     ?x - Info about command

M - Message of the day   * - Comment line

N - Enter name/qth

The A (abort) command to the BBS.  It can be used to abort output

from most BBS commands, like Download, List and Read.

The Bye command disconnects you from the BBS.  Use it when you are

done!

The C command enters the user into the Conference.  Following lines

typed will be sent to all stations in the local conference.  Control-

ZQ (^ZQ) or /EX may be used to exit the conference.  When a station

enters or exits the conference, a message noting the event is sent to

the stations in the conference.  Other Conference commands are:

^ZH produces the help text

^ZU Lists the current BBS users

^ZA #  Asks the user on the given channel number to join

the conference

The CC command allows sending a "Carbon Copy" of a given message to

other stations.  Only the originator of the message, the destination

station, or the SYSOP can do this for a given message.  Syntax is: CC

msg# call1@BBS1; call2@bbs2 ...  The @bbs parts are optional if the

calls are users of your system.  Example: cc 4725 k8eiw@wb8bii; no8m

An alternate syntax for this command is: CC msg# <filename Where

filename does not include a file type (.DIS is automatically

appended).  This file contains tocalls (and optionally @BBS) of the

stations that should get the copies.  There is one call per line of

the file.  Example:

CC 3195 <locals

The file LOCALS.DIS could contain:

k8eiw @ wb8bii

NO8M

MSYS 1.16                                                   Page 76

The Download command is used to read a file stored on the system.

The format of the command is D filename where filename is the name of

the file to download.  To see what files are available for

downloading, use W or WN command.  To get a file from a subdirectory

type

d <directory_name>/<filename)>

The G command (for grep, a Unix utility) allows you to search a

downloadable file for a specified string of characters, printing all

lines that contain the string of characters.  Case is ignored in the

comparison.  No wild cards (like ? or *) are recognized.  When you

use this command you will be asked for a file name and then a search

string.  For file name enter the name that you would use if you were

downloading the entire file.  If you do not give a file name,

Database.0 will be assumed.  When the command is used, the user will

be asked for the file name to search and then the search pattern.

The default directory FILES\ will be prefixed to the file name the

user gives UNLESS the DOS SET command has been used for filesdir.

Thus downloadable files may be pattern searched, and in the directory

that the sysop chooses to make available.

The G command is not available when the BBS is in limited service

mode.

The GX command permits search on the directory specified by the DOS

SET msys:grepdir (default is FILES).

SEE ADVANCED USERS SECTION for details.

The H command will give you a short description of the commands.  For

more information about a particular command, type ?x where x is the

letter of the command.

The I command by itself gives hardware configuration of this system.

(the information displayed comes from HELP\INFO.HLP)

For example:

This system runs on an IBM PC type computer and is written in C.

It uses TNCs running in KISS mode which allows all the fancy

stuff it does.

Location is Port Huron, Michigan

TNCs KPC-4

PK-232

Frequency Radio Power Antenna

145.03 MOCOM 70 60 W Isopole at 80 ft

145.09 TR-7400A 30 W Isopole at 30 ft

If you have any questions please leave a message.

ID gives a list of the ports and digipeaters/gateways available.

MSYS 1.16                                                   Page 77

JH command lists stations recently heard on the various ports and

stations that recently connected.  Use the P command for path to

station that have connected at some time.  For the following commands

the sponsor's callsign is given in {curly brackets}.  This is

normally NOT the call you would use.  This callsign is given to help

identify some of the "mystery" callsigns used for nodes and

digipeaters such as "MYDIGI".

JBbscommand gives a list of BBS's that have been heard along with

date and time they were heard and the path to them.

JDigipeat command gives a list of the Digipeaters that have been

heard along with the date and time they were heard and the path to

them.

JGateways command gives a list of the Gateways that have been heard

along with the date and time they were heard and the path to them.

JKanodes command gives a list of the KAnodes that have been heard

along with the date and time they were heard and the path to them.

JMsys command List other MSYS systems that have been heard.

JNetrom command gives a list of the NET/ROM nodes that have been

heard along with the date and time they were heard, the real callsign

of the NODe and the path to them (most are direct).

You can specify a port number as an argument to limit the display to

just that port.  For example:

JK 1 would display the KA Nodes heard on port 1 only.

The K command is used to kill (delete) old messages from the

system.  You can kill only those messages that are to or from your

station.  The format of the command is K, a space, and then the

number of the message to delete.  Multiple message numbers separated

by a space may also be killed.

Example: k 123 6789 1123.

KM - To delete all messages TO you.

KF - Kill  messages that have been forwarded (sysop only).

KT - Msg# to kill NTS traffic you are going to deliver.

The following "K" Commands Are SYSOP only:

K* -  Goes through ALL messages, allowing you to kill, keep or hold

any of them.  You can also read a given message (it must be in the

mail.bak directory).  When reading a message this way, use the space

bar to pause the display.

MSYS 1.16                                                   Page 78

K@ - kills all messages with given @BBS. only compares the leftmost

part of the @BBS field (the normally visible part of a

hierarchical address) instead of any part which caused

unexpected deletions.

Example: K@ ARRL

K> <call>  or K< <call> will kill all messages to or from a callsign

or "group".

L - The List command lists selected message headers. The following

formats are available:

L  - List messages since you last used the B command

LB - List bulletins (all of them, use with care!)

LC - List Catagories ("TO" fields)

LM - List messages to or from you  (List Mine)

LN - List messages with type of N  (List Not read or

forwarded)

LO - Lists msgs older then date given as YYMMDD Ex.:

LO 910204

LT - List NTS Traffic messages

LL # - List the last # messages         Example: LL 10

LU - Lists unread messages to you

L< callsign - List messages from callsign    Example:

L< WA8BXN

L> callsign - List messages to callsign Example:

L> WA8BXN

L@ callsign - List messages being sent to BBS call

Example:

L@ WA8BXN

L # - List messages >= given number          Example: L 827

L # # - List messages from # to #            Example: L 50 75

L"string" - Lists messages with given string in title (case

insensitive)

L'string' - Lists messages with given string in title (case

insensitive)                                 Examples:

L'Pfb'

L"ID's"

l"Modem users"

Note: When asked for msg#'s to read, always use the indicated number

of digits.  Use leading zero's if needed to get the requested number

of digits!  Example for a 5 digit message number:   00123

MSYS 1.16                                                   Page 79

The following are sysop only:

LF - Messages that have been forwarded but not deleted

LH - Messages marked as Held (invisible to all users)

LY - Messages that have been read but not deleted

L$ - Messages with type $

L$$ - Messages with type $ (also gives their BIDs)

L$ # # - Messages in range also giving their BIDs

LL$ # - Last # messages, showing BIDs.

L# - Bulletins completely forwarded(type #)

L? - Messages that MSYS doesn't know how to route

The L< and L> commands will also take wildcards as arguments.

Note: Messages that are Private are shown only to Sender, Addressee

or SYSOP.

If the listing from the L command exceeds the page limit you have set

with the X command, it will permit you to enter the last 2,3, or 4

numbers of the message numbers that you wish to read.  Upon

completion of the displaying of the desired messages, it will

continue with the listing of messages.

The M command may be used to display the Message of the day.

Normally this message (if not empty) is displayed when you

connect unless you have selected Expert mode (?X for more info

on Expert mode).  If you have selected Expert mode, using the M

command is the only way to see any Message of the day.  If there

is no message of the day, you will simply get the next command

prompt.  The message of the day will advise you of new features

and other important information.  Example:  M

N command can be used to register your name or QTH.  You should enter

both of these.  To enter your name type N your name.

Example: N Mike

To enter your QTH, use the command NQ your qth.

Example: NQ Kirtland, OH

To enter your ZIP or Postal Code, use NZ code.

Example: NZ 44026

To enter the primary BBS that you look for your mail on use NH

callsign.

Example: NH WA8BXN

MSYS 1.16                                                   Page 80

The P command can be used to find the path last used by a station to

connect to this system.  The format is P <callsign>.

Example: P W1AW

The information given comes from the station's use of the N

commands and information recorded by the system.  Try the P

command with your own call to check the information the system

has about you.

PC <callsign>  Will return the callsign information from the callsign

database (if enabled).

The PF  (bbs_callsign) command can be used to find out if this system

knows how to forward mail to a particular BBS (what you would

enter as @bbs in the S command).  The port number and call of the

BBS to which the message would be sent is displayed along with the

VIA list used for the connect.  If NetRom or KA Nodes are used to

connect to the BBS to which the message would be forwarded to the

fact is noted but the details are not given.  If this system does

not contain the requested BBS in its forward file you get a

message that the callsign is unknown.

Example:

PF WA8BXN

The R command is used to read active messages in the bbs.  The

following are variations of the R command:

R # #     Reads the given message numbers Ex: R 12734 11521

RM   Reads your messages (Read Mine)

R catagory  Reads the messages in a given "TO" field.  Example:

R SALE

R> callsign Reads msgs with given "TO" field

R< callsign Reads msgs from given callsign

R@ callsign Reads msgs with given @BBS       Example:

R@ ARRL

RE # Read Export  This causes the given message(s) to be

displayed with a S command line, followed by title, message

text and then /EX in place of the normal way messages are

displayed.  If this form of output is saved using the

output to file command [< filename] you can easily export

messages to an ASCII file.

RH <msg#>  Gives complete forwarding information including R:

header lines.

RP   Read Preview.  It does a normal read operation but does not

mark the message as having been read so it will show up if

you do a LU command.

RN <msg#> This is like the R command but no headers are printed,

just the message text.  Good for marginal connections when

you otherwise know the message number you want to read.

REM  Read Export Mine.  Reads all messages to you in export form

MSYS 1.16                                                   Page 81

The following R commands are SYSOP only:

RS   Reads all mail for SYSOP.  Note that when you read mail for

SYSOP (ALL CAPS) it gets changed to mail for Sysop so that

it won't continue to show up as a new msg.

RES  Read Export Sysop.  Reads all messages to SYSOP in export

form)

R*   It allows you to read all held messages.  After each is

read you are prompted to enter H to continue to holding the

message, K to kill, N to release as private, or $ to

release as a bulletin.

R+   Goes through all the held msgs using the hold file trying

to identify why each was held and allowing you to continue

to hold, kill, or release as private or bulletin each

message.

Typically, after a screefull of lines is displayed on your terminal,

you will be asked:

More? [Y]es, No or Continuous

Just press return for next screenfull, N to quit reading this message

or category), or C to display all the rest without pausing. The

number of lines per screen is set by the X command.

REPly

The REPly command is used to generate a reply to a message

without having to do a S command.  The advantage of using the

REPly command is that it will usually pick up the correct @BBS

(often with proper H route) from the R: lines in the message

being replied to.  You can use either:

REPly msg# (example: REP 1234) or

REPly msg#.  (example: REP 1234.).

In the first form you will have to supply the title.  In the

second form, a title is automatically generated of the form RE:

the-original-title Messages that passed through SMTP systems

will probably not generate the correct reply address.

The SR command can be substituted for the REPLy command, and

will perform the same function.  If you put a period after the

msg #, a title will automatically be made by preceeding the

original title with RE:

MSYS 1.16                                                   Page 82

S command      Format: Sx tocall @bbs $BID#

Where x is message type (P=private, B=bulletin, T=traffic)

If the x is omitted, and the MAkePRivate is on, the message will

automatically be made a type SP

Tocall is DEStination callsign (without - number/SSID)

@bbs is DEStination BBS (again without any - number) [OPTIONAL]

---> If @BBS part is omitted, if the station has entered a Home

BBS (NH command) that BBS will be used.  Otherwise the to

station callsign will also be used as the @BBS.

$BID# is the Bulletin IDentification Number [Valid ONLY for type

B - OPTIONAL - if omitted, MSYS will assign if needed.]

Examples:

SP call @ bbs.haddr SP K8EIW @ WB8BII.OH.USA.NA   Send a pvt. msg

ST zipcode @ NTSst  ST 44070 @ NTSOH              Send traffic msg

SB catagory @ route SB NEED @ ALLOH               Send a bulletin

SR msg#             SR 12723 or SR. 12723         Send reply.

You will next be asked for a title for the message.  After this

you type the message, keeping all lines less than 80 characters.

When you are done with the message type ^Z or /EX on a new line.

Never use SSID's in callsigns (ex: don't use WA8BXN-2, just use

WA8BXN)

Hierarchical addresses (haddr)  are of the form:

state.country.continent.

Other sub-areas may be used by BBS in a particular area, such as:

#NEOH.OH.USA.NOAM.  If you know them, use them.

When sending personal messages to a distant bbs, it is a good idea to

put the destination city and state in the title.

MSYS 1.16                                                   Page 83

If you type ^A in place of ^Z when you are done typing a

message, the message will not be stored.

Users who send messages to stations without giving any @BBS or

giving unknown @BBS are warned of the fact.

Use SB to send bulletins, ST to send traffic, etc.  All messages

sent to a specific callsign will be treated as if you used SP

(send private).  To make messages to a specific callsign

readable by everyone use SB instead of just S! Or make it to a

non-callsign (such as ALL).  If you are sending a bulletin that

may also be entered on other BBS, use the form:

SB <to> @<bbs> $<BID#>

so that a proper BID# will be assigned.

The SR <msg#> command functions identical to the REPLy <msg#>

command.

The ST Command will NOT add a BID to messages.

Notes about the @BBS:

Automatic generation of @BBS field for S command in BBS:  If the

S command contains an @BBS entry it is left untouched.  If there

is no @BBS as a parameter for the S command, the following

happens.  If the to station is found in the user file then the

home bbs of that station is substituted for the given @BBS.  If

the to station is not a known user then the to station callsign

is placed in the @BBS field.

Any messages entered into the system with no @BBS that don't get

a @BBS from the user file will have ==== for the @BBS.  Only

messages with second type character of N will cause the user

file to be searched for @BBS.  Thus bulletins will never get

their to call sign (typically ALL) looked up in the user file.

In the bbs, if a S command has an SSID in the @BBS filed it is

removed in the leftmost part of the hierarchical address.  Thus

S W1XYZ @ K1XXX-5 is taken as S W1XYZ @ K1XXX.

MSYS 1.16                                                   Page 84

T    The T  command (talk to sysop).  When a bbs user types T you

will get the connect music and then have 60 seconds to type

something or they will be put back to the bbs.  If you do type

to them then when you are done with your conversation and do ^Cd

then they will go back to the bbs.  The keyboard call can still

be used for incoming connects; note that it is accessible to all

stations, even on closed bbs ports.  Outgoing connects are the

same as they have always been.  When an incoming keyboard

connect occurs, you will automatically be taken to the F1 screen

If you wish to prevent users from ringing your bell, set MCON 0.

U    The U command gives the current users of the bbs plus the number

of active messages and users and what they are doing.

UPload

The UPload command To use it, the user must be authorized (see

user flags ).  To do an upload, the user types UP on the BBS.

MSYS will then ask for the file name to be uploaded.  If no file

name is entered, or the file exists (in the FILES directory) the

upload is terminated.  If the filename is accepted, the user

then sends the ASCII file, ending with a line that contains only

^Z.  Once the transfer is started, it may be aborted by sending

a line with ^A.

Version

The V command gives the version of this software and date and

time MSYS was started.

W    The W command lists the available files for downloading.

Examples:

W    Lists all files in C:\MSYS\FILES

W (Dir1)  Lists all files in the (Dir1) subdirectory of

C:\MSYS\FILES\(Dir1).

The directory list provided includes the dates & sizes of the

files.

MSYS 1.16                                                   Page 85

X    Command can be used to set or reset Expert status and also to

set the number of lines sent before pausing during an R command.

If no argument is given, X simply toggles and displays the new

Expert status.  When Expert status is set, the initial connect

messages are replaced by a compatibility identifier line ([MSYS-

1.16-BFH$]) and the command prompt is simply >.   If a number is

given after X, this is the number of lines to display for a R

command before asking More?  To continue displaying the current

message when More? is sent, press return.  To abort the message,

type N and then return.  This facility has been added to prevent

a long message from scrolling off your screen faster than you

can read it.  Experiment with different values until you find

one that pleases you.  To not use this facility, set the number

of lines to zero.  Examples: X 20 (pause after every 20 lines) X

0 (don't pause ever).  When you get More? in R cmd output (as a

result of using X cmd to set lines between pauses), you can type

C for continue without any more stops.

XC

Toggles the automatic display of msg catagories when you connect

to the bbs and are using non-expert mode.  Default for new user

is ON

XF and XS Commands

XF sets "fast" mode, XS sets "slow" mode.  Fast mode means that

multiple lines are placed in packets for some commands (like D,

R and L).  Slow mode means one line per packet as it has been

(the default).  XF is faster for users with good paths.  XS is

better for users with poor paths (or busy channels!).

XR

Toggles the automatic asking if you want to reply to a msg after

you read it. Can be useful if you get a lot of personal mail.

Default for new user is OFF.

YD pathname filename

YAPP download of the given file.  Users must start their YAPP

function within 30 seconds of issuing the command.

YU pathname filename

YAPP upload of the given file.  Users can not delete or replace

existing files. Users must be enabled with user bit $800 to be

able to upload files.

YW

List files in the default YAPP directory.  (See section on DOS

SET for further details on defaults.)

YW xxx    List the contents of the xxx directory under the YAPP

directory.

MSYS 1.16                                                   Page 86

*    Putting * at the beginning of a line makes it a comment.  It

also suppresses the next command prompt (but the system will be

waiting for another command).  * is useful to answer the SYSOP

if you get a: MESSAGE FROM SYSOP

< filename (sysop only)

Causes the commands found in the given file to be processed as

if they were typed from the keyboard.  This may be used as a

message import function if messages are placed in a file.  A

sample is:

SB ALL @ ARRL < W1AW $XYZ27

Sample title

This is the text of the message.  It is short.

/EX

You can of course have many messages in the file.  When end of

file is found, input is taken again from the keyboard.

> filename and >> filename

Both of these commands cause output normally sent to the screen

to be also saved in the indicated file.  Using just one > causes

a new file to be created (replacing any existing file by that

name).  Use of two > before the filename causes output to be

appended to the end of the filename given (if it exists).  To

terminate writing to the file, use either of these commands

without a file name.  While output is being saved to a file,

there will be NO PROMPT FOR NEXT COMMAND.

/ <cmd>   To be used for sysop commands while in the BBS (see remote

sysop) or:

\ <cmd>   To be used for sysop commands while in the BBS (see remote

sysop)  Either symbol works!

MSYS 1.16                                                   Page 87

REQxxx SERVERS

A set of REQxxx servers are available in the BBS they are: REQDIR,

REQFIL and REQQTH.  Messages sent to your system to REQanything will

be processed.  If a server is not available for the requested

function, a reply message will be generated stating that the server

is not available.  The servers named above are enabled by new

commands of the same names.  That is, using REQDIR ON will enable the

REQDIR server, etc.  Enabling the REQQTH server also requires that a

file name containing the Buckmaster CD ROM ASCII callbook file

(normally S:HAM0\HAMCALL.129) be specified with the CAllfile command.

The REQDIR server responds with a message to the requester containing

either the FILES directory listing or specified FILES directory

subdirectories given in the message title.  An @BBS to which the

message is to be returned may optionally be given in the title if the

reply is to go to a bbs other than the one at which the original

message was entered.  Examples:

SP REQDIR @

WA8BXN

(blank title)

^Z

would return a listing of the FILES directory to the bbs used to

originate this message.

SP REQDIR @ WA8BXN @NT8V  (title)

^Z

would return a listing of the FILES directory to the sending

station @ NT8V.

Subdirectories may be given in the title, proceeded by a slash and

separated with blanks.  The optional @BBS may also be specified.

Example:

SP REQDIR @ WA8BXN

/MODS /PGMS/BASIC

^Z

The file REQDIR.HLP included on the distribution disk should be

placed in the HELP directory.  The file REQFIL.HLP should be placed

in the FILES directory.

MSYS 1.16                                                   Page 88

The REQFIL server responds by sending the requested file.  The file

name is specified in the title of the message.  Subdirectory names

may be given if needed (don't include the leading /).  An @BBS can be

specified in the title as described for REQDIR above.

Example:

SP REQFIL @ WA8BXN

mods/kam.1 mods/kam.2

^Z

The REQQTH server responds with callbook information for calls given

in the title.  Currently only US hams are in the callbook file.

Calls must be separated by blanks.  An @BBS may be specified as

described above.

Example:

SP REQQTH @ WA8BXN

nt8v no8m K8EIW

^Z

MSYS 1.16                                                   Page 89

CALLSIGN SERVER

The callbook server has split out of MSYS.  There are three servers

available, one for the Buckmaster CD ROM, one for the RT Systems SAM

database, and one for the j-com disk files.  Note that the CAllfile

command has been removed from MSYS.  In its place you specify the

name of the file (or directory in case of j-com files) to be used

when you start the appropriate call server TSR.

To use either the Buckmaster CD ROM callsign files, or those

available from j-com, and SAM (c) RT Systems,  you must load the

appropriate TSR program before starting MSYS.  Once this is done,

accessing the call server is the same as it has been in the past

(using either the PC command in the BBS or sending a message to

REQQTH @ the bbs call).

The TSR for the CD ROM files is CALLSRV1.  It

optionally takes one pararmeter, the name of the

callsign file.  If none is given, the default is

S:\HAM0\HAMCALL.129.

The TSR for the j-com files is CALLSRV2.  It

optionally takes one parameter, the name of the

directory in which you installed the several j-

com files.  If none is given, the default is

F:HAMBASE.

The TSR for the SAM (c) files is CALLSRV3.  It

optionally takes one parameter, the name of the

directory in which you installed the several SAM

(c) files.  If none is given, the default is

F:HAMBASE.

THINGS NEEDED TO MAKE THE CD-ROM DATA BASE CALLSIGN SERVERS WORK:

Put the following in your CONFIG.SYS and reboot system.

LASTDRIVE=W

DEVICE=SONY_CDU.SYS /D:MSCD210 /U:1 /B:340 /M:P /V /L:S /Q:*

/T:*

Run the following before MSYS (probably put these statements in

AUTOEXEC.BAT):

MSCDEX /D:MSCD210 /M:4 /L:S /V

MSCDINFO

CALLSRV1 S:\HAMO\HAMCALL.129

MSYS 1.16                                                   Page 90

Put the following in  MSYS.DO:

REQQTH ON

Notes: You may have to make changes in the above statements if you

have different device drivers for your CD ROM drive or set up the

hardware options differently.  Running without interrupts works

pretty well on the CD ROM drive and if you are like me you don't have

any interrupts to give to the drive anyway.  If you don't want to

respond to REQQTH queries but only enable the PC command on the BBS,

set REQQTH OFF.

For all the servers, the title must be less than 39 characters.

THINGS NEEDED TO MAKE THE SAM (c) DATA BASE CALLSIGN SERVER WORK:

Put the following in your AUTOEXEC.BAT file.

E:\sam\api\samapi  E:\sam     This installs the SAM

(c)                      API interface.

C:\msys\callsrv3.exe E:\sam   This installs the MSYS

interface to SAM (c)

Of course, this presupposes that you have already installed the SAM

(c) data base on your E: drive.

Put the following in  MSYS.DO:

REQQTH ON

PC ON

MSYS now supports the SAM optional County data base as well.

MSYS 1.16                                                   Page 91

MODEM SUPPORT

MSYS will support one or more modem ports.  The modems must be

capable of supporting true carrier detect (CD) operation.  Both the

CTS and the DTR line must be connected!

MSYS modem forwarding protocol

When MSYS attempts to forward to another BBS, the following protocol

is used for the initial logon. When MSYS sees the other system send

"Call:" or "Callsign :" it will respond with two lines. The first is

"echo off" and the second is the callsign indicated in the forward

file. Next, when it sees "Password:" or "Password :" it responds with

a line that is the password from the forward file. It then proceeds

using the normal packet protocol lines.

Incoming Call Protocol

When carrier is detected from the modem, MSYS sends the line:

MSYS Modem Interface (c) Copyright 1993 Hub Computers Inc.

It then attempts to open MSYSMOD.MSG. If the file is found, its

contents are sent to the modem. Next MSYS prompts with "Call:". At

this point a human operator would enter their callsign value. An MSYS

connecting for purposes of forwarding would reply with "echo off"

which disables echoing by the called MSYS system and then the

callsign value would be send on a second line.  MSYS next prompts

with "Password:" and expects the password to be entered that

corresponds to the previously given call sign.  These two words are

then used to scan the MSYSPASS.MOD file to find a match.  If a match

is found, the third word on the line in the MSYSPASS.MOD file is used

as the apparent callsign with which the user will appear to have

connected to the system.

Here is an example of a normal user connecting into MSYS via modem:

MSYS                     User

-----                    --------

MSYS Modem Interface ...

[contents of MSYSMOD.MSG]

Call:

NO8M

Password:

camping

[from this point on it looks just like a packet connection was made]

MSYS 1.16                                                   Page 92

Here is an example of a MSYS system forwarding into another MSYS

system via modem:

MSYS                     Calling MSYS system

-----                    -------------------

MSYS Modem Interface ...[contents of MSYSMOD.MSG]

Call:

echo off

WA8BXN

Password:

BINGO

[normal packet protocol follows]

Use ONLY FBBC 0 and XO OFF on modem ports. Very Important!!!

The use of FBB style forwarding and/or compression on the modem port

will usually result in buffer overflows, and cause serious

consequences to the continued high quality performance of the system.

Note: To accommodate exchanges with FBB systems, use of control

characters in the call and/or password should not be needed.  The

callsign must be prefixed with a period to turn off echoing from the

FBB system. Here is an example MSYSFWD.DEF file entry

F2 K8EIW ATDP5551212 .WA8BXN MSYS

The F2 could be R2 or P2 to ask for reverse forwarding or to always

poll respectively. The 2 can be replaced by whatever port number your

modem is connected to.

K8EIW is the call of the bbs we are forwarding to.

ATDP5551212 is the telephone number to call. The P could be replaced

by T if tone instead of pulse dialing is desired.

.WA8BXN specifies the "callsign" value to be give. The period says to

FBB systems to turn off echoing. The actual callsign value used by

FBB is just the WA8BXN part.

MSYS is the "password" value that will be sent to the FBB system.

This example is for forwarding to FBB systems. When forwarding to a

MSYS system the only difference is that the period is NOT used in the

"callsign" field.

MSYS 1.16                                                   Page 93

MSYS.OPT FILES

The name of the port MUST be (in capitals) "MODEM"  Radiospeed must

be set to 1200.  Port speed must be set to the speed of the modem for

a 300 to 4800 baud modem.  For modems that can lock the port speed it

should be set to at least 19.2k for a 9.6 or 14.4k modem.

Here is an example:

port 5 at $260 int 5 speed 19200 name MODEM radiospeed 1200 tnctype 2

MSYS.DO

The CTS option MUST be turned on in the MSYS.DO file.  The following

shows the proper command for turning CTS on for port 5:

CTS 5 ON

The CTS line must be connected in the EIA cable from MSYS to the

modem!

MSYS 1.16                                                   Page 94

MSYSTNC.x FILES

There are a number of ways to set up the modem.  Here is a file for a

Zoom 2400 baud modem:

atz ---------- reset the modem to the power on defaults

atq1 --------- don't send responses

atm1 --------- speaker will be active until connect

ats0=1 ------- answer on one ring

The only problem that might show up with these files is that the

"ats0=1" *MUST* be last in all the files.  If it is another location

and someone is calling in at the time of the initialization, the

modem will answer and then ignore all the commands that follow and

come up with an improper initialization.

One 2400 baud modem was found with a unique need.  Register 27 on a

small plug-in was found to control the V22.bis routine.  If 27 was

set to one (the default), then the modem would *NOT* initiate a

V22.bis error correcting connect.  The factory default was that 27

was set to never make an error correcting connect!  The line

"ats27=0" cured the problem.  AND ... it was found on a very generic

2400 baud modem that this same register, which was not mentioned in

the documentation, worked the same way.  Check for register 27 on

your 2400 baud modem!

Here is the MSYS.OPT string used with a V32.bis Zoom modem which is

able to connect from 300 to 14.4k (with automatic fallback):

port 5 at $260 int 5 speed 19200 name MODEM radiospeed 1200 tnctype 2

Here is the MSYSTNC.2 file that then sets it up:

atz ........ reset

at&f ....... load factory defaults

atl0 ....... speaker volume low

at&c1 ...... DCD follows carrier

at&d2 ...... DTR signal setting

atq1 ....... no responses

atm1 ....... speaker on until carrier

ats0=1 ..... answer in one ring

This may or may not work with your modem.  A book and a whole bushel

basket of experimentation may be necessary.  Without the book for

your modem, your work will be cut out for you.

MSYS 1.16                                                   Page 95

One of the HAYES commands is AT&V.  This command will permit you to

see the setup for the different registers in your modem.  The

following is the display from a working Hayes Ultra 96 modem:

ACTIVE PROFILE:

B16 B1 B41 B60 E0 L1 M1 N1 Q1 T V1 W0 X4 Y0 &C1 &D2 &G0 &J0 &K3 &Q5

&R0 &S2 &T4

&U0 &X0 &Y0

S00:001 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:099

S08:002 S09:006

S10:014 S11:095 S12:050 S18:000 S25:005 S26:001 S36:007 S37:009

S38:020 S44:003

S46:002 S48:007 S49:008 S50:250 S97:030

It has been reported to us that the following string works well with

the US Robotics Sportster modem:

AT&F&B1&H1&R2Q1M1S19=2X4S0=1

The detail the individual commands is as follows:

AT&F      Load factory (rom) settings into RAM

AT&B1     Fixed serial port speed

AT&H1     Hardware Clear to Send

AT&R2     Received data hardware (RTS) flow control

ATQ1      Supressed result codes

ATS19=5   Inactivity timer set to 5 minutes

ATX4      Dial tone detect, busy/no answer detect, etc

ATS0=1    Answer on one ring

DIP switch positions 1-7 off, and 8 on.

It should be noted that some of the above commands do not exactly

follow the Hayes Command set.

There does not seem to be a problem initializing most modems with one

long string as opposed to individual "AT" commands.

Use at your own risk!  READ YOUR MODEM MANUAL!

MSYS 1.16                                                   Page 96

MODEM USAGE AT 4.8, 9.6 AND 14.4k

Hardware flow control between a modem using 9.6k or above is

critical.  MSYS now hosts full CTS support.  This means that the

modem's port speed can be set to a speed higher than the maximum

connect speed.  When a connect is achieved, the speed between the

modem and the computer remains at that higher speed no matter what

the speed of the modem to the remote person connected.  If a person

checks in with an old 300 baud modem, the speed between the computer

and the modem remains at what it was sent as.  The modem will accept

data until its buffer is full.  It will then signal the computer, via

CTS control, that it is full and will await the buffer emptying

before it tells the computer to send more data  A full RS-232 cable

is necessary.  The three wire cables that work in some TNCs will not

work here.

NETNode USAGE

Access to a network node from the modem is possible.  It is

accomplished by adding the capitalized string "NETNODE" to the

appropriate line in the MSYSPASS.MOD.  USE THIS OPTION WITH CAUTION!

An example follows:

wombat poobear WA8BXN NETNODE

A user designated with the NETNODE parameter will first be put into

the MSYS NETNODE and will have FULL capability to access the packet

network.  For such a user to access the BBS, he would give the normal

BBS command.

MSYS 1.16                                                   Page 97

FORWARDING ON THE MODEM PORT

LET'S PAUSE FOR A MOMENT ... DO NOT FORWARD WITH MSYS ON A

TELEPHONE LINE UNLESS YOU ARE ABLE TO INITIATE A ERROR CORRECTING

CONNECT WITH THE MODEM.  A NON-ERROR CORRECTING CONNECT WILL MAKE

YOUR STATION RESPONSIBLE FOR GARBLED, JUMBLED AND MESSED UP MESSAGES.

ALSO: DO NOT ATTEMPT FBB COMPATABLE OR COMPRESSED FORWARDING VIA THE

MODEM PORT.  YOU WILL OVERRUN THE AVAILABLE BUFFERS!

The FWD.DEF file defines the script MSYS uses to establish a

forwarding session.

In the FWD.DEF file, put a  F#, R#, or P# line followed by the @BBS

items to forward. The format of the F, R and P lines are identical

except for the letter.  This format is different from what would be

used for packet forwarding and is as follows:

F# callsign phone# callword password

$fwd/<call.rev>

where #        is the port number,

callsign       is the call of the station you are forwarding to,

phone#         is the string to send to the modem to dial,

callword       is the reply to give to Call:, and

password       is the reply to give to Password: (questions asked by

the destination MSYS system)

$fwd/<call.rev>     is the directory and file which contains the

forwarding information

Example:

F1 WA8BXN ATDP5551212 swiss normal

$fwd/wa8bxn.rev

NOTE: The log-in callword and password are converted to UPPER CASE.

Thus the station you are calling must have UPPER CASE values for your

callword and password.  The station you are calling must be running

MSYS 1.11 or higher.

NOTE: The included file in the above example will be in the MSYS/FWD

directory.

MSYS 1.16                                                   Page 98

FBB ON THE PHONE

FBB style forwarding (compressed or not) should NOT be used on the

modem.  Compresssed forwarding just won't work and the non compressed

mode has a high likelyhood of overrunning buffers on long messages

because of the continuous transmission stream of a block of messages.

Therefore, the FBBC parameter MUST be set to 0 for the modem port.

"Normal" forwarding to and from FBB systems is possible however

Use ONLY FBBC 0 and XO OFF on modem ports. Very Important!!!

Use of control characters in the call and/or password should not be

needed.  The callsign must be prefixed with a period to turn off

echoing from the  FBB system. Here is an example forward file entry

F2 K8EIW ATDP5551212 .WA8BXN MSYS

The F2 could be R2 or P2 to ask for reverse forwarding or to always

poll respectively. The 2 can be replaced by whatever port number your

modem is connected to.

K8EIW is the call of the bbs to which we are forwarding.

ATDP5551212 is the telephone number to call. The P could be replaced

by T if tone instead of pulse dialing is desired.

.WA8BXN specifies the "callsign" value to be give. The period says to

FBB systems to turn off echoing. The actual callsign value used by

FBB is just the WA8BXN part.

MSYS is the "password" value that will be sent to the FBB system.

This example is for forwarding to FBB systems. When forwarding to a

MSYS system the only difference is that the period is NOT used in the

"callsign" field.

FORWARDING TO AA4RE SYSTEMS

It has been reported that for MSYS to forward to a 4RE bbs via land

line, the 4RE bbs needs to change the responses in the MESSAGE.BB

file.  First, change MESSAGE 95 N to: Call:   and MESSAGE 96 N to:

Password:  These two changes have been tested between KI7HM (MSYS and

KD7HP (4RE) and are reported to work ok. (Thankyou Gentlemen!)

MSYS 1.16                                                   Page 99

ANONYMOUS CHECK-INS

It is possible to set up the modem port to take a call from an

anonymous check-in.  As long as all outgoing traffic is held, people

interested in ham radio, awaiting their license or whatever can read

and send traffic.  As long as ALL outgoing anonymous traffic is held,

there are no legal implications.  MSYS is designed so that if someone

uses the string "visit" for both the call and the password, and if

visit is in the MSYSPASS.MOD,  MSYS will allow a check in.  Here is

how the MSYSPASS.MOD should be set up:

VISIT VISIT V1SIT

Visit Visit V1SIT

visit visit V1SIT

(other calls follow)

By having all the various combinations of case, a anonymous check-in

does not have to worry about the case that is used.  Now, it is

CRITICAL that MSYSHOLD.DAT have the lines "connected=v1sit" and

"from=v1sit".  All anonymous traffic will be held.  It can then be

routinely deleted or readdressed by the sysop (with the sysop's call

being used for the sending party).

It is also necessary to use the "EU V1SIT" command to add the user to

your system.  Here is what the file will look like when done:

V1SIT is VISITOR located at VISITOR  Last new message 19485

Last connected Fri Mar 19 23:10:57 1993 on port 5

Flags=$0140 Lines=20

Postal code: VISITOR  Home BBS: NO8M  Last WP Update: Mar 10

08:41:01 1993

Path to V1SIT is Direct

Other calls than V1SIT can be used, however MSYS is programmed to

disallow changes to the V1SIT callsign's information.  An anonymous

check-in can not go in and change V1SIT's home PBBS, name, etc.

Also, please note that the user "FLAGS" for V1SIT are set at $140 to

prevent erroneous WP messages from being generated!

MSYS 1.16                                                   Page 100

It helps to have a help file that tells the anonymous users something

about how to get help.  Here is an example MSYSMOD.MSG:

*******************************************

** Welcome to the NCARC MSYS Modem Port! **

*******************************************

For information on the system, use "I".  To a list of messages on

the board, use the command "L 0".  To see what is in the

download section, use the command "W".  (Use "D FILES.LST" and

"D README.DOC" for information on how to get those files.)  A

MSYS User's Manual is available and how to get it is in the

file USERMAN.INF.)  Please DO use this modem port for

downloading the LONG files I have here ... we have PART 97 of

the FCC Regulations and other very long files that are

inappropriate for downloading by a RF link.

If you are interested in amateur radio and how to get your license

leave me a message (SP NO8M or SP SYSOP) and I will get you in

touch with someone that can help.

-----end of MSYSMOD.MSG

Anonymous use of the port will be in direct proportion to the

advertising that is done outside of amateur circles.  If you make up

fliers and distribute them, you will see port usage rise.

MSYS 1.16                                                   Page 101

ACOUSTIC COUPLED MODEMS

There are times that you may want to utilize an acoustically coupled

modem port.  This might be needed when you want to forward

automatically from a pay phone.  The MSYSFWD.DEF line in this set up

would read like this:

-----

R0 NO8M ATX1D callword password

$FWD\NO8M.REV

-----

A callword and password that is a unique, pretend user is set up.  If

the callword and password were BOZO and CLOWN, the following line

could be used in your MSYSPASS.MOD:

BOZO CLOWN N08M

The pretend callsign N08M would then be set up on the home PBBS to

forward what you wanted.  You would add the pretend call to your home

MSYSFWD.DEF like this:

-----

r5 n08m

!25

$fwd\n08m.rev

-----

This would cause the home MSYS to keep all the traffic that you

wanted for the acoustic system (and was listed in the N08M.REV)

listed as PN or B$ and ready to forward into the remote system.  Make

sure you add the pretend call to the MSYSBBSB.DAT if you will be

forwarding bulletins.

MISCELLANEOUS NOTES

ON USAGE       If you have a user who ties up a RF port for hours

reading each and every message, you may benefit by having him start

up a MSYS, check in on the phone port and get each and every message

automatically.  In that way, the user can read the stuff off-line at

their own computer.

ON R: LINES:   The above user may then try to automatically forward

messages out on that phone MSYS that was set up.  This will add a new

R: line that will be unknown to your network.  You can either begin

supporting each and every PBBS that comes and goes or demand that the

station send their message with NO R: line and that they send their

messages manually.

REQUEST

Please forward problems, concerns and hints to Steve,

NO8M@NO8M.#NEOH.OH.USA.NA.

MSYS 1.16                                                   Page 102

DATABASE HELP

Commands related to Database manipulation available on MSYS

MSYS allows BBS users to input information and search for information

in simple data bases.  The names of the databases are Database.0

through Database.999.  These files are located in the normal FILES

directory, which can be accessed by  the D command (download) on the

BBS.  These files can also be searched by the G command on the BBS.

To use this command you enter the command G (with  no operands).  You

will then be asked for the file you want to search.  Simply pressing

return at this point will select Database.0 by default .  You are

then asked for the search  string. This is a set of characters that

when found in lines of the file cause those  lines to be displayed.

Case is ignored during the search.  If a selected line ends with \

then the following line(s) are also selected for display.  Data is

entered into a data base by sending a message to MSYS with DBADD in

the title.  The text of the message will be added to the indicated

Database (Database.0) by default.  A header line is also inserted in

the data base telling the station that made the entry,  long with the

date and time.  Here are some examples:

SP MSYS   {this is a S command issued to the BBS}

DBADD 3   {the message title, Database.3 to be used}

wind damage mentor Ohio \

large tree down at 615 & 20 \

reported by mentor pd \

no injuries

^Z   {normal way to end message}

The text of this message would be added to Database.3.  Now if you

search Database 3 for either wind damage or mentor for example,  you

would get to see these lines, along with any other lines that have

wind damage or mentor in them.

MSYS 1.16                                                   Page 103

Example of using search command:

G              {BBS command to do search}

File search routine {Response from bbs}

Enter file name (or just press return for Database.0): Database.3

Enter search argument: Mentor

{lines from message above displayed}

>              {BBS ready for next command}

Another example:

Assume a file had the following lines:

Mary had a little lamb

its fleece was white as snow, \

and every where that Mary went

the lamb was sure to go.

It followed her to school one day

If you searched this for 'snow' you would get the following back:

its fleece was white as snow, \

and every where that Mary went

That's a brief outline of what can be done.  Its simple yet very

flexible.  Let me know of any applications you can think of for these

facilities.  Particular Database numbers can be assigned to different

uses.  Formats for information can be suggested for each different

Database.

MSYS 1.16                                                   Page 104

FORWARD FILE DESCRIPTION

NOTE: TO FORWARD BULLETINS YOU MUST SET UP MSYSBBS.DAT!

The MSYS forward file (MSYSFWD.DEF) is an ASCII file that can be

prepared with any program editor.  The file consists of groups of

lines separated by a line beginning with at least 3 dashes.  Each

group of lines specifies what should be sent to a given BBS along

with other information.

The first line has the following format:

Position       Content

--------        -------

1   The letter F      If you substitute R for F (F0 becomes R0

WA8BXN) then it enables reverse forwarding

request.  P0 Forces a reverse forward poll.

EXCEPT IN FBBC MODE!

2           The port number to use (a single digit)

3           A blank

4to N       Callsign of BBS (with SSID) optional:

N+1         A blank

N+2 to ?    Via list of digipeaters

Examples:

F0 W1XX V W2XX,W1XX-1

F2 WA0XYZ-2

When no nodes are being used, this first line is used as if it

were a connect command to the desired system .  When a NODe is used,

the Via digipeater list will have no effect.

If a NODe (KA NODe or NET/ROM) is to be used, the next set of

lines are a script of how to use the NODe(s).  This script may be up

to 25 lines long.  Its format will be described below.

MSYS 1.16                                                   Page 105

Next comes a list of callsigns of BBS systems that should be

forwarded  to the system given on the first line (the one beginning

with F).  There is one call per line.  No SSID should be given.  The

callsign of the system given on the first line should normally be

given in this list.  Example forward file that doesn't use any nodes:

F0 W1XX

W1XX

W2ABC

W2XXX

----------

F0 W1XX V W1XYZ

W1XX

W2ABC

W2XXX

---------

F0 W1XX V K1XXX,W1XXX

W1XX

W2ABC

W2XXX

--------------

F0 W8XXX-12

W8XXX

WA8XXX

K7XXX

------------

Note: In the previous example we try three different paths to connect

to W1XX.  If we get  the messages forwarded the first way the other

connections will not be attempted.

F0 W1XX

!01-04,12,21-23          This will forward during the times indicated

W1XX                     (Use of this line is optional, but if used,

W2ABC                    must begin with a !)

W2XXX

----------

In the list of callsigns given indicating what BBS's msgs can be

forwarded to a particular BBS if a second entry is typed on the line

then the second entry must match some part of a message title to

allow it to be forwarded.  For example,

F1 W1XX

ARRL DX

----------

@ARRL  will be forwarded only if DX appears in their title.  Case of

letters is ignored in the comparison.

MSYS 1.16                                                   Page 106

You can use * in the forward file.  If you have NTS* then all msgs

with NTS as the first 3 characters of the @BBS will be forwarded.

For example:

F1 W1XX

NTS*

----------

*  used alone is treated specially.  Only messages that have been on

the BBS for more than about 2 days will be forwarded.  This gives

time for them to be forwarded by other specified connections.

IMPORTANT  NOTE:  In the forwarding file, use of * for the BBS names

cannot be the first call and the first bbs in the list of BBB's that

can be forwarded to through the given bbs MUST  begin with a letter.

Example:

F0 WA8BXN

*         <---- won't work

WA8BXN

-----------

F0 WA8BXN

44*       <----- won't work

WA8BXN

-----------

F0 WA8BXN

WA8BXN    <----- will work

*

44*

-----------

Note: That # is a wild card character.  If you want to put #XYZ as an

entry in your forward file, you will have to use "#XYZ (the " is an

escape character that says the character that follows must match

exactly and is not treated as a wildcard character as it normally

would be).

MSYS 1.16                                                   Page 107

NODe Scripts

The script consists of several sets of lines.  The first line begins

with  @  followed immediately by the call (and Via digipeater list if

needed) of the NODe to initially connect to.  The port that will be

used is taken from the digit after the F on the first line in the

group.  If the connection is established to the NODe, the following

sets of lines are used to talk to the first NODe and possibly other

nodes called through it.

Each set of lines for a given NODe begins with a line that has a

period in position one.  The following characters on the line are

sent to the NODe as a command.  The next line in the set is optional

and if used has  #  in the first position.

Following this is the number of seconds to wait before abandoning use

of this NODe.  If this line is not given then 60 seconds will be

assumed.

The next line is required and begins with  +  in the first position.

The following characters are what are looked for in lines that come

from the NODe to indicate that connection has been established to the

station called (another NODe or the desired BBS).  An exact match to

the characters given on the line (including case) must be found

somewhere in a line received to continue.

Next come two lines beginning with  ?  in the first position.  These

lines contain strings of characters that if found anywhere in a line

that comes from the NODe that the connection was not made.  Thus

after we issue the connect command (given on the line beginning with

.) one of three things can happen: a line from the NODe that matches

the +string is found and we continue with the script or talk to the

BBS, a line from the NODe matches a ?string which means we give up on

making the connection, or a time-out occurs waiting for a match which

again means we will give up  the connection.

Example scripts:

These strings are useful for KA Nodes.

@ANOD    (issue connect command to ANOD)

.C       (issue connect to desired call)

#25      (time out after 25 seconds if no match)

+MADE    (if MADE comes from NODe, continue)

?BUSY         (if BUSY comes from NODe, abandon attempt)

?RETRIED (if RETRIED comes from NODe, abandon attempt)

For a NET/ROM NODe the following would be useful:

+to

?Failure

?Busy

MSYS 1.16                                                   Page 108

Here is another example of a complete forward file:

F1 W8XXX

W8XXX

W5XYZ

---------

F0 W4XYZ

@W7XXX

.C W9XXX

+to

?Busy

?Failure

.C W4XXX

#120

+MADE

?RETRIED

?BUSY

W4XYZ

W4XXX

-------

F1 W9XXX-2 VIA K9XXX-3,KA9XXX

W9XXX

W1XXX

W3XXX

K0XXX

----------

NOTE:  The forward connect script for a given bbs is limited  to 25

lines.

MSYS 1.16                                                   Page 109

MSYSBBSB.DAT

(YOU MUST HAVE THIS FILE TO FORWARD BULLETINS!)

This file contains a set of bit position numbers and corresponding

BBS calls to which $ BID messages will be sent.  The format of the

records is:

Bit#  Callsign(0-31)

An example file content is:

25 WB8LVP

26 KA8Z

27 KB8AST

28 KQ8M

29 NO8M

The MAXIMUM BIT# IS 31

MSYSFWD.DEF:  If you put a line beginning with $ and followed by a

file name (where BBS callsigns normally go) the given file will be

read at that point as more BBS calls.

Example:

F0 WA8BXN

WA8BXN

$BXN.DAT

ARRL

--------

In this example, the contents of BXN.DAT will be used for more calls

that can be forwarded through WA8BXN.  Due to the extra overhead of

opening, reading, and closing the file, this should only be used for

more than just a few call signs.

MSYS 1.16                                                   Page 110

(callsign).FWD/.REV Files

One way to reasonably manage large forwarding lists is to create a

file for each BBS that you forward to.  The file will take its name

from the BBS that you will be forwarding to.  For example, to

establish the list of @BBS items to forward to WB8BII, you would

create a file called WB8BII.FWD.  To help manage the lists, you would

to set up a C:/MSYS/FWD directory.  In this example,  the file will

be: C:/MSYS/FWD/WB8BII.FWD.  In this file, you will list all of the

different @BBS and hierarchical addresses you wish to forward.

Example:

File entry     Comments (NOT IN THE FILE!)

WB8BII         Mail with an @BBS of WB8BII

KA8Z           Mail with an @BBS of KA8Z

OKIPN          Mail with an @BBS of OKIPN

NEOH           Mail with an @BBS of NEOH

NTSKY          Mail with an @BBS of NTSKY

ALL*           Mail with an @BBS of ALL(anything)

"#NEOH         Mail with a HIERARCHICAL ADDRESS of .NEOH

OH             Mail with a HIERARCHICAL ADDRESS of OH

EU             Mail with a HIERARCHICAL ADDRESS of EU (Europe)

442*           Mail to ZIP CODE 442xx

The last entry in the table MUST have a carriage return after it.

There should be NO blank lines in the file, INCLUDING AT THE END OF

THE FILE!

It is suggested that perhaps @BBS and hierarchical entries be listed

in the file ahead of the more general ALL-type entries.  In the event

a logical connection is lost during a forward cycle, at least some of

the personal mail will be forwarded ahead of the less urgent "for

sale-rusty tower" messages.

If you wish WB8BII to be able to REVERSE FORWARD (i.e. connect TO you

and pull the messages listed, rename the file to:  WB8BII.REV.  This

will permit MSYS then to examine the file and to forward any messages

and bulletins to WB8BII upon request.  Don't forget, before any

bulletins can be forwarded to WB8BII, it must be assigned a number in

the MSYSBBSB.DAT file!

MSYS 1.16                                                   Page 111

To make the above file work, your MSYSFWD.DEF would look like this:

(previous entry)

---------

F0 WB8BII

$C:/MSYS/FWD/WB8BII.REV

---------

(next entry)

When you run the MERge command, the contents of the

C:/MSYS/FWD/WB8BII.REV file will be added to the contents of the

MSYSFWD.DEF file to get a combined file listing the routing to WB8BII

and all of the types of messages to be forwarded.  Because the file

is in the /FWD directory, and because it has a .REV extension, if

WB8BII should connect to you, your station will forward those items

as well.

"Reverse" forwarding is very beneficial to a busy "hub" bbs as it

will help move the mail.  It is not uncommon in a metropolitan area

to be forwarding to one bbs while several others are reverse

forwarding from you, provided the network capacity is available to

handle the traffic.

MERge: (MUST DO THIS IN ORDER TO FORWARD MAIL)

This sysop command reads MSYSFWD.DEF and copies it to MSYS.FWD,

expanding any include files ($filename).  A file is also produced,

MSYS.BBS that is a sorted list of all possible BBSs that can be

forwarded to (including things like ARRL, 44*, etc.).  This command

is for 2 reasons:  if there were any include files in the MSYS.FWD

files they did not get read when a pf command was used and secondly,

the new file is needed for hierarchical forwarding.  The leftmost

part of the hierarchical route only is used for forwarding and the

contents of the file is needed for efficient processing.  Any changes

you make to your forward file must be done to MSYSFWD.DEF followed by

use of the MERge command (IN THE BBS).

This command can NOT be done while forwarding is active.

MSYS 1.16                                                   Page 112

CALLSIGN @ BBS REPLACEMENT

This feature is used to change the routing or direction of a message.

It can be used to redirect traffic for a certain ZIP code to a

specific bbs that has capabilities for handling it, or it may be used

to redirect messages from one LAN to another.

Create a file called MSYS.REP.  In it place lines with the following

format

OLDCALL OLD@BBS   NEWCALL NEW@BBS

For example:

ALL  ALLONT   ALL  NEOH

(messages  to: ALL@ALLONT  become ALL@NEOH)

44* NTSOH @1 ---

(messages to 44*@NTSOH    become originaltocall@ ---)

NTS* * @2 @1

(messages coming in to NTS-anything get tocall and @bbs swapped)

***  IMPORTANT  *** There can be no more than 150 lines in this file!

The algorithm goes like this: When a message comes in, a check is

made comparing the to call and @bbs of the message with the first two

fields of the records in MSYS.REP.  If a match is found (wild cards

are allowed in MSYS.REP) then the last 2 fields are used to replace

the tocall and @bbs of the message.  @1 or @2 in the last two fields

in MSYS.REP have a special meaning.  @1 means the original to call

and @2 means the original @bbs.  Note that wildcards are not expanded

in these last 2 fields.  To  support this facility a new command is

also added: REPlace.  If an argument of L is specified, the current

replacement list is displayed.  If no argument is given, the current

contents of MSYS.REP are read and stored in memory.  (This is done

when  MSYS is started also).  Use the REP command after you have

edited MSYS.REP to make the changes effective.

MSYS 1.16                                                   Page 113

REVERSE FORWARDING

MSYS can respond with messages to a connected BBS when it gives F>.

To use this facility, you must have a directory C:\MSYS\FWD that

contains files for each of the bbs to which you wish to reverse

forward.  These files will be named <requesting_bbs's_call_sign>.REV.

These files contain the .@BBS values that can be reverse forwarded to

the requesting station.

So you may want to set up your forward file as follows:

[file MSYSFWD.DEF]:

F0 W8XYZ

$c:\msys\fwd\w8xyz.rev

--------

F0 K8XXX

$fwd/k8xxx.dat

--------

The file C:\MSYS\W8XYZ.REV could look like this:

W8XYZ

ARRL

ALLUS

The file C:\MSYS\FWD\K8XXX.DAT could look like this:

K8XXX

K9XYZ

ARRL

With these example files, reverse forwarding would be enabled for

W8XYZ but not K8XXX.  Recall that $ lines in the forward file

MSYSFWD.DEF cause the given file to be included at that point.

Also remember to be sure to use the MERge command  whenever  you make

ANY changes to the forward files!  Reverse forwarding of a given

message is not possible until there has been at least one normal

forwarding (either forced manually with the FOrward command or at

your forward time).  The other half of reverse forwarding (where MSYS

would send F> when it is done forwarding messages to a particular

station) is enabled with the R in  the first line of the forwarding

file (F0 becomes R0 WA8BXN).

When done forwarding MSYS just disconnects and does not send ***

done.

MSYS 1.16                                                   Page 114

Reverse forwarding checks if the bbs is currently connected.  If so

MSYS will not request reverse forwarding.

Reverse forwarding POLL (connects and requests reverse forwarding

even if you don't have anything yourself to forward) can be

implemented - EXCEPT when using FBB compatible forwarding!  Simply

place P where you had R or F before.  Thus: F0 WA8BXN becomes P0

WA8BXN

A check is made to see if asking for another message to be reverse

forwarded will overfill the maximum number of messages.

Reverse forwarding will check if the required number of attempts at

forwarding a message have occurred before reverse forwarding.  Thus

if we have OH 25 in the reverse forwarding file, a message that

hasn't undergone 25 forwarding attempts will not be reverse forwarded

by this entry .  NOTE that selective title forwarding is ignored

during reverse forwarding.

SELECTIVE FORWARDING:

Selective forwarding by title contents is limited to ONE selection

per route.  Thus if you have both ARRL LETTER and ARRL PFB in your

forward file for a given bbs,  only bulletins addressed @ARRL with

LETTER in their titles will be forwarded.  No other bulletins with

@ARRL (including those with PFB in their titles) will be forwarded.

In addition to selective forwarding title comparisons, two additional

selection criteria may be specified: number of forward attempts that

must be made before using this path or maximum message size to be

forwarded using this path.  Here some examples:

F0 KA0FPO

ARRL

NEOH  DX

KQ8M  10

NEOH  '15

OKIPN  <3000

OKIPN  '<I

-------

MSYS 1.16                                                   Page 115

This set of lines will cause the following to happen.  All @ARRL

messages will be forwarded.  Messages with @NEOH will be forwarded if

they contain DX (upper and/or lower case) in the title.  Messages

with @KQ8M will be forwarded if type is PN and there have been at

least 10 forwarding cycles with the message present on the BBS.

Messages with @NEOH will be forwarded if there is 15 somewhere in the

title.  The single quote mark is used to indicate that the number

following is not the number of attempts that must be made to forward

the message by other means.  Messages with @OKIPN that are less than

3000 bytes long will be forwarded.  And finally messages with @OKIPN

and <I in the title (again, case ignored) will be forwarded.  The

ability to not use a given forwarding possibility until there have

been the specified number of forwarding attempts is included so that

preference can be given to favored paths while still allowing

alternate paths to be automatically used if the favored ones fail.

Limits on message size have been implemented for those who may

forward to mini-BBS systems with limited storage capacity.

TCP/IP FORWARDING:

TO forward to VE3GKM-4's TCP/IP mailbox the file would look  like

the following.

F0 VE3GKM-4

@VE3GKM-4

.

+NET (may not be needed)

VE3GKM

(MORE @BBS CALLS)

---------------

MSYS 1.16                                                   Page 116

SMTP FORWARDING

Sample entry in MSYSFWD.DEF:

T 44 70 4 10   (DO NOT USE DOTS IN THE ADDRESS)

N8HSP

-----

Note that no port number is specified, it does an arp if needed.

Also note no digis (put them in the arp table if you have to have

them).  No nodes.  Just T and the IP address of the DEStination

station as shown.  This corresponds to the line like F0 N8HSP.  Next

comes the @BBS values to send to this station.  There can be more

than one.  Then the ------ that ends this entry.  The same forward

file is used for both normal  forwarding and tcp/ip stuff.  I have

also made some changes to the SMTP command.  After you use it, if you

type PRompt it will prompt you for To: (enter call@bbs) Reply-to:

(just hit return if you like the default given) Subject: (this is in

the body of the text, not the title) Then you enter your text and end

with.  (not ^Z) as instructed.  The result is a message that contains

all the fancy extra lines often found in tcp/ip messages.

Bulletins may be forwarded using SMTP.  To do so you must assign the

SMTP station an entry in the MSYSBBSB.DAT file like you would for

normal forwarding (don't use any SSIDs).  Also add the callsign of

the station in the T line in the forward file after the IP address.

Example: T 44 70 4 5 K8EIW  This should be done in the forwarding

file for all SMTP forwarding.

Bulletins are accepted from SMTP (a bulletin goes to a non-callsign).

BIDs are made up based on your message number and callsign.

Hierarchical addresses are added to msgs that come in via SMTP.

NOTE: Since BIDs are not supported in this implementation of SMTP,

bulletins can either be sent to or received from TCP/IP systems but

not both ways.

MSYS will accept <RCPT TO>: lines in either the format a%b@c or b!a@c

where c is your bbs call, a is the destination stations call and b is

the destinations bbs.  Both of these get transformed into a@b on the

MSYS message list.  The first form (the one with the %) is newly

added in 1.10 to help out in what seems to be a difference of opinion

as to the correct way to do it.

MSYS 1.16                                                   Page 117

FORWARDING NOTES

Make sure you don't have any blank lines in your various forward

files (MSYSFWD.DEF and the files in the FWD directory).

Executing commands from within the forwarding file.  In the

forwarding file at the beginning of a set of forwarding lines (i.e.,

at the beginning of the file or after a ------ line) you can place

any sysop command(s) by simply putting \ in front of them.

The MSYS.BBS file contains a number after each "BBS" that the system

knows how to forward to.  This is the ORing of all the BBS Masks that

can be used to forward the particular @BBS.  This is used to

determine when bulletins have been sent to all stations that should

be getting them; when such a bulletin is found, it gets its type

changed from $ to # so then it is no longer considered for

forwarding.

Within the forward file a !timestring line may be placed before any

callsign to limit forwarding hours for that callsign (or bulletin

route).  The first ! line as used before limits all times for a bbs

entry you forward to.  The following ! lines remain in effect until

changed by another ! line.  When you start another bbs forward entry

group the forwarding hour defaults to 0-23.  At this point I would

like a little example of what this means, so here it is (the comments

and blank lines would NOT really be in the forward file):

F0 WA8BXN this is the beginning of a forward group}

!0-16,19-23    {don't initiate forwarding during hours of 5 or 6

P.M.}

WA8BXN         {forward the @BBS any time we connect}

!0-16,22-23    {don't forward the following during early evening)

NTSOH

OH

!1-4      {forward following only in early morning}

ARRL

ALLOH

-----

BID:

When a message comes in to non-callsign without a bid a check is made

with generated BIDs of msg#_callsign and callsign_msg# to eliminate

duplicates.

Messages addressed to other than a what looks like a callsign

automatically get a BID generated for them.

Mail forwarded to BBSs identifying with either [MBL or [..  - $  ]

get BID included in S command.

MSYS 1.16                                                   Page 118

FORWARDING DISPLAY

As each of the BBSs to which msgs can be forwarded is scanned for a

given bbs you will see it.  When a message is being forwarded you

will see a line that gives its number and the to station and @bbs of

the message.  In parenthesis you will see the entry in the forward

file that is being used.  Thus if the forwarding file contained.

F0 WA8BXN

WA8BXN

44*

---------

and there was a message for K8ZZZ@44123 being forwarded you would see

something like:

Fwding msg # 123 K8ZZZ@44123(44*). QSLDX @BBS:

Messages  that arrive with @BBS of IDCALL get the @BBS field changed

to --- .  This makes them distinctive

Messages sent to REQQTH, REQDIR and REQFIL are left private if they

arrive that way.

***  There can only be up to 400 different bbs names in the

forwarding file.  To check this do a LCount on the file MSYS.BBS.

Wildcards (where you can use *) have been enhanced to allow more than

just using *.  Based upon the recommendations of AA4RE and others,

the following have been implemented:

* Matches 0 or more occurrences of anything

@ a..z, A..Z

# 0..9

+ a..z, A..Z, 0..9

? a..f, A..F, 0..9 (hex digits)

< a..z

> A..Z

$ =@#+?<>$     (any one of those characters, but I don't have any

guesses as to why this group was included)

= any one character

" the escape character: the next character must match exactly

( beginning of group . (abc) matches a b or c.

Examples: 9v1A2q   matches #@#@#@

95===  matches anything 5 characters long beginning

with 95

(WK)6   matches anything starting with W6 or K6 (case

is important)

95###   matches any 5 digi number beginning with 95

MSYS 1.16                                                   Page 119

HF FORWARDING

HF Forwarding regulations do not allow transmitting 3rd party traffic

unless there is a control operator present, the station is under

remote control, or a STA has been issued.  To aid in compliance with

the regulations, the ATtended command has been added.  When set to ON

all forwarding is allowed.  If off, ports with radio speed of 300

will forward only those messages in which the from call is the idcall

(messages that you originate) or the to station in the message is the

station you would directly connect to for forwarding.  HF reverse

forwarding is also inhibited if ATtended is OFF.  Note that the

regulations appear to allow you to receive messages unattended.  So

if you don't have a STA for HF turn AT OFF when you aren't there and

remember to turn it ON when you are.  To be on the safe side, put

ATtended OFF in your MSYS.DO file should the system reboot when you

aren't present.

MSYS NODe Forwarding

You can forward directly through your own MSYS node (assuming  it is

properly activated).  Here are some examples how to do it:

F0 W8XYZ  This is the station we are forwarding to, the number

%W8XYZ    after F is not important Then do a network connect to W8XYZ

- assuming  W8XYZ is a BBS that is in our local node

destination list

W8XYZ     stuff to forward to this bbs

-------

F0 W8XYZ

%W8XXX    {connect to node W8XXX which is in our local dest list}

+to

?busy

W8XYZ     stuff to forward to this bbs

-------

NOTE:  There should be NO BLANK LINES in this file!

MSYS 1.16                                                   Page 120

PACTOR FORWARDING

Automated forwarding should not be done using PACTOR. However,

reverse forwarding can move traffic off a remote PBBS.  A

MSYSFWD.DEF-like file is made with a filename of your choice.  This

will serve as a manual forwarding file.

You can not have the $filename.ext type of include file.

The following is an example of a proper file:

p0 no8m

wa8bxn

wb8bii

k8eiw

-----

The above causes a connect to NO8M and sends traffic for the three

listed PBBS stations.  A reverse forward prompt then results in

traffic for the connecting station to be reverse forwarded.

When you wish to manually forward, enter the following command from

the sysop keyboard:

FP filename.ext

MSYS then acts on your file and forwards via the PACTOR port.  MSYS

can have a "normal" forwarding session in progress at the same time

as it is forwarding via PACTOR.

DO NOT ENABLE DIGI ON YOUR PACTOR PORT!!

DON'T FORGET TO ENABLE CW ID IN YOUR PACTOR TNC!!

MISSING TEXT TO Message

If the text for a message is not available when it is being

forwarded, the forwarding is aborted instead of sending out a missing

text message for the text.  The STatus command will now indicate how

many times this has occurred and the last message number for which it

happened.  The MISsing command may also be used to determine how many

times this occurred without generating all of the other STatus

output.

MSYS 1.16                                                   Page 121

FBB COMPATIBLE FORWARDING

MSYS follows the standard established by F6FBB in his version 5.14

and 5.15 of his BBS software.

As described in the FBB manual, once the F letter in the SID (System

type IDentifier) has been detected, all command lines must start in

the first column with the 'F' character.  All command lines are

terminated by a return (CR) character.  When the connection to

another bbs using the FBB protocol has been made and the 'F' flag has

been received, the SID of the local station is sent with the first

proposal.  The proposal may look something like this:

FB P WA8BXN WB8BII K8EIW 12345_WA8BXN 765

F>

Where:

FB             Type of command (proposal)

P              Type of message (P = personal, B = bulletin)

WA8BXN         Sender (From field)

WB8BII         BBS of recipient (@BBS)

K8EIW          Recipient (To field)

12345_WA8BXN   BID or MID

765            Size of message in bytes

F>             End of proposal

In the case of multiple message proposals, the 'FB' line is sent for

each message proposed followed by one F>.

Upon receiving the proposals, the other BBS will accept or reject the

message with the command line:

FS -+=

where

-    means No, already have that message

+    means Yes, send the message

=    assumed by MSYS to be No, message being forwarded by another

system.

The FS line must have as many +,- signs as  FB lines in the proposal.

Each message is sent with the title on the first line, the text, and

a control Z in the last line.  There are NO blank lines between

messages.

When the other BBS has received all the messages in the group, it

acknowledges by sending its proposal and the direction of forwarding

is reversed.

MSYS 1.16                                                   Page 122

If the receiving bbs has no further messages to send, it responds

with a:   FF

This line must NOT be followed by a F>.

If the sending BBS has no further messages, it responds with a:

FQ

and then disconnects.

FBB COMPATIBLE COMPRESSED FORWARDING

The protocol utilized for the transfer of compressed messages is an

extension to the standard FBB protocol.  Compressed forwarding is

validated by the presence of the letter B in the SID [MSYS-1.14-

BFHM$].

In the case of compressed forwarding, the proposal line begins with

a:

FA

FA means that the transfer will be an ASCII compressed message.

The proposal to send a message is in the form:

FA P WA8BXN WB8BII K8EIW 12345_WA8BXN 6789

Compressed message transfer is done in a binary mode, which is

derived from the YAPP protocol.  All transfers consist of a header, a

block of data, an end of message and a checksum.  Each transfer is

equivalent to the transfer of one message of the standard protocol,

but is not followed by a control Z.  The end of file specifier is

defined by the transmission of a EOT character followed by a 1 byte

checksum..

Format of the header for a compressed message:

<SOH> 1 byte = 01 hex

Length of the header 1 byte = Length from the title,

Title of the message 1 to 80 bytes

<NUL> 1 byte = 00 hex

Offset 1 to 6 bytes

<NUL> 1 byte = 00 hex

To follow the FBB format, the title of the message is transmitted in

ASCII, not compressed.

MSYS 1.16                                                   Page 123

The offset is also transmitted in ASCII and specifies the offset at

which the data should be inserted in the file (in case of a

fragmented file).  A data block contains from one to 256 bytes.  It

begins by two bytes which specify the format:

<STX> 1 byte = 02 hex

Number of data 1 byte = 00 to FF hex. (00 if length = 256

bytes).

Data bytes 1 to 256 bytes

The last data block is followed by the end of transmission character

(EOT) and the checksum in the format:

<EOT> 1 byte = 04 hex

Checksum 1 byte = 00 to FF hex

The checksum is equal to the sum of all the data bytes of the

transmitted file, modulo 256 (8 bits) and then two's complemented.

The checking of the checksum is very simple:

The sum of the data from the file and the checksum received

modulo    256 (anded with FF) shall be equal to zero.

In case of a checksum error, the message is not accepted and the

system sends the comment

*** Checksum error

and issues a disconnect request.

The transmission sequence is repeated for each message in the group.

MSYS 1.16                                                   Page 124

W0RLI STANDARD FOR DATA COMPRESSION DURING THE FORWARDING OF MESSAGES

IN THE AMATEUR RADIO BBS NETWORK

ihave/iwant variable batch size protocol

Feature letter X, commands SS, SX, SY

LZH compression

--------------------------------------------------------------------

Command Definitions and BNF description.

--------------------------------------------------------------------

N              = INTEGER, size 32 bits.

protocol       = TBD

batch_size     = N

file_size      = N

msg_count      = <0-25>

id             = BID | MID EOL

ids            = id | ids id

byte           = <0x00 - 0xff>

bytes          = byte | bytes byte

ihave = ``                SX'' msg_count [`` '' protocol] [``                                                   '' batch_size] EOL [ids]

iwant = ``                SY'' msg_count EOL [ids]

isend = ``                SS'' file_size EOL bytes

For ihave:

Protocol and batch_size not used in initial implementation.

Default protocol is : LZH <C source provided>

For iwant:

SY means want only there.

--------------------------------------------------------------------

The basic interchange (example):

--------------------------------------------------------------------

bn are BIDs (or MID if message has no BID).

m: is master, s: is slave.

m: SX 5

m: b1

m: b2

m: b3

m: b4

m: b5

s: SY 3

s: b1

s: b3

s: b5

MSYS 1.16                                                   Page 125

m: SS 26545

m: <sends 26545 byte file>

In the case of any error, the station noticing the error must

disconnect.

--------------------------------------------------------------------

With more detail of one implementation (MB/SERVER processing)

--------------------------------------------------------------------

From Sending side:

MB:

Scan messages to find batch not larger than max. batch size.

Keep record numbers and cc: id in list.

Send ihave.

Receive iwant.

Mark all messages in ihave and not in iwant as forwarded (dup bid)

Pass iwant list to SERVER in file.

Wait for server acknowledge.

SERVER:

Create export file.

Compress export file.

Signal MB that compressed file is ready to send.

MB:

Send isend, send the file.

Receive prompt.

Mark all messages in iwant as forwarded.

From the receiving side:

MB:

Receive ihave.

Check BIDs in ihave.

for all non-dup BIDs, make iwant list.

Send iwant.

Receive compressed file.

Signal SERVER import file available.

Send prompt.

SERVER:

Decompress file, import messages.

MSYS 1.16                                                   Page 126

HIERARCHICAL FORWARDING and MSYS

There are several features in MSYS that combined with hierarchical

forwarding should almost completely automate routing of incoming

messages.  These features act at two separate times: first when a

message comes in and second during forwarding.

First let's review the way hierarchical forwarding works.  With

hierarchical forwarding the @BBS field can contain more than just the

callsign of the BBS that is the DEStination of the message.  Routing

information can also be placed in this field.  The @BBS field thus

has the syntax of part1.part2.part3.  ....  When it is time to

forward a message, the @BBS field is inspected and the leftmost part

that is found in the forwarding file is used to forward the message.

Consider as an example @N8PIH.MI.  If N8PIH is found in the

forwarding file then it is forwarded that way and the MI part is

ignored.  On the other hand, if N8PIH is not found in the forwarding

file, then if MI is found in the forward file then it is forwarded

that way.  If neither part is found in the forwarding file, then the

message will not go anywhere.  Such messages will show up (after at

least one forwarding attempt after the last time MSYS was started) if

you do a L? command.

If all messages had both the actual callsign of the DEStination BBS

and the appropriate xx part in the @BBS field then the forwarding

file could be greatly shortened.  You would need to have the calls of

the BBS stations in your state in the forward file, along with each

of the xx designators for the remaining states.  If there were say 50

BBS systems in your state then perhaps 99 different entries would be

sufficient in your forward file (ignoring for the moment bulletin

routes like ARRL).

NOTE that the limit of 400 different calls in the forward file is

more than sufficient!  Messages that are going to a BBS in your state

get routed as specified in your forwarding file; those going to

another state get routed to the BBS you have set up in the forwarding

file to get stuff for that state.  You could include a few explicit

calls for BBS systems in nearby states that you route directly to, if

you want to override the normal path for messages going to that state

if you so desire.

Now for the second part, the action taken by MSYS when a new message

arrives.  When a message comes in, MSYS looks at the @BBS field.  If

it is absent, then if the to callsign field contains something that

MSYS knows the routing for, the to field is copied to the @BBS field.

Otherwise the @BBS field is set to ===== and the message won't go

anywhere.  It will show up as noted above in a L? command.

MSYS 1.16                                                   Page 127

For forwarding NTS traffic, MSYS uses a file called BBSTONTS.BIN that

contains BBS callsigns and the states they belong in (this

information comes from several sources, more on this later).  Next

MSYS inspects the @BBS field to determine if it knows how to forward

what is in the given @BBS field.  If some part of the possibly

hierarchical specification is found in the forwarding file, it is

left alone and will be forwarded by that matching specification.  If

no match is found in the forwarding file, then the BBSTONTS.BIN file

is searched to find some part of the given @BBS field.  If a match is

found, the corresponding NTSxx is appended to the existing @BBS

field.  Thus MSYS will attempt to lookup the right state to send the

message to.  Assuming the state is found, then forwarding will take

place as described above (see the N8PIH.MI example).  If no part of

the @BBS field can be recognized, then the message will not be sent

out, but will show up when you do a L? command as described above.

The BBSTONTS.BIN look up will also handle zip codes given in the @BBS

field, making @12345 into @12345.xx as is appropriate.

Thus far we have examined what happens when in the process of

automating forwarding.  In the event the proper routing cannot be

determined by MSYS then you will have to use the Sysop EM (edit

message) command to manually add the appropriate .xx designation,

assuming you can guess it somehow.  If you are lucky, the originator

put useful information in the title of the message or may have hidden

it in the text of the message.  Maybe the callbook will be of some

use to you.  Or maybe you can just force it in the opposite direction

from which it arrived.

It needs to be emphasized that MSYS will forward on the leftmost

element matched in the @BBS ONLY.  For example, if the @BBS of a

message is KA8Z.#NEOH.OH.USA and there is an entry in the WB8BII.REV

file for file for KA8Z, and the path to WB8BII is lost, the message

will NOT alternate route to the #NEOH path.

MSYS 1.16                                                   Page 128

Creating the BBSTONTS.BIN file.

This file contains binary data and cannot easily be created with a

text editor.  Mutil function 14 takes as its input a file called

BBSTONTS.DAT and produces as output BBSTONTS.BIN.  To create

BBSTONTS.DAT, you run a function called BBSTONTS (function 13 in

MUTIL) which takes as its input BBSLIST.DAT.  BBSLIST.DAT is created

and/or updated using data from message headers that have passed

through your system and/or AD8I BBS lists.  Function 10 of MUTIL is

used to use messages as input; function 11 uses AD8I lists as input.

Lets look at a picture of these files and programs:

+---------------+

| Message files |

| from MAIL.BAK |

+---------------+

|

V

MUTIL function 10

|

+-------------+

| BBSLIST.DAT |

|  file    |

+-------------+

|

|

MUTIL function 13  <--- Other standard files:

|          STATES.DAT

|          ZIPCODES.DAT

V          ROUTES.DAT

|

|

+-------------+

| BBSTONTS.DAT|

|   file      |

+-------------+

|

|

MUTIL function 14

|

V

|

+-------------+

| BBSTONTS.BIN|

|   file      |

+-------------+

MSYS 1.16                                                   Page 129

HOUSECLEANING (HCL command)

MSYS.HCL

You should create a file called MSYS.HCL into which you put lines

describing classes of messages you want to eradicate.  Each line

should contain one or more of the following keywords:

AT= TO= FROM= AGE= TITLE= TYPE= KEEP= SYSOP=

So for example:

AT=ARRL AGE=30

would delete all messages with @BBS of ARRL that are over 30 days

old.  If specified, the values of AT TO and FROM must match exactly

for a message to be deleted.  TITLE and TYPE if specified must match

some part of their corresponding message header fields; case is

ignored.

KEEP can be used to specify a number of msgs that match the other

values given that should be saved anyway.

Example:

TO=ALL AT=ARRL TITLE=PFB KEEP=2

Would keep the newest two messages to ALL@ARRL with PFB somewhere in

their title.  NOTE:  The value of title given cannot contain blanks.

DIR is used to specify a Directory you wish to store a message into

before killing.  For Example:

TITLE=CD-ROM age=3 DIR=FILES\HARDWARE

Will store any file with "CD-ROM" in the title that is over 3 days

old in the file MSYS/HARDWARE

The title of the message will be the BID number.  This will probably

require some Sysop editing to change the title to something more

meaningful.

SYSOP=Y has to be specified in any line that you want to delete

messages that are to the idcall of the BBS.  Thus to delete all read

messages (including one to you) you could have a line:

TYPE=PY SYSOP=Y

The following line deletes read messages, except those to you:

TYPE=PY

MSYS 1.16                                                   Page 130

The effects of these lines can be far reaching.  For example, if you

put in the line:

KEEP=5

Then it will delete all but the 5 newest messages!  All this action

takes place when you type:

HCLean NOW

or at a time specified as a numeric parameter to HCLean.  HCL 120

would cause it to happen at 02:00 (number given is minutes after

00:00 [midnight] to begin housecleaning).

Please note that BID= is NOT one of the items that is used to select

messages to be killed in the MSYS.HCL file.

MSYS.HCL file as an example:

at=arrl age=21

at=okipn age=10 a

to=panet age=3 type=#

to=mods age=3 type=# dir=files/mods

at=alloh age=10 type=#

at=allbbs age=7 type=#

at=amsat age=7 title=schedule dir=files/amsat

at=amsat age=7 type=#

at=midnet age=3 type=#

at=arrl title=pfb keep=1 type=#

at=arrl title=dx keep=1 type=#

at=arrl title=kep keep=1

type=# title=sale age=2

type=# title=need age=3

type=# age=35

type=py keep=1

type=pf keep=2

type=tn age=21

type=pn age=60

to=sysop age=7 type=#

At the end of housecleaning, the file MSYSHCL.DO is done (as a DO

file) if it exists.

MSYS 1.16                                                   Page 131

MSYS NETWORK NODe INTRODUCTION

Some basic information concerning the implementation of the MSYS NODe

interface will help in making decisions related to configuration.

Two terms must be understood by the system administrator (SYSOP).

These terms are neighbor NODe and DEStination NODe.  A neighbor NODe

is a compatible NODe to which there is a direct connection path.

MSYS does not support use of digipeaters in the path to neighbor

nodes.  Normal AX.25 connections are made to neighbor nodes.  The

data exchanged by these connections consists of a mixture of

supervisory and data frames that together support the network

operation.  DEStination nodes are also network nodes and are the

nodes listed when one does a Nodes command.  DEStination nodes are

nodes that can be reached through the network either directly if they

are also neighbor nodes, or indirectly through some neighbor NODe.

Routing

Routing of frames in the network is conceptually fairly simple,

handling all the details is not!  Each NODe does a "Nodes Broadcast"

periodically (usually every 30 minutes) which is a list of nodes that

it knows how to route to.  This may be due to directly hearing a

neighbor NODe or by hearing the nodes list from neighbor NODe that

claims to have a route to each NODe it lists in its NODe broadcast.

Information from the nodes broadcast is maintained by MSYS.  When it

receives a frame that is not for itself, MSYS looks to see which

neighbor NODe can handle the DEStination indicated in the frame and

simply passes the frame on to some neighbor NODe.

Data frames while they are inside the network as handled as what are

termed "datagrams".  What this means is that during a conversation

between two end users there is no guarantee by the network that the

data frames will always go by the same path or that they will make it

through the network.  Depending on conditions in the network the path

taken by frames can change and at times frames will be arbitrarily

discarded.  The end nodes (the ones the users connect to) are

responsible for error free end to end services.  While the AX.25

protocol insures that transmission of frame between any two nodes is

error free, the network protocol establishes error free operation

between users.

MSYS network NODe functions include both the routing of frames within

the network as well as the uplink/link logic to support the error

free end to end services.  Additionally, the TCP/IP servers can make

use of the network as an alternative to the existing IP routing

facilities.  Connections from the network may be made directly to the

BBS through the enhanced NODe command set.

MSYS 1.16                                                   Page 132

An attempt has been made to make the MSYS network NODe user friendly

while still retaining compatibility with other network nodes.  To

this end there is a help command and headings on output generated by

various information listing commands.  Since MSYS is generally run at

an attended site, a Talk command is available to allow users to talk

to the Sysop directly from the network.

Basic Decisions About the Network NODe

The really big question is should you enable the network NODe

functions of MSYS? Fortunately, you have more options than just yes

or no! So perhaps the question should be reworded as should you run

the NODe in full feature, fully automated mode?  The answer for most

stations will be NO.  The answer in your particular case will depend

on the capabilities of your station and the locations of other

compatible nodes.  If you run a high profile station already (wide

coverage as a digi/K-NODe) and have compatible network nodes at

reasonable distances from you (maybe at least 30 miles away), then

you may fill a need by running with everything enabled.  On the other

hand, if the only way you can talk to anything is through a

digipeater (even when the batteries in your ht are fully charged) and

on a hazy day you can still see the antennas of at least five network

nodes, all still forwarding the same frame they received 2 years ago,

then may I suggest you run in a very controlled minimal mode.  Make

friends, not enemies through these enhancements to MSYS!

Network NODe Quickstart

This section tells the minimal things you must do to get the NODe up

and running in some (probably not optimal) fashion.  You MUST define

your network NODe callsign using the NRC command.  If you are also

active in TCP/IP you should set NRCall and MYTcpcall to be the same

thing.  You MUST use the NPorts Command to enable the ports that you

want to be able to access the NODe.  For example, NP 3 would allow

access to the NODe on ports 0 and 1 but no others.  You SHOULD define

your network NODe mnemonic identifier using the NRI command.  You

MUST enable the NODe using the NETNode ON command.  You should

probably turn AUTONode and AUTODESt ON and set OCM to 3.

This should put your system into full service automatic mode as far

as the network NODe is concerned.  It probably will not be the way

you will want to run but its a starting point.

MSYS 1.16                                                   Page 133

Manual Operations on the Network NODe Database

To REMove a DEStination NODe completely, use

REMove DEStination nodecall

To REMove a DEStination for a given neighbor NODe, use

DESt DELete DEStcall neighborcall

To REMove a neighbor NODe (and all  the  DEStination  routes through

this                          neighbor NODe), use:

NODe DELete nodecall port#

To add a new neighbor NODe, use

NODe ADD nodecall port# quality [digicall1 .. .  digicall8]

Note:  The mnemonic identifier for the neighbor NODe will be obtained

from the first nodes broadcast heard from the NODe.

Use of the digipeater calls is optional.  Although you can specify up

to 8 digipeaters, rarely will more than 1 or 2 be practical to use.

To add a new DEStination for a given neighbor NODe, use

DESt ADD [destid:]destcall neighborcall neighborport obs#

quality#

Notes:

The neighbor NODe callsign must be defined before any

DEStinations can be added for it.

destid or alias for the DEStination is optional.

obs# is the obsolescence count, 0 locks the DESt in for the

NODe.

To change the quality of an existing neighbor NODe, use:

NODe REPlace callsign port# newquality [digi1 ... digi8]

To change the obsolescence count and quality of a DEStination through

some neighbor NODe, use:

DESt REPlace DEStcall neighborcall neighborport newobs#

newquality

MSYS 1.16                                                   Page 134

One Possible Network NODe Setup

A NETNode can be run in a moderate mode that is somewhere between

fully automatic and low profile.  Here is some basic info that chould

be used: Computer is an old original IBM PC 4.77 MHz clock 640 K ram,

two 10 M hard drives, 4 serial cards and a printer card.

Port Freq.

0  223.7     High profile (150 W, beam at 80 ft .  aimed west)

1  145.01    Quite high profile (200 W omni ant at 90 ft.)

2  145.05    Low profile (10 W omni ant at 60 ft.)

3  80 M

4  40 M

5  20 M

6  serial connection to another computer

Nearby existing NetRom nodes:

Port Call           Distance

0  WB8CQR-10      15 (Miles)

1  WB8CQR-1       15

1  K8EIW-1        45

1  W8QLY          75

1  W3LIF          80

2  KB8AST-1       25

Nearby NOS stations using NetRom interface:

Port Call           Distance

1  N8HSP-4        10

1  WB8LYJ-4       15

All of these stations are usually reliable connections.  Other NetRom

nodes are usually heard but are not always reliable.  During band

openings many more are heard, some are reliable during the band

opening.

MSYS 1.16                                                   Page 135

NODE RELATED PARMS in MSYS.DO

Here are the network NODe related parameters I have in my MSYS.DO

file:

OCM 3          Transmit DESt nodes heard that have obsolescence

counts  3 to 6

NRI 460406     Right 3 bytes of my IP address [44.70.4.6] expressed

in   hex  (quite useful this way)

NRC WA8BXN-2   Callsign net NODe responds to.  MUST be the same as

TCP/IP callsign (MYTcpcall)

NPorts $47     Network NODe uses ports 0,1,2 and 6 only

PQuality 100   Default port quality 100.  A low value assigned to

nodes not manually entered that connect to mine

NODe REStore   Read saved NODe file when MSYS starts

AUTONode OFF   Do NOT record NODe broadcasts from nodes not already

in neighbor lists.  This means that new nodes heard

(during band openings for example) will not be

automatically added, unless they do actually connect

to my NODe.

AUTODESt ON    DEStination NODe broadcasts (from nodes already in

my neighbor table) will be recorded.

NETNode ON     Enable network NODe

Here is the output I get from the JN Sysop command (or R command in

the NODe):

Port Neighbor NODe Call Quality   DESts  Heard    Digipeater(s)

1  AKR:K8EIW-1         179       66     23:43

1  CLE1:WB8CQR-1       180       67     23:41

0  CLE220:WB8CQR-10    180       67     23:34

2  PRYTWP:KB8AST-1     192        2     23:25

1  YNG:W8QLY           180        7     23:36

MSYS 1.16                                                   Page 136

The general philosophy I am using is that I only want to attempt to

connect to network nodes that I have a good path to.  These are also

the nodes that I broadcast myself (and the DEStinations they claim

they can handle).  I have manually entered these nodes into my

neighbor NODe table (using NODe ADD).  I see little sense in saying

that I can really connect to a NODe I hear once because a meteor hit

at the right time! This method does not particularly limit the number

of DEStination nodes my NODe knows about because the nearby nodes

claim they can handle everything they hear.  I just let the nearby

nodes worry about finding routes to everything they claim to hear.

My system serves as a link from the one isolated NODe on 145.05 LAN

frequency to the nodes on 145.01 which give access to the Ohio 4800

Baud UHF backbone frequency.  I also serve as an IP switch to and

from the network.  I have adjusted the neighbor NODe qualities to

favor nodes (primarily TCP/IP stations) to which I am a good route.

I have set most of the normal NetRom nodes to a lower quality value

so that they will talk directly to each other most of the time rather

than trying to go through my NODe.  Should they be unable to connect

directly to each other, they will still be able to go through my

system.

If you want to run very low profile in terms of getting used as a

network NODe, leave the default of 10 set for OCMin.  This means that

your NODe broadcasts will only send out your own call. This will get

the network to recognize you but not make it think you are a good

relay to anything.  Keep AUTOD ON so that your system will know how

to reach nodes in the network.  Its probably a good idea to set AUTON

OFF and manually add the calls of the best nearby nodes you can

connect to (one is all you need).

If you are totally confused about the network operations, try to

consult with who ever runs your local NODe(s).  Most of the

parameters you have, they have (with some changes in their names).

MSYS 1.16                                                   Page 137

Miscellaneous Comments on Network NODe

If you are low on SSIDs, you might consider the following.  Set the

IDCALL to your callsign without any SSID.  Do not define a BBS,

Keyboard, or K NODe callsign.  Pick an SSID and use your callsign

with it and set as the value for MYTcpcall, NRCall and FORWARD call

To get to your BBS the network NODe BBS command can be used.  To talk

to your keyboard the BBS T command or network NODe T command can be

used.  If you want to make outgoing calls you will have to define a

keyboard callsign.

Should you use the network NODe to route TCP/IP frames?  I would say

only if you have to!  The normal IP routing is better than using the

network NODe but in those cases where using the network is the only

way to reach some distant IP station then do use the network.

Network node connects are allowed only on those ports enabled by

NPorts command.

Kanode which lists KANodes that have been heard.  An optional

parameter may be given that specifies the port for which the list is

desired.

Justheard which lists stations recently heard.  Here again a port

number may be given as an argument.  The N command is now described

as Netnodes (lists destination network nodes known).

In the network node, using the C# form of the connect command

(example: C2 WA8BXN) will always result in a normal AX.25 connect,

even if the destination station is a network node.

The network node does not record information in nodes broadcasts

heard on ports that are not enabled for network node use (NP

command).

MSYS 1.16                                                   Page 138

NET NODe PARAMETERS SUMMARY

(PARM          Default   Comments)

PQuality [port#] # 192   Quality for nodes heard on port

BInterval seconds 3600   Nodes Broadcast interval

OCMin #            10    Minimum obsolescence to broadcast

OCInit #            6    Initial obsolescence count

WQupdate  #         1    Worst quality DESt route to record

NTtl #             64    Network time to live (max hops)

TTimeout seconds   60    Transport timeout (Net FRACK)

TREtries #          3    Transport retries

TACK sec            3    Transport ACK delay (RESPonsetime)

TBdelay seconds   180    Transport busy delay

NBdcast [port#]    ON    Enable or disable node broadcasts on a per

port basis

NDwindow #         4     Net default window size (frames)

NETCon            ON     Enable connect cmd in net NODe

NETBbs            ON     Enable BBS cmd in net NODe

NETTalk           ON     Enable Talk cmd in net NODe

NETXBatchsize [port] #   Sets max batch size for RLI compression on

port

NETXOption [port#] OFF   If on, enables RLI compressed fwding on port

AUTODESt         OFF     Add new DESts for known neighbors

AUTONode         OFF     Add new neighbors heard

NODe SEnd                Force known DESt transmission

NODe SAve                Write net NODe Database to disk

NODe REStore             Read net NODe Database from disk

NODe ADD call port qual [digis]    Adds neighbor NODe

NODe REP call port qual [digis]    Changes quality of neighbor NODe

NODe DEL call port       Deletes neighbor nodes & its DESts

DESt ADD [id:] destcall neighbor call neighborport obs# qual

Adds new DEStination

DESt REP destcall neighborcall neighborport obs# quality

Changes obs & quality

DESt DEL destcall neighborcall neighborport

Deletes DESt through given neighbor

REMove DESt callsign     Deletes DESt through all neighbors

NInfo                    Gives net NODe status

NRCall callsign          Sets net NODe callsign

NRId mnemonic            Sets net NODe mnemonic (alias) identifier

NPorts (MASK)            Ports to which nodes broadcast are made

TNatime seconds          Transport no activity timeout time

C BBS or C bbscallsign   Acts like BBS command taking you to

the bbs.  You also now get a "connected to"

message with either of these commands.

MSYS 1.16                                                   Page 139

A number of commands allow setting options for network connections.

Here are the network command names and their AX.25 port related

counterpart.

NETOnly        BPorts

NETRead        MAXRead

NETMax         BMaxcon

NETFBBStime    FBBStimes

NETFBBC        FBBC

NETFBBG        FBBG

NETXBatchsize  XBatchsize

NETXOption     XOption

MSYS 1.16                                                   Page 140

PACTOR

OVERVIEW

PACTOR operates somewhat like a enhanced AMTOR.  A series of

characters is received and an acknowledgement is sent by the

receiving station.  PACTOR has a full character set that make it look

like a typical packet connect.  In addition it has a number of

features that enhance its performance such as dual speeds, internal

compression and internal memories that enhance reception.

PACTOR does not share a channel.  It can not be used for automated

forwarding.  Our own experience utilizing the mode while camping in

very poor conditions shows that PACTOR is a far superior mode of

digital communication.

HARDWARE

MSYS supports only the KAM Plus or KAMs that are upgraded to KAM Plus

TNCs.  The KAM is the only TNC that gives separate commands to allow

you to seize the transmit direction and to go into receiving mode.

Other TNCs provides only toggles that lead to uncertainty about the

state of the function being toggled.

When used with MSYS, the KAM Plus can only be used as one port

dedicated to PACTOR.  You can not do KISS or access the VHF side of

it.

The CTS line must be present in the RS-232 cable.  The radio must be

capable of PACTOR operation.  Some older radios will not switch from

transmit to receive fast enough.  (Also some newer ones switching

between vfos or modes!)  Additional cooling will probably be

necessary.  In addition, a larger than normal power supply may be

necessary.  These are points which should not be overlooked.

500 Hertz receive filtering is useful.  Our own experience has shown

excellent results using minimal power into poor antennas when working

home.  25 watts into a mobile whip is more than adequate to a home

station running 100 watts to a dipole over a 400 mile path on 40

meters ... 24 hours a day.

PACTOR SET UP

The set up change is minimal.  The name of the port in the MSYS.OPT

file must begin with "PACTOR".  For example:

PACTOR_40

or

PACTOR_HF

would be appropriate.

MSYS 1.16                                                   Page 141

PACTOR TNC PARAMETERS

The following KAM commands must be given to set the KAM into the

proper state:

MYCALL <YOUR_CALL_HERE>  This is the callsign that will be used on

the AIR.  The BBS call MAY be different!

Observe your License Class privileges!

MYPT <YOUR_CALL_HERE>    This is the call that the KAM will respond

to on the air

INTFACE TERM

DEL 0

PBBS 0                   (The TNC will reset at this point)

CWPTT ON

CWSPEED 20

FLOW OFF

XFLOW OFF

CRSUP OFF

PTHUFF ON

PMODE PACTOR

ECHO OFF

PACTOR

(The KAM Plus does not need the PERM command to remember its

settings.)

PACTOR MANUAL CONNECTS

To connect manually to another PACTOR station, the standard command

"C# callsign" is used.  You will not have keyboard access until the

connect is made or fails.  The standard "^CD" is used to disconnect

although if your connect accepts a "BYE" command or the like, that is

preferable.  After the connect is closed, there is a short delay

(about 10 seconds) where you again lose keyboard access.

PACTOR FORWARDING

Automated forwarding should not be done using PACTOR because of the

HF "attended" rules.  However, reverse forwarding can move traffic

off a remote PBBS.  A MSYSFWD.DEF-like file is made with a filename

of your choice.  This will serve as a manual forwarding file.  You

can not have the $filename.ext type of include file.  The following

is an example:

p0 no8m

wa8bxn

wb8bii

k8eiw

-----

MSYS 1.16                                                   Page 142

The above causes a connect to NO8M and sends traffic for the three

listed PBBS stations.  A reverse forward prompt then results in

traffic for the connecting station to be reverse forwarded.

The following is another example:

p0 no8m

home

wife

wa8bxn

wb8bii

k8eiw

no8m

oh

usa

-----

The above file connects to NO8M from a remote version of the NO8M

PBBS.  It sends traffic for stations who use the home NO8M to the

HOME route.  It sends traffic for KA8ZVV to her computer which is

attached via a RS-232 cable.  The other forwarding is standard.  When

you wish to forward, you utilize the following command:

FP filename.ext

MSYS then acts on your file and forwards via the PACTOR port.

USER'S VIEW WHEN CONNECTED

When connected to a MSYS PACTOR port, the user may be confused by the

fact that the link direction is controlled by MSYS.  However, other

than that, the user will see a port identical to a packet port.  The

user should not attempt to control the direction of the link.

PACTOR LIMITATIONS

No YAPP is currently possible on a PACTOR port.  You also should not

do FBB-type compressed forwarding.  No particular advantage is seen

when using the FBB group forwarding.  DO NOT ENABLE DIGI ON THE

PACTOR PORT!

MSYS 1.16                                                   Page 143

PACTOR TROUBLESHOOTING

A power supply of ample capacity for your radio is necessary.  Some

problems can be traced back to unstable or minimal power supplies.

The use of the TERM command to go in and talk to the TNC is valuable.

The TNC can be interrogated and connects can be established without

regard to MSYS settings by using TERM.

One initially confusing aspect is the use of "ECHO OFF''                                                              .  MSYS

requires echo to be off.  When in the TERM command mode, you may wish

to turn echo ON so you can see what you are typing.  Before you leave

the TERM command mode, you must remember to again set ECHO to OFF.

MSYS PACTOR SCANNING

By instituting scanning, your HF radio can be made to cover many more

frequencies and, due to that, more area.  The costs are minimal.  A

TTL-to-Serial level converter and a serial port is required.

Although the serial port would be a lot to ask for most of us, the

port is configured in such a way that there is no need for an

interrupt or normal port address.  The memory required is under 1k.

The port does not need an interrupt due to it never needing to read

data.  Data is sent to the radio but there is no need to read data

from it.

It is possible to build a simple interface to allow the computer to

choose one of four antenna lines.

IOBUS.DOC

A separate file is included with the MSYS archive that documents what

lines on a serial card are used for what purpose.  You may want to

review that file as it will be referred to often. It will also help

to have documentation for the board you are using.

INTERRUPTS

You can give it a interrupt if you can spare one.  You will be

wasting it, though, as it is not used.  If you can not spare an

interrupt, the first thing to do is to insure that no interrupt is

enabled on the card.  You may have to cut traces, set switches or do

whatever you need to do to make sure the interrupt is not enabled.

In most cards, you have two switches that connect either INT 3 or INT

4 to the electronics on the card.  In some older cards, this is done

with a jumper.  In both cases, turning the switches off or pulling

the interrupt jumper will cause it to be disabled.

There may be an instance where a interrupt is hard wired to the

electronics.  You will need to cut that trace using a knife or a hand

held rotary grinder.

MSYS 1.16                                                   Page 144

ADDRESSING

You will need to set the address jumpers or switches to address the

board as COM2.  This sets the address lines to the address $2f8.  By

cutting and soldering jumpers between two of the address lines, you

can re-address the board to be $378.  In this way, you can save the

addressing space.

This is done in this way:

$2F8     10   1111 1000

\ /

X      becomes

/ \

$378     11   0111 1000

^            ^

Bit 9         Bit 0

By cutting address bit 7 and address bit 6 and crossing them with a

jumper, you have re-addressed the serial card to recognize address

$378.

You need to cut two traces, the traces that lead to finger A23 and

A24.  Refer to the file IOBUS.DOC.  Finger A23 is soldered, via a

jumper, to the line that used to run from A24.  A24 goes to the line

that used to run from A23.

DSR LINE

MSYS checks a line in the serial port called the DSR line.  The DSR

line is attached to the PTT line through level converters.  This line

will tell MSYS when the radio has transmitted.  The radio

transmitting will indicate that the scanning must stop.

The PTT line from the KAM+ TNC is near TTL levels(0 or +5 volts).  It

must be changed to RS-232 levels (+12 or -12 volts).  This can be

done via a level converter.  When the TTL-to-RS-232 level converters

are discussed, the additional wiring necessary will be covered.

We determined that many RS-232 adapters, such as an adapter to take a

9 pin plug to a 25 pin socket, are improperly wired when it comes to

carrying the DSR line through the adapter.  If the radio does not

stop scanning, you may want to place a RS-232 LED box, such as Radio

Shack 276-1401, in line to verify you are seeing the DSR change.

MSYS 1.16                                                   Page 145

TTL TO RS-232 INTERFACE (1488/1489 version)

Supply voltages:    +5V     +12V    -12V    Ground

1488    14      1      7

1489    14             7

RS-232 connector                                   Radio

on computer control                                (TS-440 ACC1

serial port:                                        connector):

1489        1489

Pin 2 (XMIT DATA) -------[)o---------[)o----------> Pin 3

(receive data)

1     3     4     6

1488

Pin 6 (DSR)     <-------o(]--------+

3     2      |

|  2N2222

C  \

\|

|-----^^^^^---PTT line From TNC

/| B      33K ohms

E  /

+

|

Pin 7 (Ground) -------------------+----------------- Pin 1

(ground)

Note:  [)o is a level converting NAND gate

The arrow on the emitter of transistor is not shown.

Power supply design is up to you (not much current is

needed).

MSYS 1.16                                                   Page 146

TTL TO RS-232 INTERFACE (MAX232)

The MAX232 chip allows you to do the same level changes.

|------|

+12vdc ----------|--| 7805 |--|--------------|

-  |------|  -              |

.01^            ^.01           |

|            |              |

|------------|              |

|              |

gnd             |

|

|------------------------|       |

|                        |       |

|--------------|1                     16|-------|+

+-              |                        |       - 10uF

4.7uF^   gnd - )| --|2                       |       ^

|       10uF+  |                      15|-------|

|--------------|3                       |       |ground

|                        |

|--------------|4                       |

+-              |                        |

4.7uF^              |                        |

---------------|5                       |

|                        |

gnd --- | ( -----|6                       |

+10uF      |                        |

|                      10|---o(]-- PTT line

|                        |         from TNC

DB-25  -------|7                       |

pin 6         |                        |

DB-25  -------|8                      9|---[)o--  pin 3 of

pin 2         |                        |          ACC port

--------------------------

o(]  and  [)o  are 7400 NAND gates

NOTE:  JDR (800-538-5005) is advertising a newer version

of the MAX232 which requires no capacitors.

MSYS 1.16                                                   Page 147

For ICOM and Yaesu radios, the following circuit may be used from the

computer to the radio:

RS-232 Pin 2 (XMIT DATA)                Radio Serial Input

(Center of 1/8" plug for

ICOM, Pin 3 of mini-DIN

for Yaesu)

------------------------------

10 K Ohms      / C

|/

-----^v^v^v------|   2N2222 etc

B  |\

\ E

|

Ground

MSYS 1.16                                                   Page 148

SCANDEF UTILITY PROGRAM

You now need to set up the files that will be necessary.  You need to

run SCANDEF.  SCANDEF will access a ASCII file called PORTn.DEF and

modify the scanner.  It will also define the scanning frequencies,

the delay on each frequency before moving to another one, the antenna

that will be used and the time that the frequency will be scanned.

The "n" in PORTn.DEF is a number you supply to define the port.  If

your port is to be number 6, you would call the file:

PORT6.DEF.

At least 4 lines are required in the PORTn.DEF file. In order, they

are:

TSR xx         where xx is the interrupt number of the scan TSR,

usually D2

PORT yyy       where yyy is the hex address of the control serial

port

SPEED zzzz     where zzzz is the baud rate of the radio's

computer interface

freq delay antenna starthour lasthour

The last line type may be repeated up to 25 times. It contains the

frequency in MHz or KHz (14.235 or 14235 for example), the time to

spend on this frequency listening for a connect attempt in seconds,

the antenna number (0-3), the first hour to scan this frequency, and

the last hour to scan this  frequency. Only the frequency is

required. Defaults for the other values are 5 seconds, antenna 0,

start hour 0, and last hour 23. If you give any parameter beyond

frequency, you must specify all parameters to the left of it.

Here is an example file (PORT1.DEF it might be called if port 1 is

the PACTOR port):

TSR D2

port 378

speed 4800

3532.1

3737.1  10   1  0 8

3737.1  10   1  20 23

7169.6

7174.1

MSYS 1.16                                                   Page 149

This would scan 3632.1 and the two 40 meter frequencies all day.

3737.1 would be scanned from 8 PM to 8 AM only, listening 10 seconds

before going on to the next scan frequency (and using antenna 1).

Note that these frequencies are NOT appropriate for PACTOR Operation!

TURNING IT ON AND OFF

To enable scanning type (or better yet put in MSYS.DO) "PS n ON"

where n is the PACTOR port.  Repeat this for each additional port.

This command reads the appropriate PORTn.BIN file storing the

frequency parameters in memory and begins scanning.

To disable scanning type PS n OFF, where n again is the port number.

The PS command with no operands gives basic information about PACTOR

scanning such as which ports are enabled, current frequency and

number of frequencies being scanned.  PS with a port number as a

parameter gives a listing of the frequencies scanned on the given

port.

The command PS n freq will tune the radio on the given port number n

to the specified frequency.  This frequency need not be in the scan

list.  Using this form of the command also disables scanning.  The

frequency can be given either in MHz (7.0991 for example) or KHz

(7099.1).  This form of the command is useful before making a manual

connect (using the C# command) or initiating manual PACTOR forwarding

(using the FP command).  Note that this command can be imbedded in

the forward file (\ps 2 3541.2 for example). To resume scanning, use

PS 2 ON, either manually or at the end of the forward file.

If you have enough memory, you can run the SCANDEF program described

below using a command like "RUN SCANDEF PORT3" while MSYS is running.

To see the output after the program runs, type "TY $POOL".  (The RUN

command will redirect any screen output to a file called "$POOL".

You can read that file to determine the program's output.)  Then use

the "PS 3 ON" command to load the new frequency specifications. This

could all be done as a remote Sysop.

MSYS 1.16                                                   Page 150

ADDING ANTENNAS

RTS and DTR outputs from the serial port are set by the antenna

number as follows:

Ant#      RTS     DTR

----      ---     ---

0        0       0

1        0       1

2        1       0

3        1       1

+V

|

|

|                        Transistors are 2N2222 types

||>                        Relays can be 12 or 5 volts

||> Relay A

||>                        You can add a manual override

|                        switch, LED indicators, etc.

c |

\     10k

-----^^^^----- RTS

/

e |

|

|

ground

+V                                  +V

|                                   |

|                                   |

|                                   |

||>                                 ||>

||> Relay B                         ||>Relay C

||>                                 ||>

|                                   |

c|                                   |c

\     10k              10k        /

-----^^^^-------------^^^^------'

/               |                 \

e|                                   |e

|               DTR                 |

|                                   |

|                                   |

ground                              ground

MSYS 1.16                                                   Page 151

THE LAST STEP, THE TSR

Before running MSYS, an appropriate SCANTSR program must be run, such

as SCAN440.COM.  This is best done in your AUTOEXEC.BAT file.  The

SCAN440.COM program is designed to be used with the Kenwood TS-440.

Other SCANTSR programs may be found on the distribution disks,

including programs for the ICOM IC-735 and the Yaesu FT-757.

USING OTHER RADIOS

As distributed it is set up for a Kenwood TS-440.  Source code is

provided (SCANTSR.ASM) that can be modified for other radios.  If you

do modify it for some other radio and are sure of your modifications,

send WA8BXN a copy so that it can be included in future releases.

The SCANTSR program is called by MSYS when it needs to tell the radio

to go to a new frequency. It is called as an  interrupt passing the

control serial port address in DX and the new frequency as a 32 bit

number (long integer) in registers AX and BX.  AX has the most

significant 16 bits.  The TSR simply formats whatever command the

radio needs to go to the given frequency and then sends that data to

the radio using the given control serial port address.  The control

serial port is already initialized.

MSYS 1.16                                                   Page 152

MSYS GENERAL INFORMATION

BBS ONLY ACCESS

Lets say you are running with ports 0 through 3 and want to make port

2 bbs only access with at most 1 connection allowed at a time (either

another BBS forwarding to you or you forwarding but not both at the

same time).  You also don't want to generate MAIL FOR beacon on port

2.  Use the following:

FB 2 25           turn off mail for beacon port 2

BP 4              port 2 bbs only

BM 2 1            max 1 connect port 2

MCF $B            no simultaneous forwarding on port 2

Note that BP and MCF both have port masks as arguments.

CONNECTS FROM NON-AMATEUR CALLSIGNS

Connects are no longer accepted from non-amateur callsigns  (like PK-

232 and NOCALL for example).  You can still make connections to non-

call signs.

#  COMMAND

Command lines beginning with # on the BBS are ignored (they sometimes

come from KA Nodes).

CHANNEL NUMBERS

The channel numbers use a rotational assignment rather than always

starting from 0

DEVICE ERRORS

Device errors are trapped by MSYS (you would previously get the

Abort, Retry, Ignore? message).  The first occurrence of the error is

given a response of retry, the second ignore.  This will normally be

caught by particular function being performed which will terminate

with an error message, but MSYS should continue operation.

DISK FULL MESSAGE

If the disk is full when a message is sent to the system it will give

a message to the connection station that the disk is full and then

disconnect.

MSYS 1.16                                                   Page 153

DOS

If you are using MSDOS 3.3, you may need to put STACKS=0,0 in your

config.sys file.  MSYS has been run under MSDOS 5.0 and MSDOS 6.0 as

well as DRDOS 6.0 with no known problems or special commands other

than FILES=40 and BUFFERS=20.  With MSDOS 5.0 and MSDOS 6.0, you may

chose to use SMARTDRV.EXE instead of the buffers statement.  It all

depends on your personal preference and "RAM CRAM".

END OF LINE BELLS:

When you type past column 72 you will get a beep for each character.

Keep those lines less than 80 characters!

HEADERS

To create an ASCII file containing all the active message headers,

use the following sequence of commands on the bbs:

X 0       (turn off the More? message temporarily)

>msghdrs  (this is the name of the file to be created)

L$ 0      (or L 0 if you don't want to see the bids)

>         (close the file)

X 20      (restore the More? message)

KEYBOARD

When someone connects to your keyboard answer call you will get two

musical tune to alert you to the connect.  Any stations connecting to

your keyboard call when you are already connected to another station

will get a busy.  The actual number of connects allowed before busy

is given by the SYSOP command MCon.  MCon 1, for example, only allows

an incoming call to the keyboard to be answered if there are no other

keyboard connections.  MCon 0 prevents all incoming keyboard requests

(they get busy).  MCon N will allow N connections to the keyboard

until busy is returned.  When you type past column 72 you will get a

beep for each character.  Keep those lines less than 80 characters!

When a station attempts to connect to your keyboard via the several

ways possible but is sent a busy the bell will also beep to alert you

to this.

***LINKED

A line of the form ***LINKED to callsign is recognized as a

connection by the given callsign.  This occurs with some versions of

RLI systems when a user uses the C command on the RLI system to

connect to the MSYS BBS.  ***LINKED to callsign requires SYSOP

verification if SYSOP call is used.

MSYS 1.16                                                   Page 154

LOCKUPS

If the system hangs an attempt is made to reboot.  Thus you should

make sure you don't have a diskette in A: and that you have an

appropriate AUTOEXEC.BAT file set up.  It is strongly recommended

that any automatic restarts include the MUTIL 4 command to insure

that the mail file will be properly indexed.  If the mail files are

not properly indexed, MSYS will most likely hang on restart.

LOGGING

Messages sent, forwarded, and killed get logged to MSYS.LOG.  A new

log file record is being generated.  It is identical to the BBS

record except it has BYE in place of BBS.  It is generated when a

user exits the bbs by any means (disconnect, timeout, B cmd).  You

may wish to put an entry in the MSYSTODO.DAT file to rename MSYS.LOG

periodically (weekly?) as the log file CAN get very large, very fast!

MAIL BEACON

Messages to your IDCALL and to SYSOP are not displayed in the MAIL

FOR beacon.  You will see them on the screen.

MAIL.BAK

To reduce disk read/write delays when a message is received and

copied to the MAIL.BAK directory, subdirectories are automatically

created in the MAIL.BAK directory.  100 messages are backed up in

each directory.  This can substantially reduce the time it takes to

make the backup copy of incoming messages because much smaller number

of file names must be looked at to add the new file name to the

directory.  The created subdirectory names are of the form MSGxxx

where xxx is the message number divided by 100.  Thus in MSG025

messages 2500 to 2599 would be found.  The message /RESTore function

from the BBS has been modified to understand these changes.

MAX channels AND PORTS

Maximum for MAXCHANS is 50  (MSYS.OPT)

Maximum number of ports is 7 (numbered 0 to 6)

MSYS supports all IRQs (0-7) for serial ports.  I suggest the use of

IRQs 2,3,4 & 7 on PC class computers and 3,4,5 & 7 on AT class and

above computers.  Use of 01 & 6 are not recommended but are supported

in the code.  Due to the use of these interrupts by other hardware

devices/functions results will generally be bad!  I really think that

the MS-400 (and similar) shared interrupt serial interface boards

should work properly.

MSYS 1.16                                                   Page 155

MEMORY MANAGEMENT

Buffer space for logical channels is allocated when the channel is

allocated (and released when the channel is released).  The MEM

command may or may not change results depending on position of the

last user in the memory space.  If you find that you are seeing

***MEMORY LOW BUSY messages, you probably should investigate the

reasons why.  There are several things that you may try to do to

relieve the problem.  The first is to use a upper memory manager to

put as many of the TSRs as possible into upper memory.  If you use

Quarterdeck QEMM, you should run the OPTIMIZE program.  When using

QEMM, you may wish to NOT use DOS=HIGH in the CONFIG.SYS.

Another suggestion is to use the Quarterdeck program VIDRAM with the

"ON" switch set.  VIDRAM will utilize the memory just above the 640K

boundary that is normally used for graphics and add it to the

available program space.

While the default MINmem is 32000, it can be lowered slightly WITH

CAUTION.  If MINmem is set too low, you do run the risk of a user

(INCLUDING THE SYSOP!!!) logging in and utilizing more memory

resources than are available, causing immense grief and extra effort

to clean up the damaged files.  A 386 system with 6 active ports and

the SAM callsign database server using DOS 6.2, QEMM 7.03 with VIDRAM

ON, running under DesqView 2.4 starts out with approximately 100000

bytes free.  It seems to be enough to keep up to 14 channels working

with very few "busys".  If that still isn't enough, you may wish to

either try another multitasking system (Windows?) or run MSYS not

under a multitasking system.

If all that fails, or you just have a system with no expanded or

extended memory, you need to seriously consider your MSYS

configuration.  If you reduce the maximum number of messages (MUTIL

1) to a smaller number, you will increase the available memory for

users.  You may choose to limit the number of active channels.  If

you are experiencing low memory problems, you may also wish to

disable the conference feature.  While it is possible to reduce the

size of the various memory stacks used by MSYS, extreme care is

needed to avoid serious consequences.  See the section on STACKS for

further details.

Another alternative is to provide a minimum service system by running

the "small" version of MSYS, which is distributed as MSYSB.EXE.  The

'B' version does not provide all the fancy NODE OR TCP/IP features,

but is will function quite nicely as a stand-alone BBS working behind

or along side of other network switches.  Probably the major loss to

the packet community is the fact that this version will not show up

in a NetRom node list.  You will still need the main distribution

disks for all of the documentation.  MSYSB.EXE is the executable

version that replaces the "big" MSYS.EXE

MSYS 1.16                                                   Page 156

EXPANDED MEMORY USAGE

MSYS will automatically detect if either EXTENDED or EXPANDED memory

is available.  If sufficient EXPANDED memory is present (at least

320K must be available to be used) then all stacks will be around

8000 bytes and come from expanded memory.  This should free up about

50K or so of memory as shown by the MEM command.  Also each new

connection will require less memory.  If you have expanded memory and

don't want it used, start MSYS using:

MSYS NOEMM

Real expanded memory boards (that could be used with an 8086/8088

processor) will give better performance (faster) than simulating

expanded memory.  Expanded memory is used (if present) to contain the

message headers.  Now you can have 2000 msgs on your system and

memory to spare.  Be warned that with this many active msgs things

will slow down.

NOTE:  If you have problems running expanded memory try using a

different EMM program.

MSYS can also run using EXTENDED memory as well.  Of course, you will

need to utilize a EXTENDED memory manager such as HIMEM.SYS or QEMM

7.0 or perhaps any one of several other quality memory manager

programs on the market.  If you have EXTENDED memory and do not want

it used, start MSYS using:

MSYS NOEXMEM

AUTOMATIC MESSAGE HOLDING

If you create a file called MSYSHOLD.DAT you can specify

characteristics of messages to hold automatically using parameters

similar to those used in the house cleaning file.  The available

parameters are:

TO=       wildcard representation of To callsign

FROM=     wildcard representation of From callsign

AT=       wildcard representation of @BBS

SIZE=          number that is size of message

CONNECTED=  exact callsign of sending station (less SSID)

PORT=     inbound port number that message is from

BID=      This is useful for trapping known "improper" BIDs.

TITLE=    If used, this should be the last parameter on the line.

TEXT=     Names file which contains list of "bad words".

MSYS 1.16                                                   Page 157

Normally ARRL bulletins have BIDs similar to: ARLP037.  These bids

begin with a letter.  BIDs automatically assigned to bulletins that

don't have a bid usually begin with a digit.  So, to hold bulletins

that probably don't have the right bid, we could use:

AT=ARRL BID=#*

Note that #* is a wildcard specification that matches anything that

starts with a digit.

If the value given for TITLE=  is found any where in the title of a

message (and all other specifications are met) the message will be

held.

There is one other parameter that can be specified, a line containing

only the letters:

BBS

All lines ABOVE the line BBS will look at all messages coming in for

the holding criteria.  The lines below the BBS line will only apply

to messages entered by a NON-BBS station.  For this use, a BBS is

defined as a station that transmitted a [...$] line.  Therefore, all

entries below the BBS line will apply to users of your system..

Example:            Comment - (NOT IN MSYSHOLD.DAT FILE!)

SIZE=2500           holds any messages bigger than 2500 bytes

FROM=WA8BXN PORT=2  holds messages from WA8BXN that come in on port 2

CONNECTED=W8XYZ     holds all messages sent by connected user station

W8XYZ

AT=MSYS             holds all messages with @MSYS

TEXT=BADWORDS.DAT   holds all messages that have any words found in

BADWORDS.DAT file.

BBS

AT=ALL*

Note: the difference between FROM= and CONNECTED=

FROM is the from call for the message (often supplied after < in the

S  command), while

CONNECTED is the call of the station connected to the bbs sending the

message.

In the above example, a message sent to ALLUS from a local USER would

be held, but would NOT be held if entered from a connected BBS.

NOTE:  Held messages are visible only to the SYSOP  They are killable

only by SYSOP.  A message will be sent to a user (not a BBS) that the

message has been held and made invisible for SYSOP review.  The user,

at that point, has no further ability to deal with that message.

MSYS 1.16                                                   Page 158

Other options on the lines in MSYSHOLD.DAT:

* at the beginning of the line makes it into a comment

TEXT=filename operand

When specified, if other tests are met, the text and title of the

message will be compared with the contents of the given filename .

If any of the words in the file match, the message will be held.

Case is ignored in the test.

Example:

TO=ALL TEXT=WORDLIST.1

would cause all messages going to ALL  to be compared with the file

WORDLIST.1.  Put one phrase or word per line in the file, enclosed in

either single or double quote marks.  You may also put a number

between 1 and 255 at the beginning of the line before the quoted

phrase to assign a weight to the word.  If no weight is given, the

word or phrase gets a weight of 255.  Here is a sample file of words:

'war'

"900"

"send check"

50 'send'

50 "check"

"That's it"

If the sum of the weights of these "words" found in the title or

message text exceeds HOLdlevel (default 0) then the message is held.

NOTE: HOLdlevel is a new SYSOP command!

CAUTION: 'war'  would match an occurrence in the word 'forWARd' in a

message.  Its not a real problem as it just means the message will be

held not killed off.

To resolve that "problem", you may include a space either before, or

after or both before and after to "isolate" the specific word you

wish to hold. A space will also help catch words at the beginning or

end of a sentence.  For example, if our "bad" word is "dirt", the

following:

" dirt " in wordlist.1 will catch the word "dirt" but not the word

"dirty" in the sentence: The dirty bird's name is dirt.

MSYS 1.16                                                   Page 159

If the word KILL is added to the line in MSYSHOLD.DAT, the message

will be stored in the mail.bak directory and immediately killed.  For

example:

TO=DIRT KILL

will kill all messages to DIRT.  It is suggested that this form of

the command be used with care.

MSYS.DO

MSYS.DO is a file which you should create to do all of the SYSOP

commands needed to prepare your specific MSYS system.  This file is

processed as a series of commands and is far easier to do rather than

manually typing them every time you start MSYS.  When the system

starts, it automatically tries to do a DO command.  See the sample

file for more information on the possible contents.

MSYSTODO.DAT

Timed execution of commands can be done in MSYS.  You can create  a

file called MSYSTODO.DAT with commands and times they should be done.

Wild cards can be specified for the times.  Each line consists of a

time (given as exactly 8 characters)  followed by  a space, and then

the command to execute at the  given  time. The wildcard character is

? and they may only occupy positions in the  left  part of the time .

The format of the time  is  MMDDhhmm where  MM  is the month number

(1 to 12), DD is the  day  of  the month  (1 to at most 31), hh is

the hour (0 to 23) and mm is  the minute (0 to 59) .  Here are some

time examples:

MMDDhhmm  (pattern)

????????  every minute

???????0  every 10 minutes (times that are like 00 10 20 etc.  for

the minutes)

??????00  at the beginning of every hour

????1200  at noon every day

????0000  at midnight every day

???10000  at midnight of the 1st, 11th and 21st days of each month

??010400  at 4 am the first of every month

01010000  at the beginning of every year

You can also specify a day of the week in the DD field, in  place of

the day of the month .  To do this you would put D# in  the  DD field

(# is from 1 to 7 for Sunday to Saturday) .  Examples:

??D11200 Every Sunday at noon

11D52300 At 11 PM on each Thursday in November

MSYS 1.16                                                   Page 160

That is how the times are set up, now here is a complete  example

of things that could go in MSYSTODO.DAT:

????0000 nod sav

????1200 nod sav

??010000 log

??d10000 copy msys.usr msysusr.bak

???????0 dt

This file would save the network node list twice a day, make a new

log file at the beginning of every month, make a backup of the user

file every Sunday, and display the time and date every 10 minutes.

MSYSTNC.#

If you use any TNC set up files be warned that the program waits 1

sec after each line from the file is sent to the TNC to give it time

to think about it.

NOTE:

AEA TNCs made after Jan. 89 can probably be treated like KPC-x TNCs

to get them into kiss mode.  It should no longer be necessary to have

a special TNC file for them (but if it doesn't work without it, it is

suggested that the TNC file be used).  A good solution to the TNC to

kiss problem with MFJ 1270/4 TNC-2 compatibles is to install the

JKISS EPROM available from Tucson Amateur Packet Radio.  The EPROM

forces the TNC to ALWAYS be in the KISS mode, and no further action

by MSYS is needed.

Special KISS Parameters

There are 4 parameters that may be set either before the TNC is put

into the KISS mode with the MSYSTNC.x or after by commands from the

MSYS.DO file.

They are:

TXD

TXT

PPersist

SLottime

It is probably best to set them in the MSYS.DO file so that you have

the ability to determine what the value is in your TNC by examining

the parameter from the keyboard.  If MSYS has NOT set the parameter

after the TNC has gone into the KISS mode, it will show -1 as the

value.

Suggested values are TXD 30, TXT 5, PPersist 64, SLottime 10.

Your mileage may vary.

MSYS 1.16                                                   Page 161

Packet size

Data in packets is limited to 256 bytes.  Larger amounts of data in a

single packet will be discarded without causing the system to crash.

PASSWORDS

There are several PASSWORD types utilized in MSYS.

REMOTE SYSOP

First, there is the remote SYSOP password.  The phrase for this is

stored in a file called MSYS.RMT in the C:\MSYS directory. to you.

/Sysop command:  CASE SENSITIVE!

This command gets you into remote SYSOP mode which allows you to use

most of the SYSOP commands through the bbs.  The command must be

typed exactly as shown: backslash, Capital S and lower case ysop and

then return.  You will then get a line like

Usage stats: Users=4 Msgs rcvd=10 Msgs sent=5 Max=8

You MUST respond with the EXACT characters needed before SYSOP

privileges will be granted.  To this you must reply (there will be no

prompt) with the four characters corresponding to the numbers typed

in order of characters found in the file MSYS.RMT (case is

important).  For example, if your MSYS.RMT file contained MSYS is the

greatest! then you would type <space> e i t which are the

corresponding characters (the first character is numbered 0).  Thus

you would type: eit (Don't forget to count the spaces) You should now

get a prompt cmd:  If you are connected via radio, you issue /Sysop

command on bbs to verify SYSOP authority (enter 4 letters) You then

get the next bbs prompt (if you give correct letters or not).  If you

have given the correct reply then you may issue any SYSOP command by

starting the line with \ (while still in the bbs).  So when on the

bbs from the local keyboard, you can now type something like \em 1234

and you will be able to edit message 1234 without having to leave the

bbs.  As a remote SYSOP, once you give the right 4 letters, you

retain SYSOP status until you disconnect.  To get a list of available

commands, type h.  Most of the commands available duplicate those

found normally as SYSOP commands.  An added command, WHO, gives a

list of who is on the system and is the same as the new U command in

the bbs.

NOTE: /Sysop is not required from local keyboard.

Make sure you mark yourself a remote SYSOP, even if you won't be

one.(User flag $10)

MSYS 1.16                                                   Page 162

AUTOMATIC PASSWORD REPLY

If you are running a remoted MSYS system (you DO go on vacation,

don't you?), you may automate that return process by having a file

called PASSWORD.RMT in your C:\MSYS\FWD directory.  Likewise, this is

a convenient way for your neighbor SYSOP to manage your system while

you are absent.

If your system receives the password prompt (the "Usage    stats"

line) from a system that you connected to during forwarding or from

the Connect commands,  MSYS will automatically supply the correct

response.  You must have a file called PASSWORD.RMT in your FWD

directory that is the same as the MSYS.RMT file on the other system.

NOTE:  If you are using MSYS to remote SYSOP a distant bbs, it will

respond ONLY ONCE per initial connect!  For example, if you are using

MSYS to SYSOP a remote system from the campground,  your local system

will respond to the "Usage..." statement the first time.  If you

disconnect from the remote bbs and drop back to a node, then

reconnect to the remote system, your local MSYS will NOT respond to

the "Usage..." statement.  You must disconnect completely and then

re-establish the connection to the remote system for the automatic

password response to be given.  This is done to help prevent repeated

automatic attempts to decipher the password.

USER PASSWORDS

A third form of password protection is the USER password.  If the

bits for USER password have been set for a user, then you must have a

PASSWORD DIRECTORY.  The directory will contain a file for each user

with ONLY the callsign as the file name.  There are NO SSID's or

extensions used.  The sample directory and file name would look like

this:

C:\MSYS\PASSWORD\N8UDK

At a minimum, N8UDK's user bits would be set to $400.

If a user has their flag bits set with the $400 bit on, when they

connect they will get the Usage stats message line and must give the

proper 4 characters just like a SYSOP would.  The password file for a

user must be in the PASSWORD directory and has a filename of the

user's call (no SSID, no file extension).

A fourth password is the one used by the FTP function of TCP/IP.  It

is more fully described under the heading of MSYSPASS.DAT.

MSYS 1.16                                                   Page 163

A fifth password file that is necessary is the MSYSPASS.IP.  This

file lists the call and password and callsign for Telnet users.  If

this file is not present when a station makes a Telnet connect to the

bbs, rather than asking for login and password, it will simply ask

for callsign which will be used without further verification.

A sixth password file is utilized for modem access.  Discussion of

the modem passwords used in the MSYSPASS.MOD file can be found in the

documentation in the MODEM section.

TheNet & NetRom RS-232 Connections

RS-232 Connections of ports to TNCs running NetRom and TheNet should

use > as the first character in the name given in the port statement

in MSYS.OPT.

To connect the computer to a single TNC per computer port, a straight

through cable is used (with a minimum of pins 2,3 and 7).  At the TNC

end, pins 10 and 23 must be jumpered.  DO NOT CONNECT PINS 10 or 23

at the computer end!  In a NetRom/TheNet equipped TNC, the connection

between pins 10 and 23 switches the TNC interface from a normal ASCII

terminal interface to one that sends NetRom Serial protocol.

In a multiple TNC connection using the standard diode matrix

interconnect,  the computer may be connected if you reverse pins 2&3

and pins 5&20 at the computer connection.

Using MSYS into a matrix with NetRom or TheNet equipped TNCs can have

great advantages on a backbone system.  The TNCs handle traffic on

the backbone not going to/from the MSYS system without any load being

placed on the MSYS computer.  MSYS then gains the advantage of a fast

reliable connection to the adjacent nodes.

Forwarding may be accomplished by connecting to the RS-232 connected

TNC or matrix using normal script files.  When using the NetRom

Serial protocol, remember the ">" character in the port statement!

SMTP

The \+\filename include capability that is currently in the BBS S

command is also usable in SMTP with the PRompt command.  Both of

these are available only from the local keyboard.

Typing other than Y N C A or RETURN in response to More? in the BBS

is taken as a new command.

MSYS 1.16                                                   Page 164

STACK Requirement

NOTE: NOT USED WITH EXPANDED OR EXTENDED MEMORY!

The stack space needed by the forward process has increased.  The

following line can be put in MSYS.OPT  (This may not be necessary):

STACK FORWARD 8000

A smaller value (maybe 7000) might work.  Try it if you want.  If the

system randomly crashed, make it 8000.

TERM

TERM is a new ( Version 1.16) Sysop command which allows the Sysop to

directly access the hardware connected to a port.  It is extremely

limited in what it can do.  It is designed to allow you to

interrogate and test port hardware.

The syntax for this command is:

TERM #

where "#" is the port number.  TERM permits direct keyboard access to

the port for issuing commands to a modem or perhaps a TNC.  The

screen will show what is received from the hardware.  The keyboard

input will be sent to the hardware.

The F9 key will take most TNCs out of the KISS mode so that you may

communicate with the TNC command processor.  DON'T FORGET TO PUT THE

TNC BACK INTO THE KISS MODE BEFORE EXITING!  In a similar fashion,

when accessing the PACTOR port, the same rule applies - leave the TNC

in the proper mode (<PACTOR STANDBY>) before exiting.  The term mode

can also be used to access the modem, however, none of the MSYS

capture or include features are available.  To properly exit the dumb

terminal mode, depress the F10 key.

"F9" can be used to take a TNC out of the KISS mode.

When used with a KISS TNC, not much be accomplished.  The TNC can be

taken out of KISS mode, adjusted and reset to KISS but automated

events within the system may lead to undesired and unpredictable

results.  There is no recording, sending of files, etc. and there is

no plan to add them.

"F10" will end the TERM session.

MSYS 1.16                                                   Page 165

TESTING

You can connect two MSYS systems together through their serial ports

by the use of a NULL MODEM.

You can also connect a loopback by connecting pins 2 and 3 on the

same serial port.  This will allow you to see what the system is

doing without being on the air.

TITLES TRUNCATED

Titles of messages are truncated after 37 characters.

TYPE OF MESSAGE

(the "TR" field)

The algorithm uses 3 pieces of information to determine the

appropriate TR to assign: the destination "callsign", the @"bbs", and

the character after the S in the Send command.

The to "callsign" is classified as one of the following:

0 - Looks like a real callsign

1 - A zipcode (5 digits)

2 - something else (like ALL, for example)

The @"bbs" is classified as one of the following:

0 - Looks like a real callsign

1 - none specified

2 - call of this bbs

3 - something else

The character after the S in the s command is classified as:

0 - B  (as in SB for send bulletin)

1 - P  (as in SP for send private)

2 - T  (as in ST for send traffic)

3 - none (as in just plain S)

4 - something else (SW for send weather bulletin, for example)

The following message types are generated:

BN   For  bulletins going to a specific bbs - these are

forwarded to a single station that handles that bbs.

PN  For private messages

TN  For NTS traffic

xN  For Sx where x is an "something else"

N   For individual non-private messages.

B$  For bulletins going with @route (like ARRL)

x$  For bulletins sent with Sx (x is "something else")

P$  For private bulletins going with @route

MSYS 1.16                                                   Page 166

Note: [P]N below means that the type will be N is MAKEPrivate is off,

PN if MAKEPivate is ON (default).

TR Values Assigned to Messages

For messages going to Callsigns:

Character after S ->     B    P    T    None x (Other)

@BBS type:               ---  ---  ---  ---- ---------

callsign                 BN   PN   TN   [P]N xN

no bbs given             B$   PN   TN   [P]N x$

this bbs                 B$   PN   TN   [P]N x$

something else           B$   PN   TN   [P]N x$

For messages going to ZIP Codes:

Character after S ->     B    P    T    None x (Other)

@BBS type:               ---  ---  ---  ---- ---------

callsign                 BN   PN   TN   TN   xN

no bbs given             B$   P$   TN   TN   x$

this bbs                 B$   P$   TN   TN   x$

something else           B$   P$   TN   TN   x$

For messages going to SOMETHING ELSE:

Character after S ->     B    P    T    None x (Other)

@BBS type:               ---  ---  ---  ---- ---------

callsign                 BN   PN   TN   BN   xN

no bbs given             B$   P$   TN   B$   x$

this bbs                 B$   BN   TN   B$   x$

something else           B$   P$   TN   B$   x$

Messages with second character of type N are forwarded to one place

(and then killed if AUTOKill is ON, the N is changed to F otherwise).

Messages with second character of type $ are flooded to all stations

that get the specified @BBS.  When sent to all such BBSs, the second

character gets changed to #.

Messages that come in with an R: line that contains the call

(actually HCAll) of this bbs are automatically held.  The R: line

scan is terminated by the first non-R: line found in the message.

MSYS 1.16                                                   Page 167

Bulletins that arrive with a bid on the S command line are rejected

if their BID already exists in the BID file.  Bulletins are

identified as those messages that arrived via the SB command or S

non-call.  All messages other than bulletins are always received

(never rejected).  If  a message identifier ($string) exists on the S

command line it is used.  In all other cases, a message identifier is

generated internally for all non-bulletins using the bbs call and

message number from the last R: line scanned.  If the message

identifier is found in the BID file and the message was not held

because of this systems R: line, the message is not saved (but it is

acknowledged as being received entirely by sending the > prompt when

the ^Z is received).  The processing of internal message identifiers

is not indicated in the system identification line [MSYS-XXXX-H$].

If you set MIDchar to something, then the character will appear

between the H and the $.  For example, if you set MIDchar to M, then

the SID line will be [MSYS-XXXX-HM$].  This will be sent to all

stations that connect to the BBS.  Setting MIDchar to a non-null

character will also cause  the MID to be sent during forwarding if

the system to which you are forwarding has MIDchar in its SID.

MSYS 1.16                                                   Page 168

USER BIT SETTINGS:

$01   -  Expert user

$02   -  BBS

$04   -  Bad User Bit  (Disconnect immediately)

$08   -  Limited BBS  Permits only limited use of the bbs

$10   -  Remote SYSOP status

$20   -  Need to send WP Update

$40   -  Multiline packets (XF)

$80   -  Prevents use of the S command.  Users so marked can only

read      messages.

$100  -  Lock user

$200  -  Authorizes use of the Upload command in the bbs.

$400  -  Must have user password

$800  -  YAPP Upload authorized

$1000 -  Auto Reply turned on

$2000 -  No Automatic LC in BBS

$8000 -  New User flag

NOTE: These settings may be added together.

CAUTION!  They ADD in HEXADECIMAL!  800 + 200 = A00

MSYS 1.16                                                   Page 169

WP SERVER:

Generation of messages to the white pages master server is included

in MSYS.  This sends the information users give in the N, NH, NZ and

NQ commands to the WP server when it is changed (or initially given)

as well as periodically.  Note that MSYS does not respond to messages

to WP but merely passes them on without modification.  WP messages

are NOT generated for users with any of the following

characteristics: locked, homebbs of ?, bbs  system, and/or baduser.

KANTRONICS 3.0 EPROM Upgrade users:

I am told you will have to make a TNC setup file and include the

following:

INTFACE KISS

RESET

You may also need to include a line with one or more control-c

characters before these two lines in your file.

MSYS 1.16                                                   Page 170

FREQUENT PROBLEMS AND NOTES FROM THE AUTHOR

IMPORTANT:  You must use the new version of MUTIL with this version

of MSYS (i.e., run setup, function 9).  It is probably also a good

idea to delete MSYS.HRD when going to a different version of MSYS or

changing the port statements in MSYS.OPT!

If you have problems getting MSYS going here are some things to try

that have helped many who have called:

1.  Make sure you have FILES=40 and BUFFERS=20 in CONFIG.SYS

2.  Try adding STACKS=0,0 to CONFIG.SYS

3.  Get rid of anything resident (check AUTOEXEC.BAT and CONFIG.SYS)

4.  Run MUTIL 4 (do this EVERY time you start MSYS)

5.  Simplify MSYS.OPT as much as possible; remove all PORT

statements

6.  Delete your MSYS.DO file (after making a backup!)

These items should get you to the point where MSYS will work.  From

here try putting things back and see what causes failure.

OVERRUNS, FRAMING ERRORS

On occasion, it may be useful to do an ST S command to see the status

of your ports.  If you find that you have "OVERRUNS", you should

probably make some adjustments in the MSYS.OPT file for speeds.  An

overrun is a condition that occurs when the TNC delivers a character

to the serial port and the computer does not process the character in

time before another character is presented.  If the second character

is presented before the first has been taken out, the UART will

signal MSYS that there has been an overrun.  One possible solution is

to use a buffered UART such as the 16550, which MSYS can properly

utilize.  A second choice is to adjust the TNC to computer bit rate.

The "normal" thinking is to run the port as fast as possible, but it

may be useful to actually slow the TNC port to something over the

radio speed but less than port maximum.  Empirical tests have shown

that a TNC/computer speed of 4800 seems to work well on a 386-25

machine using 8250 (non-buffered) UARTs.  In any event, you may need

to adjust the TNC speed of the various ports until you minimize any

overruns.  Modem speeds do need to be at least as fast as the highest

speed on the modem, and preferably twice as fast.  To change the

speed of a port, remember it is necessary to change the value in

MSYS.OPT, then run MUTIL 9 before the changes will take effect.

MSYS 1.16                                                   Page 171

ABRUPT STOPS IN FORWARDING

It has been observed that ANY blank lines in the MSYSFWD.DEF, or any

of the .FWD or .REV files included in the MSYS.FWD file will cause

problems.  Make sure that ALL if the included files do NOT contain

blank lines at the ends of the file.

MESSAGES DON'T FORWARD

ALL forwarding is done on the @BBS.  In the hierarchical routing

process, it is possible that a bbs will contain 2 or more of the

criteria for forwarding.  MSYS will only forward based on the left-

most item matched in the @BBS.  For example, let us assume that we

forward all messages @ .#NEOH to WA8BXN.  We also forward messages @

KB8AST to NO8M.  If a message @ KB8AST.#NEOH.OH comes in to be

forwarded, MSYS will investigate the hierarchical routing and

determine that all messages @ KB8AST go to NO8M.  If, for some

reason, NO8M is not able to take the message, it will NOT be

forwarded to WA8BXN as a "second choice".  It is therefore imperative

that you not "over-specify" when setting up your forward files with

the hierarchical routing. One major advantage of hierarchical routing

is that you need not list EVERY bbs in a state or an area.  You only

need to specify the hierarchical component.

Make sure you have executed the MERge command.  Without doing this

the system will never forward.

NOTE:  You must have the file MSYSBBSB.DAT to be able to forward

bulletins.

THE SYSTEM FORWARDS MESSAGES TO IMPROPER LOCATIONS.

Check to make sure that your MSYSFWD.DEF or MSYS.FWD files do not

have improperly placed "*" characters or have imbedded empty lines at

the end of the sections.  These "empty lines" are often found at the

end of the xxx.FWD or xxx.REV files.

THE SYSTEM APPEARS TO WORK BUT:

Nothing is on the screen!

Make sure you set the display type correctly in  the MSYS.OPT file.

The command is WINDOW DISPLAY COLOR for color monitors  or

WINDOW DISPLAY MONOCHROME  for monochrome monitors.

MSYS 1.16                                                   Page 172

THE SYSTEM COMES UP AND APPEARS TO HANG.

This may be caused by a non-existent com port.  Check the lines in

your MSYS.OPT to make sure the port addresses are correct.  Also,

Rerun MUTIL 4 to insure the mail files are properly indexed.

SYSTEM REBOOTS AFTER FORWARDING

This may be caused by having too many entries in your forwarding

file.  Remember you may only have up to 400 different bbs names in

your file!

PERFORMANCE HINTS

Don't let too many messages pile up in MAIL.BAK <DIR> before you

delete them (after archiving if you wish) so the hard disk doesn't

fill up.

From time to time do a BID # command and see how many bids are

active.  If its over 3 or 4 thousand its probably time to run MUTIL

function 21 to get rid of the older bids.

Delete old log files.  It is probably wise to put a statement:

??D10000 log in your MSYSTODO.DAT file to keep the log file from

expanding to extremes.  It is not uncommon to have a log file exceed

1 megabyte in a week's time!  The "log" statement will convert the

MSYS.LOG file to a yymmdd.log, which then can be archived off line.

While you are creating the MSYSTODO.DAT file, it is also not a bad

idea to add a: ????0001 NODe SAve  just to keep your current node

list in case of a reboot.

Periodically you should also do the following:

1.  Delete old BIDs using MUTIL 21

2.  Delete old backup messages. (after running Mutil 10)

3.  Delete all files in the /mail.bin directory (while MSYS is

NOT running!)

Here are some suggestions from users of MSYS that they claim work.

Use at your own risk!

MSYS 1.16                                                   Page 173

To cure the problem of certain TNCs that don't transmit properly on

HF use radiospeed of 1200 in place of 300 in MSYS.OPT.  I can't

imagine how this could help, but someone says it did for them (they

weren't using MFJ TNCs which seem to have this kind of problem).  One

thing you might want to try if you are having problems with a TNC on

HF is to get a copy of KA9Q's net.exe program and see if you can use

its AX.25 connect command with your TNC on HF.  If that fails as well

the problem definitely is in the TNC.

MFJ KISS MODE

To put a MFJ TNC in kiss mode, try this in your MSYSTNC.# file

ax off

parity 0

awlen 8

conmode trans

kiss on

restart

If you are just bringing up MSYS for the first time and experience

crashes when mail is forwarded to you, or the size of BIDLIST.DAT is

0 bytes, do the following to solve the problem:  Shut down MSYS

(quit), delete BIDLIST.DAT, start MSYS, shut it down (the DOS DIR

command should now show a size of 130000 for BIDLIST.DAT).  You

should now be ready for normal operation.

ADDITIONAL NOTES

Paccom ROM version 1.1.5 may work better than 1.1.662

If you are using DOS 3.3, you may need to put  STACKS=0,0 in your

config.sys file.

If you find that the displayed time is different from what you have

set the time to be in DOS, put the following in AUTOEXEC.BAT and

reboot: SET TZ=EST5  (Obviously, set it to YOUR time zone!)  One more

thing - the time can (and does) often get hosed up around the first

of April and the end of October because of certain software language

routines that attempt to account for the change in Daylight Savings

time.  Not to worry (too much) because it will eventually make itself

right (with maybe a little help from you!)

You should also be aware that you may set your computer to local time

(with the TS hhmmss command) and set the time that MSYS uses to GMT

by use of the GMtime xx command where xx is your offset from GMT in

hours.

MSYS 1.16                                                   Page 174

Home BBS replacement occurs only if @BBS is blank or the call of your

bbs.

Note:  If you run into problems with newer versions of QEMM and MSYS,

try turning off the stealth option in QEMM.

OS2 REPORTED PROBLEMS

Reference a message from N8LJL @ N8ITP.OH.USA.NA

To All users of OS2 2.1 GA.  A problem has been found when using Com

3 or Com 4 on IRQ 2 on ISA System Boards.  When a DOS Session is

opened with access to a port using IRQ 2, the program may come up

initially and work fine, but when you switch back to the DeskTop and

return to the DOS Session, the screen data will not update.  This is

most common on ProComm Version 2.01.  It appears that the interrupt

control portion of the program is losing it brains as to where IRQ 2

is assigned.  IBM is currently working on the problem.  Hopefully a

resolution will not be too far down the road.

This problem has existed since the advent of OS2 2.0 GA.  It will

only affect those users who are trying to run more than 2 com ports

and require access to IRQ 2.

There is indeed an interrupt conflict, but all is not lost.  Refer to

pages 376-380 in the OS2 manual, with particular note to page 378 and

the "DOS settings" on 379.  The book missed a point that will be

important. In config.sys and an explanation of what they do:

DEVICE=C:\OS2\MOUSE.SYS SERIAL=COM1

DEVICE=C:\OS2\COM.SYS (3,3e8,5)

DEVICE=C:\OS2\MDOS\VCOM.SYS  (3,3e8,5)

The second line sets up COM3, address 3e8, to IRQ5 for OS/2 sessions.

The THIRD line does the same for DOS programs running under OS/2.

I recommend using IRQ5 for COM3 unless you have an LPT2 printer

attached.

FINAL NOTE

IF MSYS DOESN'T SEEM TO BE WORKING PROPERLY, REMOVE ALL

(EVERY LAST ONE!) RESIDENT PROGRAMS INCLUDING YOUR ENTIRE

CONFIG.SYS (keeping files=40 and buffers=20) AND

AUTOEXEC.BAT FILES AND SEE IF IT WORKS THAT WAY.  IF IT

DOES, PUT THINGS BACK IN ONE AT A TIME TO SEE WHERE THE

PROBLEM IS!

MSYS 1.16                                                   Page 175

MSYS Utility Support Programs

(c) Copyright 1993 by HUB COMPUTERS, INC.

MUTIL.EXE

When you run MUTIL you will get a screen that is similar to:

==============================================================

MSYS Utilities 1.16 by WA8BXN (C) 1993 Hub Computers, Inc.

1    Change maximum number of messages

*    Restore messages from backup directory MAIL.BAK        ***

3    Print remote sysop phrase number table

4    Check Mail index file integrity

5    Test serial ports

6    Print forwarding tree

8    Run log file analyzer

9    Run SETUP program

10   Scan message headers to create/add to BBSLIST.DAT

11   Deletes old entries from BBSLIST.DAT and BBSTONTS.DAT  files.

12   Fix mail index file (can be hazardous to use!)

13   Process BBSLIST.DAT into BBSTONTS.DAT file

14   Scan BBSTONTS.DAT to build BBSTONTS.BIN (indexed)

15   Build list of archived messages

16   Printer status

17   MERge BBSLIST.DAT files

18   Check MSYSNODE.DAT integrity

19   Convert user files 1.05 & before <===> 1.06 and later

20   Sort calls in the BBSLIST.DAT file

21   Delete Old BIDS (and MIDS)

22   Specify file name to use in place of printer

99   Quit

Enter number of selection: _

*** This function has been replaced by the /RESTore command in the

BBS

MSYS 1.16                                                   Page 176

MUTIL Function 1 - Change max number of messages

Input file(s):  MSYS.MSG

Output files(s): MSYS.MSG

The maximum number of messages that can be stored in MSYS is

determined by the MSYS.MSG mail index file.  When MSYS starts

running, it checks to see if there is an existing MSYS.MSG file.  If

there is one, it is used (and contains the header lines of the

messages that existed when MSYS was last stopped).  If there is no

existing MSYS.MSG file, then an empty one is created, with a capacity

of 100 messages.  This utility function can be used to change the

size of an existing MSYS.MSG file.  You can increase or decrease the

size of the file.  You can't make it smaller than the number of

active messages.  The maximum number of active messages has been

increased to 2000.

NOTES:  There is a cost in memory requirements for each message

possible.  Each message slot costs 138 bytes of memory;  100 messages

(default maximum) thus takes 13800 bytes.  500 messages would take

69,000 bytes and 2000 messages will take approximately 270,000 bytes

(even if there weren't that many messages active).

VALUES I USE:  I run with a 500 message maximum size.  Normally I

have around 400 active messages.

DIALOG  WITH  FUNCTION: You are first asked for the name of the

MSYS.MSG file to modify:

Enter path to MSYS Mail Index file or just press return to use

MSYS.MSG: _

Normally you will want to just press the return key.  If the current

directory from which you are running MUTIL does not contain the

MSYS.MSG file that you want to modify, you can specify a different

path (or even a different file name if you have renamed MSYS.MSG for

some reason).

This function will next copy the specified MSYS.MSG file to a file

called MSYS$$$.MSG which can be used to restore MSYS.MSG if for some

reason this function fails.

Next you are asked if you have run the Check Mail function (function

4).  You should run Check Mail just before this function since

attempting to change the maximum number of  message slots can produce

very interesting (and usually bad) results if the original MSYS.MSG

file is corrupt.  If you respond N then  this function terminates and

you can then do the Check Mail function.  If you have just run Check

Mail, then press Y to continue.  Note that you do NOT press return

after either of these letters!

MSYS 1.16                                                   Page 177

If you responded Y, then the function continues, telling the current

maximum number of messages, the current number of active messages,

and some other information.  You are then asked for the new maximum

number of messages you want.  This number must be greater or equal to

the number of active messages.  It must also be less than or equal to

2000.

If the value you ask for is acceptable, the file will be changed to

accommodate the new number.

Next you will be asked to press a key to continue (try the return

key).  This pause is put in the function so that you can see any

additional messages that were produced before the screen  is cleared.

MUTIL Function 2 - Restore Messages

MOVED INTO MSYS PROGRAM!

This function has been moved into the MSYS program with the SYSOP

command of /REST [msg#] which MUST be done while connected to the

bbs.

MSYS 1.16                                                   Page 178

MUTIL Function 3 - Print Remote Sysop Phrase

INPUT FILE(S):  MSYS.RMT

OUTPUT FILE(S): Hardcopy printer output (optional)

This function prints the contents of the MSYS.RMT file (used  for

remote sysop verification).  The output of this function can be just

the screen or also to the printer.  The output consists of printing

the content of remote sysop phrase followed by a table that gives the

character at each position in the phrase.

DIALOG WITH FUNCTION:

You are asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

SAMPLE OUTPUT:

Sample phrase.

0   S

1   a

2   m

3   p

4   l

5   e

6

7   p

8   h

9   r

10  a

11  s

12  e

13  .

USAGE EXAMPLE:

If  you did the \Sysop command as a remote user to get sysop

privileges and got the following reply:

Usage stats: Users=7 Msgs rcvd=3 Msgs sent=6 Max=12   you would type

the following response:

pp e

MSYS 1.16                                                   Page 179

MUTIL Function 4 - Check Mail Index File Integrity

(CHKMAIL)

INPUT FILE(S):  MSYS.MSG

MAIL\MSG*.DAT

OUTPUT FILE(S): MSYS.MSG

This function checks the integrity of the Mail index file MSYS.MSG.

It checks for such things as message slots on both the free and

active lists, loops in either of these lists and  active message

numbers that do not have text files in the MAIL directory (and  vice-

versa).  It attempts to fix any errors if it can (and its generally

pretty good at doing it).

NOTE:  It will not delete any message text files in the MAIL

directory if they don't  have active headers.  They will be reported

but you will have to delete them yourself if you really don't want

them.  You probably won't really want them, but  you decide!

NOTE:  This function may be run in a batch file through MUTIL

(without having to give its function number interactively) by

running:

MUTIL 4

SAMPLE NORMAL OUTPUT:

CHKMAIL - An MSYS Utility to verify mail index integrity

MSYS Mail Index Checker

head=0 free=1 next=2

Reading Index file:

There are 100 total message headers

Pass 1:

No cycles detected

No headers on both lists

All headers accounted for

No message files without headers

All active headers have files

Press a key to continue

COMMENTS:  I would strongly suggest you run this function EVERY time

MSYS crashes for any reason (it should not crash often, if at all).

To be on the safe side, many MSYS SYSOPs always run it when they

start MSYS.

MSYS 1.16                                                   Page 180

MUTIL Function 5 - Test serial ports

INPUT FILE(S):  None

OUTPUT FILE(S): None

This function provides a very basic test of serial interfaces.  It is

designed mostly to see if you got the addresses/int  numbers right

for your machine.  It is not intended to be a brutal test of their

operation!

DIALOG WITH FUNCTION:

After a table is displayed that lists the normal addresses for serial

cards and corresponding interrupts, you will be asked if your machine

has an 8086/8088 CPU chip in it.  If its a basic PC/XT compatible

system, respond Y.  If you have an AT (80286) or higher level

machine, respond with N.  If you have an 8088/8086 based system (PC

class) then it will look for ints from  2,3,4, and 7.  If you say not

8088/8086 (AT class and above) it will look for ints from 3,4,5, and

7.  Depending on your response, certain interrupt handlers will  be

installed.  You will then be asked for the address of the serial card

you wish to check.  The answer you give must be in HEXADECIMAL.  A

valid response might be 3F8, for example.  A simple check is made to

see if there might be a serial port at the address you specify.  It

is easily fooled if you specify the address of some other kind of I/O

interface.

If  there seems to be a device at the address you gave, a quick check

will be made to see if it looks like an 8250 UART chip is used in

your serial interface.  Internal loop back tests cannot be done with

the newer UART chips.  If the test fails when you select internal

loopback, try the external loopback option.  This requires a

connector that will connect pins 2 & 3 together in place of the TNC

you would normally connect.  External loopback can be used to test

all types of UART chips.

Next you will next be asked for a baud rate to test at.  Here  you

give a decimal number as an answer.  It probably doesn't make much

difference what you give . (Don't try 0!).  I usually test at 1200

baud.

Now its your turn to type characters.  As indicated, press Esc  to

terminate the test.  Normal response should be something like:

Had IRQ 4 1 time(s),  A received

If you press the letter 'A' key.  If interrupts aren't working you

would see:

Received A without interrupt.

NOTE: This is not a performance test, just an address test.

MSYS 1.16                                                   Page 181

MUTIL Function 6 - Print forwarding tree

INPUT FILE(S):  MSYS.FWD

OUTPUT FILE(S): Printer hard copy (optional)

This function displays your forwarding file in two ways to tell you

what callsigns you forward to each bbs you forward to and also to

which bbs you forward each callsign.

DIALOG WITH FUNCTION:

You are asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

The first part of the output consists of lines like

AD8I ---> KA8Z WB8BII

which means that messages with @AD8I are sent to either KA8Z or

WB8BII(which ever one you can connect to first).

The second part of the output consists of lines like

KA3NVP <---   KA3NVP NTSNJ NTSNY NTSPA

which means that you forward messages with KA3NVP NTSNJ NTSNY and

NTSPA in some part of the @BBS field to KA3NVP.

NOTE:     To pause the display, you must use ^NumLock (^S  won't

work).         To resume scrolling, press any other key.

MSYS 1.16                                                   Page 182

MUTIL Function 8 - Run Log File Analyzer

INPUT FILE(S):  MSYS.LOG

MSYS.MSG

OUTPUT FILE(S): Printer hard copy (optional)

This program produces a listing of BBS usage.

DIALOG WITH FUNCTION:

You  are  asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

SAMPLE OUTPUT:

==============================================================

From Wed Mar 01 15:23:48 1989 to Mon May 229 00:57:03 1989

Station  Connects     Sends   Reads     Kills     Forwards

HCLean      0           0       0        27         0

N8UDK       3           1       1         0        12

NT8V        0           1       1         0         0

WA8BXN      4           0       0         1         9

WB8ZZS    168          82      31        24         0

LAST READ       MSG# TO   @BBS  Count  Title

May 18            1 TEST         1    test

Mar 23            7 TEST         2    test

May 28            2 TEST         3    test

Mar 02         8073 TEST         7    no title

MSGINs 84  MSGKILLs 97  MSGFWDs 21  MSGREADs 33 RESTARTs 331  QUITs

311  CONNECTS 175

The following messages were received but not killed:

1   2   3   20   21   8073

(These should be active messages on your BBS!)

==============================================================

NOTES:  This data is taken from my test system which uses  WB8ZZS as

its call (Thanks to my wife Ruth!) Many strange things happen on this

system as I am testing new software changes.

The first line of the report gives the period of time covered  by the

contents of the MSYS.LOG file that was read.

The next section of the report lists stations that were active on the

BBS and what they did.  The headings mean the following:

Station - Callsign of the station

Connects- Number of times the station connected (or the

SYSOP used the BB command).

Sends  -  Number of messages sent by this station

Reads  -  Number of messages read by this station

Kills  -  Number of messages killed by this station

Forwards- Number of messages forwarded to this station

MSYS 1.16                                                   Page 183

NOTE:  The callsign HCLean indicates activity by the House Cleaning

routine.

The next section lists messages that were read during the period,

sorted in increasing order of number of reads .  The meaning of the

headings is:

LAST READ - Date the message was last read

MSG#    - The number of the message on the bbs

TO      - The "TO" station callsign of the message

@BBS    - The "@" field of the message.

Count   - The number of times the message was read

Title   - The title of the message

NOTE:  Private messages are not shown,  only bulletins.  This

section is useful to see what interest there is in the various types

of messages.

The next section provides overall counts of various activities:

MSGINs      The number of messages that were sent to the BBS

MSGKILLs    The number of messages that were killed

MSGFWDS     The number of messages forwarded by the BBS to other

systems.  Note: If a bulletin is forwarded to 4

different systems, it counts as 4 forwards

MSGREADs    Total number of Read commands used on BBS

RESTARTs    Number of times MSYS was started

QUITs       Number of times MSYS was gracefully shut down using

^F4 or Quit command.  Restarts - quits is the number

of crashes for whatever reason.

CONNECTS    Number of user connects to BBS + SYSOP BB cmds

The final section of the report gives a list of message numbers that

were assigned to messages sent to the system but not killed.

Normally these message numbers should be active messages on the BBS.

If they aren't, they may be messages to restore since they got lost

somehow.

MSYS 1.16                                                   Page 184

MUTIL Function 9 - Run Setup Program

INPUT FILE(S):  MSYS.OPT

OUTPUT FILE(S): MSYS.DEF

This function sets default initialization parameters.  IT MUST BE RUN

BEFORE YOU CAN RUN MSYS THE FIRST TIME and any time you change

MSYS.OPT.  It is documented elsewhere in this document.

DIALOG WITH FUNCTION:

You are asked if you want hard copy output (on the printer).  Respond

Y or N as desired.

MSYS 1.16                                                   Page 185

MUTIL Function 10 - Scan Message Headers

to create/add to BBSLIST.DAT

INPUT FILES:   MAIL.BAK\MSG*.DAT (or specified directory) BBSLIST.DAT

(if it exists)

OUTPUT FILE:   BBSLIST.DAT

This program scans backup mail message files that are (or were

originally found) in the MAIL.BAK directory to determine what BBS

stations have been "seen" (using the R: lines added by each BBS as it

forwards a message).

DIALOG WITH FUNCTION:

You will be asked to give the name of the directory that contains the

messages to be scanned or to press return to use the MAIL.BAK

directory directly.

NOTE: This function may be run from a batch file by using either:

MUTIL 10

or:

MUTIL 10 dirname

In the first form (MUTIL 10) it will be assumed that the input files

are in the MAIL.BAK directory.  In the second form (MUTIL 10 dirname)

the files are assumed to be in the given directory name.

OUTPUT FILE FORMAT:

The first line of BBSLIST.DAT gives the date this function was run to

create or add to the file.

The remaining lines in the file consist of the following fields:

callsign of bbs

location of bbs

postal code of bbs

number of times this call has been seen

Note: If you choose to edit this file, be sure to keep things in the

same columns.  Also note that the callsigns are lined up using the

digit of the callsign.  The list is sorted  by  digit in callsign,

letters after the digit, and then letters before the digit (pretty

much callbook order).  Calls that do not begin with A, K, N, or W are

grouped together at the end of the list, with an * in the first

position in the record (non-US calls).

MSYS 1.16                                                   Page 186

MUTIL Function 11 - Deletes Old Entries

From BBSLIST.DAT and BBSTONTS.DAT

This utility function will prompt the user to determine what the

oldest record date you wish to keep in a yymmdd sequence.  It will

then edit the files to delete BBS records older than the specified

date.  It might be a good idea to then run MUTIL Function 10 to add

any new BBS's to the list before running MUTIL Function 13.

MUTIL Function 12 - Fix Mail Index File

***** WARNING ***** This function can DESTROY the mail index file

INPUT FILE(S): MSYS.MSG

MAIL\MSG*.DAT

OUTPUT FILE:   MSYS.MSG

This function allows manual inspection, checking and modification of

the mail index file.  It is a utility I wrote to see what destroyed a

mail file and perhaps fix it.  There is one function that may be

useful to most SYSOPS, that allows you to specify the next message

number to be assigned to incoming messages.  You may want to do this

if you were running some other bbs and want to continue using message

numbers from where you left off.  Personally I would suggest that you

don't do this but rather start with the default value 1.  But its up

to you!

NOTE:  Make a copy of your existing MSYS.MSG file before using this

function!

NOTE:  Most of the checking and repair functions have been automated

in the CHKMAIL function (MUTIL Function 4).

DIALOG WITH FUNCTION: You are first asked for the name of the

MSYS.MSG file to modify:

Enter MSYS message header file name or simply press return for

MSYS.MSG: _

Normally you will want to just press the return key.  If the current

directory from which you are running MUTIL does not contain the

MSYS.MSG file that you want to modify,  you can specify a different

path (or even a different file name if you have renamed MSYS.MSG for

some reason).

MSYS 1.16                                                   Page 187

To change the number that will be assigned to the next message, use

the following procedure:

Select F8 (Manual changes)

Select F5 (Change Next)

Enter new value for next message number to be used.

Select F10 (Return to previous menu)

Select F9 (Save changes and quit)

RENUMBERING MESSAGES

All active messages may be renumbered using MUTIL function 12.

You should renumber your messages when you reach about message number

64000.  As message numbers increase above this they will wrap around

back to 0 if you don't use this function.  Some of the L command

options will not work properly if newer messages have numbers smaller

than older ones.  To renumber the messages, go through the following

steps:

1 - Start the MUTIL program

2 - Type 12 and return

3 - Press return to accept use of MSYS.MSG

4 - Press F8 for manual changes

5 - Press F6 for Renumber messages

6 - Type new starting message number (1 is good) then

press return

7 - Press F10 to return to previous menu

8 - Press F9 to save changes

9 - Press return to go back to main menu

10 - Type 99 and return to exit

It should be noted that any files in the mail.bin directory should

also be deleted when renumbering messages.

MSYS 1.16                                                   Page 188

MUTIL Function 13 - BBSLIST.DAT to BBSTONTS.DAT

INPUT FILES:  BBSLIST.DAT

ZIPCODES.DAT

STATES.DAT

ROUTES.DAT

OUTPUT FILE:  BBSTONTS.DAT

This function builds a list of BBS calls (and optionally zip codes)

and the corresponding xx designator to which they should be

forwarded.

If you select processing US BBS calls by state the data found in

BBSLIST.DAT is scanned for state and zip code.  The state is

identified as being something after a comma in the location field

that matches the list of state two letter codes found in  the

STATES.DAT file.  If only one state is found and it corresponds to

the zip code found, then the BBS call is automatically assigned to

xx  where xx is the state abbreviation.  If either of these pieces of

information is missing, or they do not match the test fails and you

are asked to decide what state the bbs belongs in.

DIALOG WITH FUNCTION:

You will be asked if you want to include state routings for ZIPCODES.

You should reply Y the first time you are creating the BBSTONTS.DAT

file.  Otherwise reply N.  If you delete BBSTONTS.DAT for some

reason, you should reply Y the first time you create a new one.

If the state of the BBS cannot be automatically determined, you will

get a screen that looks like this:

==============================================================

WB8ZZS   KIRTLAND OH        44026   (4)

State/ZIP check failed - Zipstate OH

Enter two letter state code then <Return key> or press <Escape key>

then <Return key> to ignore or just press <Return key> to use OH

=============================================================

The first line of this screen is the line from BBSLIST.DAT that is

being considered.  The next line gives the state that corresponds to

the zip code (if available).  The third line gives the guess that can

be accepted if you just press return.  If the guess is good, hit

return.  If you want to use a different state, type the two letter

code for that state and then press return.  If you want to omit this

call from the output file, press some non-letter key before pressing

return.  Space bar, return works well.

MSYS 1.16                                                   Page 189

In this example, the state could not be identified because  there is

no comma between the city and the state.  (It would be nice if

everyone used standardized R: lines!) The guess of OH was made from

the zip code alone.

When the US calls have been processed, you will be asked if you want

to process non-US calls.  If you have earlier chosen not to process

US calls by states/zipcodes, the US calls will be processed with all

the other calls if you choose to process non-US calls.  This feature

is useful for non-US hams that handle all US messages as just going

to the US.

If you choose to process the calls at this point, you will be

presented with screens that  list an entire entry from the

BBSLIST.DAT file.  Following this line will be the callsign of the

BBS identified from the line.  Using the call prefixes found in the

ROUTES.DAT file, a guess will be made as to what routing should be

used for the given call.  You may then choose to ignore the call, to

use the routing obtained from the call prefix (if any) or to type the

complete route you desire.

A sample screen might look like this:

=============================================================

*  WB8ZZS   KIRTLAND OH        44026   (4)

Callsign:    WB8ZZS

Possible route: USA.NA

Press <Escape key> <Return key> to ignore

Type desired routing then press <Return key> or

just press <Return key> to use USA.NA

=============================================================

BBSTONTS.DAT FILE FORMAT:

This  is an ASCII file containing in bytes 1 to 9 a BBS  call  or

zipcode (left justified, possibly with wildcard * indicator), and the

corresponding designator to which messages for that BBS  call or zip

code should be forwarded beginning in position 10 .  Example lines:

89*     NV

WB8ZZS  OH

VK1XX   AUS.AS

NOTE:  Bulletin routing designator (like ARRL) should NOT appear in

this file.

NOTE:  The file is sorted in ascending order using the entire record

as key.

NOTE:  There should be no duplicates of calls/zip codes found in the

first 9 bytes of the records.

MSYS 1.16                                                   Page 190

MUTIL Function 14 - BBSTONTS.DAT to BBSTONTS.BIN

INPUT FILE(S):  BBSTONTS.DAT

OUTPUT FILE(S): BBSTONTS.BIN

This function builds an index of the calls in  the BBSTONTS.DAT file,

placing the index at the beginning of the BBSTONTS.BIN file.  After

the index, the contents of BBSTONTS.DAT is copied to BBSTONTS.BIN.

DIALOG WITH FUNCTION:

This function does not require any keyboard input.

NOTE:  The format of the resulting file is such that no attempt

should be made to edit it.  With care, BBSTONTS.DAT may be edited and

the result then processed by this function.

NOTE:  The file BBSTONTS.BIN is used by MSYS.  The files BBSTONTS.DAT

and BBSLIST.DAT are NOT used directly used by MSYS and may be stored

on a diskette, if desired, when MSYS is running.  I just keep them in

the normal MSYS directory.

MUTIL Function 15 - Build List of Archived Messages

This function will look at messages found in a given directory (in

MAIL.BAK format) and construct a file of the message headers.  The

result is a file of all your messages like doing a L command on the

BBS for every message that has passed through your system.  You can

use the grep function on it to find messages with some particular

string of characters in their header line.  I use it to find a

message when I think to myself "I remember there was a message about

...  one time".  Then knowing the message number you can go back to

your archive diskettes and extract the complete text.

MUTIL Function 16 - Printer Status

This function provides a very simple printer test to see if its

connected and can be initialized.

MSYS 1.16                                                   Page 191

MUTIL Function 17 - MERge BBSLIST.DAT Files

This function takes as its input your current BBSLIST.DAT file (which

it first renames to BBSLIST.BAK) and another file you specify  in the

same format as BBSLIST.DAT) and merges the two files into one called

BBSLIST.DAT.  I use this function to merge the BBSLIST.DAT file that

some of the users of MSYS have sent me.  This allows me to include in

my file some bbs calls that normally would not have been seen in

messages passing through my system.

MUTIL Function 18   Check MSYSNODE.DAT Integrity

This function performs the following functions:

MSYSNODE.DAT viewing and repair program

0     Printer echo toggle (now OFF)

1     List active neighbors

2     List all neighbor slots

3     List active known destinations

4     List all known destination slots

5     Data consistency check

6     View given neighbor or destination

99     Exit

Any output from the program will be directed to the screen unless the

printer has been toggled on.

MUTIL Function 19 - Convert User Files

1.05 & Before To 1.06 and later

Version 1.05 and earlier used a different format for storing user

files.  This utility probably will be of little use to most current

users.

MSYS 1.16                                                   Page 192

MUTIL Function 20 - Sort BBSLIST.DAT File

This utility will sort the callsigns in the BBSLIST.DAT file.  It

will sort the bbs callsigns based on number, suffix after the number,

prefix to the number and generate a new file called BBSLIST.NEW  At

the conclusion of this operation, it will suggest that you rename the

files in the proper manner.  This utility has been revised in MUTIL

1.16 to operate faster and to permit more BBS calls.

MUTIL Function 21 - Delete Old BIDs (and MIDs)

This function deletes old BIDs/MIDs from the file BIDLIST.DAT.  It

asks for the number of days worth of bids that should be kept.  Run

it periodically (maybe once a week when you do your hard disk

backups).  The bid file can hold up to 6500 entries  The more it has,

the slower it works.  The maximum number of BIDs in the BID file is

settable.

MUTIL Function 22 - Specify Printer File Name

The file you specify will be opened and will receive any output that

would normally go to the printer.  Any existing data in the given

file will be lost.  If you want to abort this  function, simply press

return when asked for the file name.

MSYS 1.16                                                   Page 193

SAMPLES OF MSYS FILES

(c) Copyright 1993 by HUB COMPUTERS, INC.

MSYS.OPT:

port 0 at $3E0 int 7 speed 4800 name 145.01 RADIOSPEED 1200 TNCTYPE 1

port 1 at $2E8 int 7 speed 9600 name 223.68 RADIOSPEED 1200 TNCTYPE 1

port 2 at $3E8 int 7 speed 4800 name 145.05 RADIOSPEED 1200 TNCTYPE 1

port 3 at $2F0 int 7 speed 9600 name >AKRNODE RADIOSPEED 9600

port 4 at $2F8 int 3 speed 19200 name MODEM RADIOSPEED 1200 TNCTYPE 2

port 5 at $3F8 int 4 speed 9600 name PACTOR_80M

numchans 15

bbs alias akrbbs

bbs call wb8bii

id call k8eiw

id every 30

forward call wb8bii-1

answer call k8eiw-15

kanode call wb8bii-3

sysop name Don

sysop qth Cuyahoga Falls

sysop zip 44223

process id window 2

monitor outgoing on

window display color

window 0 size 1 1 80 13

Window 6 size 1 23 48 25

make

MSYS 1.16                                                   Page 194

MSYS.DO:

*----------------CALLSIGN STUFF-------------------------------

HCA WB8BII.#NEOH.OH.USA.NOAM

HOST K8EIW

NRI AKRON

NRC K8EIW

MYT K8EIW

myip 44 70 4 5

mya akrbbs

*-----------------TNC PARAMETERS------------------------------

monsf on

check 0500

*------------Turn on CTS for Modem and PACTOR ports

cts 4 1

cts 5 1

*------------Set FRACK for ALL ports

frack 12

*------------Reset FRACK for high-speed matrix port

frack 3 5

resp 1

slot 10

tac 2

txt 5

txd 0 35

txd 1 30

txd 2 35

txd 3 5

paclen 236

maxfr 0 4

maxfr 1 7

maxfr 2 4

maxfr 3 7

PP 64

DPORTS $00

DIGI OFF

*-----------------NET/ROM PARAMETERS -------------------------

*------------Adds New Neighbors Heard if On

AUTOn off

*------------Broadcast Interval 30 minutes

BIN 1800

*------------KaNode Turned off

KPORTS $0

*------------Net Default Window Size

ND 7

*------------Enable BBS in NetNode

NETB 1

*------------Enable Connect Comand in NetNode

NETC ON

*------------Net Full Function Hours

NETFBBS 0-23

MSYS 1.16                                                   Page 195

*------------NetNode On

NETN ON

*------------Network Connect for BBSs Only (Off permits ALL)

NETO 0

*------------Mail-for beacons turned off if 1 (port MASK)

NOBE $0

*------------NetRom PORTS 0,1,2,3

NPORTS $F

*------------Network Time to Live

NTL 15

*------------Obsolescence Count Initial

OCI 6

*------------Obsolescence Count Min for Broadcast

OCM 5

*------------Path Quality of heard nodes port 0

PQ 0 120

*------------Path Quality of heard nodes port 1

PQ 1 120

*------------Path Quality of heard nodes port 2

PQ 2 120

*------------Path Quality of heard nodes port 3

PQ 3 200

*------------TCP ON PORTS 0,1,2,3

TPORTS $F

*------------Min. DESt Quality to record

WQ 120

*--------------------BBS PARMS--------------------------------

*------------Kill on Forward OFF

AUtokill OFF

*------------Turns on AX25 Level 2

AX25L2 on

*------------Turns On AX25 Level 2 backoff (RNR)

AX25b $F

*------------Turns on BBS Node broadcast to network

BBSNODE 1

*------------Turns on BID function

BID 1

*------------Display Time for F2 Key (0=infinite)

f2timer 0

*------------FBB Compatibility/Compression On (SEE NEXT STATEMENT!)

FBBC 2

*------------FBB Compatibility/Compression OFF for MODEM PORT (PORT

4)

FBBC 4 0

*------------FBB Forward Group Number of Messages

FBBG 5

*------------Forward Time interval (- = every x minutes)

fti -10

*------------Network FBB Compatability/Compression On

NETFBBC 2

*------------Network FBB Forward Group Number of Messages

NETFBBG 5

MSYS 1.16                                                   Page 196

*------------Network RLI Batch Size

NETXB 5000

*------------Network RLI Compression Option On

NETXO ON

*------------House Clean at x Minutes after 00:00

HCL 310

*------------Heard List turn off point if Dispatches < #

HE 10

*------------Initializes Printer before each File Printed (0=NO)

IN 0

*------------Makes Messages Private by default

MAKEP ON

*------------Maximum Bulletin Age

MAXB 10

*------------Allow Connects to SYSOP Keyboard

MCON ON

*------------Min Memory for Memory Low Busy Msg

MINM 32000

*------------Turns on Message Trailer Function

MS on

*------------Establishes New User default bits

NEW $00

*------------Turns on CallSign Server

PC ON

REQQTH ON

REQDIR ON

REQFIL ON

*------------Adds Received BID to R: line

RBid ON

*------------Screen Save Setting

SC 10

*------------Sets TimeZone (z)

TIMezone GMT

*------------Adjusts GMT offset from local time(CHANGE WITH DST!)

GMtime 5

*------------Turns on SYSOP Verification

VERIFY on

*------------Adjust color of Sysop BBS screen

WB 3 1

WF 3 14

*------------RLI Batch Size (non-compressed bytes)

XB 5000

*------------RLI Compression Option On

XO ON

*----------------NET/ROM NEIGHBORS----------------------------

NODE ADD AKR:K8EIW-1 3 192

NODE ADD #AKRU:K8EIW-2 3 200

NODE ADD #AKRU2:WB8BII-9 3 200

NODE ADD AKROH:WB8BII-10 3 200

AUTODEST On

NETNODE ON

NODE RESTORE

MSYS 1.16                                                   Page 197

DESt REP AKR:K8EIW-1 K8EIW-1 3 0 230

*-------------ARP ROUTING TABLE-------------------------------

arp add ka8tef-4 -2 44 70 16 1 /24

arp add ka8z-4 -2 44 70 4 4

arp add k8eiw-4 -2 44 70 4 32

arp add wa8bxn-2 -2 44 70 4 6

arp add wb8lvp-2 -2 44 70 4 57

*------------Default TCP/IP ROUTE--------------

DRoute k8eiw-4 3

MSYS.OTD:

<<< RADIO CLUB MEETING TONIGHT @ 8:00 PM - CITY BUILDING >>>

***

MSYSK.OTD

< Please try  ### W8UPD  Gateway 145.01 <-> 223.68 >

MSYS.REP:

NTS* ##### @2 @1    COMMENTS (NOT IN FILE!!!)

440* * @1 NO8M      This sends ALL 440xx Zip Code to NO8M

44001 * @1 N8ITP    This redirects 44001 Zip Code to N8ITP

44003 * @1 KB8AST   This redirects 44003 Zip Code to KB8AST

4403* * @1 N8ITP    This redirects 4403x Zip Code to N8ITP

44030 * @1 KB8AST   This redirects 44030 Zip Code to KB8AST

The result of the above is that all ZIP coded

mail not specifically addressed to the 44001,03

or 3x Zip Codes will go to NO8M.

441* * @1 W8GRG

44107 * @1 NO8M

46* * @1 KA8TEF     This redirects all Zip Code 46xxx to KA8TEF

* OHNET @1 ALLOH

* ARL @1 ARRL

* ALLUSA @1 ALLUS

* USA @1 ALLUS

* K8EIW @1 WB8BII

* WB8CQR @1 W8GRG

* WB8BXN @1 WA8BXN

NOTE:  THERE SHOULD BE NO BLANK LINES IN THE FILE!

MSYS 1.16                                                   Page 198

MSYS.RMT:

PASSWORD TEST PHRASE 0123456789

(This file can be more than 2K in length if you desire!  If you do

much remote sysop activity, we STRONGLY urge a long password and

utilize MSYS or MSYST as the remote terminal server. )

MSYSBBSB.DAT:

1  KA8Z

2  NO8M

3  KB8DRN

7  W8GRG

8  WA8BXN

9  NG8P

10

16 AD8I

17 W8CQK

18 KA0FPO

19

22 WA8DCE

23 K1LT

31 W8UPD

NOTE: 31 is the MAXIMUM number of BBS's you can forward bulletins to!

MSYS 1.16                                                   Page 199

MSYSFWD.DEF:

FILE CONTENTS  COMMENTS  (DO *NOT* INCLUDE COMMENTS IN FILE!)

F1 w8upd       This is a straight forward out port 1 to a

w8upd          local PBBS.  Only Messages @ W8UPD will be forwarded.

-----          Necessary dashes! (5 minimum - 79 maximum)

F3 kq8m

@k8eiw-2       Forward through an adjacent TheNet node.

.c #cledx      Connect command given to TheNet node.

+to            Necessary triggers.

?from

?with

.c kq8m

#120           This extends the wait for connect to 120 seconds

+to            seconds after the .C KQ8M command was issued.

?with

?Busy

kq8m

-----

R3 ka0fpo      This is a forward with a request for Reverse Forward

@k8eiw-2

.c #cledx

+to

?from                         This example shows how to use the

?with                         include file KA0FPO.REV.  The .rev

.c kq8m-1                     extension indicates ka0fpo may

#120                          reverse forward from this system.

+to

?with

?Busy

.c ka0fpo

#120

+to

?with

?Busy

$c:\msys\fwd\ka0fpo.rev

------

f1 wa8dce

!25                           This indicates that WA8DCE will never

$c:\msys\fwd\wa8dce.rev       be forwarded to, but will reverse

all                           forward messages.

--------

MSYS 1.16                                                   Page 200

MSYSHOST.NET:

44.70.4.4 [ka8z] ka8z-4 ka8z.ampr.org        #Tom Barnes,canton,ka8z

44.70.4.6 [wa8bxn] wa8bxn wa8bxn.ampr.org    #Mike Pechura,lake

county,@wa8bxn

44.70.4.39 [kb7yw] kb7yw kb7yw.ampr.org #Fred Peachman, brookfield,

@wb8lvp

44.70.16.1 [ka8tef] ka8tef-4 ka8tef.ampr.org #Phil,Republic Ohio,

@ka8tef

NOTE:  ALL ENTRIES FOR EACH HOST MUST BE ON ONE LINE!

MSYSPASS.DAT:

guest * 3 c:/msys/public

anonymous * 3 c:/msys/public

ka8z tom 7 c:/msys/public

mike wa8bxn 7 c:/msys/public

fred kb7yw 7 c:/msys/public

ka8tef phil 7 c:/msys/public

The format is: call pass privilege base directory

And is used for FTP access to the system.

MSYSPASS.IP

wa8bxn apple wa8bxn

terry n8hsp n8hsp

fred kb7yw kb7yw

swiss normal no8m

The format is: call pass callsign

and is used for Telnet access to the bbs.

MSYS 1.16                                                   Page 201

Typical MSYSTNC.1

(AEA PK-232)

VHF OFF

HB 300

AWLEN 8

PARITY 0

CONMODE TRAN

TRACE OFF

HID OFF

BE 0

passall off

RAWHDLC ON

HP OFF

PP ON

KISS ON

RESTART  OR  HOST ON         !!!CONSULT YOUR TNC MANUAL!!!

NOTE:  The MSYSTNC.x files are associated with the corresponding

tnctype.x specified in the MSYS.OPT file.  The above file COULD be

named MSYSTNC.2 depending on your configuration.  Only the Kantronics

TNCs are fixed at MSYSTNC.0

NOTE: Consult your TNC manual to properly specify the proper

parameters for your tnc to put it into the KISS mode.

The TNC-2 clones, such as the MFJ 1270B or 1274, may benefit greatly

by using the JKISS EPROM which forces the tnc to be in the KISS mode

all the time.  The JKISS file is included on the distribution disk or

is available as an EPROM from the Tucson Amateur Packet Radio

organization.

MSYS 1.16                                                   Page 202

INFORMATION FOR ADVANCED USERS

REPLACEABLE DIRECTORY NAMES

Beginning in MSYS 1.04 you can specify alternate directory names to

be used by MSYS in place of the standard ones by using appropriate

DOS set commands.  The directory names that can be replaced and their

default values are:

Default             Set DOS variable

Dir Name            to replace

--------            ----------------

FILES               MSYS:FILES

FWD                 MSYS:FWD

GREP                MSYS:GREPDIR

HELP                MSYS:HELP

MAIL                MSYS:MAIL

YAPP                MSYS:YAPPDIR

As an example, assume you have a RAM disk at drive E: and have set up

a directory called HELP on E: and copied the normal help files to

this directory.  You can then issue the following DOS command before

starting MSYS:

set MSYS:HELP=E:HELP

and now MSYS will get the help files from the E:HELP directory.

NOTE:  You must be very careful in using a RAMDISK since its contents

are easily lost.  Also, you should probably not us a RAMDISK that

takes space out of the normal 640K address space.

NOTE:  You are not limited to using RAMDISKS in specifying the disk

and/or directory to be used for the above directories.  You could

specify floppy and/or hard disks as well.

To establish a separate directory that is "GReppable", use the DOS

SET command similar to this:

SET MSYS:GREPDIR=C:MSYS\GREP

Don't forget to create the C:\MSYS\GREP directory!

MSYS 1.16                                                   Page 203

REPLACEABLE FILE NAMES

You can also replace the following file names of files normally found

in  the "MSYS" (default) directory.  Judicious choices can speed up

operation and/or let you make use of multiple disk drives.  Poor

choices can be a disaster!

Default File   Set This DOS

Name           Parameter to replace:   Notes

-------------  --------------------- -----

MAIL.BAK       MSYS:MAIL.BAK          (6)

MAIL.BIN       MSYS:MAIL.BIN

BBSTONTS.BIN   MSYS:BBSTONTS.BIN      (1)

MSYS.FWD       MSYS:MSYS.FWD          (2)

MSYS.HRD       MSYS:MSYS.HRD          (3)

MSYS.LOG       MSYS:MSYS.LOG          (3)

MSYS.MSG       MSYS:MSYS.MSG          (4)

MSYS.USR       MSYS:MSYS.USR          (4)

MSYSHOST.NET   MSYS:MSYSHOST.NET      (5)

MSYSBBSB.DAT   MSYS:MSYSBBSB.DAT      (2)

MSYSPASS.DAT   MSYS:MSYSPASS.DAT      (5)

Notes:

1. Very good candidate for ram disk (read only)

2. Good to put on ram disk if you are careful

3. These can be made NUL if you don't want them

4. If you like to live very dangerously, consider putting them on

ram disk.  Otherwise keep them on hard disk .  Updated frequently

during normal operation, they are very important files.

5. Good for ram disk if you run tcp/ip much.  Read mostly.

6. A very strong suggestion would be to put this file on a different

disk partition.  Backup message files tend to get forgotten and

expand at a very high rate.  If they fill up the primary MSYS

disk, MSYS will no longer be able to accept messages.  If the

MAIL.BAK directory is on a different drive, the most that can

happen when that drive is full is that MSYS just will not make

backup copies of the incoming messages.

Example 1.  Lets move the mail.bak directory to the D: drive.  It is

appropriate to put the SET command in the AUTOEXEC.BAT so that it

will happen on a reboot.  You would put the statement:

SET MSYS:MAIL.BAK=D:\MAIL.BAK

in the AUTOEXEC.BAT file.  MSYS will then look for the mail backup

directory on the D: drive.

CAUTION  MUTIL Functions always assume the C:\MSYS directory path!

When doing MUTIL functions, you MUST specify the changed path!

MSYS 1.16                                                   Page 204

Example 2.  Assume you still have a ram disk at E: and have copied

the  normal contents of BBSTONTS.BIN to that disk.  You could do the

following DOS command:

SET MSYS:BBSTONTS.BIN=E:BBSTONTS.BIN

to cause MSYS to read the file from your RAMDISK.

NOTE: With all these changes you can make, you can configure a lot in

terms of files now.  Be careful of file sizes when thinking about

where to put things.  MSYS.LOG grows endlessly as does the MAIL.BAK

directory.  These two are VERY good candidates for another drive so

as to NOT accidentally run out of disk space and bring the bbs to a

humbled state.

NOTE:  The programs in MUTIL assume the files are where they should

be and totally ignore any replacements you may have made of file

names and/or directory names.

NOTE: If you are not absolutely sure of what you are doing, don't (do

anything with these facilities).

NOTE: Before you try any of this, MAKE BACKUPS OF EVERYTHING!

MSYS 1.16                                                   Page 205

MSYS MSG FILE FORMAT:(For the adventuresome only!)

(If you know what you are doing, this will make some sense to you)

int head;   /* slot number of head of active list */

int free;   /* slot number of head of free list */

int next;   /* next message number to be assigned */

/* Each slot has the following definition: */

struct msghdrdef {

unsigned  mmsgnr;

unsigned char mmsgtype[3];

int  mmsgsize;

unsigned char mmsgto[7];

unsigned char mmsgfrom[7];

unsigned char mmsgatbbs[39];

unsigned char mmsgdate[7];

unsigned char mmsgtitle[40];

long mmsgbits;

unsigned char mmhbbs[8];

unsigned char mmsgtries;

unsigned char mmsgbid[15];

int  mmsglink;

};

/* Note: word alignment is forced for all ints & longs */

MSYS 1.16                                                   Page 206

ROUTES.DAT FILE FORMAT:

This is an ASCII file that contains callsign prefixes and routings.

The callsign prefix begins in the first byte.  Comparisons are made

with the BBS call for the length of the prefix from this file.  The

routing field begins in byte 9 (a tab character can be used to get

here).  Here are some sample entries:

4X    ISR.AS

9M2   MYS.AS

A     USA.NA

The idea here is that incoming messages for a given bbs will get the

routing appended to the @BBS field.  Thus if someone sent a message

to 4X1ABC  @  4X1XYZ  the @ field would become @4X1XYZ.ISR.AS

It is expected in your forward file you would either have provisions

to route the country part (ISR) to some bbs,  or the continent part

(AS) to some bbs.  Thus to route foreign messages, you might just

have all the continent designators (like EU,  AS, NA  etc.) in your

forwarding file or you can be more selective. Remember that you can

have some countries along with the continent routings in your forward

file.  The part of the @BBS from left to right that is found in the

forwarding file is the one that is used.  If you do a lot of direct

forwarding to various countries that can be accommodated; if you just

dump all  foreign stuff  on a single station that is easy too.  Feel

free to modify the contents of ROUTES.DAT as you see fit;  I have

just included country prefixes for countries for which I seen message

headers pass through my system.

MSYS 1.16                                                   Page 207

The following program may be used to create a BBSLIST.DAT file

from the BBSTONTS.DAT file supplied on distribution disk:

10 OPEN "BBSTONTS.DAT" FOR INPUT AS 1

20 OPEN "BBSLIST.NEW" FOR OUTPUT AS 2

30 IF EOF(1) THEN CLOSE:END

40 LINE INPUT #1,A$

50 I=1:C$="":  WHILE  MID$(A$,I,1)<>" "  :C$=C$+MID$(A$,I,1):

I=I+1: WEND

60 D$=MID$(A$,35,6)

70 H$=MID$(A$,10,25)

90 IF LEFT$(D$,2)="99" THEN 30

100 X$=LEFT$(C$,1): IF X$>="0" AND X$<="9" THEN 120

110 X$=MID$(C$,2,1): IF X$>="0" AND X$<="9" THEN C$=" "+C$

120 PRINT #2,C$;TAB(8);H$;TAB(74);D$

130 GOTO 30

You may use a text editor to extract the program from this file. The

result of this program is put in a file called BBSLIST.NEW.  It may

be renamed to BBSLIST if you don't have one already.  Or you can

merge your file with this one by using the following commands:

copy BBSLIST.DAT+BBSLIST.NEW TEMP

del BBSLIST.DAT

ren TEMP BBSLIST.DAT

Then run MUTIL function 20 to sort the file and discard the

duplicates from the BBSLIST.NEW file.

MSYS under WINDOWS 3.1

To run MSYS in the background mode with Windows 3.1 you must make a

PIF file called MSYS.PIF.  The secret is when you get ready to save

the MSYS.PIF file, show the path as:

c:\windows\msys.pif

NOT

c:\msys\msys.pif.

Try using the variable 130 for background and 110 for foreground to

give the best results for timesharing.

MSYS 1.16                                                   Page 208

MSYS under DesqView

To run MSYS under DesqView 386 Version 2.4 on a 386 or a 486 machine,

try these values in the "Add Program" (AP) setup:

O (for Other) F1 (to specify Program Information)

PROGRAM NAME = MSYS

Keys To Use = MS (or your choice!)

Memory Size = (as big as you can make it - try 620K)

Program  = C:\MSYS\MSYS.EXE (be sure to specify proper drive and

sub-directory)

Directory = C:\MSYS

Writes text directly to screen     [Y]

Displays graphics information [N]

Virtualize text/graphics (Y,N,T)   [Y]

Uses serial ports (Y,N,1,2)        [Y]

Requires floppy diskette           [N]

F1 for advanced options

Starting Height:                   25

Starting Row.                       0

Starting Width.                    80

Starting Column:                    0

Close on exit (Y,N,blank)          [ ]

Uses its own colors                [Y]

Allow Close Window  command        [Y]

Runs in background (Y,N,blank)     [ ]

Uses math  coprocessor             [Y]

Keyboard conflict (0-F):           [0]

Share   CPU   when   foreground    [Y]

Share EGA when foreground/zoomed   [Y]

Can be swapped out (Y,N,blank)     [ ]

Protection level (0-3)             [0]

To run MSYS under DesqView on a 386 machine, try these values in the

"performance" section:

foreground 3

background 2

common memory 15

dos buffer for ems 5

optimize communications Y

allow swapping of programs N

manage printer contention N

MSYS 1.16                                                   Page 209

If you choose to use QEMM as your memory manager, the following line

in your CONFIG.SYS file should give satisfactory results:

DEVICE=C:\QEMM\QEMM386.SYS RAM ROM

NOTE:  A WORD OF CAUTION:

When using QEMM-386, the /ST:M option in the DEVICE=  line will cause

MSYS to not load properly.  There may be other options which will

also cause grief.  These suggestions are not necessarily the optimum

settings for your system,  but they have been known to work.

If you are running QEMM version 7, and are NOT running under

DesqView, running the command:  LOADHI /LINK before loading MSYS will

make additional memory available to MSYS.

If you are running QEMM and are running under DesqView, if you have

no other need for graphics, you may use the: VIDRAM ON command to

make additional memory available to MSYS.

MSYS 1.16                                                   Page 210

SYSOP COMMANDS RELATED TO TCP/IP

MYIpaddr

Takes four integers in the range 0 to 255.  It sets the IP

address that the system will respond to.  Example: MYI 44 70 4 6

If MYIpaddress is not specified, or set to all zeros, the TCP/IP

support will be inactive.

MYTcpcall

Takes a callsign (with SSID) that will be used when sending arp

and TCP/IP frames.  It can be the same as the ID  call in most

cases.

TElnet

Takes an IP address and optional server number as parameters.  A

symbolic IP address may given to be looked up in the file

MSYSHOST.NET.  Default server is 23.  This command is used to

establish keyboard to keyboard connection with a TCP/IP station.

Examples: TE [44.70.4.10]

te hsp   (hsp would have to be defined in

MSYSHOST.NET)

TElnet operates much like C# normally used to connect to AX.25

stations.  ^Cd is used to disconnect.

TPorts

Takes a port mask value as an argument.  This number is used the

same way as in the other similar commands: each port corresponds

to a port (port 0 is the rightmost bit) and when the bit is set

to 1 for a given port that port is available for TCP/IP use

(particularly ARP broadcasts).

ARP

Used without an argument lists the known callsign/IP address

correspondence determined by TElnet connects.  An optional

argument, CLEAR, may be used to erase all of the ARP entries.

FTP

Takes a host-id as an argument (it can be either an IP address

in the square brackets or a symbolic IP address).  It

establishes a connection to the FTP server of the specified

system.  FTP is the File Transfer Protocol used in TCP/IP.

MSYS 1.16                                                   Page 211

SMTP

Allows local access to the SMTP server (mainly for test

purposes).

Note: The following servers are currently implemented in MSYS:

7     Echo

9     Discard

21    BBS

23    Telnet

25    Simple Mail Transfer Protocol

87    Local keyboard

Files related to TCP/IP

MSYSHOST.NET

This is an ASCII file you create with an editor (such as the

MSYS EF command).  Each line in the file begins with an IP

address that includes the periods but not the square brackets

around it.  Following this there is at least one space and/or

tab character, then the list of symbolic names (or aliases, or

nicknames, what ever you want to call them).  You may place

comments on the line by beginning the comments with a pound sign

(#).  Example entry line for this file:

44.70.4.6 bxn mike wa8bxn # kirtland, oh

This line would allow "bxn" "mike" or "wa8bxn" to be used as

arguments in the TElnet and FTP commands. Note that WA8BXN here

just happens to look like a callsign.  The actual callsign (and

SSID) associated with a particular IP address is determined by

ARP (address resolution protocol) broadcasts.  These symbolic

names you define in this file simply relieve you of having to

remember (and type) complete IP addresses.  Nothing more,

nothing less!  See sample for MSYSHOST.NET file.

MSYS 1.16                                                   Page 212

MSYSPASS.DAT

This file contains the user names and passwords for FTP users.

Each line contains 3 or 4 items: username password access

directory Username is the name the user will use to log in.  It

can be anything for a given user.  It could be their callsign or

something else.  The password can also be anything.  If you put

* as the password in the file for a particular, any password

given by the user will be accepted.  The third entry on each

line, access, is a number that says how much access the user

will have to your system: 1 means can only read files, 3 means

read and write (but not replace or delete) and 7 means read,

write, replace and delete.  You shouldn't set 7 for any user

since the passwords given by a user can be monitored by others.

The last item which should always be given is the root directory

that the user will have. They may never go closer to the real

root than what you give here.

Here are some examples:

anonymous * 1 files      This allows user anonymous to read

any files in MSYS/FILES and any of

its subdirectories.  Any password can

be

given for this user.  Anonymous with *

for the password is an expected

"standard" entry for  TCP/IP systems to

give access to totally public files.

Include it in your system.

apple runtime 3          This allows user apple (who must give

password runtime) read/write access

to your entire disk (DEFAULT DIRECTORY

IS THE REAL ROOT DIRECTORY!)

MSYS 1.16                                                   Page 213

USER ACCESS level chart

Level 1: CD DIR GET PWD QUIT

Level 3: PUT MKDIR

Level 7: PUT (to replace existing file) DELE RMDIR

If you are going to allow level 7 access (i.e., can do everything) I

would suggest you also assign a directory that doesn't allow access

to any needed files.  For example, you might set up the following

entry in the MSYSPASS.DAT file:

WA8BXN MIKE 7 /MSYS/FILES/WA8BXN

You will have to create a directory WA8BXN in the files directory.

Now stations can log in with WA8BXN MIKE and do anything in

/MSYS/FILES/WA8BXN and any directories found in this directory.

If you set up ANONYMOUS * 1  /MSYS/FILES then the user ANONYMOUS can

read anything put in the WA8BXN subdirectory of FILES (as well as in

FILES itself) but not write or otherwise change anything.  The rest

of your files should be pretty well protected.

FTP COMMANDS:

When you use the FTP command to connect to another system you can

type: HELP to get a list of the supported commands.  They currently

include:

CD dirname     To move around in the directory structure

CD             with no arguments gives the current directory

DELE filename  To delete given file

DIR filespec   gives a directory listing

GET filename   downloads a the given file

MKDIR          makes a directory

PUT filename   uploads the given file  Syntax is:

put [localfilename] remotefilename

If the optional localfile name is omitted it is

assumed to be the same as the remote file name.

PWD            gives the current directory

QUIT           disconnects

RMDIR dirname  removes the given directory

FTP function   will give an estimate on the number of bytes

transferred during GET and PUT commands.

MSYS 1.16                                                   Page 214

IP Routing:

What does this mean?  Its sort of like digipeaters for the TCP/IP

world.  If a TCP/IP user sets you up as a route for IP frames going

to a particular IP address MSYS will now repeat them if it has (or

can get through an ARP request broadcast) a  known route to the

DEStination IP address.  Consider the following fictitious example:

W9AAA [44.60.1.1] is in Indiana and wishes to connect to W3AAA

[44.80.1.1] in Pennsylvania but can't do it directly.  W9AAA can

reach W8AAA [44.70.1.1] in Ohio who can in turn reach W3AAA.  W9AAA

will set up the route to [44.80.1.1] to be W8AAA (an  MSYS system).

When attempting to connect to W3AAA (using Telnet,  for example) will

send out a SYN IP  frame (connect request) to callsign W8AAA but with

an IP address of [44.80.1.1], the IP address of W3AAA.  W8AAA upon

hearing this will check its ARP table to see if it knows how to get

to [44.80.1.1].  If the entry is not there, W8AAA will do an ARP

broadcast on all the ports enabled for TCP/IP.  W3AAA will respond on

one of them and this will put an entry in the ARP table.  Now when a

SYN is heard from W9AAA it will be repeated by W8AAA and thus heard

by W3AAA.

To use IP ROUTING yourself in MSYS you use the arp add command.  When

adding a given IP address you would put the call of the TCP/IP

station you want to use as an IP repeater in place of the call that

belongs with the IP address.  You can still use normal AX.25

digipeaters (up to 8) to reach the TCP/IP station.  If the path from

W8AAA to W3AAA in the above example was not direct,  then the

following arp entry could be done at W8AAA, assuming that K8AAA is a

normal AX.25 digipeater: arp add W3AAA port# 44 80 1 1 K8AAA

NOTE:  For more on TCP/IP refer to The KA9Q Internet Software

Package.

In the TCP/IP related ARP command, use: -2 as the port number for

routing through the network interface.

TCP/IP connects are rejected if not enough memory or MCON is

exceeded.

MSYS 1.16                                                   Page 215

EDITVIEW (ED Command)

(c) Copyright 1993 by HUB COMPUTERS, INC.

Commands    The ED command used from the MSYS command mode can be

used to edit small text files.  The file is limited to records each

80 bytes or less in length.  By default up to 200 records may be in

the file.  This can be modified by giving the maximum number of lines

after the file name.  The intent of this editor is to do little

things without having to take down the system to run a more powerful

editor.  These might include editing message texts, the forward file,

etc.

ED is a full window editor (it adjusts its operation to fill the

window it is run in).  It must be run in a window that is 80

characters wide.  Thus using the default window definitions it can be

run in the normal command window (0) or you can use the CMdwindow

command to first go to window 4 (the one you get when you press F4)

and then use the ED command.  You will then be using the entire

screen for your editing.

ED is pretty much a what you see is what you get editor.  There are

two modes of operation.  When you start the editor you are in full

screen or edit mode in which changes you make to the screen are also

made to the file.  In this mode you get a line at the bottom of the

window that gives such information as the current line number and the

name of the file being edited.  The other mode is command mode in

which you give commands to be done by the editor.  To enter command

mode press the Escape key.  The bottom line will change to Edit Cmd:

and await your command.  The command you type will appear on this

bottom line.  To get a list of available commands type Help.  You

should see something that looks like the following:

Find - locates given string .  Example:  F wa8bxn

Notes: Not case sensitive .  F10 from edit mode is find

same string again.

Insert - puts blank line at beginning of file

QUIT - Abandons edit

SAVE - Saves file and exits

DELete  -  Delete specified # of lines from  file  beginning with

current line

Return key in edit mode with insert on inserts blank lines

Press a key to return to edit

MSYS 1.16                                                   Page 216

Use of the keypad to move around in the file is what is expected of

the labeled keys, with the following enhancements.  If you are

somewhere in a line and you press the Home key,  you go to the

beginning of the current line.  If you are the beginning of the

current line and press Home,  you go to the beginning of the screen.

If you are at the beginning of the screen, pressing Home takes you to

the beginning of the file.

MSYS 1.16                                                   Page 217

EDITFILE (EF Command)

(c) Copyright 1993 by HUB COMPUTERS, INC.

The EF command used from the MSYS command mode can be used to edit

small text files.  The file is limited to records each 80 bytes or

less in length.  By default up to 200 records may be in the file.

This can be modified by giving the maximum number of lines after the

file name.  The intent of this editor is to do little things without

having to take down the system to run a more powerful editor .  These

might include editing message texts, the forward file, etc.  This

isn't a full screen editor but rather a line editor that was

influenced in its design by the UNIX editor.  Plain and simple

nothing fancy is found in it.  To begin editing, use the command EF

filename when you have the cmd: prompt.  The filename can contain

drive and path information.  If the file does not exist you will be

given the option to create one by that name.  To create a new file

this way, use the Append command.  Type the lines of the new file and

then a line with a period at the beginning to exit append mode.  Then

use the  commands Write and Quit (each followed by the return key)

and you have created a new file.  All of the commands can be

shortened to the first letter of the command, except for the DElete

command which requires at least the first two letters.

Here is information about the edit commands:

#  (i.e.,  a line number) makes that line the current line and also

displays it

+ # Makes the current line # more than it is now .  If # is omitted,

then the current line is incremented by one.

- # Same as + # except current line number is decremented.

<ESCAPE> Using the Escape key followed by the Return key abandons

edit with no changes made to the file (unless you used the Write

command yourself).

<RETURN> Pressing the Return key alone displays the next line and

makes it the current line.

Append enters input mode, placing the following lines of input  AFTER

the current line.  A line beginning with a period terminates append

mode.

Bottom makes the last line the current line and displays it.

MSYS 1.16                                                   Page 218

Change can be used to replace part of the current line.  The first

non-blank character after the command is the quote character.  The

operands are quote-character string-to-replace quote-character

replacement-string quote-character .  For example,

C /YXZ/abc/

would replace XYZ in the current line with abc.  The line is

printed after it is displayed.

DElete can be used to delete the current line.  If a number is

specified as an operand, it is the number of lines to delete

beginning with the current line.

Down moves the current line down the number of lines specified (or 1

if none is specified).

Find uses the first non-blank character after the command word as the

beginning of the search string.  The search is from the current line

on in the file.  Case is NOT important.  If a match is found the line

is displayed and that line becomes the current line.  If no match is

found the current line remains unchanged.

Help displays the list of commands.

Insert is like Append except that the new lines are placed BEFORE

the current line.

Near displays the contents of the file 5 lines before and 4 lines

after the current line which remains unchanged.

Print with no operands displays the current line.  Give 1 operand it

displays the line with that number.  Given 2 operands (separated with

a blank or comma) it displays from the first to the second line

number.  The current line number remains unchanged.

Quit is used to exit the editor after the file has been written.

Top makes the first line (numbered 0) the current line and displays

it.

Up decrements the current line number by the number given or 1.

MSYS 1.16                                                   Page 219

View displays the current line and the 9 lines that follow it.  The

current line remains unchanged.

Write causes the file to be written to disk with any changes that

have been made.

^F is used to search again for the same search string give with last

use of Find.

To start the editor, type EFile filepathname [# of lines max].

Examples:

ef msys.fwd 250

ef mail/msg25.dat

efile a:f1.c

MSYS 1.16                                                   Page 220

Epilog

Notes on sending me messages with questions/suggestions/etc.:  I do

try to respond to your personal messages.  Keep in mind I do get

quite a few at times.  If you want an answer from the author, you

need to send the question to the author personally!  Messages that

contain demands (add this feature or correct this bug immediately and

put out a new release or I won't use your program) generally are

saved in the bit bucket.  For a free program, please be considerate!

Try to put just one thought/question in each message.  If you bundle

up a whole bunch, I don't respond until I can address all your

issues.  If you send follow-up messages, please give a sentence or

two summary of the problem so that I may understand what you are

talking about.  Just saying "I tried what you suggested and it still

doesn't work, anything else to try?" doesn't give me much of a clue

about the situation.  Although you may have only one problem you are

dealing with and can easily remember its entire history,  I don't

have that luxury here!  If you find what you think is a bug,  please

try to tell me how to reproduce it, what you are seeing, and what you

think should be happening.  The harder the problem, the longer it

takes me to work on it and respond.  When you run into any sort of

problem, simplify  everything as much as possible.  Remove all

statements from config.sys, autoexec.bat, msys.opt and msys.do that

are not essential to reproduce the bug.  Believe me, this will

probably help you find at least half of the problems and their

solutions yourself.

Use and Enjoy!

MSYS 1.16                                                   Page 221

INDEX

!, 118                               Automatic Password Reply, 163

!timestring, 118                     AUTONode, 38, 133, 139

#, 36, 107, 153                      AX25 CHECK, 20

$POOL, 67                            AX25 FRACK, 20

*, 87                                AX25 MAXFRAMES, 20

*** done, 114                        AX25 RETRIES, 20

*comment, 36                         AX25Backoff, 38

+, 108                               AX25L2v2, 38

.FWD, 111                            BADuser, 38

.REV, 111                            BBS, 212

/, 87                                BBs, 38

/Sysop, 162                          BBS ALIAS, 19

<, 87                                BBS CALL, 19

<Esc>, 74                            BBS callsign, 63

>, 17, 87, 164                       BBS ONLY ACCESS, 153

>>, 87                               BBSLIST.DAT, 186, 187, 192

?, 74, 77, 108                       BBSNode, 39

@, 108                               BBSTimeout, 39

@BBS, 84                             BBSTONTS, 129

@home, 36                            BBSTONTS.BIN, 128, 129, 191, 204

^A, 84                               BBSTONTS.DAT, 53, 187, 189, 190,

^Z, 84                                191

A, 76                                BDigi, 39

ABortfwd, 36                         BElloff, 39

ACOUSTIC COUPLED MODEMS, 102         BID, 39, 84, 118, 168

ADDUser, 36                          BIDLIST.DAT, 39

Alt-F1, 75                           BINterval, 40

Alt-F2, 75                           BInterval, 139

Alt-F3, 75                           blank lines, 118

AMTOR, 141                           BMaxcon, 40

ANONYMOUS CHECK-INS, 100             BOOT, 40

ANSWER CALL, 19                      BPorts, 40

APpend, 36                           BText, 40, 62

ARChive, 36                          BUDCall, 38

archived messages, 191               BUDCalls, 41, 59

ARP, 36, 60, 211                     BUDList, 41

arp, 117                             BUFFERS, 171

ARP ADD, 37                          BUSY, 40

ARP CLEAR, 36                        Bye, 76

ARP DEL, 37                          C, 76, 139

ARP REP, 37                          C script, 42

ATtended, 37, 120                    C#, 43

AUTODESt, 37, 133, 139               CALLSIGN SERVER, 90

AUTOEXEC.BAT, 90, 204                Callsign Server, 63

AUTOHold, 37                         CALLSRV1, 90

AUTOKill, 38, 167                    CALLSRV2, 90

Automatic Message Holding, 157       CALLSRV3, 90

MSYS 1.16                                                   Page 222

callword, 98                         DOS SET, 77

Cancel, 74                           Download, 77

CC, 76                               DPorts, 47

CD ROM, 90                           DRoute, 47

CGA, 68                              DRoute DELete, 47

Change maximum number of             DRSI CARDS, 27

messages, 177                       DTimedate, 47

CHANNEL NUMBERS, 153                 DUmpbbs, 47

CHAnsta, 41                          Echo, 212

CHEck, 41                            EDit, 47

Checksum error, 124                  EDITFILE (EF Command), 218

CHKMAIL, 180                         EDITVIEW (ED Command), 216

CL B#, 41                            EFile, 47

CLrscrn, 41                          EMsghdr, 48

CMdwindow, 42                        END OF LINE BELLS, 154

COLOR, 72                            EU, 100

CONFIG.SYS, 90, 171                  EUser, 49

COPy, 42                             EXPANDED MEMORY, 157

CRetries, 42                         EXTENDED, 157

Ctrl/F2, 75                          F:HAMBASE, 90

Ctrl/F4, 65, 75                      F1, 75

Ctrl/F5, 75                          F2 Time, 49

Ctrl/F6, 75                          F6, 75

CTS, 43, 94                          F8, 75

DATABASE HELP, 103                   FA, 123

Database.0, 77, 103                  FB, 122

datagram, 132                        FBB, 99, 115

DBADD, 103                           FBB COMPATIBLE COMPRESSED

DEADletter, 44                        FORWARDING, 123

DEBug, 45                            FBB COMPATIBLE FORWARDING, 122

DELEfile, 45                         FBB ON THE PHONE, 99

Delete Old BIDS, 193                 FBBC, 50, 105

DELUDays, 45                         FBBStimes, 50

DELUSer, 45                          FCall, 50

DesqView, 209                        FF, 123

DESt, 73                             FFile, 50

DESt ADD, 45, 134, 139               FIle, 50

DESt DEL, 45, 134, 139               FILES, 171, 203

DESt REP, 45, 134, 139               Files related to TCP/IP, 212

DEStination, 117, 134                files=40, 9

DEStination NODe, 132                Fix mail index file, 187

Device Errors, 153                   FMsg, 51

DFree, 46                            FOrward, 51

DIGipeats, 46                        FORWARD CALL, 19, 138

DIR, 130                             Forward File Description, 105

DIRectory, 46                        FORWARDING DISPLAY, 119

Discard, 212                         FORWARDING NOTES, 118

DISconnect, 46                       FORWARDING ON THE MODEM PORT, 98

Disk Full Message, 153               FQ, 123

DOcmdfile, 46                        FRACK, 71

DOS, 154                             FRAck, 51

DOS 3.3, 174                         FREE, 51

MSYS 1.16                                                   Page 223

FREQUENT PROBLEMS, 171               JGateways, 54, 78

FS, 122                              JH, 78

FTIme, 51                            JHeard, 54

FTP, 201, 211                        JKanodes, 55, 78

FTP COMMANDS, 214                    JKISS EPROM, 161

FTPgm, 52                            JMsys, 55, 78

FUllduplex, 52                       JNetrom, 55, 78

Function key usage, 35, 75           JTcp/ip, 55

FWD, 203                             Justheard, 138

fwd directory, 118                   K, 78

G, 104                               K*, 78

GEtmsgotd, 52                        K<, 79

GMT, 70                              K>, 79

GMtime, 52                           K@, 79

GR, 77                               KAM Plus, 141

GREP, 203                            KANODE CALL, 19

GRep, 52                             KANTRONICS 3.0 EPROM, 170

grepdir, 77                          KEYBOARD, 154

GX, 77                               KEyboard, 55

H, 77                                KF, 78

HApnstat, 52                         KILL, 160

HARDWARE, 141                        Kill, 78

HCAll, 52, 167                       KILLproc, 55

HCL, 130                             KM, 78

HCLean, 53, 184                      KPorts(MASK), 56

HEADERS, 154                         KT, 78

HEard, 53                            L, 79

HELP, 14, 203                        L#, 80

HF Forwarding, 120                   L$, 80

Hierarchical, 117                    L$ # #, 80

HIERARCHICAL FORWARDING, 127         L$$, 80

HOLdlevel, 53, 159                   L<, 79

HOStname, 53                         L>, 79

HReplace, 53                         L?, 80

I, 77                                L@, 79

ID, 53, 77                           LB, 79

ID CALL, 20                          LC, 79

ID EVERY, 20                         LCount, 56, 119

IGNORE, 54                           LF, 80

IMport, 54                           LH, 80

INitpr, 54                           LINKED, 154

IP, 70                               LL, 79

IP Routing, 215                      LL$ #, 80

IPNCheck, 54                         LM, 79

IPNFrack, 54                         LN, 79

IPNRetry, 54                         LO, 79

IPTtl, 54                            LOADHI /LINK, 210

IRQs, 155                            Local keyboard, 212

J, 138                               LOCKkbd, 56

j-com, 90                            LOCKUPS, 155

JBbs, 54, 78                         log file analyzer, 183

JDigipeat, 54, 78                    LOGclose, 56

MSYS 1.16                                                   Page 224

LT, 79                               MONOCHROME, 72

LU, 79                               MONOutgoing, 59

LUsers, 56                           MONPorts, 59

LY, 80                               MONSframe, 59

M, 80                                MONUframe, 59

MAIL, 14, 203                        MONxx, 59

MAIL BEACON, 155                     More?, 164

MAIL.BAK, 14, 66, 155, 204           MS-400, 155

MAIL.BIN, 14, 204                    MSgtrailer, 59

MAKEPrivate, 56, 167                 MSYS COMMAND LINE, 12

MASK, 71                             MSYS Command Summary, 28

MASK INFORMATION, 25                 MSYS DIRECTORY LAYOUT, 14

MAStersrv, 56                        MSYS GENERAL INFORMATION, 153

MAX, 155                             MSYS MSG FILE FORMAT, 206

max hops, 63                         MSYS Network NODe, 132

MAXBullag, 56                        MSYS NODe Forwarding, 120

MAXCHANS, 155                        MSYS PACTOR SCANNING, 144

MAXFrames, 57                        MSYS SYSOP Commands, 36

maximum number of messages, 177      MSYS.BBS, 14, 112, 118

MAXRead, 57                          MSYS.DEF, 16

MCForward, 57                        MSYS.DO, 91, 120, 136, 160, 195

MCON, 85                             MSYS.EXE, 14

MCOn, 57                             MSYS.FWD, 58, 112, 172, 204

MDir, 57                             MSYS.HCL, 130

MEMory, 58                           MSYS.HRD, 171, 204

MEMORY LOW BUSY, 156                 MSYS.LOG, 204

MEMORY MANAGEMENT, 156               MSYS.MSG, 177, 204

MERge, 9, 58, 112, 192               MSYS.OPT, 15, 171

MEXpert, 58                          MSYS.OPT FILES, 94

MFJ KISS MODE, 174                   MSYS.OTD, 198

MHclear, 58                          MSYS.REP, 113, 198

MIDchar, 58                          MSYS.RMT, 72, 162, 163, 179, 199

MINmem, 58                           MSYS.USR, 204

MISsing, 58                          MSYS:BBSTONTS.BI, 204

MISSING TEXT TO Message, 121         MSYS:FILES, 203

MODE MONO, 9                         MSYS:FWD, 203

MODEM, 17, 94                        MSYS:GREPDIR, 203

Modem                                MSYS:HELP, 203

Hayes Ultra 96, 96                  MSYS:MAIL, 203

US Robotics Sportster, 96           MSYS:MAIL.BAK, 204

Zoom, 95                            MSYS:MAIL.BIN, 204

MODEM SUPPORT, 92                    MSYS:MSYS.FWD, 204

MONBframe, 59                        MSYS:MSYS.HRD, 204

MONCC, 59                            MSYS:MSYS.LOG, 204

MONCF, 59                            MSYS:MSYS.MSG, 204

MONIFrame, 59                        MSYS:MSYS.USR, 204

MONITor, 59                          MSYS:MSYSBBSB.DAT, 204

MONITOR BFRAMES, 21                  MSYS:MSYSHOST.NET, 204

MONITOR IFRAMES, 20                  MSYS:MSYSPASS.DAT, 204

MONITOR OUTGOING, 21                 MSYS:YAPPDIR, 203

MONITOR PORTS, 21                    MSYSB.EXE., 156

MONITOR SFRAMES, 20                  MSYSBBS.DAT, 105

MSYS 1.16                                                   Page 225

MSYSBBSB.DAT, 102, 110, 172,         NET NODe Parameters Summary, 139

199, 204                            NETBbs, 60, 139

MSYSFWD.DEF, 9, 58, 105, 110,        NETCon, 60, 139

112, 114, 172, 200                  NETFBBC, 60, 140

MSYSHCL.DO, 131                      NETFBBG, 61, 140

MSYSHOLD.DAT, 100, 157               NETFBBStime, 61, 140

MSYSHOST.NET, 63, 201, 204           NETMax, 140

MSYSK.OTD, 198                       NETNode, 61, 97, 133

MSYSMOD.MSG, 101                     NETOnly, 61, 140

MSYSPASS.DAT, 163, 201, 204, 213     NETRead, 61, 140

MSYSPASS.IP, 164, 201                NetRom RS-232 Connections, 164

MSYSPASS.MOD, 92, 97, 100, 164       NetRom Serial protocol, 17, 164

MSYSTNC.#, 161                       NETTalk, 61, 139

MSYSTNC.1, 18, 202                   Network time to live, 63

MSYSTNC.2, 95                        NETXBatchsize, 61, 139, 140

MSYSTNC.x FILES, 95                  NETXOption, 139, 140

MSYSTODO.DAT, 71, 160, 173           NETYapp, 61

MUTIL, 129, 171                      NEWUser, 61

MUTIL 4, 171, 173                    NH, 80

MUTIL Function 1, 177                NInfo, 61, 139

MUTIL Function 10, 186               NNode, 62

MUTIL Function 11, 187               NOBEacons, 62

MUTIL Function 12, 187               NOBUlls, 62

MUTIL Function 13, 189               NODe, 62, 132, 133, 134, 135

MUTIL Function 14, 191               NODe ADD, 62, 134, 139

MUTIL Function 15, 191               NODe DEL, 62, 139

MUTIL Function 16, 191               NODe DELete, 134

MUTIL Function 17, 192               NODe REP, 62, 134, 139

MUTIL Function 18, 192               NODe REStore, 62, 139

MUTIL Function 19, 192               NODe SAve, 62, 139

MUTIL Function 2, 178                NODe Scripts, 108

MUTIL Function 20, 193               NODe SEnd, 139

MUTIL Function 21, 193               NODETimeout, 62

MUTIL Function 22, 193               NOEMM, 12, 157

MUTIL Function 3, 179                NOEXMEM, 12

MUTIL Function 4, 180                NOHEARD, 12

MUTIL Function 5, 181                NON-AMATEUR CALLSIGNS, 153

MUTIL Function 6, 182                NONODE, 12

MUTIL Function 8, 183                NORAMHDRS, 12

MUTIL Function 9, 185                NPorts, 62, 138, 139

MUTIL.EXE, 8, 176                    NQ, 80

MYAlias, 60                          NRCall, 62, 133, 138, 139

MYBbs, 60                            NRId, 63, 139

MYCall, 60                           NRS, 17

MYIpaddr, 60, 211                    NTS traffic, 78

MYKnode, 60                          NTtl, 63, 139

MYTcp, 60, 133, 138, 211             NUMCHANS, 19

N, 80                                NZ, 80

NAME, 16                             obsolescence, 63

NBdcast, 60, 139                     OCInit, 63, 139

NDwindow, 60, 139                    OCMin, 63, 137, 139

neighbor NODe, 132                   OS2 REPORTED PROBLEMS, 175

MSYS 1.16                                                   Page 226

OVERVIEW, 141                        REMOTE SYSOP, 72, 162

P, 81                                REMove, 65, 134

Packet size, 162                     REMove DESt, 139

PAClen, 63                           REName, 65

PACTOR, 141                          RENUMBERING MESSAGES, 188

PACTOR FORWARDING, 142               REPLACEABLE DIRECTORY NAMES, 203

PACTOR LIMITATIONS, 143              REPLACEABLE FILE NAMES, 204

PACTOR MANUAL CONNECTS, 142          REPlaces, 65

PACTOR SET UP, 141                   REPly, 82

PACTOR TNC PARAMETERS, 142           REQDIR, 66, 88

PACTOR TROUBLESHOOTING, 144          REQFIL, 66, 88

password, 14, 98, 100, 162           REQMaxfil, 66

PASSWORD DIRECTORY, 163              REQQTH, 66, 88, 90

PASSWORD.RMT, 72, 163                REQxxx SERVERS, 88

PATh, 63                             RES, 82

PC, 63, 90                           RESPonsetime, 66

PF, 81                               RESTore, 66

PForward, 63                         RETries, 66

PIng, 63                             REVERSE FORWARD, 111

PMessage, 63                         REVERSE FORWARDING, 114

poll, 115                            Reverse forwarding, 115

Port speed, 94                       RH, 81

POrts, 64                            RM, 81

PPersist, 64, 161                    RN <msg#>, 81

PQuality, 64, 139                    ROUTES.DAT FILE FORMAT, 207

PRInt, 64                            Routing, 132

Print forwarding tree, 182           RS, 82

Print remote sysop phrase number     RUn, 67

table, 179                          S, 83

printer, 193                         SAM (c), 90

Printer status, 191                  SAMPLES OF MSYS FILES, 194

PROcesses, 64                        SB, 84

PRTmsgs, 64                          SCreensave, 68

PScan, 65                            SELECTIVE FORWARDING:, 115

PUBLIC, 14                           SEnd, 68

PWindow, 65                          servers, 212

QEMM, 175, 210                       SET, 204

Qmsg, 65                             Set, 204

QUIT, 65, 75                         SET TZ, 174

R, 81                                SETUP, 10

R # #, 81                            setup, 185

R*, 82                               Simple Mail Transfer Protocol,

R<, 81                                212

R>, 81                               SLottime, 68, 161

R@, 81                               SMTP, 68, 82, 117, 164, 212

RADIOSPEED, 17                       SMTP FORWARDING, 117

Radiospeed, 94                       SNow, 68

RBid, 65                             SOrt, 68

RDir, 65                             Special KISS Parameters, 161

Readbackup, 65                       SR, 82

Register 27, 95                      SSID, 138

REM, 81                              ST, 84

MSYS 1.16                                                   Page 227

STACK, 22, 165                       VIDRAM, 156, 210

STACKS, 154, 157                     visit, 100

STatus, 69, 121                      W, 85

SUspendtrace, 69                     W0RLI STANDARD FOR DATA

SYSOP, 82                             COMPRESSION, 125

SYSOP NAME, 23                       WAtch, 72

SYSOP QTH, 23                        WBackgrnd, 73

SYSOP ZIP, 23                        WColors, 73

SYSTEM REBOOTS AFTER FORWARDING,     WForegrnd, 73

173                                 WHO, 73, 162

T, 85                                WHY, 73

TABs, 69                             Wildcards, 119

TACK, 69, 139                        wildcards, 80

TBdelay, 70, 139                     WINDOW, 21

TCP/IP, 60, 71, 133, 138, 211        Window, 73

TCP/IP FORWARDING:, 116              WINDOW DISPLAY MONOCHROME, 9

TElnet, 70, 211                      WINDOWS 3.1, 208

Telnet, 201, 212                     WP, 100

Test serial ports, 181               WP SERVER, 170

TESTING, 166                         WPAge, 73

TFrames, 70                          WPEvery, 73

TImezone, 70                         WQupdate, 73, 139

Titles Truncated, 166                WRecord, 74

TNatime, 70, 139                     WSize, 74

TNC-2, 72                            X, 49, 86

TOdo, 71                             XBatchsize, 74

TPorts, 71, 211                      XC, 49, 86

Trace25, 71                          XF, 49, 86

TraceIP, 71                          XOption, 74

TREtries, 71, 139                    XR, 49, 86

TRUncate, 71                         XS, 86

TS, 71                               YAPP, 10, 14, 86, 203

TSR, 90                              Yapp, 74

TTimeout, 71, 139                    YAPP download, 86

Tucson Amateur Packet Radio,         YAPP upload, 86

161, 202                            YD, 86

TXD, 161                             YU, 86

TXDelay, 71                          YW, 86

TXT, 161

TXTail, 72

TYpe, 72

Type of Message, 166

U, 85

UPload, 85

USER BIT SETTINGS, 169

USER Commands, 76

USER PASSWORD, 163

Users, 72

V, 85

V1SIT, 100

VErifysysop, 72

VIdeo, 72

MSYS 1.16                                                   Page 228