USERMAN.DOC - ARES/Data Remote Packet User Information

Be sure to also read QUIKREF.DOC - Packet Operator Quick Reference

Version 1.6

OVERVIEW OF ARES/Data

Briefly, ARES/Data may be regarded as a specialized multiple-port,

multiple-connect database with a specific command set tailored to the

handling of information input, search, listing, and summary requests.

In addition, the system provides a full-featured conference bridge so

that all connected stations may converse conveniently with one another.

The ARES/Data network is a star network with the ARES/Data database

machine at the hub.  It looks something like this:

_______ARES/Data Database Machine_______

|           |             |             |

Port A       Port B       Port C        Port D

|           |             |             |

radio       radio         radio         radio

..          ...           ...           ....

. .         . . .         . . .         . . . .

. . .       .  .  .       .  .  .       .  .  . .

. .   .     .  . .  .     .  . .  .     .  . .  . .

P P    P    P  P P  P     P  P P  P     P  P P  P P

Each "P" represents a remotely-connected packet station which is

connected to the ARES/Data database machine.  All the remotely-connected

stations have shared access to the data in the system.  In particular,

the packet operators can utilize two groups of functions provided by

ARES/Data which are described in detail in this file:

I.  send/receive status requests and current information to/from

the ARES/Data database station

II. send/receive short messages to/from other remotely connected

stations or the sysop (Conference Bridge)

A.  CONNECTING TO THE ARES/Data DATABASE STATION

TNC SETTINGS FOR REMOTELY CONNECTED PACKET STATIONS:  These are

quite important for efficient operation of a master/slave network

like ARES/Data!  Use the commands appropriate for your type of TNC:

TAPR, AEA, PACCOMM:                     WA8DED:

DWAIT 25 (250 MS)                      * W 25 (250 MS)

MAXFRAME 1                             * O 1

FRACK 10 (10 SEC)                      * F 10 (10 SEC)

RETRY 10                               * N 10

AX25L2V2 ON                            * V 2

RESPTIME 10 (1.0 SEC)                  * @T2 100

TXDELAY 40 (400 ms)                    * T 40 (400 ms)

NOTE: Set paclen larger than the longest input line you expect to use.

For example, if you will have 4 20-character fields plus an 80 char.

message, then to include four commas use

PACLEN 164

Next, using the ARES/Data callsign that has been chosen by the sysop,

simply initiate a connection.

(Note: if the sysop is running a G8BPQ switch at the main database

station, this first connect will be to the switch.  Usual commands

like NODES, ROUTES, etc. are available.  In addition, a new command,

ARESDATA, will connect you to the ARES/Data application.)

After a successful connect, acknowledged by "ARES/Data System

Online", you may begin to interact with the database or use the

conference bridge.  By the way, always be patient in waiting for a

response from the system - as you can see from the diagram above,

there may be a large number of operators using the system, and at

1200 baud, you may have to wait for responses at times.  If you are

collecting information from a group of voice operators (see Section

D.4), activate your voice net on the designated simplex frequency if

needed.  Note that in this case the packet operator needs two

antennas (one for packet and one for voice) with the antennas

arranged to eliminate de-sense (unless the packet portion is set up

on an alternate band, which is preferable).

B.  WHAT INFORMATION IS STORED IN THE DATABASE

ARES/Data organizes the incoming data into "records", which you can

view as a group of pieces of information about some particular

person, place, or thing.  Each record is given a unique "record

number" by the program.  Here is what can be stored in each record:

field1,field2,field3,field4,message

where each of fields 1-4 can be a 20-character string, and the

message string can be 80 characters long.  The distinction between

the "fields" and the "message" is that the "fields" are organized

internally by the program so that the packet operator can request

searches and summaries on the information in any one of the four

fields.  Searches and summaries cannot be performed on the

information in the message field.

If this sounds a little abstract, don't worry.  It is one of the

major virtues of the system that the meaning of the stored

information is not defined in advance.  In this manner, ARES/Data

can be used in a variety of ways, depending upon the particular

disaster or emergency at hand.  In a given event, the sysop can

issue a "labels" command that gives particular meaning to each of

the fields and the message, so that all know how ARES/Data is being

used for that event.

For example, in an evacuation, you may want to keep track of

evacuees at shelters.  Then you may want the fields to mean:

Name, Shelter, Status, PhoneNumber, Contact person.

On the other hand, if there is a multiple-casualty incident, you may

want the fields to mean:

Triage number, Sex/Age, Ambulance, Hospital, Condition.

See the paper on ARES/Data in the 7th ARRL Computer Networking

Conference Proceedings (1988) for more examples.

C.  GENERAL RULES FOR CURRENT INFORMATION INPUT

(NOTE:  If the database has been set READONLY by the sysop, you will

not be able to enter information, change information, or delete

information.  However, you can still search and list the database

entries.)

Enter the four fields and any message, in order, with separators

between the fields.  The only valid separator is the comma.  Within

a field, leading and trailing blanks are ignored, but imbedded

blanks ARE significant.  If no value is desired for a particular

field, just skip the field by adding an extra comma.  The database

will fill that field with ten blank characters.

D.  SYNTAX FOR CURRENT INFORMATION INPUT:

field1,field2,field3,field4,message<cr>

(<cr> means carriage return)

1.  FIELD1 - FIELD4

The four fields are very general.  Each can have up to 20

characters, with imbedded blanks.  Entries can be in upper

or lower case, or a mixture, but are converted to UPPER case

before being stored in the database.  The meaning of each field is

defined in real-time by the ARES officials, depending upon the

nature of the event.  The sysop can issue a "labels" command that

will give specific names to each of the four fields to help the

operators remember what they mean.  Similarly, the remote packet

operator can type "labels<cr>" to see the current label

definitions.

2.  MESSAGE

MESSAGE is a free-form field that can be up to 80 characters long.

It could contain a message, a phone number, an address, or other

information deemed useful for the incident.

AUTOMATIC ORIGIN FEATURE:

When a record is initially entered into ARES/Data, the callsign of

the originating station is automatically added to the beginning of

the message field in the format "<W1AW>   rest of message here   ".

This origin identifier automatically changes if another station

alters the record later.  As a result of this, the effective length

of the message field is 80 minus the number of characters in the

origin identifier, which is usually more like 72 characters.

3.  EXAMPLES OF DATA INPUT

4085553195,joe,12,sj34<cr>

Johnson,mary,93445,sj13, home 2333 Alsace Ln SJ 617-555-1212<cr>

All of the input information is stored in the database

as a record of the status and location of a particular person,

place, or thing at a particular time and date.  The time and date

are added automatically by the ARES/Data program.  Further STATUS INPUT

packets for the same person, place, or thing will also be saved

in the database.  The time and date will identify the most recent

information.

4.  DETAILED EXAMPLE INPUT TO THE ARES/Data DATABASE

Suppose that the four fields have been defined to be:  LAST NAME,

FIRST NAME, SHELTER NUMBER, and basic physical CONDITION.  This

example also assumes that a voice operator is telling the packet

operator what to type in.  If voice operators are not in use, the

packet operator simply types the information in directly.

Voice Operator says:                        Packet Operator types:

This is current info,                         (nothing)

The last name is Johnson                      Johnson,

The first name is Joe                         Joe,

The shelter is RS03 (Riverside Shelter 3)     RS03,

The condition is "okay"                       ok,

(optional) The message is "came with dog"     came with dog

which should look like this on your terminal:

Johnson,Joe,RS03,ok,came with dog<CR>

The ARES/Data Database will acknowledge your input, for example, by

"1450: data input accepted, #234."

or, by sending an error message requiring a re-entry.  The

acknowledgment contains the current time and the record

number for that person's entry in the database.  Note that

the program adds the actual time and date to each entry, which you

can see when you list the record (see below).

If you later enter more information for the same person, you

would re-enter the same last and first names, and then enter

new values for any other fields that have changed.  BOTH entries

will be kept as a sort of "audit trail" of what has happened to

that person.  Alternatively, you can just change the value in the

specific field that has changed (see the next section).

E.  CORRECTING AND/OR UPDATING INFORMATION

If you accidentally enter incorrect information into the database,

or if the information in a particular record has changed, you have

several options.  You can delete the entire record, or you can

change the value in a specific field of a specific record.

1. DELETING A BAD RECORD:

You can ask the sysop to delete the bad entry by typing:

tell sysop ooops, typo. pse delete #234.<CR>

OR, if the sysop has enabled the remote delete feature, you

can delete this entry yourself, and then re-enter the correct

data.  To do this, you first make sure you know the record number,

then use the delete command:  "d nnnn<CR>"

d 234<CR>

This function is always enabled at the sysop keyboard.  Its use by

remotely connected packet stations is controlled initially by the

configuration file during program startup.  Thereafter, the sysop

can disable or enable this function as necessary.  Be extremely

careful in using this command!  Always list the record first

before deleting to be sure you have the right one.

2. CHANGING (UPDATING) A PARTICULAR FIELD OF A PARTICULAR RECORD:

The syntax for updating fieldm of record nnnn is:

#nnnn,m=new text for fieldm<cr>

where m = 1-4 for the first four fields, and m = 5 for the message.

For example, suppose you needed to change the value in field3

of record 235 to "shelter 9".  You would type:

#235,3=shelter 9<cr>

Note that when you correct or update a single field like this, the

time and date for the record are not changed.  ARES/Data responds

by showing you what the old values for record 235 were, and what

the new values are according to your update command.

F.  INFORMATION THAT MAY BE REQUESTED FROM THE ARES/Data DATABASE

The packet operator can request several types of searches of the

ARES/Data database.  S/he can request a search for a specific value

of any one of the four main fields.  In this case, the ARES/Data

program sends back to the packet operator a status report listing

all entries in the database having the specified value for the

selected field.  In addition, the operator can request a "wildcard"

search, which looks for any entries in a specific field that START

with a particular string.  The Packet Op can also request a summary

for any one of the four fields, which is a list of the number of

entries in the database for each distinct value of that field (see

Section H).  The operator can list single records in the database by

specifying the record number.

For example, suppose that for a particular incident, the sysop has

designated field1 to be the person's last name.  Suppose the packet

operator needs to find the information on all people in the system

with a given last name.  The operator sends a search request for

field1, stating which last name s/he is interested in.  The

ARES/Data system will respond with all entries with the given name;

one line for every entry in the database matching that name.  If the

operator knows only the beginning of the name or if the name is

long, the wildcard search is very useful.

G. SEARCH REQUESTS

1.  SYNTAX FOR SEARCH REQUESTS

/1,value<cr>                      for field 1

/2,value<cr>                      for field 2

/3,value<cr>                      for field 3

/4,value<cr>                      for field 4

or

?1,value<cr>                      for field 1

?2,value<cr>                      for field 2

?3,value<cr>                      for field 3

?4,value<cr>                      for field 4

This type of packet instructs the database machine to look in the

database for ALL entries with the same entry as "value" in the

specified field.  The string "value" must exactly match what was

originally typed in for the selected field, with leading and

trailing blanks removed, and without regard for case.  The initial

character of the search request can be "/" or "?" - use whichever

is most convenient.  The two formats are handled identically.  A

status report listing all information for each match is sent back

to the requesting packet station.  The first line gives the search

value and the field number.  At the end of the report, the line:

"ARES/Data Search done at 1534, nn hits."

is sent, which signifies no more information coming, and

that "nn" matches (or hits) were found in the database.

2.  WILDCARD SEARCH OR PARTIAL SEARCH

The syntax for a wildcard or partial search is:

/n,val*<cr>

where "n" is the field number (1-4), and "val*" means that you want to

search for all entries in fieldn that start with the characters

"val".  The response from the system is identical to that for an

exact search request.  This is very useful if a particular field

has been defined to hold more than one piece of information.  For

example, suppose field 1 is defined to be "Lastname-Firstname" so

that Bill Jones would be entered by the line:

Jones-Bill, shelter3, OK, 444-555-1212, message<cr>

Now if you did not know Mr. Jones' first name, you could still

search for him in the database by typing

?1,Jone*<cr>

and you would retrieve all records whose first field began with

the characters "JONE".

3.  DETAILED EXAMPLE OF SEARCH REQUEST

Assume that the label definitions are the same as in D.4

above.  This example applies to a last name search request

(field1).

Voice Operator says:                        Packet Operator types:

This is a request for last name search        /1,

The name is johnson                           johnson

which should look like this on your terminal:

/1,johnson<CR>

The ARES/Data database will acknowledge your request by either

stating that there are no entries in the database for that value

for field1, or by sending a status report which looks like:

Exact Search for value "JOHNSON" in Last name

Recno    DT/Time  Last name, First name, Shelter, Condition, Msg

234      23/1124  JOHNSON,JOE,RS03,OK,<W6ABC> CAME WITH DOG

ARES/Data Search done at 1530, 1 hits.

Some versions of ARES/Data may have disabled the search features

for certain fields to save disk(ette) space and speed access.

(And for some situations, it won't make sense to be able to

search on any field).  If searches have been disabled, you will

get a message like this one:

Cannot search on this field, index file not in use.

H.  SUMMARY REQUESTS

$1<cr>                      produces a  list of all distinct

values for field1, with the number

of entries in the database for each

$2<cr>                      similar, except summarize on field 2

$3<cr>                      similar, except summarize on field 3

$4<cr>                      similar, except summarize on field 4

For example, suppose field 3 were defined to be the shelter name.

After the packet operator types "$3<cr>", ARES/Data sends a summary

on field 3, which may be interpreted as a list of shelters, with the

number of people that have checked in to each shelter.

I.  LISTING SPECIFIC ENTRIES (RECORDS) IN THE DATABASE

l nnnn<cr>                  Lists record nnnn

The response will be a short header showing the labels for the

various fields, and then the complete information for record nnnn.

l nnn,mmm<cr>               Lists records numbered from nnn to mmm

l all<cr>                   List ALL records in the database.

WARNING: be careful with this command, as it may cause a

large number of packets to be sent on the channel.  You can

stop an undesired "list all" by simply disconnecting from the

ARES/Data machine.  This will cause no harm.  Then just

reconnect.

J.  DOWNLOADING FILES

You may download a file from a special public directory on the database

machine by typing:

get filename<cr>

This facility is intended to be used and controlled by the sysop in

the sense that s/he controls what is in the public subdirectory and

whether this feature is on or off.  One file that is currently

provided with ARES/Data is an "info" file, which gives more

information of interest to general users.  If the sysop has copied

this file to the public subdirectory, you can download it by typing:

get info<cr>

If the sysop deems that it is useful, this complete documentation

file can be downloaded by typing "get userman.doc<cr>", but please

remember that a lot of packets will be generated by this operation.

To get a listing of the files available for downloading from the

public area, type

dir<cr>

K.  CONFERENCE BRIDGE (roundtable - "users" and "tell" commands)

This feature allows any connected station to send messages to other

connected stations or to the sysop.

1.  Users command:

The users command, in the form "users<cr>" or "u<cr>", returns a

list of the callsigns of currently logged-on packet stations.  The

response is of the form:

Users at WN6I-1 (AX0):  N6KL  W6BB-3  W6XYZ  WB6MRQ-7

Users at WN6I-4 (DR0):  0:N6KL-3 1:N5BZK 3:AA4RE-12

Note that there is one line for each port defined in the ARES/Data

system, so that you can see who is using which port.  The

callsigns used by ARES/Data for the verious defined ports do not

have to be identical.  After the database callsign, the port name

defined by the sysop during startup is shown in parentheses.  Note

also that for the DRSI packet adapter, several radios and even

several boards can be attached to the database machine.  All the

users connecting to the DRSI adapters are treated as being on one

port of the ARES/Data network.  You can refer specifically to the

user on DRSI sub-port 1 by putting a "1:" in front of the

callsign:  "1:N5BZK".  In general, however, this is not really

necessary, since as far as ARES/Data is concerned, "N5BZK" or

"BZK" will do just as well (see below).

2.  Tell command:

The Tell command allows connected packet stations to use ARES/Data as a

conference bridge, or roundtable.  The general format is:

tell callsign message<cr>      or:

t callsign message<cr>

For example:

tell w6bb-3 The food truck just arrived at SJ12<cr>

The message "The food truck just arrived at SJ12" is sent to the

connected station W6BB-3 prefaced by a time stamp and the call of the

station originating the tell command.  In this case, if the tell

command was sent by W6XYX,  W6BB-3 sees:

1230 W6XYZ> The food truck just arrived at SJ12

It is not necessary to enter the entire callsign - just the suffix or

some other substring will do.  If several connected callsigns contain the

substring, each station will get the message.  The special callsign

"*" or "all" is used to send a message to all connected stations.  The

special callsign "sysop" sends the message to the sysop at the ARES/Data

database station.

END USERMAN.DOC