TN210.DOC

FORWARD

***********

With this release one will find extensive revision over that in the earlier

TheNet Plus version 2.06 documentation.   Based on comments received as well as

in-depth observation of network activity, added material on operating practices

and suggested approaches to  laying out modern inexpensive backbone  trunks has

been added.   An effective LAN layout is also described.    The section on node

operation and configuration has been expanded.

A program appended to this release  is  PATHCALC.EXE,  a shareware  program

written  by Jon Adams, NW6H.   PATHCALC will help  NodeOps in evaluating the RF

path  quality of existing  links and is extremely useful in selecting new radio

links.

Another  program  appended is R95.EXE.    R95 allows modification of BINARY

files  to  an  ASCII format suitable for transmission over packet BBS networks.

Large files can be ZIPPED, split into smaller files,  then  recombined  on  the

receiving  end.    R95 doesn't  create the  problem of harsh sounds and monitor

lockups  that binary transfers via TCP can cause.    James Jones, WD5IVD is the

author of RADIX 95.

SOFTWARE NOTICE

TheNet Plus is designated as freeware and is released for amateur radio use

only.     Permission is hereby granted to freely copy and distribute (on a non-

profit basis)  all,  or  portions  of  the  files associated with this release.

If  partial  distributions  of this documentation package are made, please give

the  appropriate  author credit.    Copies of TheNet Plus  source code  are not

available at this time.    Those interested  in  source  code  for the original

TheNet version 1.0 series may direct requests to:

NORD><LINK

3300 Braunschweig

Hinter dem Berge 5

Germany

WARNING:  Use of  TheNet Plus in  non-conformance  of  local network policy

may  result  in  unsatisfactory  system  performance.     ALWAYS coordinate the

operation  of new nodes on existing networks prior to placing them in service.

ACKNOWLEDGEMENTS

Special  appreciation goes to NORD><LINK's Hans Georg Giese DF2AU, and Mike

Roehner DC4OX,  authors of TheNet.     We also gratefully acknowledge the ideas

and comments submitted by many individuals.  These include:  Scott Cronk N7FSP,

Mark Frey VE3DTE,  Eric Meth VE3EI, Don Nelsch K8EIW, Dick Sission W5ONL,  Alan

MacPherson VE3KYZ,  John Langtry VE3NEC, John Papay K8YSE,  Dick Schiavo KF4WQ,

Bob  Moorman K5VMX,   Mykle Raymond N7JZT,  Dick Taylor N5NGZ,  Rich Ide N7LDI,

Brett  Mueller KB5CDX,  Juan Pineda EA3CIW, Roy Gould N5RG, Dick Wareing K3AKK,

Dave Weik KB7ZZ,  Dick Jenkins N2AYY, Steve Weidner WB7WHF, Wally Foster N6CDJ,

Fred McKenzie K4DII,  Hartley Gardner W1OQ  and,  Tad Torborg KA2DEW  and  Rich

Place WB2LJR,  of the North East Digital Association.   There has been a lot of

interest in this project and our apologies if we have overlooked anyone.

INTRODUCTION

The  PURPOSE  of  packet  nodes  in  a  network  is  to act as relay points

extending the range of  users.   Compared to early digipeater  networks  (which

typically  spanned a  maximum  distance of  8 - 10 hops),  nodes allow users to

travel significantly farther.  As of this writing Alan MacPherson, VE3KYZ holds

the distance record of thirty seven sequential hops through the system.  Alan's

route was from Ontario, Canada to Arizona, USA.

Nodes are  also  useful in allowing  frequency  changes in a radio network.

They  can  be  configured to route circuits between combinations of VHF/UHF/HF.

Nodes can accomplish changing  from  a radio  route to a wire-line route.  This

sometimes is done over satellite or telephone circuits.

A  prerequisite for proper node software development is to reach a sensible

balance  between  network loading and providing useful features for the  users.

It is tempting  for software developers to design  nodes  with  many "bells and

whistles".    However, our RF networks  are for the  most  part overloaded with

BBS, Packet cluster, and  TCP/IP  traffic.  Until the networks are upgraded for

greater  capacity,  it's important that node design be oriented toward reducing

unnecessary node overhead.

We  believe TheNet Plus achieves this goal.   Each TheNet Plus  feature was

carefully weighed in terms of usefulness versus the generation of added network

congestion.

TheNet Plus  conforms  to  the International  Standards  Organization (ISO)

model  for  Open  Systems  Interconnection  (OSI).    OSI is a modular software

construction model for data communications.   It is a  functional guideline for

universal intercomputer communication.   The OSI model was developed during the

1970s' and  formally adopted as an international standard in 1983.   This model

is known as CCITT X.200 or ISO 7498.

The  TheNet Plus code is written into a 27C256 Electrically Programmed Read

Only Memory (EPROM)  chip.  This EPROM directly replaces U-23 that contains the

TNC program in TAPR TNC-2 or clones.    Thus, a single chip change-out converts

the TNC into an economical and powerful node switch.

The new node features and networking suggestions in this release came about

through personal observations and user input.  Our goal is to provide practical

nodeware along with information on the art of NodeOping.  To this end, we would

appreciate feedback.  Our points of contact are:

Software:                                 Documentation:

*********                                 **************

Packet: NJ7P@NJ7P.AZ.USA.NA                       N7OO@NJ7P.AZ.USA.NA

U.S. Mail:  William A. Beech, NJ7P                Jack Taylor

PO Box 38                             RR-2 Box 1690

Sierra Vista, AZ  85636               Sierra Vista, AZ  85635

Spanish language coordinator and distribution:

**********************************************

Packet: NS7G@NJ7P.AZ.USA.NA

U.S. Mail:  Rod Lopez

2302 6th st.

Douglas, AZ  85607