TN211_17.DOC

HOST INTERFACE

******************

When  a  terminal  is  attached  to the node's RS-232 terminal port, one is

accessing  the Host Interface.  Its function is to allow the NodeOp  to perform

housekeeping operations.  The node automatically  accepts one as the authorized

SYSOP whenever it senses the terminal connection.

Besides the regular USERS commands, the Host Interface has specific

host commands available.  These are:

ESC C - to connect to the node.

ESC D - to disconnect from the node

While accessing the Host Interface, one can (without qualification) perform

all the other SYSOP functions that are available remotely.   The Host interface

is set up as a single user application.   It can be accessed by giving an ESC C

command via a terminal attached to the RS-232 port.   A user can access only if

the Host is not  actively  CONNECTED to the node AND if the Host mode Parameter

is enabled (set to 1).   Under these circumstances a user would CONNECT  to the

node  as normally done.   Once  the "connected  to" response  is  received, the

user  would enter another "C" command.   The node will then respond again  with

"connected to" <nodecall>.   At this time communication  can take place between

user and  the Host terminal.   Outside users  in this mode  will NOT be able to

SYSOP the node.   Should the Host terminal be CONNECTED to the node when a user

attempts the second "C" command, the response will be "Host Busy."

An  interesting  application of Host mode is to connect the RS-232 port of a

"stand alone" node to a Kantronics weather station.   The node's Host mode Parm

is enabled and appropriate instructions are listed in the INFO section.

The following display is from a Host mode to Kantronics weather station:

ELPWX:KB5GZZ-1} the El Paso Weather Station

Send C to connect & then DATA for current readings

Send HELP for help

Operating in North Central El Paso

MaxData is 6

ELPWX:KB5GZZ-1} TheNet Plus 2.08B (692)

Circuit(ELP:N5RG-1 N7OO-11)

ELPWX:KB5GZZ-1} Routes:

0 ELP95:WA5PIE-1 192 2

> 0 ELP4:N5FAZ-5 192 27

ELPWX:KB5GZZ-1} Heard:

No One

ELPWX:KB5GZZ-1} Connected to ELPWX:KB5GZZ-1

01/01/92  16:28:04 TPCB= +0083.0 DEGF

01/01/92  16:28:04 TF  = +0055.7 DEGF

01/01/92  16:28:04 RG  = 0000.0 IN

01/01/92  16:28:04 WS  = 00004 MPH

01/01/92  16:28:04 WD  = 00041 DEGN NE

wxn:

Type "help" followed by parameter/command to get a short description.

BYE       DATA      DMETRIC   DUSA      HELP      SYSOP

wxn:

BYE

Place TNC or NETROM in command mode and disconnect.

wxn:

DATA

DATA [[sensor]] [n [step]]]] [start mmddhhmm]] ...]]

The DATA command is used to retrieve sensor readings from the

WeatherNode. The optional values (sensor, n, step, and start

date/time) can be specified to select only specific sensors or

specify the number of samples, and the step between samples to be

displayed. If the DATA command is given without any arguments, the

WeatherNode will return the last reading for each defined sensor.

Specifying n will cause n samples of the preceding sensor to be

displayed (If the sensor is left blank, n samples of all defined

sensors will be displayed). If step is specified, the WeatherNode

will skip step-1 readings between the displayed values (i.e. step=2

displays every other reading, step=3 displays every third reading,

etc).  You may optionally specify a start time, using the format

start mmddhhmm where mm is the month (01-12), dd is the day (01-31),

hh is the hour (00-23) and mm is the minutes (00-59).  The WeatherNode

will then display the requested data, starting at the date and time

you have specified.

Valid sensors are tf (temperature of external sensor), tp

(temperature of PC mounted sensor), ws (wind speed), wd (wind

direction), and rg (rain gauge).

wxn:

DMETRIC

Read data in metric units from acquisition buffer specifying a series of

sensor type, number of samples, and sample step.  Temperature displayed

in degrees C, wind speed displayed in kph, and rain gauge displayed in

centimeters.

wxn:

DUSA

Read data in USA units from acquisition buffer specifying a series of

sensor type, number of samples, and sample step.  Temperature displayed

in degrees F, wind speed displayed in mph, and rain gauge displayed in inches.

wxn:

HELP

Read the manual.

wxn:

SYSOP

Displays 3 password codes and if correct password entered, grants

access to full command set.  If incorrect password, displays attempt

failed unless 3 attempts failed, then displays access denied and inhibits

access except for data command for 15 minutes.

wxn:

sysop

wxn:

bye