BANKSWITCHING FOR TheNet X1

What  follows  is two versions of how to  do  the  bankswitching. Saying things in two different ways is a neat way of making  sure that ambiguities are exposed, so here goes. The first version  is by me, the second by Bob G8HBE.

===============================================================================

For  the  reduced TNC2 version, the instructions  are  simply  as follows :

1.   	Bend out pin 1 of the EPROM so that when inserted  into the  socket it will not contact pin 1 of the socket  or any other pin.

2.   	Connect  a wire from the SIO-0 DTRA pin ( pin 16  )  to the  bent  out  pin ( pin 1 ) of the  eprom.  The  DTRA signal  should also appear on pin 8 of the  TAPR  modem disconnect header.

The  status  led will flicker as it now shows the  state  of  the bankswitch signal.

One  word of caution - if you can, just check the signal on pin 1 of the eprom - make sure it switches fast and cleanly - i suspect that if it does not, errors will occur.

================================================================================

Robert Smith

38 Norris Close,

Chiseldon,

Nr Swidon,

Wiltshire.

SN4 0LR

6 October 1991

BANKSWITCHING for TheNet X1

So  that the 27C512 does not get damaged by the bending of pin  1 and  soldering  it I have made the modification to  the  TNC-200, Tiny 2 and MFJ1274 type TNC's as follows.

To  modify your TNC you will require a piece of  thin  connecting wire about 100 mm long and a 28 pin IC socket,  you may also need a little bit of inserlating tap.

Before starting the modification make sure that power to the  TNC has  been disconnected and that the lithium battery link as  been removed.

Remove the 27C256, U23 in a MFJ1274/TNC200 or U2 on a Tiny 2  and put it in a safe place.

Taking your New 28 pin IC socket, bend pin 1 outwards and  solder the end of the wire to the bent out pin.

Plug  the  IC socket into the socket you took  the  27C256  from, making sure that you plug it in the correct way round.

Depending  on  the type of IC socket that is mounted  on  the  PC board  you may find that pin 1 on both IC sockets may touch,   if this seems to be the case put a little pieces of inserlating  tap between them.

Also on the Tiny 2 make sure that the bent pin does not touch the CPU chip.

Now  connect the other end of the wire to pin 16 on the  Z80  SIO chip.

This  signal can also be found on pin 8 of the  modem  disconnect header which is J5 on a Tiny 2, also the same signal can be found on  pin  5  of U6 (74HC14) on the Tiny 2. On  a  TNC200  you  can connect the wire to the side of R51 nearest U23, this is just  20 mm from pin 1 of U23 on the TNC200 board.

If in doubt use a test meter and check the continuity from pin 16 on  the Z80 SIO chip to the point where you are going  to  solder the wire.

Once  you have done this you can plug the new  27C512  Programmed with TheNet X into the new mounted IC sockets.

Re-insert the lithium battery link, Connect your computer to  the RS232  socket,  12  volts to the TNC and switch  on,  if  all  is working  well  the the STA light will be flickering and  after  a second a message will appear on you screen.

Modification complete.

G8HBE    6-Oct-91