----------------------------------------------------------------------

From   : Pierre Cornelis, ON7PC   rue Ballings, 88     B-1140 Brussels

To     : All

Date   : October, 1992

Subject: WA8DED AND THEFIRMMARE COMMAND REFERENCE CARD

--------------------------------------------------------------------

WA8DED  AND  THEFIRMWARE  COMMAND  REFERENCE  CARD :

====================================================

1. Commands :

-------------

The following commands must be precedeed by ESCAPE :

Command       Parameter         Description

---------------------------------------------------------------------

A (1)           0             Auto linefeed disabled

1             Auto linefeed enabled

C       Cs1 [Cs2 ... Cs9]     Connect path (0=unproto path)

D                             Disconnect

E (1)           0             Echo input disabled

1             Echo input enabled

* F (4)          1-15           Frame acknowledge (seconds)

time = frame ack*(2*number of digi+1)

G               0             Get information (host mode)

1             Get link status (host mode)

# H                             Displays the heard list

0             Disable the updating of the list

1             Enable the updating of the list

2             Clears the list

10             Maximum calls in the list

* I              Cs             Tnc source callsign

JHOST (0)       0             Terminal mode enabled

1             Host mode enabled

K                             Displays the date and time

0             Disable the stamp function

1             Enable the stamp function

2             Enable stamp and monitor

20.02.88          Set date with european format

02/20/88          Set date with US format

17:36:00          Set time

L              0-4            Display channel status

M (IU)        NIUSC+-         Monitor mode

N  none

I  I frames

U  UI frames

S  supervisory frames

C  monitor while connected

+  callsigns to be included (max 8)

-  callsigns to be excluded (max 8)

* N (10)         0-127          Number of tries (0=forever)

* O (4)          1-7            Number of outstanding I frames

P (64)         0-255          P-persistence value

QRES                          Re-start firmware

R (1)           0             Repeater disabled

1             Repeater enabled

S (0)          0-4            Select channel (0=unproto)

T (30)         0-127          Transmitter delay (10ms)

U (0)   0     [text]          Unattended mode disabled

1     [text]          Unattended mode enabled

V (2)           1             Version 1 protocol initiated

2             Version 2 protocol initiated

W (10)         0-127          Slot time interval (10ms)

X (1)           0             Transmitter PTT disabled

1             Transmitter PTT enabled

Y (4)          0-4            Maximum connections

Z (3)           0             Flow disabled, xon/off disabled

1             Flow enabled, xon/off disabled

2             Flow disabled, xon/off enabled

3             Flow enabled, xon/off enabled

@  B                          Display number of free buffers

@  D  (0)       0             Full duplex disabled

1             Full duplex enabled

@  K                          Enable the KISS-Mode

@  S                          Display current link state

@  T2 (100)    0-65535        Timer T2 interval (10ms)

@  T3 (18000)  0-65535        Timer T3 interval (10ms)

@  V  (0)       0             Callsign validation disabled

1             Callsign validation enabled

*  These commands are applicable to each connection channel

#  Max 600 records with the format : call-ssid, first heard, last

heard, I frames, RR frames, REJ frames and RNR frames

2. TheFirmware version 2.3 DAMA :

---------------------------------

On September 91 NORD><LINK published a new TheFirmware version including

the DAMA mode. The "Demand Assigned Multiple Access" protocol was

described by Detlef Schmidt DK4EG in the 9th ARRL Network Computer

Conference.

When a DAMA node is recognized, this TF eprom turns automatically to the

DAMA mode. Two new concepts are important :

RTT : Round Trip Timer is the time that the node wait before sending data

to anaother user

The new commands are :

Command       Parameter         Description

---------------------------------------------------------------------

B (120)                       Delay (in sec) after which the TNC turns

back to normal (non-DAMA) mode.

F (700)                       Frack in 10 ms start for non-DAMA mode

Start value in 10 ms for SRTT in DAMA mode

@ A1 (7)                      alpha 1 parameter for DAMA

@ A2 (15)                     alpha 2 parameter for DAMA

@ A3 (2)                      beta    parameter for DAMA

@ I (60)                      Maximum I poll frame lenght

@ M (1)         0-1           Terminal mode 7th bit or not

3. TheFirmware version 2.4 DAMA :

---------------------------------

This version has been released in September 92.

- the SMACK mode (KISS with CRC) has been added

- the AX25 version 1 is not possible

- if F (FRACK) is lower that 16, the number is considered to be in seconds.

4. Default parameters for TF 2.4a:

----------------------------------

The table starts at $0040.

db    1Bh             ; Command Character                     ( Escape )

db    '      ',60h    ; Callsign                              ( <empty> )

db    '      '        ; Mnemonic                              ( <empty> )

db    4               ; Maximum number of connections         (      4 )

db    03h             ; Monitormode                           (    03h )

;   Bit 0 - I frames

;   Bit 1 - UI frames

;   Bit 2 - Supervisory frames

;   Bit 3 - Monitor while connected

db    1               ; Digipeater, disable/enable            (      1 )

db    32              ; P-Persistence, 0..255                 (     32 )

db    10              ; Slottime, 10ms                        (     10 )

db    25              ; TX delay, 10ms                        (     25 )

db    03h             ; Flow Control                          (    03h )

;   Bit 0 - Flow Control on

;   Bit 1 - XON/XOFF on

db    1               ; Transmitter PTT disable/enable        (      1 )

db    1               ; Auto linefeed disable/enable          (      1 )

db    1               ; Echo command line disable/enable      (      1 )

db    1               ; Version 2 disable/enable              (      1 )

db    2               ; Maximum unacknowledge frames          (      2 )

db    10              ; Maximum try count                     (     10 )

db    0               ; Validate callsign                     (      0 )

db    0               ; Full duplex enable/disable            (      0 )

db    0               ; 8 bit character enable/disable        (      0 )

db    0               ; UI poll, 0=UI 1=UI+                   (      0 )

db    0               ; Stamp                                 (      0 )

;   0 off

;   1 on status frame

;   2 on status and monitor frames

db    0               ; Transmit flags during pauses          (      0 )

db    0               ; Heard list, enable/disable            (      0 )

dw    100             ; for ^ , max. number of calls          (    100 )

dw    150             ; Timer 2, 10ms                         (    150 )

dw    18000           ; Timer 3, 10ms                         (  18000 )

db    7               ; Alpha 1                               (      7 )

db    15              ; Alpha 2                               (     15 )

db    3               ; Alpha 3                               (      2 )

dw    250             ; Initial RTT, 10 ms                    (    250 )

db    60              ; Paclen for I-POLL                     (     60 )

dw    120             ; Start value for the DAMA timer, sec   (    120 )

Note 1:  The secondary station id must be shifted  left  one  bit

and or'ed with 60H.

73 and happy packetting,

Pierre , ON7PC @ ON7RC

---------------------------------------------------------------------

***END