Cut file at lines of the form "-- filename.ext --" and write to file

with that name.

-- spacetrk.tex --

\documentstyle[11pt,fleqn]{article}

\oddsidemargin 0in

\evensidemargin 0in

\textwidth 6.5in

\textheight 9in

\topmargin -36pt

\headheight 12pt

\headsep 24pt

\parindent 0pt

\def\dfrac#1#2{{\displaystyle{#1\over#2}}}

\def\dt{(t-t_o)}

\def\Epo{(E+\omega)}

\def\dddot#1{#1}

\def\dn{\dot{n}}

\def\dr{\dot{r}}

\def\dv{\dot{v}}

\def\df{\dot{f}}

\def\dM{\dot{M}}

\def\dC{\dot{C}}

\def\dD{\dot{D}}

\def\dom{\dot{\omega}}

\def\dOm{\dot{\Omega}}

\def\dal{\dot{\alpha}}

\def\dxi{\dot{\xi}}

\def\de{\dot{e}}

\def\deta{\dot{\eta}}

\def\dpsi{\dot{\psi}}

\def\ddn{\ddot{n}}

\def\dde{\ddot{e}}

\def\ddxi{\ddot{\xi}}

\def\ddeta{\ddot{\eta}}

\def\ddD{\ddot{D}}

\def\dddn{\stackrel{{\ldotp\ldotp\ldotp}}{n}}

\def\ppa{a''}

\def\ppn{n''}

\def\ppao{a''_o}

\def\ppno{n''_o}

\def\Mvec{{\bf M}}

\def\Nvec{{\bf N}}

\def\Uvec{{\bf U}}

\def\Vvec{{\bf V}}

\def\rvec{{\bf r}}

\def\drvec{\dot{\rvec}}

\def\L{{I\hspace{-3pt}L}}

\title{SPACETRACK REPORT NO. 3\\[12pt]

Models for Propagation of\\

NORAD Element Sets}

\author{ {\sc Felix R.\@ Hoots}\\

{\sc Ronald L.\@ Roehrich}\\[12pt]

{\sc December 1980}\\[12pt]

Package Compiled by\\

TS Kelso}

\date{31 December 1988}

\begin{document}

\pagenumbering{roman}

\addcontentsline{toc}{section}{ABSTRACT}

\maketitle

General perturbations element sets generated by NORAD can be used to predict

position and velocity of Earth-orbiting objects.  To do this one must be

careful to use a prediction method which is compatible with the way in which

the elements were generated.  Equations for five compatible models are given

here along with corresponding FORTRAN IV computer code.  With this information

a user will be able to make satellite predictions which are completely

compatible with NORAD predictions.

\newpage

\addcontentsline{toc}{section}{CONTENTS}

\tableofcontents

\newpage

APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED

\vspace{1in}

Requests for additional copies by agencies of the Department of Defense, their

contractors, and other government agencies should be directed to

the:\footnote[1]{Editor's note: These are the addresses from the original document

and may no longer be valid.}

\begin{quote}

Defense Documentation Center\\

Cameron Station\\

Alexandria VA  22314

\end{quote}

All other persons and organizations should apply to the:

\begin{quote}

Department of Commerce\\

National Technical Information Service\\

5285 Port Royal Road\\

Springfield VA  22161

\end{quote}

Address special inquiries to:

\begin{quote}

Project Spacetrack Reports\\

Office of Astrodynamics\\

Aerospace Defense Center\\

ADC/DO6\\

Peterson AFB CO  80914

\end{quote}

\newpage

\parindent 2em

\parskip 12pt

\pagenumbering{arabic}

\section{INTRODUCTION}

NORAD maintains general perturbation element sets on all resident space

objects.  These element sets are periodically refined so as to maintain a

reasonable prediction capability on all space objects.  In turn, these element

sets are provided to users.  The purpose of this report is to provide the user

with a means of propagating these element sets in time to obtain a position

and velocity of the space object.

The \underline{most important} point to be noted is that not just any

prediction model will suffice.  The NORAD element sets are ``mean'' values

obtained by removing periodic variations in a particular way.  In order to

obtain good predictions, these periodic variations must be reconstructed (by

the prediction model) in exactly the same way they were removed by NORAD.

Hence, inputting NORAD element sets into a different model (even though the

model may be more accurate or even a numerical integrator) will result in

degraded predictions.  The NORAD element sets \underline{must} be used with

one of the models described in this report in order to retain maximum

prediction accuracy.

All space objects are classified by NORAD as near-Earth (period less than 225

minutes) or deep-space (period greater than or equal 225 minutes).  Depending

on the period, the NORAD element sets are automatically generated with the

near-Earth or deep-space model.  The user can then calculate the satellite

period and know which prediction model to use.

\section[The Propagation Models]{THE PROPAGATION MODELS}

Five mathematical models for prediction of satellite position and velocity are

available.  The first of these, SGP, was developed by Hilton \& Kuhlman (1966)

and is used for near-Earth satellites.  This model uses a simplification of

the work of Kozai (1959) for its gravitational model and it takes the drag

effect on mean motion as linear in time.  This assumption dictates a quadratic

variation of mean anomaly with time.  The drag effect on eccentricity is

modeled in such a way that perigee height remains constant.

The second model, SGP4, was developed by Ken Cranford in 1970 (see Lane and

Hoots 1979) and is used for near-Earth satellites.  This model was obtained by

simplification of the more extensive analytical theory of Lane and Cranford

(1969) which uses the solution of Brouwer (1959) for its gravitational model

and a power density function for its atmospheric model (see Lane, et al.\

1962).

The next model, SDP4, is an extension of SGP4 to be used for deep-space

satellites.  The deep-space equations were developed by Hujsak (1979) and

model the gravitational effects of the moon and sun as well as certain

sectoral and tesseral Earth harmonics which are of particular importance for

half-day and one-day period orbits.

The SGP8 model (see Hoots 1980) is used for near-Earth satellites and is

obtained by simplification of an extensive analytical theory of Hoots (to

appear) which uses the same gravitational and atmospheric models as Lane and

Cranford did but integrates the differential equations in a much different

manner.

Finally, the SDP8 model is an extension of SGP8 to be used for deep-space

satellites.  The deep-space effects are modeled in SDP8 with the same

equations used in SDP4.

\section[Compatibility with NORAD Element Sets]{COMPATIBILITY WITH NORAD ELEMENT SETS}

The NORAD element sets are currently generated with either SGP4 or SDP4

depending on whether the satellite is near-Earth or deep-space.  For element

sets sent to external users, the value of mean motion is altered slightly and

a pseudo-drag term ($\dn/2$) is generated.  These changes allow an SGP user

to make compatible predictions in the following manner.  If the satellite is

near-Earth, then the pseudo-drag term used in SGP simulates the drag effect of

the SGP4 model.  If the satellite is deep-space, then the pseudo-drag term

used in SGP simulates the deep-space secular effects of SDP4.

For SGP4 and SDP4 users, the mean motion is first recovered from its altered

form and the drag effect is obtained from the SGP4 drag term ($B^*$) with the

pseudo-drag term being ignored.  The value of the mean motion can be used to

determine whether the satellite is near-Earth or deep-space (and hence whether

SGP4 or SDP4 was used to generate the element set).  From this information the

user can decide whether to use SGP4 or SDP4 for propagation and hence be

assured of agreement with NORAD predictions.

The SGP8 and SDP8 models have the same gravitational and atmospheric models as

SGP4 and SDP4, although the form of the solution equations is quite different.

Additionally, SGP8 and SDP8 use a ballistic coefficient ($B$ term) in the drag

equations rather than the $B^*$ drag term.  However, compatible predictions

can be made with NORAD element sets by first calculating a $B$ term from the

SGP4 $B^*$ drag term.

At the present time consideration is being given to replacing SGP4 and SDP4 by

SGP8 and SDP8 as the NORAD satellite models.  In such a case the new NORAD

element sets would still give compatible predictions for SGP, SGP4, and SDP4

users and, for SGP8 and SDP8 users, would give agreement with NORAD

predictions.

\section[General Program Description]{GENERAL PROGRAM DESCRIPTION}

The five ephemeris packages cited in Section Two have each been programmed in

FORTRAN IV as stand-alone subroutines.  They each access the two function

subroutines ACTAN and FMOD2P and the deep-space equations access the function

subroutine THETAG.  The function subroutine ACTAN is a two argument (quadrant

preserving) arctangent subroutine which has been specifically designed to

return the angle within the range of 0 to $2\pi$.  The function subroutine

FMOD2P takes an angle and returns the modulo by $2\pi$ of that angle.  The

function subroutine THETAG calculates the epoch time in days since 1950 Jan

0.0 UTC, stores this in COMMON, and returns the right ascension of Greenwich

at epoch.

One additional subroutine DEEP is accessed by SDP4 and SDP8 to obtain the

deep-space perturbations to be added to the main equations of motion.

The main program DRIVER reads the input NORAD 2-line element set in either

G-card internal format or T-card transmission format and calls the appropriate

ephemeris package as specified by the user.  The DRIVER converts the elements

to the units of radians and minutes before calling the appropriate subroutine.

The ephemeris package returns position and velocity in units of Earth radii

and minutes.  These are converted by the DRIVER to kilometers and seconds for

printout.

All physical constants are contained in the constants COMMON C1 and can be

changed through the data statements in the DRIVER.  The one exception is the

physical constants used only in DEEP which are set in the data statements in

DEEP.

In the following sections the equations and program listing are given for each

ephemeris model.  Every effort has been made to maintain a strict parallel

structure between the equations and the computer code.

\section[The SGP Model]{THE SGP MODEL}

The NORAD mean element sets can be used for prediction with SGP.  All symbols

not defined below are defined in the list of symbols in Section Twelve.

Predictions are made by first calculating the constants

\[a_1=\left(\dfrac{k_e}{n_o}\right)^{\frac23}\]

\[\delta_1=\dfrac34J_2\dfrac{a_E{}^2}{a_1{}^2}\dfrac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^{\frac32}}\]

\[a_o=a_1\left[1-\dfrac13\delta_1-\delta_1{}^2

-\dfrac{134}{81}\delta_1{}^3\right]\]

\[p_o=a_o(1-e_o{}^2)\]

\[q_o=a_o(1-e_o)\]

\[L_o=M_o+\omega_o+\Omega_o\]

\[\dfrac{d\Omega}{dt}=-\dfrac32J_2\dfrac{a_E{}^2}{p_o{}^2}n_o\cos i_o\]

\[\dfrac{d\omega}{dt}=\dfrac34J_2\dfrac{a_E{}^2}{p_o{}^2}n_o(5\cos^2i_o-1).\]

The secular effects of atmospheric drag and gravitation are included through

the equations

\[a=a_o\left\{\dfrac{n_o}{n_o+2\left(\dfrac{\dn_o}2\right)

\dt+3\left(\dfrac{\ddn_o}6\right)\dt^2}\right\}^{\frac23}\]

\[e=\left\{\begin{array}{ll}

1-\dfrac{q_o}a, & \mbox{for } a > q_o\\[12pt]

10^{-6}, & \mbox{for } a \leq q_o

\end{array}\right\}\]

\[p=a(1-e^2)\]

\[\Omega_{s_o}=\Omega_o+\dfrac{d\Omega}{dt}\dt\]

\[\omega_{s_o}=\omega_o+\dfrac{d\omega}{dt}\dt\]

\[L_s=L_o+\left(n_o+\dfrac{d\omega}{dt}+\dfrac{d\Omega}{dt}\right)

\dt+\dfrac{\dn_o}2\dt^2+\dfrac{\ddn_o}6\dt^3\]

where $\dt$ is time since epoch.

Long-period periodics are included through the equations

\[a_{yNSL}=e\sin\omega_{s_o}-\dfrac12\dfrac{J_3}{J_2}\dfrac{a_E}p\sin i_o\]

\[L=L_s-\dfrac14\dfrac{J_3}{J_2}\dfrac{a_E}pa_{xNSL}\sin i_o

\left[\dfrac{3+5\cos i_o}{1+\cos i_o}\right]\]

where

\[a_{xNSL}=e\cos\omega_{s_o}.\]

Solve Kepler's equation for $E+\omega$ (by iteration to the desired accuracy),

where

\[\Epo_{i+1}=\Epo_i+\Delta\Epo_i\]

with

\[\Delta\Epo_i=\dfrac{U-a_{yNSL}\cos\Epo_i+a_{xNSL}\sin\Epo_i-\Epo_i}

{-a_{yNSL}\sin\Epo_i-a_{xNSL}\cos\Epo_i+1}\]

\[U=L-\Omega_{s_o}\]

and

\[\Epo_1=U.\]

Then calculate the intermediate (partially osculating) quantities

\[e\cos E=a_{xNSL}\cos\Epo+a_{yNSL}\sin\Epo\]

\[e\sin E=a_{xNSL}\sin\Epo-a_{yNSL}\cos\Epo\]

\[e_L{}^2=(a_{xNSL})^2+(a_{yNSL})^2\]

\[p_L=a(1-e_L{}^2)\]

\[r=a(1-e\cos E)\]

\[\dr=k_e\dfrac{\sqrt{a}}re\sin E\]

\[r\dv=k_e\dfrac{\sqrt{p_L}}r\]

\[\sin u=\dfrac{a}r\left[\sin\Epo-a_{yNSL}-a_{xNSL}

\dfrac{e\sin E}{1+\sqrt{1-e_L{}^2}}\right]\]

\[\cos u=\dfrac{a}r\left[\cos\Epo-a_{xNSL}+a_{yNSL}

\dfrac{e\sin E}{1+\sqrt{1-e_L{}^2}}\right]\]

\[u=\tan^{-1}\left(\dfrac{\sin u}{\cos u}\right).\]

Short-period perturbations are now included by

\[r_k=r+\dfrac14J_2\dfrac{a_E{}^2}{p_L}\sin^2i_o\cos 2u\]

\[u_k=u-\dfrac18J_2\dfrac{a_E{}^2}{p_L{}^2}(7\cos^2i_o-1)\sin 2u\]

\[\Omega_k=\Omega_{s_o}+\dfrac34J_2\dfrac{a_E{}^2}{p_L{}^2}\cos i_o\sin 2u\]

\[i_k=i_o+\dfrac34J_2\dfrac{a_E{}^2}{p_L{}^2}\sin i_o\cos i_o\cos 2u.\]

Then unit orientation vectors are calculated by

\[\Uvec=\Mvec\sin u_k+\Nvec\cos u_k\]

\[\Vvec=\Mvec\cos u_k-\Nvec\sin u_k\]

where

\[\Mvec=\left\{\begin{array}{l}

M_x=-\sin\Omega_k\cos i_k\\

M_y=\cos\Omega_k\cos i_k\\

M_z=\sin i_k

\end{array}\right\}\]

\[\Nvec=\left\{\begin{array}{l}

N_x=\cos\Omega_k\\

N_y=\sin\Omega_k\\

N_z=0

\end{array}\right\}.\]

Then position and velocity are given by

\[\rvec=r_k\Uvec\]

and

\[\drvec=\dr\Uvec+(r\dv)\Vvec.\]

A FORTRAN IV computer code listing of the subroutine SGP is given below.

\newpage

\input{SGP.FOR}

\newpage

\section[The SGP4 Model]{THE SGP4 MODEL}

The NORAD mean element sets can be used for prediction with SGP4.  All

symbols not defined below are defined in the list of symbols in Section

Twelve.  The original mean motion ($\ppno$) and semimajor axis ($\ppao$) are

first recovered from the input elements by the equations

\[a_1=\left(\dfrac{k_e}{n_o}\right)^{\frac23}\]

\[\delta_1=\dfrac32\dfrac{k_2}{a_1{}^2}\dfrac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^{\frac32}}\]

\[a_o=a_1\left(1-\dfrac13\delta_1-\delta_1{}^2

-\dfrac{134}{81}\delta_1{}^3\right)\]

\[\delta_o=\dfrac32\dfrac{k_2}{a_o{}^2}\dfrac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^{\frac32}}\]

\[\ppno=\dfrac{n_o}{1+\delta_o}\]

\[\ppao=\dfrac{a_o}{1-\delta_o}.\]

For perigee between 98 kilometers and 156 kilometers, the value of the

constant $s$ used in SGP4 is changed to

\[s^*=\ppao(1-e_o)-s+a_E\]

For perigee below 98 kilometers, the value of $s$ is changed to

\[s^*=20/\mbox{XKMPER}+a_E.\]

If the value of $s$ is changed, then the value of $(q_o-s)^4$ must be replaced

by

\[(q_o-s^*)^4=\left[[(q_o-s)^4]^{\frac14}+s-s^*\right]^4.\]

Then calculate the constants (using the appropriate values of $s$ and

$(q_o-s)^4$)

\[\theta=\cos i_o\]

\[\xi=\dfrac{1}{\ppao-s}\]

\[\beta_o=(1-e_o{}^2)^{\frac12}\]

\[\eta=\ppao e_o\xi\]

\[C_2=\begin{array}[t]{l}(q_o-s)^4\xi^4\ppno(1-\eta^2)^{-\frac72}

\left[\ppao\left(1+\dfrac32\eta^2+4e_o\eta+e_o\eta^3\right)\right.\\[12pt]

\left.+\dfrac32\dfrac{k_2\xi}{(1-\eta^2)}

\left(-\dfrac12+\dfrac32\theta^2\right)(8+24\eta^2+3\eta^4)\right]

\end{array}\]

\[C_1=B^*C_2\]

\[C_3=\dfrac{(q_o-s)^4\xi^5A_{3,0}\ppno a_E\sin i_o}{k_2e_o}\]

\[C_4=\begin{array}[t]{l}2\ppno(q_o-s)^4\xi^4\ppao\beta_o{}^2(1-\eta^2)^{-\frac72}

\biggl(\left[2\eta(1+e_o\eta)

+\dfrac12e_o+\dfrac12\eta^3\right]-\dfrac{2k_2\xi}{\ppao(1-\eta^2)}\times\\[12pt]

\left[3(1-3\theta^2)

\left(1+\dfrac32\eta^2-2e_o\eta-\dfrac12e_o\eta^3\right)

+\dfrac34(1-\theta^2)(2\eta^2-e_o\eta-e_o\eta^3)\cos 2\omega_o\right]\biggr)

\end{array}\]

\[C_5=2(q_o-s)^4\xi^4\ppao\beta_o{}^2(1-\eta^2)^{-\frac72}

\left[1+\dfrac{11}4\eta(\eta+e_o)+e_o\eta^3\right]\]

\[D_2=4\ppao\xi C_1{}^2\]

\[D_3=\dfrac43\ppao\xi^2(17\ppao+s)C_1{}^3\]

\[D_4=\dfrac23\ppao\xi^3(221\ppao+31s)C_1{}^4.\]

The secular effects of atmospheric drag and gravitation are included through

the equations

\[M_{DF}=M_o+\left[1+\dfrac{3k_2(-1+3\theta^2)}{2\ppao{}^2\beta_o{}^3}

+\dfrac{3k_2{}^2(13-78\theta^2+137\theta^4)}{16\ppao{}^4\beta_o{}^7}\right]

\ppno\dt\]

\[\omega_{DF}=\begin{array}[t]{l}\omega_o+\left[-\dfrac{3k_2(1-5\theta^2)}{2\ppao{}^2\beta_o{}^4}

+\dfrac{3k_2{}^2(7-114\theta^2+395\theta^4)}{16\ppao{}^4\beta_o{}^8}\right.\\[12pt]

\left.+\dfrac{5k_4(3-36\theta^2+49\theta^4)}{4\ppao{}^4\beta_o{}^8}\right]\ppno\dt

\end{array}\]

\[\Omega_{DF}=\Omega_o+\left[-\dfrac{3k_2\theta}{\ppao{}^2\beta_o{}^4}

+\dfrac{3k_2{}^2(4\theta-19\theta^3)}{2\ppao{}^4\beta_o{}^8}

+\dfrac{5k_4\theta(3-7\theta^2)}{2\ppao{}^4\beta_o{}^8}\right]\ppno\dt\]

\[\delta\omega=B^*C_3(\cos\omega_o)\dt\]

\[\delta M=-\dfrac23(q_o-s)^4B^*\xi^4\dfrac{a_E}{e_o\eta}

[(1+\eta\cos M_{DF})^3-(1+\eta\cos M_o)^3]\]

\[M_p=M_{DF}+\delta\omega+\delta M\]

\[\omega=\omega_{DF}-\delta\omega-\delta M\]

\[\Omega=\Omega_{DF}-\dfrac{21}2\dfrac{\ppno k_2\theta}{\ppao{}^2\beta_o{}^2}

C_1\dt^2\]

\[e=e_o-B^*C_4\dt-B^*C_5(\sin M_p-\sin M_o)\]

\[a=\ppao[1-C_1\dt-D_2\dt^2-D_3\dt^3-D_4\dt^4]^2\]

\[\L=\begin{array}[t]{l}

M_p+\omega+\Omega+\ppno\left[\dfrac32C_1\dt^2+(D_2+2C_1{}^2)\dt^3\right.\\[12pt]

+\dfrac14(3D_3+12C_1D_2+10C_1{}^3)\dt^4\\[12pt]

\left.+\dfrac15(3D_4+12C_1D_3+6D_2{}^2+30C_1{}^2D_2+15C_1{}^4)\dt^5\right]

\end{array}\]

\[\beta=\sqrt{(1-e^2)}\]

\[n=k_e\bigg/a^{\frac32}\]

where $\dt$ is time since epoch.  It should be noted that when epoch perigee

height is less than 220 kilometers, the equations for $a$ and $\L$ are

truncated after the $C_1$ term, and the terms involving $C_5$, $\delta\omega$,

and $\delta M$ are dropped.

Add the long-period periodic terms

\[a_{xN}=e\cos\omega\]

\[\L_L=\dfrac{A_{3,0}\sin i_o}{8k_2a\beta^2}(e\cos\omega)

\left(\dfrac{3+5\theta}{1+\theta}\right)\]

\[a_{yNL}=\dfrac{A_{3,0}\sin i_o}{4k_2a\beta^2}\]

\[\L_T=\L+\L_L\]

\[a_{yN}=e\sin\omega+a_{yNL}.\]

Solve Kepler's equation for $\Epo$ by defining

\[U=\L_T-\Omega\]

and using the iteration equation

\[\Epo_{i+1}=\Epo_i+\Delta\Epo_i\]

with

\[\Delta\Epo_i =

\dfrac{U-a_{yN}\cos\Epo_i+a_{xN}\sin\Epo_i-\Epo_i}

{-a_{yN}\sin\Epo_i-a_{xN}\cos\Epo_i+1}\]

and

\[\Epo_1=U.\]

The following equations are used to calculate preliminary quantities needed

for short-period periodics.

\[e\cos E=a_{xN}\cos\Epo+a_{yN}\sin\Epo\]

\[e\sin E=a_{xN}\sin\Epo-a_{yN}\cos\Epo\]

\[e_L=(a_{xN}{}^2+a_{yN}{}^2)^{\frac12}\]

\[p_L=a(1-e_L{}^2)\]

\[r=a(1-e\cos E)\]

\[\dr=k_e\dfrac{\sqrt{a}}re\sin E\]

\[r\df=k_e\dfrac{\sqrt{p_L}}r\]

\[\cos u=\dfrac{a}r\left[\cos\Epo-a_{xN}+

\dfrac{a_{yN}(e\sin E)}{1+\sqrt{1-e_L{}^2}}\right]\]

\[\sin u=\dfrac{a}r\left[\sin\Epo-a_{yN}-

\dfrac{a_{xN}(e\sin E)}{1+\sqrt{1-e_L{}^2}}\right]\]

\[u=\tan^{-1}\left(\dfrac{\sin u}{\cos u}\right)\]

\[\Delta r=\dfrac{k_2}{2p_L}(1-\theta^2)\cos 2u\]

\[\Delta u=-\dfrac{k_2}{4p_L{}^2}(7\theta^2-1)\sin 2u\]

\[\Delta\Omega=\dfrac{3k_2\theta}{2p_L{}^2}\sin 2u\]

\[\Delta i=\dfrac{3k_2\theta}{2p_L{}^2}\sin i_o\cos 2u\]

\[\Delta\dr=-\dfrac{k_2n}{p_L}(1-\theta^2)\sin 2u\]

\[\Delta r\df=\dfrac{k_2n}{p_L}\left[(1-\theta^2)\cos 2u-\dfrac32(1-3

\theta^2)\right]\]

The short-period periodics are added to give the osculating quantities

\[r_k=r\left[1-\dfrac32k_2\dfrac{\sqrt{1-e_L{}^2}}{p_L{}^2}(3\theta^2-1)\right]

+\Delta r\]

\[u_k=u+\Delta u\]

\[\Omega_k=\Omega+\Delta\Omega\]

\[i_k=i_o+\Delta i\]

\[\dr_k=\dr+\Delta\dr\]

\[r\df_k=r\df+\Delta r\df.\]

Then unit orientation vectors are calculated by

\[\Uvec=\Mvec\sin u_k+\Nvec\cos u_k\]

\[\Vvec=\Mvec\cos u_k-\Nvec\sin u_k\]

where

\[\Mvec=\left\{\begin{array}{l}

M_x=-\sin\Omega_k\cos i_k\\

M_y=\cos\Omega_k\cos i_k\\

M_z=\sin i_k

\end{array}\right\}\]

\[\Nvec=\left\{\begin{array}{l}

N_x=\cos\Omega_k\\

N_y=\sin\Omega_k\\

N_z=0

\end{array}\right\}.\]

Then position and velocity are given by

\[\rvec=r_k\Uvec\]

and

\[\drvec=\dr_k\Uvec+(r\df)_k\Vvec.\]

A FORTRAN IV computer code listing of the subroutine SGP4 is given below.

These equations contain all currently anticipated changes to the SCC

operational program.  These changes are scheduled for implementation in March,

1981.

\newpage

\input{SGP4.FOR}

\newpage

\section[The SDP4 Model]{THE SDP4 MODEL}

The NORAD mean element sets can be used for prediction with SDP4.  All symbols

not defined below are defined in the list of symbols in Section Twelve.  The

original mean motion ($\ppno$) and semimajor axis ($\ppao$) are first

recovered from the input elements by the equations

\[a_1=\left(\dfrac{k_e}{n_o}\right)^{\frac23}\]

\[\delta_1=\dfrac32\dfrac{k_2}{a_1{}^2}\dfrac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^{\frac32}}\]

\[a_o=a_1\left(1-\dfrac13\delta_1-\delta_1{}^2-\dfrac{134}{81}\delta_1{}^3\right)\]

\[\delta_o=\dfrac32\dfrac{k_2}{a_o{}^2}\dfrac{(3\cos^2i_o-1)}{(1-e_o{}^2)^{\frac32}}\]

\[\ppno=\dfrac{n_o}{1+\delta_o}\]

\[\ppao=\dfrac{a_o}{1-\delta_o}.\]

For perigee between 98 kilometers and 156 kilometers, the value of the

constant $s$ used in SDP4 is changed to

\[s^*=\ppao(1-e_o)-s+a_E.\]

For perigee below 98 kilometers, the value of $s$ is changed to

\[s^*=20/\mbox{XKMPER}+a_E.\]

If the value of $s$ is changed, then the value of $(q_o-s)^4$ must be replaced

by

\[(q_o-s^*)^4=\left[[(q_o-s)^4]^{\frac14}+s-s^*\right]^4.\]

Then calculate the constants (using the appropriate values of $s$ and

$(q_o-s)^4$)

\[\theta=\cos i_o\]

\[\xi=\dfrac{1}{\ppao-s}\]

\[\beta_o=(1-e_o{}^2)^{\frac12}\]

\[\eta=\ppao e_o\xi\]

\[C_2=\begin{array}[t]{l}(q_o-s)^4\xi^4\ppno(1-\eta^2)^{-\frac72}

\left[\ppao(1+\dfrac32\eta^2+4e_o\eta+e_o\eta^3)\right.\\[12pt]

\left.+\dfrac32\dfrac{k_2\xi}{(1-\eta^2)}

\left(-\dfrac12+\dfrac32\theta^2\right)(8+24\eta^2+3\eta^4)\right]

\end{array}\]

\[C_1=B^*C_2\]

\[C_4=\begin{array}[t]{l}

2\ppno(q_o-s)^4\xi^4\ppao\beta_o{}^2(1-\eta^2)^{-\frac72}

\biggl(\left[2\eta(1+e_o\eta)+\dfrac12e_o+\dfrac12\eta^3\right]

-\dfrac{2k_2\xi}{\ppao(1-\eta^2)}\times\\[12pt]

\left[3(1-3\theta^2)\left(1+\dfrac32\eta^2-2e_o\eta-\dfrac12e_o\eta^3\right)

+\dfrac34(1-\theta^2)(2\eta^2-e_o\eta-e_o\eta^3)\cos 2\omega_o\right]\biggr)

\end{array}\]

\[\dM=\left[1+\dfrac{3k_2(-1+3\theta^2)}{2\ppao{}^2\beta_o{}^3}

+\dfrac{3k_2{}^2(13-78\theta^2+137\theta^4)}{16\ppao{}^4\beta_o{}^7}\right]\ppno\]

\[\dom = \left[-\dfrac{3k_2(1-5\theta^2)}{2\ppao{}^2\beta_o{}^4}

+\dfrac{3k_2{}^2(7-114\theta^2+395\theta^4)}{16\ppao{}^4\beta_o{}^8}

+\dfrac{5k_4(3-36\theta^2+49\theta^4)}{4\ppao{}^4\beta_o{}^8}\right]\ppno\]

\[\dOm_1=-\dfrac{3k_2\theta}{\ppao{}^2\beta_o{}^4}\ppno\]

\[\dOm=\dOm_1+\left[\dfrac{3k_2{}^2(4\theta-19\theta^3)}{2\ppao{}^4\beta_o{}^8}

+\dfrac{5k_4\theta(3-7\theta^2)}{2\ppao{}^4\beta_o{}^8}\right]\ppno.\]

At this point SDP4 calls the initialization section of DEEP which calculates

all initialized quantities needed for the deep-space perturbations (see

Section Ten).

The secular effects of gravity are included by

\[M_{DF}=M_o+\dM\dt\]

\[\omega_{DF}=\omega_o+\dom\dt\]

\[\Omega_{DF}=\Omega_o+\dOm\dt\]

where $\dt$ is time since epoch.  The secular effect of drag on longitude of

ascending node is included by

\[\Omega=\Omega_{DF}-\dfrac{21}2\dfrac{\ppno k_2\theta}{\ppao{}^2\beta_o{}^2}

C_1\dt^2.\]

Next, SDP4 calls the secular section of DEEP which adds the deep-space secular

effects and long-period resonance effects to the six classical orbital

elements (see Section Ten).

The secular effects of drag are included in the remaining elements by

\[a=a_{DS}[1-C_1\dt]^2\]

\[e=e_{DS}-B^*C_4\dt\]

\[\L=M_{DS}+\omega_{DS}+\Omega_{DS}+\ppno \left[

\dfrac32C_1\dt^2\right]\]

where $a_{DS}$, $e_{DS}$, $M_{DS}$, $\omega_{DS}$, and $\Omega_{DS}$, are the

values of $n_o$, $e_o$, $M_{DF}$, $\omega_{DF}$, and $\Omega$ after deep-space

secular and resonance perturbations have been applied.

Here SDP4 calls the periodics section of DEEP which adds the deep-space lunar

and solar periodics to the orbital elements (see Section Ten).  From this

point on, it will be assumed that $n$, $e$, $I$, $\omega$, $\Omega$, and $M$

are the mean motion, eccentricity, inclination, argument of perigee, longitude

of ascending node, and mean anomaly after lunar-solar periodics have been

added.

Add the long-period periodic terms

\[a_{xN}=e\cos\omega\]

\[\beta=\sqrt{(1-e^2)}\]

\[\L_L=\dfrac{A_{3,0}\sin i_o}{8k_2a\beta^2}(e\cos\omega)

\left(\dfrac{3+5\theta}{1+\theta}\right)\]

\[a_{yNL}=\dfrac{A_{3,0}\sin i_o}{4k_2a\beta^2}\]

\[\L_T=\L+\L_L\]

\[a_{yN}=e\sin\omega+a_{yNL}.\]

Solve Kepler's equation for $\Epo$ by defining

\[U=\L_T-\Omega\]

and using the iteration equation

\[\Epo_{i+1}=\Epo_i+\Delta\Epo_i\]

with

\[\Delta\Epo_i =

\dfrac{U-a_{yN}\cos\Epo_i+a_{xN}\sin\Epo_i-\Epo_i}

{-a_{yN}\sin\Epo_i-a_{xN}\cos\Epo_i+1}\]

and

\[\Epo_1=U.\]

The following equations are used to calculate preliminary quantities needed

for short-period periodics.

\[e\cos E=a_{xN}\cos\Epo+a_{yN}\sin\Epo\]

\[e\sin E=a_{xN}\sin\Epo-a_{yN}\cos\Epo\]

\[e_L=(a_{xN}{}^2+a_{yN}{}^2)^{\frac12}\]

\[p_L=a(1-e_L{}^2)\]

\[r=a(1-e\cos E)\]

\[\dr=k_e\dfrac{\sqrt{a}}re\sin E\]

\[r\df=k_e\dfrac{\sqrt{p_L}}r\]

\[\cos u=\dfrac{a}r\left[\cos\Epo-a_{xN}+\dfrac{a_{yN}(e\sin E)}{1+\sqrt{1-e_L{}^2}}\right]\]

\[\sin u=\dfrac{a}r\left[\sin\Epo-a_{yN}-\dfrac{a_{xN}(e\sin E)}{1+\sqrt{1-e_L{}^2}}\right]\]

\[u=\tan^{-1}\left(\dfrac{\sin u}{\cos u}\right)\]

\[\Delta r=\dfrac{k_2}{2p_L}(1-\theta^2)\cos 2u\]

\[\Delta u=-\dfrac{k_2}{4p_L{}^2}(7\theta^2-1)\sin 2u\]

\[\Delta\Omega=\dfrac{3k_2\theta}{2p_L{}^2}\sin 2u\]

\[\Delta i=\dfrac{3k_2\theta}{2p_L{}^2}\sin i_o\cos 2u\]

\[\Delta\dr=-\dfrac{k_2n}{p_L}(1-\theta^2)\sin 2u\]

\[\Delta r\df=\dfrac{k_2n}{p_L}\left[(1-\theta^2)\cos 2u-\dfrac32(1-3\theta^2)\right]\]

The short-period periodics are added to give the osculating quantities

\[r_k=r\left[1-\dfrac32k_2\dfrac{\sqrt{1-e_L{}^2}}{p_L{}^2}(3\theta^2-1)\right]

+\Delta r\]

\[u_k=u+\Delta u\]

\[\Omega_k=\Omega+\Delta\Omega\]

\[i_k=I+\Delta i\]

\[\dr_k=\dr+\Delta\dr\]

\[r\df_k=r\df+\Delta r\df.\]

Then unit orientation vectors are calculated by

\[\Uvec=\Mvec\sin u_k+\Nvec\cos u_k\]

\[\Vvec=\Mvec\cos u_k-\Nvec\sin u_k\]

where

\[\Mvec=\left\{\begin{array}{l}

M_x=-\sin\Omega_k\cos i_k\\

M_y=\cos\Omega_k\cos i_k\\

M_z=\sin i_k

\end{array}\right\}\]

\[\Nvec=\left\{\begin{array}{l}

N_x=\cos\Omega_k\\

N_y=\sin\Omega_k\\

N_z=0

\end{array}\right\}.\]

Then position and velocity are given by

\[\rvec=r_k\Uvec\]

and

\[\drvec=\dr_k\Uvec+(r\df)_k\Vvec.\]

A FORTRAN IV computer code listing of the subroutine SDP4 is given below.

These equations contain all currently anticipated changes to the SCC

operational program.  These changes are scheduled for implementation in March,

1981.

\newpage

\input{SDP4.FOR}

\newpage

\section[The SGP8 Model]{THE SGP8 MODEL}

The NORAD mean element sets can be used for prediction with SGP8.  All symbols

not defined below are defined in the list of symbols in Section Twelve.  The

original mean motion ($\ppno$) and semimajor axis ($\ppao$) are first

recovered from the input elements by the equations

\[a_1=\left(\dfrac{k_e}{n_o}\right)^{\frac23}\]

\[\delta_1=\frac32\frac{k_2}{a_1{}^2}\frac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^\frac32}\]

\[a_o=a_1\left(1-\dfrac13\delta_1-\delta_1{}^2

-\dfrac{134}{81}\delta_1{}^3\right)\]

\[\delta_o=\dfrac32\dfrac{k_2}{a_o{}^2}\dfrac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^{\frac32}}\]

\[\ppno =\dfrac{n_o}{1+\delta_o}\]

\[\ppao=\dfrac{a_o}{1-\delta_o}.\]

The ballistic coefficient ($B$ term) is then calculated from the $B^*$ drag

term by

\[B=2B^*/\rho_o\]

where

\[\rho_o=(2.461\times 10^{-5})\mbox{ XKMPER kg/m$^2$/Earth radii}\]

is a reference value of atmospheric density.

Then calculate the constants

\[\beta^2=1-e^2\]

\[\theta=\cos i\]

\[\dM_1=-\dfrac32\dfrac{\ppn k_2}{\ppa^2\beta^3}(1-3\theta^2)\]

\[\dom_1=-\dfrac32\dfrac{\ppn k_2}{\ppa^2\beta^4}(1-5\theta^2)\]

\[\dOm_1=-3\dfrac{\ppn k_2}{\ppa^2\beta^4}\theta\]

\[\dM_2=\dfrac3{16}\dfrac{\ppn k_2{}^2}{\ppa^4\beta^7}(13-

78\theta^2+137\theta^4)\]

\[\dom_2=\dfrac3{16}\dfrac{\ppn k_2{}2}{\ppa^4\beta^8}(7-

114\theta^2+395\theta^4)+\dfrac54\dfrac{\ppn k_4}{\ppa^4\beta^8}(3-

36\theta^2+49\theta^4)\]

\[\dOm_2=\dfrac32\dfrac{\ppn k_2{}^2}{\ppa^4\beta^8}\theta(4-19\theta^2)

+\dfrac52\dfrac{\ppn k_4}{\ppa^4\beta^8}\theta(3-7\theta^2)\]

\[\dot\ell=\ppn+\dM_1+\dM_2\]

\[\dom=\dom_1+\dom_2\]

\[\dOm=\dOm_1+\dOm_2\]

\[\xi=\dfrac1{\ppa\beta^2-s}\]

\[\eta=es\xi\]

\[\psi=\sqrt{1-\eta^2}\]

\[\alpha^2=1+e^2\]

\[C_o=\dfrac12B\rho_o(q_o-s)^4\ppn\ppa\xi^4\alpha^{-1}\psi^{-7}\]

\[C_1=\dfrac32\ppn\alpha^4C_o\]

\[D_1=\xi\psi^{-2}/\ppa\beta^2\]

\[D_2=12+36\eta^2+\dfrac92\eta^4\]

\[D_3=15\eta^2+\dfrac52\eta^4\]

\[D_4=5\eta+\dfrac{15}4\eta^3\]

\[D_5=\xi\psi^{-2}\]

\[B_1=-k_2(1-3\theta^2)\]

\[B_2=-k_2(1-\theta^2)\]

\[B_3=\dfrac{A_{3,0}}{k_2}\sin i\]

\[C_2=D_1D_3B_2\]

\[C_3=D_4D_5B_3\]

\[\dn_o=C_1\left(2+3\eta^2+20e\eta+5e\eta^3+\dfrac{17}2e^2+34e^2\eta^2+D_1D_2B_1

+C_2\cos 2\omega+C_3\sin\omega\right)\]

\[C_4=D_1D_7B_2\]

\[C_5=D_5D_8B_3\]

\[D_6=30\eta+\dfrac{45}2\eta^3\]

\[D_7=5\eta+\dfrac{25}2\eta^3\]

\[D_8=1+\dfrac{27}4\eta^2+\eta^4\]

\[\de_o=-C_o\left(4\eta+\eta^3+5e+15e\eta^2+\dfrac{31}2e^2\eta+7e^2\eta^3+D_1D_6B_1

+C_4\cos 2\omega+C_5\sin\omega\right)\]

\[\dal/\alpha=e\de\alpha^{-2}\]

\[C_6=\dfrac13\dfrac{\dn}{\ppn}\]

\[\dxi/\xi=2\ppa\xi(C_6\beta^2+e\de)\]

\[\deta=(\de+e\dxi/\xi)s\xi\]

\[\dpsi/\psi=-\eta\deta\psi^{-2}\]

\[\dC_o/C_o=C_6+4\dxi/\xi-\dal/\alpha-7\dpsi/\psi\]

\[\dC_1/C_1=\dn/\ppn+4\dal/\alpha+\dC_o/C_o\]

\[D_9=6\eta+20e+15e\eta^2+68e^2\eta\]

\[D_{10}=20\eta+5\eta^3+17e+68e\eta^2\]

\[D_{11}=72\eta+18\eta^3\]

\[D_{12}=30\eta+10\eta^3\]

\[D_{13}=5+\dfrac{45}4\eta^2\]

\[D_{14}=\dxi/\xi-2\dpsi/\psi\]

\[D_{15}=2(C_6+e\de\beta^{-2})\]

\[\dD_1=D_1(D_{14}+D_{15})\]

\[\dD_2=\deta D_{11}\]

\[\dD_3=\deta D_{12}\]

\[\dD_4=\deta D_{13}\]

\[\dD_5=D_5D_{14}\]

\[\dC_2=B_2(\dD_1D_3+D_1\dD_3)\]

\[\dC_3=B_3(\dD_5D_4+D_5\dD_4)\]

\[\dom=-\dfrac32\dfrac{\ppn k_2}{\ppa^2\beta^4}(1-5\theta^2)\]

\[D_{16}=D_9\deta+D_{10}\de+B_1(\dD_1D_2+D_1\dD_2)

+\dC_2\cos 2\omega+\dC_3\sin\omega+\dom(C_3\cos\omega-2C_2\sin 2\omega)\]

\[\ddn_o=\dn\dC_1/C_1+C_1D_{16}\]

\[\dde_o=\begin{array}[t]{l}

\de\dC_o/C_o-C_o\left\{\left(4+3\eta^2+30e\eta+\dfrac{31}2e^2+21e^2\eta^2\right)\deta

+(5+15\eta^2+31e\eta+14e\eta^3)\de\right.\\[12pt]

+B_1\left[\dD_1D_6+D_1\deta\left(30+\dfrac{135}2\eta^2\right)\right]

+B_2\left[\dD_1D_7+D_1\deta\left(5+\dfrac{75}2\eta^2\right)\right]\cos\omega\\[12pt]

\left.+B_3\left[\dD_5D_8+D_5\eta\deta\left(\dfrac{27}2+4\eta^2\right)\right]\sin\omega

+\dom(C_5\cos\omega-2C_4\sin 2\omega)\right\}

\end{array}\]

\[D_{17}=\ddn/\ppn-(\dn/\ppn)^2\]

\[\ddxi/\xi=2(\dxi/\xi-C_6)\dxi/\xi+2\ppa\xi\left(\dfrac13D_{17}\beta^2-2C_6e\de+\de^2+e\dde\right)\]

\[\ddeta=(\dde+2\de\dxi/\xi)s\xi+\eta\ddxi/\xi\]

\[D_{18}=\ddxi/\xi-(\dxi/\xi)^2\]

\[D_{19}=-(\dpsi/\psi)^2(1+\eta^{-2})-\eta\ddeta\psi^{-2}\]

\[\ddD_1=\dD_1(D_{14}+D_{15})+D_1\left(D_{18}-2D_{19}

+\dfrac23D_{17}+2\alpha^2\de^2\beta^{-4}+2e\dde\beta^{-2}\right)\]

\[\dddn_o=\begin{array}[t]{l}

\dn\left[\dfrac43D_{17}+3\de^2\alpha^{-2}+3e\dde\alpha^{-2}

-6(\dal/\alpha)^2+4D_{18}-7D_{19}\right]\\[12pt]

+\ddn\dC_1/C_1+C_1\biggl\{D_{16}\dC_1/C_1+D_9\ddeta+D_{10}\dde+\deta^2(6+30e\eta+68e^2)\\[12pt]

+\deta\de(40+30\eta^2+272e\eta)+\de^2(17+68\eta^2)\\[12pt]

+B_1[\ddD_1D_2+2\dD_1\dD_2+D_1(\ddeta D_{11}+\deta^2(72+54\eta^2))]\\[12pt]

+B_2[\ddD_1D_3+2\dD_1\dD_3+D_1(\ddeta D_{12}+\deta^2(30+30\eta^2))]\cos 2\omega\\[12pt]

+B_3\left[(\dD_5D_{14}+D_5(D_{18}-2D_{19}))D_4+2\dD_4\dD_5+D_5\left(\ddeta D_{13}+\dfrac{45}2\eta\deta^2\right)\right]\sin\omega\\[12pt]

+\dom[(7C_6+4e\de\beta^{-2})(C_3\cos\omega-2C_2\sin 2\omega)+2C_3\cos\omega\\[12pt]

-4C_2\sin 2\omega-\dom(C_3\sin\omega+4C_2\cos 2\omega)]\biggr\}

\end{array}\]

\[p=\dfrac{2\ddn_o{}^2-\dn_o\dddn_o}{\ddn_o{}^2-\dn_o\dddn_o}\]

\[\gamma=-\dfrac{\dddn_o}{\ddn_o}\dfrac1{(p-2)}\]

\[n_D=\dfrac{\dn_o}{p\gamma}\]

\[q=1-\dfrac{\dde_o}{\de_o\gamma}\]

\[e_D=\dfrac{\de_o}{q\gamma}\]

where all quantities are epoch values.

The secular effects of atmospheric drag and gravitation are included by

\[n=\ppno+n_D[1-(1-\gamma\dt)^p]\]

\[e=e_o+e_D[1-(1-\gamma\dt)^q]\]

\[\omega=\omega_o+\dom_1\left[\dt+\dfrac73\dfrac1{\ppno}Z_1\right]+\dom_2\dt\]

\[\Omega=\Omega_o''+\dOm_1\left[\dt+\dfrac73\dfrac1{\ppno}Z_1\right]+\dOm_2\dt\]

\[M=M_o+\ppno\dt+Z_1+\dM_1\left[\dt+\dfrac73\dfrac1{\ppno}Z_1\right]+\dM_2\dt\]

where

\[Z_1=\dfrac{\dn_o}{p\gamma}\left\{\dt+\dfrac1{\gamma(p+1)}[(1-\gamma\dt)^{p+1}-1]\right\}.\]

If drag is very small ($\dfrac{\dn}{\ppno }$ less than $1.5 \times 10^{-6}$/min) then the

secular equations for $n$, $e$, and $Z_1$ should be replaced by

\[n=\ppno+\dn\dt\]

\[e=e''_o+\de\dt\]

\[Z_1=\dfrac12\dn_o\dt^2\]

where $\dt$ is time since epoch and where

\[\de=-\dfrac23\dfrac{\dn_o}{\ppno}(1-e_o).\]

Solve Kepler's equation for $E$ by using the iteration equation

\[E_{i+1}=E_i+\Delta E_i\]

with

\[\Delta E_i=\dfrac{M+e\sin E_i-E_i}{1-e\cos E_i}\]

and

\[E_1=M+e\sin M+\dfrac12e^2\sin 2M.\]

The following equations are used to calculate preliminary quantities needed for

the short-period periodics.

\[a=\left(\dfrac{k_e}n\right)^{\frac23}\]

\[\beta=(1-e^2)^{\frac12}\]

\[\sin f=\dfrac{\beta\sin E}{1-e\cos E}\]

\[\cos f=\dfrac{\cos E-e}{1-e\cos E}\]

\[u=f+\omega\]

\[r''=\dfrac{a\beta^2}{1+e\cos f}\]

\[\dr''=\dfrac{nae}\beta\sin f\]

\[(r\df)''=\dfrac{na^2\beta}r\]

\[\delta r=\dfrac12\dfrac{k_2}{a\beta^2}[(1-\theta^2)\cos 2u

+3(1-3\theta^2)]-\dfrac14\dfrac{A_{3,0}}{k_2}\sin i_o\sin u\]

\[\delta\dr=-n\left(\dfrac ar\right)^2\left[\dfrac{k_2}{a\beta^2}(1-\theta^2)\sin 2u

+\dfrac14\dfrac{A_{3,0}}{k_2}\sin i_o\cos u\right]\]

\[\delta I=\theta\left[\dfrac32\dfrac{k_2}{a^2\beta^4}\sin i_o\cos 2u

-\dfrac14\dfrac{A_{3,0}}{k_2a\beta^2}e\sin\omega\right]\]

\[\delta(r\df)=-n\left(\dfrac ar\right)^2\delta r+na\left(\dfrac ar\right)\dfrac{\sin i_o}\theta\delta I\]

\[\delta u=\begin{array}[t]{l}

\dfrac12\dfrac{k_2}{a^2\beta^4}\left[\dfrac12(1-7\theta^2)\sin 2u-3(1-5\theta^2)(f-M+e\sin f)\right]\\[12pt]

-\dfrac14\dfrac{A_{3,0}}{k_2a\beta^2}\left[\sin i_o\cos u(2+e\cos f)

+\dfrac12\dfrac{\theta^2}{\sin i_o/2\:\cos i_o/2}e\cos\omega\right]

\end{array}\]

\[\delta\lambda=\begin{array}[t]{l}

\dfrac12\dfrac{k_2}{a^2\beta^4}\left[\dfrac12(1+6\theta-7\theta^2)\sin 2u

-3(1+2\theta-5\theta^2)(f-M+e\sin f)\right]\\[12pt]

+\dfrac14\dfrac{A_{3,0}}{k_2a\beta^2}\sin i_o

\left[\dfrac{e\theta}{1+\theta}\cos\omega-(2+e\cos f)\cos u\right]

\end{array}\]

The short-period periodics are added to give the osculating quantities

\[r=r''+\delta r\]

\[\dr=\dr''+\delta\dr\]

\[r\df=(r\df)''+\delta(r\df)\]

\[y_4=\sin \dfrac{i_o}2\sin u+\cos u\sin \dfrac{i_o}2\delta u+\dfrac12\sin u\cos \dfrac{i_o}2\delta I\]

\[y_5=\sin \dfrac{i_o}2\cos u-\sin u\sin \dfrac{i_o}2\delta u+\dfrac12\cos u\cos \dfrac{i_o}2\delta I\]

\[\lambda=u+\Omega+\delta\lambda.\]

Unit orientation vectors are calculated by

\[U_x=2y_4(y_5\sin\lambda-y_4\cos\lambda)+cos\lambda\]

\[U_y=-2y_4(y_5\cos\lambda+y_4\sin\lambda)+sin\lambda\]

\[U_z=2y_4\cos \dfrac{I}2\]

\[V_x=2y_5(y_5\sin\lambda-y_4\cos\lambda)-sin\lambda\]

\[V_y=-2y_5(y_5\cos\lambda+y_4\sin\lambda)+cos\lambda\]

\[V_z=2y_5\cos \dfrac{I}2\]

where

\[\cos \dfrac{I}2=\sqrt{1-y_4{}^2-y_5{}^2}.\]

Position and velocity are given by

\[\rvec=r\Uvec\]

\[\drvec=\dr\Uvec+r\df\Vvec.\]

A FORTRAN IV computer code listing of the subroutine SGP8 is given below.

\newpage

\input{SGP8.FOR}

\newpage

\section[The SDP8 Model]{THE SDP8 MODEL}

The NORAD mean element sets can be used for prediction with SDP8.  All symbols

not defined below are defined in the list of symbols in Section Twelve.  The

original mean motion ($\ppno$) and semimajor axis ($\ppao$) are first

recovered from the input elements by the equations

\[a_1=\left(\dfrac{k_e}{n_o}\right)^{\frac23}\]

\[\delta_1=\frac32\frac{k_2}{a_1{}^2}\frac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^\frac32}\]

\[a_o=a_1\left(1-\dfrac13\delta_1-\delta_1{}^2

-\dfrac{134}{81}\delta_1{}^3\right)\]

\[\delta_o=\dfrac32\dfrac{k_2}{a_o{}^2}\dfrac{(3\cos^2i_o-1)}

{(1-e_o{}^2)^{\frac32}}\]

\[\ppno =\dfrac{n_o}{1+\delta_o}\]

\[\ppao=\dfrac{a_o}{1-\delta_o}.\]

The ballistic coefficient ($B$ term) is then calculated from the $B^*$ drag

term by

\[B=2B^*/\rho_o\]

where

\[\rho_o=(2.461\times 10^{-5})\mbox{ XKMPER kg/m$^2$/Earth radii}\]

is a reference value of atmospheric density.

Then calculate the constants

\[\beta^2=1-e^2\]

\[\theta=\cos i\]

\[\dM_1=-\dfrac32\dfrac{\ppn k_2}{\ppa^2\beta^3}(1-3\theta^2)\]

\[\dom_1=-\dfrac32\dfrac{\ppn k_2}{\ppa^2\beta^4}(1-5\theta^2)\]

\[\dOm_1=-3\dfrac{\ppn k_2}{\ppa^2\beta^4}\theta\]

\[\dM_2=\dfrac3{16}\dfrac{\ppn k_2{}^2}{\ppa^4\beta^7}(13-

78\theta^2+137\theta^4)\]

\[\dom_2=\dfrac3{16}\dfrac{\ppn k_2{}2}{\ppa^4\beta^8}(7-

114\theta^2+395\theta^4)+\dfrac54\dfrac{\ppn k_4}{\ppa^4\beta^8}(3-

36\theta^2+49\theta^4)\]

\[\dOm_2=\dfrac32\dfrac{\ppn k_2{}^2}{\ppa^4\beta^8}\theta(4-19\theta^2)

+\dfrac52\dfrac{\ppn k_4}{\ppa^4\beta^8}\theta(3-7\theta^2)\]

\[\dot\ell=\ppno+\dM_1+\dM_2\]

\[\dom=\dom_1+\dom_2\]

\[\dOm=\dOm_1+\dOm_2\]

\[\xi=\dfrac1{\ppa\beta^2-s}\]

\[\eta=es\xi\]

\[\psi=\sqrt{1-\eta^2}\]

\[\alpha^2=1+e^2\]

\[C_o=\dfrac12B\rho_o(q_o-s)^4\ppn\ppa\xi^4\alpha^{-1}\psi^{-7}\]

\[C_1=\dfrac32\ppn\alpha^4C_o\]

\[D_1=\xi\psi^{-2}/\ppa\beta^2\]

\[D_2=12+36\eta^2+\dfrac92\eta^4\]

\[D_3=15\eta^2+\dfrac52\eta^4\]

\[D_4=5\eta+\dfrac{15}4\eta^3\]

\[D_5=\xi\psi^{-2}\]

\[B_1=-k_2(1-3\theta^2)\]

\[B_2=-k_2(1-\theta^2)\]

\[B_3=\dfrac{A_{3,0}}{k_2}\sin i\]

\[C_2=D_1D_3B_2\]

\[C_3=D_4D_5B_3\]

\[\dn_o=C_1\left(2+3\eta^2+20e\eta+5e\eta^3+\dfrac{17}2e^2+34e^2\eta^2+D_1D_2B_1

+C_2\cos 2\omega+C_3\sin\omega\right)\]

\[\de_o=-\dfrac23\dfrac{\dn}{\ppn}(1-e)\]

where all quantities are epoch values.

At this point SDP8 calls the initialization section of DEEP which calculates all

initialized quantities needed for the deep-space perturbations (see Section

Ten).

The secular effect of gravity is included in mean anomaly by

\[M_{DF}=M_o+\dot\ell\dt\]

and the secular effects of gravity and atmospheric drag are included in

argument of perigee and longitude of ascending node by

\[\omega=\omega_o+\dom\dt+\dom_1Z_7\]

\[\Omega=\Omega_o+\dOm\dt+\dOm_1Z_7\]

where

\[Z_7=\dfrac73Z_1/\ppno\]

with

\[Z_1=\dfrac12\dn_o\dt^2.\]

Next, SDP8 calls the secular section of DEEP which adds the deep-space secular

effects and long-period resonance effects to the six classical orbital elements

(see Section Ten).

The secular effects of drag are included in the remaining elements by

\[n=n_{DS}+\dn_o\dt\]

\[e=e_{DS}+\de_o\dt\]

\[M=M_{DS}+Z_1+\dM_1Z_7\]

where $n_{DS}$, $e_{DS}$, $M_{DS}$ are the values of $n_o$, $e_o$, $M_{DF}$

after deep-space secular and resonance perturbations have been applied.

Here, SDP8 calls the periodics section of DEEP which adds the deep-space lunar

and solar periodics to the orbital elements (see Section Ten).  From this point

on, it will be assumed that $n$, $e$, $I$, $\omega$, $\Omega$, and $M$ are the

mean motion, eccentricity, inclination, argument of perigee, longitude of

ascending node, and mean anomaly after lunar-solar periodics have been added.

Solve Kepler's equation for $E$ by using the iteration equation

\[E_{i+1}=E_i+\Delta E_i\]

with

\[\Delta E_i=\dfrac{M+e\sin E_i-E_i}{1-e\cos E_i}\]

and

\[E_1=M+e\sin M+\dfrac12e^2\sin 2M.\]

The following equations are used to calculate preliminary quantities needed for

the short-period periodics.

\[a=\left(\dfrac{k_e}n\right)^{\frac23}\]

\[\beta=(1-e^2)^{\frac12}\]

\[\sin f=\dfrac{\beta\sin E}{1-e\cos E}\]

\[\cos f=\dfrac{\cos E-e}{1-e\cos E}\]

\[u=f+\omega\]

\[r''=\dfrac{a\beta^2}{1+e\cos f}\]

\[\dr''=\dfrac{nae}\beta\sin f\]

\[(r\df)''=\dfrac{na^2\beta}r\]

\[\delta r=\dfrac12\dfrac{k_2}{a\beta^2}[(1-\theta^2)\cos 2u

+3(1-3\theta^2)]-\dfrac14\dfrac{A_{3,0}}{k_2}\sin i_o\sin u\]

\[\delta\dr=-n\left(\dfrac ar\right)^2\left[\dfrac{k_2}{a\beta^2}(1-\theta^2)\sin 2u

+\dfrac14\dfrac{A_{3,0}}{k_2}\sin i_o\cos u\right]\]

\[\delta I=\theta\left[\dfrac32\dfrac{k_2}{a^2\beta^4}\sin i_o\cos 2u

-\dfrac14\dfrac{A_{3,0}}{k_2a\beta^2}e\sin\omega\right]\]

\[\delta(r\df)=-n\left(\dfrac ar\right)^2\delta r+na\left(\dfrac ar\right)\dfrac{\sin i_o}\theta\delta I\]

\[\delta u=\begin{array}[t]{l}

\dfrac12\dfrac{k_2}{a^2\beta^4}\left[\dfrac12(1-7\theta^2)\sin 2u-3(1-5\theta^2)(f-M+e\sin f)\right]\\[12pt]

-\dfrac14\dfrac{A_{3,0}}{k_2a\beta^2}\left[\sin i_o\cos u(2+e\cos f)

+\dfrac12\dfrac{\theta^2}{sin i_o/2\:\cos i_o/2}e\cos\omega\right]

\end{array}\]

\[\delta\lambda=\begin{array}[t]{l}

\dfrac12\dfrac{k_2}{a^2\beta^4}\left[\dfrac12(1+6\theta-7\theta^2)\sin 2u

-3(1+2\theta-5\theta^2)(f-M+e\sin f)\right]\\[12pt]

+\dfrac14\dfrac{A_{3,0}}{k_2a\beta^2}\sin i_o

\left[\dfrac{e\theta}{1+\theta}\cos\omega-(2+e\cos f)\cos u\right]

\end{array}\]

The short-period periodics are added to give the osculating quantities

\[r=r''+\delta r\]

\[\dr=\dr''+\delta\dr\]

\[r\df=(r\df)''+\delta(r\df)\]

\[y_4=\sin \dfrac{I}2\sin u+\cos u\sin \dfrac{i_o}2\delta u+\dfrac12\sin u\cos \dfrac{i_o}2\delta I\]

\[y_5=\sin \dfrac{I}2\cos u-\sin u\sin \dfrac{i_o}2\delta u+\dfrac12\cos u\cos \dfrac{i_o}2\delta I\]

\[\lambda=u+\Omega+\delta\lambda.\]

Unit orientation vectors are calculated by

\[U_x=2y_4(y_5\sin\lambda-y_4\cos\lambda)+cos\lambda\]

\[U_y=-2y_4(y_5\cos\lambda+y_4\sin\lambda)+sin\lambda\]

\[U_z=2y_4\cos \dfrac{I}2\]

\[V_x=2y_5(y_5\sin\lambda-y_4\cos\lambda)-sin\lambda\]

\[V_y=-2y_5(y_5\cos\lambda+y_4\sin\lambda)+cos\lambda\]

\[V_z=2y_5\cos \dfrac{I}2\]

where

\[\cos \dfrac{I}2=\sqrt{1-y_4{}^2-y_5{}^2}.\]

Position and velocity are given by

\[\rvec=r\Uvec\]

\[\drvec=\dr\Uvec+r\df\Vvec.\]

A FORTRAN IV computer code listing of the subroutine SDP8 is given below.

\newpage

\input{SDP8.FOR}

\newpage

\section[The Deep-Space Subroutine]{THE DEEP-SPACE SUBROUTINE}

The two deep-space models, SDP4 and SDP8, both access the subroutine DEEP to

obtain the deep-space perturbations of the six classical orbital elements.

The perturbation equations are quite extensive and will not be repeated here.

Rather, this section will concentrate on a general description of the flow

between the main program and the deep-space subroutines.  A specific listing

of the equations is available in Hujsak (1979) or Hujsak and Hoots (1977).

The first time the deep-space subroutine is accessed is during the

initialization portion of SDP4/ SDP8 and is via the entry DPINIT.  Through this

entry, certain constants already calculated in SDP4/ SDP8 are passed to the

deep-space subroutine which in turn calculates all initialized (time

independent) quantities needed for prediction in deep space.  Additionally, a

determination is made and flags are set concerning whether the orbit is

synchronous and whether the orbit experiences resonance effects.

The next access to the deep-space subroutine occurs during the secular update

portion of SDP4/ SDP8 and is via the entry DPSEC.  Through this entry, the

current secular values of the ``mean'' orbital elements are passed to the

deep-space subroutine which in turn adds the appropriate deep-space secular

and long-period resonance effects to these mean elements.

The last access to the deep-space subroutine occurs at the beginning of the

osculation portion (periodics application) of SDP4/SDP8 and is via the entry

DPPER.  Through this entry, the current values of the orbital elements are

passed to the deep-space subroutine which in turn adds the appropriate deep-

space lunar and solar periodics to the orbital elements.

During initialization the deep-space subroutine calls the function subroutine

THETAG to obtain the location of Greenwich at epoch and to convert epoch to

minutes since 1950.  All physical constants which are unique to the deep-space

subroutine are set via data statements in DEEP rather than being passed

through a COMMON.

A FORTRAN IV computer code listing of the subroutine DEEP is given below.

These equations contain all currently anticipated changes to the SCC

operational program.  These changes are scheduled for implementation in March,

1981.

\newpage

\input{DEEP.FOR}

\newpage

\section[Driver and Function Subroutines]{DRIVER AND FUNCTION SUBROUTINES}

The DRIVER controls the input and output function and the selection of the

model.  The input consists of a program card which specifies the model to be

used and the output times and either a G-card or T-card element set.

The DRIVER reads and converts the input elements to units of radians and

minutes.  These are communicated to the prediction model through the COMMON

E1.  Values of the physical and mathematical constants are set and

communicated through the COMMONs C1 and C2, respectively.

The program card indicates the mathematical model to be used and the start and

stop time of prediction as well as the increment of time for output.  These

times are in minutes since epoch.

In the interest of efficiency the DRIVER sets a flag (IFLAG) the first time

the model is called.  This flag tells the model to calculate all initialized

(time independent) quantities.  After initialization, the model subroutine

turns off the flag so that all subsequent calls only access the time dependent

part of the model.  This mode continues until another input case is

encountered.

The DRIVER takes the output from the mathematical model (communicated through

the COMMON E1) and converts it to units of kilometers and seconds for

printout.

The function subroutine ACTAN is passed the values of sine and cosine in that

order and it returns the angle in radians within the range of 0 to $2\pi$.

The function subroutine FMOD2P is passed an angle in radians and returns the

angle in radians within the range of 0 to $2\pi$.  The function subroutine

THETAG is passed the epoch time exactly as it appears on the input element

cards.\footnote[1]{If only one year digit is given (as on standard G-cards) the

program assumes the 80 decade.  This may be overridden by putting a 2 digit

year in columns 30--31 of the first G-card.}  The routine converts this time

to days since 1950 Jan 0.0 UTC, stores this in the COMMON E1, and returns the

right ascension of Greenwich at epoch (in radians).

FORTRAN IV computer code listings of the routines DRIVER, ACTAN, FMOD2P,

and THETAG are given below.

\newpage

\input{DRIVER.FOR}

\newpage

\input{ACTAN.FOR}

\newpage

\input{FMOD2P.FOR}

\newpage

\input{THETAG.FOR}

\newpage

\section[Users Guide, Constants, and Symbols]{USERS GUIDE, CONSTANTS, AND SYMBOLS}

The first input card is the program card.  The format is as follows:

\begin{verbatim}

Column     Format     Description

1         I1         Ephemeris program desired

1 = SGP

2 = SGP4

3 = SDP4

4 = SGP8

5 = SDP8

2-11      F10.0      Prediction start time

12-21      F10.0      Prediction stop time

22-31      F10.0      Time increment

\end{verbatim}

\noindent

All times are in minutes since epoch and can be positive or negative.  The

second and third input cards consist of either a 2-card transmission or 2-card

G type element set.  Either type can be used with the only condition being

that the two cards must be in the correct order.  For reference a format sheet

for the T-card and G-card element sets follows this section.

The values of the physical and mathematical constants used in the program are

given below.

\begin{tabbing}

Variable name \quad \= distance units/Earth radii \quad \= \kill

\underline{Variable name} \> \underline{Definition} \> \underline{Value}\\[12pt]

CK2 \> $\dfrac12J_2a_E{}^2$ \> 5.413080E-4\\[12pt]

CK4 \> $-\dfrac38J_4a_E{}^4$ \> .62098875E-6\\[12pt]

E6A \> 10$^{-6}$ \> 1.0 E-6\\[12pt]

QOMS2T \> $(q_o-s)^4\mbox{ (er)}^4$ \> 1.88027916E-9\\[12pt]

S \> $s\mbox{ (er)}$ \> 1.01222928\\[12pt]

TOTHRD \> 2/3 \> .66666667\\[12pt]

XJ3 \> $J_3$ \> -.253881E-5\\[12pt]

XKE \> $k_e\left(\dfrac{\mbox{er}}{\mbox{min}}\right)^{\frac32}$ \> .743669161E-1\\[12pt]

XKMPER \> kilometers/Earth radii \> 6378.135\\[12pt]

XMNPDA \> time units/day \> 1440.0\\[12pt]

AE \> distance units/Earth radii \> 1.0\\[12pt]

DE2RA \> radians/degree \> .174532925E-1\\[12pt]

PI \> $\pi$ \> 3.14159265\\[12pt]

PIO2 \> $\pi/2$ \> 1.57079633\\[12pt]

TWOPI \> $2\pi$ \> 6.2831853\\[12pt]

X3PIO2 \> $3\pi/2$ \> 4.71238898

\end{tabbing}

\noindent

where er = Earth radii.  Except for the deep-space models, all ephemeris

models are independent of units.  Thus, units input or output as well as

physical constants can be changed by making the appropriate changes in only

the DRIVER program.

Following is a list of symbols commonly used in this report.

\begin{quote}

$n_o$ = the SGP type ``mean'' mean motion at epoch \\[12pt]

$e_o$ = the ``mean'' eccentricity at epoch \\[12pt]

$i_o$ = the ``mean'' inclination at epoch \\[12pt]

$M_o$ = the ``mean'' mean anomaly at epoch \\[12pt]

$\omega_o$ = the ``mean'' argument of perigee at epoch \\[12pt]

$\Omega_o$ = the ``mean'' longitude of ascending node at epoch \\[12pt]

$\dn_o$ = the time rate of change of ``mean'' mean motion at epoch \\[12pt]

$\ddn_o$ = the second time rate of change of ``mean'' mean motion at epoch \\[12pt]

$B^*$ = the SGP4 type drag coefficient \\[12pt]

$k_e$ = \parbox[t]{5.25in}{$\sqrt{GM}$ where $G$ is Newton's universal

gravitational constant and $M$ is the mass of the Earth} \\[12pt]

$a_E$ = the equatorial radius of the Earth \\[12pt]

$J_2$ = the second gravitational zonal harmonic of the Earth \\[12pt]

$J_3$ = the third gravitational zonal harmonic of the Earth \\[12pt]

$J_4$ = the fourth gravitational zonal harmonic of the Earth \\[12pt]

$\dt$ = time since epoch \\[12pt]

$k_2 = \dfrac12J_2a_E{}^2$ \\[12pt]

$k_4 = -\dfrac38J_4a_E{}^4$ \\[12pt]

$A_{3,0} = -J_3a_E{}^3$ \\[12pt]

$q_o$ = parameter for the SGP4/SGP8 density function \\[12pt]

$s$ = parameter for the SGP4/SGP8 density function \\[12pt]

$B$ = \parbox[t]{5.25in}{$\dfrac12C_D\dfrac Am$, the ballistic coefficient for SGP8

where $C_D$ is a dimensionless drag coefficient and $A$ is the average cross-%

sectional area of the satellite of mass $m$} \\[12pt]

\end{quote}

\newpage

\section[Sample Test Cases]{SAMPLE TEST CASES}

For reference a sample test case is given for each of the five

models.\footnote[1]{The test cases were generated on a machine with 8

digits of accuracy.  After a one day prediction, the test cases have only 5 to

6 digits of accuracy.}  The input used was standard T-cards and the output is

given at 360 minute intervals in units of kilometers and seconds.

When implemented on a given computer, the accuracies with which the test cases

are duplicated will be dominated by the accuracy of the epoch mean motion.

If, after reading and converting, the epoch mean motion has an error $\Delta n

= j \times 10^{-k}$ radians/time, then the predicted positions at time t may

differ from the test cases by numbers on the order of

\[\Delta r=\Delta n\dt(6,378.135) \mbox{ kilometers}\]

\newpage

\begin{verbatim}

1 88888U          80275.98708465  .00073094  13844-3  66816-4 0    8

2 88888  72.8435 115.9689 0086731  52.6988 110.5714 16.05824518  105

SGP  TSINCE              X                Y                Z

0.            2328.96594238   -5995.21600342    1719.97894287

360.00000000    2456.00610352   -6071.94232177    1222.95977784

720.00000000    2567.39477539   -6112.49725342     713.97710419

1080.00000000    2663.03179932   -6115.37414551     195.73919105

1440.00000000    2742.85470581   -6079.13580322    -328.86091614

XDOT             YDOT             ZDOT

2.91110113      -0.98164053      -7.09049922

2.67852119      -0.44705850      -7.22800565

2.43952477       0.09884824      -7.31889641

2.19531813       0.65333930      -7.36169147

1.94707947       1.21346101      -7.35499924

\end{verbatim}

\newpage

\begin{verbatim}

1 88888U          80275.98708465  .00073094  13844-3  66816-4 0    8

2 88888  72.8435 115.9689 0086731  52.6988 110.5714 16.05824518  105

SGP4 TSINCE              X                Y                Z

0.            2328.97048951   -5995.22076416    1719.97067261

360.00000000    2456.10705566   -6071.93853760    1222.89727783

720.00000000    2567.56195068   -6112.50384522     713.96397400

1080.00000000    2663.09078980   -6115.48229980     196.39640427

1440.00000000    2742.55133057   -6079.67144775    -326.38095856

XDOT             YDOT             ZDOT

2.91207230      -0.98341546      -7.09081703

2.67938992      -0.44829041      -7.22879231

2.44024599       0.09810869      -7.31995916

2.19611958       0.65241995      -7.36282432

1.94850229       1.21106251      -7.35619372

\end{verbatim}

\newpage

\begin{verbatim}

1 11801U          80230.29629788  .01431103  00000-0  14311-1

2 11801  46.7916 230.4354 7318036  47.4722  10.4117  2.28537848

SDP4 TSINCE              X                Y                Z

0.            7473.37066650     428.95261765    5828.74786377

360.00000000   -3305.22537232   32410.86328125  -24697.17675781

720.00000000   14271.28759766   24110.46411133   -4725.76837158

1080.00000000   -9990.05883789   22717.35522461  -23616.89062501

1440.00000000    9787.86975097   33753.34667969  -15030.81176758

XDOT             YDOT             ZDOT

5.10715413       6.44468284      -0.18613096

-1.30113538      -1.15131518      -0.28333528

-0.32050445       2.67984074      -2.08405289

-1.01667246      -2.29026759       0.72892364

-1.09425066       0.92358845      -1.52230928

\end{verbatim}

\newpage

\begin{verbatim}

1 88888U          80275.98708465  .00073094  13844-3  66816-4 0    8

2 88888  72.8435 115.9689 0086731  52.6988 110.5714 16.05824518  105

SGP8 TSINCE              X                Y                Z

0.            2328.87265015   -5995.21289063    1720.04884338

360.00000000    2456.04577637   -6071.90490722    1222.84086609

720.00000000    2567.68383789   -6112.40881348     713.29282379

1080.00000000    2663.49508667   -6115.18182373     194.62816810

1440.00000000    2743.29238892   -6078.90783691    -329.73434067

XDOT             YDOT             ZDOT

2.91210661      -0.98353850      -7.09081554

2.67936245      -0.44820847      -7.22888553

2.43992555       0.09893919      -7.32018769

2.19525236       0.65453661      -7.36308974

1.94680957       1.21500109      -7.35625595

\end{verbatim}

\newpage

\begin{verbatim}

1 11801U          80230.29629788  .01431103  00000-0  14311-1

2 11801  46.7916 230.4354 7318036  47.4722  10.4117  2.28537848

SDP8 TSINCE              X                Y                Z

0.            7469.47631836     415.99390792    5829.64318848

360.00000000   -3337.38992310   32351.39086914  -24658.63037109

720.00000000   14226.54333496   24236.08740234   -4856.19744873

1080.00000000  -10151.59838867   22223.69848633  -23392.39770508

1440.00000000    9420.08203125   33847.21875000  -15391.06469727

XDOT             YDOT             ZDOT

5.11402285       6.44403201      -0.18296110

-1.30200730      -1.15603013      -0.28164955

-0.33951668       2.65315416      -2.08114153

-1.00112480      -2.33532837       0.76987664

-1.11986055       0.85410149      -1.49506933

\end{verbatim}

\newpage

\section[Sample Implementation]{SAMPLE IMPLEMENTATION}

These FORTRAN IV routines have been implemented on a Honeywell-6000 series

computer.  This machine has a processing speed in the 1MIPS range and a 36 bit

floating point word providing 8 significant figures of accuracy in single

precision.  The information in the following table is provided to allow a

comparison of the relative size and speed of the different models\footnote[1]%

{The timing results are for the test cases in Section Thirteen with a one day

prediction.  Times may vary slightly with orbital characteristics and, for

deep-space satellites, with prediction interval.}.

\begin{verbatim}

core used     CPU time per call (milliseconds)

Model     (words)          Initialize       Continue

SGP          541               .8              2.7

SGP4       1,041              1.9              2.5

SDP4       3,095              5.1              3.6

SGP8       1,601              1.8              2.2

SDP8       3,149              5.4              3.2

\end{verbatim}

\newpage

\addcontentsline{toc}{section}{ACKNOWLEDGEMENTS}

\section*{ACKNOWLEDGEMENTS}

The authors would like to express our appreciation to Bob Morris for his

helpful comments, to Linda Crawford for supplying one of the function

subroutines, and to Don Larson for his help in validating the computer

programs.  Finally, we wish to acknowledge the skill and patience of Maria

Gallegos who accomplished the unenviable task of typing this report.

\newpage

\addcontentsline{toc}{section}{REFERENCES}

\begin{thebibliography}{99}

\bibitem{}

Brouwer, D., ``Solution of the Problem of Artificial Satellite Theory without

Drag'', {\em Astronomical Journal\/} {\bf 64}, 378--397, November 1959.

\bibitem{}

Hilton, C.G. and Kuhlman, J.R., ``Mathematical Models for the Space Defense

Center'', Philco-Ford Publication No. U-3871, 17--28, November 1966.

\bibitem{}

Hoots, F.R., ``A Short, Efficient Analytical Satellite Theory''.  {\em AIAA\/}

Paper No.\ 80-1659, August 1980.

\bibitem{}

Hoots, F.R., ``Theory of the Motion of an Artificial Earth Satellite'',

accepted for publication in {\em Celestial Mechanics}.

\bibitem{}

Hujsak, R.S., ``A Restricted Four Body Solution for Resonating Satellites with

an Oblate Earth'', {\em AIAA\/} Paper No.\ 79-136, June 1979.

\bibitem{}

Hujsak, R.S. and Hoots, F.R., ``Deep Space Perturbations Ephemeris

Generation'', Aerospace Defense Command Space Computational Center Program

Documentation, DCD 8, Section 3, 82--104, September 1977.

\bibitem{}

Kozai, Y., ``The Motion of a Close Earth Satellite'', {\em Astronomical

Journal\/} {\bf 64}, 367--377, November 1959.

\bibitem{}

Lane, M.H. and Cranford, K.H., ``An Improved Analytical Drag Theory for the

Artificial Satellite Problem'', {\em AIAA\/} Paper No.\ 69-925, August 1969.

\bibitem{}

Lane, M.H., Fitzpatrick, P.M., and Murphy, J.J., ``On the Representation of

Air Density in Satellite Deceleration Equations by Power Functions with

Integral Exponents'', Project Space Track Technical Report No.\

APGC-TDR-62-15, March 1962, Air Force Systems Command, Eglin AFB, FL.

\bibitem{}

Lane, M.H. and Hoots, F.R., ``General Perturbations Theories Derived from the

1965 Lane Drag Theory'', Project Space Track Report No.\ 2, December 1979,

Aerospace Defense Command, Peterson AFB, CO.

\end{thebibliography}

\end{document}

-- spacetrk.aux --

\relax

\@writefile{toc}{\string\contentsline\space {section}{ABSTRACT}{i}}

\@writefile{toc}{\string\contentsline\space {section}{CONTENTS}{ii}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {1}INTRODUCTION}{1}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {2}The Propagation Models}{1}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {3}Compatibility with NORAD Element Sets}{2}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {4}General Program Description}{2}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {5}The SGP Model}{3}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {6}The SGP4 Model}{10}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {7}The SDP4 Model}{21}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {8}The SGP8 Model}{31}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {9}The SDP8 Model}{46}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {10}The Deep-Space Subroutine}{57}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {11}Driver and Function Subroutines}{68}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {12}Users Guide, Constants, and Symbols}{75}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {13}Sample Test Cases}{78}}

\@writefile{toc}{\string\contentsline\space {section}{\string\numberline\space {14}Sample Implementation}{84}}

\@writefile{toc}{\string\contentsline\space {section}{ACKNOWLEDGEMENTS}{85}}

\bibcite{}{1}

\bibcite{}{2}

\bibcite{}{3}

\bibcite{}{4}

\bibcite{}{5}

\bibcite{}{6}

\bibcite{}{7}

\bibcite{}{8}

\bibcite{}{9}

\bibcite{}{10}

\@writefile{toc}{\string\contentsline\space {section}{REFERENCES}{86}}

-- spacetrk.toc --

\contentsline {section}{ABSTRACT}{i}

\contentsline {section}{CONTENTS}{ii}

\contentsline {section}{\numberline {1}INTRODUCTION}{1}

\contentsline {section}{\numberline {2}The Propagation Models}{1}

\contentsline {section}{\numberline {3}Compatibility with NORAD Element Sets}{2}

\contentsline {section}{\numberline {4}General Program Description}{2}

\contentsline {section}{\numberline {5}The SGP Model}{3}

\contentsline {section}{\numberline {6}The SGP4 Model}{10}

\contentsline {section}{\numberline {7}The SDP4 Model}{21}

\contentsline {section}{\numberline {8}The SGP8 Model}{31}

\contentsline {section}{\numberline {9}The SDP8 Model}{46}

\contentsline {section}{\numberline {10}The Deep-Space Subroutine}{57}

\contentsline {section}{\numberline {11}Driver and Function Subroutines}{68}

\contentsline {section}{\numberline {12}Users Guide, Constants, and Symbols}{75}

\contentsline {section}{\numberline {13}Sample Test Cases}{78}

\contentsline {section}{\numberline {14}Sample Implementation}{84}

\contentsline {section}{ACKNOWLEDGEMENTS}{85}

\contentsline {section}{REFERENCES}{86}