NOTES

for

Amateur Radio

"DX Gazetteer"

&

"DX Bearing & Distance Tables"

w2iol

06 April 1993

CONTENTS

1.  General Information on the ``DX Bearing'' & ``DX

Gazetteer'' Tables.................................

2.  Paper, Pictures and PCs............................

3.  Amateur Radio Prefix References....................

4.  DX QTH Latitude & Longitude Coordinates............

5.  Short Path Bearings, Returns & Distances...........

6.  Long Path Data from Short Path Data................

7.  VUCC Grid Locators.................................

8.  ITU Regions........................................

9.  Continents.........................................

10.  Amateur Radio Zones................................

11.  ``STATUS''.........................................

12.  DX Awards..........................................

13.  USA Third Party Traffic Treaties & Reciprocal

Licensing/Operating Agreements.....................

14.  Time & Time Again..................................

15.  HF Time & Frequency Standard Stations..............

16.  National Radio Quiet Zone (NRQZ)...................

17.  Northern California DX Foundation's (NCDXF) HF

Beacons............................................

18.  Suggestions and Errors.............................

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

1.  General Information on the ``DX Bearing'' & ``DX Gazetteer''

Tables

a.  The Amateur Radio DX Gazetteer includes latitude and longitude

coordinates, and status in DXCC, United Nations, ITU, etc.  This

is the version at ARRL as "dxcc-w2iol-a-p" and "dxcc-w2iol-q-z"

and "dx_wiol.dat" on the Ham CD-ROM.

b.  The DX Bearing Tables include Great Circle bearings, returns and

distances, time differences (DX - local), DXCC status, etc.

The Bearing tables can be produced for worthy causes at no cost.

c.  CONTENTS for BEARING & GAZETTEER TABLES

DATA SET

______________________

DXCC "Country", "City"

Antarctic Bases

Continental U.S.A.

NCDXF HF Beacons

d.  The Gazetteer and Bearing tables are available in prefix or

"country/city" order.  The prefix order is 0-9-A-Z.  The

"country/city" order is used for the gazetteer at ARRL and on

the CD-ROM.  For a handy prefix order list see K2DI's DXCC list.

e.  A special "Field Day" bearing version in prefix order that

covers the US and Canada by ARRL sections and divisions is also

available.

2.  Paper, Pictures and PCs

Someone once said that a picture is worth 30 dB words.  All the

bearing and distance information, accurate enough for almost all ham

radio purposes, can be found graphically very easily from an

azimuthal-equidistant global map centered on or anywhere near (by

many, many miles) the home QTH.  Such maps are much more enlightening

as to what is happening than tables, such as DX Bearing Tables.  They

also make a nice wall decoration and a good introductory discussion

piece, especially for non- (potential) hams.  The ARRL Operating

Manual has about 20 such maps that are centered on various parts of

the world.

Suppliers of personalized A-E maps advertise in the ham magazines.

The ARRL and the Callbook sell multi-colored versions centered on

Kansas, the center of the continental USA.  Other versions, centered

on major cities such as New York, are available from the US Government

Printing Office and elsewhere if you look around.

Great circle bearings from point A to point B do not take into

account other propagation paths such as the "Gray Line" or the

sunrise/sunset terminator path possible between some locations.

Computer DX aids such as "Gray Line Advantage" by MFJ, "The DX Edge"

by Xantek or Geoclock by Ralph Alghren, can show such paths (and start

conversations).

The latitude and longitude coordinates from these tables can be

used as input to any of several "Maximum Usable Frequency" (MUF)

programs to determine the best probable frequencies to attempt contact

between two points on earth for given solar flux and sunspot

conditions for a given day of the year and time of day.  See the ARRL

Amateur Radio Handbook and ARRL Antenna Handbook.

A number of personal computer programs combine bearings,

distance, MUF and gray line propagation calculations and the day/night

map display.  Thus, the value of this set of tables is not in

providing bearings, which can be obtained easily from the maps or

calculated using any one of dozens of available calculator and

computer programs, but in gathering a variety of DX data together in

one place.

3.  Amateur Radio Prefix References

For general references for amateur radio prefixes, see:  The ARRL

Operating Manual, "DXCC" By ARRL, Callbooks, "Worked-100-Nations" by

Worldradio, "DX Dynasty Award" by 73, and the Directory of Islands by

RSGB.  For the latest prefixes see the DX columns in the magazines

(QST, Ham Radio, CQ and 73) and the many DX newsletters, such as the

DX News-Sheet.  Geoff Watts, Norwich, England, publishes a number of

detailed reference guides including, "The ``DXNS'' Prefix-Country-Zone

List," "The ``DXNS'' DXCC Countries Guide," "The ``DXNS'' CQ & ITU

Zones Guide," and "The ``DXNS'' Soviet Oblast Guide."  To identify the

home country for new, strange prefixes that appear without notice, see

the ITU prefix allocation tables for each country in the ARRL

Operating Manual, the ARRL Amateur Radio Handbook, or in the

Callbooks, etc.

4.  DX QTH Latitude & Longitude Coordinates

Latitude & Longitude

- In Bearing tables, printed to 0.01 degree (truncated,

not rounded)

- In Gazetteer tables, printed to nearest minute

For references for latitude and longitude coordinates, see:  US

Department of the Interior Geological Survey (USGS) topological maps,

The National Atlas of the United States, Polar Regions Atlas by the

Central Intelligence Agency, and also various atlases by Rand McNally,

Prentice-Hall, The Times, and other publishers.  For a capsule

description of political and geographical points of interest, see

Chapmans World Gazetteer.  The Rand McNally College World Atlas, is a

pocket size paperback for 5 dollars.  Another compact reference is The

Portable World: A Complete Pocket World Atlas, (in the United Kingdom

sold under the name Philips Small World Atlas) by Willett, Gaylard and

Prince-Smith, published by Avon Books for 9 dollars.  These altases

all have an index of locations with latitude and longitude

coordinates.  Not all atlases list coordinates.  The USGS USA Place

Names data base is now available on IBM PC-compatible disks and CD-

ROMs.

Recommended Reading for DX'ers:  George J. Demko, former director

of the U. S. Office of the Geographer, with Jerome Agel and Eugene,

has a book, Why In The World: Adventures in Geography, (Anchor

Doubleday, 1992) that should be read by any DX'er.  In addition, Jon

Fisher has two little books called Uninhabited and Deserted Islands

and The Last Frontiers on Earth - Strange Places Where You Can Live

Free, (Loopanics Unlimited, 1983 & 1985).  These books describe the

location, geography, climate and general living conditions of many of

the obscure little islands and odd spots on Earth that we call "DX

countries."  There are very good reasons why many are normally

uninhabited by sane folks (read non-hams) and why DX-peditions are

necessary to put them on the air.

5.  Short Path Bearings, Returns & Distances

a.  "BRNG" = Great Circle Bearing from local QTH to DX QTH.

= Short Path Bearing (degrees)

(measured clockwise from geographic or true North)

b.  "RTRN" = Return = Great Circle "Reverse" or "Reciprocal" Bearing

from DX QTH to local QTH

= Short Path Return (degrees)

(measured clockwise from geographic or true North)

c.  "DSTNC" = Great Circle Distance for Short Path in

"km" = kilometers, "sm" = statute miles, or "nm" =

nautical miles.

6.  Long Path Data from Short Path Data

a.  (Long Path Angles) = (Short Path Angles) + or - (180 degrees)

c.  (Long Path Distance) ~#= (Earth Circumference) - (Short Path Distance),

Where Earth Circumference = Average = 40030 km = 24874 sm = 21614 nm

Earth          Earth

Radius     Circumference

__________|____________|_______________

|  6378.16 km|     40075.2 km

Equatorial|  3963.20 sm|     24901.5 sm

|  3443.92 nm|     21638.2 nm

__________|____________|_______________

|  6356.77 km|     39940.8 km

Polar     |  3949.91 sm|     24818.0 sm

|  3432.38 nm|     21566.3 nm

__________|____________|_______________

|  6371.02 km|     40030.3 km

Average   |  3958.77 sm|     24873.7 sm

|  3440.08 nm|     21614.7 nm

__________|____________|_______________

The bearing and distance calculations are for an idealized

spherical earth with the "average" circumference.  If the truer oblate

spherodial shape were used instead, the distances could change by

10-20 miles for the longer ranges (about 1 part in 300 depending on

distance and direction) and the bearing and return bearing, to the

nearest degree anyway, may not change at all.  Unless you are building

a big rhombic antenna, you will never know the difference.

7.  VUCC Grid Locators

"VUCC Grid"   VHF/UHF Century Club grid location in IARU "Maidenhead" format

by Folke Rosvall, SM5AGM (QST, W1XX, Jan & Oct 83)

Characters   Type         Area (Long. by Lat.)   Name

_____________________________________________________________

1, 2      letters           20d by 10d        "field"

uppercase

A-R, S*

3, 4      numbers            2d by  1d        "square"

0-9

5, 6      letters           5m by 2.5m        "sub-square"

lowercase|#-

a-x

* - Grid Locators define an area while latitude and longitude

define a point.  Unofficial W2IOL conventions (not approved by SM5AGM)

for Grid Locators treat the North and South geographic poles1 as small

"point" grids.  The South Pole "grid" is large enough to cover the

Amundsen-Scott base, KC4AAA.2 The North Pole "grid" would cover a

polar expedition camp site.

"Grid"      Pole      Notes

______________________________________________

AS00aa   North Pole   "S" unique to North Pole

AA00aa   South Pole   KC4AAA

|#- - Another unofficial W2IOL convention is the use of lowercase

letters for characters 5 and 6, as a means to better distinguish them

from the uppercase letters for characters 1 and 2.

__________

1. The north and south magnetic and geomagnetic poles are

in the data base.  They move around by many miles.

2. The geographic poles wiggle around a little, too.

Someone usually is assigned to move the real pole stuck

at the South Pole a few feet every so often.

__________

Examples:

Latitude      Longitude      Grid Locator

_________________________________________

40d 46.9m N   074d 41.4m W   =  FN20ps

40d 46.9m S   074d 41.4m E   =  ME79if

40d 46.9m S   105d 18.6m E   =  OE29pf

40d 46.9m N   105d 18.6m W   =  DN70is

Only the first four characters that are used for VUCC awards are

printed in the tables.  There are 32,400 distinct 2d by 1d grid

squares.  (If we counted the unofficial "point" grids for the North

and South poles, there would be 32,402.)

The ARRL World Locator Atlas by SM5AGM has maps showing all the

grid squares and also Basic language programs that can be used to

calculate the grid locator from latitude and longitude and vice versa,

as well as bearings and distances given grid locators.  See also:  The

ARRL Grid Locator (for US and Canada) map.  The "VHF Plus" column in

CQ magazine for December 1992 discusses the history of grid square

locator systems.

8.  ITU Regions

"ITU"       = International Telecommunications Union, Geneva

"ITU rgn"   = ITU "Frequency Allocation Regions"

ITU REGION   AREA (Brief Description)

________________________________________________________________________________

1        Europe, Africa, Iceland, "Middle East", Turkey, U.S.S.R., Mongolia

2        North America, South America, Greenland, North Pacific

3        Asia-not-in-1, Iran, Australia, South Pacific.

For more precise boundaries, see the ITU Region map in QST,

October 1988.

9.  Continents

The International Amateur Radio Union (IARU) "Worked All

Continents" (WAC) awards program uses the following definitions for

"continents."

CT   Continent

_________________________________________________________

Af   Africa

An   Antarctica (not used for WAC)

(Af, As, SA & Oc used for Antartica.)

As   Asia

Eu   Europe

NA   North America

SA   South America

Oc   "Oceania"

= Australia + (most of Indian and Pacific Oceans)

See Callbook maps for continent boundaries.  The Callbook has the

Egyptian Sinai peninsula in Africa, while the encyclopedias have it in

Asia.  Amazingly, the normal "civilian" atlases rarely show continent

boundaries.

10.  Amateur Radio Zones

"CQ"   "CQ" Zones (1 to 40)

Used for CQ Magazine "Worked-All-Zone" awards

"IT"   "ITU" Zones 1 to 75 defined by the ITU

= ITU "Geographical Broadcast Reception Zones"

+

"IARU/Callbook" Zones 76 to 90

for

ocean areas not specified by ITU|#-.

Used for the IARU "Radiosport Contest"

& for "World ITU Zones" awards by K6YK

|#- - These undefined-by-ITU  areas usually have no land area above

water.  One exception is Minami Torishima (JD) that is located in the

Pacific in Callbook-defined zone 90 and near-but-not-in ITU-defined

zone 45.  It is sometimes listed as ITU zone 45 and sometimes 90.  See

the ARRL Operating Manual and Callbook maps for "CQ" and "ITU" zone

boundaries.

11.  ``STATUS''

#

ABRV   RELATIONSHIP                                      Entities

_________________________________________________________________

UN     United Nations member*                                180

IT     ITU member*                                          165?

DX     DXCC by American Radio Relay League                   330

= "DX Century Club"

Is     "Island DX Award" (IDX) by                            169

Whidbey Island DX Club

73     "DX Dynasty Award" = DXDA by 73 Magazine              402

WR     "Worked-100-Nations" = W-100-N by Worldradio          226|#-

3P     USA 3rd Party Traffic Treaties                         43

RL     USA Reciprocal Licensing & Operating Agreements        71

* Almost all UN members are also ITU members, and vice versa.

Exceptions include:

Switzerland & Vatican City that are ITU members, but not UN.

Switzerland is an Observer at the UN.

Bhutan, Dominica, Namibia, Seychelles & Vanuatu are UN members,

but not ITU.

Radiosporting magazine's "DX Centurion Award" program uses the

DXCC entity list.

|#- The W-100-N total does not include Malyj Vysotskij (4J1) or

Rotuma Island (3D2) or many other amateur radio DX "countries."  The

W-100-Nation intention is to include only sovereign nations, with

their own independent governments.  Colonies, dependents, military or

research bases, and properties of the sovereign entities are counted

as the same as the home nation.  Thus, the W-100-N entity total is the

smallest of the worldwide DX awards.

The Whidbey Island DX Club's "Island DX Awards" program uses the

DXCC entities that are defined as islands by the National Geographic

Society.  The Directory of Islands by the Radio Society of Great

Britain (RSBG), that is used for the "Islands on the Air" (IOTA)

Award, lists 12 pages of islands for the world.

Most of the deleted DXCC entities, those that still have a

recognizable geographic location, appear in the tables under the

current valid entity.

During 1990, North and South Yemen joined, East and West Germany

re-united and East and West Berlin re-united.

In 1991, the Soviet Union (USSR) broke up into 15 separate

countries.  Ten of them have formed a loose "Commonwealth of

Independent States" (CIS).  Azerbaijan, Estonia, Georgia, Latvia and

Lithuania are going their own ways for now.  "CIS" has replaced "USSR"

in the tables.

In 1992, Yugoslavia disintegrated into four entities.  In January

1993, Czezhoslovakia peacefully broke into the Czech Republic and the

Slovak Republic.  Several other groups are trying to claim their own

territories within existing countries or lapping across two or more

nations.  Several of these potential new "countries" are listed in the

tables under the existing entity.

During the '70s and '80s the former Canadian Northwest

Territories (NWT) districts of McKenzie and Franklin were divided into

the districts of Baffin, Invuvik, Keewatin, Kitikmeot and Ft. Smith.

Most of the eastern part of NWT is now becoming the new Inuit (Eskimo)

territory of Nunavut with the capital at either Resolute or Iqaluit.

The western NWT may be given a new name, perhaps Denendeh.

The DXCC Advisory Council is continually reviewing petitions for

acceptance of new places for DXCC status.

12.  DX Awards

For information on:

AWARD    TITLE                   SPONSOR

___________________________________________________

DXCC     DX Century Club         ARRL

IOTA     Islands on the Air      RSGB, G3KMA

WAC      Worked All Continents   IARU

WAZ      Worked All Zones        CQ

WITUZ    World ITU Zones         K6YK

WPX      World Prefixes          CQ

"WUN"?   Worked UN(?)            Kansas Radio Club?

and many, many, many more amateur radio DX award programs see:  the

ARRL Operating Manual, The DX Awards Directory by K1BV, and the

"Awards" columns in 73, CQ, Worldradio, and many DX newsletters.

13.  USA Third Party Traffic Treaties & Reciprocal Licensing/Operating

Agreements

"3P"   "3PTT"   USA Third Party Traffic Treaties

"RL"   "RLOA"   USA Reciprocal Licensing / Operating Agreements

These agreements and treaties usually also include territories of

the given country.  For example:  British and French territories

follow the home country.  For Antarctic base stations, see individual

home countries listed for 3PTT or RLOA.

For Emergencies or Special Events, temporary agreements are

sometimes arranged with countries other than those listed.  Otherwise:

Third Party Traffic, even an informal greeting, is illegal.

For the United Kingdom, Third Party Traffic is legal only with

pre-arranged "GB" prefix stations.  "GB3" prefixes are excluded from

this agreement!  An example is GB8SD, ("Scottish Day") (Kilmarnock or

Glasgow, Scotland) arranged for message traffic to and from the annual

Bellcore Pioneers "Scottish Day at Bonnie Brae" benefit event for the

Bonnie Brae boy's home (Millington, NJ, USA).  Pitcairn Island (VR6)

follows an informal agreement for allowing handling of messages

dealing with health, safety and vital supplies.

See QST for June 1990 (page 58), and CQ for March 1990 (pages

90-92) and the Callbooks for more on 3rd Party Traffic Treaties and

Reciprocal Licensing/Operating Agreements and late changes.

14.  Time & Time Again

One part of daily planning for DXing is figuring out when the

hams of the world are likely to be on the air operating versus

sleeping or working.  The world daylight/nighttime maps by "Gray Line

Advantage," "The DX Edge" and "Geoclock" and a number of other dx

programs show this very clearly.  You can see where hams are working

noon-time DX on 20 meters or grayline at dawn or dusk, or sound asleep

in the middle of the night.  If you don't have either of these aids or

an equivalent, you figure it out with a quick mental calculation with

the time information in these tables.

World "time zones", used for everyday living, "clock time," are

defined as twenty-four one-hour-wide (15d) (usually) longitudinal

slices.  Clock time increases eastward of of the "prime" or

"Greenwich" meridian (0d E/W) that passes through the Greenwich

Observatory, Greenwich, England, and decreases westward.  "Ideal" one

hour zones are centered on a meridian of longitude that is a multiple

of 15d longitude.

Geographical and political boundaries don't match this ideal of

time zones centered on 15d intervals, so most time zone boundaries zig

and zag to reflect human needs and political realities.  Several time

sub-zones are offset x hours plus 30 minutes to better match local

sunrises and sunsets.  Liberia (-0045) and the Chatham Islands (+1245)

are offset by 15 (or 45) minutes.  The World Book Encyclopedia has one

of the clearest world time zone maps.  Saudia Arabia observes "Sun

Time" that goes by actual sunset and sunrise times (in the +0300 hour

time zone band).  Arctic and Antarctic polar regions, with months of

daylight and darkness at a time, usually observe GMT.  Even though it

spans 31/2 time zones, the People's Republic of China uses, or at

least is supposed to use, only the time zone (+0800) for Biejing.  For

the Chinese in the far west, this is equivalent to the people in

California trying to live on Boston time.

The time zone at Greenwich has been commonly called "Greenwich

Mean Time", or more often just "GMT."  (In military jargon, the term

"ZULU" time is also often used to refer to "GMT".)  Local standard

times are referred to as being "ahead" (+hhmm, east) of GMT, or

"behind" (-hhmm, west) of GMT.  The actual date changes at the

"International Date Line" that zigs and zags near 180d E/W longitude

where locations to the east of Greenwich are 12 hours ahead and

locations to the west of Greenwich are 12 hours behind.  When crossing

the Date Line, one "gains" a day going west or "loses" a day going

east.

"Universal Coordinated Time" or "UTC" (from the French,

"Universelle Tiempes Coordinate") is now the proper term for the

reference time at the prime/Greenwich Meridian.3 UTC is adjusted every

so often by a "leap second" to compensate for variations in the

earth's rotation.  Standard time stations, like WWV, follow these

adjustments.  There are other "standard times" that are used for

astronomy and other scientific purposes.

To make better use of available daylight as the sunrise and

sunset times shift through the year, many locations shift their clocks

one hour ahead of "Local Standard" time from mid-spring to mid-autumn

to "Daylight Savings Time" (DST) or "Summer Time".

The "DX Bearing" and "DX Gazetteer" tables use the following

conventions:

a.  "Greenwich Mean Time" (GMT)/(UTC)

"GMT"     = (DX QTH Standard Time) - GMT

"+hhmm"   = DX QTH Standard Time ahead (east) of GMT

by hh hours & mm minutes

"-hhmm"   = DX QTH Standard Time behind (west) of GMT

by hh hours & mm minutes

b.  Time Differences:

DX QTH Standard relative to Local (Home) QTH Standard

"TMDF"    = "Standard Time Difference"

= (DX QTH Standard Time) - (Local QTH Standard Time).

"+hhmm"   = DX QTH Standard Time ahead (east) of Local

"-hhmm"   = DX QTH Standard Time behind (west) of Local

c.  "DST" = Daylight Savings Time or Summer Time

"+N"   Entities in the northern hemisphere that

advance local standard time by one hour

between March/April and September/October.

"+S"   Entities in the southern hemisphere that

advance local standard time by one hour

between September/October and March/April.

__________

3. UTC is one of family of "universal times" that also

includes UT0, UT1 and UT2, that account for the

variations in the Earth's orbit around the sun and are

used for various scientific purposes.

__________

England, France, Spain and Mexico observe

"Summer"/"Savings" time year around.  Several of the European

ex-USSR countries have changed their time zone from +0300 to

+0200.

In the US, by state law, Arizona uses Mountain Standard

Time (MST, -07:00) from October to April and Pacific Daylight

Savings Time (PDST, -07:00) from April to October.  However,

Indian reservations in Arizona do observe normal Mountain DST.

So the towns of Page, Tsaile and Whiteriver may observe DST.  In

addition, Kentucky allows local option for DST.  The story on

how the US time zone and savings time system came about is told

in the article, "Standard and Daylight-saving Time," by Bartky

and Harrison in Scientific American, May 1979.

15.  HF Time & Frequency Standard Stations

Twenty-seven high frequency (2-30 mHz) time and frequency

standard stations are indicated in curly brackets ({}).  For examples:

{ WWV }, { WWVH }, { CHU }, { JJY }, ...

The "city" latitudes and longitudes used in the tables in these

cases are actually the "standard" station coordinates, not the "city"

center.  For example, standard station RWM (55d 48m N 38d 18m E) is

about 20 km east of downtown Moscow (Moskva), USSR (55d 45m N 37d 42m

E).

The radio based navigation systems, LORAN-C, Omega, Transit and

NAVSTAR/Global Positioning System (GPS), and the CIS (ex-Soviet)

GLONASS depend on maintaining extremely accurate time and frequency

and can be used as standards for most purposes.  See Reference Data

for Engineers:  Radio, Electronics, Computer, and Communications

published by H. W. Sams and Reference Manual for Telecommunications

Engineering by R. L. Freeman, Recommendations and Reports of the CCIR,

1986, Volume VII, "Standard Frequencies and Time Signals," and also

NIST (former NBS) Special Publication 432, "NIST Time and Frequency

Services."

16.  National Radio Quiet Zone (NRQZ)

a.  NRQZ Boundaries:

Parts of Virginia & West Virginia

Bounded by

37d 30m N to 39d 15m N in Latitude &

78d 30m W to 80d 30m W in Longitude.

b.

The purpose of the NRQZ is to protect:

NRAO   National Radio Astronomy Observatory

Green Bank, WV at 38d 26.4m N  079d 49.8m W

&

NRL    Naval Research Laboratory

Sugar Grove, WV at 38d 30.7m N  079d 18.6m W

For more information on the NRQZ, see "Happenings," pages 53-54,

QST, June 1981.

17.  Northern California DX Foundation's (NCDXF) HF Beacons

The Northern California DX Foundation (NCDXF) supports beacon

stations on 20 meters at 14.100 MHz at various locations around the

world.  Whether the sunspots are hot or not, these beacons serve to

indicate openings to various parts of the world, help calibrate your

"S" meter, and act as in-band time and frequency standards.  In early

1989, the USA FCC approved similar beacons for 21.150 and 28.200 mHz.

Other countries have not acted as of yet.

Schedule:  Each beacon occupies a one-minute time slot on a

rotating basis with the others.  For example, 4U1UN/B at the U.N.

headquarters in New York City is on the air at every "0" minute; i.e.

at the top of the hour, at 10 minutes after, at 20 minutes after, etc.

For now, there is no station for the nine minute slot.  (Other

stations may be off the air occasionally for maintenance, having been

stolen, etc).

Output sequence:  Each message begins and ends with a CW id at

100 watts.  In between, the beacon broadcasts a series of continuous

tones at 100, 10, 1, and 0.1 watts.

TIME   CALL      LOCATION

_________________________________

00    4U1UN/B   New York

01    W6WX/B    California

02    KH6O/B    Hawaii

03    JA2IGY    Japan

04    4X4TU/B   Israel

05    OH2B      Finland

06    CT3B      Madeira Island

07    ZS6DN/B   South Africa

08    LU4AA/B   Argentina

09    _____/B   [ None for now ]

Also, at minute 02, the W6WX beacon at Stanford University

broadcasts on 21.150 mMHz.  At minute 03, it transmits on 28.200 MHz

and it is then silent until its turn on 14.100 MHz at minute 11, etc.

The other beacons will eventually follow on 21.15 and 28.20 MHz.

Reports:  The DX Foundation is eager to get reception reports.

They've even worked up a way for individual stations to best evaluate

day-to-day propagation changes.  (It's based on assigning numbers to

the different power levels and adding up the total you can hear.)

For more information, contact:

Northern California DX Foundation

P.O. Box 2368

Stanford, CA 94309.

Put "Beacon Info" or some such on the envelope so it will immediately

be routed to the Beacon Manager without someone having to open your

letter to find out what it's about.

The foundation is more and more these days going by the name "The

DX Foundation" to avoid confusion with the Northern California DX

Club, which is an entirely separate group.  So, the formal name is

still NCDXF, but don't let it throw you if you hear the shorter

version.

Happy beaconing!

[These "Beacon" notes are courtesy of Patty Winter, N6BIS.]

See also:

ARRL Operating Manual,

"20 Meter Beacons Revisited" by W5AE, QST, December 1988.,

"Tuning the 20 Meter Beacon Network" by K4TWJ, CQ, March 1984, &

"Worldwide Beacon Net: ..." by W6ISQ & K6RU, QST, June 1983.

18.  Suggestions and Errors

Please refer suggestions and errors to W2IOL,

rcmcc@canis.bellcore.com.

06 April 1993