InterFlex Systems Design -- Superior Software for AEA and Kantronics TNCs

<<   Welcome to PkGOLD and KaGOLD  >>

This functional demo program can be used several times. You can try

multiple connects, Shell to DOS, Cut/Paste, and do many experiments

with the program. The program will run for 30 minutes at a time, which

is plenty of time to work through each of the following suggested

exercises.

SOUND DEMO

The program starts by introducing you to the "Sound Demo" which makes

HF "sounds" on your PC speaker, and explains what you are hearing.

To skip the sound demo, answer "N" (no) to the first question. You

will be given a chance to run the "live" demo after that.

Things to Try

Get the program going: PKDEMO or KADEMO (depending on which you have).

Go through the Sound Demo, or Quit the Sound Demo to enter the Live Demo.

Enter the right COM port and Baud rate. Then your callsign and name.

You'll do this each time you run the program, the release edition saves

this (and other settings) to disk and "comes up" much faster.

The program will establish a link to your TNC. It may take a minute

or so the first time you run the program, but once you've got things

working, the program "comes up" very quickly.

LESSON #1: Single Connect.

Push [F1] for the Help Index. Use the Cursor or the first few letters

of a help topic to "pick" the topic. Cross-reference items are highlighted

by the cursor. To backup from a cross-reference item, use [Alt-F1].

Read about Packet Mode, Packet Operating, Packet Parameters if you wish.

Now let's do a single connect:

1.  Type in just a callsign, e.g. WA4EGT or type in a path, such

as WA4EGT  or  WA4EGT VIA DANAPT if you need to digipeat.

Note: If you don't know who you can connect to, use the

Mheard function, [Alt-F2]. Notice that when you depress the

[Alt] key, the "keyline" below changes to show you what each

key does. [Alt-F2] is shown as MHEARD.

2.  Hit the [F7] key (connect key) when you've typed in a callsign.

Don't type C CALLSIGN, just CALLSIGN then [F7].

3.  After connecting, use [Alt-F7] to disconnect. Choose NORMAL

disconnect. Notice that if frames are still pending, the

disconnect is suspended until the frames are sent and

acknowledged.

If there is still time in the countdown sequence, hit [Alt-F7]

again, and select to "cancel the disconnect" request.  Then

you will have to do step #3 again to initiate another disconnect

request.

4.  Go to the NETWORK VIEW screen using [F2] if you are not there

already.

5.  Go to the PREVIOUS SESSIONS screen using [F3]. Notice that

the text of your previous session is there.

6.  Return to the connected sessions using the [F4] key.

LESSON #2: Multiple Connect

1.  Type in a callsign or path (callsign VIA ...) and hit [F7].

2.  Do step 1 at least once more, so that you have two (or more)

sessions going. Don't worry about being on a session screen when

you start another session. [F7] creates session windows on the fly

(unlike other programs with fixed parameters).

3.  Switch between sessions using the [F4] "NEXT" key.

4.  Switch to the NETWORK VIEW screen using the [F2] key. Notice

the full-screen.

5.  See a list of stations to which you are connected by depressing

the Ctrl key (by itself).

6.  Go to each session using the [F4] key. Enter the operator's name

using the [Alt-n] key sequence.  Do this for each session. Names

and QTH data is saved in a log file in the release version.

7.  Go back to the NETWORK VIEW screen using [F2].

8.  If text "comes in" from a session, and you are on another session

screen, or the Previous Sessions or Network View screen, you will

see a message "F4: <callsign>" in the bottom of the banner outline.

Push [F4] "next" to go to that session immediately.

9.  Go to each session using the [F4] key, and start typing a line

but do not hit the [Enter] key to send it quite yet.  Switch

to each session and put in a partial line.  Notice that each

session has it's own "text entry/command entry" line. You can

move the cursor using the "arrow" keys, overwrite or insert

text.  The program saves the cursor "state" for each session

as you switch from session to session.

10.  Initiate a disconnect on each session. When all are disconnected,

go to the previous sessions screen using [F3] and review the

sessions.

LESSON #3: NET/ROM Node

1.  Identify a node, or several nodes that you use and can hit

easily.  Determine the node ALIAS. There are a number of

advantages to using node ALIAS names, rather than callsigns,

such as:

a.  You can establish multiple connects to the ALIAS that

are not possible using the node callsign.

b.  When a routed node disappears from the nodelist, you

will be informed with "invalid callsign" if aliases are

used. If just the callsign is used, and the routed node has

been dropped or disappeared, the node attempting the

route will drop back to an ordinary connect attempt and

eventually retry out.  Any callsign used and not found

in the nodelist is assumed to be a direct connect from

the node, and not a routed connect. Using ALIASES assures

that if the node has been dropped from the routes, you

will be immediately informed.

2.  Assume the node is called  NRNDE.  You want to connect to W1AW

after connecting to NRNDE. Type:

NRNDE|W1AW

on the command/text entry line and hit [F7]. The program will

establish a connect to NRNDE-n and then issue a connect to

W1AW automatically.

3.  Establish a few connects using the NRNDE node. You will notice

a couple of things:

a.  Each connect goes to a different SSID of NRNDE. For example

you might see NRNDE-10, and NRNDE-13 etc. The program

assures that if you use the "or bar (|)" technology, that

multiple connects to the are established automatically.

b.  The connect progress is shown to you on the session screen.

c.  As you make progress, you will notice the callsign in the

status banner indicates the current connected station.

4.  If you have a two hop connect, like NRNDE and then to SECNDE,

you can type:

NRNDE|SECNDE|W1AW  [F7]

The program will establish a connect to NRNDE-n, then request

a connect to SECNDE, and finally ask for a connect to W1AW.

Note: BPQ nodes may require using a "port" number in the

connect request (except when specifying a routed node in its

internal node list).  If W1AW is on port 1 of a BPQ node in

the example above, you would type:

NRNDE|SECNDE|C 1 W1AW  [F7]

5.  If you need to Digipeat into the first node, and the digipeater

is N6WIK, try:

NRNDE VIA N6WIK|SECNDE|W1AW [F7]

LESSON #4: Quick Connects

1.   Using the connect paths from prior examples, you can now

setup a few "quick connects" or "point and shoot" connects.

Once in the program, hit [Alt-S] for setup. Then highlight

QUICK CONNECTS, then choose ADD.

2.   Enter the name of the remote station, hit [Enter], then

type in the path.  For example:

Name:   ARRL HdQrtrs

Path:   NRNDE VIA N6WIK|SECNDE|W1AW

3.   Do this for each callsign. Some "paths" may simply be the callsign

of a station that is nearby, and doesn't require any special

digipeating or node usage.

When done, use the [ESC] key to bring you back to the program

window area.

4.   Press the [F7] key with a blank entry on the command line. This

causes the program to present you with a "pick list" of your

quick connects.  Highlight one, and hit [Enter] to start the

connect process.  Do this for all of the entries in your Quick-

Connects list.

5.   Note: For emergency use, when "Tactical" callsigns are contemplated,

enter the tactical callsign under as the NAME, and the FCC call

in the PATH. Also enter the Tactical callsign as the "NAME" of

the station in your NAMES list (created by [Alt-n] or in the

setup area under Names).

LESSON #5: Changing Parameters

1.   Enter a parameter name, such as MYALIAS and hit [F10], the

command key.  The program will display the contents of the

parameter.

2.   Type a parameter and a value, such as MYALIAS FIRESTN followed

by [F10]. This will enter the parameter value.

3.   Type DISPLAY [F10] to see a list of parameter groups. Highlight

one and hit [ENTER].  You may now move the cursor to a parameter

and alter it's value.  Typing DISPLAY M [F10] jumps immediately

to the Monitor parameters.

4.   Hit [F10] by itself (no parameters on the command entry line).

You will see (at a minimum) STARTUP and SHUTDOWN.  These are

the names of special parameter files with the extension .TNC.

Highlight STARTUP, hit [Enter] and notice that all of the

startup parameters are loaded into your TNC.

5.   Use [Alt-S] to get to the setup menu. Select TNC files.

Highlight STARTUP and hit [Enter].  This is a .TNC file. You

can edit it (we suggest not editing it unless you know what

you wish to change).  Use [F1] help to display your options.

6.   In the setup area, under TNC files, you will notice <new> and

<current>.  Selecting <new> allows you to create a parameter

file with settings you may wish to try.  Experiment.

LESSON #6: Sending CQ

1.   On the Network View screen, hit [Alt-U] to enter the CQ/UNPROTO

mode.  Now anything you type on the screen followed by the

[Enter] key is sent out as "unproto" or unnumbered information

frames.

2.   Look at the value of your UNPROTO by typing UNPROTO [F10].

If it is not CQ, then set unproto to CQ by typing:

UNPROTO CQ [F10]

3.   Each time you leave the Network View screen, the CQ/Unproto

switch is turned off. This is done to avoid sending frames

that are by accident. If you stay on the Network View screen,

and are in the CQ mode, all text you type is sent out as

[UI] frames.  You might set your UNPROTO to CQ VIA (some call).

Then you can see your CQ frames.

LESSON #7: Changing Program Options

1.   Use [Alt-S] to bring up the setup menu. Select Setup from

the menu.  Explore the various menu areas.

2.   Look at the Alt-n messages. Notice that ?CALL and ?NAME are in one

or more messages. These are "macros" that are filled in by the

program with the current CALL and NAME of the connected station.

ALT-N messages are only active when connected or in [Alt-U] mode.

3.   Look through other settings. There are several.

LESSON #8: Maildrop Access

1.   Use [Alt-F8] to access your maildrop.

2.   Use the L)list, R)ead, K)ill etc. commands.

Leave the maildrop with "B" for Bye.

binary file transfer, conferencing etc.

LESSON #9: For Multimode TNC (PK232, KAM) owners

1.   Try the other modes. Use the enclosed "TNC" files if you

wish. Hit [F10] (with no parameters on the command line)

and highlight some mode, then hit ENTER.

2.   Or change modes by simply typing the mode name then [F10].

3.   The [ESC] key is the transmit key down and up key. In non-packet

modes, start typing. When you want to key down and begin

transmitting hit the ESC key. Keep typing away. When you've

finished typing what and want your system to unkey after sending

the text, hit ESC again. Your text will continue being transmitted

until the text is sent, at which time it will unkey.

To leave ARQ AMTOR, just type AMTOR [F10] or [Ctrl-F4]. To switch

modes, type in the mode name then [F10], or use the TNC files for

that mode by hitting [F10] and selecting a mode.tnc file.

4.   AMTOR ARQ: Put the other station's SELCAL on the text entry line

(e.g. WEGT) and push the [F7] or "connect" key. The TNC will begin

Phasing in ARQ.

To end an ARQ session use [Alt-F7] and select a "disconnect" mode.

5.   FEC: On the PK-232, get into AMTOR mode. On the KAM, use [FEC] mode.

Type in text, when you're ready to transmit FEC, just hit [ESC].

6.   BAUDOT: Type BAUDOT [F10]. Set the shift and baud rate.

LESSON #10: Why use a version of our GOLD software?

We believe that our PkGOLD and KaGOLD software is the best available.

We invite you to compare our products to ANY other program, including

"new" program from the hardware companies.

We were the first to offer binary file transfers, first to support the new

Maildrop features in the AEA TNCs, first to have software for the dsp-2232,

and first to offer conference mode, YAPP transfers, auto-recognition and

exchange of data, and more.

LAST LESSON: Ordering PkGOLD or KaGOLD

1.   Get your VISA or MASTERCARD in front of you.

2.   Call (714) 496-6639 and ask for a copy of the latest software

for your TNC

or

3.   Send us a check, money order, or your VISA or MasterCard

number to:

InterFlex Systems Design Corp.

Post Office Box 6418

Laguna Niguel, CA  92607-6418

Be sure to include your name, address, and callsign.

VISA or MasterCard orders, please include a telephone

number to call if any problems arise.

Note: Price: $79.95 + $5 shipping. CA residents add 7.75%.

Subject to change. Indicate whether you want PkGOLD Enhanced

or KaGOLD DualPort. Or buy both for $129.95 plus shipping and

sales tax (if in California).

Good luck. We appreciate your business, and look forward to providing

you the best software available.