THENET @EU       de:ON5ZS  17.12.91 04:59 101   3423 Bytes

Token Ring precautions

*** Bulletin-ID: 16C107DB0GE ***

911217/0431Z ON1BWP, 911217/0135Z ON6AR , 911216/1338Z ON1AEO

911216/1219z ON4UBO, 911216/1203z DB0IZ , 911216/1146z DB0GE

de ON5ZS @ DB0GE

Hello everybody,

Here follows a suggestion for wiring the token ring, because it really worths !

1) modification of the rc232 card (pc).

--------------------------------------

Among other components you will find a 1489, this is a LINE RECEIVER.

Pins 2, 5, 12, 9 are called "RESPONSE CONTROL", and on most of the serial

cards,  these connections are NOT used. Using correctly these connections

you can reduce the interferences on the RXData line. So, first add 4

capacitors of 330 pF between pins 2, 5, 12, 9 and ground.

2) Connection between computer and TNC.

---------------------------------------

If you use more wire in the same cable, or in a screened cable, the RXData

signal can be degraded by the TXData signal either by induction and/or capacity.

We suggest to use TWO coaxial cables, one for RXD and another for TXD.

3) Baudrate generator.

----------------------

All the UARTs (on the computer and on the TNCs) need a clock frequency of

16 times the baudrate. We suggest to use the SAME SIGNAM for all UARTs.

This has the following advantages:

- The baudrate are exactly the same for all the UARTs and this contribute

to reduce the error rate.

- You can change the token ring baud rate without ANY hardware modification.

You can also modify this while TNN is running without loosing any

token.

You will find this signal on pin 9 of the UART in your computer. But

you have to be carefull and you need to buffer this signal. This will

also protect your UART and recover the clock signal waveform.

-  modification on your computer (serial card) :

+------+           \    1/6 74HC04

|  P   | pin 9    |  \

|  C   o ---------|   o------/\/\/\/\----------> to TNC #1

|      |          |  /         100 ohms     (small coaxial cable)

|  u   |           /

|  a   |

|  r   |

|  t   |

|      |

+------+

-  modification on each TNC

1/6 74HC14

\

1K            |  \                   +-----+

clock in >------/\/\/\/-+-----|   o ------+          |  Z  |

(from computer          |     |  /        |          |  8  |

or preceding TNC)  33pF ===     /          |          |  0  |

|                 |   pin 27 |     |

|                 +----------o  s  |

---                |          |  i  |

|          |  o  |

/         |          +-----+

/  |        |

clock out <-----------------o   |--------+

(to next TNC)                \  |

\   (1/6 74HC14)

73 from Daniel (ON5ZS @ LX0PAC)