THENET @EU       de:ON5ZS  17.12.91 01:04 101  17075 Bytes

RS232_PC.C - Version II

*** Bulletin-ID: 16C106DB0GE ***

911217/0112Z ON1BWP, 911217/0011Z ON6AR , 911216/1331Z ON1AEO

911216/1208z ON4UBO, 911216/1157z DB0IZ , 911216/1143z DB0GE

de ON5ZS @ DB0GE

/************************************************************************/

/*									*/

/*									*/

/*    *****			  *****					*/

/*	*****			*****					*/

/*	  *****		      *****					*/

/*	    *****	    *****					*/

/*  ***************	  ***************				*/

/*  *****************	*****************				*/

/*  ***************	  ***************				*/

/*	    *****	    *****	    TheNetNode			*/

/*	  *****		      *****	    (PC)			*/

/*	*****			*****	    Public Domain		*/

/*    *****			  *****	    NORD><LINK			*/

/*									*/

/* This software ist public domain ONLY for non commercial use		*/

/*									*/

/* RS232_PC.C	-	Timer Routinen NUR fuer PC !			*/

/*			DIESES MODUL IST HARDWAREABHAENGIG !		*/

/*									*/

/************************************************************************/

#include "all.h"

#include <stdio.h>

#include <dos.h>

/*----------------------------------------------------------------------*/

/* Routinen zur Verwaltung der RS232 Schnittstelle			*/

/* 									*/

/* Die Schnittstelle wird im Interrupt betrieben, wovon das aufrufende	*/

/* Programm aber nichts merkt. Ausgaben erfolgen ueber einen Ringpuffer	*/

/* und Eingaben werden ueber einen separaten Buffer gepuffert. Status	*/

/* liefert daher immer den Buffer Status, nicht den aktuellen Status	*/

/* der Hardware. Bufferueberlauf wird gemeldet, sonst wird nichts	*/

/* unternommen.								*/

/*									*/

/* 13-02-90, DF2AU							*/

/*									*/

/* 07-08-90, DF2AU: 2 Stop Bits						*/

/* 29-09-90, DF2AU: nur RTS toggle					*/

/* 28-10-90, DF2AU: Fehler in reset_rs232() beseitigt			*/

/* 01-01-91, DF2AU: Adresse COM1 wird aus Bios geholt			*/

/*           DF2AU: ((inportb(uart +5) & 0x20) == 0x20)(tx interr.)     */

/* 04-11-91, ON5ZS: interrupt function for 8250/16450 AND 16550 +       */

/*           	    max_buf_rx and max_buf_tx			        */

/*                  modifications in rs232_in,rs232_out,                */

/*                      init_rs232,clear_rs232,reset_rs232              */

/* 01-12-91, ON5ZS: faster version , without test

/*----------------------------------------------------------------------*/

#define BUFFERSIZE	0x3000		/* Groesse jedes Buffers	*/

#define INT_NUMMER	4		/* Interrupt Leitung		*/

extern int      is_16550;               /* flag ON if uart is 16550      */

int 		uart;      		/* Basisadresse der Hardware	*/

static char 	in_buffer[BUFFERSIZE];	/* Zeichen von RS232 an Progr.	*/

static char 	out_buffer[BUFFERSIZE];	/* Zeichen von Progr. an RS232	*/

int     	in_buffer_in;		/* einschreiben in in_buffer	*/

int     	in_buffer_out;		/* auslesen aus in_buffer	*/

int     	out_buffer_in;		/* einschreiben in out_buffer	*/

int     	out_buffer_out;		/* auslesen aus out_buffer	*/

int             out_aktiv = 0;          /* = 0 after interrupt & 0 tx byte*/

void interrupt 	(*old_int)();		/* alter Interrupt Pointer	*/

/*----------------------------------------------------------------------*/

/* Interrupt Service by ON5ZS   					*/

/*----------------------------------------------------------------------*/

void interrupt rs232_int()

{

unsigned char  iir;       /* Interrupt Indentification Registre */

if (is_16550 == 0)

/************************************************************************

* <<<<<<<<<<<<<< interrupt function for 8250 and 16450 >>>>>>>>>>>>>>>> *

************************************************************************/

{while ((iir = inportb(uart +2)) != 1)        /* while interrup pending */

{switch (iir)

{case 0x04:

/**************************************************************

* interrupt type is - Receiver data available                *

* interrupt reset with : read rx buffer register             *

**************************************************************/

if ((inportb(uart +5) & 0x01) == 0x01)       /* is data ready ? */

{                             /* read data + clear interrupt */

in_buffer[in_buffer_in++] = inportb(uart);

if (in_buffer_in == BUFFERSIZE) in_buffer_in = 0;

}

break;

case 0x02:

/**************************************************************

* interrupt type is - Transmitter Holding Registre Empty     *

*   source int. is - Transmitter Holding Registre Empty      *

* interrupt reset with : reading iir if source               *

*                        or write in tx holding register     *

**************************************************************/

if (out_buffer_in != out_buffer_out)

{if ((inportb(uart +5) & 0x20) == 0x20)

{outportb(uart, out_buffer[out_buffer_out++]);

if (out_buffer_out == BUFFERSIZE) out_buffer_out = 0;

}

}

else

out_aktiv = 0;

break;

case 0x06:

/************************************************************

* interrupt type is - receiver line status                 *

*   source int is  - Overun error   or  - Parity erorr     *

*                  - framming error or  - Break interrupt  *

* interrupt reset with : reading the Line Status Register  *

************************************************************/

inportb(uart +5);                /* reset this interupt */

break;

case 0:

/**************************************************************

* interrupt type is - Modem stauts  (normal never activ)     *

*     source int is - CTS or DSR or Ring or DCD              *

* interrupt reset with : reading Modem Status Register       *

**************************************************************/

inportb(uart + 6);                          /* reset interrupt */

break;

}   /* end switch  */

}      /* end while   */

}         /*  end if     */

else

/*************************************************************************

* <<<<<<<<<<<<<<<<<<<<< interrupt function for 16550 >>>>>>>>>>>>>>>>>> *

************************************************************************/

{while ((iir = inportb(uart +2)) != 0xC1)    /* while interrup pending */

{switch (iir)

{case 0xC4:

case 0xCC:

/**************************************************************

* interrupt type is - Receiver data available (0xC4)  or     *

*                   - Caractere timeout   (0xCC)             *

* interrupt reset with : read rx buffer register  Drops      *

*                       below the trigger Level (0xC4)       *

*                     : read all the caracter in rx buffer   *

**************************************************************/

/* while data ready ? */

while((inportb(uart +5) & 0x01) == 0x01)

{in_buffer[in_buffer_in++] = inportb(uart);

if (in_buffer_in == BUFFERSIZE) in_buffer_in = 0;

}

break;

case 0xC2:

/**************************************************************

* interrupt type is - Transmitter Holding Registre Empty     *

*    source int. is - Transmitter Holding Registre Empty     *

* interrupt reset with : reading iir if source               *

*                        or write in tx holding register     *

**************************************************************/

if (out_buffer_in == out_buffer_out )

out_aktiv = 0;

else				 /* while FIFO not FULL */

{while( (out_buffer_in != out_buffer_out) &&

((inportb(uart +5) & 0x20) == 0x20)

)

{outportb(uart, out_buffer[out_buffer_out++]);

if (out_buffer_out == BUFFERSIZE) out_buffer_out = 0;

}

}

break;

case 0xC6:

/************************************************************

* interrupt type is - receiver line status                 *

*    source int is  - Overun error   or  - Parity erorr    *

*                   - framming error or  - Break interrupt *

* interrupt reset with : reading the Line Status Register  *

************************************************************/

inportb(uart +5);                /* clear this interupt */

break;

case 0xC0:

/************************************************************

* interrupt type is - Modem stauts  (normal never activ)   *

*    source int is - CTS or DSR or Ring or DCD             *

* interrupt reset with : reading Modem Status Register     *

************************************************************/

inportb(uart + 6);                         /* reset interrupt */

break;

}    /* end switch   */

}       /* end while    */

}          /* end else     */

outportb(0x20, 0x20);                 /* Int. Controller wieder frei */

}            /* end function */

/*----------------------------------------------------------------------*/

/* Resetten der Schnittstelle						*/

/* OHNE Baudrate setzen!						*/

/* Buffer initialisieren                           			*/

/* Reset auf TNCs geben							*/

/*----------------------------------------------------------------------*/

void clear_RS232()

{

disable();                    	/* Int vorsichtshalber abschalt.*/

if (is_16550 ==1)

{outportb(uart +2,0xC7 );       /* FIFO ON , reset rx and tx FIFO */

outportb(uart +2,0xC1);              /* trigger level = 14 bytes */

}

inportb(uart);                           /* Datenregister leeren */

in_buffer_in = in_buffer_out = out_buffer_in = out_buffer_out = 0;

out_aktiv = 0;

enable();			                       /* int wieder an */

}

/*----------------------------------------------------------------------*/

/* Initialisieren der Schnittstelle					*/

/* OHNE Baudrate setzen!						*/

/* Buffer initialisieren, Interrupt Vektoren setzen			*/

/*----------------------------------------------------------------------*/

void init_rs232()

{

uart = * ((int far *) MK_FP(0x40, 0x00));

outportb(uart +2,0x01);                    /* enable FIFO mode (16550) */

if ((inportb(uart +2) & 0xC0) == 0xC0)         /* test if FIFO enabled */

{is_16550 = 1;

printf("uart is a 16550\n");

outportb(uart +2,0xC7);           /* FIFO ON, clear rx and tx FIFO */

outportb(uart +2,0xC1);           /* trigger level : 14 bytes      */

}

else

{is_16550 = 0;

printf("uart is a 8250/16450\n");

}

inportb(uart);	                         /* Datenregister leeren */

outportb(uart +3, 0x07);                                 /* DLAB auf 0 */

outportb(uart +1, 0x07);                     /* Interrupt: Rx, TX, RLS */

outportb(uart +4, 0x08);                       /* Interrupt frei (out2)*/

in_buffer_in = in_buffer_out = out_buffer_in = out_buffer_out = 0;

out_aktiv = 0;

disable();

old_int = getvect(8 + INT_NUMMER);

setvect((8 + INT_NUMMER), rs232_int);              /* Vektor setzen */

outportb(0x21, inportb(0x21) & (~(1 << INT_NUMMER)));

enable();			                      /* Interrupt frei */

}

/*----------------------------------------------------------------------*/

/* Schnittstelle sperren						*/

/*----------------------------------------------------------------------*/

void reset_rs232()

{

disable();

outportb(0x21, inportb(0x21) | (1 << INT_NUMMER));

outportb(uart +1, 0x00);                /* keine Interrupt Freigabe */

outportb(uart +4, 0x08);                   /* Out2 sperrt Interrupt */

if (is_16550 == 1) outportb(uart +2,0x00);              /* FIFO OFF */

setvect((8 + INT_NUMMER), old_int);

enable();

}

/*----------------------------------------------------------------------*/

/* RTS Leitung aktivieren						*/

/*----------------------------------------------------------------------*/

void rts_off()

{

outportb(uart +4, 0x0a);                /* Out2 an: Interrupt Freigabe */

}

/*----------------------------------------------------------------------*/

/* RTS Leitung desaktivieren						*/

/*----------------------------------------------------------------------*/

void rts_on()

{

outportb(uart +4, 0x08);                /* Out2 an: Interrupt Freigabe */

}

/*----------------------------------------------------------------------*/

/* Baudrate auf 9600 Baud, 8 Bit, keine Paritaet setzen, 2 Stop		*/

/*----------------------------------------------------------------------*/

void setbaud_9600()

{outportb(uart +3, 0x87);	/* auf Baudraten Register	*/

outportb(uart +0, 0x0C);	/* Teilerfaktor			*/

outportb(uart +1, 0x00);

outportb(uart +3, 0x07);	/* 8 Bit, keine Paritaet, 2 Stop*/

printf("Token ring speed 9600 baud\n");

}

/*----------------------------------------------------------------------*/

/* Baudrate auf 19200 Baud, 8 Bit, keine Paritaet setzen, 2 Stop	*/

/*----------------------------------------------------------------------*/

void setbaud_19200()

{outportb(uart +3, 0x87);	/* auf Baudraten Register	*/

outportb(uart +0, 0x06);	/* Teilerfaktor			*/

outportb(uart +1, 0x00);

outportb(uart +3, 0x07);	/* 8 Bit, keine Paritaet, 2 Stop*/

printf("Token ring speed 19200 baud\n");

}

/*----------------------------------------------------------------------*/

/* Baudrate auf 38400 Baud, 8 Bit, keine Paritaet setzen, 2 Stop        */

/*----------------------------------------------------------------------*/

void setbaud_38400()

{outportb((uart +3), 0x87);     /* auf Baudraten Register */

outportb((uart +0), 0x03);     /* Teilerfaktor           */

outportb((uart +1), 0x00);

outportb((uart +3), 0x07);     /* 8 Bit, keine Paritaet, 2 Stop*/

printf("Token ring speed 38400\n");

}

/*----------------------------------------------------------------------*/

/* Baudrate auf 57600 Baud, 8 Bit, keine Paritaet setzen, 2 Stop        */

/*----------------------------------------------------------------------*/

void setbaud_57600()

{outportb((uart +3), 0x87);     /* auf Baudraten Register       */

outportb((uart +0), 0x02);     /* LSB Teilerfaktor             */

outportb((uart +1), 0x00);     /* MSB                          */

outportb((uart +3), 0x07);     /* 8 Bit, keine Paritaet, 2 Stop*/

printf("Token ring speed 57600\n");

}

/*----------------------------------------------------------------------*/

/* Baudrate auf 115.200 Baud, 8 Bit, keine Paritaet setzen, 2 Stop      */

/*----------------------------------------------------------------------*/

void setbaud_115()

{outportb(uart +3, 0x87);     /* auf Baudraten Register       */

outportb(uart +0, 0x01);     /* LSB Teilerfaktor             */

outportb(uart +1, 0x00);     /* MSB                          */

outportb(uart +3, 0x07);     /* 8 Bit, keine Paritaet, 2 Stop*/

printf("Token ring speed 115200 baud\n");

}

/*----------------------------------------------------------------------*/

/* Zeichen aus Buffer an Programm liefern				*/

/*----------------------------------------------------------------------*/

char rs232_in()

{

char	c;

/* attend le 1 ier caractere */

while (in_buffer_in == in_buffer_out);    /* ggfs. warten auf Zeichen	*/

c = in_buffer[in_buffer_out++];

if (in_buffer_out == BUFFERSIZE) in_buffer_out = 0;

return(c);

}

/*----------------------------------------------------------------------*/

/* Zeichen an Schnittstelle geben, ggfs im Buffer speichern		*/

/*----------------------------------------------------------------------*/

void rs232_out(char c)

{

int temp;

int next_buffer_in ;

next_buffer_in = out_buffer_in +1 ;

if (next_buffer_in == BUFFERSIZE) next_buffer_in = 0;

disable();

if (out_aktiv == 0)

{enable();

out_aktiv = 1;

outportb(uart,c);

}

else

{ out_buffer[out_buffer_in] = c;

out_buffer_in = next_buffer_in;

enable();

}

}

/*----------------------------------------------------------------------*/

/* Status Eingabe							*/

/*----------------------------------------------------------------------*/

int rs232_in_status()

{ return(!(in_buffer_in == in_buffer_out));}

/*----------------------------------------------------------------------*/

/* Ende von RS232_PC.C */