THENET @ALL      de:DF2AU  31.01.92 13:19 991   3059 Bytes

TheNet and TheFirmware Bug Report

*** Bulletin-ID: 299118DK0MAV ***

920131/0128Z ON1BWP, 920131/0119Z ON6AR , 920128/1604Z PI8HWB

920125/1905Z PI8PAC, 911001/1100z PI8ZAA, 910930/1725z PI8UTR

910929/1814Z PI8AIR, 910929/1749z DK0MWX, 910929/1731z DB0EAM

910929/1713z DB0KG , 910929/1707z DK0MAV

de DF2AU @ DK0MAV

Hi,

all known versions of TheNet and all versions of TheFirmware up to TF22D have

a nice bug. The status of DCD is sometimes read incorrect. This may cause the

TNC to start transmitting although DCD is active or to refrain from

transmitting on a free channel. The error will occur if the DCD pin changes

polarity while the interrupt caused by the previous edge has not been serviced.

The next "slow" edge of DCD will give the correct status again. So that bug

is not lethal but just nasty. Some modems are that slow that it will

never happen.

The bug is sitting in the Level-1 code. This code is common to all

Z80-SIO versions, including the TNCu2e (HK21).

This is the incorrect version:

SIBESC:   push  AF              ; Register sichern

push  BC

push  DE

push  HL

push  IX

in    A,(SIBCTL)      ; Status SIO Kanal B (RS232) holen

ld    B,A             ; und merken

ld    A,10H           ; Reset External/Status Interrupts

out   (SIBCTL),A

ld    HL,SIBRR0

ld    A,(HL)          ; alter Status

ld    (HL),B          ; neuen Status merken

xor   B

ld    C,A             ; C = was hat sich geaendert

bit   3,C             ; Abfrage DCD-Wechsel-Interrupt

jr    Z,SIBE1

And this is what it should look like:

SIBESC:   push  AF              ; Register sichern

push  BC

push  DE

push  HL

push  IX

--->      ld    A,10H           ; Reset External/Status Interrupts

--->      out   (SIBCTL),A

--->      in    A,(SIBCTL)      ; Status SIO Kanal B (RS232) holen

--->      ld    B,A             ; und merken

ld    HL,SIBRR0

ld    A,(HL)          ; alter Status

ld    (HL),B          ; neuen Status merken

xor   B

ld    C,A             ; C = was hat sich geaendert

bit   3,C             ; Abfrage DCD-Wechsel-Interrupt

jr    Z,SIBE1

That's all. Reassemble and have fun.

73, georg, DF2AU @ DK0MAV