------------------------------------------------------------------------

From    : Pierre Cornelis, ON7PC    rue Ballings, 88    B-1140 Brussels

Phone : + 32 2 241 5805

To      : All

Date    : March 30, 1993

Subject : TheNetNode version 1.49 documentation

------------------------------------------------------------------------

************************************************************************

*                                                                      *

*    *****                       *****                                 *

*      *****                   *****                                   *

*        *****               *****                                     *

*          *****           *****                                       *

*  ***************       ***************                               *

*  *****************   *****************                               *

*  ***************       ***************                               *

*         *****            *****            TheNetNode                 *

*       *****                *****          (PC) + (ST)                *

*     *****                    *****        Public Domain              *

*   *****                        *****      NORD><LINK                 *

*                                                                      *

* This software is public domain ONLY for non commercial use           *

*                                                                      *

************************************************************************

*********************************************************************

*                                                                   *

*  This documentation is partially a translation of the original    *

*  documentation  "Benutzerhandbuch zu der Packet-Radio Knoten      *

*  Software TheNetNode (PC/ST) V1.50  from DF9GU, DL9HCJ, DB8AS,    *

*  DL8ZAW and DB2OS  and partially some tips to install it quickly. *

*                                                                   *

*  This documentation has been written by Pierre Cornelis ON7PC.    *

*                                                                   *

*  Finally this paper reflects also the modifications brought by    *

*  Daniel Nagels ON5ZS member of the SB><LINK (South Belgian Link). *

*                                                                   *

*  Some bugs were fixed, but the major improvements brought by      *

*  ON5ZS increase drastically the speed on the token ring and       *

*  reduce the token recoveries.                                     *

*                                                                   *

*  The nodes ON5PL and ON7RC  are running 57600 Bd token rings,     *

*  ON5ZS is running 76800 Bd, and  we expect higher baudrates !     *

*                                                                   *

*  The publication of this paper has been granted by Hans Georg     *

*  Giese DF2AU of the NORD><LINK group.                             *

*                                                                   *

*********************************************************************

1. INTRODUCTION :

=================

Foreword :

----------

TheNetNode has been developed by the NORD><LINK (DC4OX, DF2AU and

DL8ZAW). One version has been compiled for the IBM-PC, and another for

the Atari-ST .

For the IBM-PC version 1.22, please contact:

Hans Georg Giese, DF2AU

Hinter dem Berge 5, D-3300 BRAUNSCHWEIG

(or DF2AU @ DK0MAV)

or for the IBM-PC version 1.49 with DAMA, please contact:

Henning HAPPE, DG9FU

Sechsacker 20, D-3502 Vellmar

(or DG9FU @ DB0EAM)

or    Daniel NAGELS, ON5ZS

rue des Carrieres 2 , B-5241 WANZE

(or ON5ZS @ LX0PAC  or ON5ZS @ ON4DXL)

to get the original and latest files.

And for the Atari-ST version, please contact:

Stefan BRENTJES DK6PX,

Jahnstrasse 59, D-6580 Idar Oberstein

(or DK6PX @ DB0GE)

... do not forget a few DEM to support the shipping charges !

As TheNetNode 1.49 is distributed as a public domain software with the

sources, you may find various versions of it, and sometimes in diverging

ways, so that it would be a good idea to report problems, bugs and

improvements directly to one person : Henning HAPPE DG9FU, so that he

could distribute a new general release ... that's for the benefit of us

all !

TheNetNode 1.49  or TNN 1.49 is, as it's name suggest it, TheNet with all

known options and also something more ... We assume that you know how

TheNet works, and we will focus on the special topics.

The PC version works essentially as a L3/L4  node that runs on an PC

XT or AT or compatible. The complete TheNet protocol has been

transferred from the TNC into the computer who can performs the tasks

faster than the Z80 used in the TNC. This doesn't mean that you will not

need anymore the TNC. The TNC now controls the 'radio side' under a

special release of the KISS protocol that activates the PTT, sends and

receives data, and transfers them to the computer.

In order to transfer data, a 'token ring' is used. The computer sends a

so called 'token', this is a typical byte pattern, on the ring. All the

TNC are now wired in serie, and the token generated by the computer

reaches the first TNC in the ring. If the TNC received something from

the radio, it holds the token, it sends first the received data, and

then put the token at the end. Each TNC waits for the token before it

transmits it's data, so that each TNC is transparent for the data of the

preceding TNC. As soon as the TNC recognizes the token, it keeps the

token, inserts it's own data, and put then the token back at the end.

After some time, the data finally reach the computer where TheNet

protocol is performed.

The outgoing data are provided with a port address and transmitted after

the token and they are transmitted again in the ring. The TNC make a

test to recognize it's address, if it recognizes his port address, the

data are removed from the data block and brought over to the radio with

the suitable parameters.

On the serial port (token ring), transmission rates of 9.600, 19.200,

38.400,  57.600, 115.200  bit/sec are possible. As well as the software

is able to run with a UART 8250, it is highly recommended to replace

this chip with a 16550 for transmission rates higher than ..... bit/sec.

The 16550 has a buffer of 16 bytes, so that the chip can be programmed

to wait for 14 bytes before sending an IRQ, this means 14 times less

IRQs. The software recognizes the UART type (8250 or 16550)

automatically.

But with 3 full duplex links at 9600 bit/s, a simplex link at 2400

bit/s, a simplex link at 1200 bit/s one mailbox at 9600 bit/s, a user

access at 9600 bit/s and a user access at 9600 bit/s, the token ring may

reach the saturation point from time to time.

+-------------------------------------------------------------------+

|   NOTE FROM ON7PC : For small nodes ( 1 user access, 1 server,    |

|   and 2 network links) you may use a 286 at 16 MHz, but for big   |

|   nodes with a lot of traffic (2 users accesses and 4 network     |

|  links) we suggest you to use a 386 at 40 MHz.                    |

+-------------------------------------------------------------------+

This will be enough as general introduction.

The advantage of the ring is that each TNC does send ONLY to ONE other,

or looks from the opposite way, each TNC receives ONLY data from ONE

other. This means that the level and collision problems that appeared in

the diode matrix are here totally eliminated.

Comparing with a system with more TheNet TNCs connected via a diode

matrix or a polling system, TheNetNode offers the advantage that the

whole node appears now only with an UNIQUE callsign and SSID from,

whatever access is been used. This provide a better presentation in the

neighbors node tables. In TheNetNode there is also only one node table

for the whole node. Finally, the  routing inside TheNetNode  goes

directly from the input port to the output port (instead of going by hop

from one TheNet TNC to the other).

But there are not only advantages ... If a TNC fails the whole ring is

paralyzed if we do not take some precaution. This is also why a special

KISS software was developed.

On the distribution diskette you will find KISS17.EPR. This is the EPROM

dump for the TNC's. After the text "KISS - modifiziert fuer Token-Ring -

V 1.7 (DL8ZAW 11.05.91)" , you will find :

at address $5C the KISS port number (between $00 and $7F),

Program a different number for each TNC. Kiss port

number $00 is intended to be the general user access.

Then program other eproms with $01, $02, $03, etc ....

at address $5D the TX-delay in 10 msec ($00 to $FF),

at address $5E the P-Persistence value ($00 to $FF),

at address $5F the slottime ($00 to $FF),

at address $60 the TX-Tail ($00 to $FF),

at address $61 the Full duplex (00 = simplex, 01 = duplex).

at address $62 the PTT wait time (lower byte)

at address $63 the PTT waiting time (higher byte)

This is only useful if FULLDUPLEX is active. At the end of

the transmission, this timer starts. If there is a new

packet to be sent, it will be sent immediately (TX

delay =  0) , this will save a lot of synchronization

time . If you want to use this function, you have to

connect a 22 microfarad capacitor at pin 11 of SIO

(RTS-NOT).

Example: Address $62 = $50 and address $63 = $46 : this

gives a PTT waiting time of $4650 = 18000 ms = 3 Min.

So, the transmitter will stay on, and if there is no

traffic for a more than 3 minutes, it will go off the

air.

at address $64 the DAMA mode (00= off , 01 = on)

Please take care that your TNC2 has always a 32 kbyte RAM and that you

use a clock frequency > 6 MHz if the baud rate is 9600 bit/sec

fullduplex.

Watchdog : version with MAX691 :

As long as the TNC is working properly there are transitions between +10

V to -10 V on pin 5 of the RS232 connector. These transitions will be

detected by a MAX691. If the TNC fails, the transition will disappear

and a relay will by pass the defective TNC. Pin 8 of the RS232 is

connected to the reset transistor of the TNC and so the MAX691 can

attempt a TNC reset. If the TNC comes to be alive again, the relay will

be deactivated and the TNC will be connected. As the MAX691 allows only

0 to +5 V levels, a resistor, diode and zener diode are needed. Sysops

may receive the schematic of the watchdog circuit. Henning DF9GU has

also developed a "Ringplatine", please contact him.

Watchdog : version with 4060 :

Data to the TNC and to the computer are watched with two 4060 low cost

chips. These are 14 stage counters with oscillators. It will be wired so

that they are always counting. If the counter reaches a number so that

pin 3 (Q14) is high, a relay can be activated, that makes a reset on the

computer or remove the power supply from the TNCs. The counter is still

counting, and when pin 13 (Q9) is active, the 4060 is reset via pin 12.

On this pin 12, the RS232 is also connected, so that in normal

condition, the 4060 is always reset. This version of watchdog is cheaper

than the previous one, and works also if one TNC hangs while the

computer is still running. One disadvantage is that you have to go to

the node and fix the problem in case one of the TNC is really out of

order.

After a restart, the TNC puts a start message after the token, after

that he will receive all parameters from the computer. The occurrence

will also be registered and can be shown with the Statistic command.

The computer is also equipped with a similar watchdog circuit.

On the distribution diskette you will find

TNN149.EXE  the program

TNN149.CFG  the configuration file ,

TNN149.PAS  contains a password, callsign and identifiers

TNN149.STA  contains the information you got with the (S)tat command.

Parameter 19 gives the value of the saving interval for

this file. Parameter 19 is expressed in 10 minutes

intervals, and it is recommended to give the value "6" to

parameter 19, so that the Statistics are saved every

hour. Note that the process and the token ring are

suspended during saving on diskette !

When you boot the program, it will look for TNN149.CFG, TNN149.PAS and

TNN149.STA. If these files are not available, you will receive an error

message as

UART is a 16550

Version for PC UART Xtal 1.8432 MHz (standard value)

Length of Password : 80

WARNING : Fehler in TNN149.CFG ***

TheNetNode (PC) V1.49c (XX0XX)

Copyright by NORD><LINK (DC4OC/DB2OS/DF2AU/DL8ZAW/DL9HCJ/DF6LN)

This version is compiled for 8 KISS ports.

Convers mode by DK5SG.

login:

and the program will load the default parameters.

When you change parameters, they are automatically updated on the

diskette, so that you find the last correct parameters at each boot.

For the future we plan to include the version number in the files like

TNN149.CFG, TNN149. PAS, TNN149.STA, so that it will be possible to make

the difference between versions.

You can either control the program locally from the console or remotely.

In both case you need to establish your credentials as a sysop. From the

console the user will be request to type a log-in  and a password. As

default values the login "thenet" and the password "Geheim" are used. If

both are correct you will receive a message like

Welcome to TheNetNode (PC) V1.xx [DATE].

With ESC C you will be connected as HOST and you can use the whole

command set of TheNetNode or modify whatever parameter.

+-------------------------------------------------------------------+

|  Original version : default login    : thenet                     |

|                     default password : Geheim                     |

+-------------------------------------------------------------------+

From the radio side (remotely) you must first connect the node then type

"SY", you will then receive a text like "ON7PC de ON7PC (18:30)> 62 36

65 13 34". Now you have to reply these numbers with the correct

characters that you introduced with ESC P. You can ask this several

times and if just one of the attempts is correctly answered, the

authentication will be credited. This has been introduced as a "crazy

guard".

The default password is "1234567890123456...." . The SYSOP.PRO keeps a

logbook with the commands given by the sysop. If this file becomes too

long you can delete it with "DOS DEL path\SYSOP.PRO".

ESKAY (SP), TURBO-PR  and  Graphic Packet (GP) have special features who

reply automatically the password request, so that you never have to

count on your fingers to find out the password reply key !

TheNet was able to keep a 25 uplink and a 25 circuit table, now

TheNetNode is able to keep a 100 uplink and a 100 circuit table.

Starting with TNN release 1.45 some features have been enhanced : The

MH-List is saved as MH.TAB on current disk. This list is updated on each

packet from the user. Normally the current disk shall be the virtual

disk C:, and of course if you use a floppy disk you will get problems !

The reading of "U +" list needs a .TMP file. All .HLP and .TXT files

will be copied in a RAM disk.

2. INSTALLATION AND TIPS :

==========================

This chapter is not part of the original documentation, it has been

written by myself ON7PC, it will provide a  short "quick start"

information for TheNetNode on a IBM-PC (for the Atari it looks very

similar, but they may be some differences in the Operating System, etc

..).

1.  From experience an XT is just good enough for VERY SMALL nodes with a

maximum of 4 ports running at 1200 Bd.

For small nodes ( 1 user access, 1 server, and 2 network links) you

may use a 286 at 16 MHz.

But for big nodes with a lot of traffic (2 users accesses and 4

network links) we suggest you to use a 386 at 40 MHz.

You need also 1 floppy disks (A:),  but NO hard disk, NO video card,

NO monitor, NO keyboard (but remember to modify the SETUP with

"keyboard not installed" !), but at least 2 Mb of RAM.

2.  Format and copy the operating system on a new diskette ("FORMAT A:

/S"). Prepare a few other diskettes for the backups !

3.  The root should contain :

DOS          <DIR>

TNN          <DIR>

AUTOEXEC BAT            # see paragraph 5

COMMAND  COM

CONFIG   SYS            # see paragraph 4

CMD      TMP

COLBOOT  COM            # see paragraph 5

WAITN    COM            # see paragraph 5

and of course the two hidden system files (IO.SYS and MSDOS.SYS)

(The # sign is set before the comments and should be removed in the

real file, of course !)

4. The CONFIG.SYS file : We are using a 386 computer with 4 Mbytes RAM,

running under MS-DOS 5.0, and we have 2 floppy drives a: and b: , no

hard disk.

For 286 machines :

DEVICE=A:\DOS\HIMEM.SYS

DOS=HIGH,UMB

BUFFERS=15

FILES=15

DEVICEHIGH=A:\DOS\RAMDRIVE.SYS 256 256 /e

For 386 and 486 machines :

DEVICE=A:\DOS\HIMEM.SYS

# Manages the programs to use the EXTENDED memory . Must be

the first "device" command !!

DOS=HIGH,UMB

# DOS is installed in upper memory area

DEVICE=A:\DOS\EMM386.EXE 2048 RAM

# simulates EXPANDED memory, with 2048 kbytes , and access to

both EXPANDED and EXTENDED (RAM).

BUFFERS=30

FILES=30

DEVICEHIGH=A:\DOS\RAMDRIVE.SYS 1024 256 100 /A

# creates a RAM disk of 1024 kbytes, in sectors of 256 bytes,

with a maximum of 100 entries, and in EXPANDED memory (/A).

this RAM disk will be called as "c:" drive.

The size of this RAM disk depend upon your requests: if you

want long help files you will need a large RAM disk !

Even for ON7RC who is a very verbosant node, only 160 k

bytes are used ...

LASTDRIVE C:

... please refer to your DOS manual.

5.  Here is the AUTOEXEC.BAT we use. Please remember that we have 2

floppy disks. The b: drive contains all CTEXT.*, *.TXT and *.HLP

files as well as my Norton Commander and some of my favorite

utilities . All the other files  are on drive a: under a  DOS

subdirectory and a TNN subdirectory. The a: disk is a system disk

and contains also CONFIG.SYS and AUTOEXEC.BAT. The virtual disk is

C:.

A:\DOS\RTS

# RTS.EXE is a small (optional) utility who toggle the RTS

pin in order to reset the watchdog circuit (see ON5ZS).

PATH a:\;a:\dos;a:\TNN;b:\nc;

A:\DOS\CHKDSK A: /F < A:\DOS\NO.BAT > C:\CHKDSK.RPT

# makes a report of the disk a:

A:\DOS\RTS

A:\DOS\KEYBFR               # your keyboard layout

VERIFY OFF

PROMPT $P$G

copy a:\command.com c:

set comspec=c:\command.com

mem     # shows the memory configuration and free space

a:\dos\begin a:\tnn\reset.log

# record each re-start in RESET.LOG (see ON5ZS)

a:\dos\rts

copy b:\tnn\ctext.* c:      # ... all the CTEXT files ...

a:\dos\rts

copy b:\tnn\*.txt c:        # ... all the *.TXT files ...

a:\dos\rts

copy b:\tnn\*.hlp c:        # ... all the *.HLP files ...

copy a:\tnn\reset.log c:

a:\dos\rts

c:

a:\tnn\tnn149                # ... the program itself ...

a:\dos\rts

a:\dos\end a:\tnn\reset.log

# record each exit from this way in RESET.LOG (see ON5ZS)

A:\DOS\WAITN 20

# just wait a few seconds ... and allows the sysop to type

a CTRL-C to interrupt the autoexec procedure.

A:\DOS\COLBOOT

# utility that reboots the system ... so if for any reason

the program ends, you will have try to reboot...

6.  The DOS subdirectory should contain only the few specific DOS files

needed:

BEGIN    EXE

DISKCOPY COM     # ... from MS-DOS 5.0

(needed for back-ups!)

CHKDSK   EXE     # ... from MS-DOS 5.0

END      EXE

HIMEM    SYS     # ... from MS-DOS 5.0

KEYBFR   COM

MEM      EXE     # ... from MS-DOS 5.0

NO       BAT     # answers "NO" when using CHKDSK

RAMDRIVE SYS     # ... from MS-DOS 5.0

RTS      EXE     # toggles RTS (written by ON5ZS)

EMM386   EXE     # ... from MS-DOS 5.0

CMD      TMP

7.  The TNN subdirectory should contain following files :

MHEARD   TAB      # file created by TNN

TNN149   CFG      # the configuration file

TNN149   EXE      # the program itself

TNN149   PAS      # password, callsign and identifiers

TNN149   STA      # information you got with (S)tat command.

two log files :

RESET    LOG      # contains date/time of resets

SYSOP    PRO      # logbook of sysop commands

and :

AKTUELL  TXT      # contains info about the current node

AX25V1   TXT      # warning for user attempting to use AX25

version 1

CTEXT    TXT      # connect-Text (only send if allowed see POrt

command)

CTEXT     n       # connect text for each port (has priority on

the CTEXT.TXT)

CVSHELP  TXT      # help information on conference mode,

HARDWARE TXT      # hardware information (reply to the HArdware

command),

HELP     TXT      # help file  (reply to the Help command)

INFO     TXT      # information file (reply to the Info command)

MAP      TXT      # map (reply for Map command).

POLLDISC TXT      # text sent when a non DAMA user has a too short

FRACK and that he already receives the number

of warnings  set by the Poll-UI-Count

parameter. The variable "%n" can be included

to inform the user of the last wrong FRACK

time used !

QUIT     TXT      # text sent after the Quit command.

SOFTWARE TXT      # software information (reply to the SOftware

command).

and finally a lot of help files that can be edited by the sysop.

There is basically one help text for each command, but more can

been added too. An index of all .HLP files appear with 'H INDEX' :

9600BAUD HLP

AKTUELL  HLP

BCNODES  HLP

BEACON   HLP

CONNECT  HLP

CONVERS  HLP

CQ       HLP

DAMA     HLP

HARDWARE HLP

INFO     HLP

LINKS    HLP

MAP      HLP

MHEARD   HLP

NODES    HLP

PARMS    HLP

PATH     HLP

PORT     HLP

QUIT     HLP

ROUTES   HLP

SOFTWARE HLP

STAT     HLP

TIME     HLP

USER     HLP

VERSION  HLP

The program can of course run without all those CTEXT.*, *.TXT or

*.HLP files but are used if you like a userfriendly and complete

system. So you can start your first try with just a few of these

files, and complete them later on, when your node is running

properly ....

To make your first steps easier, please delete TNN149.CFG  or rename

it as CFG.149 ... and delete TNN149.STA or rename it as STA.149.

+-------------------------------------------------------------------+

|   NOTE FROM ON7PC :                                               |

|   On some distribution floppies you may only find the TNN*.*      |

|   files on one subdirectory and the other files on a separate     |

|   subdirectory. This can make your editing work more easy !       |

|   But please remember to put all the files as described           |

|   hereabove in the TNN subdirectory.                              |

|   Upgrading as also been done by distribuing only the TNN*.*      |

|   files to the involved sysops ...                                |

+-------------------------------------------------------------------+

8.  Prepare a DB25 female connector and connect pin 2 (TD) and pin 3

(RD), plug it on the COM1 port of your PC, a  RS232 led tester will

show the activity and with a dual trace oscilloscope you will really

see the tokens going around ...

9.  The RS232 card in your computer is also very important ... As serial

card we recommend to start with a "conventional" serial card who use

a 8250 or a 14450 chip. Nowadays it can be hard to find these "old"

cards, but they work better than newer ones with VLSI chips offering

a lot of functions.

It is also possible to replace the 8250 or 16450 with a 16550 who

has two 16 bytes buffers. Daniel Nagels ON5ZS written a RS232_PC.C

that takes the benefit of the power of the 16550. This routine has

been adopted by NORD><LINK and used in this version 1.49. Using a

16550 makes it now possible to run the transmission speed of the

token ring  from 9600 up to 115200 Bd. Please read the notes from

ON5ZS.

Some cards support 2 serial ports, 1 printer port and 1 game I/O,

and they use a 16450 and a 16451 : the 16451 performs the serial

interface (COM1) and the parallel interface (LPT1). We suggest you

to keep the 16451 in place, and to replace the 16450 by a 16550.

Then modify the address and interrupts so that the 16451 will be

used for COM2 (address $2F8 and IRQ 3 - not used in TNN) and that

the the 16550  will be used  as COM1(address $3F8 and IRQ 4). This

can usually NOT be done with the provided jumpers, and you will have

to solder 4 small pieces of wire directly on the printed circuit

board in order to get the correct addresses and IRQ decoding (this is

what I call the 'surgery work' and fortunately you shall not have to

cut the tracks today !). If you succeed you will also get your LPT1

(address $378 and IRQ 7) ready for IO remote control (see further in

this documentation) !

For computers using 8250 or 16450 chips we DO NOT recommend to use

speeds above 38400 bit/sec !!

Now boot your system ....

10. You should now receive on your screen:

uart is 16550A

*** CONFIGFILE - Open-Error ***

WARNING: Fehler in TNN149.CFG ***

*** TNN149.STA - Open-Error ***

*** CONFIG.NET - Open-Error ***

WARNING: Fehler in TNN149.STA ***

MESSAGE : Tokenring running at 19200 bits/s

TheNetNode(PC) V1.49a (XX0XX)

Copyright by NORD><LINK (DC4OX/DB2OS/DF2AU/DL8ZAW/DL9HCJ/DF6LN)

This version is compiled for 8 KISS ports

Convers-Mode by DK5SG

login:

OK fine ! TNN recognized the UART chip as well as the speed. And as we

deleted the TNN149.CFG and TNN149.STA files from  the diskette (A:)

it could of course  not be copied on the virtual disk (C:) and that

is the reason of the two warning messages. We will now correct that.

But first answer to the login with 'thenet <CR>' (all lower case

letters !), you will receive

password:

Answer with 'Geheim <CR>' (the first letter is a higher case !)

+-------------------------------------------------------------------+

|  Original version : default login    : thenet                     |

|                     default password : Geheim                     |

+-------------------------------------------------------------------+

You will receive:

Welcome to TheNetNode (PC), Version 1.49a, (May 17 1992, 21:03:13)

Type <ESC> I0 <the_node_call>   (e.g. <ESC> I0 ON7RC), to set the

callsign, and you can also modify the alias (<ESC> I1), the login

(<ESC> I2) and the password (<ESC> I3), the converse identifier or

callsign (<ESC> I4)  and the special identifier to not receive the

CTEXT (<ESC> I5). Type now <ESC> C , and you will see:

CONNECTED to ON7RC

ON7RC de ON7RC (08:55)>

Type H to get the HELP menu (if HELP.TXT exists) , try and experience

all the commands you want.

From this point we will please ask you to refer to the original

documentation "Benutzerhandbuch zu der Packet-Radio Knoten  Software

TheNetNode (PC/ST) V1.50"  from DF9GU, DL9HCJ, DB8AS, DL8ZAW and

DB2OS ... or to the other chapters in this paper.

To exit, type 'Q', the node will respond with

DISCONNECTED fm ON7RC

you are back in the host mode, type <ESC> QUIT (not documented in the

original paper !) to exit TheNetNode.

All parameters will now be saved in the TNN149.CFG, TNN149.PAS and

TNN149.STA even if these file did not exist before. Only TNN149.PAS

can be read as an ASCII file with Norton Commander. TNN149.CFG and

TNN149.STA are binary files.

The next time   you run TheNetNode all the parameters will be

automatically loaded from these file.

Now you can start to edit the *.HLP and *.TXT files as you want. Do

not use an editor ending the file with a 'CTRL-Z' character, we use

here the Norton Commander editor, and it works fine ! Don't matter,

once your node will be on the air, you will receive questions and

criticism from  the users about those help file, you will modify

them once again ....

11. Modify (i.e. with NORTON UTILITIES or PCTOOLS) the KISS dump :

Please first check the  version of the KISS eprom, the ONLY GOOD

eprom is the version 1.7, check the eprom dump and you must  find a

text like "KISS - modifiziert fuer Token-Ring - V 1.7 (DL8ZAW

11.05.91)".

As well as this is the ONLY GOOD eprom, there is still a bug ! All

versions of TNN are configured to use a token-ring with 2 stop bits

(modification from DF2AU since August 7, 1990). But all KISS eprom

(included this last 1.7 release) are with 1 stop bit. Now use Norton

Utilities and check for the following sequence 18h 04h 44h 02h 10h

...  that is a release for ONE stop bit, now replace this sequence

with  18h 04h 4Ch 02h 10h to make a TWO stop bit eprom ... In other

words replace the 44h with 4Ch ! Save this as your new "master  dump

with 2 stop bits" ... (thanks to ON5ZS !)

The parameter list start just after the closing bracket of the text

"KISS - modifiziert fuer Token-Ring - V 1.7 (DL8ZAW 11.05.91)" and

you have now to prepare eprom dumps for each TNC:

at address $5C the KISS port number (between $00 and $7F),

at address $5D the TX-delay in 10 msec ($00 to $FF),

at address $5E the P-Persistence value ($00 to $FF),

at address $5F the slottime ($00 to $FF),

at address $60 the TX-Tail ($00 to $FF),

at address $61 the Full duplex (00 = simplex, 01 = duplex).

at address $62 the PTT wait time (lower byte)

at address $63 the PTT waiting time (higher byte)

at address $64 the DAMA mode (00= off , 01 = on)

Note that the KISS port number, and the DAMA bit is very important,

other parameters as TXdelay, P-Persistence, slottime, etc ... are

used by default, for a few seconds, till TheNetNode has transferred

the parameters from TNN149.CFG to the TNC.

Program the necessary KISS eproms, insert them gently in the TNCs.

12. KISS eprom for full duplex :

There is a bug in KISS version 1.7 and you can not use it for full

duplex links (to local BBS, DXCluster, TCP/IP, ...). For this

purpose we recommend you to use the old KISS version 1.5.

The parameters are in the same sequence after the text "KISS -

modifiziert fuer Token-Ring - V 1.5 (DF2AU 23.12.90)", but there is

no PTT waiting time or DAMA mode !

13. Wiring the token-ring :

PC                                                  PC

COM1      TNC#1       TNC#2       TNC#3            COM1

RS232 pin nr  2       2   3       2   3       2   3               3

|       |   |       |   |       |   |               |

+--->---+   +--->---+   +--->---+   +--.. etc ... --+

For the very first tests, the watchdogs are not necessary !

Note that the TNC with the address "0" has a very specific function

... if your node receive a connect request to a destination that is

not in the node table and that is not in the link table, it will be

switched to the TNC with the KISS address "0" ... So this TNC must

be connected to the local access ( and to comply with the IARU

Region 1  Recommendations this should be on 70 cm ...). If you have

more user accesses, they will be attached to other port, but the

main 70 cm port shall be attached to port 0 ...

14. Parameters fine tuning :

Parameters may be changed either from the keyboard or remotely, in

this case you need to establish your credentials  with the 'SYSOP'

command. The default password or remote authentication is

'12345678901234 etc     ...' so just respond with the 'units' of the

numbers ...

To give the value 51 to parameter 2, just type 'T 2 51'.

Modification to parameters relative to each port is done the same

way, the second  column of the parameter table (that you get after a

'P' command) gives the starting parameter number, e.g. to give the

port 2 , MaxFrame parameter the value 6 type 'T 47 6'.

Experiencing nodes from a while, ON5ZS and myself recommend you to

first try the following parameter setting and then if necessary to

try out yourself :

For the level 1 parameters :

user link     network    interlink   wired link

link       9600 Bd   to BBS, TCPIP

---------------------------------------------------------------

TXDelay        25 ($1E)     25 ($1E)      2 ($02)      2 ($02)

PPersist       92 ($5C)    127 ($7F)    255 ($FA)    255 ($FA)

Slot time      10 ($0A)      5 ($05)      3 ($03)      5 ($05)

TXTail          3 ($03)      3 ($03)      3 ($03)      3 ($03)

These parameters may, but must not necessary, be burned into the

KISS eprom !

For the level 2 parameters :

user link    network   interlink   wired link

link        9600 Bd   to BBS, TCPIP

---------------------------------------------------------------

MaxFr           2            3            6            3

L2Retry        10           10           10            5

T2-Timer      219           54           54           13

For the level 3 parameters :

06: Lifetime    20 to 30

07: T-timeout        120

08: T-Retry       2 or 3

09: T-AckDelay    2 or 3

10: T-BsyDelay        60

11: T-Window          18

12: NoAckBuf          35

13: T3-Timer       32000

On the 'User access' and on the ports connected to the BBS, to the

DX-Cluster or other servers,  we highly recommend to NOT BROADCAST

the routing table (see the BCNODES command).

15. In our country (Belgium) we are faced with a well known language

problem ... In order to have the same structure as the original one,

and to allow the user to get the help message in his own language,

we translated the help files and we renamed them. For example the

original file in English CONNECT.HLP has been translated in French

under CONNECTF.HLP. The last letter is a "language identifier" and

we propose F" for  French, "N" for  Dutch ("Nederlands") and so on

... If the user forgets  the identifier he will receive the help

file in English ! Of course file names are limited to 8 characters,

and we should have SOFTWARE.HELP (in English) and SOFTWARF.HELP (in

French). Using the  "HELP INDEX" command we find all these file

names ! This however increase the size needed on the virtual disk

...

16. Watchdog information :

Assuming the token ring is at 9600 Bd, on pin 4 (RTS) of the COM1

port of the PC there will be practically a square wave of about 110

ms period and of an amplitude of +12V to -12V. This signal is halted

by the console routines (screen or keyboard activation).

On pin 5 of the RS232 ports of the TNCs there will be practically a

square wave of 20 ms period and of an amplitude of +10V to -10V.

17. Next bootings :

Now if all of the above is correct, you can start your node. Your node

will be active, even if you do not give the login ("thenet") and the

password ("Geheim"), but if you want to use the console (keyboard

and screen) of your node to make connections, to modify parameters

and files, then you must identify yourself as sysop by giving the

login and the password.

18. In case of problems :

Monitoring the token and the frames in the token-ring (TD and RD of

your PC) can help you to fix the problems. Try one TNC after the

other and exclude suspected TNCs with a DB25 connector with pin 2

and 3 short circuited.

19. Problem with DCD :

If for any reason (squelch of your tcvr too low, continuous carrier

on the RF channel, ...) the DCD in ON, the token ring will still

work for a while. But as information transferred by the token ring

to the TNC, this information is saved in the memory (RAM), as DCD in

true, the tcvr will not transmit, and the frames will stay in the

memory. After some time the memory can get full. Then there will be

even not enough  memory just to buffer the frames flowing in the

token-ring ... then the system will hang ...

If you see a token going out from the computer, but no token going

out of the ring this may be one of the reasons ....

20. How to upload the next version ?

If your node is at some distance, you may want to do this remotely,

you can do that in a very easy way, but be careful otherwise you

would have to go in a hurry to you node to reboot the previous

version. Keep always a copy of the previous version on a floppy disk

in a safe place !

You can upload either on the virtual disk (faster and recommended but

you will need to copy the uploaded file from the virtual disk to the

floppy disk) or on the floppy disk (slower, but no need to copy from

the virtual disk to the floppy) but in either case, you will need as

much free space as the length of the file you want to upload.

Assuming you didn't give any other command, you will be on drive c:

(the virtual disk),

a. first change to drive a: with DOS A:

b. be sure there is sufficient free space on a: with DOS DIR A:

c. use the command LOAD TNN149.NEW to upload the program,

d. if you are using ESKAY (or SP), type  <ESC> SB file_name (i.e.

<esc> SB C:\....\TNN149.EXE) to start the binary transfer and

follow the instructions. The YAPP protocol doesn't work !

e. But remember that a transfer of 200 k bytes (about the length of

TNN149.EXE) on a good link at 1200 Bd requires about 45 minutes

... so keep coooool ! . After that time the new file will then be

on drive A: (the floppy disk) under the name TNN149.NEW.

f. use the command DOS RENAME A:\TNN\TNN149.EXE TNN149.OLD to keep a

back up of the old version,

g. use the command DOS RENAME A:\TNN\TNN149.NEW TNN149.EXE

h. cross your fingers ... (hi)

i. and finally restart the node with RESET SYSTEM

21. The bottleneck :

As fast as you can go there is always a bottleneck ...

A 386 motherboard running at 25, 33 or even 40 MHz is affordable,

and we told you how to get the most out of your computer with the

serial card modification.

As from experience the bottleneck seems to be in the TNC ! If you

really want to get traffic on your node, and want to use 9600 Bd

backbones, you need to start with high speed TNCs.  Modifications

have been described in your BBS.

Mostly you will need to :

- replace the 4.9152 MHz Xtal with a 9.8304 MHz,

- replace the slow Z80 CPU with one who goes up to 10 MHz one

(this will appear in the complete type number of the chip) ,

- replace the slow Z80 SIO-0 with a 8 MHz one,

- use a fast RAM currently you will find chips having access time

of 100 ns or even 80 ns (both will do it ), and,

- use an EPROM with an access time of 150 ns or faster.

Still faster ... use 10 MHz Z80 CPUs, 8 MHz Z80 SIOs, 80ns RAMs and

120ns EPROM, we (ON5ZS and myself ON7PC) do run TNC from Eisch

Electronic at a clock frequency of 15 MHz !!

Remember that temperature DECREASE the speed performance of

components!

We have a good experience with the TNC-2 from EISCH Electronic ,

Abt-Ulrich-Str 16 , D-7900 ULM GOEGGLINGEN, but even with those TNCs

there are some improvements you can do easily (decouple the RAM with

a 0.1 microfarad place an additional  SHORT wire between the grounds

of CPU, SIO, EPROM, ...).

Other TheNetNode sysop reported good experience with the TNC2-C CMOS

from LANDOLT Electronic, Wingerstrasse 114, D-6457 MAINTAL.

And we are looking for faster TNCs ...

22. Testing set up : Of course you will be interested to test TNN

current version or updates at home, or to check your TNC and eproms

before that they will be installed in the node, ... here is the

configuration I use at home for that purpose. I have a 486 DX

computer running at 33 MHz with 4 Mbytes RAM, two floppies and a

hard disk. My CONFIG.SYS look like :

DEVICE=c:\DOS\HIMEM.SYS

DOS=HIGH, UMB

BUFFERS=30

FILES=30

DEVICEhigh=C:\DOS\ANSI.SYS

DEVICEhigh=C:\DOS\EMM386.EXE 2048 RAM

DEVICEhigh=C:\DOS\RAMDRIVE.SYS 360 512 /e

DEVICEhigh=C:\DOS\EGA.SYS

DEVICEhigh=MOUSE.SYS

I do not start TNN with AUTOEXEC.BAT but rather with TNN149.BAT and

I do not  use all the *.HLP and *.TXT files that I use for my node

ON7RC. Drive d: is the virtual disk , note that the smaller size of

the virtual disk. All NORD><LINK stuff goes into a THENET

subdirectory and the TNN version 1.49 files are not on

C:\THENET\TNN149\*.*   So here is what it looks like :

COPY C:\COMMAND.COM D:

SET COMSPEC=C:\COMMAND.COM

MEM

COPY C:\THENET\TNN149\CTEXT.* D:

COPY C:\THENET\TNN149\*.TXT D:

COPY C:\THENET\TNN149\*.HLP D:

D:

C:\THENET\TNN149\TNN149

CD C:\

3. SYSOP COMMANDS :

===================

Following commands can be performed at the keyboard of the node or

remotely after establishing your credentials (see SYSOP command).

Notes :

a. the commands may be abbreviated to the capital letters as shown

below,

b. values in angle brackets <> must be replaced by their actual values,

c. values in square brackets [] are optional,

d. | separate various options, you have to choice ONE of them,

e. + means add entry to a list and - remove from a list,

f. 1 means enable and 0 disable.

AUTO <port_number>

This command, only available in version  TNN 1.49-on5zs released

after 25/12/92, computes automatically the IRTT and T2. First

introduce correctly the following parameters :

- TXDelay, Persist and Slottime, (see PARM command)

- DAMA or not, and  baud rate on the radio port (see POrt command)

- baud rate on the token ring (see <esc> T command)

and then type simply 'AUTO <port_number>' and the IRTT and T2 will

be adjusted.

BCnodes <+|-> <port> <call> <digipeater>

This command allows to send the broadcast to other NET/ROM, TheNet

or TheNetNode station, directly or via stations who do not use the

L3/L4 protocol as used in NET/ROM, TheNet and TheNetNode. The

BCNODES with the + sign allows to add the station with the specified

callsign  via the specified port and digipeater in the broadcast

list. The BCNODES command with the - sign removes the entry in the

broadcast list.

An entry of the form "BC + 11 NODES" causes a  broadcast on port 11

and addressed to the callsign NODES, and NODES is a key word

recognized by NET/ROM, TheNet and TheNetNode systems. Usually the

broadcast must only be done on the backbone links and not on the

user access links. The interval between  broadcasts is  set by the

parameter 5 ("Broadcast"). If you are monitoring the RF channel

corresponding to the port number,  using a terminal program as ESKAY

(SP),  you will see on that node a broadcast table looking like :

ON7RC>NODES ctl UI^ pid CF

NODES Broadcast de BRU:

DB0BM     Julich via ON5ZS       93

DB0DOZ    NH     via ON5ZS       93

DB0EAM    KS     via ON5ZS       82

DB0FBB    DTMD   via ON5ZS       82

DB0HAG    DOBOX  via ON5ZS       81

DB0II     MG     via ON5ZS      119

DB0KV     Kleve  via ON5ZS       93

DB0OVN    Kaarst via ON5ZS       93

DB0PKE    Kevela via ON5ZS       93

DB0QS     DinTCP via ON5ZS       82

DK0MWX    MGBox  via ON5ZS       93

ON7RC>NODES ctl UI^ pid CF

NODES Broadcast de BRU:

LX0PAC    ON-lx  via ON5ZS      150

ON1BWP    SAB    via ON1BWP      78

ON1KGX    KGX    via ON7AN      150

ON1KGX-5  CLRBBS via ON7AN      147

ON4DXL    DX-Lge via ON5ZS      129

ON4KVI-5  SATbbs via ON5ZS      129

ON5PL     CORN   via ON5ZS      189

ON5VL     LGE    via ON5ZS      162

ON5VL-5   LGEBBS via ON5ZS      127

ON5ZS     D0     via ON5ZS      220

ON7AN     CLR    via ON7AN      220

ON7RC>NODES ctl UI^ pid CF

NODES Broadcast de BRU:

ON7RC-5   BRUBBS via LOCAL      192

PI1GOR-6  GOR6   via PI1HWB     109

PI1HWB    Breda  via PI1HWB     140

PI1LIM    HRL    via ON5ZS      119

PI1NYM-5  NYMS   via PI1HWB      94

PI1NYM-6  NYME   via PI1HWB      89

PI8BRD    Breda  via PI1HWB     139

PI8EHV    EHV    via PI1HWB     120

PI8ESA    ESA70  via PI1HWB      94

PI8FLK    FLK    via PI1HWB      78

PI8GTB    NBwTCP via PI1HWB     101

ON7RC>NODES ctl UI^ pid CF

NODES Broadcast de BRU:

PI8HRL    RMNC   via ON5ZS      119

PI8HRL-8  Hrlbbs via ON5ZS      119

PI8HRL-9  NETROM via ON5ZS      119

PI8HVH    HVH    via PI1HWB      93

PI8NOS    HVS    via PI1HWB     108

PI8NYM    NYM430 via PI1HWB      89

PI8RMD    RMD    via PI1HWB      89

PI8RNI    DeBilt via PI1HWB      94

PI8VLI    Mburg  via PI1HWB      83

|       |          |           +--- best quality

|       |          +--- best quality neighbor

|       +--- ident (mnemonic), the # sign indicates hidden nodes

+--- destination node

BCnodes !

Starts the broadcast to be transmitted immediately to all the ports.

Perform this command each time you restart TNN after maintenance to

inform your partners that you are again active !

Beacon <port> = <text>

Set or change the beacon text for the particular port. The beacon

can be enabled or not with the parameter 18 (1=enabled 0 = disabled)

Beacon <port> <time> <tlm> <name>

Telemetry beacons can be send to enable to supervise the node.

<time> is the time interval in minutes (recommended value = 10 min)

TLM 0 = No telemetry,

1 = telemetry

2 = telemetry + link statistics

<name> is the name of the unnumbered info frame (recommended = QST)

For example a command  like  BE 1 10 2 QST will activate the beacon

on port 0 and you will see  the following unproto frames with the

telemetry as :

ON7RC>QST ctl UI^ pid F0

TheNetNode (PC) V1.49a-2 (BRU:ON7RC)

20.03.93 14:30:43 Up=2/19:32 Fmem=71792 Free=4363 Tok=91 R=151 L=23

C=18

with   - the date and time in the node,

- Fmem is the free memory in bytes

- Free is the number of free blocks in TNN's buffer

- TOK = 91  is the token/second

- R = 151 is the number of token recoveries

- L = 24 is the number of links

- C = 15 is the number of circuits

and the link statistics as :

ON7RC>LSTAT ctl UI^ pid F0

ON7AN:  24690996  L3rtt= 380 (390)  L3srtt= 940  L2srtt= 460  Baud=

151

ON7RC>LSTAT ctl UI^ pid F0

ON5ZS:  24690996  L3rtt= 880 (4460)  L3srtt= 1950  L2srtt= 580

Baud= 2047

ON7RC>LSTAT ctl UI^ pid F0

ON1BWP: 24690996  L3rtt= 4010 (33170)  L3srtt= 6650  L2srtt= 5650

Baud= 19

with  - a callsign as ON5ZS, that is the neighbor node

- 24690996 is the running time in 10 msec (that is 68

days, 14 hours, etc ...)

- L3rtt = 880 is the computed level 3 RTT

- (4460) is the previous level 3 RTT

- L3srtt = 1950 is the latest level 3 smoothed RTT

- L2srtt = 580 is the latest level 2 smoothed RTT

- Baud = 2047 is the average speed measured during one L3 RTT

interval

The BE command without argument give the current status, for

example:

BRU:ON7RC> Beacons:

0 10 1 QST

1 10 2 QST

Those information can be saved with the <ESC> MM function of ESKAY

for further analysis or statistics, just type <ESC> MM ON7RC>QST or

<ESC> MM ON7RC>LSTAT.

CLEAR

Clears the token ring.

CLEAR *

Clears all the statistics.

Note : If you had problems and fixed them, it can be interesting to

have a look on the behavior of the node. In order to clear only the

statistics of that link you can type 'STAT - <call>' and then 'STAT

+ <call>' .

CVshost <+|-> <port> <call> <digipeater>

Allow or disable a neighbor node to take part to the converse

mode.

DOS <command>

Only for the version running on a IBM PC or compatible.

Performs the internal DOS command, for example

DOS DIR

DOS d:

DOC CD d:\path

DOS DEL d:\path\filename.ext

DOS COPY d1:path1\filename1.ext1 d2:path2\filename2.ext2

DOS MD  d:path\dirname

DOS RD  d:path

DOS REN d:path\filename1.ext1 filename2.ext2

This command must be used very carefully, you can breakdown

completely  your node.

EDIT <text>

Allows to edit text files.

ERASE <text>

Erase text files.

IO <port_bit> <1|0>

The 8 bits of LPT1 (at address $378 and IRQ 7) can be used to

remote control  antennas, transceiver, or to reset computers. The

ACK (pin 10 of the printer connector DB25 or CENTRONICS) must be

tied to ground. A relay driver chip as the ULN2003 may be used to

control the relays.

For example if you want to bring the bit on a high level and then

back to a low level in order to make a reset of the connected BBS :

ON7PC de ON7RC (21:25)>IO 2 1

BRU:ON7RC> IO-Port-Status:

0 1 2 3 4 5 6 7                      bit 2 is then High, and

0 0 1 0 0 0 0 0                 ---> the relay #2 is excited ...

ON7PC de ON7RC (21:26)>IO 2 0

BRU:ON7RC> IO-Port-Status:

0 1 2 3 4 5 6 7                      bit 2 is then Low, and

0 0 0 0 0 0 0 0                 ---> the relay #2 is released.

In the version of ON5ZS, the IO bit is saved in the configuration

file (TNN149.CFG).

LINKS <+|-> <port> <call>

Add or remove the specified callsign in the link table. This allows

to establish connections to nodes other than NET/ROM TheNet or

TheNetNode , and to make links to servers (Mailboxes, DX Packet

Clusters, TCP/IP servers, etc ...)

In the NORD><LINK version, adding a exclamation mark ('!') will lock

that link.

In  the version of ON5ZS, adding a exclamation mark ('!') will allow

all SSID to be switched to that port, e.g. 'L + 11 ON4CP !' will

switch all frames to ON4CP, and whatever the SSID, to port 11.

LOAD <filename>

This command allows to upload a file to the node. After you give

this command, the program will wait after the  #BIN#<length>   where

length is the length of your program in bytes. The node will send

you a #OK#  and you can start your binary transmission. The

procedure expected is those used in Autobin-THP or in the "SB" from

ESKAY. After loading the binary file, the node will give you the

checksum.

LOcal <+|-> <Call> <Alias> <Quality> <Port>

Add or remove links to the node list that do NOT send a

broadcast,i.e.

In fact  a command "LO + ON7RC-5 BRUBBS 255 5" will add

artificially, at intervals equal to the broadcast interval, an

entry in the node table exactly as "N ON7RC-5 + BRUBBS 255 5 port

neighbor digi" should do on a L3/L4 node.

If a connect to that destination is done, the L3/L4 connection is

modified to a L2 connection at that node.

This can be used for mailboxes, DX-Clusters or RMNC. Maximum 5

calls are allowed.

The LO command without argument gives the list of local nodes,e.g.

BRU:ON7RC> Local nodes:

BRUBBS:ON7RC-5 192 5

MHEARD = <value>

Sets the maximum number of station heard in the MHEARD list.

NODE <nodecall> <+|-> <ident> <quality> <count> <port> <neighbor>

[<digicall>]

Works as in previous NET/ROM and TheNet versions to add or remove

manually a node from the  node list. The identifier (max 6

characters) may start with a '#' to be hidden or it may be a '*' is

no identifier is used.

If the  count parameter is set to '0' the entry will be locked. The

port is the <port_number> defined by the PORT command (see below)

and the digicall (max 2 callsigns) is the path to the neighbor.

These informations are NOT saved in TNN149.CFG, and must be repeated

on each restart !

PARAMETER <number> <value>

Modifies the value of the parameter number

PATH <+|-> <call1> <call2>

Tells the node to add or to remove a fixed digipeater path. For

example at DB0KH, DB0DA must be called via DB0MW then DB0KH  will

add 'PATH + DB0DA DB0MW' to his path list.

PORT <rx_port_number> <tx_port_number> <mode> <port_name>

Defines each port with the

<rx_port_number>, this refers to a logical number in TheNetNode

<tx_port_number>, this refers to a logical number in TheNetNode

<mode> represents the following bits :

bit 0 : value = 1   : 1 = TNC OK          0 = TNC locked

bit 1 : value = 2   : 1 = user port       0 = link port

bit 2 : value = 4   : 1 = CTEXT on        0 = CTEXT off

bit 3 : value = 8   : 1 = DAMA on         0 = DAMA off

bit 4 : value = 16  : 1 = digipeater on   0 = digipeater off

bit 5 : value = 32  : 1 = RMNC

bit 6 : value = 64  : 1 = No link via digipeater

bit 7 : value = 128 : 1 = T2 dynamic

<port_name> is a name for each port. It will be used for

connections  between ports , the port_name is maximum 6 digits,

and  VHF, UHF, SHF are recommended for the user access ports,  and

for backbones  we recommend to use the suffixes separated by a

hyphen, for example for  the backbone link between DB0ONA and

ON5ZS we recommend to use ONA-ZS , and finally for links specially

dedicated to servers we recommended to use TCP/IP, CLUSTER, BBS

etc ...

DAMA on : to activate the DAMA mode on a port. At present time,

there is only ONE DAMA priority table. You may declare 2 ports

with the DAMA mode, but they must share the same RF channel (same

L1 support). It is not possible to have 2 (or more) DAMA ports on

2 (or more) RF channels. For example you can have a 1200 Bd and a

9600 Bd sharing the same 70 cm user access frequency.

Bit 5, 6 and 7 are implemented in the version of ON5ZS :

RMNC : simple FRACK shall be used,

No link via digipeater : a SABM frame with a VIA digipeater will

be ignored,

T2 dynamic : T2  varies with the number of frames staying in the

RX buffer. If there are to much frames in the buffer, T2 will

increase, and the SRTT of the partner too. This will be used for

RMNC and BBS system with the TF24 firmware (F6FBB, DieBox, ...)

When the declared sysop ask 'PO' he will receives an extended

list of the ports :

BRU:ON7RC> KISS-TNC-Ports:

00 00 UHF       User CTEXT DAMA ------ --- ---------- ------ 1200

01 01 U4800     User CTEXT ---- ------ --- No_Upl_via ------ 4800

02 02 S4800     User CTEXT ---- ------ --- ---------- ------ 4800

04 04 Cluster   Link ----- ---- ------ --- ---------- T2_dyn 4800

05 05 BBS       Link ----- ---- ------ --- ---------- T2_dyn 19200

06 06 BRU-HF    Link ----- ---- ------ --- ---------- T2_dyn 4800

10 10 Dochamps  Link CTEXT ---- ------ --- ---------- ------ 9600

12 12 Breda     Link CTEXT ---- ------ --- No_Upl_via ------ 1200

13 13 RC-bwp    Link CTEXT ---- ------ --- No_Upl_via ------ 1200

14 14 Charleroi Link CTEXT ---- ------ --- ---------- ------ 9600

PROMPT <string>

In the string you can use what ever kind of string, the following

symbolic strings are recognized :

%a  for the nodes identifier

%c  for the user call

%C  for the user call with SSID

%d  for the node call

%D  for the node call with SSID

%n  for the users name

%r  for a carriage return

%t  for the time

%0  to clear the prompt

The most used (and recommended) form of prompt is

'PROMPT %c de %d  (%t)  >' and if the node call is ON7RC and that

ON5TH will be connected, the  prompt looks like

'ON5TH de ON7RC (18:30) >'

READ <filename>

Reads the file

RESET SYSTEM

The RESET command performs a warm start-up reset of TheNetNode.

RESET <port>

For a reset of that particular TNC attached to that port

ROUTES <port> <nodecall>[digicall] <+|-> <quality>

Works as in previous NET/ROM and TheNet versions to add or remove

manually a route to the route list.

These informations are NOT saved in TNN149.CFG, and must be repeated

on each restart !

STAT <+|-> <call>

Add or remove the statistics for the link with <call>. <call> must

be a station where L3/L4 protocol is used.

SUSPEND <+|-> <port> <callsign> <number_of_connections>

Suspend the user to make a connection with the node to that node AND

limit to the number of connections.

Two pseudo port number may be used : 255 to lockout on ALL ports,

and 254 to allow only L2 connections. When the suspended user

attempts to make a connection, he shall receive the SUSPEND.TXT and

will be immediately disconnected.

SYSOP

The SYSOP command allows an authorized control operator to establish

his credentials prior to make the described commands. The node

will respond with a list of 5 random numbers. The sysop must then

establish his credentials  by giving the 5 characters corresponding

to the five positions in the 80 characters string defined as

password (see also <esc> p command). There is no acknowledgment of

success or failure.

TEST <port>

Send a 4 k bit sequence of 1 and 0 to that port.

TOS <command>

Works as the DOS but for Atari ST !

4. THE ESCAPE COMMANDS :

========================

These command can only be performed from the keyboard at the node. They

follows the earlier WA8DED and the well known TheFirmware command sets

but some of the commands are new, or have been slightly modified ....

ALT X        Stops the program

ESC C        Connect the keyboard as host.

ESC D        Disconnect.

ESC F <path> Modify the path for CONFIG.NET and DOS commands.

ESC I   Shows the identification of the node, the four parameters can be

modified separately with

ESC I0 <node_callsign>        e.g. ESC I0 ON7RC

ESC I1 <node_identifier>      e.g. ESC I1 BRU

ESC I2 <login>                e.g. ESC I2 this_is_my_node

ESC I3 <password>             e.g. ESC I3 TOP_SECRET

By default, the login is "thenet" and the password "Geheim". But

as a matter of facts, if your node is installed in a room where,

only the sysop have access, you can simplify the login and the

password to just one letter !

+--------------------------------------------------------------+

|   NOTE from ON7PC :                                          |

|                                                              |

|  Please use no SSID in the call, in other words use the -0 . |

|  If you want to use the same callsign as user for "in live"  |

|  keyboard connection please choose another SSID as -4 or -6  |

|                                                              |

|  About the Node Identifier or Mnemonic :                     |

|                                                              |

|  When NET/ROM was developed, the original purpose of the     |

|  identifier, was to give a geographical information. One of  |

|  the suggestions was to use the identifier of the nearest    |

|  "airport" .                                                 |

|  For ON7RC in Brussels, we adopt "BRU" as the Brussels       |

|  airport. But there is not an airport in each city ...       |

|  So I would suggest to choose carefully your ID.             |

|                                                              |

|  Good examples are CLR for Charleroi (ON7AN),                |

|                                                              |

|  If you, as Sysop, want to check the new compiled version,   |

|  or  want to check the protocol, at home, just for a few     |

|  hours,  please use "TEST" as identifier, moreover place a   |

|  "#"  sign before the ID, so that your node will not appear  |

|  in the node lists of your neighbors.                        |

|                                                              |

|  TCP/IP users choose "TCPIP" as identifier. Nice idea, but   |

|  what ever version of TCP/IP you use, please keep it on the  |

|  same "TCPIP" identifier !                                   |

|                                                              |

|  In NET/ROM and TheNet, small letters are NOT accepted       |

|  inside the mnemonic. Some sysop used that to have the       |

|  benefit of the mnemonic but to prevent the use of it        |

|  So these sysop included small letters,  please STOP these   |

|  stupid behaviors !                                          |

|                                                              |

|  Some other sysops use WW-locators ... but as well as        |

|  locators give the precise location, what do you think       |

|  that the users should more appreciate (or remember) :       |

|  to type J020EU or BRUSSE ? BRUSSE is certainly more         |

|  meaningful than J020EU.                                     |

|  Please, sysops,  STOP using WW locators as mnemonics.       |

|                                                              |

|  Legal or not ? That is another question that we heard !     |

|  I should say yes IT IS legal because every 9.5 min the node |

|  sends a beacon with callsign and identifier, so that the    |

|  node is identified legally ! Not one PTT administration     |

|  did any objection so don't get yourself in trouble ...      |

|                                                              |

|  Just for convenience ... it is more easy to type 3 letters  |

|  than  5 or 6 letters, more over you don't carry about       |

|  numeric pad or not, about "shifting" for the numbers...     |

|                                                              |

|  And much more ... using the identifier or mnemonic you can  |

|  make multiple connections to the node, in our example, you  |

|  can of course connect ON7RC, and BRU, but also BRU-1,       |

|  BRU-2, BRU-3, ...  up to BRU-15. You can make up to 17      |

|  simultaneous  connection to your node. The SSID used after  |

|  BRU must NOT be defined in the configuration of the node,   |

|  This is a standard feature, but I agree, it is not well     |

|  documented in the original papers of NET/ROM, TheNet        |

|  or even TheNetNode ...                                      |

+--------------------------------------------------------------+

ESC K   Gives date and time

ESC K0  Monitoring without date and time

ESC K1  Time and date will be added in monitoring

ESC L       Shows the status

ESC Ln      Shows the status of channel n

ESC LOGOUT  As there is a login procedure, the logout inhibit

undesirable  actions on the keyboard, this can be useful if the

node is located in the shack of a radio club .... (This command

is not documented in the original papers).

ESC M x Controls the monitoring :

ESC M N    No monitor

ESC M I    Monitors only information frames

ESC M U    Monitors only unnumbered information frames

ESC M S    Monitors only supervisory frames

ESC MUSIC  Monitor all kind of frames

ESC <+|-> MUSIC + ON5ZS ON6NR  add or remove monitoring of

all frames with ON5ZS or ON6NR in the address field.

+---------------------------------------------------------+

|    VERY USEFUL TO DETECT LINK AND PROTOCOL PROBLEMS !  |

+---------------------------------------------------------+

ESC P       Gives the password for example ESC PMary had 26 lambs ...

modify the password to "Mary had 26 lambs ...". Note that

there is no space between the 'P' and the first letter of

the password. By default the password is  "1234567890123456

..." .

ESC QUIT     Saves all the configuration parameters and quits

TheNetNode.

ESC R0       No trace of token recoveries

ESC R1       Trace the token recoveries.

+---------------------------------------------------------+

|      VERY USEFUL TO DETECT TOKEN RING PROBLEMS !       |

+---------------------------------------------------------+

ESC S        Shows the used channel

ESC S n      Switch to channel n

ESC T        Shows the token ring speed

ESC T 19200  Sets the token ring speed at 19200 bit/s

ESC Y0       No connection to the host

ESC Y1 - Y4

5. THE USER COMMANDS :

======================

Aktuell

Aktuell gives red hot information about the node ...

(Please note that the German word "aktuell" means "current" or

"today's information"  a little bit in the way of "message of the

day",  well known by TCP/IP users ... and do not confuse "aktuell"

with the  English word "actual" who has a meaning of "real").

BCnodes

The BCNODES command shows you the list of the ports where the

broadcast is sent to. For example :

BRU:ON7RC> Broadcast:

10 NODES

12 NODES

10 NODES

13 NODES

14 NODES

BEacon

Displays the text, that is transmitted as a BEACON, for example :

TheNetNode (PC) V1.22d-1a-on5zs (BRU:ON7RC)

TNN  sends this  beacon every 10 minutes on every port and monitored

with a terminal program as ESKAY (SP) it looks like:

ON7RC>ID ctl UI^ pid F0

TheNetNode (PC) V1.22d-1a-on5zs (BRU:ON7RC)

But TNN can also send two other beacons, the telemetry beacon and

the link statistics.

CONNECT

This is a mode that allows anyone to establish a connection to

another packet station (level 2) or another node (level 3/4).  To

use CONNECT you can give it either a callsign (ie. ON5ZS) or a

TheNet node name (or mnemonic) (ie. NMR).

There are mainly 4 ways to make connects, and we will explain this

is the same order that TNN will do :

- if the call is in the node list, TNN will connect that node

(TheNet or TheNetNode) . Check first with the 'N' command, if for

example DB0ONA appears in the list you can make a connection to

that node simply by typing 'C DB0ONA'.  If no match is found the

next search will be done in the link table :

- the sysop can define some L2 links, first check these links with

the 'L' command, if for example you find in this list ON4DXB, you

can initiate a connection to ON4DXB simply by typing 'C ON4DXB'.

When a match is found in the link table, a connect to that user

via the listed path is initiated.  It is not necessary for the

originating station to know the whole path to reach another user.

You need only to give the destination callsign. If there is no

match found either in the node table or in the link table, TNN

will check the MH list.

- if a match is found in the MH list, a connection shall be attempt

to that call on the right channel ...

- finally if the  call is not in the MH list, a connection shall be

done on port 0 ! That is the reason why it is important to use

port 0 as the main users access (user accesses are on 70 cm as IARU

Recommendation !)

The possible responses are

Connected to ....     if the connection is established

Failure with ....     if the station don't answer

Busy from........     if the station has no more user channel

available

Node busy             if the node is busy

Link table full       if the link table is full

Circuit table full    if the circuit table is full

Host table full       if the host table is full

If you are using this TheNetNode, TheNet version 1.16, when you quit

the connected node, with a (Q)uit command, you will be reconnected

to the last connected node except if you use the 'C! callsign'

command for connecting that node.

CONvers

With the CONVERS command a user can enter into a conference mode

with other users.  Whatever you type to the conference will be sent

to all the other members of your conference group.  Each message is

preceded by the callsign of the sender of that message.

Since TNN v1.47 the converse module from DK5SG has been adopted, and

it is now possible to converse with other user connected to other

TNN nodes, making the whole network as a big converse node !

A list of all the commands available from within the CONVERS mode

can be seen by typing '/HELP'.

Upon entering CONVERS, you find yourself connected with all the

users who are currently on CONVERS channel 0.  The normal operation

way is to ask the user list with '/W' and either to choose a free

channel and to wait that somebody join you, or to join a group

already in conference.

Here are the commands while in CONVERS mode:

/?, /HELP            Returns the help information.

/BYE, /EXIT, /QUIT   Exits from CONVERS mode.

/CHANNEL n           Changes the CONVERS channel to n.

0-32767 are valid channel numbers.

/INVITE user         This command is used to ask another user to

join you on your current channel.  It makes

no difference whether the other user is on a

different CONVERS channel or just connected

to the Node. The user will receive a msg as:

"*** Message from <call> at <time> ... Please

join  convers channel <channel>" and the

other station will receive a msg like

"*** Invitation sent to <call> @<host_node>".

/LINKS [LONG]        list all connections on the OTHER TNN nodes.

/MSG user text...

/WRITE user text...  Sends a message containing 'text' to 'user'.

/WHO [QUICK] [LONG]  Lists out all the users and the channel they

are listening on.

/WHO QUICK -  only the users on your channel.

/WHO LONG  - gives a detailed list of all

users.

In addition to the CONVERS mode commands you can also issue all the

regular  TheNetNode commands without leaving CONVERS mode.  This is

done by preceding  the command with a '!'.   ie. '!USER' gives a

list of node users, '!NODES' returns the nodes list.

All of the commands can be abbreviated.

The /QUIT command terminates the CONVERS mode session and returns

you regular node command level.

CQ

The CQ command is used to start a CQ call.  You can also include a

short text to be sent out as UI Frames addressed to CQ ,  ie.  if

ON4AB enters 'CQ Why doesn't anyone answer me...', then the  text

'Why doesn't anyone answer me...' is sent out port 0 (the user

access  port) as UI Frames:

fm ON4AB-15 to CQ ctl UI^ pid F0

Why doesn't anyone answer me...

In the user list the following entry will appear :

Uplink(ON4AB)           <-->        CQ(ON4AB-15)

There are 2 ways that a person can connect to ON4AB :  first, if

another  user tells his station 'C ON4AB-15' or second, if a user

connects to the node and then tells the node: 'C ON4AB-15'.

Both of these methods will work as long as ON4AB doesn't enter

another command to the node.

HArdware

The "HARDWARE" command or just "HA", returns a list of the hardware

at this node.  It can contain setup of the node including radio

equipment, antennas, etc.

Help

The "HELP" command gives a help file (highly recommended for

newcomers hi !).

Help INDEX

Gives a list of all exhaustive help files, each of them can been

read with HELP <file_name>.

Help <file_name>

Gives the help for the various commands.

Info

The "INFO" command or just "I" returns information about local node,

the name of the SYSOP to contact for problems on this node.

Links

The "LINKS" or just "L" command, returns the current active L2 links

along with the port it is using, for example you could got :

BRU:ON7RC> Links:

P6 (BRU-HF)     ON7RC-6

P4 (Cluster)    ON4DXB

P1 (U4800)      ON5ZS

P1 (U4800)      ON1KOT-5

P12 (Breda)      PI1HWB

P13 (RC-bwp)     ON1BWP

P1 (U4800)      ON7AN

P5 (BBS)        ON7RC-5

P2 (S4800)      ON1BJC

P1 (U4800)      ON1BJC-15

P1 (U4800)      DG5YET-15

P1 (U4800)      ON7PC-5

P1 (U4800)      ON7PC-4

P1 (U4800)      ON6UG-13

P1 (U4800)      ON1BLM-15

P1 (U4800)      F1NJL-15

P1 (U4800)      ON1BUW-15

P1 (U4800)      ON4KTP-15

P1 (U4800)      PA3FSY-15

P1 (U4800)      ON6KY

P1 (U4800)      ON1BUW

P1 (U4800)      ON6IW-15

P1 (U4800)      ON6GB-1

P1 (U4800)      ON7WE-15

P1 (U4800)      ON4ANT-15

P1 (U4800)      ON4ABP-15

P1 (U4800)      ON4AKD-15

P1 (U4800)      ON5EB-6

P1 (U4800)      PI8DXH-15

P1 (U4800)      ON1KZZ-15

P1 (U4800)      ON1AMM-15

P1 (U4800)      ON4ACG-15

P1 (U4800)      ON1DC-15

P1 (U4800)      ON5RF

P12 (Breda)      PI1HWB

P1 (U4800)      ON5VY-15

P1 (U4800)      ON4TH-15

P1 (U4800)      PA3EBT-15

P1 (U4800)      ON4PJ-15

P1 (U4800)      ON7PC-15

P1 (U4800)      ON5IL-15

P1 (U4800)      ON7PC-1

P2 (S4800)      ON6MH

P1 (U4800)      ON5CN-15

P1 (U4800)      ON1KGX-10

P1 (U4800)      DD3IA-15

P1 (U4800)      ON4ACT-15

P1 (U4800)      ON6JG-15

P1 (U4800)      ON5PV-15

|     |          |      |

|     |          |      +----- "!" = all SSID accepted (for RMNC)

|     |          +------------ callsign

|     +----------------------- port name

+----------------------------- port number

But one of the most important features of this LINK command is to

provide a way to make connections with  RMNC/Flexnet nodes and that

is the case of ON4CP.

LOcal

The LOCAL or "LO" command shows the defined local links. The L3/L4

connection is then modified on L2 at the node. This can be used for

mailboxes, or RMNC digipeaters. Example :

BRU:ON7RC> Local nodes:

BRUBBS:ON7RC-5 240 5

Map

The "MAP" or "M" command returns a small chart of the current links

around this node.  A user can use this command to get a quick look

at the accessible mailboxes, nodes, TCP/IP servers, DX-Cluster etc

...

Sometimes a drawing is better than 10 000 words ...

MHeard

Shows the callsigns of stations heard by the node.  Only stations

heard direct by node are added to the MH List.

Normally just the last 10 callsigns heard are given. The

presentation looks like:

+---  maximum size of the list

|    +--- maximum possible size of the list

|    |

BRU:ON7RC> MHEARD (100/1210)

20.03.93 14:23 (U4800)      [     41983,   1374095] ON7PC-4

20.03.93 14:23 (UHF)        [     18534,    130613] ON1BBJ

20.03.93 14:23 (UHF)        [      8477,   2038041] ON4KST

20.03.93 14:23 (UHF)        [      9246,   1062196] ON1BOR

20.03.93 14:23 (UHF)        [     33411,   2201037] ON4PJ

20.03.93 14:09 (UHF)        [     53655,   4183815] ON1KZZ

20.03.93 14:07 (UHF)        [     15920,    388501] ON1ABS-7

20.03.93 13:52 (U4800)      [   1875054,   8727415] ON7PC

20.03.93 13:52 (UHF)        [    319719,   5500712] ON7KO

20.03.93 13:41 (U4800)      [    183839,   7288491] ON6KY

|      |      |                |           |     |

|      |      |                |           |     +--- callsign

|      |      |                |           +--- number of

|      |      |                |             transmitted bytes

|      |      |                +--- number of received bytes

|      |      +--- port name

|      +--- time (UTC)

+--- date

Optionally you can give a number after the command to list more

stations heard. For example 'MH 20' would return the last 20

callsigns heard.

When a callsign is used instead of a number, the command returns the

last time that callsign was heard.  SSIDs are not taken into

account.

Nodes

Returns all the currently known nodes, that use the NETROM/TheNet-L3

protocol.  As far as the quality is higher than the min-Quality

(parameter 2), the information in the nodes list is automatically

sent out over BROADCASTS to the TheNet neighbor nodes. A typical

presentation looks like :

ON7PC de ON7RC (14:25)> N

BRU:ON7RC> Nodes:

Julich:DB0BM         NH:DB0DOZ        KS:DB0EAM      DTMD:DB0FBB

DOBOX:DB0HAG        MG:DB0II      Kleve:DB0KV     Kaarst:DB0OVN

Kevela:DB0PKE    DinTCP:DB0QS      MGBox:DK0MWX     ON-lx:LX0PAC

SAB:ON1BWP       KGX:ON1KGX    CLRBBS:ON1KGX-5  DX-Lge:ON4DXL

SATbbs:ON4KVI-5    CORN:ON5PL        LGE:ON5VL     LGEBBS:ON5VL-5

D0:ON5ZS        CLR:ON7AN     BRUBBS:ON7RC-5     GOR6:PI1GOR-6

Breda:PI1HWB       HRL:PI1LIM      NYMS:PI1NYM-5    NYME:PI1NYM-6

Breda:PI8BRD       EHV:PI8EHV     ESA70:PI8ESA       FLK:PI8FLK

NBwTCP:PI8GTB      RMNC:PI8HRL    Hrlbbs:PI8HRL-8  NETROM:PI8HRL-9

HVH:PI8HVH          :PI8HWB       HVS:PI8NOS    NYM430:PI8NYM

RMD:PI8RMD    DeBilt:PI8RNI          :PI8THT-7   Mburg:PI8VLI

:PI8YRC

Additionally, if a callsign is given, the routing information for

that node is returned, e.g  N ON5ZS  gives

Routes to D0:ON5ZS

--Qua-Obs-Po--Neighbor--

> 220 6   10  ON5ZS

|   |  |   |    |

|   |  |   |    +--- callsign of the neighbor

|   |  |   +--- port number

|   |  +--- obsolescence counter

|   +--- quality

+--- used (">") or not

In this case the node callsign (ON5ZS) is the same as the neighbor,

so a direct link exists between this node (ON7RC) and ON5ZS !

"NODES #" will output the quality and obsolescence count as well.

ON7PC de ON7RC (14:26)> n #

BRU:ON7RC> Nodes:

Julich:DB0BM    93/5        NH:DB0DOZ   93/5        KS:DB0EAM   82/5

DTMD:DB0FBB   82/5     DOBOX:DB0HAG   81/5        MG:DB0II   119/5

Kleve:DB0KV    93/3     Kaarst:DB0OVN   93/5    Kevela:DB0PKE  93/5

DinTCP:DB0QS    82/5     MGBox:DK0MWX   93/5     ON-lx:LX0PAC  150/5

SAB:ON1BWP   78/4       KGX:ON1KGX   150/5  CLRBBS:ON1KGX-5 147/5

DX-Lge:ON4DXL   129/5   SATbbs:ON4KVI-5 129/5     CORN:ON5PL   189/5

LGE:ON5VL    162/5   LGEBBS:ON5VL-5  127/5       D0:ON5ZS   220/5

CLR:ON7AN    220/5   BRUBBS:ON7RC-5  192/0    GOR6:PI1GOR-6 109/5

Breda:PI1HWB   140/5      HRL:PI1LIM   119/5     NYMS:PI1NYM-5 94/2

NYME:PI1NYM-6 89/5     Breda:PI8BRD   139/5      EHV:PI8EHV  120/2

ESA70:PI8ESA   94/5       FLK:PI8FLK   78/2    NBwTCP:PI8GT   101/5

RMNC:PI8HRL   119/5   Hrlbbs:PI8HRL-8 119/5  NETROM:PI8HRL-9 119/5

HVH:PI8HVH   93/5          :PI8HWB   1/1        HVS:PI8NOS  108/5

NYM430:PI8NYM   89/2       RMD:PI8RMD   89/2    DeBilt:PI8RNI  94/5

:PI8THT-7  1/4      Mburg:PI8VLI   83/5          :PI8YRC  1/4

The NODE command can also be used with wildcards, example "NODES

PI*" will give all the list of all nodes starting with PI.

ON7PC de ON7RC (14:26)> N PI*

BRU:ON7RC> Nodes:

GOR6:PI1GOR-6 109/5   Breda:PI1HWB   140/5     HRL:PI1LIM   119/5

NYMS:PI1NYM-5 94/2     NYME:PI1NYM-6 89/5    Breda:PI8BRD   139/5

EHV:PI8EHV   1202   ESA70:PI8ESA   94/5      FLK:PI8FLK   78/2

NBwTCP:PI8GTB   10/5    RMNC:PI8HRL   119/5  Hrlbbs:PI8HRL-8 119/5

NETROM:PI8HRL-9 119/5    HVH:PI8HVH   93/5         :PI8HWB   1/1

HVS:PI8NOS   108/  NYM430:PI8NYM   89/2      RMD:PI8RMD   89/2

DeBilt:PI8RNI   94 5        :PI8THT-7 1/4     Mburg:PI8VLI   83/5

:PI8YRC   1/4

"NODES *" will display all the known nodes.

The information is always given in the form IDENT:Callsign. IDENT is

at the most 6 characters long and should reflect a short geographical

description of the location of the node.

The quality value is in the range between 0 and 255. To convert it

in percent, simply divide the quality by 256 and multiply it by

100%. So 255 is a very good quality, and 76 means that only 30% of

the frames arrive without errors. A quality of 0 means a locked

entry (done by the sysop) and a quality of 1 means that the node has

been heard but didn't transmit his broadcast table.

Parms

PARMS give a table with all the parameters of TheNetNode:

The first part of the table is common to all ports :

BRU:ON7RC> Parms:

01: Max-Nodes   170     02: min-Quality 77      03: Obs-Init    6

04: min-BCast   4       05: Broadcast   600     06: Lifetime    30

07: T-Timeout   240     08: T-Retry     2       09: T-AckDelay  3

10: T-BsyDelay  180     11: T-Window    18      12: NoAckBuf    35

13: Timeout     3600    14: T3-Timer    18000   15: CallCheck   1

16: SaveConfig  6       17: Alpha 1     7       18: Alpha 2     15

19: BetaMin x10 25      20: BetaMax     6       21: Paclen      236

22: DAMA Enable 1       23: DAMA Tout   80      24: DAMA Maxcnt 20

25: Poll-UI-Cnt 10      26: L3-RTT-Int  175     27: L3-Alpha    15

28: dummy       8

01: Max-Nodes       170

Maximum allowable number of destinations in the node routing

table. Every node takes 36 bytes of memory.

02: min-Quality     77

Poorest route quality that will be automatically added to the

node routing table. Only nodes having a higher quality are used

to update the node list. If min-Quality is set to 0, the

automatic updating is suspended

03: Obs-Init        6

Initial value given to the obsolescence count of a route that

has been  newly added or updated by the node's automatic routing

table mechanism. On each broadcast interval, where the Neighbor

node is not heard, the obsolescence counter is decremented. If

the obsolescence counter is 0 then the node is discarded.

04: min-BCast       4

Minimum obsolescence count threshold below which a route will

not be included in the node's automatic routing broadcasts.

05: Broadcast       600  ( = 10 min)

Number of seconds between automatic routing broadcasts issued by

the node. If this parameter is set to 0 the broadcast is

suspended (NOT RECOMMENDED AT ALL !!)

06: Lifetime        30

Initial value of the 'time-to-live' field in the network header

of all network-layer frames originated by this node. The

time-to-live is decrement by each intermediate node that relays

the frame. And  if the time-to-live ever reaches 0, the frame is

discarded.

07: T-Timeout       240

Number of seconds between transport-layer retries.

08: T-Retry         2

Maximum number of transport-layer tries attempted before a

circuit failure is reported.

09: T-AckDelay      3

Number of seconds of delay used by the transport layer from the

time it receives an information message until it sends an

information-acknowledge message.

10: T-BsyDelay      180

Maximum number of seconds that the transport layer will remain

'choked' as the result of an incoming message that has the choke

flag bit set.

11: T-Window        18

Number of incoming out-of-sequence information messages (frames)

that the transport layer will buffer while waiting for the next

expected information message to arrive.

12: NoAckBuf        35

Maximum allowed backlog of messages (frames) that the

transport layer will buffer before it sends a choke message.

13: Timeout         3600  ( 3600 sec = 60 min)

Maximum number of seconds that a transport layer circuit or a

link layer connection can remain idle before it is automatically

disconnected.

14: T3-Timer        18000   (= 180 000 msec = 3 min)

Maximum no activity period (in 10 ms increments) permitted by

the link layer before it issues a poll to make sure the link is

still intact.

15: CallCheck       1

Callsign validation (= 1) or not (= 0). The validation

conditions are:

- call must have 4 to 6 characters

- all characters must be numbers or letters

- at least one or two numbers

- the last character must be a number

- the SSID must be in the range of 0 to 15

16: SaveConf        6

Time (in 10 min increments) between automatic save of CONFIG.NET

and statistics.

17 : Alpha 1        7

Used to compute the actual RTT.

18 : Alpha 2        15

Used to compute the actual RTT.

19 : Beta x10       25

Used to compute the actual FRACK, FRACK = Beta x SRTT

20 : BetaMax        6

21 : Paclen         236

Define the length of the frames send from the node e.g. when a

Help is requested.

22 : DAMA Enable     1

0 = disable and 1 = enable DAMA mode.

23 : DAMA Tout      80

Time in msec that the node waits for the reception of an ACK,

and before going to the next polled user.

24 : DAMA Mxcnt     20

Maximum value of the counter for no activity.

25 : POLL-UI-Cnt    10

Number of warnings sent from the node to the use in order to

ask non DAMA users to increase their FRACK. After this number

of counts, the users is disconnected and receive the

POLLDISC.TXT

26 : L3-RTT-INt     175

Time in 10 msec between L3 measurement frames.  Will be used to

compute automatically the quality in the  next future ...

27 : L3 Alpha  15

28 : Dummy      8

Used for testing purpose by developers ...

The second table gives parameters for each port :

Port #   Qual TXDel PPer Slot TXTail Fulldup IRTT MaxFr L2Ret T2Timer

0  30:   175    25    128  5     3     0     236   2    10     219

1  40:   0      15    128  5     3     0     246   4    10     54

2  50:   0      25    128  5     3     0     189   4    10     54

4  70:   0      2     255  3     3     1     115   2    10     54

5  80:   0      2     255  3     3     1     100   4    10     27

6  90:   0      2     255  3     3     1     115   2    10     54

10 130:  220    3      255  3     3     0     75    3    20     27

12 150:  140    30     255  10    3     0     557   3    10     219

13 160:  78     30     255  10    3     0     456   3    10     219

14 170:  220    2      255  3     3     0     74    3    20     27

Quality             175

Default quality of the radio channel connected to the node HDLC

port.

TXDelay             25

TX-Delay in ms.

P-Per               128

Together with slottime (next parameter) defines the exponential

delay algorithm used by the node when keying up its transmitter.

Slot                5

Slottime in ms.

TXTail              3

TX-Tail in Bytes

Fulldup             0

Full duplex enabled (= 1) or not (= 0).

IRTT                236

Initial Round Trip Timer .

MaxFrame            2

Maximum number of outgoing frames that the link layer will send

without receiving acknowledgment.

L2Retry             10

Maximum number of tries that the link layer will attempt before

reporting a link failure.

T2-Timer            219

Delay (in 10 ms increment) used by the link layer from the time

it receives an information frame until it sends an

acknowledgment (RR, RNR or REJ).

Rem: The indicated values here above are 'standard' values.

Path

Shows the nodes or users whose path have been defined. This path

will be used for every connection to that node or user.

For example if the sysop gives the path to DB0KG with :

'path + db0fe-7 db0fe-1

'path + db0kg db0fe-7

a connection issued by a user as 'C DB0KG' will be done via DB0FE-1

and DB0FE-7. In this case the connection uses Level 2 protocol.

POrt

The "PORTS" or just "PO" command gives the port list, along with

the port name and the KISS address.

BRU:ON7RC> KISS-TNC-Ports:

00  00 UHF        User CTEXT DAMA -------

01  01 U4800      User CTEXT ---- -------

02  02 S4800      User CTEXT ---- -------

04  04 Cluster    Link ----- ---- -------

05  05 BBS        Link ----- ---- -------

06  06 BRU-HF     Link ----- ---- -------

10  10 Dochamps   Link CTEXT ---- -------

12  12 Breda      Link CTEXT ---- -------

13  13 RC-bwp     Link CTEXT ---- -------

14  14 Charleroi  Link CTEXT ---- -------

|   |     |        |     |     |   |

|   |     |        |     |     |   +- L2 digipeating enabled or not

|   |     |        |     |     +- DAMA mode enabled or not

|   |     |        |     +- CTEXT enabled or not

|   |     |        +- type  "User", "Link" or "off"

|   |     +- port name :

|   |        - VHF, UHF, SHF , etc ...

|   |        - or node suffix followed by the neighbor node

|   |          for example on port 14, we have the link between

|   |          ON7RC and ON7AN, so we have "RC-AN"

|   +- tx port number ( = KISS address)

+- rx port number ( = KISS address)

Port 0 is, by concept, reserved for the local access on 70 cm. If

there are more user accesses, port 0 is the most important (the one

on 70 cm as recommended by IARU Region 1 !) and the other can be

attached to other ports.

Quit

Ends the connection to a remote node.  You are returned to the

previous node.  You can then re-connect to the same remote node or

to another one if you wish.

Routes

Returns the path to the neighbor nodes and which port to use.

The typical display is :

Routes of BRU:ON7RC

Node------Qua-Po-Dst-Act-Lck-SRTT-L3SRTT-(10mS)---- Route---

LOCAL      80  5   1  no  no    -      -

ON7AN     210 14  19 yes  no   81      0

ON5ZS     220 10  12 yes  no   59    180

ON1BWP     77 13   1 yes  no  199      0

|        |   |   |  |   |    |      |              |

|        |   |   |  |   |    |      |              +--- non TheNet

|        |   |   |  |   |    |      |                   digipeater

|        |   |   |  |   |    |      +--- L3SRTT in 10 msec

|        |   |   |  |   |    +--- SRTT

|        |   |   |  |   +--- locked or not

|        |   |   |  +--- active or not

|        |   |   +--- number of destination nodes on this port

|        |   +--- port number

|        +--- quality of this route

+--- callsign

SOftware

Returns a description of the software in use by this node.

Statistics

Returns a short list of statistics under the headings data bytes,

Info frames, RR frames, REJ frames and RNR frames, it also shows

the number of necessary token recoveries, invalid frames and

incomplete frames.

The typical display is:

BRU:ON7RC> Statistics: (17.03.93 18:58:35 - 20.03.93 14:24:17)

Token recoveries : 151    20.03.93 13:50:23

Token ring speed : 57600 bit/s.

Fifo mode (16550): ON

Token/sec        : 84   -  min/max: 59/104

Msg buffer used  : 5460   max: 131220   available: 267720 Bytes

Invalid frames   : 20    20.03.93 12:48:51

Runtime          : 67 hours

Links/port  : 4/0  1/1  0/2  10/4  2/5  1/6  1/10  1/12  1/13  1/14

Max tries before recovery : 500

-Port----RX-----TX-- : -Port----RX-----TX-- : -Port----RX-----TX--Kb

UHF      4535  42403 : U4800    2943  24054 : S4800       0      0

Cluster 12340   8753 : BBS    201017  51025 : BRU-HF   2232   5499

Dochamp 76295 139499 : Breda   21018  42592 : RC-bwp  13932  43489

Charler  27619   53212 :

Total activity: 772469 Kbytes

For link statistics type 's *'

As you can see the msg buffer is really  huge compared with other

systems (TheNet or RMNC).

Please note the activity : 772469 kbytes or 772 Mb in 19 days (the

period is show here below : 01 Mar to 20 Mar ...) or 40 Mbytes a

day !

So if you want now the complete statistics, type 's*' and you will

get :

BRU:ON7RC> Statistics: (17.03.93 18:58:35 - 20.03.93 14:24:26)

Token recoveries : 151    20.03.93 13:50:23

Token ring speed : 57600 bit/s.

Fifo mode (16550): ON

Token/sec        : 84   -  min/max: 59/104

Msg buffer used  : 6480   max: 131220   available: 267720 Bytes

Invalid frames   : 20    20.03.93 12:48:51

Runtime          : 67 hours

Links/port       : 4/0  1/1 0/2 10/4 2/5 1/6 1/10 1/12 1/13 1/14

Max tries before recovery : 500

-Port-----RX----TX-- : -Port-----RX----TX-- : -Port----RX----TX--Kb

UHF      4535  42403 : U4800    2943  24055 : S4800       0      0

Cluster 12341   8753 : BBS    201017  51025 : BRU-HF   2232   5499

Dochamp 76295 139500 : Breda   21018  42592 : RC-bwp  13932  43489

Charler  27619   53212 :

Total activity: 772472 Kbytes

Link to ON5ZS            01.03.93 06:14:38 - 20.03.93 14:24:24

RX:   76145 Kbytes    595855 i   481100 rr    10393 rej       0 rnr

TX:  134503 Kbytes    767680 i   444450 rr     6654 rej       0 rnr

Link to ON7AN            01.03.93 06:14:38 - 20.03.93 14:24:18

RX:   27619 Kbytes    175254 i   201963 rr      272 rej       0 rnr

TX:   50759 Kbytes    278159 i   159186 rr      498 rej       0 rnr

Link to ON7RC-5          01.03.93 06:14:38 - 20.03.93 14:24:05

RX:  198282 Kbytes    867763 i   305087 rr     2061 rej      38 rnr

TX:   50468 Kbytes    302086 i   441457 rr     4689 rej   13100 rnr

Link to ON7RC-6          01.03.93 06:14:38 - 20.03.93 14:24:06

RX:    2232 Kbytes     14232 i    28957 rr        0 rej      13 rnr

TX:    5496 Kbytes     28364 i    12923 rr       14 rej       0 rnr

Link to ON1BWP           01.03.93 06:14:38 - 20.03.93 14:24:19

RX:   13884 Kbytes    118401 i   124304 rr     5192 rej       2 rnr

TX:   40949 Kbytes    222963 i   129577 rr     3106 rej       0 rnr

Link to ON4DXB           01.03.93 06:14:38 - 20.03.93 14:24:24

RX:   12341 Kbytes    144865 i   132147 rr        1 rej       0 rnr

TX:    8753 Kbytes    130421 i   138235 rr        3 rej      23 rnr

Link to PI1HWB           01.03.93 06:14:38 - 20.03.93 14:24:24

RX:   21017 Kbytes    185948 i   188753 rr     2375 rej       0 rnr

TX:   40124 Kbytes    294997 i   144871 rr      536 rej       0 rnr

+-----------------------------------------------------------------+

| NOTE FROM ON7PC :                                               |

|                                                                 |

|   As you can see you have a track of what is happening on your  |

|   node and the command User + will give you much more info !    |

|   No any other kind of node lets you REALLY evaluate the        |

|   quality of the links as TNN does !                              |

+-----------------------------------------------------------------+

Time

Returns the date and time from the real time clock of the computer.

User

This command returns a list of all stations currently connected to

the node. A typical display will be :

BRU:ON7RC> TheNetNode (PC) V1.49a-2 (4323)

Uplink(ON4DXB)                  <-->  Circuit(D0:ON5ZS ON4DXB)

Uplink Cluster

Circuit(CLR:ON7AN ON4TH)        <-->  Downlink(ON4TH-15 ON4DXB)

Downlink Cluster

Uplink(ON4KST)                  <-->  Downlink(ON4KST-15 ON4DXB)

Downlink Cluster

Circuit(D0:ON5ZS SP6STY)        <-->  Downlink(SP6STY-15 ON7RC-6)

Uplink @ ON5ZS via LX0PAC DB0DQ SR6BBS

Downlink BRU-HF

Circuit(CORN:ON5PL DF9KT-1)     <-->  Downlink(DF9KT-14 ON4DXB)

Uplink @ ON5PL via DB0PRA DB0NET

Downlink Cluster

Circuit(D0:ON5ZS DG5NEX)        <-->  Downlink(DG5NEX-15 ON4DXB)

Uplink @ ON5ZS via LX0PAC DB0KT

Downlink Cluster

Uplink(ON7RC-5)                 <-->  Circuit(CLR:ON7AN ON7RC-5)

Uplink(ON1BBJ)                  <-->  Downlink(ON1BBJ-15 ON4DXB)

Downlink Cluster

Circuit(LGE:ON5VL ON4AHQ)       <-->  Downlink(ON4AHQ-15 ON4DXB)

Downlink Cluster

Circuit(PI8HWB PI8DXH)          <-->  Downlink(PI8DXH-15 ON4DXB)

Downlink Cluster

Circuit(ESA70:PI8ESA PA3EZH)    <-->  Circuit(ON-lx:LX0PAC PA3EZH)

Circuit(PI8HWB PI8HWB-6)        <-->  Circuit(SAB:ON1BWP PI8HWB-6)

Circuit(CLR:ON7AN ON6GB-14)     <-->  Downlink(ON6GB-1 ON4DXB)

Downlink Cluster

Circuit(D0:ON5ZS LX2PA)         <-->  Downlink(LX2PA-15 ON4DXB)

Uplink @ ON5ZS via LX0PAC-1

Downlink Cluster

Uplink(ON5ZS)

Uplink(ON7PC-4)

Uplink U4800

Circuit(D0:ON5ZS DL1RSL-14)

Uplink @ ON5ZS via LX0PAC DB0WZB DB0BLN

--> more info with "U +"

ON7PC de ON7RC (14:27)> U +

where:

BRU    is the Mnemonic or "ident" of TheNetNode,

ON7RC  is the official callsign of TheNetNode,

(4323) represents the number of free buffers, each is 36 bytes long.

Uplink indicates that the user with callsign in () entered the

network on this node eventually via an AX.25 L2 digipeater,

Uplink is a L2 connection,

Downlink indicates that the user with the callsign in () exits the

network on this node eventually via an AX.25 L2 digipeater,

Downlink is a L2 connection,

Circuit on the right side column means that the user with callsign

in () is coming from the following node and has entered that

node with the callsign that follows, Circuit is a L3/L4

connection,

Circuit on the left side column means that a connection to the node

mentioned in () before is active  and that the user there has

the following callsign,

Host shows a connection to the operator terminal of the node,

CQ appears only in the right column and indicates that this user is

calling "CQ".

<--> represents an active patchcord in the node

<..> indicates a CQ or a connection in-progress.

An entry without anything in the right side column shows that the

user is connected with the command interpreter (L7) of this

node.

User +

This command is only available in the version of ON5ZS. It shows two

tables : one for level 2 connections, the other for level 4

connections.

The level 2 table looks like :

L2 - User:

Port SrcCall DstCall LS RxTxTr SRTT    RxB     TxB Baud ConTime Pri

--------------------------------------------------------------------

12 ON7RC     PI1HWB  IXF 0 0 0 507 1196959  4223089 178 67:28:26 -

10 ON7RC     ON5ZS   IXF 0 0 0  76 4495585 16676967 697 67:29:01 -

4 ON7RC    ON4DXB   IXF 0 0 0  73   27219    57573  14 13:14:23 -

4 ON4AHQ-15 ON4DXB  IXF 0 0 0  91   14365      298   7  4:20:06 -

1 ON7RC     ON7PC-4 IXF 0 0 0 217     160    19316 248  0:10:28 -

0 ON7RC     ON4KST  IXF 0 0 0 147     573   394633 111  7:52:07 4

0 DL1RSL-1  F6BUE-2 SET 0 0 1 236       0        0   0  0:00:01 1

4 ON4KST-15 ON4DXB  IXF 0 0 0  82   25999      442   9  6:05:48 -

14 ON7RC     ON7AN   IXF 0 1 0  58 2361335  2661952 273 40:50:40 -

4 ON4TH-15  ON4DXB  IXF 0 0 0 107    4307       27  13  0:43:51 -

4 PI8DXH-15 ON4DXB  IXF 0 0 0  87    3969      365  16  0:34:12 -

5 BRU-3     ON7RC-5 IXF 0 0 0  32   17052   150703 388  0:57:35 -

13 ON7RC     ON1BWP  IXF 0 4 0 672  824109  4636233 226 53:37:37 -

4 DG5NEX-15 ON4DXB  IXF 0 0 0 115    1876        1   1  3:11:09 -

4 DF9KT-14  ON4DXB  IXF 0 0 0 115     355        0   1  0:26:41 -

4 ON6GB-1   ON4DXB  IXF 0 0 0 114    3749        6   7  1:07:46 -

0 ON7RC     ON1BBJ  IXF 0 0 0 147      17     7021   8  1:56:19 5

4 ON1BBJ-15 ON4DXB  IXF 0 0 0 112    6471        8   7  1:56:15 -

4 LX2PA-15  ON4DXB  IXF 0 0 0 115    4453        7  12  0:49:13 -

6 SP6STY-15 ON7RC-6 IXF 0 0 0 115    1768       14  13  0:17:39 -

where:

SrcCall  is the source call for the L2 connection

DestCall is the destination call for L2 connection

LS       is the link status with :

SET = Link-Setup

FMR = Frame Reject

DRQ = Disconnect Request

IXF = Info Transfer

REJ = REJ Sent

WAK = Waiting Acknowledge

DBS = Device Busy

RBS = Remote Busy

BBS = Both Busy

WDB = Waiting Ack And Device Busy

WRB = Waiting Ack And Remote Busy

WBB = Waiting Ack And Both Busy

RDB = REJ Sent and Device Busy

RRB = REJ Sent and Remote Busy

RBB = REJ Sent and Both Busy

RX       is the number of received frames waiting in the node

TX       is the number of frames waiting to be transmitted

Tr       is the number of retries

SRTT     is the smoothed Round Trip Timer

RxB      is number of bytes received

TxB      is number of bytes transmitted

Baud     is effective baud rate computed from RxB, TxB and ConTime

ConTime  is the connect time

Pri      is the DAMA priority (0 = highest priority)

The level 4 table looks like :

L4 - User:

Call     Node             S  Rx Tx Tr Win  RxB   TxB  Baud  ConTime

--------------------------------------------------------------------

DF9KT-1   CORN  :ON5PL  IXF  0  0  0  18     10   1440   7  0:26:54

ON4DXB    D0    :ON5ZS  IXF  0  0  0  18  57475  54420  18 13:14:22

PA3EZH    ESA70 :PI8ESA IXF  0  0  0  10      9   1204  26  0:06:09

PI8HWB-6        :PI8HWB IXF  0  0  0  10   7744   7170 101  0:19:31

PI8HWB-6  SAB   :ON1BWP IXF  0  2  0  12   3258  15468 133  0:18:40

SP6STY    D0    :ON5ZS  IXF  0  0  0  18     24   4224  30  0:18:36

PI8DXH          :PI8HWB IXF  0  2  0  10    374   8584  34  0:34:19

DL1RSL-14 D0    :ON5ZS  IXF  0  1  0  18     10    624  59  0:01:25

ON4AHQ    LGE   :ON5VL  IXF  0  0  0  18    307  29414  15  4:20:15

PA3EZH    ON-lx :LX0PAC IXF  0  0  0  18    373      0   7  0:04:51

DG5NEX    D0    :ON5ZS  IXF  0  0  0  18     10   4436   3  3:14:13

LX2PA     D0    :ON5ZS  IXF  0  1  0  18     16    588  25  0:50:18

ON6GB-14  CLR   :ON7AN  IXF  0  0  0  12     15   8172  15  1:08:32

ON4TH     CLR   :ON7AN  IXF  0  0  0  12     36   9288  27  0:44:33

ON7RC-5   CLR   :ON7AN  IXF  0  0  0  12 118602  34086 353  0:57:34

where:

Call     is  the users call

Node     is the node call where the user is connected to

S        is the L4 status with :

SET = circuit setup

IXF = info transfer

DRQ = disconnect request

Rx       is the number of received frames waiting in the circular

buffer

Tx       is the number of transmitted frames waiting in the circular

buffer

Tr       is the number of transport retries

Win      is the transport window size

RxB      is the number of received bytes since the begin of the

connection

TxB      is the number of transmitted bytes since the begin of the

connection

Baud     is the effective baudrate

ConTime  is the connection time

Version

Returns the actual version number and the date and time of the last

compilation, for example :

BRU:ON7RC> TheNetNode (PC), Version 1.22d-1a-on5zs, (Mar 05 1992,

11:16:50)

ON7PC de ON7RC (21:26)>

6. CONNECTION OF SERVERS TO THE NODE :

=======================================

Connecting a BBS, a TCP/IP or a DX-Cluster in the node : If you want to

add those kind of servers, here is how we did :

further to the token ring

|

+---------------+   +------------+           +------------+   |

| BBS type      |   |            |TD--->---RD|            |->-+

| F6FBB     COM1|---| TNC2 with  |RTS-->--DCD| TNC2 with  |

| or TheBox     |   | TF18 eprom |DCD--<--RTS| KISS eprom |

| call1         |   |            |RD---<---TD|            |-<-+

+---------------+   +------------+           +------------+   |

|

+---------------+   +------------+           +------------+   |

|               |   | TNC2  with |TD--->---RD|            |->-+

| TCP/IP    COM1|---| WA8DED epm |RTS-->--DCD| TNC2 with  |

| call2         |   |            |DCD--<--RTS| KISS eprom |

|               |   |   (KISS)   |RD---<---TD|            |-<-+

+---------------+   +------------+           +------------+   |

|

+--------------------------------+                            |

|               +----------------|           +------------+   |

|               | DRSI card with |TD--->---RD|            |->-+

| DX-Cluster    | with modified  |RTS-->--DCD| TNC2 with  |

| call3         | RS232 to TTL   |DCD--<--RTS| KISS eprom |

|               | levels         |RD---<---TD|            |-<-+

|               +----------------|           +------------+   |

+--------------------------------+                            |

^           ^

|           |       from token ring

(#)         (#)

(#) these are the normally 'radio ports' (also called 'HDLC') without

modem and where connections are on TTL level, preferably on NRZI code,

at a lower speed than the token ring.

Moreover, the TD signal was "muted", in other words the signal appear

only when the corresponding RTS is high. This was done, on the Eisch

TNC, by using one of the free gate of the 74HC32 ( OR gate ). One input

is connected to Pin 6 of the Modem connector and the other input was

connected to the SIO pin 17 (RTSA not).

On the Z80 SIO itself, RTSA not (pin 17) and CTSA not (pin 18) are

connected together, this signal is referred as "RTS" on the schematic

here above. The DCDA not (pin 19) is connected to a 4k7 pullup resistor,

and this signal is refereed as "DCD" on the schematic here above.

Remember that your KISS TNC needs a clock frequency of 6 MHz to run 9600

Bd.

Remember also that these are TTL levels, and to avoid damage, insert a

resistor in series (about 470 ohms) with if each TTL input and do NOT

connect TTL signals with RS232 signals ... please be careful !!

The corresponding TNC2 KISS and the parameters should be set at :

- full duplex,

- persistence = 255,

- slot time = 0,

- TXdelay = 0,

- T2 timer = 0.

Call1, call2 and call3 should be different !

+-------------------------------------------------------------------+

|  The original version of TNN149 have still a bug : you cannot use |

|      the same call and a different SSID for the mailbox !         |

|                                                                   |

|    One provisional solution is to connect the BBS on port 0,      |

|    but then you CAN NOT follow the fundamental purpose to use     |

|                the port 0 as user default port !                  |

|                                                                   |

|  The version 1.48a revised by ON5ZS doesn't have this bug !       |

+-------------------------------------------------------------------+

7. ROUND TRIP TIMER :

=====================

A few words about this new technique, that replace the FRACK by a

dynamic value. This technique is also used in the KA9Q TCP/IP package

...

The RTT (Round-Trip-Time or RTT is the time between BEGIN of the last

Info frame and the END of the acknowledge.

+--------------------------------------+

| txd | frame #1 | frame #2 | frame #3 |

+--------------------------------------+

+-----------+

| txd | ACK |

+-----------+

<--------- Round Trip Time --------->

To avoid large variation of this parameter, a smoothed value is used

(SRTT) and is computed from :

SRTT' = (ALPHA1 * SRTT + RTT) / (ALPHA1 + 1)    for decreasing RTT

or SRTT' = (ALPHA2 * SRTT + RTT) / (ALPHA2 + 1)    for increasing RTT

For increasing values RTT is quickly adapted, but for decreasing  values

RTT is slowly adapted. Proposed values are Alpha 1 = 7, Alpha 2 = 15.

During computing only plausible value are used, TNN consider a value

'plausible' if it is  less than (RTT *10) and greater than (RTT/10).

It is interesting to make some simulation

a) first suppose we start from a stable situation where RTT = SRTT = 100

and suppose  that the measured RTT is 30, and stays 30 for the next

packets, the following table show SRTT vs. the number of packets and vs.

the value of the parameter alpha :

packet           0    1    2    3    4    5    6    7    8    9   10

measured RTT :  100   30   30   30   30   30   30   30   30   30   30

---------------------------------------------------------------------

Alpha = 0    |  100   30   30   30   30   30   30   30   30   30   30

1    |  100   65   48   39   34   32   31   31   30   30   30

2    |  100   77   61   51   44   39   36   34   33   32   31

3    |  100   82   69   60   52   47   42   39   37   35   34

4    |  100   86   75   66   59   53   48   45   42   39   38

5    |  100   88   79   71   64   58   53   50   46   44   41

6    |  100   90   81   74   68   62   58   54   50   47   45

7    |  100   91   84   77   71   66   61   57   54   51   48

8    |  100   92   85   79   74   69   65   61   57   54   52

9    |  100   93   87   81   76   71   67   63   60   57   54

10    |  100   94   88   83   78   73   70   66   63   60   57

11    |  100   94   89   84   79   75   72   68   65   62   59

12    |  100   95   90   85   81   77   73   70   67   64   61

13    |  100   95   90   86   82   78   75   72   69   66   63

14    |  100   95   91   87   83   80   76   73   70   68   65

15    |  100   96   92   88   84   81   78   75   72   69   67

16    |  100   96   92   88   85   82   79   76   73   71   68

17    |  100   96   92   89   86   83   80   77   74   72   70

18    |  100   96   93   90   86   83   81   78   75   73   71

19    |  100   96   93   90   87   84   81   79   76   74   72

20    |  100   97   93   90   88   85   82   80   77   75   73

We chose a starting SRTT value of 100, to make it easier to have an idea

in '%'.

As you can see if Alpha = 0 then the SRTT follows the RTT 'exactly',

when alpha is high the adaptation is slow, and values of Alpha greater

than 20 seems really too slow. The suggested values of Alpha1 = 7 (used

for decreasing values) and Alpha 2 =15 (for increasing values) seems

really good values ...

b) now suppose a stable situation where SRTT = 100, and suddenly the

RTT is 200, but the RTT returns to 100 just after that :

packet           0    1    2    3    4    5    6    7    8    9   10

measured RTT :  100  200  100  100  100  100  100  100  100  100  100

---------------------------------------------------------------------

Alpha =  0   |  100  200  100  100  100  100  100  100  100  100  100

1   |  100  150  125  112  106  103  102  101  100  100  100

2   |  100  133  122  115  110  107  104  103  102  101  101

3   |  100  125  119  114  111  108  106  104  103  103  102

4   |  100  120  116  113  110  108  107  105  104  103  103

5   |  100  117  114  112  110  108  107  106  105  104  103

6   |  100  114  112  110  109  108  107  106  105  104  104

7   |  100  112  111  110  108  107  106  106  105  104  104

8   |  100  111  110  109  108  107  106  105  105  104  104

9   |  100  110  109  108  107  107  106  105  105  104  104

10   |  100  109  108  108  107  106  106  105  105  104  104

11   |  100  108  108  107  106  106  105  105  105  104  104

12   |  100  108  107  107  106  106  105  105  104  104  104

13   |  100  107  107  106  106  105  105  105  104  104  104

14   |  100  107  106  106  105  105  105  104  104  104  104

15   |  100  106  106  105  105  105  105  104  104  104  103

16   |  100  106  106  105  105  105  104  104  104  104  103

17   |  100  106  105  105  105  104  104  104  104  104  103

18   |  100  105  105  105  104  104  104  104  104  103  103

19   |  100  105  105  105  104  104  104  104  103  103  103

20   |  100  105  105  104  104  104  104  104  103  103  103

As you can see even a modification of 200 % has just a small influence

on the two next packets.

When the node starts a connection there is no RTT value, and then the

node uses another parameter called IRTT as "initial" RTT . It is

suggested to use a IRTT as close as possible of the mean probable value.

If the IRTT is so low that the partner has no time to reply with an ACK,

the node will start an endless series of retries. And if IRTT is too

high the connection would appear as to be 'dead' ! As you can see on the

figure here above the minimum IRTT can be computed, the IRTT should be :

the duration of one I frame          (274 bytes * 8 bits ) / baud rate

+ T2

+ delay due to PPERSIST/SSLOTIME

+ ack frame                          (24 bytes * 8 bits) / baud rate

In TNN 1.49-on5zs released after 25/12/92 the computing can be done

automatically by the node, after you have introduced all the parameters

(TXDelay, Persist, Slottime, baud rate on the radio port, baud rate on

the token ring, dama or not and T2) simply type 'AUTO <port_number>' and

the IRTT will be adjusted.

The ALPHA parameter allows to adapt the SRTT in function of the actual

RTT.

The retry timer (FRACK) is computed from SRTT :

T1 = (BETA + (BACKOFF * Tries)/10) * SRTT  * k

BETA  is another adjustable parameter. The number of TRIES is the number

of retries till this frame. BACKOFF is a constant  and Tries is the

number of retries.

k is in function of the number of users.

For testing purpose a new parameter L3-RTT-Int is used, and it gives the

RTT of the link in msec as a reply to the "N" command. And L2-SRTT (in

msec) is shown with the "R" command.

8. BETA and FRACK DYNAMIC :

===========================

In the old formula T1 = SRTT * BETA . In a situation where both station

transmit I frames with ACK, the BETA  must be at least 2, otherwise

there IS NO CHANCE to acknowledge the frames and the node can go into an

endless RETRIES state.

A BETA of 3 is in most case too high, that's why the version of ON5ZS

use 2.5 . In order to use a  decimal value, the units have been changed

and BETA is expressed in "x 10" .

+-------------------+                             +------------+

| txd | I #1 | I #2 |                             | txd | I #3 |

+-------------------+                             +------------+

+------------+

| txd | I #5 |

+------------+

<-- Round Trip Time --->

<----- Round Trip Time ----->

In order to use a FRACK that is higher and higher as the function of the

number of retries increase, a new formula has been adopted :

T1 = (SRTT * 2) + (SRTT * (BetaMax - 2) * TRIES) /N2

|        New formula       |       Old formula

|        BetaMax = 5       |        Beta = 3

|     T1   cumulated time  |    T1    cumulated time

-----------+--------------------------+-------------------------

retry #1   |    500       500         |    750       750

retry #2   |    575      1075         |    750      1500

retry #3   |    650      1725         |    750      2250

retry #4   |    725      2450         |    750      3000

retry #5   |    800      3250         |    750      3750

retry #6   |    875      4125         |    750      4500

retry #7   |    950      5075         |    750      5250

retry #8   |   1025      6100         |    750      6000

retry #9   |   1100      7500         |    750      6750

retry #10  |   1175      8375         |    750      7500

This prevents that the node races.

9. T2 DYNAMIC :

===============

The flux is generally controlled by RNR and RR, but it is also possible

to make T2 dynamic in order to do the same.

On the link where there is a RMNC partner or where there is a BBS, the T2

dynamic method will be useful.

If the number of frames in the buffer is greater than 2, then T2 is

computed from :

T2 = T2 + ( 15 * T2 * Nr_of_frames_in_buffer / NoAckBuffer)

If you partner is a NET/ROM, TheNet , TheNetNode or G8BPQ node, then

this method is obvious because the flux is controlled by the L3.

Have fun with TheNetNode and hope you enjoy it as much as we do !

+-----------------------------------------------------------------+

| NOTE FROM ON7PC :                                               |

|                                                                 |

|   This is a "beta" version and if you find some errors, please  |

|   drop me a message ...                                         |

+-----------------------------------------------------------------+

73 de Pierre ON7PC @ ON7RC

-------------------------------------------------------------------------