EZSSTV.DOC  --  November 1994

-----------------------------

Amateur SSTV has been around for decades but most people were scared

away because it was so expensive and complicated.  It isn't anymore.

Many hams are now discovering this interesting mode and many old-timers

from the early days are returning.

In the 1980's it was necessary to spend well over a kilobuck to

buy special equipment dedicated to SSTV.  In the last couple years

several PC-based systems have appeared.  These all use fairly simple

interfaces to get audio in and out of the computer and do most of

the work in software.

This EZ SSTV software will allow you to receive the two currently most

popular SSTV modes.  The nets often use Robot 36 color because it is

reasonably quick and universally available. During more leisurely times,

Scottie S1 seems to be the favorite for its much better image quality.

What you need to watch SSTV

---------------------------

* This software.

* An IBM PC or compatible with at least:

- '386 or better CPU. (Might be able to sneak by with '286.)

- 640K memory.

- Color VGA display.

- Spare serial port (COM1 or COM2) for the interface.

- A mouse is handy but not necessary.

* A cheap and simple interface described below.  If you

already have a receive interface for one of these it

should work fine:

- HAMCOM.

- VESTER (see note below).

- JVFAX (single op-amp version).

- FAXCAP.

- SSC PC SSTV.

- SSC PC HF Fascimile.

- AEA FAX.

This software will also work with the AEA PK-900.  See notes

toward end of file for details.

The Interface

-------------

A well stocked junk box probably has all of these.  Otherwise, you can

get everything at your local Radio Shack store. The resistor and capacitor

values are not at all critical.  Use either a 25 or 9 pin connector

depending on the type of serial port available.

part #          $

------        -----

U1      741 or similar op amp           276-007         0.79

R1      47K resistor                    271-1342        0.39

C1      0.01 uF capacitor               272-131         0.49

J1      DB25S                           276-1548        1.99

or  DB9S connector                  276-1538        1.29

-       Shielded cable for audio         ?               ?

There are many variations on this basic circuit.  Others use diodes

for reverse voltage protection, capacitors for power filtering, and/or

transformers for breaking up ground loops.  This is trimmed to the

bare essentials but still seems to work well.

U1 - top view

(pins pointing away from you)                25    9

notch on this end           J1    name   pin  pin

and/or dot near pin 1             ----   ---  ---

+----U----+         +-----+

| 1     8 |    +----|--O  |  RTS    4    7

audio from         | |            |         |    |    |     |

receiver    -------| |---+--------| 2     7 |----+    |     |

| |   |        |         |         |     |

C1       |   +----| 3     6 |---------|--O  |  DSR    6    6

<    |    |         |         |     |

>   | +--| 4     5 |         |     |

R1  <    | |  +---------+         |     |

>   | |                      |     |

<    | +----------------------|--O  |  DTR   20    4

>   |                        |     |

|   |                        |     |

audio ground ------------+---+------------------------|--O  |  GND    7    5

+-----+

Let's see some pictures

-----------------------

Attach the interface to a serial port and to the audio output of your

receiver or transceiver.  Tune into one of the SSTV frequencies:

80 meters:      3.845   3.857   MHz

40 meters:      7.171

20 meters:      14.230  14.233

15 meters:      21.340

10 meters:      28.680

2 meters:       144.5

20 and 80 meters are generally your best bet.  There are nets every

Saturday at 15:00 and 18:00 UTC on 14.230 MHz.

Type "EZSSTV" to run the program.  You can use either the mouse or keyboard.

Mouse Operation

---------------

Click on the "Recv" button.  You will be asked which serial port the

interface is attached to.  Click on either the "COM1" or "COM2" button.

If you click on the serial port used for the mouse, the mouse will

stop working.  In this case, exit the program using the keyboard and

start over again.

Sit back and enjoy the pictures.  The receive mode will automatically

switch between Robot 36 and Scottie S1 if a valid VIS code is received.

The software also has automatic fine tuning which will track signals

up to 100 Hz off frequency.

Just try the buttons and menus to see what they do.  This is a lot

quicker and more interesting than reading detailed descriptions.

You will notice that some features are "grayed out" and disabled.  All

these features are in the full version of Pasokon TV.

Keyboard Operation

------------------

Notice that the "Recv" button has a red label.  Whenever there is a

button with a red label, the 'Enter' key can be used to select this

default button.

Press 'Enter' to start receiving.  Press "1" or "2" depending on where

the interface is located.

Keyboard equivalents:

F1      - Robot 36 second color mode.

F4      - Scottie S1 mode.

Alt-D   - Desk menu, contains only "About" item in this version.

Alt-F   - File menu, read files, quit, etc.

Alt-E   - Edit menu, modify existing images.

Alt-M   - Make menu, make test patterns.

Alt-C   - Config menu, display information and set options.

left & right arrows     - Select images along bottom of screen.

up arrow                - Full screen image display.

It is not necessary to pull down a menu to access one of its functions.

For example, you can simply press "Q" for quit instead of Alt-F then "Q".

VESTER SSTV Interface

---------------------

A minor change is required to use the VESTER SSTV interface with the

EZ SSTV software.  This modification should not affect operation of

the interface with the VESTER software.

Simply add a 1N914 or similar diode:

* Cathode end (bar) to U1, pin 7.

* Anode end to RTS pin of J1.

(This is pin 4 of a 25 pin connector.)

(This is pin 7 of a 9 pin connector.)

AEA PK-900

----------

This multimode TNC is capable of receiving WEFAX and SSTV.  You can use

use it for SSTV reception by following these steps:

1. Connect transceiver to radio port 1, which is where you'd

normally connect an HF transceiver.  Even if using a VHF radio

still connect it to radio port 1.

2. Connect the PK-200 to a serial port on your computer in the

usual way.

3. Run a terminal program.  Send these commands to the PK-900:

radio 1/0

analog

It should now be in the proper mode to demodulate the SSTV

signals.

4. Exit out of the terminal program and run EZSSTV.  Follow usual

instructions above.

When finished with SSTV operation, perform a hardware reset on the PK-900

to remove all remnants of the "analog" mode.  It has been reported

that some PK-900 control programs don't work properly after the TNC

has been in "analog" mode.  The solution is a hardware reset on the

PK-900; a software reset doesn't seem to be adequate.

For more information on SSTV

----------------------------

See the January 1993 issue of QST or the SSTV section of the 1995 ARRL

Handbook for Radio Amateurs or contact:

John Langner WB2OSZ

115 Stedman St. # E

Chelmsford, MA 01824-1823

U.S.A.

telephone: (508) 256 6907

Internet e-mail: johnl@world.std.com