WHATS-UP.DOC Release 1.00 Page 1

_______

____|__     |                  (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

WHATS-UP (c) Joe Kasser, G3ZCZ, 1990-1991

Joe Kasser G3ZCZ/W3

POB 3419

Silver Spring, Md. 20918

Voice (301) 593 6136

Data BBS (301) 593 9067

Compuserve (CIS) 70531,1405

WHATS-UP  is a tool which allows you to capture, decode,  display

and  extract for analysis telemetry from the  following  Orbiting

Satellites Carrying Amateur Radio (OSCAR).

UoSAT-OSCAR 11  1200 baud ASCII Telemetry,

AMSAT-OSCAR 13  50 baud RTTY Z blocks Telemetry,

AMSAT-OSCAR 16, WEBER-OSCAR  18 and LUSAT-OSCAR 19  Binary  Tele-

metry,

DOVE-OSCAR 17   ASCII TLM packet Telemetry,

Fuji-OSCAR 20   Status and Telemetry.

WHATS-UP  will also allow you to process any telemetry  data  you

have  from  the  now reentered UoSAT-OSCAR 9  and  Fuji-OSCAR  12

spacecraft.

WHATS-UP is a table driven program which uses configuration files

to  allow  maximum  flexibility.  If you have  a  Kenwood  Radio,

WHATS-UP  will  tune the Radio's VFO A to the  spacecraft  beacon

frequency (plus a presettable  offset) and  set the receive  mode

for the satellite of interest.  If  the Radio responds correctly,

WHATS-UP will allow you to read back the  VFO  A frequency during

the  pass and capture the data to a Doppler measurement file  for

later import to a spreadsheet.

This  version of the program is configured for a PK-232. It  will

intercept  the  Microsat  Binary telemetry  from  AMSAT-OSCAR  16

(PACSAT),  WEBER-OSCAR 18 (WEBERSAT) and LUSAT-OSCAR  19  (LUSAT)

and display and process them as if they were DOVE-OSCAR 17  ASCII

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 2

telemetry. NOTE: AMSAT have STILL NOT FINALIZED or PUBLISHED  the

Binary Telemetry Format for the Microsats.

The program is distributed as a Shareware product. You may freely

copy  and  share  the product for noncommercial  use,  with  your

friends, classmates, associates and radio hams. If you decide  to

use  the  product, you are asked to become a registered  user  by

completing  the  registration  form and sending  it,  and  $35.00

(drawn on a US Bank please) or equivalent in foreign currency  to

the author.

Upon  receipt  of your registration, you will  receive  one  free

update disk, telephone and mail (electronic and regular) support.

Please  do not use packet radio mail for commercial messages,  or

even  those  that may be construed as such by  individual  SYSOPS

through whose BBSs the messages may pass.

This product may not be sold or distributed with another  product

without the express written permission of Joe Kasser, G3ZCZ.  Joe

Kasser,  G3ZCZ  will  only  support  unmodified  copies  of  this

software.  Your comments and suggestions for changes are  however

welcome. If you are the first to suggest a change that is  imple-

mented,  you will be sent a complimentary copy of the  disk  with

the change incorporated.

European Radio Amateur users may register (24.00 Pounds Sterling,

includes VAT) and obtain support from Terry Dansey at  ReadyCrest

Ltd.,  PO  Box 75, Chatham, Kent, ME5 9DL,  England.   ReadyCrest

Ltd.   accepts  credit  cards  (Access,  Visa,   MasterCard   and

Eurocard).  Telephones:  Voice 44 (0)634-687168, FAX  44  (0)634-

687178, Data (BBS) 44 (0)634-200931.

Potential  Commercial  and Educational Institution  Users  please

contact  Joe Kasser directly for modifications and/or details  of

Site licensing.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 3

Table of Contents

1.0 Introduction

1.1 Capabilities of WHATS-UP

1.2 Program Requirements

1.3 Packet Link Quality Measurements

1.4 Copying Non-packet Telemetry

1.5 Obtaining Updates

1.6 Limit Checking

2.0 Customization

2.1 Bringing WHATS-UP up for the First Time.

2.2 Editing the Configuration File.

2.3 Starting the program.

2.4 Screen Areas

2.5 Setting Up Customized Display Pages

3.0 Modes

3.1 Standby Mode

3.2 Interactive Mode

3.3 Real Time Mode

3.4 Orbital Dynamics Mode

3.5 Playback Mode

3.6 (Data) Extraction Mode

3.7 Audio Warnings and Orbit Data Displays

4.0 Menus

4.1 Function Keys

4.2 Modes menu

4.2.1 Extract From Playback File

4.2.2 Interactive Mode

4.2.3 Orbital Parameters

4.2.4 Playback Mode

4.2.5 Real Time Mode

4.2.6 Selections Menu

4.2.7 Exit to Dos

4.3 Selections Menu

4.3.1 Change Display Page

4.3.2 Edit Menu

4.3.3 Files Menu

4.3.4 Help Menu

4.3.5 Jump to DOS

4.3.6 Modes Menu

4.3.7 Orbits Menu

4.3.8 Radio Menu

4.3.9 Spacecraft Menu

4.3.10 PK232 Menu

4.3.11 Utilities Menu

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 4

4.3.12 Exit to DOS

4.4 Edit menu

4.4.1 Edit Spacecraft Doppler File

4.4.2 Edit Any File

4.4.3 Edit Orbital Data File

4.4.4 Edit Spacecraft Configuration File

4.4.5 Pick Capture-to-disk File

4.4.6 Edit Capture-to-disk File

4.4.7 Edit WHATS-UP.SYS

4.4.8 Edit Two Files

4.4.9 Edit Doppler Channel File

4.4.10 Edit Doppler Data File

4.4.11 Pick Spacecraft Configuration .SYS File

4.5 Files Menu

4.5.1 Change Directory Path

4.5.2 Change Playback File

4.5.3 View Playback File

4.5.4 Show Capture-to-disk Files

4.6 Orbits Menu

4.6.1 Pick AMSAT Format Element Set

4.6.2 Edit WHATS-UP.2LN

4.6.3 Load Element File

4.6.4 Pick NASA 2 Line Element Set

4.6.5 View Spacecraft Orbit Elements

4.7 Radio Menu

4.7.1 Turn Doppler Tracking On/Off

4.7.2 Change Doppler Interval

4.8 Spacecraft Menu

4.8.1 picK Spacecraft

4.8.2 Change Spacecraft

4.8.3 Pick Ops. Schedule

4.8.4 Show Ops. Schedule

4.9 TNC Menu

4.9.1 UoSAT ASCII Beacon

4.9.2 Phase 3 RTTY Beacon

4.9.3 Set Morse Code (CW)

4.9.4 Fuji/MicroSat ASCII Packet

4.9.5 Configure PK232

4.10 Utilities menu

4.10.1 Change Directory Path

4.10.2 Change Beacon Frequency

4.10.3 Show Space on Disk

4.10.4 Reset Header Counters

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 5

4.10.5 Show Defaults

4.10.6 Set PC Clock

4.10.7 Show Files

4.10.8 Show Color Chart

4.10.9 Reconfigure WHATS-UP

4.10.10 Sound CW String

5.0 Orbital Elements

5.1 Basics

5.2 Orbital Trajectories

5.3 Types of orbits

5.4 Orbital Elements

5.5 Angle of Inclination.

5.6 Right Ascension of Ascending Node (RAAN).

5.7 Eccentricity and Semimajor Axis

5.8 Decay Rate

5.9 Argument of Perigee.

5.10 Epoch Time (of Ascending Node) and Revolution Number.

5.11 The Mean Motion.

5.12 The Catalog Number

5.13 Mean Anomaly

5.14 Anticipated Spacecraft Lifetimes

6.0 The Spacecraft

6.1 Receiving system components

6.1.1 Antennas

6.1.2 Receivers

6.1.3 Terminal Units or Modems

6.2 Receiving Signals from DOVE

6.3 Receiving Signals from UoSAT-OSCAR 11

6.4 Receiving Signals from AMSAT-OSCAR 13

6.5 Receiving PSK Modulated Signals in the 70 cm Band

7.0 Decoding Active Spacecraft Telemetry

7.1 DOVE-OSCAR 17

7.2 UoSAT-OSCAR 11

7.3 AMSAT-OSCAR 13

7.4 AMSAT-OSCAR 16, WEBER-OSCAR 18 and LUSAT-OSCAR 19

7.5 Fuji-OSCAR 20

7.6 AMSAT-OSCAR 21

8.0 Spacecraft No Longer Active

8.1 Fuji-OSCAR 12

9.0 File formats

9.1 Configuration File

9.1.1 Your callsign

9.1.2 Default spacecraft Name

9.1.3 Station Latitude

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 6

9.1.4 Station longitude

9.1.5 Station Altitude

9.1.6 Station minimum antenna elevation for acquisition

9.1.7 Station minimum usable pass time

9.1.8 Early warning time

9.1.9 Default Kepler file

9.1.10 UTC offset

9.1.11 Default directory path

9.1.12 Default extracted data file

9.1.13 Default   file  name  with  list  of   telemetry

parameters to extract file

9.1.14 TNC Type

9.1.15 Serial port to TNC

9.1.16 PC serial port to Radio

9.1.17 PC serial baud rate

9.1.18 PC port data bits

9.1.19 PC port Stop bits

9.1.20 PC Port parity bits

9.1.21 status (top) window color

9.1.22 Telemetry Page color

9.1.23 outgoing window color

9.1.24 incoming window color

9.1.25 prompt window color

9.1.26 Alarm window color

9.1.27 Bottom window color

9.1.28 Emphasis color

9.1.29 Prompt color

9.1.30 Option color

9.1.31 Parameter changed color

9.1.32 Parameter limit exceeded color

9.1.33 Orbit element window color

9.1.34 Orbit  element  window Orbit element  window  In

range color

9.1.35 Orbit element window early warning color

9.1.36 Orbit element window next one up color

9.1.37 Orbit alert dit time

9.1.38 Orbit alert note

9.1.39 Flag Sound

9.1.40 Doppler display Flag

9.1.41 Spacecraft Configuration File Linkages

9.1.42  * Comment line

9.1.43 The remaining lines

9.2 Spacecraft Parameter Files

9.2.1 Spacecraft ID.

9.2.2 Spacecraft Suffix

9.2.3 Beacon Frequency

9.2.4 Doppler Measurement File

9.2.5 Spacecraft  Identification in  Keplerian  Element

File

9.2.6 Doppler Measurement Sample Interval

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 7

9.2.7 Initial Frequency Offset

9.2.8 TNC configuration Code

9.2.9 Modulation

9.2.10 Data Type

9.2.10 Receiver Type

9.2.11 Selected or default display page number

9.2.12 Page Definitions

9.2.13 Telemetry Parameter Configuration

9.2.13.1 TLM_Channel

9.2.13.2 TLM_Segment_ID

9.2.13.3 TLM_Description

9.2.13.4 TLM_Eqn_Type

9.2.13.5 TLM_Ceof_C

9.2.13.6 TLM_Ceof_B

9.2.13.7 TLM_Ceof_A

9.2.13.8 TLM_Units

9.2.13.9 TLM_Page

9.2.13.10 TLM_Row

9.2.13.11 TLM_Col

9.2.13.12 TLM_Width

9.2.13.13 TLM_Dec

9.2.13.14 TLM_Limit_Check

9.2.13.15 TLM_Limit_Low

9.2.13.17 TLM_Limit_High

9.2.13.18 TLM_Negative_Blank

9.2.13.99 The Telemetry Identifier Line

9.2.13.99.1 The Line Identifier

9.2.13.99.2 The Segment Identifier

9.2.13.99.3 Spare/Not Used

9.2.13.99.4 Location of Segment Identifier

in Line

9.2.13.99.5 Spare/Not Used

9.2.13.99.6 Spare/Not Used

9.2.13.99.7 Spare/Not Used

9.2.13.99.8 Segment Identifier String

9.2.13.99.8.1 Fuji

9.2.13.99.8.2 DOVE

9.2.13.99.9 Spare

9.2.13.99.10 Number of Lines

9.2.13.99.11 First Line

9.2.13.99.12 Last Line

9.2.13.99.13 Header Lines

9.2.13.99.14 Header Display Flag

9.2.14 Digital Telemetry Status Channels

9.2.14.1 Status_Channel

9.2.14.2 Status_Display_Page

9.2.14.3 Status_Text

9.2.14.4 Status_Bit_Mask

9.2.14.5 Status_Row

9.2.14.6 Status_Col

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 8

9.2.14.7 Status_ON_Text

9.2.14.8 Status_OFF_Text

9.2.14.9 Status_ON_Color

9.2.14.10 Status_OFF_Color

9.2.15 Packet/Link Parameters

9.2.15.1 Packet_title

9.2.15.2 Packet_Type

9.2.15.3 Packet_Lines

9.2.15.4 Packet_Page

9.2.15.5 Packet_Color

9.2.15.6 Packet_Row

9.2.15.7 Packet_Col

9.2.15.8 Link_Page

9.2.15.9 Link_Row

9.2.15.10 Link_Col

9.2.15.11 Binary_Byte_Count

9.3 Telemetry Channel Extraction File

9.4 Extracted Telemetry Data File

9.5 Doppler Data File

9.5.1 The Time

9.5.2 The Doppler Mark

9.5.3 The Frequency

9.5.4 The Doppler Shift

9.5.5 The Measured Shift

9.6 Kepler Element Files (*.2LN)

9.7 AMSAT Format Element File (*.AMS)

9.8 Spacecraft Operations File

10.0 Glossary

11.0 References and Further Reading

APPENDIX

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 9

1.0 Introduction

WHATS-UP  is a tool which provides you the Radio Amateur, or  the

Educator,   with  the  capability  to  perform   experiments   in

spacecraft orbital dynamics as well as monitoring the environment

onboard several of the Orbiting Satellites Carrying Amateur Radio

(OSCAR) during individual passes or over long periods of time.

Spacecraft  in  the OSCAR series send back volumes  of  Telemetry

daily  and apart from the odd Command station, few if  any  Radio

Amateurs  or Educators seem to be doing anything with  it.  Radio

Amateurs  tend to concentrate on the communications  capabilities

of the spacecraft and ignore their telemetry completely. If  they

do  listen  to  a beacon, it's usually just  to  check  that  the

transponder is on, heaven forbid - to actually copy any data.

The  telemetry  can  tell  us a story. It can  tell  us  what  is

happening to both the spacecraft and its environment. As such  it

has  a tremendous educational potential which has  remained  just

that - a potential for at least the last six years.

Before  every satellite launch the equations and format  for  the

spacecraft telemetry are published by AMSAT. The telemetry  tells

us  about  the health and welfare of the spacecraft  itself,  and

something  about  the  payload.  Spacecraft  health  and  welfare

information tells us about the battery, solar cells and on  board

computer  status. Payload information can range from  information

about  transponder loading/utilization to data  from  instruments

that  measure  the  environment of the space in  and  around  the

satellite.  Battery Telemetry is used by the command stations  to

determine  when  the  spacecraft  can  be  used,  and  when   the

transponders  should be shut down. The number of individuals  not

associated  with  command stations who  have  decoded  spacecraft

telemetry  and published their findings can be counted using  the

fingers  of one hand. There's a lot of computing power out  there

that  has  the  potential  to  process  telemetry  and   discover

something new, but does not have the access to the data. WHATS-UP

is  an attempt to provide that data to the average Radio  Amateur

and Educator.

Capturing,  decoding  and  displaying  telemetry  from   orbiting

spacecraft in real time, in the classroom, is an excellent way of

introducing  space  science  to  students.   Signals  from  these

spacecraft are downlinked on frequencies that can be received  on

regular vhf/uhf scanner radio receivers.

WHATS-UP  provides  an introduction as to how this  can  be  done

using  readily  available  low cost  equipment.   General  topics

discussed    cover   telemetry,   the   spacecraft    themselves.

Groundstation  hardware topics include receiving antennas,  radio

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 10

receivers  and  modems.  Software topics  discussed  include  the

software  used to track the spacecraft and the software  used  to

both decode and display the data in real time as well as that for

post  pass analysis.  Excluding the Personal Computer,  a  simple

telemetry  capturing  groundstation can be set up for  less  than

$500.00 in equipment costs.

There is no substitute from the excitement of hands-on experience

in  awakening  an interest in space.  While OSCARs in  the  main,

transmit  down to the ground (downlink) telemetry in morse  code,

several  of them also utilize standard computer literate  digital

data  schemes  such  as BAUDOT or ASCII  codes.   The  thrill  of

receiving  a  signal from space soon fades however  if  the  data

cannot  be  understood.  Even after the data  has  been  decoded,

watching  the  temperatures on-board a spacecraft  as  it  passes

overhead  is  also  of little interest, but,  what  can  be  made

interesting is receiving and capturing the data over many days or

even months and looking for trends and relationships.

You  can capture telemetry just by listening to a spacecraft  and

copying  morse  code by ear writing it down with a  pencil  on  a

piece  of  paper.  All you need to do is listen  on  the  correct

frequency  at  the  right time and you will  hear  some  signals.

Write  down  what you hear.  Apart from the thrill of  copying  a

signal  from  a satellite, there's not much else to do  with  the

data.  While you can look up the conversion equations and  get  a

snapshot  of what is happening, it soon becomes tedious and  only

dedicated souls do it regularly.

When  AMSAT  were  building the  Phase  3A  spacecraft,  personal

computing was in its infancy.  That was before the Apple 2 or the

TRS-80; that was the day of the home built S-100 machine, and the

AMSAT-GOLEM-80  Project.  At that time, Karl Meinzer, DJ4ZC,  had

written a native German high level computer language called  IPS.

This  language was very similar to Forth, but contained a  multi-

tasking kernel. This language was programmed into the  spacecraft

flight  computer  and also used in the ground  command  stations.

Using  a  computer  on the spacecraft allowed  AMSAT  to  provide

telemetry  that  could  be captured by computer  on  the  ground.

While  Phase  3A  never made it into orbit  (the  launch  vehicle

malfunctioned),  subsequent spacecraft continued to  provide  the

same facility.  Today UoSAT-OSCAR 11 (UO-11), AMSAT-OSCAR 13 (AO-

13),  UoSAT-OSCAR 14 (UO-14), AMSAT-OSCAR 16 (AO-16),  DOVE-OSCAR

17  (DO-17), WEBER-OSCAR 19 (WO-18), LUSAT-OSCAR 19  (LO-19)  and

Fuji-OSCAR 20 (FO-20) are all sending back telemetry that you can

capture  with your PC and amateur radio equipment.  As  most  non

radio   amateurs   do  not  understand   morse   code,   WHATS-UP

concentrates   on  those  spacecraft  which   downlink   computer

compatible  telemetry and can be received with relatively  simple

equipment.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 11

1.2 Capabilities of WHATS-UP

WHATS-UP contains the following features:

* Display of spacecraft orbital elements and tracking data.

* Will automatically set your Kenwood receiver to spacecraft

beacon  frequency  when the spacecraft  comes  above  your

horizon.

* Audio  warning of spacecraft acquisition of  signal  (AOS)

and loss of signal (LOS).

* Real-time, Interactive and Playback modes.

* Automatic Capture-to-disk of raw telemetry.

* Extracts   telemetry  channel  data  to  a   database   or

spreadsheet readable file for further analysis.

* Link quality measurement.

* Capability to display and print the raw telemetry as it is

received.

* Up  to 16 user configurable display pages  (screens).  You

set  the position on the page (width of  engineering  unit

field,  and number of decimal places) that a parameter  is

displayed at.

* Wild card page (parameter shows up on all pages).

* Selectable  display of Engineering units or Hex  byte  for

each display page.

* Display of raw packets (i.e. STATUS)

* Color  changes  if  a  parameter  value  changed   between

successive frames.

* Audio  and  visual alarms if a  telemetry  value  exceeds,

falls below or falls outside a preset limit value(s).

* Dumb  split  screen  terminal mode  with  user  selectable

number of window rows.

* Customizable colors, PC to TNC baud rate, data parity  and

stop bits.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 12

* Default spacecraft configuration files.

* UTC Time of day clock display (in HH:MM:SS format)

1.2 Program Requirements

IBM PC or clone with at least 256k memory.

A  Radio receiver and a Packet TNC with an RS-232  interface

is only required for real time data capture. This version is

configured  for the PK-232 because only the PK-232  has  the

capability   for  receiving  the  telemetry  from  all   the

spacecraft supported by the program.

The  program DOES require that the packetized  telemetry  be

received,  and captured-to-disk with the packet header on  a

different  line  to the contents of the  packet  (HEADERLINE

ON). I also suggest that you turn the date/time stamp on  so

that  you  will be able to playback your  data  and  extract

selected  values and their corresponding time codes  into  a

file  that  can be read into your  spreadsheet  program  for

further trend analysis.

This version (the default) is set up to display all  packets

as  wild  cards (i.e will show up on all  pages),  and  then

display several temperatures and solar cell array currents.

By  putting  the correct parameters  in  the  SPACECRAFT.SYS

file, you can set up any of the pages to display any of  the

telemetry channel data in any row and column in that page.

1.3 Packet Link Quality Measurements

The  packet  link quality part allows the  following  to  be

done.  You  can  define which of the  packets  you  want  to

display/count  (If  you  just want to  count  them  and  not

display  them, set the page value to 99). You can then  view

an incrementing count counter each time a selected packet is

received.  For  example, using DOVE-OSCAR 17, you  can  also

display  the contents of the STATUS, WASH or BCRXMT  packets

in any display page. This feature allows you to get an  idea

of how good your receiving system is.

1.4 Copying Non-packet Telemetry

Packet  telemetry,  by definition is error  free.  The  RTTY

blocks from AMSAT-OSCAR 13 on the other hand may be received

with  errors.  These errors may take the form of garbled  or

missing  characters. If your link is bad, and such  hits  do

occur,  you should edit the capture-to-disk file before  you

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 13

playback the telemetry and decode/display the data.

If you get bad data in real time, look at the raw data.  You

will be able to see the quality of the link.

1.5 Obtaining Updates

It  is  anticipated  that  WHATS-UP is  going  to  grow  and

incorporate  features for decoding and displaying data  from

other  spacecraft. Better Microsat binary  telemetry  decode

and  display  capability will be added when  AMSAT  announce

that  the format has stabilized, and  sufficient  registered

users express interest in having it. To stay on the  mailing

list and receive an update as it is released, register  your

copy,  then send in a disk containing at least  1  Megabytes

(zipped)  of  captured  data from  the  spacecraft  of  your

choice.  If  you  would like to  exchange  data  with  other

educational  institutions  or  users so as  to  be  able  to

analyze  more  data  than  you can get  on  a  single  pass,

indicate  that  fact and we will try and put you  in  direct

touch with others who are similarly inclined.

1.6 Limit Checking

Limits  are only checked for parameters being displayed  (in

Engineering  units).  With  this approach, you  can  set  up

different  pages for different on-board subsystems, you  can

also  set up different display pages of the same  parameters

for daylight, darkness and terminator crossing passes,  with

different limit values to draw your attention to changes.

2.0 Customization

2.1 Bringing WHATS-UP up for the First Time.

WHATS-UP  comes  configured  to use Com1 in the  PC  as  the

serial  port between the PC and the PK-232, and to use  Com2

as  the  serial port between the PC and the  Kenwood  Radio.

WHATS-UP  is also configured to assume DOVE as  the  default

spacecraft.

If  you  type 'WHATS-UP' followed by the  'Enter'  key,  the

program  will load and start to do things.  If  the  default

serial  ports  are  correct for your  system,  you  need  do

nothing further, other than change the colors of the windows

to your preference.

2.2 Editing the Configuration File.

The configuration file is called WHATS-UP.SYS. You must edit

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 14

it  using option 'S' in the Edit Menu or with an ASCII  word

processor  (in the non document mode) to set up the  correct

parameters on the RS-232 link between your TNC and your  PC.

See  Section  10 for details of what parameter is  on  which

line of the WHATS-UP.SYS file.

Before  you make any changes, make sure that you do not  try

and edit your original, always work from a copy.

These  are the minimum set up items to change in the  WHATS-

UP.SYS file.  Refer to Section 9 for fuller details of  what

items are on which lines.  Bring up the Selections menu, the

access  the  Edit Menu. choose the option  for  editing  the

WHATS-UP.SYS  file. As you enter items on the  lines  listed

below, delete the entries currently in place.

Line 1:  Enter your callsign instead of the default one. Use

Capital letters (Upper case). If you do not have  a

callsign,  pick  a mneumonic that  represents  your

school  or  organization.  You may  use  up  to  10

characters.   The  callsign entered here,  will  be

shown  at the top of the status window,  next  time

you  load WHATS-UP, and will also (more  important)

be  appended  to  your  capture-to-disk  files   to

identify  the  ground station  which  received  the

data.

Line 3:  Enter  the  latitude  of  your  location.  In   the

southern hemisphere, use a negative number.

Line 4:  Enter the longitude of your station in degrees West

of Greenwich in the range 0 to 359.9.

Line 5:  Enter  the  station  altitude above  sea  level  in

meters. If you don't know, don't change anything.

Line 10: Enter the difference between local time in your  PC

and UTC. For example, eastern standard time is 5.

Line 16: If  you  are  not using Com1  as  the  serial  port

between  the PC and the TNC, enter the port  number

on this line.

Line 17: If you have a compatible radio with RS-232  control

capability  and  are not using Com2 as  the  serial

port  between the PC and the Radio, enter the  port

number on this line.

Line 18: If  you are not using 1200 baud between the PC  and

the TNC enter the actual value on this line.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 15

Save  the file at this time. WHATS-UP will now work for  you

in  its  minimal mode. Read the manual to  learn  about  its

capabilities and use section 9 when you customize it to suit

yourself.

2.3 Starting the program.

You may start the program in three ways as follows.

2.3.1 Default

Type 'whats-up' and return (without the '  characters).

This brings the program up in the default mode. It will

read the whats-up.sys file to determine the  spacecraft

being monitored, and then prompt you for the mode.

2.3.2 User Chosen spacecraft

Type  'whats-up spacecraft' and return (without  the  '

characters). The program reads the spacecraft.sys  file

to load the parameters for the Microsat of choice,  and

then prompts you for the mode.

Examples of the command are :

'WHATS-UP DOVE' or 'WHATS-UP Fuji20'

2.3.3 Custom Mode

Type 'whats-up spacecraft mode' and return (without the

' characters). This brings the program up in the custom

mode.  It will read the whats-up.sys file to  determine

the  spacecraft being monitored, and then start  up  in

the  mode  you set. Valid modes are 'p', 'i',  'o'  and

'r'.

Examples of the command are :

'WHATS-UP DOVE R' or 'WHATS-UP Fuji20 R'

If  you  place  a  command  line  like  this  in   your

autoexec.bat  file, should you be copying telemetry  in

an  unattended manner and a power failure  take  place,

the system will boot up into the correct WHATS-UP  mode

when power is restored.

When the program loads, the first thing it does is load

the  Keplerian  data  from the default  data  file  and

compute the current position of all the spacecraft. The

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 16

following  typical  messages will be send  during  this

activity.

MIR Loaded

RS-10/11 Loaded

AO-13 Loaded

UO-14 Loaded

AO-16 Loaded

DO-17 Loaded

Computing Next Pass for AO-10

Computing Next Pass for UO-11

Computing Next Pass for MIR

Computing Next Pass for RS-10/11

Computing Next Pass for AO-13

Computing Next Pass for UO-14

Computing Next Pass for AO-16

Computing Next Pass for DO-17

If  the default Keplerian data file is not  present  on

the disk, the orbital mode will be disabled.

2.4 Screen Areas

The  screen is divided into four window areas.   The  Status

window occupies the top line. The prompt window occupies the

bottom  line.  Raw data are always shown in the  lower  data

area. This window also shows any outgoing text that you  may

type  at the keyboard or any commands WHATS-UP sends to  the

TNC.   The data area in the top half of the screen  contains

the  processed data (real time and playback), the  raw  data

(interactive) and the orbital data display.

2.5 Setting Up Customized Display Pages

The  ability  to customize a display page is a  unique  tool

provided  by  WHATS-UP.  This  tool  allows  you  to   group

telemetry information so you can see how the monitor  points

you are interested in behave, and visually see the  changes.

You can locate information in any position on the screen and

in  any color you wish. You can also perform limit  checking

on  the  data,  and  have  the  "out-of-limit"  show  up  in

different colors and generate audible alarms.

Setting  up display pages is an orderly procedure using  the

information  contained  in  Section 9.   When  you  want  to

customize  the  displays,  make  a  copy  of  the   supplied

spacecraft configuration file and work with it. Look at  the

format  of  the  file, read Section 9.2  to  sure  that  you

understand what is in the file. Use the editor to adjust the

items  in the spacecraft configuration file.  After  editing

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 17

the  file, reload the file to enter the  changed  parameters

into  WHATS-UP  to see the effects of your  changes.  Repeat

this procedure until you are satisfied.

First,  from the utilities menu, view the colors and  decide

on the color scheme for the window and data.

Lay out the display page on graph paper for a screen  window

of 13 rows by 78 columns.

Decide  on the name of the page. If it is a new  page  enter

the  name on a new line after the existing pages before  the

line  that  ends in an '*' character. Do not delete  the  '*

character.   After you have entered the page name, insert  a

comma character ',' and a number for the default page color.

An example from the DOVE.SYS file for such an entry is shown

below.

PAYLOAD TRANSMITTER/RECEIVERS, 30

2.5.1 Customizing Analog Telemetry Channel Displays.

The  next  few  lines  contain  the  analog   telemetry

decoding  display information.  Review Section  9.2  to

see  what has to be inserted in each of  the  elements.

The  analog  telemetry lines are  terminated  with  the

following  line, which must be present even if no  data

lines are present.

'* end of telemetry'

2.5.2 Customizing Status Telemetry Channel Displays.

Section 9.2.14 describes the meaning of each item.  The

status   telemetry  lines  are  terminated   with   the

following  line, which must be present even if no  data

lines are present.

'* end of status bytes'

2.5.3 Customizing Packet Header Displays.

The  packet  header  configuration  lines  contain  the

analog telemetry decoding display information.  Details

of  the  meaning  of each item  are  given  in  section

9.2.15. The packet header lines are terminated with the

following  line, which must be present even if no  data

lines are present.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 18

'*  end of packet types'

If you want to change the colors that channels out of limit,

or  channels  in which the data  changed  between  telemetry

frames are displayed in, you must change the entries in  the

WHATS-UP.SYS file.  207 is a respectable value for the color

to  display  data  for which the  limit  has  been  exceeded

(flashing white on red background).

3.0 Modes

WHATS-UP  is designed to operate in a number of modes  as  listed

below.  Each mode has its own type of window display.  To  change

things  or select features, touch the 'Esc' key to bring  up  the

top level menu.  Menus in WHATS-UP are organized as a  hierarchy,

each  option  will  take you as deep as necessary  to  make  your

selection.

3.1 Standby Mode

The  standby  Mode  presents  you with  the  Modes  Menu  as

described in Section 4.  This mode is activated if you bring

up WHATS-UP without specifying a mode in the command line.

3.2 Interactive Mode

The  Interactive mode is a dumb terminal. You can use it  to

give commands to the TNC. You should also use it to set  the

'HEADERLINE ON'. In this mode, you will see the raw  packets

on the channel. You can also use this mode as a regular  TNC

program  (If  you do, you ought to get your  head  examined,

because LAN-LINK will do the job much better). The  capture-

to-disk  will turn on when the first packet is  copied,  and

will turn off two minutes after the last.

3.3 Real Time Mode

The  Real-time mode converts and displays engineering  data.

You  can  display  up to 16 (configured  by  you)  pages  of

information.  Information  that changes  between  successive

frames, is shown in a different color. Information that  has

exceeded a preset (by you) limit is shown in an alarm  color

(default:  blinking red). The capture-to-disk will  turn  on

when  the  first  packet is copied, and will  turn  off  two

minutes after the last.

3.4 Orbital Dynamics Mode

The  orbital  dynamics  mode  gives you  a  display  of  the

positions  of the spacecraft with respect to your  location.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 19

If  the  spacecraft  are out of range,  then  you  see  time

related information. If the spacecraft are in range then you

also  see position related information.  The time  that  the

spacecraft  comes  above your horizon is  when  you  acquire

signals. This time is known as Acquisition of Signals  (AOS)

time. You lose signals when the spacecraft drops below  your

horizon.  This  time  is known as  Loss  of  Signals  (LOS).

WHATS-UP  allows you to define an early warning  time  (EWT)

(in minutes) before AOS.  The period of time between AOS and

LOS is known as the pass.

Orbit positional information is only as accurate as your  PC

clock and the reference Keplerian data supplied by AMSAT and

other sources. You can expect an accuracy of within a minute

or  two if your PC clock is correct. WHATS-UP only  provides

time  displays accurate to within a minute or two.  Its  not

worth going for greater accuracy in most cases.

The information displayed in the orbital mode window are  as

listed below.

WNDOW     Time  till AOS (if out of range, or time till  LOS

if in range.

AOS       Time  of  AOS. This item is not displayed  if  the

spacecraft is in range.

LOS       Time of LOS.

PASS      The  amount of time that the pass will  last.   If

the  spacecraft  is  in  range,  then  this   item

displays the minutes remaining till LOS.

MA        Mean Anomaly, or position of the spacecraft in its

orbit (see Section 5).

ALT       The altitude of the spacecraft above sea level. An

Up arrow next to it indicates that the altitude is

increasing, a down arrow, that it is decreasing.

The  following information is only displayed for  spacecraft

in range.

AZ        Azimuth  -  Pointing  angle  to  spacecraft  along

horizon (0 = North, 90 = East).

EL        Elevation  -  Pointing angle  to  spacecraft  from

horizon to zenith.

RANGE     Distance between your station and the  spacecraft.

An Up arrow next to it indicates that the altitude

is   increasing,   a  down  arrow,  that   it   is

decreasing.

DPLR      Doppler shift on spacecraft's beacon signal.

A typical example of the display is shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 20

UO-11    02:05 00:08 00:19 00:11  143     681

AO-13    04:51       02:54 08:01  122   37723  47.92  24.91

=DO-17    03:32 01:35 01:45 00:10  206     792

The  first column provides information about  the  automatic

sequencer.

A  check mark indicates that the spacecraft is the  selected

one for data to be captured.

An  equals  sign indicates that WHATS-UP  is  configured  to

configure  itself to tune to its beacon signal  and  capture

data at AOS for that spacecraft.

An   less  than/equals  sign  indicates  that  WHATS-UP   is

configured to configure itself to tune to its beacon  signal

and capture data at EWT for that spacecraft.

The display is color coded as follows.

Spacecraft out of range are displayed in the default  window

color.  Spacecraft in range are displayed in the 'in  range'

color. Spacecraft within EWT are is displayed up in the  EXT

color. The spacecraft designator for the next spacecraft  to

come in range is displayed in the 'next one up' color.

These  colors  may be configured to your preference  in  the

WHATS-UP.SYS file.

3.5 Playback Mode

The Playback mode allows you to play back captured telemetry

with 4 speeds (speedy, slow, slower and snail's pace).

3.6 (Data) Extraction Mode

In this mode, data is extracted from a playback file into  a

file that can be read into a spreadsheet. If you answer  the

prompt  for the default file with a  non-existent  filename,

WHATS-UP will prompt you for individual channel numbers.  To

terminate the sequence and begin the extract mode, touch the

'Enter' key without entering a channel number.

Note: Start and stop times are text string matches.

3.7 Audio Warnings and Orbit Data Displays

WHATS-UP provides audible warnings in morse code of AOS, LOS

and  EWT. Each warning consists of a letter followed by  the

spacecraft designator. An 'A' prefixes AOS, an 'L'  prefixes

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 21

LOS and a 'Q' prefixes EWT.

WHATS-UP also shows orbit data associated with the  selected

spacecraft in the bottom window. This data which  duplicates

the  line shown in the orbital mode, is there to be used  in

the realtime and interactive modes.

4.0 Menus

WHATS-UP  provides various features organized in a  hierarchy  of

menus. To bring up the top menu, touch the 'Esc' key.

4.1 Function Keys

The  following function keys are active in  the  operational

modes described in Section 3:

FK 1 capture to disk Toggle

FK 2 type of display Engineering Units/Raw Byte Toggle

FK 3 select display page

FK 4 switches  raw data display window in and  out,  if

window lines are set correctly.

Alt-B send a 'Break' to the TNC

Alt-C connect to another packet station

Alt-D disconnect from another packet station

Alt-P Printer on/off toggle

Alt-S Sound on/off toggle

Alt-X Quit Mode

left arrow decreases playback speed

right arrow increases playback speed.

4.2 Modes menu

When  you bring up the Modes menu you will be prompted  with

the following options.

E Extract From Playback File

I Interactive Mode

O Orbital Parameters

P Playback Mode

R Real Time Mode

S Selections Menu

X Exit to Dos

Type  the letter associated with the option to  perform

it.

Each of the options are described below.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 22

4.2.1 Extract From Playback File

This option begins up the Extraction mode.

4.2.2 Interactive Mode

This option begins up the Interactive mode.

4.2.3 Orbital Parameters

This option begins up the Orbital mode.  This option is

only active if Keplerian data files are in the  defined

directory path.

4.2.4 Playback Mode

This option begins up the Playback mode.

4.2.5 Real Time Mode

This option begins up the Real-time mode.

4.2.6 Selections Menu

This option begins up the Selections Menu.

4.2.7 Exit to Dos

This option allows you to terminate WHATS-UP and return

to DOS.

4.3 Selections Menu

This menu presents you with the following typical options.

C Change Display Page

E Edit Menu

F Files Menu

H Help Menu

J Jump to DOS

M Modes Menu

O Orbits Menu

R Radio Menu

S Spacecraft Menu

T PK232 Menu

U Utilities Menu

X Exit to DOS

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 23

4.3.1 Change Display Page

This  option allows you to change the display page  for

the  Real-time,  Playback  and  Extraction  Modes.   It

performs the same operation as Function key 3 (F3) when

those modes are active.

4.3.2 Edit Menu

This option begins up the Edit Menu.

4.3.3 Files Menu

This option begins up the Files Menu.

4.3.4 Help Menu

This option displays help information for the  function

keys.

4.3.5 Jump to DOS

This  option allows you to jump into a DOS  Shell.  You

return to WHATS-UP by typing the DOS command 'EXIT'.

4.3.6 Modes Menu

This option begins up the Modes Menu.

4.3.7 Orbits Menu

This  option begins up the Orbits Menu. This option  is

only  available  if  an orbit element  file  (*.AMS  or

*.2LN) is present.

4.3.8 Radio Menu

This option begins up the Radio Menu if a Kenwood Radio

is present and the interface is operational.

4.3.9 Spacecraft Menu

This option begins up the Spacecraft Menu.

4.3.10 TNC or PK232 Menu

This option begins up the TNC Menu.

4.3.11 Utilities Menu

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 24

This option begins up the Utilities Menu.

4.3.12 Exit to DOS

This option allows you to terminate WHATS-UP and return

to DOS.

4.4 Edit Menu

WHATS-UP  contains an ASCII text editor suitable  for  files

less than 64k in size. It is built based on Borland's  Turbo

Pascal  Editor Toolbox and the commands are compatible  with

Sidekick  and Wordstar. A summary of the editor commands  is

shown below.

F10 Switch Windows (if more than one window open)

Backspace Delete left char

Del Delete Char

Enter New line

Ins Toggle insert mode

PgUp Move cursor up one page

PgDn Move cursor down one page

^A Move cursor left one word

^C Move cursor down one page

^D Move cursor right one character

^E Move cursor up one line

^F Move cursor right one word

^G Delete one character

^H Delete left one character

^I Tab

^L Find/replace next occurrence

^P Insert a control character into the text

^M New line

^N Insert line

^R Move cursor up one page

^S Move cursor left one character

^T Delete one word after cursor

^V Toggle insert mode

^W Scroll up

^X Move cursor down one line

^Y Delete line at cursor

^Z Scroll down

^K^B Mark beginning of block

^K^C Copy block to position of cursor

^K^D Save file and exit edit

^K^H Hide block marker

^K^K Mark end of block

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 25

^K^Q Abandon file and exit edit

^K^R Read file into screen

^K^T Mark single word as block

^K^V Move block to position of cursor

^K^W Write block to disk file

^K^Y Delete block

^K 1..9 Set marker 1 .. 9

^Q^A Find text and replace

^Q^B Move to beginning of block

^Q^C Move to end of file

^Q^D Move to right of line

^Q^E Move to top of window

^Q^F Find text

^Q^I Toggle autoindent mode

^Q^K Move to end of block

^Q^R Move to top of file

^Q^S Move to left of line

^Q^X Move to bottom of window

^Q^Y Delete to end of line

^Q 1..9 Jump to marker 1..9

The  following options can be set in the  Find/Find  and

Replace (^Qf and ^Qa) operations.

# locates #th occurrence

G global replace

N replace without Y/N question

U ignore upper case/lower case

W match whole words only

Notes

1    The  ^  key  in front of  a  character  identifies  the

character  as  a 'control' character. To  activate  it,

hold down the Control key AND the character key.

2    Some of the commands require two keystrokes.

3    Use  ^P to embed a control character in the  text.  For

example, if you are creating a file containing commands

to  be sent to the TNC, to enter a control-C  character

into the file, use the ^P^C sequence of keystrokes.

WHILE  USING THE EDITOR, WHATS-UP can't receive and  process

characters from the TNC. All other features ARE INHIBITED OR

LOCKED OUT.

The  Edit menu allows you to call up the various  files  for

editing as shown by this typical display.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 26

D DOVE.DOP

F Any File

K WHATS-UP.2LN

M c:dove.SYS

P Pick ***.D17 File

R c:910313.D17

S WHATS-UP.SYS

T Two Files

W Arrays

X whats-up.txt

Y Pick ***.SYS File

Type the letter corresponding to your choice.

4.4.1 Edit Doppler File

This  option lets you edit the file containing  Doppler

measurements.

4.4.2 Any File

This option lets you edit any file.

4.4.3 Edit Keplerian Element File

This  option lets you edit the default file  containing

Keplerian data.

4.4.4 Edit Spacecraft Configuration File

This  option  lets  you edit the  file  containing  the

spacecraft  configuration data, such as display  pages,

and automatic radio tuning options.

4.4.5 Pick Capture-to-disk File

This option lets you pick a capture-to-disk file to  be

edited.  When  you implement this option  you  will  be

presented with a list of available files.

4.4.6 Edit Capture-to-disk File

This  option  lets you edit the  file  containing  data

captured during the last pass.

4.4.7 Edit WHATS-UP.SYS

This option lets you edit the WHATS-UP.SYS file.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 27

4.4.8 Two Files

This  option lets you edit any two files.

4.4.9 Edit Doppler Channel File

This  option  lets  you edit the  file  containing  the

channel  numbers of the data to be extracted  from  the

raw telemetry, processed and written to disk.

4.4.10 Edit Doppler Data File

This option lets you edit the file containing extracted

data.

4.4.11 Pick Spacecraft Configuration File

This  option lets you pick a  spacecraft  configuration

file  to be edited. When you implement this option  you

will be presented with a list of available files.

4.5 Files Menu

This menu presents you with the following typical options.

A Change Directory Path

F Change Playback File

V View Playback File

Z Show *.D17 Files

4.5.1 Change Directory Path

This  option  allows  you  to  temporarily  change  the

directory path.

4.5.2 Change Playback File

This option allows you to change the playback file.  To

select a file, move the cursor down to the desired file

and  push the 'Enter' key. If you have more files  than

fit  in  the window, touch the 'PgDn'  key  to  display

another window full.

4.5.3 View Playback File

This  option  allows you to view the  contents  of  the

playback file.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 28

4.5.4 Show Spacecraft data Files

This  option  shows you the data files for  the  chosen

spacecraft in the default directory path.

4.6 Orbits Menu

This menu presents you with the following typical options.

A Pick AMSAT Format Element Set

E Edit WHATS-UP.2LN

L Load Element File

N Pick NASA 2 Line Element Set

V View Spacecraft Orbit Elements

4.6.1 Pick AMSAT Format Element Set

This  option  allows  you to change  the  AMSAT  Format

Keplerian  Element  file. To select a  file,  move  the

cursor  down to the desired file and push  the  'Enter'

key.  If  you have more files than fit in  the  window,

touch the 'PgDn' key to display another window full.

4.6.2 Edit Default Keplerian Element File

This  option lets you edit the default file  containing

Keplerian data.

4.6.3 Load Element File

This  option  loads the data in the element  file  into

WHATS-UP.   When  you activate the option you  will  be

prompted as follows.

Which Element File ? WHATS-UP.2LN

WHATS-UP   will  supply  the  default  name,  you   may

overwrite  it to supply the name of another  file.  You

use  this option to load a file that is not located  in

the default directory.

4.6.4 Pick NASA 2 Line Element Set

This  option  allows you to change the  2  Line  Format

Keplerian  Element  file. To select a  file,  move  the

cursor  down to the desired file and push  the  'Enter'

key.  If  you have more files than fit in  the  window,

touch the 'PgDn' key to display another window full.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 29

4.6.5 View Spacecraft Orbit Elements

This option lets you view the data associated with  the

spacecraft. When you activate this option, you will  be

presented  with  a  menu window containing  a  list  of

spacecraft  designators.  Move the cursor to the  one  of

interest and touch the 'Enter' key to select it.

A typical display is shown below :-

1 20440U 90  5  E 91059.65616971  .00001077  00000-0  44042-3 0  2017

2 20440  98.6806 140.0431 0012003 123.1299 237.1040 14.29083383 57498

Catalog ID:               20440   Apogee:                 803.312

Element Set:                201   Perigee:                786.093

Epoch Year:                1991   Period:                  100.76

Epoch Day:           59.6561697   Semi Major Axis:       7172.862

Drag:                0.00001070

Inclination:            98.6806   Epoch Age:               22.263

RAAN:                  140.0431   Current Date:            81.919

Eccentricity:         0.0012003   Current Orbit #:           6067

Argument of Perigee:   123.1299

Mean Anomaly:          237.1040

Mean Motion:         14.2908338

Epoch Orbit #:             5749

If  the default element file is in AMSAT format (*.AMS)  the

two line display is not shown.

4.7 Radio Menu

This  menu  is  only present if you  have  a  Kenwood  Radio

defined as your Radio Receiver for the spacecraft.  The menu

presents you with the following typical options.

D Turn Doppler Tracking ON

I Change Doppler Interval

4.7.1 Turn Doppler Tracking ON/OFF

This  option lets you turn the Doppler tracking on  and

off.

4.7.2 Change Doppler Interval

This  option  lets  you change the  time  interval  (in

minutes)  between successive samples of the  radio  VFO

frequency.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 30

4.8 Spacecraft Menu

This menu presents you with the following typical options.

K picK Spacecraft

M Change Spacecraft

P Pick Ops. Schedule

S Show Ops. Schedule

4.8.1 picK Spacecraft

This  option  allows you to choose  another  spacecraft

from a list of those in the default directory path.  To

select one, move the cursor down to the desired one and

touch the 'Enter' key. For example, amongst the default

files supplied are DOVE.SYS and FUJI.SYS. To select the

DOVE or the Fuji-20 spacecraft, move the cursor down to

the 'DOVE.SYS' or 'Fuji.SYS' line and touch the 'Enter'

key.

4.8.2 Change Spacecraft

This option allows you to choose another spacecraft. To

select   a   another  one,  enter  the  name   of   the

spacecraft.sys  file. For example, amongst the  default

files supplied are DOVE.SYS and FUJI.SYS. To select the

DOVE or the Fuji-20 spacecraft, type 'DOVE' or 'Fuji'

4.8.3 Pick Ops. Schedule

This option allows you to pick an operations  schedule.

You  would  use  it  to look  at  the  schedule  for  a

spacecraft other than the one currently selected.

4.8.4 Show Ops. Schedule

This  option  allows you to look at  the  schedule  the

spacecraft currently selected.

4.9 TNC Menu

This menu presents you with the following typical options.

A UoSAT ASCII Beacon

B Phase 3 RTTY Beacon

C Set Morse Code (CW)

M Fuji/MicroSat ASCII Packet

T Configure PK232

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 31

4.9.1 UoSAT ASCII Beacon

This  option  will  configure the PK-232  to  copy  the

UoSAT-OSCAR 11 telemetry. Note: you require a  hardware

change  in  the PK- 232 to make sense of  the  received

data.

4.9.2 Phase 3 RTTY Beacon

This  option  will  configure the PK-232  to  copy  the

UoSAT-OSCAR 13 Baudot Beacon.

4.9.3 Set Morse Code (CW)

This  option  will configure the PK-232 to  copy  morse

code.  You  use  this if you want to  copy  morse  code

telemetry.  Note the decoding formats are not  provided

in  this  program,  so  you will  have  to  decode  the

telemetry  in  some other way, such as by  hand  or  by

means of a spreadsheet.

4.9.4 Fuji/MicroSat ASCII Packet

This option will configure the PK-232 to copy the  DOVE

and Fuji ASCII format PACKET telemetry. You should  not

use  WHATS-UP to capture AMSAT/UoSAT  binary  telemetry

because WHATS-UP filters the ^J and ^M (carriage return

and line feed characters) from the incoming datastream.

4.9.4 AMSAT-OSCAR 21 1100 baud PSK ASCII Packet

This option will configure the PK-232 for 1100 baud  on

the  Radio Port. The PK-232 does not have an 'HB  1100'

command  yet, that is coming in a forthcoming  firmware

release,  hence the approach used here. You will  still

need   a  PSK  decoder  to  copy  the  data  from   the

spacecraft.

Note:   at  the time this software  was  released,  the

display  formats  for RM-1 were not  available,  so  no

configuration  file is available for RM-1  in  WHATS-UP

0.60.

Note:  at the time this software was released,  it  was

unclear if the data are HDLC or some other kind of  PSK

similar to AMSAT-OSCAR 13's 400 baud PSK data.  If  the

data  are  not  HDLC,  you will not  be  able  to  copy

anything usable on the PK-232.

Note:  In view of the two caveats above,  don't  expect

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 32

too  much from this mode. In particular, do not go  out

to  purchase a PSK modem for use with  this  spacecraft

without verifying that the data are receivable.

4.9.5 Configure PK232

This  option  configures  the PK-232  to  copy  the  UI

packets transmitted by the Packet spacecraft.

4.10 Utilities menu

This menu presents you with the following typical options.

A Change Directory Path

B Change Beacon Frequency

D Show Space on Disk

R Reset Header Counters

S Show Defaults

U Set PC Clock

Z Show Files

* Show Color Chart

! Reconfigure WHATS-UP

@ Sound CW String

4.10.1 Change Directory Path

This  option  allows  you  to  temporarily  change  the

directory  path  to  the  spacecraft   capture-to-disk,

configuration and schedule files.

4.10.2 Change Beacon Frequency

This option allows you to temporarily change the beacon

frequency displayed in the status window.

4.10.3 Show Space on Disk

This option allows you to see how much space is left on

a disk with exiting from the program.

4.10.4 Reset Header Counters

This   option   applies  to   spacecraft   transmitting

packetized telemetry. When activated, the option resets

the packet counters to zero. Use this before a pass  to

see  how many packets of each type are received  during

the pass.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 33

4.10.5 Show Defaults

This option allows you to display the default settings.

4.10.6 Set PC Clock

This  option lets you change the time in the  PC.  When

you  do so, WHATS-UP will recalculate the positions  of

the spacecraft.

4.10.7 Show Files

This  option lets you display the files in the  default

directory  path. Use this if WHATS-UP tells you that  a

file does not exist and you are sure that it does.

4.10.8 Show Color Chart

This   option   allows  you  to   display   the   color

combinations.  Use this to see what how  the  different

color  combinations  appear on your  screen,  note  the

numbers associated with each color, then exit from  the

program  and  edit  the WHATS-UP.SYS  file  using  your

editor  in its ASCII (non document)mode to  change  the

colors to those you desire.

4.10.9 Reconfigure WHATS-UP

This option reloads the configurations from the  WHATS-

UP.SYS  file. Use it after editing the file to see  the

effect of your changes.

4.10.10 Sound CW String

This  option asks you to enter some characters  at  the

keyboard.  When you do so, it then sounds them  off  in

morse code. You use this option to adjust the speed  of

the morse code used in the AOS, LOS and EWT signals.

5.0 Orbital Elements

5.1 Basics

As  an object moves in space it is subject to gravity.   The

object itself has mass and attracts other objects and is  at

the  same time attracted by the mass in the  other  objects.

Sir Isaac Newton formulated the law of gravity, which can be

described in the following manner.

All   bodies  attract  each  other  with  a   force   called

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 34

gravitational  attraction.   The  strength  of  the   mutual

attraction between two bodies is dependent on their  masses,

and  the  distance between the bodies. In  fact  the  closer

together that the two bodies are, the greater is the  mutual

attraction.   Mathematically  this can be expressed  as  the

gravitational  attraction  between two  bodies  is  directly

proportional  to their masses and is inversely  proportional

to the square of the distance between them.

Planet  Earth  is  an object moving in space  and  exerts  a

gravitational force.  It pulls anything close to it  towards

the  center  of  the  Earth  at  an  increasing  speed.   An

increasing speed is known as acceleration.  The acceleration

due  to  gravity at the surface of the Earth  is  given  the

value of 1 Gravity (G).

5.2 Orbital Trajectories

If  the Planet pulls everything towards itself,  what  keeps

things in orbit around it?  For example, if you throw a rock

up  into the air, gravity and air resistance (drag) slow  it

down  and  it falls back to Earth.  If you  shoot  a  bullet

towards  the  horizon it will travel much further  than  the

stone  did,  but will still fall to Earth  (unless  it  hits

something first).

If you launch a rocket towards the horizon, the rocket  will

accelerate as long as the fuel lasts.  When the rocket  fuel

is  exhausted,  the  rocket will continue  to  travel  in  a

straight  line  until other forces alter its path.   If  the

rocket   is  low  enough,  the  molecules  of  air  or   the

atmospheric  drag  tends to slow it down. At any  height  it

will still be pulled back by gravity.  The force of  gravity

always acts towards the center of the Earth.  On its own the

rocket  will  travel  in a straight  line.   Gravity  acting

downwards  will  curve  the path of the  rocket  around  the

Earth.

The  force  of gravity will pull the rocket  away  from  its

horizontal  path  and  cause  it  to  fall  in  a  downwards

direction.  Now the surface of the Earth is also curved  and

also curves away in a downwards direction.  If speed of  the

rocket  is  such  that  the rate  of  descent  (due  to  the

gravitational  attraction  of  the Earth) is  equal  to  the

curvature of the Earth, the rocket will always remain at the

same  height , namely, it will be in a circular  orbit.   If

the rocket does not have enough speed it will gradually fall

back  to  Earth, and if it has more speed, it will  tend  to

rise above the Earth.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 35

If the rocket continues to burn fuel the speed of the rocket

increases and the path it takes rises away from the  surface

of the Earth.  When the fuel is exhausted, gravity is  still

there and still slows down the rocket.  As it slows down its

path  curves  more  sharply  until  at  its  furthest  point

(apogee)  it  is traveling parallel to the  surface  of  the

Earth.   It  is  however traveling  slower  than  the  speed

necessary to maintain a circular orbit at that altitude  and

starts  to  curve  back towards the  Earth.   The  force  of

gravity  now  acts  in a  downward  and  (slightly)  forward

direction and speeds up the rocket until at its lowest point

(perigee) it once again is traveling parallel to the surface

of  the  Earth  but now has enough speed to  rise  away  the

surface.  This process repeats each time around and produces

an elliptical orbit.(In this orbit, the center of the  Earth

is one focus of the ellipse.

To  place  a satellite into an orbit, it must be  given  the

right  amount of speed for the desired orbital  altitude  so

that  the  orbital velocity at apogee is such that  it  just

balances  the  gravitational pull of the Earth.   Since  the

force  of  gravity  decreases  with  altitude,  the  orbital

velocity  is  also different at  different  altitudes.   Any

rocket can lift a small mass to a much higher altitude  than

it can lift a large mass.  How much and how high will depend

on the rocket itself.

5.3 Types of orbits

Different orbits are used for different purposes.   Circular

and elliptical orbits come in various forms depending on the

angle that the plane of the orbit makes with the equator  of

the Earth.  This angle is known as the angle of  inclination

of  the orbit (with respect to the equator).  A polar  orbit

has an angle of inclination such that the spacecraft in that

orbit   can  see  the  polar  regions  of  the   Earth.    A

geostationary orbit is one which has an angle of inclination

parallel  to  the equator and an altitude  of  22,240  miles

(35,790  km)  and the spacecraft moves in its orbit  at  the

same speed as a point on the surface of the Earth below  it.

The spacecraft thus appears to be stationary with respect to

the Earth below it.

The direction in which the satellite moves around the  Earth

determines  the type of orbit.  One which travels along  its

orbit  in  the same direction as the rotation of  the  Earth

(eastwards)  is  said to be in a direct or  prograde  orbit.

One  which  travels  against  the  rotation  of  the   Earth

(westwards) is said to be in a retrograde orbit.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 36

The  movement  of  an  object  in  space  can  be  described

mathematically.  In the early 17th century, the  only  known

objects in space were the Sun, the Moon, the Planets, a  few

comets  and  the stars.  It was then  that  Johannes  Kepler

formulated  three laws that first described the movement  of

the Planets about the Sun.

Kepler's Laws are stated in the following paragraphs.

1.   Each  Planet  revolves about the Sun in an  orbit  that

forms a circumference of an ellipse with the Sun at one

focus of the ellipse.

2.   The  line from the center of the Sun to the  center  of

the Planet (called the radius vector) sweeps out  equal

areas  in equal periods of time as the  Planet  travels

along the circumference of the ellipse.

3.   The  square  of the time taken for a Planet  to  travel

around  the  circumference of the  ellipse  (period  of

revolution of the orbit of a Planet) is proportional to

the  cube of the mean distance of the Planet  from  the

Sun.

In  the first law, the focus within the Sun is  actually  at

the  center of mass of the Earth-Sun system and not  at  the

center of the Sun.

5.4 Orbital Elements

The position of an object in space can be expressed in terms

of  its relationship with other bodies.  Each orbit  can  be

described  in terms of a number of parameters  which  supply

enough information to accurately locate the satellite.   Six

basic  parameters  are used to describe the  position  of  a

satellite  in  an  elliptical orbit  are  described  in  the

following paragraphs.  They are Angle of inclination,  Right

Ascension of Ascending Node (RAAN), Eccentricity,  Semimajor

Axis, Argument of Perigee and Epoch time of Ascending  Node.

Consider each one in turn.

5.5 Angle of Inclination.

The  angle of inclination of an orbit is the  angle  between

the  plane  of the orbit and the equator of  the  Earth.   A

satellite  moving  in  a  direct  orbit  has  an  angle   of

inclination  between  0  and 90 degrees,  one  moving  in  a

retrograde  orbit has an angle of inclination of between  90

and 180 degrees.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 37

The  maximum  northern and southern latitudes reached  by  a

satellite  are  equal  to the angle of  inclination  of  its

orbit.

5.6 Right Ascension of Ascending Node (RAAN).

While  the spacecraft is moving around the Earth, the  Earth

is at the same time rotating on its own axis, and is  itself

traveling in an orbit about the Sun.  The Right Ascension of

Node is needed as a fixed reference point in the sky.

Astronomers  use the term celestial sphere to  describe  the

sky  for two reasons. First, from where we are  standing  on

the surface of the Earth, the sky seems to be painted on the

inside  surface  of  a  sphere  (with  the  stars  in  fixed

positions  on  the  sphere). Second,  have  you  ever  known

scientists  to use a short commonly used word when they  can

use long ones?

The orbital plane of a spacecraft intersects the  equatorial

plane  of  the Earth in two places (one on each side of  the

globe).  A line drawn between these two points and continued

out  to  the celestial sphere is called the line  of  nodes.

The  two  points  on  the line of  nodes  where  the  planes

intersect  are called the point of nodes.  Most globes  (and

maps)  show  the  north pole upwards.  When  the  spacecraft

crosses  the  equatorial plane (passes  above  the  equator)

going northward it is ascending from south to north and that

node point is known as the ascending node.  Conversely  when

the  satellite  continues on its way and  travels  half  way

around the world it crosses the other node on the equatorial

plane  descending to the southern hemisphere.   This  second

node is called the descending node.

The Earth is in an inclined orbit around the Sun, just  like

a satellite is in orbit around the Earth.  The Earth has  an

ascending  and descending node around the Sun, in a  similar

manner  to  a  spacecraft in orbit around  the  Earth.   The

orbital  plane of the Earth is known as the  Ecliptic.   The

direction in space from the center of the Earth through  the

intersection  of  the ecliptic and  the  Earth's  equatorial

plane  out  to  the celestial sphere is  called  the  vernal

equinox or the First Point of Aries because it points to the

constellation  of Aries (which is so far away that (for  all

practical purposes) it is in fixed direction).

The  angle between the line of nodes for the ascending  node

of  the  orbit  of  the  spacecraft  continued  out  to  the

celestial sphere and the vernal equinox when measured in  an

easterly (right as opposed to westerly/left) direction along

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 38

the earth's equator is defined as the Right Ascension of the

Ascending Node (RAAN).

5.7 Eccentricity and Semimajor Axis

In  a circle, the radius of the circumference  is  constant.

This means that an object traveling along the  circumference

is always at a constant distance from the center or focus of

the  circle.  The general shape of an orbit is  an  ellipse.

Unlike  a  circle,  an ellipse has two  focal  points.   The

distance between each of the focal points of the ellipse and

an object on the circumference is constant.  A line  through

the  two  focal points and the circumference is  called  the

Semimajor  Axis.   The  longest line  perpendicular  (at  90

degrees   to)  the  semimajor  axis  passing   through   the

circumference of the ellipse is called the Semiminor Axis.

The  mathematical  term describing the overall shape  of  an

ellipse is called Eccentricity.  When the eccentricity of an

ellipse  is 0 the length of the semimajor axis is  equal  to

the  length  of  the  semiminor axis  and  the  shape  being

described is a circle.  A value of eccentricity greater than

0  means that there is a difference between the  lengths  of

the  axes  and  the  shape  of  the  ellipse  flattens  out.

Eccentricity describes the shape of the orbit and the length

of  the semimajor axis describes the size of the orbit.   If

these  two  parameters  are known, the  apogee  and  perigee

values for the orbit can be calculated.

5.8 Decay Rate

The  Earth has an atmosphere.  It is dense at  ground  level

and thins out with increasing altitude.  The Earth's gravity

attracts  the molecules of gas in the atmosphere  and  stops

them  from  escaping.  As the satellite  travels  along  its

elliptical  orbit  around the Earth, its  altitude  changes.

When it is close to perigee, it bumps into molecules of air.

The  lower  it gets the greater the drag on it by  the  air.

When  the orbital radius decreases as a result of drag,  the

potential  energy  of the spacecraft also  decreases  as  it

comes closer to the earth. This decrease in potential energy

reappears  in  the  form  of heat  energy  imparted  to  the

atmosphere  and  to the skin of the spacecraft,  and  in  an

increase of the kinetic energy of the spacecraft. It is this

latter  that causes the velocity of the spacecraft to  speed

up.

The  rate of change of speed through the atmosphere  at  the

perigee  pass  will depend on the type of orbit and  on  the

altitude of the perigee point.  In general, satellites  with

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 39

low  perigee  points will be more affected  than  satellites

with  higher perigee points.   The effect of this  over  the

long term is to lower the apogee point. Over the long  term,

the  satellite tends to spend more time in the denser  parts

of the atmosphere which then tends to circularize the  orbit

(at   the  perigee)  at  which  point  the  air  drag   acts

continuously on the spacecraft and the orbit  disintegrates.

The  rate  of change of the orbit measured at  a  particular

epoch is called the decay rate.

5.9 Argument of Perigee.

A line drawn between the perigee of an elliptical orbit  and

the center of the Earth is called the line of perigee.  This

line  also  passes through the apogee and is  the  semimajor

axis  of the orbit.  The angle between the line  of  perigee

and the line of nodes is called the argument of perigee.  It

is a measurement of the angular distance between them and is

measured in the ascending direction from the line of  nodes.

The argument of perigee thus establishes the position of the

ellipse itself within the orbital plane.

5.10 Epoch Time (of Ascending Node) and Revolution Number.

The  Epoch  time is a time when the  satellite  crosses  its

perigee point.  This time is given as a Julian date, and  is

the reference time for when the orbital elements are  valid.

The  Epoch  Revolution or orbit number is the  orbit  number

(since first perigee crossing) for which the element set  is

valid.

5.11 The Mean Motion.

The  mean motion a satellite is a measurement of the  number

of  orbits  completed in a day.  It is equal  to  twice  the

value of PI divided by the time that the spacecraft takes to

complete one revolution of its orbit (orbital period).

5.12 The Catalog Number

The catalog number is the number given to the object.

5.13 Mean Anomaly

As  the  satellite  travels along its  orbit,  its  position

changes.   The angle (measured in the direction  of  forward

rotation)  between  the position of the spacecraft  and  the

line  of perigee is called the true anomaly.  The  speed  of

the spacecraft is different at different parts of the orbit.

Calculations  are simpler if the speed is considered  to  be

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 40

constant.   The constant value is the average speed  of  the

spacecraft   in  its  orbit.   The  mean  anomaly   is   the

hypothetical  position of the satellite in its orbit  (along

the ellipse) if it is assumed to be traveling at its average

speed.

5.14 Anticipated Spacecraft Lifetimes

There  is not much point in setting up equipment to  receive

data  from  these  spacecraft if they are not  going  to  be

around  for  a reasonable amount of  time.  Past  experience

points  to three main factors limiting the operational  life

of  an OSCAR, namely orbital decay, battery life, and  total

radiation dosage.

5.14.1 Orbital Decay

The  last OSCAR to plunge back into the  earth's  atmosphere

while still active was UoSAT-OSCAR 9. It was launched into a

500km  orbit,  and  lasted  eight  years.  A  chart  in  the

Satellite Experimenter's Handbook shows a lifetime of 40000-

50000 days for spacecraft at the Microsat altitude, or some-

where  around  120 years; so orbital decay is not  our  main

worry.  There has been some concern about  AMSAT-OSCAR  13's

orbit, which is expected to decay somewhere between 1992 and

1995.  AMSAT  however are currently planning  a  replacement

spacecraft so the investment in receiving equipment will not

be in vain.

5.14.2 Battery lifetime.

Battery  breakdown  has  caused the demise  of  all  amateur

spacecraft  except  UoSAT-OSCAR 9. OSCARs 1, 2  and  3  were

limited to whatever charge was in the batteries when launch-

ed.  Once  Solar  cells and  nickel-cadmium  batteries  were

flown,  the limiting factor became the number of  times  the

battery cells could be charged and discharged.

The  batteries in OSCAR spacecraft in low earth  orbit  have

lasted between five and eight years. UO 9 was still  looking

good  when  it  recentered the earth's  atmosphere  after  8

years.   UoSAT-OSCAR  11,  launched  in  March,  1984,   has

batteries  with  the same part number as those used  in  the

Microsats. These latter batteries were procured in much  the

same  manner, and were matched and tested by the same  group

of VITA volunteers in Canada that performed the function for

UoSAT-OSCAR  11. The batteries on UoSAT-OSCAR 11 have  shown

no  signs of weakening after almost six years in  orbit,  so

the  prognosis  for UoSAT-OSCAR 11 and the  Microsats  looks

good.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 41

5.14.3 Radiation Damage.

Anything above the protection of the atmosphere is subjected

to exposure to the radiation due to the direct and secondary

effects   of  high  energy  particles,  from  the  sun   and

elsewhere.  The part of the spacecraft most  susceptible  to

such  damage is the memory in the  on-board-computer  (OBC).

Such damage manifests itself as a bit flipping from a 0 to a

1 or from a 1 to a 0, which is correctable. Since the memory

chips used are byte-wide, many types of single-chip failures

can  be  avoided in a manner analogous to  locking  out  bad

sectors on a hard disk.  UoSAT-OSCAR 11, which uses somewhat

similar  technology  chips, has survived  almost  six  years

despite  a  failure of one small section of  memory  several

years ago.

Another  part of the OBC is the controller itself  which  is

susceptible to a particle hitting it in a place that  causes

uncorrectable errors, such as a wrong operation internal  to

the microprocessor, or a CMOS latchup which can result in  a

high  current  being  drawn which  overheats  the  part  and

destroys it.

The spacecraft are more likely to fail due to the cumulative

effects  of  this constant bombardment, which  is  known  as

total dose. Once the total dose reaches a certain point, the

gates  in the transistors that make up the computer and  its

memories will no longer switch. This situation has  occurred

in  AMSAT-OSCAR  10.  Predicting when this  might  occur  is

difficult  because  of  lack  of  knowledge  concerning  two

things;  the  total  radiation dose in this  orbit  and  the

tolerance limits for the parts used.

The  MicroSats are in a much more benign orbit  than  AMSAT-

OSCAR 10 which with its 4000 km perigee, spends more time in

the Van Allen radiation belts than was planned.  AMSAT-OSCAR

10's  memories therefore failed sooner than hoped,  but  the

other  electronics, the transponders, batteries,  and  solar

arrays   live  on.  Through  no-longer   actively   attitude

controlled, its transponders are still usable many weeks  of

the year when its batteries are being charged by the sun.

AMSAT-OSCAR  13 is in a better orbit with a  lower  perigee,

and  should  not suffer radiation degradation  before  other

effects shorted its lifespan.

6.0 The Spacecraft

OSCAR spacecraft downlink signals in the amateur 145 MHz and  430

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 42

MHz  bands  modulated by means of Frequency Shift  Keying  (FSK),

Frequency  Modulation (FM) or Phase Shift Keying (PSK).  Some  of

the characteristics of the downlinks of suitable OSCARs currently

operational  are shown in Table 6-1.  UoSAT-OSCAR 11  and  AMSAT-

OSCAR  13  send back BAUDOT or ASCII data while  AMSAT-OSCAR  16,

DOVE-OSCAR-17,  WEBER-OSCAR 18, LUSAT-OSCAR 19 and Fuji-OSCAR  20

downlink packetized telemetry.

Table 6-1 Some of the Characteristics of OSCAR Downlinks.

Spacecraft  Beacon     Modulation      Data    Note

Frequency  Type            Rate

(MHz)

UO-11       145.825     FM    ASCII  1200 Baud   1

435.025     FM    ASCII  1200 Baud   1

AO-13       145.812    FSK    BAUDOT   50 Baud   2

PSK    ASCII   400 Baud   2

435.651    FSK    BAUDOT   50 Baud   2

PSK    ASCII   400 Baud   2

AO-16       437.025    PSK    AX.25  1200 Baud   3

437.025    PSK    AX.25  1200 Baud   3

DO-17       145.825    FM     ASCII  1200 Baud

WO-18       437.100    PSK    AX.25  1200 Baud   3

437.075    PSK    AX.25  1200 Baud   3

LO-19       437.150    PSK    AX.25  1200 Baud   3

437.125    PSK    AX.25  1200 Baud   3

FO-20       435.912    PSK    AX.25  1200 Baud

Notes

1.   Spacecraft  also broadcasts bulletins and Various  Telemetry

formats.

2.   Spacecraft  downlink  modulation is changed according  to  a

pre-published schedule.

3.   Alternate  (back  up)  beacon  frequency,  may  be  used  on

Wednesdays.

Before  discussing the equipment needed to receive  signals  from

the  spacecraft, a brief word about the spacecraft themselves  is

in  order.   Since  these OSCARs rode  into  space  as  secondary

payloads,  the orbits that they are in are close to those of  the

primary   payload  and  are  not  optimized  for  amateur   radio

communications.  The exception is AMSAT-OSCAR 13 which  contained

a motor which was used by radio amateurs to boost the  spacecraft

from  the  orbit  the rocket placed it in  into  its  operational

orbit.   The  ones that are in low earth orbits can  be  received

with  simple  equipment, but are in range for  short  periods  of

time, AMSAT-OSCAR 13 in an elliptical orbit is in range for  many

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 43

hours each day, but needs more sophisticated receiving equipment.

The  orbital parameters of the OSCARs under discussion are  shown

in Table 6-2.

Table 6-21 Orbital Parameters of the OSCARs

Spacecraft       Apogee   Perigee Inclination  Period

(km)     (km)    (Degrees)   (Minutes)

UoSAT-OSCAR 11     699     670     98.0          98.3

AMSAT-OSCAR 13  39,000   2,500     26.1         686.65  [1]

AMSAT-OSCAR 16     804     780     98.7         100.8

DOVE-OSCAR 17      804     780     98.7         100.8

WEBER-OSCAR 18     804     780     98.7         100.8

LUSAT-OSCAR 19     804     780     98.7         100.8

FUJI-OSCAR 20    1,745     912     99.05        112.0

Notes

1  686.65 minutes is 11 hrs, 26 min.

6.1 Receiving system components

Consider  the different components or building  blocks  that

are used in the different receiving configurations.

6.1.1 Antennas

Antennas receive signals, and each kind of antenna has  some

degree  of directive and polarization.  When the  spacecraft

rises   above   the  local  horizon,  the   ground   station

experiences acquisition of signals (AOS).  At this time  the

groundstation is receiving signals coming from a  particular

direction (azimuth). As the spacecraft rises in the sky, the

elevation  angle of the received signals changes, until  the

spacecraft drops below the observer's horizon and the ground

station experiences loss of signals (LOS). As seen from  the

ground,   the  spacecraft  rises  from  a  horizon  in   one

direction,  travels in an arc across the sky and sets  at  a

different  horizon in a different direction.  Each pass  for

each  spacecraft  is  different.   Antennas  for   receiving

signals from spacecraft must thus be able to receive signals

coming in from almost any angle.

Antennas   in  this  context,  fall  into  two   categories,

omnidirectional and rotatable.  The simple turnstile antenna

is horizontally polarized and has a good response to signals

arriving from high angles and can be built for about  $2.00.

The  ground  plane  and  J  Pole  antennas  are   vertically

polarized and have a good response to signals arriving  from

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 44

low  angles.  These antennas however do not have much  gain.

Yagi  Beam  Antennas however have gain with respect  to  the

turnstile or ground plane, but only in specific  directions.

You can think of the gain in some directions as being  moved

into the direction that the antenna is pointed at.  The gain

of  the  antenna depends on the number of  elements  in  the

antenna,  and the higher the gain, the narrower the area  of

the gain (lobe) is.  Consequently, these beam antennas  must

be  moved  to keep the spacecraft in the main  lobe  of  the

antenna.  Since the need for keeping the antenna pointed  at

the spacecraft depends on the beam width of the antenna, the

lower the gain of the antenna the less accurate the tracking

need  be.   Luckily  the orbits help out  in  this  respect.

UoSAT-OSCAR  11 in low earth orbit, which means it  is  fast

moving,  needs  only  a small amount of gain,  so  TV  style

rotators  can be employed to point antennas with  between  2

and  4  elements,  while  AMSAT-OSCAR  13  which  is  in  an

elliptical  orbit,  moves so slowly for nearly 8 of  its  11

hour  orbit,  that again, TV style rotators can be  used  to

point higher gain antennas with between 8 and 11 elements.

Building  your  own  antennas  is  an  easy  and  worthwhile

project.  Antennas for these OSCARS are simple and not  very

critical  with respect to the materials used.  They  can  in

fact be built from recycled junk.

6.1.2 Receivers

Receivers  fall into two kinds, FM and linear. FM  receivers

are  used  for  reception of the FM signals  from  DOVE  and

UoSAT-OSCAR  11,  while  linear  receivers  are  needed  for

reception  of  the  FSK  and  PSK  signals  from  the  other

spacecraft.  All  vhf/uhf scanner radios are  FM  receivers.

The linear receivers need single side band (SSB) capability,

something   normally  found  in  short  wave   communication

receivers.  As a result of the growing popularity of amateur

satellite  communications,  suitable  vhf/uhf   transmitter-

receivers (transceivers) have been on the market for several

years,  however these transceivers are expensive listing  in

the  $800  to  $1200  range.   An  alternative  approach  to

reception  is  to use a short wave  communications  receiver

listing around $500 together with a front end  downconverter

which lists at under $100.  The short wave radio can also be

used  to tune in, not only the world of amateur  radio,  but

news  broadcasts from overseas; a totally different  are  of

classroom activity.

"Expensive"  is  a relative term. These  days,  many  people

think  nothing  of spending $1000 on a stereo system  or  on

equipment for photography or other hobbies.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 45

6.1.3 Terminal Units or Modems

Digital  radio  links work much in the same way  as  digital

signals are transferred over the telephone line. However  in

this  case, instead of a phone wire, a radio link  is  used.

Both  links  use modems to convert the  serial  input/output

digital  RS-232 signals of the computer to the  audio  tones

used on the communications link.

Packet radio signals are demodulated by a radio modem  known

as a Terminal Node Controller (TNC). The device is connected

in between the radio and the computer and provides hams with

two way digital communications. A packet only TNC lists  for

between $120 and $200. For reception of the PSK signals from

AMSAT-OSCAR  16,  WEBER-OSCAR 18, LUSAT-OSCAR 19  and  Fuji-

OSCAR  20, PSK Modems are available either as add-ons  to  a

regular  TNC or as stand alone units, listing  between  $150

and $700.

The  BAUDOT Radio Teletypewriter (RTTY) signals from  AMSAT-

OSCAR 13 can be demodulated by an RTTY Terminal Unit.  These

devices  are listed at between $100 and $300. On  the  other

hand a multi- mode communications controller listing between

$250 and $700 can be used for AMSAT-OSCAR 13 as well as DOVE

and  the other spacecraft.  AMSAT-OSCAR 13 downlinks  BAUDOT

because   that   is   the   most   commonly   used   digital

communications   mode  used  by  radio  amateurs   at   high

frequencies (short waves)

The modem for UoSAT-OSCAR 11 is a little more difficult,  as

its  ASCII encoding is the reverse of the standard  used  in

the USA. This is because the spacecraft was built in the  UK

and its use of tones to represent data reflects the encoding

used in a popular tape interface (in the UK) at the time the

spacecraft  was built (1982- 1984).  Still a  do-it-yourself

circuit needs a few integrated circuits, is simple to build,

easy to test, and very low cost (under $50).

Summarizing the costs of the items mentioned above, the list

prices  fall  between a low and high cost depending  on  the

amount you wish to pay.  The summary is shown in Table  6-3.

It  should  be noted that the high price items  may  not  be

better   than   the  cheaper  ones,  particularly   in   the

educational  environment.   This table is of course  only  a

guide,  since  you will probably end up  with  something  in

between.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 46

Table 6-3 Range of Equipment List Prices

Item                     Low Price       High Price

Antenna                       $2.00        $100.00

Receiver                    $100.00       $1200.00

Radio Modem (TU/TNC)        $150.00        $700.00

Tracking Software            $25.00        $350.00

Telemetry Decoding Software  $35.00         $35.00

---------------------------------------------------

TOTAL                       $312.00       $2385.00

THE

Consider  the equipment needed to receive signals from  each

of the spacecraft in turn.

6.2 Receiving Signals from DOVE

DOVE  (DO-17) which transmits on a frequency 145.825 MHz  is

the easiest spacecraft to receive usable signals from.  This

frequency  is  available  on most hand  held  scanners,  and

signals  are strong enough to be heard on nothing more  than

the simple antenna provided with the scanning radio when  it

is  purchased.  However, the thrill of  receiving  satellite

signals  wears  off very quickly without any means  to  know

what those signals mean.

A  somewhat  better system is needed  for  reliable  regular

reception  of usable signals. A basic receiving  system  for

DOVE  is  shown in Figure 6.1.  DOVE's  signals  are  strong

enough  that  the ground station does not  need  a  tracking

antenna;  an  omnidirectional antenna  is  sufficient.   The

antenna  can  be  a ground plane, a turnstile  or  a  J-pole

design. A preamplifier should be used to compensate for  any

losses in the cable between the antenna and the receiver, or

any  fades  in the strength of the  received  signals.   Any

scanning radio which receives narrow band FM can be used  as

the  receiver.  This is the same type of modulation used  on

the  public service channels.  If the scanner can  hear  the

police and other services and can tune to 145.825 MHz,  then

it  is capable of receiving signals from DOVE.  The  digital

signals  from  DOVE are encoded as audio tones  and  need  a

modem to convert them to the RS-232 digital signals that can

be  interfaced  to the serial port of a PC.   This  type  of

modem  is known in Radio Amateur circles as a Terminal  Unit

(TU).

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 47

Figure 6.1 Basic Receiving System for DOVE-OSCAR 17.

OMNIDIRECTIONAL

ANTENNA

|

|

\|/

SCANNER

RECEIVER ---------> TNC ---------> COMPUTER

The signals are sent as packets using a modified version  of

the  X.25  protocol called AX.25.  Radio Amateurs  use  this

protocol  for  communications,  and  DOVE  employs  it   for

telemetry  transmission purposes so that any  Radio  Amateur

equipped  for packet radio communications is  also  equipped

for receiving signals from DOVE.

6.3 Receiving Signals from UoSAT-OSCAR 11

The  same  basic  radio receiving  system  used  to  receive

signals  from  DOVE can be used to copy the  telemetry  from

UoSAT-OSCAR 11. This spacecraft however has a lower  powered

transmitter than that of DOVE and consequently has a  weaker

signal  strength  on the ground. This  lower  signal  level,

coupled  with  the fact that the modulation is  plain  ASCII

data means that errors will be seen in the received data due

to  signal fades.  Better antennas are needed  for  reliable

reception, and antennas that track or move and always  point

at the spacecraft are desirable.

The TU used for UoSAT-OSCAR 11 is different to that used for

DOVE  due to the different data encoding (ASCII  instead  of

AX.25).

6.4 Receiving Signals from AMSAT-OSCAR 13

So  far  all the spacecraft considered have been  low  earth

orbits.   AMSAT-OSCAR 13 however is in an  elliptical  orbit

with  a high apogee.  It also downlinks telemetry as  BAUDOT

and  ASCII data. While signals from this spacecraft  can  be

heard  on  the simple DOVE type of  receiving  configuration

with  an omnidirectional receiving antenna, the signals  are

weak  and  barely audible, i.e. they are in  the  noise  and

cannot  be  received  in  usable  form  without  a  tracking

antenna.

6.5 Receiving PSK Modulated Signals in the 70 cm Band

Receiving  signals from AMSAT-OSCAR 16, WEBER-OSCAR  18  and

LUSAT-OSCAR  19   as  well as from  Fuji-OSCAR  20  requires

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 48

somewhat  more complex equipment. These spacecraft  transmit

on  downlink frequencies in the 70 cm or 430 MHz  band.   As

they  use  PSK,  the  receiver  has  to  be  a  conventional

communications receiver. This can be either a communications

receiver   designed   for  that  frequency   range,   or   a

conventional  short  wave  receiver with a  front  end  down

converter.  A PSK modem attached to the TU is also required.

Typical  receiving configurations for these  satellites  are

shown in Figures 6-2 and 6-3.

Figure 6-2 Basic Receiving System for PSK Modulation.

OMNIDIRECTIONAL

ANTENNA

|

|

|

\|/                    PSK MODEM

VHF/UHF                    | |

COMMUNICATIONS                | |

RECEIVER ----------------> TNC -------->  COMPUTER

Figure  6-3  Alternate   Basic  Receiving  System  for   PSK

Modulation.

OMNIDIRECTIONAL

ANTENNA

|

|

DOWNCONVERTER

|

|

SHORT WAVE                 PSK MODEM

COMMUNICATIONS                  | |

RECEIVER                     | |

|                         | |

|-----------------------> TNC --------------> COMPUTER

The difference between the two approaches is that the  first

uses a communications receiver designed for the 70 cm  band;

the second approach uses a general short wave receiver and a

front end down converter.

7.0 Decoding Spacecraft Telemetry

Apart  from UoSAT-OSCAR 11 and WEBER-OSCAR 18 none of  the  OSCAR

spacecraft are designed for "Science" purposes.  Their  telemetry

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 49

consists  of spacecraft housekeeping parameters,  monitoring  on-

board temperatures, voltages and currents.  While much use can be

made of these data, there isn't much real science data available.

Even  the scientific spacecraft are in the main unusable  by  the

average listener because information about the scientific payload

is  not  readily  available.   Let's  make  a  start  with  these

spacecraft, then look to a follow on activity. An OSCAR does  not

have  to be a separate spacecraft. The Soviet Union has  provided

their  amateurs  with payload space aboard two of  their  weather

satellites [8]. NASA could do the same for an amateur  scientific

spacecraft  which would monitor radiation, the  earth's  magnetic

field  and  solar  activity;  such data being  of  use  to  radio

amateurs  for predicting propagation and providing  schools  with

data  about  the  earth's environment.  NASA has  a  'Mission  to

Planet Earth' project to provide an Earth Observation Platform in

1997.   An   attached  secondary  payload   to   that   platform,

transmitting  packetized  scientific telemetry  data  (with  well

publicized formats) in the 145 MHz amateur band or in the 136 MHz

scientific  band could really bring not only the  space  program,

but  the  educational  and scientific use of  space,  into  every

educational  institution in the country.  In the mean time,  this

section discusses the usable OSCAR spacecraft and their telemetry

and the corresponding decoding equations.

The  satellites  have been built by  different  organizations  at

different  times and each uses different data formats.  DOVE  and

Fuji-OSCAR  20 transmit use ASCII Packet format, yet  while  DOVE

transmits  the  data in Hexadecimal format,  Fuji-OSCAR  20  uses

Decimal Format. AMSAT-OSCAR 16, WEBER-OSCAR 18 and LUSAT-OSCAR 19

transmit  their  telemetry  in  pure  binary  format.   By  using

packetization, the data quality is checked in the link itself and

bad packets are not normally passed to the computer from the TNC.

AMSAT-OSCAR 13 does not have any error checking at all, so it  is

up  to the receiving station to visually inspect the data  before

trying  to convert it to engineering units. UoSAT-OSCAR  11  also

transmits  its telemetry as ASCII text, but the designers of  the

spacecraft  recognized that the downlink was prone to  error  and

incorporated a checksum in its data format.

This  section discusses the data formats and  decoding  equations

associated with  several of the OSCAR spacecraft.

7.1 DOVE (DOVE-OSCAR 17)

DOVE-OSCAR 17 was launched January 22, 1990. Its prime mission is

to  provide  an easily received Digital  Orbiting  Voice  Encoded

beacon  for educational and scientific use.  Dr. Torres  'Junior'

de Castro, PY2BJO sponsored the DOVE experiment. DOVE-OSCAR 17 is

licensed  in Brazil with the callsign PT2PAZ.  DOVE-OSCAR 17  was

built by AMSAT, occupies less than a cubic foot of space,  masses

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 50

8.5 kg and contains a V40 microprocessor and 8 Megabytes of  RAM.

Essentially  it is a loaded PC Clone in orbit.   AMSAT-OSCAR  16,

DOVE-OSCAR  17, WEBER-OSCAR  18 and LUSAT-OSCAR 19  are  commonly

known as Microsats and were constructed as a set by AMSAT  during

1989.   Each  of the Microsats contains bar magnets  which  align

them  along  the earth's magnetic field and is spun  around  that

axis by photon pressure from the sun acting on the  communication

antennas  which  are painted white on one side and black  on  the

other.

Unfortunately a combination of two on-board hardware failures and

lack of available manpower in AMSAT (a volunteer organization for

all  practical purposes) have kept DOVE's voice off the air.   At

this  time DOVE only transmits packet telemetry.   DOVE-OSCAR  17

transmits  telemetry  in several different packets  as  shown  in

Figure 7-1.

Figure 7-1 A typical example of a DOVE-OSCAR 17 Telemetry Frame.

23-Jan-91  02:49:23  DOVE-1*>TIME-1:

PHT: uptime is 173/00:36:26.  Time is Wed Jan 23 02:47:30 1991

23-Jan-91  02:49:26  DOVE-1*>TLM:

00:59 01:59 02:87 03:31 04:59 05:5A 06:6E 07:52 08:6D 09:72 0A:A2

0B:DC 0C:E9 0D:D8 0E:02 0F:26 10:CC 11:A8 12:01 13:04 14:AD 15:94

16:98 17:94 18:96 19:98 1A:94 1B:91 1C:9B 1D:98 1E:25 1F:5F 20:BA

23-Jan-91  02:49:27  DOVE-1*>TLM:

21:95 22:82 23:24 24:1E 25:2A 26:01 27:02 28:02 29:01 2A:02 2B:02

2C:01 2D:29 2E:02 2F:9E 30:CA 31:9E 32:11 33:CE 34:C4 35:9A 36:A8

37:A6 38:B6

23-Jan-91  02:49:28  DOVE-1*>STATUS:

80 00 00 8F 00 18 CC 02 00 B0 00 00 0C 0E 3C 05 0B 00 04 04

23-Jan-91  02:49:28  DOVE-1*>LSTAT:

I P:0x3000 o:0 l:13081 f:13081, d:0

23-Jan-91  02:49:28  DOVE-1*>WASH:

wash addr:0680:0000, edac=0xd6

Telemetry  data are transmitted in 57 (3A hex)  channels  in  two

segments  in the TLM packets. The format is AA:BB where  'AA'  is

the hexadecimal channel number and 'BB' the hexadecimal value  of

the telemetry at the time it was sampled.

The telemetry decoding equations for DOVE are shown in Table 7.1-

1. The Equations are in the form of a quadratic equation,

Y = A*N^2 + B*N + C

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 51

where:    N = Telemetry Count (00 - FF);

A, B, C = Equation Coefficients;

Y = Result (In Specified Units).

Note N must be converted from hexadecimal to decimal units before

performing  the calculation.  A sample decoded page is  shown  in

Table 7.1-2.

Table 7.1-1 DOVE Telemetry Decoding Equations

Spacecraft:  DOVE-1:                              Rev: 1

Date: 1/7/90

HEX   Description:      C:           B:           A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx E/F Audio(W)+0.000       +0.0246      0.000       V(p-p)

1    Rx E/F Audio(N)+0.000       +0.0246      0.000       V(p-p)

2    Mixer Bias V:  +0.000       +0.0102      0.000       Volts

3    Osc. Bisd V:   +0.000       +0.0102      0.000       Volts

4    Rx A Audio (W):+0.000       +0.0246      0.000       V(p-p)

5    Rx A Audio (N):+0.000       +0.0246      0.000       V(p-p)

6    Rx A DISC:     +10.427      -0.09274     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +9.6234      -0.09911     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000100    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.05075     0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +101.05      -0.6051      0.000       Deg. C

15    +X (RX) temp:  +101.05      -0.6051      0.000       Deg. C

16    Bat 1 V:       +1.7932      -0.0034084   0.000       Volts

17    Bat 2 V:       +1.7978      -0.0035316   0.000       Volts

18    Bat 3 V:       +1.8046      -0.0035723   0.000       Volts

19    Bat 4 V:       +1.7782      -0.0034590   0.000       Volts

1A    Bat 5 V:       +1.8410      -0.0038355   0.000       Volts

1B    Bat 6 V:       +1.8381      -0.0038450   0.000       Volts

1C    Bat 7 V:       +1.8568      -0.0037757   0.000       Volts

1D    Bat 8 V:       +1.7868      -0.0034068   0.000       Volts

1E    Array V:       +7.205       +0.07200     0.000       Volts

1F    +5V Bus:       +1.932       +0.0312      0.000       Volts

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 52

20    +8.5V Bus:     +5.265       +0.0173      0.000       Volts

21    +10V Bus:      +7.469       +0.021765    0.000       Volts

22    BCR Set Point: -8.762       +1.1590      0.000       Counts

23    BCR Load Cur:  -0.0871      +0.00698     0.000       Amps

24    +8.5V Bus Cur: -0.00920     +0.001899    0.000       Amps

25    +5V Bus Cur:   +0.00502     +0.00431     0.000       Amps

26    -X Array Cur:  -0.01075     +0.00215     0.000       Amps

27    +X Array Cur:  -0.01349     +0.00270     0.000       Amps

28    -Y Array Cur:  -0.01196     +0.00239     0.000       Amps

29    +Y Array Cur:  -0.01141     +0.00228     0.000       Amps

2A    -Z Array Cur:  -0.01653     +0.00245     0.000       Amps

2B    +Z Array Cur:  -0.01137     +0.00228     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: +0.06122     +0.00317     0.000       Amps

2E    BCR Output Cur:-0.01724     +0.00345     0.000       Amps

2F    Bat 1 Temp:    +101.05      -0.6051      0.000       Deg. C

30    Bat 2 Temp:    +101.05      -0.6051      0.000       Deg. C

31    Baseplt Temp:  +101.05      -0.6051      0.000       Deg. C

32    FM TX#1 RF OUT:+0.0256      -0.000884    +0.0000836  Watts

33    FM TX#2 RF OUT:-0.0027      +0.001257    +0.0000730  Watts

34    PSK TX HPA Temp+101.05      -0.6051      0.000       Deg. C

35    +Y Array Temp: +101.05      -0.6051      0.000       Deg. C

36    RC PSK HPA Temp+101.05      -0.6051      0.000       Deg. C

37    RC PSK BP Temp:+101.05      -0.6051      0.000       Deg. C

38    +Z Array Temp: +101.05      -0.6051      0.000       Deg. C

39    S band TX Out: -0.0451      +0.00403     0.000       Watts

3A    S band HPA Temp+101.05      -0.6051      0.000       Deg. C

Table  7.1-2 Sample Decoded (General Housekeeping) Page  of  DOVE

Telemetry

PHT: uptime is 177/12:34:12.  Time is Sun Jan 27 14:45:16 1991

-X Array Cur   : 0.174 A             Array V        :22.829 V

+X Array Cur   : 0.000 A             +Z Array V     :23.836 V

-Y Array Cur   : 0.000 A             Ext Power Cur  : 0.000 A

+Y Array Cur   : 0.000 A             BCR Input Cur  : 0.480 A

-Z Array Cur   : 0.000 A             BCR Output Cur : 0.314 A

+Z Array Cur   : 0.251 A             BCR Set Point  :   119

IR Detector    :    56               BCR Load Cur   : 0.241 A

+Z Array Temp  :   3.0 C

+Y Array Temp  :   4.8 C              Battery 1 V  : 1.330 V

Battery 2 V  : 1.346 V

+2.5V  VREF    : 2.506 V              Battery 3 V  : 1.337 V

Ground REF     : 0.020 V              Battery 4 V  : 1.325 V

Battery 5 V  : 1.350 V

Bat 1 Temp   :   3.0 C               Battery 6 V  : 1.431 V

Bat 2 Temp   : -24.8 C               Battery 7 V  : 1.343 V

TX#1 RF OUT  :   0.0 W               Battery 8 V  : 1.344 V

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 53

TX#2 RF OUT  :   3.7 W

The  TIME-1  packets  identify  the  time  that  the  data   were

downlinked.   The Uptime value in the time packet tells  you  how

long the software has been running (since last upload).

The STATUS packets contain information about spacecraft  on-board

status.  These  bytes, have in the past, been  changed  by  AMSAT

without   prior  or  post  notification  to  the  radio   amateur

community.   The   LSTAT  packet  contains   engineering   status

information which has not been published other than that the last

value "d:0" means the spacecraft is not set up as a digipeater.

The  WASH packets provide engineering information about  the  on-

board RAM memory.

7.2 UoSAT-OSCAR 11

UoSAT-OSCAR  11 which was launched March 1, 1984, is  similar  to

and  is  a follow on to the now reentered UoSAT-OSCAR 9.  It  was

designed and built at the Department of Electronic and Electrical

Engineering  at the University of Surrey, England.  It was  built

to develop scientific experimentation and space education.  While

much invaluable experience has been received by the UoSAT people,

not much has been published in the general educational and  radio

amateur  press about its on-board experiments and telemetry  data

formats.   As such, apart from a small group of dedicated  users,

UoSAT-OSCAR  11  seems to have been ignored by  the  majority  of

radio amateurs and educational institutions.

UoSAT-OSCAR 11 carries four on-board experiments:- a Digital Com-

munications Experiment, a Space Dust Experiment, a Charge Coupled

Device (CCD) Video Camera Experiment and a Digitalker Experiment.

The Digital Communications Experiment demonstrated the concept of

store-and-forward digital communications using spacecraft in  low

earth  orbit.  The Space Dust Experiment measures the  impact  of

dust particles, and calculates the momentum of the particles. The

CCD  Video  Camera Experiment takes pictures of the  earth  at  a

resolution of 384 x 256 pixels with 128 gray levels.  This exper-

iment  does not seem to have returned any usable  pictures.   The

Digitalker  Experiment  provides clear digitized  voice  using  a

fixed vocabulary and is switched on from time to time.

UoSAT-OSCAR   11  transmits  a  number  of  different  types   of

telemetry.  WHATS-UP  can only decode and display  the  real-time

telemetry.  Should you tune in signals from UoSAT-OSCAR  11,  you

may get anything. Hang in there, sooner or later it will transmit

real-time  telemetry, if not on one pass, then on the  next.  The

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 54

real-time telemetry is transmitted as 1200 baud ASCII data  using

FSK.  The  sense of the modulation is inverted  with  respect  to

convention  due  to the wide popularity of a  BBC  computer  tape

interface (which used the inverted modulation) in England at  the

time the spacecraft was built.  This means that either a  special

modem has to be built to receive the data or the PK-232 has to be

modified  before it will copy signals from UoSAT-OSCAR  11.  This

modification is needed because the PK-232 RXR parameter does  not

work   above  300  baud  due  to  a  hardware  limitation.   This

modification  performs  the equivalent of the  RXR  operation  in

hardware by taking advantage of an unused inverter inside the PK-

232.

What  has to be done is to wire U15 pins 1 and 2 to a  switch  in

series  with  the output to JP4. Adding a green LED to  show  the

state  of the switch (at a glance) is optional. The steps are  as

follows:

1. Drill  a 1/4 inch hole in the front panel of the PK-232  above

the red DCD LED.

2. Mount  a Double Pole Double Throw (SPDT) switch on  the  front

panel  of  the PK-232 where it will not interfere  with  other

components  (near the upper left corner by the AEA logo  above

the threshold control).

3. Carefully  cut the circuit board trace connecting U15's pin  6

to the inside pin of JP4.  This trace is easiest to cut  where

it comes out from under R20 on the top (component side) of the

PC  board.  Be sure to double check that this is  the  correct

trace  with an ohm meter.  Cut with care (with a sharp  X-acto

knife) so adjacent traces are not touched.

4. Solder a jumper wire between U15's pins 1 and 6.

5. Connect the center of one pole on this switch to the inner pin

of  JP4  or  the  trace that goes to  it  (which  was  cut  to

disconnect it from U15 pin 6).

6. Connect  the  corresponding  switch  contact  which  will   be

hardware "RXR OFF" to U15 pin 6.

7. Connect  the  corresponding  switch  contact  which  will   be

hardware "RXR ON" to U15 pin 2.

8. Mount  a green LED on the front panel of the PK-232 above  the

red DCD LED.

9. Wire  one leg of the LED to the other pole of the switch,  the

other end to a 1K ohm resistance (test the LED first to  make

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 55

sure you wire it the correct way).

10. Wire the other end of the resistance to +12V near the voltage

regulator.  Do not wire it to the battery back up voltage  (if

the  LED stays lit when you turn the PK-232 off, you wired  it

wrongly).

11. Wire one side of the switch to ground so the green LED lights

when the REVERSE position is selected.

To copy the UoSAT-OSCAR 11 FM AFSK ASCII Telemetry on 145.825 Mhz

set  the  new hardware RXR switch in the "reverse"  position.  Be

sure  to  return  your RXR switch to "normal" when  you  want  to

return to regular operation, as this switch is in the signal path

in all modes when the PK-232's internal modem is used.

A  typical  telemetry  frame starts with a  non  printing   ASCII

character  (1E  hexadecimal) followed by the  identification  and

date.  The  date  code  can be  deciphered  using  the  following

YYMMDDWHHMMSS format where YY is the last two digits of the year,

MM,  the month, DD, the day of the month, W, the day of the  week

(Sunday = 0), the remainder being hours,minutes and seconds.  All

times  are given in UTC. A blank line follows, then follow  seven

lines worth of ten channels per line. The format of each line  is

as shown below.

NNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDCNNDDDC

where NN is the channel number, DDD the data and C a checksum  to

validate  the data.  The checksum is needed because there  is  no

error  checking built into the link. Each channel thus  comprises

six  digits.  The  checksum  is  computed  for  each  channel  by

'exclusive  or'ing  (XOR) each nibble (4 bits) of each of  the  6

characters in that channel. A zero result means that the data was

received  correctly,  a  non-zero  result  means  the  data   was

corrupted.  Figure  7.2-1 contains an example of the  raw  UoSAT-

OSCAR  11  data showing some of the errors due to  noise  on  the

downlink.

Figure 7.2-1 Example  of a Received Raw UoSAT-OSCAR 11  Telemetry

Data Frame

00519D0141370267650361400404660503;4 6019E07045608040C08036C

10519C11298312000313056114069A15529A!6188;175452185905195058

20519F21220322662223000124001725000726093E27541528564D294681

30519E31041732287C33568B34007035217236276637393D38426B39455E

40649F41117242647343061044162545000146000247444748454949422x

50456251108D52634653284p54663215000056p00357451258447A59460E

60826A615FC1625F4A63334164440265160466174267700668000E69000F

UOSAT-2           9101281004625

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 56

7.3 AMSAT-OSCAR 13

AMSAT-OSCAR  13 which was launched June 15, 1988 was built  as  a

joint  venture between radio amateurs in the USA and  in  Germany

organized  as  the Radio Amateur Satellite  Corporation  (AMSAT).

AMSAT-OSCAR  13 is a spin stabilized long life long  range  radio

amateur    communications   satellite   which   provides    daily

intercontinental communications  capability for hours at a  time.

It  contains  a number of analog and  digital  transponders  with

communications links on several frequencies. An on-board computer

based on the RCA 1802 microprocessor controls the spacecraft  and

generates the downlink telemetry.  Schedules are published in the

amateur  radio  press  which  provide  information  as  to  which

transponder is active at any time during the orbit.   AMSAT-OSCAR

13  also  contains a motor which was used by  radio  amateurs  to

boost the spacecraft from the orbit the rocket placed it in  into

its operational orbit.

AMSAT-OSCAR  13  transmits telemetry in a number of ways  on  two

beacons.  The two meter beacon is on a frequency of  145.812  MHz

and  carries  CW,  50  baud RTTY,  and  400  baud  PSK  telemetry

according to a published schedule.  WHATS-UP can only decode  and

display  the 50 baud RTTY telemetry which is transmitted  in  the

form  of Z blocks shown in Table 7.3-1.  The block is  identified

as  a 'Z' block by the letter 'Z' before the words "HI.  THIS  IS

AMSAT OSCAR 13" which identify the spacecraft.

The block starts with the letter 'Z' in the first line.  Time and

status  information  follows, then the  first  telemetry  channel

begins several lines down and is shown with the value of 193. Six

lines   of  telemetry,  each  line  containing  ten  values   are

transmitted,  with  a  blank  line  separating  the  two  halves.

Following the telemetry lines, the spacecraft may transmit  plain

text information of general interest.  Although the Z block  only

contains  60 channels, the equations for decoding all 128 of  the

telemetry  channels  are presented in Table 7-3.2  in  which  the

Channel Numbers are shown in Hexadecimal.

Table 7.3-1 AMSAT-OSCAR 13 RTTY Telemetry Block

Z  HI. THIS IS AMSAT OSCAR 13

05.02.54  8661

.0086   .0000   .07B9

64  6   0   1   16  218 1

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 57

193 170 158 143 181 144 147 140 200 7

147 7   7   7   165 29  100 7   149 7

10  7   145 115 34  7   153 129 122 180

152 73  7   145 137 55  7   183 136 151

7   154 137 169 211 142 127 100 9   140

161 7   173 149 150 154 14  131 127 210

HI THIS IS AMSAT OSCAR 13 08SEP90

NEW AO13 SCHEDULE FROM 17OCT90 AFTER MOVE TO LON 180  LAT 0

MODE B  MA 000 TO 095

MODE JL MA 095 TO 125

MODE LS MA 125 TO 130

MODE S MA 130 TO 135

MODE BS MA 135 TO MA 140

MODE B  MA 140 TO 256

The Bytes in the block are identified in the followings ways:

C  - unsigned count (0 to +255)

Cs - signed count (-128 to +127, 2s complement)

Cx - signed count (+63 to -192, #3F=+63, #FF=-1, #80=-128,

#7F=-129, #40=-192, modified 2s complement)

All  temperature  channels  are  decoded  identically  using  the

equation T = (C-120)/1.71  (in Degrees Centigrade).  All channels

measuring   currents  use  a  linear  equation   with   different

calibrations  constants.  Three  equations  are  used   providing

maximum current values of 1A, 2.5A and 5A, as follows:

1A:  I = (C-15)*4.854 mA

2.5A:  I = (C-15)*12.135mA

5A:  I = (C-15)*24.27 mA

Table 7.3-2 AMSAT-OSCAR 13 Telemetry Decoding Equations

#    Label          Equation             Remarks

00   Uin-BCR        U=(C-10)*167mV   U-Panel: +0.6V @ 0.35A

+0.7V @ 1A

01   Tx-PWRout-L    Average power=(261-C)^2 / 724 Watts

rectified envelope voltage.

02   T-Rx-U         Temp             Mode-B receiver temperature.

03   ---

04   Uout-BCR       U=(C-10)*79.5mV  BCR output voltage.

05   ---

06   T-TX-U         Temp             Mode-B transmitter

temperature.

07   I-14V-ST       5A               Transponder separation bus.

08   U-10V-C        U=(C-10)*53.2mV  Computer supply

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 58

09   Press He-Hi    off              Helium tank pressure.

0A   T-IHU          Temp             Integrated Housekeeping Unit

0B   I-14V-S        1A               Separation bus, 14V that

supplies

torquer and LIU.

0C   BCR-Oscill1    >~6=OK           BCR status. No count = not

working. Typically C=80.

0D   Press He-Lo    off              Helium regulator output.

0E   T-BCR          Temp             Battery Charge Regulator.

0F   I-10V-C        1A               10V continuous power supply.

10   BCR-Oscill2    As channel 0C

11   Press Tank     off              N2O4 tank pressure.

12   T-SEU          Temp             Sensor Electronics Unit

13   IbatCharge     2.5A             Positive Current to battery.

14   L-Sensor (A)   U=(C-10)*8.53mV  Light-Sensor Antenna Side.

15   Motor Valve    off

16   T-ABAT1        Temp             Auxiliary battery 1

17   I-BCR-OUT      5A               14V line to battery and

other consumers.

18   L-Sensor (M)   U=(C-10)*8.53mV  Light-Sensor Motor Side.

19   ---

1A   T-ABAT2        Temp             Auxiliary battery 2

1B   I-BCR-IN       Equivalent to total current of all panels,

not available due to sensor failure.

1C   Spin rate      C>131,  Spin rate=479/(C-109)-2 rpm

C<=131, Spin rate=(131-C)*0.85+20 rpm

Sensor angular position

oscillator. Lock indication

channel #47.

1D   Rx-L-AGC       Gain reduction=(C-75)^2 / 1125 dB

1E   T-MBAT         Temp             Mean battery (normally in

use)

1F   I-Panel6       1A               Solar panel 6

20   Tx-PWRout-U    Average power=(287-C)^2 / 1796 Watts

As channel 01.

21   T-He-Tank      Temp

22   T-Panel1       Temp

23   I-Panel5       1A

24   Rx-U-AGC       Gain reduction=(C-71)^2 / 2465 dB

25   T-Tx-L         Temp             Mode-JL transmitter.

26   T-Panel3       Temp

27   I-Panel4       1A

28   ---

29   T-Rx-L         Temp             Mode-L receiver.

2A   T-Panel5       Temp

2B   I-Panel3       1A

2C   U-14V-ST       U=(C-10)*66.8mV  Transponder separation bus.

2D   T-RUDAK        Temp             RUDAK temperature.

2E   T-top          Temp             Arm 1, Antenna side.

2F   I-Panel2       1A

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 59

30   U-9V-U         U=(C-10)*54mV    Internal 9 volt bus from

Mode-B transponder.

31   T-wall-arm2    Temp

32   T-bottom       Temp             Arm 1, Motor side.

33   I-Panel1       1A

34   ---

35   T-wall-arm1    Temp

36   T-N2O4         Temp

37   ---

38   U-ABAT         U=(C-10)*78.5mV  Auxiliary battery.

39   T-S-xpnder     Temp             Mode-S transponder.

3A   T-L-Sensor     Temp             Light sensor antenna side.

3B   ---

3C   U-9V-L         U=(C-10)*45.4mV  As channel 30, Mode-L

3D   T-AZ50-Tank    Temp

3E   T-nutation damper  Temp         Arm 3.

3F   ---

40   ES-Sensitivity 2MUX0            Earth Sensor sensitivity

threshold.

bit   significance

0 LSB  20mV

1      37mV      Hysteresis 200mV

2      75mV      Threshold  600mV

3     150mV

4     300mV

5     600mV

6     1.2V

7 MSB 2.4V

41   Antenna/SERI   2MUX1            Antenna relays and SERI

resistors.

bit   significance

0 LSB Hi-gain  2m  to U

1     Hi-gain 70cm to L

2     --+ SERI-1  load resistor for

3     -+! SERI-2  both Light-Sensors

!! resistance

00  7.5 Ohm

01  3.9 Ohm

10  2.3 Ohm

11  5.9 Ohm

42   RUDAK-Status   2MUX2            IN-B (ex Motor-PWR).

C=82, Standard-ROS  (S)

C=78, Emergency-ROS (N, Not-ROS)

C=80, Primitive-ROS (P)

43   S&RUDAK-CNTL   2MUX3            Mode-S and RUDAK interface.

bit   significance

0 LSB RUDAK OFF

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 60

1       "   NMI/

2       "   Byte Clock

3       "   Byte count reset

4      ---

5     Mode-S Beacon ON

6       "    Squelch defeated

7 MSB   "    Squelch Hi Sensitivity

44   BCR-Sin        2MUX4            Array voltage offset.

U=29.1+(Cs*100)mV  (16.3V...41.8V)

45   BCR-Sout       2MUX5            Battery knee voltage offset.

U=14.98+(Cx*20)mV  (11.14...16.2V = 192...63)

46   BCR-relays     2MUX6

bit   significance

0 LSB BCR-2 ON

1     Auxiliary battery charging

2     Auxiliary battery connected,

Main battery disconnected.

47   SS-1           C=255 or C=0, PLL locked

Sun-Sensor angular position

oscillator, Slit antenna

side.

48   SS-2           Time offset from SS-1

49   Flag-SS        C=1, SS-1        Sun sensor data.

C=2, SS-2

4A   SPIN-RAW                        Raw spin count (1/256).

4B   Sensor-control bit   significance  (OUT4)

0 LSB --+ MUX-CTRL for sensor elec. module

1     -+!

!!

00 - Sun data

01 - spin ref./spin counter

10 - ES lower beam

11 - ES upper beam

2     Earth sensor positive edge select.

(Strobes value of spin count at

transition.)

3     Motor Instrumentation ON.

(Pressure sensors, motore valve

indicator.)

4     0.3V Sun Sensor Sensitivity

5     0.6V  "    "         "

6     1.2V  "    "         "

7 MSB 2.4V  "    "         "

(Max threshold #F = 1 solar constant)

4C   SS-correction

4D   Last ES-A      Z                last ES-pulse Antenna side.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 61

4E     "            O#               (Orbit number and MA value)

4F   Last ES-M      Z                last ES-pulse Motor side.

50     "            O#               (Orbit number and MA value)

51   Lockoutrange   Within +- C counts from sun sensor pip, earth

sensor handler ignores data (Spin count 1

circle = 256 counts.)

52   ES-A           Strobed spin count at edge selected,

Antenna Side beam.

53   Update Flag1   Indicates update, Antenna Side beam.

54   ES-M           As channel 52     Motor Side beam.

55   Update Flag2   As channel 53      "    "    "

56   S/C STATUS     bit   significance

0 LSB LIU power ON

1     S/A plug status 0=Safe, 1=Arm

2     RUDAK-out (lock)

3     Mode-S Squelch open

4     ---

5     Memory Soft error Counter

6       "         "      "

7 MSB   "         "      "

57   ---

58   ---

59   ---

5A   ---

5B   N              no of 20ms per dot, morse speed.

5C   n              running count of units for morse.

5D   ---

5E   TRANSPONDER    bit   significance  (OUT7)

0 LSB GB OFF

1     GB FSK (1=+170Hz)

2     DPSK OFF

3     EB ON

4     --+ PSK source

5     -+! for GB (EB: don't care)

!!

00 - no PSK

01 - ranging

10 - EB source

6     Low power transponder ON

7 MSB Passband OFF (Beacons and Mode-J +3dB)

5F   ---

60   MODUS          bit   significance  (magnet control)

0     magnet system ON

1     underspin magnet

61   M-Soll         magnet vector desired angle to the despun sun

(clockwise as seen from top, 1 circle = 256)

62   M-Out          bit significance (OUT3, also L,S,J control)

0 LSB polarity Arm 1

1     polarity Arm 2

2     polarity Arm 3

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 62

3     Magnet power ON

4     Mode-J ON

5     ---

6     Mode-S ON

7 MSB Mode-L ON

63   O-FRAC-lo      Fractional Z increment in 20ms

64   O-FRAC-hi                       Counts down to 0 from preset

value. 255th Z has different

value of O-FRAC. ~7000

counts/Z.

65   O/256          Z from perigee

66   O#-lo          Orbit number

67   O#-hi

68   UHR            10ms             UTC

69                  sec

6A                  min

6B                  hour

6C                  day              1st  January  1978 = AMSAT

day 0.

6D                  256day

6E   SU0            10ms             IPS stopwatch 0.

6F                  sec

70                  min

71                  min*256

72   SU1            10ms             IPS stopwatch 1.

73                  sec

74                  min

75                  min*256

76   SU2            10ms             IPS stopwatch 2.

77                  sec

78                  min

79                  min*256

7A   SU3            10ms             IPS stopwatch 3.

7B                  sec

7C                  min

7D                  min*256

7E   Event-Lo                        Used as event ID word in

7F   Event-Hi                        intermediate buffer.

7.4 AMSAT-OSCAR 16, WEBER-OSCAR 18 and LUSAT-OSCAR 19

AMSAT-OSCAR  16, WEBER-OSCAR 18 and LUSAT-OSCAR 19 were  launched

January 22, 1990 together with DOVE-OSCAR 17.  AMSAT-OSCAR 16 and

LUSAT-OSCAR 19 are designed to provide a platform for experiments

with  digital  store-and-forward communications techniques  as  a

follow-on to the Digital Communications Experiment of UoSAT-OSCAR

11.   LUSAT-OSCAR 19 is sponsored by AMSAT in  Argentina.  WEBER-

OSCAR  18 is an engineering project of the Center  for  Aerospace

Studies at Weber State University in Utah.  It has the capability

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 63

for  digital communications but is not used as such. It  contains

an on-board video camera which has returned pictures of the earth

using  a non standard format picture transmission format.  WEBER-

OSCAR  18 also carries a number of experiments. The  Spectrometer

experiment  is  designed  to observe  the  spectrum  of  sunlight

reflected  off the earth's atmosphere and surface.  The  Particle

Impact Detector is a piezoelectrical crystal mounted on the  side

of  the spacecraft which produces an output voltage each  time  a

microparticle   impact  occurs.   The   Magnetometer   Experiment

contains two orthogonal flux gate magnetometers. As they were not

calibrated  they  can  only provide  information  about  relative

changes in the magnetic environment of the spacecraft. As in  the

case  of  UoSAT-OSCAR 11, data about the  experiments  and  their

telemetry  calibrations is lacking in the general  amateur  radio

press.

These spacecraft downlink telemetry in the 437MHz band similar to

that  of  DOVE but using a BINARY format in  an  unpublished  and

apparently  "changeable  at any time  without  notice"  sequence.

WHATS-UP  intercepts  the binary telemetry and converts it  to  a

pseudo  DOVE format as shown in Table 7.4-1 before  decoding  and

displaying   the  information.   The  decoding  equations   first

published by AMSAT in The AMSAT Journal are given in Tables  7.4-

2, 7.4-3 and 7.4-4.

Table 7.4-1 Example of Pseudo DOVE Display of Intercepted  Binary

Telemetry and other intercepted Microsat Packets

27-Jan-91  17:04:18  LUSAT-1*>TIME-1:

PHT: uptime is 005/14:46:21.  Time is Sun Jan 27 17:03:13 1991

27-Jan-91  17:04:22  LUSAT-1*>AMARG:

Jan 23, 1600Z.

AART driver loaded.

Reload will continue in a short time

73, LU7XAC

AMARG Control Team

27-Jan-91  16:41:11  N8ITP>PACSAT-1*>N8ITP [D]

27-Jan-91  16:41:12  N8ITP>PACSAT-1*>N8ITP (UA)

27-Jan-91  16:41:50  PACSAT-1*>TIME-1:

PHT: uptime is 004/10:14:01.  Time is Sun Jan 27 16:40:51 1991

27-Jan-91  16:41:54  PACSAT-1*>AMSAT:

Jan 22, 8:03 UTC - Reload in progress.  Digi on.  No BBS till the

weekend.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 64

NK6K

27-Jan-91  16:41:54  PACSAT-1*>LSTAT:

I P:0x3000 o:0 l:13884 f:13884, d:1 st:0

27-Jan-91  16:42:48  N8ITP>PACSAT-1*>N8ITP [C]

27-Jan-91  16:42:50  N8ITP>PACSAT-1*>N8ITP (UA)

27-Jan-91  16:42:55  N8ITP>PACSAT-1*>N8ITP:

test1122334455

27-Jan-91  16:44:47  KC4EBR>PACSAT-1*>KC4SA:

What's shaking?>

27-Jan-91  17:04:22  LUSAT-1*>BCRXMT:

vmax=762169 battop=766771 temp=218292

27-Jan-91  17:06:31  LUSAT-1*>LSTAT:

I P:0x3000 o:0 l:13417 f:13417, d:0 st:2

27-Jan-91  17:06:38  LUSAT-1*>TIME-1:

PHT: uptime is 005/14:48:41.  Time is Sun Jan 27 17:05:33 1991

27-Jan-91  17:06:41  LUSAT-1*>TLM:

00:DC 01:0E 02:30 03:0F 04:3A 05:10 06:DE 07:11 08:84 09:12

0A:00 0B:13 0C:E4 0D:14 0E:A9 0F:15 10:A8 11:16 12:6C 13:17

14:64 15:18 16:69 17:19 18:69 19:1A 1A:66 1B:1B 1C:6D 1D:1C

1E:5F 1F:1D 20:62 21:1E 22:D9 23:1F 24:62 25:20 26:BA 27:21

28:B0 29:22 2A:79 2B:23 2C:2C 2D:24 2E:24 2F:25 30:28 31:00

27-Jan-91  17:09:22  LUSAT-1*>BCRXMT:

vmax=762523 battop=766771 temp=198465

14-Feb-91  03:02:29  N4HY>LUSAT-1*>N4HY (UA)

14-Feb-91  03:02:38  N4HY>LUSAT-1*>N4HY:

sdfjal;jfal;sdfjal;sfjalfj;aldfjl;asdfjl;asdfj;alfjal;sd

14-Feb-91  03:02:48  N4HY>LUSAT-1*>N4HY [D]

14-Feb-91  03:02:49  N4HY>LUSAT-1*>N4HY (UA)

14-Feb-91  03:03:09  LUSAT-1*>COMLUS:

COMMAND 9 ACK

14-Feb-91  03:06:51  LUSAT-1*>STATUS:

31 34 2D 46 65 62

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 65

Table 7.4.2 AMSAT-OSCAR 16 Telemetry Decoding Equations

Spacecraft:  PACSAT-1:                            Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:        A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +9.202       -0.08990     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +9.179       -0.09277     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +9.837       -0.08838     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +9.779       -0.09144     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.817      -0.09911     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000250    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.0500      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +101.05      -0.6051      0.000       Deg. C

15    +X (RX) temp:  +101.05      -0.6051      0.000       Deg. C

16    Bat 1 V:       +1.8225      -0.0038046   0.000       Volts

17    Bat 2 V:       +1.9418      -0.0046890   0.000       Volts

18    Bat 3 V:       +1.8699      -0.0041641   0.000       Volts

19    Bat 4 V:       +1.7403      -0.0032880   0.000       Volts

1A    Bat 5 V:       +1.8792      -0.0042492   0.000       Volts

1B    Bat 6 V:       +2.0499      -0.0054532   0.000       Volts

1C    Bat 7 V:       +1.9062      -0.0045331   0.000       Volts

1D    Bat 8 V:       +1.7536      -0.0033192   0.000       Volts

1E    Array V:       +8.055       +0.06790     0.000       Volts

1F    +5V Bus:       +2.035       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.464       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 66

22    BCR Set Point: -6.1130      +1.1270      0.000       Counts

23    BCR Load Cur:  -0.0477      +0.00767     0.000       Amps

24    +8.5V Bus Cur: -0.00179     +0.000894    0.000       Amps

25    +5V Bus Cur:   -0.00104     +0.00406     0.000       Amps

26    -X Array Cur:  -0.00995     +0.00243     0.000       Amps

27    +X Array Cur:  -0.02370     +0.00254     0.000       Amps

28    -Y Array Cur:  -0.02220     +0.00273     0.000       Amps

29    +Y Array Cur:  -0.01810     +0.00259     0.000       Amps

2A    -Z Array Cur:  -0.02230     +0.00221     0.000       Amps

2B    +Z Array Cur:  -0.02000     +0.00232     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.02345     +0.00355     0.000       Amps

2E    BCR Output Cur:+0.00869     +0.00303     0.000       Amps

2F    Bat 1 Temp:    +101.05      -0.6051      0.000       Deg. C

30    Bat 2 Temp:    +101.05      -0.6051      0.000       Deg. C

31    Baseplt Temp:  +101.05      -0.6051      0.000       Deg. C

32    PSK TX RF Out: -0.0291      +0.00361     +0.0000869  Watts

33    RC PSK TX Out: +0.0055      +0.00172     +0.0001180  Watts

34    PSK TX HPA Temp+101.05      -0.6051      0.000       Deg. C

35    +Y Array Temp: +101.05      -0.6051      0.000       Deg. C

36    RC PSK HPA Temp+101.05      -0.6051      0.000       Deg. C

37    RC PSK BP Temp:+101.05      -0.6051      0.000       Deg. C

38    +Z Array Temp: +101.05      -0.6051      0.000       Deg. C

39    S band TX Out: -0.0088      +0.00435     0.000       Watts

3A    S band HPA Temp 0.000       +1.000       0.000       Counts

Table 7.4.3 WEBER-OSCAR 18 Telemetry Decoding Equations

Spacecraft:  WEBER-1:                            Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:          A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +11.087      -0.08949     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +10.322      -0.09448     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +10.348      -0.09004     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 67

6    Rx A DISC:     +11.387      -0.09535     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.746      -0.09348     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.03523     0.000       Volts

B    +5V Rx Current:+0.000       +0.000234    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0133      0.000       Volts

D    8.5V BUS:      +0.000       +0.0524      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000033    0.000       Amps

10    +10V Bus:      +0.000       +0.0767      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +100.01      -0.5980      0.000       Deg. C

15    +X (RX) Temp:  +100.01      -0.5980      0.000       Deg. C

16    Bat 1 V:       +1.8292      -0.0037196   0.000       Volts

17    Bat 2 V:       +1.8202      -0.0036943   0.000       Volts

18    Bat 3 V:       +1.8050      -0.0036721   0.000       Volts

19    Bat 4 V:       +1.8576      -0.0038979   0.000       Volts

1A    Bat 5 V:       +1.8095      -0.0037439   0.000       Volts

1B    Bat 6 V:       +1.8979      -0.0041754   0.000       Volts

1C    Bat 7 V:       +1.8246      -0.0038126   0.000       Volts

1D    Bat 8 V:       +1.7486      -0.0030475   0.000       Volts

1E    Array V:       +7.800       +0.06790     0.000       Volts

1F    +5V Bus:       +1.838       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.793       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

22    BCR Set Point: -6.1963      +1.1277      0.000       Counts

23    BCR Load Cur:  -0.0405      +0.00620     0.000       Amps

24    +8.5V Bus Cur: +0.00384     +0.000830    0.000       Amps

25    +5V Bus Cur:   -0.00763     +0.00394     0.000       Amps

26    -X Array Cur:  -0.00140     +0.00210     0.000       Amps

27    +X Array Cur:  +0.00946     +0.00226     0.000       Amps

28    -Y Array Cur:  -0.01018     +0.00224     0.000       Amps

29    +Y Array Cur:  -0.01168     +0.00239     0.000       Amps

2A    -Z Array Cur:  -0.01516     +0.00237     0.000       Amps

2B    +Z Array Cur:  -0.02111     +0.00239     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.02189     +0.00332     0.000       Amps

2E    BCR Output Cur:-0.03019     +0.00327     0.000       Amps

2F    Bat 1 Temp:    +100.01      -0.5980      0.000       Deg. C

30    Bat 2 Temp:    +100.01      -0.5980      0.000       Deg. C

31    Baseplate Temp:+100.01      -0.5980      0.000       Deg. C

32    PSK TX RF Out: +0.2104      -0.01203     +0.0001786  Watts

33    RC PSK TX Out: +0.0340      -0.00969     +0.0002198  Watts

34    PSK TX HPA Temp+100.01      -0.5980      0.000       Deg. C

35    +Y Array Temp: +100.01      -0.5980      0.000       Deg. C

36    RC PSK HPA Temp+100.01      -0.5980      0.000       Deg. C

37    RC PSK BP Temp:+100.01      -0.5980      0.000       Deg. C

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 68

38    +Z Array Temp: +0.0000      +1.0000      0.000       Counts

Table 7.4.5 LUSAT-OSCAR 19 Telemetry Decoding Equations

Spacecraft:  LUSAT-1:                             Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:         A:       Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +9.802       -0.08779     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +8.429       -0.09102     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +9.291       -0.08317     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +9.752       -0.08310     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.110      -0.08610     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000250    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.0508      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +93.24       -0.5609      0.000       Deg. C

15    +X (RX) Temp:  +93.24       -0.5609      0.000       Deg. C

16    Bat 1 V:       +1.7343      -0.0029740   0.000       Volts

17    Bat 2 V:       +1.7512      -0.0032113   0.000       Volts

18    Bat 3 V:       +1.7790      -0.0034038   0.000       Volts

19    Bat 4 V:       +1.7286      -0.0030036   0.000       Volts

1A    Bat 5 V:       +1.8114      -0.0036960   0.000       Volts

1B    Bat 6 V:       +1.7547      -0.0032712   0.000       Volts

1C    Bat 7 V:       +1.7151      -0.0030739   0.000       Volts

1D    Bat 8 V:       +1.6846      -0.0028534   0.000       Volts

1E    Array V:       +8.100       +0.06790     0.000       Volts

1F    +5V Bus:       +2.035       +0.0312      0.000       Volts

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 69

20    +8.5V Bus:     +5.614       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

22    BCR Set Point: +3.7928      +1.0616      0.000       Counts

23    BCR Load Cur:  -0.0244      +0.00628     0.000       Amps

24    +8.5V Bus Cur: +0.00412     +0.000773    0.000       Amps

25    +5V Bus Cur:   +0.02461     +0.00438     0.000       Amps

26    +X Array Cur:  -0.01614     +0.00232     0.000       Amps

27    -X Array Cur:  -0.01158     +0.00238     0.000       Amps

28    -Y Array Cur:  +0.00278     +0.00206     0.000       Amps

29    +Y Array Cur:  +0.00136     +0.00218     0.000       Amps

2A    -Z Array Cur:  +0.00370     +0.00209     0.000       Amps

2B    +Z Array Cur:  -0.00793     +0.00216     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.00901     +0.00283     0.000       Amps

2E    BCR Output Cur:+0.00663     +0.00344     0.000       Amps

2F    Bat 1 Temp:    +93.24       -0.5609      0.000       Deg. C

30    Bat 2 Temp:    +93.24       -0.5609      0.000       Deg. C

31    Baseplt Temp:  +93.24       -0.5609      0.000       Deg. C

32    PSK TX RF Out: +0.1059      +0.00095     +0.0000834  Watts

33    RC PSK TX Out: +0.0178      +0.00135     +0.0000833  Watts

34    PSK TX HPA Temp+93.24       -0.5609      0.000       Deg. C

35    +Y Array Temp: +93.24       -0.5609      0.000       Deg. C

36    RC PSK HPA Temp+93.24       -0.5609      0.000       Deg. C

37    RC PSK BP Temp:+93.24       -0.5609      0.000       Deg. C

38    +Z Array Temp: +93.24       -0.5609      0.000       Deg. C

39    LU Bcn Temp A: +93.24       -0.5609      0.000    *  Deg. C

3A    LU Bcn Temp D: +93.24       -0.5609      0.000    ** Deg. C

3B    Coax Rly Stat: +0.000       +1.0000      0.000       Counts

3C    Coax Rly Stat: +0.000       +1.0000      0.000       Counts

* Note 1:  Thermistor located near box center adjacent to LU

thermistor channel no. 5.

**  Note  2:   Thermistor  located  near -X face of  box  on  the

experiment baseplate.

7.5 Fuji-OSCAR 20

Fuji-OSCAR  20  which  was  launched on February  7,  1990  is  a

communications   satellite   in   low   earth   orbit   providing

simultaneous analog and digital communications capability.  Fuji-

OSCAR  20 was built in Japan for Japanese radio amateurs  and  is

the second Japanese built OSCAR.

On  February  7 1990, the National Space  Development  Agency  of

Japan (NASDA) put the Marine Observation Satellite (MOS) 1b  into

orbit.  The launch vehicle also carried two secondary   payloads,

Fuji-OSCAR 20 and the Deployable Boom and Umbrella Test   (DEBUT)

spacecraft which is similar in shape and weight to Fuji-OSCAR 20.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 70

MOS-1b  was  placed into a circular polar orbit, then  DEBUT  and

Fuji-OSCAR 20 separated from the launch vehicle at  0233,   above

Santiago, Chile. First signals from the spacecraft were  received

in Tokyo around 0309 UTC.

Fuji-OSCAR  20  is similar in construction to Fuji-OSCAR  12.  In

fact, much of it was originally constructed as a backup to  Fuji-

OSCAR 12 and designated as JAS-1B. It has since been modified and

improved as a result of the lessons learned during the flight  of

Fuji-OSCAR  12.  Fuji-OSCAR 12 was known as Fuji-1 in  Japan,  so

this  spacecraft is known by the Japanese as Fuji-2 and as  Fuji-

OSCAR 20 by the rest of the world.

Fuji-OSCAR 20's  planned service life is 5 years.  It  is  in   a

sunsynchronous elliptical polar orbit, having a perigee of  about

900  km  and  an apogee of about 1740 at  an  inclination  of  99

degrees.  The  Period of the orbit is about  105  minutes.   This

orbit  is  optimal  for MOS-1b which is  to  study  oceanographic

resources and observe agricultural environmental conditions.   In

this  orbit, the spacecraft passes over a given line of  latitude

at  approximately  the same time each day.  In  this  orbit,  the

spacecraft is shielded from the sun by the earth for about 33% of

the  time.  This  eclipse means that the  solar  cells  can  only

provide power for about 70 minutes in each orbit and that the on-

board  nickel  cadmium  storage  batteries  have  to  power   the

spacecraft for the remaining 35 minutes.

Fuji-OSCAR 20  weighs about 50 kg. and is  a  polyhedron   shaped

spacecraft  440mm  in  diameter and 470mm in  height  covered  by

approximately  1500  gallium arsenide solar cells  which  provide

about  11  Watts of power to keep the 11  series-connected  NiCad

cells  (rectangular) with a capacity of 6 AH charged.  There  are

26  sides to the polyhedron which almost makes it  spherical  for

all  practical  purposes other than sticking solar cells  to  it.

Fuji-OSCAR  12  was  the same shape but only  carried  about  600

cells.  This larger number of cells means that Fuji-OSCAR 20  has

a positive power budget and should not need to be switched off to

recharge.

The  Power  supply converts the raw bus voltage of +11 to  +18  V

(+14 V average) to the three regulated voltages (+10 V, +5 V,  -5

V)  used by the rest of the satellite with an efficiency  greater

than 70%.

The  attitude of the satellite is maintained by using the  torque

generated  by the interaction of two permanent magnets  with  the

earth's  magnetic field. This is a fairly conventional  technique

used  in the OSCAR series. Temperature stability is  achieved  by

using thermal insulation.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 71

Fuji-OSCAR 20 carries two Mode J transponders, both of which  may

be operational at the same time.  One transponder is analog (Mode

JA), the other is digital (Mode JD).

The  frequencies and capabilities of the analog  transponder  are

similar  to those of Fuji-OSCAR 12.  It consists of  an  inverted

hetrodyne  linear  translator  with  a  passband  100  kHz  wide,

operating  with a mode J Uplink passband of 145.9 to 146.00  MHz,

and a corresponding Downlink Passband of 435.9 to 435.8 MHz.  The

spacecraft  has a Transmitter Output of approximately 1 watt.   A

ground station needs an Uplink EIRP of about 100 W to communicate

through  the  transponder.  The JA telemetry  beacon  is  on  the

nominal frequency of 435.795 MHz with a power output of about 100

mW and can use CW or PSK modulation.  Fuji-OSCAR 20 is using  the

callsign  8J1JBS and the beacons transmit telemetry in  the  same

manner as Fuji-OSCAR 12.

The   digital  transponder  provides   store-and-forward   packet

communication  using  AX.25  link  level  protocol,  version   2.

Stations   who  used Fuji-OSCAR 12 are able to use Fuji-OSCAR  20

without making any modifications to their equipment.  The  uplink

requires Bi-phased Manchester code on an FM signal, at a bit rate

of   1200  bps. There are four Uplink  Frequencies:  145.85  MHz,

145.87 MHz, 145.89 MHz, 145.91 MHz.  The necessary ground station

Uplink  EIRP is also about 100 W.  The transponder has an  output

power of about 1 W on a downlink frequency of 435.91 MHz and uses

NRZI PSK at 1200 bps. The same PSK modem used to copy  Fuji-OSCAR

12  or  the  Microsats is needed to  copy  Fuji-OSCAR  20.    The

downlink channel also carries packet telemetry.

The 144 MHz receiving antenna is a ring turnstile mounted at  the

bottom of the side panels. The 435 MHz transmitting antenna is  a

turnstile antenna mounted at the top of satellite.  Both antennas

are  circularly  polarized.   Ground tests have  shown  that  the

transmitting  antenna is more omnidirectional than that of  Fuji-

OSCAR 12, however due to the structure of the  hybrid   circuitry

which  allow  both transponders to share the  same  antenna,  the

sense  of the circular polarization on the downlink is  different

for  each  mode.   As  the  apparent  polarization  is  different

depending  on  the  geometry  between  the  spacecraft  and   the

groundstation, you will probably have to change between left hand

and   right  hand  circular  polarization  during  a  pass.   The

spacecraft  is designed so that you can usually keep  the  uplink

and downlink polarization the same.

Mode  JA has provided strong transatlantic signals and  many   CW

and  SSB QSOs. Mode-JD was switched on for the first time  during

Orbit #95.  To Digipeat via Fuji-OSCAR 20 you don't need to use a

digipeater  call.  With the present version of the software,  all

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 72

AX.25  frames  with a valid CRC heard by the spacecraft  will  be

digipeated.

The  spacecraft also carries a BBS which is accessed by means  of

the same commands used to access a terrestrial WA7MBL/W0RLI/AA4RE

type  of BBS.  You access the BBS by connecting to 8J1JBS on  any

of the four uplink channels. When you do connect to it, make sure

that you disconnect before LOS because Fuji-OSCAR 20 only  allows

16  simultaneous connections. Stations that hang in  there  after

the  satellite  drops below their local horizon block  access  by

other  stations  and  have been  christened  'Zombies'.  The  BBS

program  is  a modified version of the BBS  program  written  for

Fuji-OSCAR 12 and allows the use of 4 banks (1Mbyte) of memory. A

typical list of messages copied by KI6QE is shown in Figure  7.5-

1.

Figure  7.5-1  Typical Message Listing from the  BBS  (copied  by

KI6QE)

Fuji-OSCAR 20/JAS1b Mailbox ver. 2.00

commands [B/F/H/M/R/U/W]

Use H command for Help

JAS>JAS>NO.  DATE   UTC    FROM    TO      SUBJECT

0086 04/13 05:15  WB6GFJ  W6SHP   Welcome

0085 04/13 05:14  WB6LLO  KI6QE   SOFTWARE

0084 04/13 05:14  WB6GFJ  W9FMW   Our Chat

0082 04/13 03:38  W9FMW   WA4EJR  MESSAGE ON CIS

0080 04/13 03:36  KG6EX   N1GCR   From Ashley

0078 04/13 03:32  KG6EX   KD8SI   From Ashley

0077 04/13 03:31  KG6EX   N8AM    From Ashley

0076 04/13 03:30  KG6EX   DD4YR   From Ashley

0075 04/13 03:27  KG6EX   DL1CR   From Ashley

0074 04/13 03:25  KG6EX   G3RUH   From Ashley

---------------------------------------------------------

The  spacecraft telemetry is transmitted either as CW or as  PSK.

The  CW  telemetry monitors 12 analog data points and  33  status

points, the PSK telemetry monitors 29 analog data points and   33

status points.  Telemetry data from Fuji-OSCAR 20 is  transmitted

on both the mode JA  and JD beacons. Mode JA sends data by  Morse

code  on the beacon signal of  435.795  MHz, repeating one  frame

every  one  minute.  Mode  JD sends a telemetry  packet  every  2

seconds on the digital  downlink channel of 435.91  MHz when  the

telemetry  mode is operating, otherwise, one frame is  downlinked

every one minute. The spacecraft  can downlink up to 30 items  of

data  and  31  items  of status in the telemetry.   The  Mode  JA

beacon  however only carries 12 data elements and most of  status

bytes.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 73

Mode JA Telemetry Data

The  Mode  JA beacon transmits the telemetry data in  the  format

shown below. These data are sent by Morse code with a "HI HI"  at

the  beginning  of  each  frame,  with  a  speed  of  about   100

characters every minute,  and always in this format repeatedly.

HI  HI  1A  1B  1C  1D

2A  2B  2C  2D

3A  3B  3C  3D

4A  4B  4C  4D

5A  5B  5C  5D

The  number  identifies the group, the letters A  through  D  are

decimal values expressed in two digits. Let this two-digit be  N,

for  each item, true value or engineering  value is  obtained  by

decoding  N as shown below.  For example, a value of 123  for  1A

means   group 1 and 23 is the measured value of the  solar  array

current.   Groups  4 and 5 contain status information  about  the

bird,   where  A,  B,  C  and  D  represent    octal    two-digit

combinations of 00 through 37. This corresponds to a  combination

of five binary  digits. Each bit shows status of each  designated

item  in the order from MSB (Most Significant Bit) to LSB  (Least

Significant  Bit).   The decoding equations for the  CW  Mode  JA

telemetry are shown in Figure 7.5.2.

Figure 7.5-2 Fuji-OSCAR 20 Mode JA Telemetry Conversion Equations

CH  DESCRIPTION                       CALIBRATION   UNITS

1A  total solar array current          19x(N+0.4)   mA

1B  battery charge/discharge current  -38x(N-50)    mA

1C  battery voltage                   (N+4)x0.22    V

1D  center tap voltage of battery     (N+4)x0.1     V

2A  bus voltage                       (N+4)x0.2     V

2B  +5 V regulator voltage            (N+4)x0.062   V

2C  JTA output power                  2.0x(N+4)^1.618mW

2D  calibration voltage               (N+4)/50      V

3A  battery temperature               1.4x(67-N)  deg. C

3B  baseplate temperature #1          1.4x(67-N)  deg. C

3C  baseplate temperature #2          1.4x(67-N)  deg. C

3D  baseplate temperature #3          1.4x(67-N)  deg. C

The   status  byte conversions are shown in Figure  7.5-3.   This

method  is used because all items whose status is represented  in

this  manner  only  have  two possible situations, either  ON  or

OFF, or  binary values 0 or 1. For example, if the first item  of

status 4A  were 423, the 4 identifies group 4, and the 23  should

be  thought   of as its equivalent binary   code  (10011).   This

shows the status in the  order of MSB to LSB, or bit 4 to bit  0.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 74

Using  the  decoding data 423 can be decoded as follows.

1: Beacon is PSK,

0: Engineering data #2 is blank,

0: Engineering  data  #1 is blank,

1: JTD power is ON,

1: JTA power is ON.

Figure 7.5-3 Fuji-OSCAR 20 Mode JA System Status Bytes

CH BIT  DESCRIPTION     STATE

1     0

4A  0   JTA power       ON    OFF

4A  1   JTD power       ON    OFF

4A  2   Eng. data #1    ---   ---

4A  3   Eng. data #3    ---   ---

4A  4   Beacon          PSK   CW

4B  0   UVC             ON    OFF

4B  1   UVC level       1     2

4B  2   Battery         tric  full

4B  3   Battery logic   tric  full

4B  4   Main relay      ON    OFF

4C  0   PCU             bit 1 (LSB)

4C  1   PCU             bit 2 (LSB)

4C  2   PCU             manual  auto

4C  3   Eng. data #3    ---   ---

4C  4   Eng. data #4    ---   ---

4D  0   Memory bank #0  ON    OFF

4D  1   Memory bank #1  ON    OFF

4D  2   Memory bank #2  ON    OFF

4D  3   Memory bank #3  ON    OFF

4D  4   Computer power  ON    OFF

5A  0   Memory select   bit 1 (LSB)

5A  1   Memory select   bit 2 (MSB)

5A  2   Eng. data #5    ---   ---

5A  3   Eng. data #6    ---   ---

5A  4   Eng. data #7    ---   ---

5B  0   Solar panel #1  lit   dark

5B  1   Solar panel #2  lit   dark

5B  2   Solar panel #3  lit   dark

5B  3   Solar panel #4  lit   dark

5B  4   Solar panel #5  lit   dark

5C  0   JTA CW beacon   CPU   TLM

5C  1   Eng. data #8    ---   ---

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 75

5C  2   Eng. data #9    ---   ---

5C  3   Eng. data #10   ---   ---

5C  4   Eng. data #11   ---   ---

5D  0   Eng. data #12   ---   ---

5D  1   Eng. data #13   ---   ---

5D  2   Eng. data #14   ---   ---

5D  3   Eng. data #15   ---   ---

5D  4   Eng. data #16   ---   ---

Telemetry  data  are downlinked on Mode JD by means  of  packets.

These data are transmitted the ASCII format shown in Figure  7.5-

4.   In  the  ASCII telemetry  (RA and RB) XXX   is  a  3   digit

decimal   number  with a range between 000 to 999.  This   number

represents  the value of N in Table 5 for channels denoted #00  -

#26.

Figure  7.5-5 contains the equations for converting the  received

data  into   engineering   values.   The  YYY  bytes  are   three

hexadecimal  bytes  of  system  status data, denoted #27a -  #29c

and can be decoded as shown in Figure 7.5-6. The SSS byte in  the

last  row  are binary  status data, denoted #30a - #39c.   Figure

7.5-7 provides  the information needed to decode them in a manner

similar to the Mode JA status points shown in Figure 7.5-3.

Figure 7.5-4.  Fuji-OSCAR 20 Mode JD PSK telemetry data format

JAS-1b FF YY/MM/DD HH:MM:SS

XXX XXX XXX XXX XXX XXX XXX XXX XXX XXX

XXX XXX XXX XXX XXX XXX XXX XXX XXX XXX

XXX XXX XXX XXX XXX XXX XXX YYY YYY YYY

SSS SSS SSS SSS SSS SSS SSS SSS SSS SSS

where,   FF  is  the  Frame Identifier,  which  may  contain  the

following types:

RA: Realtime telemetry, - ASCII

RB: Realtime telemetry, - Binary

SA: Stored telemetry, - ASCII

SB: Stored telemetry, - Binary

M0: Message #0

M1: Message #1

...........

M9: Message #9

YY/MM/DD is year/month/day, and HH:MM:SS is hour/minute/second,

all in UTC.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 76

Figure 7.5-5.  Fuji-OSCAR 20 Mode JD Telemetry Decoding Equations

CH   DESCRIPTION                  CALIBRATION/UNITS

#00  total solar array current     1.91x(N-4)mA

#01  battery charge/discharge     -3.81x(N-508)mA

#02  battery voltage              Nx0.022V

#03  battery center voltage       Nx0.009961V

#04  bus voltage                  Nx0.02021 V

#05  +5 V regulator voltage       Nx0.00620 V

#06  -5 V regulator voltage      -Nx0.00620 V

#07  + 10 V regulator voltage     Nx0.0126  V

#08  JTA output power             5.1x(N-158)mW

#09  JTD output power             5.4x(N-116)mW

#10  calibration voltage #2       N/500 V

#11  offset voltage #1            N/500 V

#12  battery temperature          0.139x(669-N)deg. C

#13  JTD temperature              0.139x(669-N)deg. C

#14  Baseplate Temperature #1     0.139x(669-N)deg. C

#15  Baseplate Temperature #2     0.139x(669-N)deg. C

#16  Baseplate Temperature #3     0.139x(669-N)deg. C

#17  Baseplate Temperature #4     0.139x(669-N)deg. C

#18  temperature calibration #1   N/500 V

#19  offset voltage #2            N/500 V

#20  Solar Cell Panel Temp   #1   0.38x(N-685)deg. C

#21  Solar Cell Panel Temp   #2   0.38x(N-643)

#22  Solar Cell Panel Temp   #3   0.38x(N-646)

#23  Solar Cell Panel Temp   #4   0.38x(N-647)

#24  -------------------------

#25  temperature calibration #2   N/500 V

#26  temperature calibration #3   N/500 V

---------------------------------------------------------

Figure 7.5-6.  Fuji-OSCAR 20 Mode JD HEX System Status Bytes

CH   DESCRIPTION

#27a  Spare (TBD)

#27b  Spare (TBD)

#27c  Spare (TBD)

#28a  Spare (TBD)

#28b  Spare (TBD)

#28c  error count of memory unit #0

#29a  error count of memory unit #1

#29b  error count of memory unit #2

#29c  error count of memory unit #3

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 77

Figure 7.5-7.  Fuji-OSCAR 20 Mode JD BINARY System Status Bytes.

CH    DESCRIPTION             STATE

1      0

#30a  JTA power             on     off

#30b  JTD power             on     off

#30c  JTA beacon            PSK    CW

#31a  UVC status            on     off

#31b  UVC level             1      2

#31c  main relay            on     off

#32a  engineering data #1     -----

#32b  battery status        tric   full

#32c  battery logic         tric   full

#33a  engineering data #2     -----

#33b  PCU status            bit 1 (LSB)

#33c  PCU status            bit 2 (MSB)

#34a  memory unit  #0       on    off

#34b  memory unit  #1       on    off

#34c  memory unit  #2       on    off

#35a  memory unit           on     off

#35b  memory select         bit 1 (LSB)

#35c  memory select         bit 2 (MSB)

#36a  engineering data #3     ------

#36b  engineering data #4     ------

#36c  computer power        on    off

#37a  engineering data #5     ------

#37b  solar panel #1        lit  dark

#37c  solar panel #2        lit  dark

#38a  solar panel #3        lit  dark

#38b  solar panel #4        lit  dark

#38c  solar panel #5        lit  dark

#39a  engineering data #6     ------

#39b  CW beacon source      CPU   TLM

#39c  engineering data #7     ------

A typical set of raw PSK telemetry packets are those captured  by

KI6QE  and shown in Figure 7.5-8.  A typical decoded and  display

page  from WHATS-UP is shown in Figure 7.5-9 for a different  set

of raw data.

Figure 7.5-8 Fuji-OSCAR 20 PSK telemetry (as copied by KI6QE)

03-Apr-90  17:40:32  8J1JBS*>BEACON:

JAS1b RA 90/04/03 17:45:18

554 433 700 686 757 837 841 823 398 666

617 001 503 516 526 523 526 523 654 000

683 675 685 684 999 643 875 316 002 000

110 111 000 000 100 000 001 011 111 000

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 78

03-Apr-90  17:40:34  8J1JBS*>BEACON:

JAS1b RA 90/04/03 17:45:20

566 427 699 705 746 837 841 824 541 659

617 001 503 516 526 523 526 523 654 000

683 675 686 683 999 642 874 316 002 000

110 111 000 000 100 000 001 011 111 000

Figure  7.5-9 Sample Decoded Display (General Housekeeping)  Page

from Fuji-OSCAR 20.

JAS1b RA 91/01/13 00:40:58

Solar Panel Temp #1:  15.20 Deg.C Total Array Current:1105.89 mA

Solar Panel Temp #2:  31.92 Deg.C Battery Charge     : 102.87 mA

Solar Panel Temp #3:  32.68 Deg.C Battery Voltage    : 14.806 V

Solar Panel Temp #4:  29.64 Deg.C Battery Center     :  6.744 V

Baseplate Temp. #1 :  40.73 Deg.C Bus Voltage        : 17.259 V

Baseplate Temp. #2 :  41.42 Deg.C   +5 V Regulator   :  5.214 V

Baseplate Temp. #3 :  40.87 Deg.C   -5 V Regulator   :  0.000 V

Baseplate Temp. #4 :  41.14 Deg.C  +10 V Regulator   : 10.471 V

Temperature Cal. #1:   1.30 V     Offset Voltage #1  :  0.000 V

Temperature Cal. #2:   1.29 V     Offset Voltage #2  :  0.000 V

Temperature Cal. #3:   1.75 V     Calibration Volt #2:  1.230 V

Battery Temp.      :  45.04 Deg.C JTA TX Output Power:   0.46 W

JTD Temperature    :  42.12 Deg.C JTD TX Output Power:   3.52 W

7.6 AMSAT-OSCAR 21 (RM-1)

AMSAT-OSCAR  21  (AO-21) was launched in  February  January  1991

from  the North Cosmodrome at Plesetsk.  The orbit is a  slightly

elliptical  polar  orbit  with  an  apogee  of  1000  km  at   an

inclination  of 83 degrees. The period of the orbit will  be  105

minutes.

AMSAT-OSCAR  21 is the first international OSCAR in  which  radio

amateurs from the Soviet Union took part.  RM1 stands for  "RADIO

M-1",  which  is the official prelaunch name of  AMSAT-OSCAR  21,

emphasizes  that  the  spacecraft was built by,  and  for,  Radio

Amateurs around the world.

The  idea  of a joint effort between the two groups, one  in  the

USSR  and  the second, in Germany first appeared  in  the  spring

1989.  The discussions about what and how things had to  be  done

lasted till the meeting of the representatives of the two  groups

in  Surrey in July 1989 when the preliminary agreement about  the

cooperation  was  signed. The final version  of  the  cooperation

agreement  was later signed in the autumn of 1989 after  much  of

the work had been completed.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 79

According  to the mutual agreement, Amsat-U-Orbita developed  and

made  the  linear  transponder,  command  radio  link,  telemetry

system, power supply system and decided all the problems with the

official and other government organizations about the location of

the equipment and launching.

The RUDAK group of AMSAT-DL developed and built the digital part,

called RUDAK-2 which contains a digipeater and an AX.25  mailbox.

It   also  provides  other  possibilities  for   experiments   in

transmitting  of  information using modern digital  methods.   It

also contains its own RF input and output circuits.

The  ground command station was developed by  the  Amsat-U-Orbita

and Amsat-U-Sputnik groups. The Rudak group however provided some

special digital part for it.

During  the  launch and orbital test period ground,  the  command

stations  will be situated in Molodechno at UC1CWA and in  Moscow

at  RK3KP. The ground command stations for Rudak-2 only  will  be

situated  near Munich at DK1YQ and near Hanover, at  DB2OS.   The

final  agreement  was signed on behalf of Amsat-U-Orbita  by  the

technical director of project "RADIO-M1" - V.Chepyzhenko,  RC2CA;

and on behalf of AMSAT-DL, by their president, K. Meinzer, DJ4ZC.

The  Project Manager for the RUDAK-2 is Hanspeter Kuhlen,  DK1YQ.

The  coordinators  for  the project are  P.  Guelzow,  DB2OS  and

L.Labutin, UA3CR.

AMSAT-OSCAR  21  is an attached  secondary  payload  (Piggy-back)

aboard a USSR geological research satellite which provides a Mode

B  communications  transponder in low earth orbit as well  as  an

orbiting experimental digital communications capacity.

The AMSAT-OSCAR 21 Specifications are as listed below.

Dimension  and shape: Cylinder of height about 4 meters  and

diameter 1.8 meters

System   configuration:  Professional  geological   research

equipment,   telemetry  system,  command   link   equipment,

transponders  and  power supply,  thermal  control.  Amateur

linear  and digital transponders, telemetry system,  command

link equipment, power supply.

Attitude  control:  Satellite attitude  will  be  maintained

using  a  gravity gradient approach in the form of a  rod  9

meters long pointing away from the earth.

Planned service life: 3 years.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 80

Two  sets  of the equipment are installed aboard  the  satellite:

Linear  transponder  #1 is Mode B and contains  the  RUDAK-2  and

other  subsystems,  while  Linear  transponder  #2  mode  B  with

subsystems The Primary transponder is Linear Transponder #1,  the

second  one  is a spare which can be put into  operation  in  the

event  of  a failure of the primary system. Mode B in  low  earth

orbit  provides  a very good communications link  as  anyone  who

worked AMSAT-OSCAR 7 Mode B will testify.

The Transponder RF Frequency Assignments and Beacon Data for  the

Primary  Payloads  are shown in Table 7-6.1, and  those  for  the

Backup  payload in Table 7.6-2.  The 1100 bps is not a  misprint,

it  is  real. Apparently this data rate is used by a  popular  PC

tape  cassette interface in the Soviet Union. Because this PC  is

simple  and  cheap for the Hams in the USSR,  the  AMSAT-U-Orbita

team  decided  to use it on this spacecraft, much  in  a  similar

manner to the use of reversed AFSK tones in UoSAT-OSCAR 11 due to

the  wide availability of a popular interface for those tones  in

the  United  Kingdom.  It is unclear as of the  time  of  writing

this,  if the telemetry is HDLC or some  proprietary  synchronous

PSK such as the 400 baud AMSAT-OSCAR 13 downlink. As such you may

not be able to copy this telemetry.

Table 7.6-1 Primary Payload

Beacons and telemetry #1

CW telemetry 8 channels  145.822 MHz 0.2 Watts

Digital telemetry        30 channels 145.952 MHz 0.4 Watts

1100 bps,BPSK/FM, deviation 2kHz

Digital telemetry Rudak-2 145.983 MHz 3.0 Watts

BPSK 1200 bps AX.25 (like FO-20)

Transponders #1

Linear transponder:      inversely hetrodined translator

Uplink passband          435.102 to 435.022 MHz

Downlink passband        145.852 to 145.932 MHz

Transmitter output max   10 Watts

Bandwith (3db)           80 kHz

Uplink EIRP required about 100 Watts

Digital transponder Rudak-2: digipeater and store&forward  packet

communication (AX.25), telecommunications experiment with digital

signal  processing  up to nearly 20 kHz, 1 MByte RAM  disc,  four

separate uplink channels.

Uplink frequencies:

RX-1  435.016 MHz 1200bps,FSK,NRZIC/Biphase-M

RX-2  435.155 MHz (AFC) 2400 bps,BPSK, Biphase-S

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 81

RX-3a 435.193 MHz (AFC) 4800 bps,RSM

RX-3b 435.193 MHz (AFC) 9600 bps,RSM

RX-4  435.041 MHz (digital AFC) RX for RTX-DSP

Downlink frequency: 145.983 MHz 3 Watts

The downlink can be switched to the following operating modes:

Mode 1: 1200 bps, BPSK, NRZI,(NRZ-S) (like FO-20)

Mode 2: 400 bps, BPSK, Biphase-S (Like AO-13 beacon)

Mode 3: 2400 bps, BPSK, Biphase-S

Mode 4: 4800 bps, RSM, NRZIC (Biphase-M) (like 4800 bps uplink)

Mode 5: 9600  bps,  RSM, NRZI (NRZ-S) +Scrambler (like  9600  bps

uplink)

Mode 6: CW keying (only for special events)

Mode 7: FSK  (F1 or F2B),e.g. RTTY, SSTV, FAX,  etc.(for  special

events)

Mode 8: FM  modulated  by  D/A signals  from  DSP-RISC  processor

(speech)

Table 7.6-2 Secondary Payload

Beacons and Telemetry #2

CW telemetry 8 channels            145.948 MHz 0.2 Watts

Digital telemetry 30 channels    145.838 MHz 0.4 Watts 1100  bps,

BPSK/FM, deviation 2kHz

Digital telemetry  30 channels    145.800 MHz 2.0 Watts 1100  bps

BPSK/FM, deviation 2kHz

Transponder #2

Linear transponder:      inversely heterodined translator

Uplink passband          435.123 to 435.043 MHz

Downlink frequencies      145.866 to 145.946 MHz

Transmitter output max   10 Watt max.

Bandwith (3db)           80 kHz

Uplink EIRP required about 100 Watts

The spacecraft uses two antennas.  The 435 MHz receiving  antenna

which  is shared by the analog and digital modes is a Helix  with

up to +3 db gain using Right Hand circular Polarization.  The 145

MHz  transmitting antenna is a Half wave dipole.  AMSAT-OSCAR  21

draws  up  to 100 Watts from the main spacecraft's  power  supply

system.  The Primary RM-1 payload including the RUDAK draw up  to

47 Watts, the secondary payload draws up to 40 Watts. The Primary

AMSAT-OSCAR  21 payload including the RUDAK weighs  approximately

28  kg, the secondary payload weighs approximately 22  kg.   Both

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 82

payloads are about 480x400x300 mm^3.

The  CW telemetry started up as soon as the  combined  spacecraft

separated  from  the  launch  vehicle and  was  powered  by  main

satellite power system. A CW Morse-Code telemetry frame  consists

of  the call RS14 and 8 channels of four digits in the  following

format:

RS14 S0AB=S1AB=S2AB=S3AB=S4AB=S5AB=S6AB=S7AB

Channels 0 to 6 contain analog telemetry data. Channel 7 contains

engineering calibration parameters.

The  first  digit (S) identifies which system  the  telemetry  is

from.  A prefix of 7 identifies a general status, a prefix  of  5

identifies  a command status. The second digit (0 to 7)  are  the

numbers  of the line (channel).  The remaining digits (A  and  B)

are  the analog telemetry data which can be decoded according  to

the equations shown in Table 7.6-3.

Table 7.6-3 AMSAT-OSCAR 21 CW Telemetry decoding parameters

Channel        Parameter                Formula   Unit

0        Transponder power output      0.05*N    Watts

1        Transponder PA Temperature    N         Deg. C

2         +24 V Regulated              N         Volt

3         +16 V Regulated              N         Volt

4         +9  V Regulated              N         Volt

5         +24 V Regulated              N         Volt

6        Inside Temperature            N         Deg. C

7        Engineering Value             N         *

A typical frame such as

"RS14=7080=7137=7224=7316=7409=5524=5032=57PPRS14" may be decoded

as shown below.

RS14

7080 - 4 Watts   - general

7137 - 37 Deg. C - general

7224 - 24 Volts  - general

7316 - 16 Volts  - general

7409 - 9 Volts   - general

5524 - 24 Volts  - command

5032 - 32 Deg. C - command

57PP             - command

Examples of other frames copied and edited by G3ZCZ/W3 are

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 83

28-Feb-91 01:40 RS14=7032=7121=7224=7316=7410=7500=7600=77PP

01-Mar-91 02:19 RS14=7028=7121=7224=7316=7410=7500=7600=77PP

The  AMSAT-OSCAR 21 Digital telemetry consists of  30  parameters

monitoring  on-board  conditions and 2  calibration  verification

points. If you intend to receive, decode and display the  digital

telemetry you will need an FM receiver, a TNC, a PSK modem, and a

computer  or terminal.  Even then you may not be able  to  decode

the  data  since  it may not be downlinked in  HDLC  format.  The

decoding  equations for the digital telemetry are shown in  Table

7.6-4.  The raw digital telemetry is expected to show up on  your

screen in the format shown in Table 7.6-5.

Table 7.6-4 Decoding   Equations  for  AMSAT-OSCAR   21   Digital

Telemetry (Version 26-Dec-90)

Line Parameter                  Formula     Unit  Hex-Format-Line

1   "Zero" of the comparator      0C         N/A      0000

2   Reference voltage             6D         N/A      0010

3   Transponder #1 HF output pwr  0.2N^2    Watt      0020

4   Transponder #1 PA temperature 0.8*N     Deg. C    0030

5   DC/DC converter temperature   0.8*N     Deg. C    0040

6   +14 V Regulated               10*N      Volt      0050

7   +24 V Regulated               10*N      Volt      0060

8   +16 V Regulated               10*N      Volt      0070

9   +12 V Regulated               10*N      Volt      0080

10  +9 V Regulated                10*N      Volt      0090

11  +7.5 V Regulated              10*N      Volt      00A0

12  +5 V Regulated                10*N      Volt      00B0

13  +9 V Regulated (linear)       10*N      Volt      00C0

14  +9 V Regulated (digital)      10*N      Volt      00D0

15  Service                          N      *         00E0

16  Service                          N      *         00F0

17  Transponder #2 HF output pwr  0.2*N^2   Watt      0100

18  Transponder #2 PA temperature 0.8*N     Deg. C    0110

19  +24 V Regulated               10*N      Volt      0120

20  +16 V Regulated               10*N      Volt      0130

21  +10 V Regulated               10*N      Volt      0140

22  +9 V Regulated                10*N      Volt      0150

23  +7.5 V Regulated              10*N      Volt      0160

24  Status command link           1         *         0170

25  Status command link           2         *         0180

26  Status command link                     *         0190

27  Status command link                     *         01A0

28  RPC +5V for Rudak-1           2.47*N    Volt      01B0

29  RPC +5V for Rudak-RTX         2.47*N    Volt      01C0

30  RPC +5V for Ramdisk           2.47*N    Volt      01D0

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 84

31  RPC +14V total supply current 627-289*N mA        01E0

32  RPC module temperature        56.7*N-49.5 Deg. C  01F0

Note:  RPC  - Rudak Power Conditioner.  The Service  and  command

link channels are reserved for use by the ground control team.

Table 7.6-5 Expected Format of Raw AMSAT-OSCAR 21 PSK Telemetry

0000 0C E6 0C E6 0C E6 0C E6 0C E6 0C E6 0C E6 0C E6

0010 6D E6 6D E6 6D E6 6D E6 6D E6 6D E6 6D E6 6D E6

0020 i E6 i E6 i E6 i E6 i E6 i E6 i E6 i E6

....................................................

01F0 j E6 j E6 j E6 j E6 j E6 j E6 j E6 j E6

i...j - the value of the parameter in the hex format, repeated 8

times  E6 - separation

The  RUDAK  system is a message  store-and-forward  package.  Its

downlink is not expected to contain any telemetry. An example  of

some RUDAK signals copied by W3/G3ZCZ a few days after launch  is

shown in Table 7.6-6.

Table 7.6-6 Example of AMSAT-OSCAR 21 RUDAK Signals.

26-Feb-91  02:31:36  RUDAK*>BEACON:

* RUDAK-II / AMSAT OSCAR 21/RM1

* Up: 435.016MHz (1200)

*     435.155MHz (2400)

* Mailbox=RUDAK Mheard=RUDAK-1

26-Feb-91  02:32:37  RUDAK*>BEACON:

RUDAK-Telemetry (91-02-25 23:03:00):

Used stack entries: 0

Uplink Carrier Detect (during last minute): 0%

26-Feb-91  02:33:36  RUDAK*>BEACON:

* RUDAK-II / AMSAT OSCAR 21/RM1

* Up: 435.016MHz (1200)

*     435.155MHz (2400)

* Mailbox=RUDAK Mheard=RUDAK-1

26-Feb-91  02:34:37  RUDAK*>BEACON:

RUDAK-Telemetry (91-02-25 23:05:00):

Used stack entries: 0

Uplink Carrier Detect (during last minute): 0%

26-Feb-91  02:35:09  RUDAK,*>WB5BZE (UA)

26-Feb-91  02:35:11  RUDAK*>WB5BZE:

+-------------------------------------------------------+

+  Welcome to the RUDAK II Bulletin Board System V0.04  +

+-----------

26-Feb-91  02:35:13  RUDAK*>WB5BZE:

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 85

--------------------------------------------+

Logged in at 91-02-25 23:05:33,  1 User

This is a preliminary release.

Please rep

26-Feb-91  02:35:15  RUDAK*>WB5BZE:

ort deficiencies to DL2MDL.

73 de AMSAT-UA/AMSAT-DL/RUDAK-Group.

Enter H for help.

WB5BZE de RUDAK>

26-Feb-91  02:35:36  RUDAK*>BEACON:

* RUDAK-II / AMSAT OSCAR 21/RM1

* Up: 435.016MHz (1200)

*     435.155MHz (2400)

* Mailbox=RUDAK Mheard=RUDAK-1

26-Feb-91  02:35:51  RUDAK*>WB5BZE [D]

26-Feb-91  02:35:56  RUDAK*>WB5BZE [D]

26-Feb-91  02:36:32  KF4WQ>RUDAK*>KF4WQ [C]

26-Feb-91  02:36:37  RUDAK*>BEACON:

RUDAK-Telemetry (91-02-25 23:07:00):

Used stack entries: 0

Uplink Carrier Detect (during last minute): 0%

26-Feb-91  02:37:35  RUDAK*>WB5BZE (UA)

26-Feb-91  02:37:36  RUDAK*>BEACON:

* RUDAK-II / AMSAT OSCAR 21/RM1

* Up: 435.016MHz (1200)

*     435.155MHz (2400)

* Mailbox=RUDAK Mheard=RUDAK-1

8.0 Spacecraft No Longer Active

This  section  is provided for post mission analysis  of  digital

data from spacecraft that are  no longer active in case you  have

access  to  data from those satellites and wish to the  tools  in

WHATS-UP to view and analyze the data.

8.1 Fuji-OSCAR 12

The  Fuji-OSCAR 12 PSK Telemetry Data Format. It  is  practically

identical  to the Fuji-OSCAR 20 format.  The format is  shown  in

Table  8.1-1, the decoding equations are presented in Table  8.1-

2.

Table 8.1-1 Fuji-OSCAR 12 PSK Telemetry Frame Format

JAS-1 FF YY/MM/DD HH:MM:SS

xxx xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx yyy yyy

sss sss sss sss sss sss sss sss sss sss

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 86

FF := Frame Identifier   RA: Realtime Telemetry - ASCII

RB: Realtime Telemetry - Binary

SA: Stored Telemetry - ASCII

SB: Stored Telemetry - Binary

M0: Message #0

M1: Message #1

.......

M9: Message #9

YY/MM/DD  =  Date

HH:MM:SS  =  Time (The command station attempts to keep the clock

as close as possible to UTC)

[ Following is valid only for RA and SA frames ]

xxx =  000 - 999  Format:  3  digit  decimal  (Analog  Data)   28

samples  in  row  0 column 0 thru  row  2  column  7

(denoted #00 - #27 below)

y =  0 - F   one byte Hex (System Status Data)

9 samples in row 2 column 8  thru  row 2 column 9

(denoted #28a - #29c below)

s =  0 or 1  Binary Status Data

30 samples in row 3  thru  row 3 column 9

(denoted #30a - #39c below)

Table 8.1-2 Fuji-OSCAR 12 Telemetry Calibration Equations

Ch.     Item                          Equation    Units

#00     Total Solar Array Current     1.91 * ( N - 4 )    mA

#01     Battery Charge/Discharge      3.81 * ( N - 528 )  mA

#02     Battery Voltage               N * 0.0210    V

#03     Half-Battery Voltage          N * 0.00937   V

#04     Bus Voltage                   N * 0.0192    V

#05     + 5 V. Regulator Voltage      N * 0.00572   V

#06     - 5 V. Regulator Voltage      N * -0.00572  V

#07     +10 V. Regulator Voltage      N * 0.0116    V

#08     JTA Power Output              5.1 * ( N  - 158 )  mW

#09     JTD Power Output              5.4 * ( N - 116 )   mW

#10     Calibration Voltage #2        N / 500  V

#11     Offset Voltage #1             N / 500  V

#12     Battery Temperature           0.139 * ( 689 - N )  Deg. C

#13     JTD Temperature               0.139 * ( 689 - N )  Deg. C

#14     Baseplate Temperature #1      0.139 * ( 689 - N )  Deg. C

#15     Baseplate Temperature #2      0.139 * ( 689 - N )  Deg. C

#16     Baseplate Temperature #3      0.139 * ( 689 - N )  Deg. C

#17     Baseplate Temperature #4      0.139 * ( 689 - N )  Deg. C

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 87

#18     Temperature Calibration #1    N / 500  V

#19     Offset Voltage #2             N / 500  V

#20     Facet Temperature #1          0.38 * ( N - 684 )  Deg. C

#21     Facet Temperature #2          0.38 * ( N - 684 )  Deg. C

#22     Facet Temperature #3          0.38 * ( N - 690 )  Deg. C

#23     Facet Temperature #4          0.38 * ( N - 683 )  Deg. C

#24     Facet Temperature #5          0.38 * ( N - 689 )  Deg. C

#25     Temperature Calibration #2    N / 500  V

#26     Temperature Calibration #3    N / 500  V

#27     Depth of Battery discharge    ( N - 500 ) / 189   AH

Fuji-OSCAR 12 System Status Telemetry Bytes

Ch.     Item

#28a    Spare (TBD)

#28b    Spare (TBD)

#28c    Memory Unit #0 error count

#29a    Memory Unit #1 error count

#29b    Memory Unit #2 error count

#29c    Memory Unit #3 error count

Fuji-OSCAR 12 Binary Status Data Points

Ch.    Item                     1         0

#30a   JTA Power                On        Off

#30b   JTD Power                On        Off

#30c   JTA Beacon               PSK       CW

#31a   UVC Status               On        Off

#31b   UVC Level                1         2

#31c   Main Relay               On        Off

#32a   Engineering Data #1      ----      ----

#32b   Battery Status           Tric      Full

#32c   Battery Logic            Tric      Full

#33a   Engineering Data #2      ----      ----

#33b   PCU Status                Bit 1 (LSB)

#33c   PCU Status                Bit 2 (MSB)

#34a   Memory Unit #0           On        Off

#34b   Memory Unit #1           On        Off

#34c   Memory Unit #2           On        Off

#35a   Memory Unit #3           On        Off

#35b   Memory Select             Bit 1 (LSB)

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 88

#35c   Memory Select             Bit 2 (MSB)

#36a   Engineering Data #3      ----      ----

#36b   Engineering Data #4      ----      ----

#36c   Computer Power           On        Off

#37a   Engineering Data #5      ----      ----

#37b   Solar panel #1           Lit       Dark

#37c   Solar panel #2           Lit       Dark

#38a   Solar panel #3           Lit       Dark

#38b   Solar panel #4           Lit       Dark

#38c   Solar panel #5           Lit       Dark

#39a   Engineering Data #6      ----      ----

#39b   CW beacon source         CPU       TLM

#39c   Engineering Data #7      ----      ----

9.0  File formats

This section contains details of the contents of the different

files used by WHATS-UP.

9.1 Configuration File

The contents of the Configuration file (WHATS-UP.SYS) are as

follows:

1    Your callsign (e.g. G3ZCZ)

2    Default spacecraft configuration file (e.g. Dove)

3    station latitude (e.g. 35.00)

4    station longitude (e.g. 74.00)

5    station altitude (e.g. 100)

6    station minimum antenna elevation for acquisition  (e.g.

0)

7    station minimum usable pass time (e.g. 5)

8    early warning time (e.g. 5)

9    default Kepler file (e.g. whats-up.2ln)

10   UTC offset (e.g. EST = 5)

11   default directory path (e.g C:)

12   default extracted data file (e.g. whats-up.txt)

13   default file name with list of telemetry parameters  to

extract file (e.g. ARRAYS)

14   TNC Type (e.g. PK-232)

15   PC serial port to TNC (e.g. 1)

16   PC serial port to Radio (e.g. 2)

17   PC serial port baud rate (e.g. 1200)

18   PC Serial port data bits (e.g. 8)

19   PC Serial port Stop bits (e.g. 1)

20   PC Serial port parity (e.g. 0)

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 89

21   status (top) window color (e.g. 79)

22   Telemetry Page color (e.g. 14)

23   outgoing window color (e.g. 14)

24   incoming window color (e.g. 30)

25   prompt window color (e.g. 15)

26   alarm window color (e.g. 15)

27   bottom window color (e.g. 79)

28   Emphasis color (e.g. 14)

29   prompt color (e.g. 14)

30   option color (e.g. 78)

31   parameter changed color (e.g. 95)

32   parameter limit exceeded color (e.g. 14)

33   Orbit element window color

34   Orbit  element  window Orbit element  window  In  range

color

35   Orbit element window early warning color

36   Orbit element window next one up color

37   Orbit alert dit time

38   Orbit alert note

39   Flag Sound

40   Doppler display Flag

41   The  next few lines link the  spacecraft  configuration

files  and  contain:- SC ID,  Spacecraft  configuration

file name, autotune parameter.

The * that follows denotes the last line of SC ID data.

You  must  configure WHATS-UP before you try any  Real  Time

activity.

The items in the configuration file are described below.

9.1.1 Your callsign

This item is the callsign displayed at the top of the screen

and  appended to the capture files when  capture-to-disk  is

turned on.

9.1.2 Default spacecraft Name

This item is the default spacecraft name (e.g. Dove). WHATS-

UP  adds the ".SYS" to the end of the name  (e.g.  DOVE.SYS)

and loads that configuration file at start up.

9.1.3 Station Latitude

This  item  is your station latitude (e.g.  35.00).  In  the

southern hemisphere, enter a negative number.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 90

9.1.4 Station longitude

This  item  is  your station longitude in  degrees  WEST  of

Greenwich. (e.g. 74.00)

9.1.5 Station Altitude

This item is the altitude of your station antenna above  sea

level in Meters. (e.g. 100)

9.1.6 Station minimum antenna elevation for acquisition

This item is the minimum angle of elevation of your antennas

at  which you can acquire a signal. If you are on top  of  a

hill  or tall building the angle my be less than 0,  if  you

are in a valley, it may be greater, such as 5 degrees.

9.1.7 Station minimum usable pass time

This  item is the minimum time in minutes for a usable  pass

for collecting data (e.g. 5).

9.1.8 Early warning time

This item is the early warning time in minutes you want  for

notice  that  a spacecraft is about to come  up  above  your

local horizon (e.g. 5).

9.1.9 Default Kepler file

This  item is the default file name that contains the  orbit

element data used when the program is first turned on  (e.g.

whats-up.2ln).

9.1.10 UTC offset

This  item  is the time difference (in  hours)  between  the

local time in your PC and GMT or Universal Coordinated  Time

(UTC).  WHATS-UP always displays UTC time. (e.g. EST = 5)

9.1.11  Default directory path (e.g C:)

This item is the directory path for the spacecraft  capture-

to-disk  (YYMMDD.S/C), spacecraft configuration (*.SYS)  and

spacecraft operations schedule files (*.OPS).

9.1.12  Default extracted data file

This  item  is the name of the file to which data  which  is

extracted from a playback file will be written to in a comma

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 91

delimited  format  so that the data can be imported  into  a

spreadsheet  package  for  further  analysis  (e.g.   whats-

up.txt).

9.1.13  Default file name with list of telemetry  parameters

to extract file

This  item  is the name of the file containing  the  channel

numbers to extract from a playback file for further analysis

(e.g. ARRAYS)

9.1.14 TNC Type

This  item is the type of TNC or Multi-mode  controller  you

are using. WHATS-UP currently supports the PK-232.

9.1.15 Serial port to TNC

This item is the Communications port number between the  TNC

and the PC (e.g. 2).

9.1.16 PC serial port to Radio

This item is the Communications port number between the  TNC

and  the  Radio (e.g. 1).  The  parameters   used  are  4800

baud, 8  data  bits,  No parity.  This item is hard coded to

meet the Kenwood standard.

9.1.17 PC serial baud rate

This  item is the baud rate used between the TNC and the  PC

(e.g. 1200).

9.1.18 PC port data bits

This  item is the number of data bits used between  the  TNC

and the PC (e.g. 8).

9.1.19 PC port Stop bits

This  item is the number of stop bits used between  the  TNC

and the PC (e.g. 1 or 2).

9.1.20 PC Port parity bits

This item is the parity setting used between the TNC and the

PC Values are defined as

0 No parity,

1 Odd,

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 92

2 Even,

3 Mark,

4 Space.

The following items are the color values used for  different

windows or messages.

9.1.21 status (top) window color (e.g. 79)

9.1.22 Telemetry Page color (e.g. 14)

9.1.23 outgoing window color (e.g. 14)

9.1.24 incoming window color (e.g. 30)

9.1.25 prompt window color (e.g. 15)

9.1.26 Alarm window color (e.g. 15)

9.1.27 Bottom window color (e.g. 79)

9.1.28 Emphasis color (e.g. 14)

9.1.29 Prompt color (e.g. 14)

9.1.30 Option color (e.g. 78)

9.1.31 Parameter changed color (e.g. 95)

9.1.32 Parameter limit exceeded color (e.g. 14)

9.1.33 Orbit element window color

9.1.34 Orbit  element window Orbit element window  In  range

color

9.1.35 Orbit element window early warning color

9.1.36 Orbit element window next one up color

9.1.37 Orbit alert dit time

This item is the speed of the morse code annunciator used to

alert you of satellite AOS, EWT and LOS.

9.1.38 Orbit alert note

This item is the tone of the morse code annunciator used  to

alert you of satellite AOS, EWT and LOS.

9.1.39 Flag Sound

This  item controls the sounds coming from the PC.  A  1  is

on, 0 is off.

9.1.40 Doppler display Flag

This  item  determines  the  type  of  Doppler  display   as

follows:-

0 display beacon frequency,

1 display Doppler shift.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 93

9.1.41 Spacecraft Configuration File Linkages

The  next few lines contain the information  which  WHATS-UP

uses  to  link  the spacecraft configuration  files  to  the

Keplerian  elements  for orbit determination  and  automatic

selection of spacecraft at AOS time.  The lines contain data

as shown below.

UO-11,UOSAT11.SYS, 0

AO-13,OSCAR13Z.SYS,0

AO-16,PACSAT.SYS,  0

DO-17,DOVE.SYS,    1

WO-18,WEBER.SYS,   0

LO-19,LUSAT.SYS,   2

FO-20,FUJI.SYS,    0

The  first item is the Keplerian element identifier for  the

spacecraft as used in the orbit element files.

The second item is the FULL configuration file name for  the

spacecraft.

The  third  item  is the automatic  AOS  selection  flag  as

itemized below.

'0', automatic selection is inhibited.

'1', WHATS-UP selects that spacecraft configuration file  at

EWT  time  (if it is not selected at  that  time),  and

tunes  the  radio  to the  beacon  frequency  +  offset

defined in that file.

'2', WHATS-UP selects that spacecraft configuration file  at

AOS  time  (if it is not selected at  that  time),  and

tunes  the  radio  to the  beacon  frequency  +  offset

defined in that file.

9.1.42  * Comment line

This  line  must be present and signals  WHATS-UP  that  the

default  parameters  have been read and that  the  following

lines contain commands to be sent to the TNC on starting  up

the program.

9.1.43  The remaining lines are commands sent to  the  PK232

when you configure the TNC. Note to avoid lock ups FLOW  and

XFLOW MUST be OFF. Typical commands are as follows:-

HEAD ON

ECHO OFF

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 94

DAYSTAMP ON

MONITOR 6

MSTAMP ON

FLOW OFF

XFLOW OFF

MFILTER None

9.2 Spacecraft Parameter Files

You  need  a Different spacecraft parameter  file  for  each

spacecraft.  Spacecraft  parameter files are  named  by  the

spacecraft  and  given the extension  '.SYS'.  Examples  are

'DOVE.SYS'  and 'Fuji20.SYS'. These files determine how  the

individual channels are decoded, and where, in which  screen

page,  and  in which color the decoded data  are  displayed.

Some  of  the items are unique to WHATS-UP and some  to  the

particular spacecraft.

The  contents  of the SPACECRAFT.SYS file are  as  described

below.

9.2.1 Spacecraft ID.

This is the call sign of the spacecraft. For example,

Spacecraft      ID

DOVE-OSCAR 17  DOVE-1

Fuji-OSCAR 12  8J1JAS

Fuji-OSCAR 20  8J1JBS

In  the case of the Microsats and Fuji-OSCAR 12/20, and  any

other  using packet telemetry, WHATS-UP searches the  packet

headers to detect the spacecraft by this callsign.

9.2.2 Spacecraft Suffix

This becomes the filetype for the capture-to-disk files. The

default suggestions are as shown below.

Spacecraft          Suffix

UoSAT-OSCAR 11      U11

Fuji-OSCAR 12       F12

AMSAT-OSCAR 13      O13

AMSAT-OSCAR 16      O16

DOVE-OSCAR 17       D17

WEBER-OSCAR 18      W18

LUSAT-OSCAR 19      L19

Fuji-OSCAR 20       F20

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 95

AMSAT-OSCAR 21      O21

9.2.3 Beacon Frequency

This is the frequency of the spacecraft beacon (in MHz) that

you  are  monitoring.  It will be displayed  in  the  status

window.

9.2.4 Doppler Measurement File

This  is  the  default name of the file used  to  store  the

Doppler frequency measurements.

9.2.5  Spacecraft Identification in Keplerian Element  File,

examples are as follows.

UO-11

AO-13

AO-16

DO-17

WO-18

LO-19

FO-20

9.2.6 Doppler Measurement Sample Interval

This  is  the  default sample  interval  (in  seconds)  used

between Doppler frequency measurements.

9.2.7 Initial Frequency Offset

This  is  the  default value (in kHz) added  to  the  beacon

frequency,   and  output  to  the  Radio  Receiver  when   a

particular spacecraft is selected. The default value is 5 to

tune  the radio 5 kHz above the beacon. This will allow  the

signal  to slide into lock, whereby the TAPR PSK Modem  will

lock on and follow the frequency for the rest of the pass.

9.2.8 TNC configuration Code

This item tells WHATS-UP how to configure the PK-232 for the

spacecraft as follows:-

0    N/A - illegal value

1    FM Packet 1200 baud

2    RTTY BAUDOT 50 baud

3    ASCII 1200 baud

4    PSK Packet 1200 baud

5    FM Packet 1100 baud

6    CW 20 words per minute

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 96

9.2.9 Modulation

This  is  the  type of modulation used  by  the  spacecraft.

Current options are :-

F FM

P PSK.

9.2.10 Data Type

This is the type of data downlinked by the spacecraft.

A ASCII as used by UoSAT-OSCAR 11 and AMSAT-OSCAR 13.

P Packet  as used by DOVE-OSCAR 17, Fuji-OSCAR 20  and  the

Microsats.

9.2.10 Receiver Type

This  is  the  manufacturer of the radio  receiver  you  are

using.   This version of WHATS-UP only supports the  Kenwood

series.

9.2.11 Selected or default display page number

This  is  the  default display page for  the  Real-time  and

Playback modes, when WHATS-UP is first loaded.

9.2.12 Page Definitions

These are the page definitions, with two items on the  line.

The  format  is PAGE_TITLE, Page_Color, as  in  the  example

below.

SPACECRAFT HOUSEKEEPING, 30

9.2.13 Telemetry Parameter Configuration

The   next  set  of  items  are  the   Telemetry   parameter

configurations (maximum = 99). You must have at least one of

these  lines in the file. If you want a value to show up  in

more  than one page (other than the wild card [0]) you  must

enter it twice (once per page). Typically each row  contains

17 items in the format shown below.

TLM_Channel, TLM_Segment_ID, TLM_Description,  TLM_Eqn_Type,

TLM_Ceof_C,  TLM_Ceof_B,  TLM_Ceof_A,  TLM_Units,  TLM_Page,

TLM_Row,   TLM_Col,  TLM_Width,  TLM_Dec,   TLM_Limit_Check,

TLM_Limit_Low, TLM_Limit_High, TLM_Negative_Blank.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 97

Each item is described in the following sections.

9.2.13.1 TLM_Channel

This is the channel number of the telemetry data in the

frame.  The  DOVE channel number is  hexadecimal  (e.g.

'0F'), Fuji is decimal. Each entry must be two digits.

A special identifying TLM_Channel is defined in  WHATS-

UP. If the value is '99 then the segment identifier and

position  of the segment identifier is defined  in  tow

positions in the line. This special channel  identifies

the type of telemetry frame.

9.2.13.2 TLM_Segment_ID

This is the segment identifier as described in  Section

9.2.13.99 below.

9.2.13.3 TLM_Description

This  item  is the text string or  description  of  the

telemetry channel that will be displayed on the  screen

page. (e.g. '+Z Array Temp.')

9.2.13.4 TLM_Eqn_Type

This item tells WHATS-UP the type of equation to use to

decode the telemetry.

Type 1 is the general purpose equation used by AMSAT-NA

in the Microsats.

Fuji uses two other types of equations (2 and 3).  They

are in the formats of

Y = D*(N+E), and Y = F*(G-N). If you know some  algebra

you can convert both of Fuji's equations to the  Format

used  by AMSAT, but since a computer is  involved,  why

not  let it do the job. You do however have to  convert

an equation of the form Y=(N+a)/b.

AMSAT-OSCAR 13 also needs three more types (4 5 and  6)

of equations to decode the RTTY Z blocks.

A  type  1 equation in WHATS-UP is a quadratic  of  the

form

Y = A*N^2 + B*N + C,

where: N = raw telemetry data value

A,  B,  C = Equation Coefficients; Y,  N  are  decimal

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 98

values.

A  type  2 equation in WHATS-UP has the format  in  the

form of

Y = B*(A+N) + C

where  C,  B,  A are coefficients;  Y,  N  are  decimal

values.

A  type  3 equation in WHATS-UP has the format  in  the

form of

Y = B*(A-N) + C

where  C,  B,  A are coefficients; Y,  N  are  decimal

values.

A  type  4 equation in WHATS-UP has the format  in  the

form of

Y = B*(N+A)^2 +C

where  C,B,  A  are coefficients;  Y,  N  are  decimal

values.

A  type  5 equation in WHATS-UP has the format  in  the

form of

Y = B*(A-N)^2 + C

where  C,  B,  A are coefficients; Y,  N  are  decimal

values.

A type 6 equation in WHATS-UP is a special case. It  is

the  AMSAT-OSCAR 13 (Channel 1C) Spin Rate equation  as

specified below.

if N > 131

then Y := 479/(N - 109) - 2

else Y := (131 - N) * 0.85 + 20;

where  N is the raw decimal number in Channel 1C and  Y

is the spin rate.

9.2.13.5 TLM_Ceof_C

This item is the equation Coefficient C.

9.2.13.6 TLM_Ceof_B

This item is the equation Coefficient B.

9.2.13.7 TLM_Ceof_A

This item is the equation Coefficient A.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 99

9.2.13.8 TLM_Units

This  item is the Units text string (e.g. '.C') in  the

screen display. However if the Tlm_Channel is '99' then

this item is the segment identifier string.

9.2.13.9 TLM_Page

This  item is the Display page number. A 0 is  a  'wild

card' which will be displayed on every page.

9.2.13.10 TLM_Row

This item is the Display page row. It identifies  which

row in the screen the data element will be displayed.

9.2.13.11 TLM_Col

This  item  is the Display page column.  It  identifies

which  column  in  the screen the  data  item  will  be

displayed.

9.2.13.12 TLM_Width

This  item is the Display width for Engineering  Units.

It tells WHATS-UP how many characters wide the  display

is  to  be. You can set it to any value you  want.  For

example,  you  can  display  a  voltage  as  '1.3'   or

'1.28567'.  Before  you  widen the  display  too  much,

remember the sampling accuracy of the analog-to-digital

converter in the spacecraft.

9.2.13.13 TLM_Dec

This  item  defines  the number  of  digits  after  the

decimal point in the display.

9.2.13.14 TLM_Limit_Check

This  item tells WHATS-UP to perform limit checking  on

the  telemetry channel. It may have several  values  as

described below.

0 = do nothing,

1 = check below low limit,

2 = check above high limit,

3 = check for EITHER [below low limit] or [above  high

limit]).

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 100

9.2.13.15 TLM_Limit_Low

This item is the Low limit value (e.g. -4.00).

9.2.13.17 TLM_Limit_High

This item is the High limit value (e.g. +10.6).

9.2.13.18 TLM_Negative_Blank

This  item is a 1 if you want computed negative  values

to  be  displayed as a zero. Use this for  example,  in

Solar  Cell Voltage computations, when negative  values

are  produced by the equation supplied even though  the

negative  values are not real. The negative values  are

produced because the equation used to convert the  data

is not valid at low or zero values of light.

A  line with an '*' as the first character  terminates  this

section.

9.2.13.99 The Telemetry Identifier Line

The  Telemetry  identifier  line has the same  format  as  a

regular line, but contains different parameters as follows:

9.2.13.99.1 The Line Identifier

This  item  has to be '99' to identify  the  telemetry

identifier line.

9.2.13.99.2 The Segment Identifier

This  is used when the spacecraft transmits  telemetry

in  more  than one segment. It  contains  the  segment

identifier showing which segment the channel is  down

linked in.

9.2.13.99.3 Spare/Not Used

9.2.13.99.4 Location of Segment Identifier in Line

This  item  contains  the  location  of  the   segment

identifier in the first line of the telemetry data.

9.2.13.99.5 Spare/Not Used

9.2.13.99.6 Spare/Not Used

9.2.13.99.7 Spare/Not Used

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 101

9.2.13.99.8 Segment Identifier String

This item is the segment identifier string as follows.

9.2.13.99.82.1 Fuji

The Fuji frame contains one real time  segment

(Segment   1)   in  a   frame   addressed   as

8J1JBS>BEACON. A typical frame is shown below.

19-Apr-90 17:14:34 8J1JBS*>BEACON:

JAS1b RA 90/04/19 17:13:58

609 430 687 676 744 837 845 829 498 681

617 001 505 516 526 524 526 523 654 000

683 675 686 695 999 643 875 471 099 000

110 111 000 000 111 100 001 111 111 000

The  segment identifier is in the seventh  and

eighth  characters  of the first line  of  the

data.  A segment identifier of  that  position

identifies  the  second segment.  The  segment

identifier  is the 'RA' located on  the  first

line  of the data just after the  JAS1b  where

the  'R' in 'RA' is the seventh  character  in

the line.

Any   telemetry  frame  addressed  to   BEACON

received  without that segment  identifier  is

assumed by WHATS-UP to be Segment 2.

9.2.13.99.8.2 DOVE

DOVE  transmits telemetry in two  frames  each

addressed  as DOVE-1>TLM. The  Microsat  ASCII

frame thus contains two segments. Two  typical

segments of DOVE telemetry are shown below.

DOVE-1>TLM [01/29/90 22:08:46]:

00:59 01:59 02:86 03:30 04:58 05:58 06:6D 07:45 08:6C 09:66 0A:A1

0B:D9 0C:E8 0D:D8 0E:01 0F:23 10:CC 11:A8 12:00 13:01 14:A8 15:94

16:96 17:94 18:95 19:96 1A:93 1B:90 1C:9A 1D:98 1E:23 1F:5E 20:BC

DOVE-1>TLM [01/29/90 22:08:47]:

21:98 22:7B 23:24 24:21 25:2E 26:00 27:00 28:00 29:00 2A:00 2B:00

2C:00 2D:29 2E:00 2F:9B 30:C8 31:9C 32:11 33:DA 34:C0 35:95 36:A4

37:A4 38:B2 39:96 3A:00

The default segment identifier used by  WHATS-

UP  is in the first and second  characters  of

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 102

the   first  line  of  the  data.  A   segment

identifier   of  '00'  identifies  the   first

segment,  and anything else in  that  position

identifies the second segment.

AMSAT-OSCAR  13 RTTY Telemetry is  transmitted

in the form of Z blocks shown below.

Z  HI. THIS IS AMSAT OSCAR 13

05.02.44  4661

.0086   .0000   .07B9

64  6   0   1   16  218 1

193 170 158 143 181 144 147 140 200 7

147 7   7   7   165 29  100 7   149 7

10  7   145 115 34  7   153 129 122 180

152 73  7   145 137 55  7   183 136 151

7   154 137 169 211 142 127 100 9   140

161 7   173 149 150 154 14  131 127 210

HI THIS IS AMSAT OSCAR 13 08SEP90

NEW AO13  SCHEDULE FROM 17OCT90 AFTER MOVE TO  LON

180  LAT 0

MODE B  MA 000 TO 095

MODE JL MA 095 TO 125

MODE LS MA 125 TO 130

MODE S MA 130 TO 135

MODE BS MA 135 TO MA 140

MODE B  MA 140 TO 256

Note the spacing between the lines and the non

telemetry information at the end of the block.

UoSAT-OSCAR   11's   ASCII   telemetry   looks

different,  so WHATS-UP is told where to  look

for the telemetry by the following items.

9.2.13.99.9 Spare

9.2.13.99.10 Number of Lines

This  item is used in non packet telemetry  to

tell WHATS-UP how many lines of data are in  a

frame.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 103

9.2.13.99.11 First Line

This  item is used in non packet telemetry  to

tell WHATS-UP which line in the frame contains

the first line of data.

9.2.13.99.12 Last Line

This  item is used in non packet telemetry  to

tell WHATS-UP which line in the frame contains

the last line of data.

9.2.13.99.13 Header Lines

This  item is used in non packet telemetry  to

tell WHATS-UP how many lines there are in  the

header.

9.2.13.99.14 Header Display Flag

This  item tells WHATS-UP to display  the  non

packet telemetry header . A '1' means  display

it, a '0' means do not display it.

9.2.14 Digital Telemetry Status Channels

The  next  set  of lines instruct WHATS-UP  how  to  display

digital telemetry status in the Fuji-OSCAR 20 frame. Digital

status  channels  contain a number of status  points.  Fuji-

OSCAR  12  and 20 have three data elements in  each  digital

status  channel.  Typically, each line in  the  WHATS-UP.SYS

file contains ten items in the following format.

Status_Channel,      Status_Display_Page,       Status_Text,

Status_Bit_Mask,  Status_Row,  Status_Col,   Status_ON_Text,

Status_OFF_Text, Status_ON_Color, Status_OFF_Color.

Each item is described in the following sections.

9.2.14.1 Status_Channel

This  is  the  channel number  of  the  digital  status

telemetry  data  in the frame. Each entry must  be  two

digits.

9.2.14.2 Status_Display_Page

This  item is the Display page number. A 0 is  a  'wild

card' which will be displayed on every page.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 104

9.2.14.3 Status_Text

This is the text that is displayed in the page.

9.2.14.4 Status_Bit_Mask

This  is the bit mask to mask out the position  of  the

desired  bit. The masks are in decimal. ie. 1,  10  and

100.

9.2.14.5 Status_Row

This item is the Display page row. It identifies  which

row in the screen the data element will be displayed.

9.2.14.6 Status_Col

This  item  is the Display page column.  It  identifies

which  column  in  the screen the  data  item  will  be

displayed.

9.2.14.7 Status_ON_Text

This  item  is  the text that  is  displayed  when  the

spacecraft telemetry contains a 1 value.

9.2.14.8 Status_OFF_Text

This  item  is  the text that  is  displayed  when  the

spacecraft telemetry contains a 1 value.

9.2.14.9 Status_ON_Color

This  item is the color the text is displayed  in  when

the spacecraft telemetry contains a 1 value.

9.2.14.10 Status_OFF_Color

This  item is the color the text is displayed  in  when

the spacecraft telemetry contains a 0 value.

These  lines  also terminate with an '*' character.   A  few

typical lines from a Fuji.SYS file are shown below.

30,3,JTA Power    :,100, 4, 1, ON  , OFF ,11,10

30,3,JTD Power    :, 10, 4,25, ON  , OFF ,11,10

30,3,JTA Beacon   :,  1, 4,45, PSK , CW  ,11,10

38,3,Solar Panel 3:,100,12, 1, LIT , DARK,11,10

38,3,Solar Panel 4:, 10,12,25, LIT , DARK,11,10

38,3,Solar Panel 5:,  1,12,45, LIT , DARK,11,10

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 105

Note the blanks in the Text ON and OFF positions. The blanks

are used to ensure that a word such as "LIT" which  contains

three  letters fully overwrites a word which  contains  four

characters such as "DARK".

9.2.15 Packet/Link Parameters

The  next  set  of  items  are  the  Packet/Link  Parameters

configurations  (maximum = 16). If you want a value to  show

up in more than one page (other than the wild card [0])  you

must  enter  it twice (once per page).  Typically  each  row

contains 10 items in the sequence shown below.

Packet_title,  Packet_Type,   Packet_Lines,

Packet_Page,   Packet_Color,  Packet_Row,

Packet_Col,    Link_Page,     Link_Row,

Link_Col,      Binary_Byte_Count.

Each item is described in the following sections.

9.2.15.1 Packet_title

This  item is the name of UNP address  (e.g.  TLM,WASH,

BCXRT).

9.2.15.2 Packet_Type

This  item  is used by WHATS-UP to define the  type  of

telemetry   .  The  following  assignments  have   been

allocated but are not necessarily used in this  release

of WHATS-UP.

0    AMSAT-OSCAR 13 Non Packet Decimal telemetry.

1    AMSAT  Microsat  Packet Telemetry (TLM)  with  the

format  CC:DD where CC is the hexadecimal  channel

number and DD the hexadecimal data.

2    AMSAT Microsat Packet Telemetry, hexadecimal ASCII

STATUS telemetry.

3    Fuji  Packet Telemetry format of decimal  data  in

which  the  line  and the  position  on  the  line

identify the channel. WHATS-UP allows for up to 60

channels.

4    AMSAT Microsat Packet Telemetry hexadecimal Binary

STATUS telemetry.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 106

5    AMSAT Microsat Binary TLM Packet Telemetry.

6    UoSAT-OSCAR 9/11 ASCII Telemetry.

9.2.15.3 Packet_Lines

This item is the number of lines of text in the packet.

for example, the AMSAT TLM packets contain three lines,

the WASH packets contain only one.

9.2.15.4 Packet_Page

This  item  is the page that the raw  contents  of  the

packet will be displayed on. A '0' is a wild card which

will make WHATS-UP display it on every page. By careful

use of this item, you can display both raw and  decoded

packet data on the same page.

9.2.15.5 Packet_Color

This item is the color that the raw packet data will be

displayed in.

9.2.15.6 Packet_Row

This item is the row position that the raw packet  will

be displayed in, on the selected page.

9.2.15.7 Packet_Col

This  item is the column position that the  raw  packet

will be displayed in, on the selected page.

9.2.15.8 Link_Page

This item is the Display page for the cumulative  count

of the packet type. The wild card '0' applies.

9.2.15.9 Link_Row

This  item is the row position that the  packet  header

will be displayed in, on the selected page.

9.2.15.10 Link_Col

This item is the column position that the packet header

will be displayed in, on the selected page.

9.2.15.11 Binary_Byte_Count

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 107

This item is the number of bytes in a binary packet. AS

no  standard  currently  exists, we have  to  tell  the

computer how many bytes to expect.

These lines also terminate with an '*' character.

9.3 Telemetry Channel Extraction File

The  contents of this file are the defaults for  extracting  data

from  the playback file. A typical set are shown below.  WHATS-UP

does  a  string  match,  and looks for  the  first  time  that  a

particular string occurs. You may thus use the contents of a time

packet, or the time mark in a header.

ZCZC (default start time string) {start of file}

NNNN (default stop time string) {end of file}

2F

35

38

The  first line contains the start string. 'ZCZC' tells  WHATS-UP

to start extracting at the beginning of the file. The second line

contains   the  stop  string.  'NNNN'  tells  WHATS-UP  to   stop

extracting  at the end of the file. The remaining lines  are  the

individual channels (uppercase letters), one channel per line.

9.4 Extracted Telemetry Data File

This an ASCII string, comma delimited file which can be  imported

into your spreadsheet.  The format of the file is such that  each

line starts with a date code or packet header. Then each  channel

and  the  datum  associated  with that  channel  follow  for  all

channels  in the packet segment or non packet frame for each  and

every  channel displayed in the selected page on the screen.   If

the data from that channel is not displayed on the screen in  the

extract mode, the data will not be extracted.

A typical example of three lines from this file is shown below.

"01-Mar-91  03:39:46  DOVE-1*>TLM:","35",    6.7,"38",   -9.7

"01-Mar-91  03:39:56  DOVE-1*>TLM:","35",    6.7,"38",  -10.3

"01-Mar-91  03:40:06  DOVE-1*>TLM:","35",    6.7,"38",  -10.3

9.5 Doppler File

Doppler data are stored in this file. If the file does not exist,

it is created when needed. IF it does exist, data are appended to

the  file. The first line of data identifies the time, place  and

spacecraft as shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 108

"*** 22-Feb-91 02:57  Doppler Track STARTED for WEBER @ G3ZCZ"

The  next  line  provides the headings for the  five  columns  as

follows.

"Time","Doppler    Mark","Frequency","Doppler    Shift","Measured

Shift"

9.5.1 The Time

The Time is obtained from the PC clock.

9.5.2 The Doppler Mark

The Doppler Mark is a sequential count incrementing at  each

sample.  It can be used to provide an 'X' axis in  a  graph.

Elapsed  time  will be the Doppler mark  multiplied  by  the

Doppler sampling interval in seconds.

9.5.3 The Frequency

The Frequency is the measured radio frequency.

9.5.4 The Doppler Shift

The Doppler Shift is the calculated/predicted Doppler  Shift

in kHz.

9.5.5 The Measured Shift

The  Measured Shift is the difference (in kHz)  between  the

measured  radio  frequency and the beacon frequency  in  the

configuration file.

These  parameters  should allow you to plot  Doppler  curves  and

determine  orbit  parameters using the time of  closest  approach

technique.

Typical entries in the file are shown in the following lines.

"02:57:32", 2, 437110740, 8.1494,10.740

"02:57:37", 3, 437110740, 8.1354,10.740

"02:57:42", 4, 437110740, 8.1202,10.740

"02:57:47", 5, 437109860, 8.1045,9.860

"02:57:52", 6, 437110670, 8.0880,10.670

"02:57:57", 7, 437110620, 8.0709,10.620

"02:58:13", 8, 437110620, 8.0525,10.620

"02:58:18", 9, 437109780, 7.9868,9.780

.

.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 109

"03:07:17", 115, 437092570, -7.7884,-7.430

"03:07:22", 116, 437092570, -7.8204,-7.430

"03:07:27", 117, 437092570, -7.8511,-7.430

"03:07:32", 118, 437092570, -7.8802,-7.430

"*** 22-Feb-91 03:07  Auto End"

The lines of data end with a termination statement.

All  text items are enclosed in double quotation marks  (").  All

elements are delimited by comma characters (,).

If you plan to do Doppler measurements on the Microsats, then you

may be interested in the following information.

The  following  frequencies were taken after the  completion  and

final  tuning  of  the  completed  transmitter  assemblies.   The

measurements  were  made at a temperature of 23 deg.  C.  As  the

frequency does change with temperature the current frequency will

be slightly different.

PACSAT:   Normal PSK TX:        437.02625 MHz

Raised Cosine TX:     437.05130 MHz

S-Band TX:           2401.14280 MHz

DOVE:     FM TX No. 1:          145.82516 MHz

FM TX No. 2:          145.82438 MHz

S-Band TX:           2401.22050 MHz

WEBERSAT: Normal PSK TX:        437.07510 MHz

Raised Cosine TX:     437.10200 MHz

LUSAT:    Normal PSK TX:        437.15355 MHz

Raised Cosine TX:     437.12580 MHz

9.6 Kepler Element Files (*.2LN)

The following outlines the "NASA Two-Line" Keplerian data format.

Data for each satellite consists of three lines in the  following

format:

Example:

Mir

1 16609U          91 36.87776287 0.00038608           39705-3 0  2481

2 16609  51.6077 232.9299 0024950 205.6681 154.3223 15.64092124284608

Line 1 contains an eleven-character name.

Lines 2 and 3 are the standard Two-Line Orbital Element  Set

used  by NASA and NORAD.  The format  description

is:

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 110

Line 2

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

10-11     International  Designator  (Last  two  digits  of

launch year)

12-14     International  Designator (Launch number  of  the

year)

15-17     International Designator (Piece of launch)

19-20     Epoch Year (Last two digits of year)

21-32     Epoch  (Julian Day and fractional portion of  the

day)

34-43     First  Time  Derivative  of the  Mean  Motion  or

Ballistic  Coefficient  (Depending  on  ephemeris

type)

45-52     Second  Time Derivative of Mean  Motion  (decimal

point assumed; blank if N/A)

54-61     Radiation  pressure coefficient.  (Decimal  point

assumed)

63-63     Ephemeris type

65-68     Element number

69-69     Check Sum (Modulo 10)

(Letters,  blanks, periods = 0; minus sign  =  1;

plus sign = 2)

Line 3

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

09-16     Inclination (Degrees)

18-25     Right Ascension of the Ascending Node (Degrees)

27-33     Eccentricity (decimal point assumed)

35-42     Argument of Perigee (Degrees)

44-51     Mean Anomaly (Degrees)

53-63     Mean Motion (Revs per day)

64-68     Revolution number at epoch (Revs)

69-69     Check Sum (Modulo 10)

All other columns are blank or fixed.

9.7 AMSAT Format Element File (*.AMS)

The  AMSAT format file is the AMSAT file as received by means  of

packet radio. WHATS-UP scans the text for the word  "Satellite:".

When  it finds it, it assumes that the next few lines  carry  the

data in the fixed order.  You thus, do not need to edit the file,

just  capture it to disk, and save with a filetype of 'AMS'.  For

example the following file could be saved as ORBS-19.AMS.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 111

If you need to enter data in by hand, edit an AMSAT format file.

4116] B   BID: ORBS-019.D

Date: 20 Jan 91 02:58:24 Z

From: N4QQ@N4QQ

To: ALL@AMSAT

Subject: Orbital Elements   019.MICROS

R:910120/0258z 18616@N4QQ.MD.USA [Silver Spring,Md]  Z:20901

From: N4QQ@N4QQ.MD.USA

To: ALL@AMSAT

HR AMSAT ORBITAL ELEMENTS FOR THE MICROSATS

FROM N3FKV HEWITT, TX      JANUARY 19, 1991

TO ALL RADIO AMATEURS BT

Satellite: AO-16

Catalog number: 20439

Epoch time:      91014.08680572

Element set:     188

Inclination:       98.6853 deg

RA of node:        94.6412 deg

Eccentricity:    0.0010690

Arg of perigee:   259.0593 deg

Mean anomaly:     100.9381 deg

Mean motion:   14.28942714 rev/day

Decay rate:       3.95e-06 rev/day^2

Epoch rev:            5098

Satellite: DO-17

Catalog number: 20440

Epoch time:      91014.07510019

Element set:     187

Inclination:       98.6867 deg

RA of node:        94.6574 deg

Eccentricity:    0.0010787

Arg of perigee:   258.7019 deg

Mean anomaly:     101.2949 deg

Mean motion:   14.29004143 rev/day

Decay rate:       4.60e-06 rev/day^2

Epoch rev:            5098

9.8 Spacecraft Operations File

This  is a text file which can usually be obtained from a  packet

radio bulletin. A Typical file is shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 112

AO-13 TRANSPONDER SCHEDULE

Mode-B : MA 060 to MA 165 :

Mode-JL: MA 165 to MA 190 :

Mode-LS: MA 190 to MA 195 :

Mode-S : MA 195 to MA 200 : <= Mode B is Off - no swishing!

Mode-BS: MA 200 to MA 205 : <= QRP on BS please.

Mode-B : MA 205 to MA 256 :

Omnis  : MA 240 to MA 060 :

This schedule is expected to continue through 27 March 91

10.0 Glossary

AMSAT     The Radio Amateur Satellite Corporation

AO        AMSAT-OSCAR

AOS       Acquisition of Signals

ARRL      American Radio Relay League

CCD       Charge Coupled Device

DO        DOVE-OSCAR

DOVE      Digital    Orbiting    Voice   Encoder,    also    used

interchangeably with DOVE-OSCAR or DO

EWT       Early Warning Time

FM        Frequency Modulation

FO        Fuji-OSCAR

FSK       Frequency Shift Keying

LO        LUSAT-OSCAR

LOS       Loss of Signals

OSCAR     Orbiting Satellite Carrying Amateur Radio

AO        AMSAT-OSCAR

PSK       Phase Shift Keying

RTTY      Radio Teletypewriter

SSB       Single Side Band

TNC       Terminal Node Controller

TU        Terminal Unit (Radio Modem)

UO        UoSAT-OSCAR

WO        WEBER-OSCAR

11.0 References and Further Reading

The  best book is The Satellite Experimenter's  Handbook,  Martin

Davidoff,  K2UBC,  ARRL  1990, 2nd  Edition.  Other  articles  of

interest are listed below.

A  Turnstile Antenna for Two Meters, Joe Kasser,  G3ZCZ,  73

Magazine, June 1978.

Antennas for Microsat Ground Stations, Dick Jansson, WD4FAB,

The AMSAT Journal, Volume 13, Number 1, March 1990.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP.DOC Release 1.00 Page 113

Satellite Antennas from Recycled Junk, Howard Sodja,  W6SHP,

Proceedings  of the OSCAR Seminar, September 29 & 30,  1990.

Available from Project OSCAR Inc.

Microcomputer Processing of UoSat-OSCAR 9 Telemetry,  Robert

J.  Diersing, N5AHD,  The Satellite Anthology, Pages  46-51,

ARRL, 1988.

The  First Flock of Microsats, Tom Clark, W3IWI,  Jan  King,

W3GEY,  Bob  McGwier,  N4HY and the AMSAT  team,  The  AMSAT

Journal, Volume 12 Number 1, May 1989.

Ariane  Launch  Vehicle Malfunctions - Phase  3A  Spacecraft

Lost!, Tom Clark, W3IWI, Joe Kasser, G3ZCZ, Orbit  Magazine,

Volume 1 Number 2, June/July 1980.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

APPENDIX 1 Obtaining Further Information Page 114

For   further  information  about  any of the spacecraft and  the

Radio   Amateur    Satellite  program,  photocopy  and  mail  the

following form together  with  a self addressed stamped  envelope

(SASE) to :-

To:

[ ]       Project OSCAR Inc. POB 1136, Los Altos, CA. 94023-1136.

[ ]       AMSAT-UK, 94 Herongate Road, Wanstead Park, London  E12

5EQ.  Telephone (081) 989 6741.

I  read about the Radio  Amateur Satellite   program in  WHATS-UP

and  am interested in it.  Please  send  me  further  information

about  the  program,   and   details  of   membership   in   your

organization.

CALL ________________ TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY ___________________________________________

STATE _________ POSTCODE ______________________

Comments and questions ...

From WHATS-UP (c) G3ZCZ

COPYRIGHT Joe Kasser, G3ZCZ 1991.

APPENDIX 2 Amateur Radio Software by Joe Kasser G3ZCZ

PC-HAM 3.52

PC-HAM contains a number of programs some of which are  described

below.

LOGBOOK

Full  blown  logging package. With automatic check  of  logs  for

awards such as DXCC. Allows you to recall any entry by call  sign

within  seconds.  Indexed displays, QSLing, Contest  mode  QSLing

(prints the lot) and lots more written in dBASE3, but a  compiled

LOGBOOK.EXE file is supplied together with the source code.

CONTEST

Keeps Dupes in memory, logs QSO's to disk in format which can be

processed by the LOGBOOK package.

CQSS

Sweepstakes  game.  Work  the ARRL Sweepstakes  contest  on  your

computer.   You  are  located  just  outside  Washington  DC.   A

propagation   model is built in to the program. This  program  is

REQUIRED  training   for all sweepstakes  operators.  Program  is

based  on  the one  described in detail in 'Software for  Amateur

Radio'   by   Joe Kasser  G3ZCZ,  published by  TAB  Books,  Blue

Ridge  Summit,  PA. 17214.

WHATSON

Predict HF Propagation. Contest mode with printout to whole world

at hourly intervals. Needs BASIC

LAN-LINK 1.59

LAN-LINK  will  capture AMSAT related bulletins  from  packet

radio BBSs automatically.  Amongst its many features are :-

Zmodem binary file transfer capability.

Function  key  and  Menu driven.

Automatic  logbook  entries  for Packet  and   Mailbox/Beacon

Mode   AMTOR   Connects,  semiautomatic logbook  entries  for

other modes. Log file can be processed by the Logbook Package

of  PC-HAM.   Contest  operation,  sends   standard   message

and  automatically increments  QSO count. Automatic optimized

configuration  of  the TNC  for each communications mode.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

APPENDIX 2 Amateur Radio Software by Joe Kasser G3ZCZ

All  mode Function  key  'OVER' feature (End). There  are  10

files with fixed names (LAN-LINK.001  through   LAN-LINK.010)

which  may  be viewed and transmitted by means  of   function

keys. They may also be edited from the Edit Menu.

Time   display  and  event  scheduler.  ASCII  Text   Editor.

Customizable  Colors.   Access  to the TNC  Command  Mode  is

provided in  case  the user wishes to override any defaults.

Automatic  capture  to  disk of all  packet  radio  connects.

Automatic indication  of the number of Packet connects. Local

Area   Network  (LAN) message store and  Forward  capability.

Capable   of  automatic connect attempts to download  a   QTC

from another station in the LAN. Capable of automatic connect

attempts  to   a   packet  BBS  to  download  your   incoming

messages,   when   your  callsign  appears on  the  BBS  mail

beacon annunciator.

Capable   of automatically requesting Bulletins  on  subjects

that  interest   you  from your local  packet  BBS.  Digipeat

monitoring  and capture. Alert signal to let  you  know  when

a   predetermined  call  shows up  in  a  packet  header   on

frequency.

Conference  Mode  in  multiconnect  situations.  Bridge  Mode

in  multiconnect   situations.   Path  determination  to   DX

station   via  :QMH:.  Indicator  that  a  specific   station

designated   as   the 'target'  call   connected   in  Packet

Mode,   or  linked  to  AMTOR Beacon/Mailbox while  you  were

away.  Automatic   NET/ROM  and  KA Node path set   up   from

LAN-LINK.DIR call/path directory file.

Selective answering machine and MAILBOX using  NC/L   command

dialogue. Screen indication  of  connect  by desired  station

(target call).

Automatic  Beacon Mode CQ caller. Will call CQ   repetitively

and  either work the connect and keep going after  disconnect

or  signal  you  when  a reply  is  received.  SAREX  special

features. :QRA: trigger to determine who else is on channel.

Automatic  AMTOR SELCAL determination.

Function key  change  from monitoring AMTOR FEC CQ's to QSO's

in  progress  (chirpcopy). NAVY MARS RTTY  file  transmitting

protocols  for the PK-232. Log files in dBASE 3  format.  AND

MORE !!!!!

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP APPENDIX 3 SHAREWARE Page 117

The  Association of Shareware Professionals (ASP) has established

standards for its members and for any organization which has "ASP

Approved"  status. The ASP wants to make sure that the  shareware

principle  works for you. If you are unable to resolve a  problem

with  an  ASP  member  or  organization  (other   than  technical

support),  the ASP may be able to help. Please write to

The  ASP Ombudsman, 545 Grover Rd, Muskegon, MI. 49442, USA.

You  are encouraged to copy the floppy disk and share  it  freely

with  others.  You have the luxury of trying out the  product  at

your own pace and in the comfort of your own home or workplace.

After  you  have used the material for  a  reasonable  evaluation

period  (90  days),  you should either  discontinue  use  of  the

material  or register your copy.  Your support is  important  and

greatly  appreciated.  With it, Shareware authors are  encouraged

to  design and distribute new products. Without it, a great  deal

of high quality, low cost software will cease to be available.

Why pay at all?

*  You receive support from the author.

*  You receive a CURRENT copy of the program.

*  Your input and ideas help shape future products.

*  A sense of pride and ownership in having honestly

participated in the Shareware revolution.

*  You help to keep software prices down by supporting a

distribution method which doesn't depend on expensive

advertising campaigns.

Be  aware of the following restrictions, designed to protect  the

community  of Shareware users and to prevent greedy  people  from

taking unfair advantage of the trust, hard work and good will  of

Shareware authors.

1.   No  price or consideration may be charged for the  material.

However, a distribution cost may be charged for the cost  of

the diskettes, shipping and handling, not to exceed $6.

2.   The  files and programs on the disks may not be modified  or

deleted.

3.   The  material  cannot  be sold as part of  some  other  more

inclusive package.

4.   The material cannot be "rented" or "leased" to others.

5.   The end user must be told clearly in writing on the  outside

of  the package and in all advertising that the  diskette(s)

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP APPENDIX 3 SHAREWARE Page 118

are "Shareware."

6.   The package must contain a written explanation that the disk

is   for  evaluation  purposes,  and  that   an   additional

"registration  fee"  is  expected  by  the  author,  if  the

material is used beyond an initial evaluation period.

7.   In the case of distribution via any telecommunications link,

the following must be done:

An error checking protocol must be used.

The individual files must be combined into, and  transferred

in a library or archive format.

8.   Shareware  distribution  is  permitted only  in  the  United

States, Canada, England, and Australia.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP 1.00 Registration form

WHATS-UP 1.00 Registration Fee $35.00

LAN-LINK 1.59 Registration Fee $35.00

PC-HAM   3.52 Registration Fee $36.50

Register them all for $95.00.

Evaluation Copies free, you pay only for the disk/postage. For an

evaluation  copy  (unregistered  full  blown  version)  of   each

software  send a formatted disk and SASE to Joe Kasser.  Overseas

users,  send  $5.00 ($10.00 covers all three)  instead.  Try  the

software  first, and only register your copy if you make  use  of

it.

To: Joe Kasser G3ZCZ,  P O BOX 3419,  SILVER SPRING,  MD 20918.

CALL ________________ TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY ___________________________________________

STATE _________ POSTCODE ______________________

TNC TYPE _____________________ Home BBS __________

I enclose a check for ______. Please send me the latest version

of ______________, and register me as a user.

I enclose a disk/sase or $5.00. Please send me the latest version

of ______________________ to evaluate. If I like it and use it, I

plan to become a registered user.

Disk Size 5.25 _____(360k) 5.25 _____(1.2Mb) 3.5 ______(730k)

I AM NOT/AM currently using Shareware Version _____

which I obtained from _________________________________________.

Signature _______________________

Classroom Registration    6 to 10 copies    $195.00.

Radio Club Registrations 10 or more copies, 10% discount.

50 or more copies, 25% discount.

Designate  one  contact  person  to  receive  classroom  or  club

registered  updates, and  supply  a list of names and call  signs

of club  members  who will be using the Registered Club Copy.

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP Index Page 120

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP Index Page 121

Access, 2, 9, 14, 72, 85, 116

Acquire, 19, 90

Acquisition, 6, 11, 19, 43, 88, 90, 112

AFSK, 55, 80

Alarm, 6, 18, 89, 92

Alert, 6, 89, 92, 116

Altitude, 6, 14, 19, 35, 38, 40, 88, 90

AMSAT-OSCAR, 1, 5, 10, 12, 31, 40, 41, 42, 43, 44, 45, 47, 49, 50,

56, 57, 62, 65, 78, 79, 80, 81, 82, 83, 84, 94, 95, 96, 97, 98,

102, 105, 112

Analog, 17, 56, 69, 71, 72, 81, 82, 86

Antenna, 6, 43, 44, 46, 47, 48, 58, 59, 60, 61, 71, 81, 88, 90, 112

AOS, 11, 19, 20, 33, 43, 92, 93, 112

Apogee, 29, 35, 38, 39, 43, 47, 70, 78

Array, 12, 51, 52, 60, 65, 66, 67, 68, 69, 73, 76, 78, 86, 97

Azimuth, 19, 43

Band, 5, 44, 46, 47, 48, 49, 52, 63, 66, 112

BPSK, 80, 81

Capture-to-disk, 4, 11, 12, 14, 18, 26, 32, 89, 90, 94

Checksum, 49, 55

Clock, 5, 12, 19, 32, 33, 60, 86, 108

Coefficients, 51, 65, 66, 68, 97, 98

Color, 5, 6, 8, 11, 16, 17, 18, 20, 32, 33, 89, 92, 94, 96, 103,

104, 105, 106

Colors, 11, 13, 16, 17, 18, 20, 33, 116

Column, 12, 20, 86, 99, 104, 106, 110

Command, 9, 10, 15, 18, 23, 31, 64, 72, 79, 82, 83, 84, 86, 116

Configuration, 1, 3, 4, 5, 6, 7, 12, 13, 16, 17, 26, 27, 31, 32,

47, 79, 88, 89, 90, 93, 95, 96, 108, 115

Display, 1, 3, 6, 7, 10, 11, 12, 13, 16, 17, 18, 19, 20, 21, 22,

23, 25, 26, 27, 28, 29, 31, 33, 53, 56, 63, 77, 78, 83, 89, 92,

96, 99, 103, 104, 106, 116

Dit, 6, 89, 92

Doppler, 1, 4, 6, 8, 19, 26, 27, 29, 89, 92, 95, 107, 108, 109

DOVE, 5, 7, 13, 15, 17, 26, 30, 31, 44, 45, 46, 47, 49, 50, 51, 52,

63, 88, 89, 93, 94, 97, 101, 107, 109, 112

DOVE-OSCAR, 1, 5, 10, 12, 42, 43, 47, 49, 50, 62, 94, 96, 112

Downlink, 10, 41, 42, 48, 49, 55, 56, 63, 71, 72, 80, 81, 84

Eccentricity, 5, 29, 36, 38, 110, 111

Edit, 3, 4, 12, 13, 14, 22, 23, 24, 25, 26, 27, 28, 33, 110, 111,

116

Elevation, 6, 19, 43, 88, 90

Epoch, 5, 29, 36, 39, 110, 111

Equations, 9, 10, 49, 50, 51, 56, 57, 63, 65, 66, 68, 73, 75, 76,

82, 83, 85, 86, 97

Event, 62, 80, 116

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP Index Page 122

EWT, 19, 20, 21, 33, 92, 93, 112

Extract, 1, 3, 6, 12, 20, 21, 22, 88, 91, 107

FSK, 42, 44, 54, 61, 80, 81, 112

Fuji-OSCAR, 1, 5, 10, 42, 43, 45, 47, 49, 69, 70, 71, 72, 73, 74,

75, 76, 77, 78, 85, 86, 87, 94, 96, 103, 112

Hex, 11, 50, 51, 65, 66, 68, 76, 84, 86

Hexadecimal, 49, 50, 51, 55, 56, 75, 97, 105

Inclination, 5, 29, 35, 36, 37, 43, 70, 78, 110, 111

Interactive, 3, 11, 16, 18, 21, 22

J-pole, 46

Kenwood, 1, 11, 13, 23, 29, 91, 96

Kepler, 6, 8, 36, 88, 90, 109

Keplerian, 6, 15, 16, 19, 22, 26, 28, 93, 95, 109

LAN-LINK, 18, 115, 116, 119

Latitude, 5, 14, 70, 88, 89

Launch, 9, 10, 34, 69, 70, 79, 82, 84, 110, 113

Limit, 3, 6, 7, 11, 13, 16, 18, 89, 92, 96, 99, 100

Longitude, 6, 14, 88, 90

LUSAT-OSCAR, 1, 5, 10, 42, 43, 45, 47, 49, 50, 62, 68, 94, 112

Microsat, 1, 4, 13, 15, 30, 31, 40, 63, 101, 105, 106, 112

Mode-B, 57, 59, 112

Modem, 32, 45, 46, 48, 54, 55, 71, 83, 95, 112

Octal, 73

Orbit, 3, 4, 6, 10, 19, 20, 21, 23, 28, 29, 34, 35, 36, 37, 38, 39,

40, 41, 42, 44, 47, 50, 53, 56, 61, 62, 69, 70, 71, 78, 79, 80,

89, 90, 92, 93, 108, 113

Packet, 1, 2, 3, 4, 8, 12, 17, 18, 21, 30, 31, 32, 45, 47, 49, 50,

53, 71, 72, 80, 94, 95, 96, 102, 103, 105, 106, 107, 110, 111,

115, 116

Parameter, 6, 7, 11, 14, 54, 82, 83, 84, 89, 92, 94, 96

Perigee, 5, 29, 35, 36, 38, 39, 41, 43, 62, 70, 110, 111

Period, 19, 29, 36, 39, 43, 70, 78, 79, 117, 118

PgDn, 24, 27, 28

PgUp, 24

Playback, 3, 4, 11, 12, 13, 16, 20, 21, 22, 23, 27, 90, 91, 96, 107

Propagation, 49, 115

PSK, 5, 31, 32, 42, 44, 45, 47, 48, 52, 56, 61, 66, 67, 69, 71, 72,

74, 75, 77, 80, 83, 84, 85, 87, 95, 96, 104, 109, 112

RAAN, 5, 29, 36, 37, 38

Real-time, 11, 18, 22, 23, 53, 54, 96

COPYRIGHT Joe Kasser, G3ZCZ 1991.

WHATS-UP Index Page 123

Row, 7, 8, 12, 75, 86, 96, 99, 103, 104, 105, 106

RTTY, 1, 4, 12, 30, 31, 45, 56, 81, 95, 97, 102, 112, 116

RUDAK, 58, 59, 79, 80, 81, 83, 84, 85

Satellite, 1, 9, 10, 35, 36, 37, 38, 39, 40, 44, 46, 56, 69, 70,

71, 72, 79, 80, 82, 92, 109, 110, 111, 112, 113, 114

Scanner, 9, 44, 46, 47

Schedule, 4, 30, 32, 42, 56, 57, 90, 102, 112

Segment, 7, 96, 97, 99, 100, 101, 102, 107

Semimajor, 5, 36, 38, 39

Semiminor, 38

Spacecraft, 1, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 18, 19,

20, 21, 22, 23, 26, 27, 28, 29, 30, 31, 32, 33, 35, 37, 38, 39,

40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 51, 53, 54, 56, 63, 65,

66, 68, 69, 70, 71, 72, 78, 80, 81, 82, 85, 88, 89, 90, 93, 94,

95, 96, 99, 100, 104, 107, 111, 113, 114

Status, 1, 6, 7, 8, 9, 11, 12, 14, 16, 17, 32, 50, 53, 56, 58, 61,

64, 72, 73, 74, 75, 76, 77, 82, 83, 86, 87, 89, 92, 95, 103, 104,

105, 117

Telemetry, 1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18,

20, 27, 31, 32, 42, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56,

57, 63, 65, 66, 68, 71, 72, 73, 75, 76, 77, 79, 80, 81, 82, 83,

84, 85, 86, 87, 88, 89, 91, 92, 94, 96, 97, 99, 100, 101, 102,

103, 104, 105, 106, 107, 113

Turnstile, 43, 44, 46, 71, 112

UoSAT-OSCAR, 1, 5, 10, 31, 40, 41, 42, 43, 44, 45, 47, 48, 49, 53,

54, 55, 62, 63, 80, 94, 96, 102, 106, 112, 113

Valid, 15, 39, 72, 86, 100

WEBER-OSCAR, 1, 5, 10, 42, 43, 45, 47, 48, 49, 50, 62, 63, 66, 94,

112

Yagi, 44

COPYRIGHT Joe Kasser, G3ZCZ 1991.

######