GETTING STARTED

CHAPTER  2  -  GETTING STARTED

NOTATIONS USED IN THE 3by5 OWNER'S MANUAL

[xxxxx]  Square brackets enclose characters you type

at the keyboard.

<Enter>  Press the Enter key.

<Esc>    Press the Escape key (labeled Esc).

<Del>    Press the Delete key (labeled Del).

<Ins>    Press the Insert key (labeled Ins).

<PgUp>   Press the Page Up key (labeled PgUp).

<PgDn>   Press the Page Down key (labeled PgDn).

<Ctrl>   Hold the control key (labeled Ctrl) while

pressing another key.

<Scroll Lock> Press Scroll Lock key.

<Num Lock>    Press Num Lock key.

<Caps Lock>   Press Caps Lock key.

-----------------------------------------------------

|      Any text enclosed in a box represents text     |

|      shown on the video screen                      |

-----------------------------------------------------

"|"      The OR character.  For example, "dog|cat" means

"dog" or "cat".

"&"      The AND character.  For example, "dog&cat" means

"dog" and "cat".

Note:  The characters * ? +  cannot be used in a search key,

they are reserved for future use.

7

3by5

INSTALLATION PROCEDURES

Equipment Needed to Run 3by5

IBM PC, PC/XT, AT, or 100% IBM compatible.

256K RAM and at least one disk drive.

PC DOS Version 2.0 or higher.

A printer is optional.

Files on the 3by5 diskette

The dual  sided, double  density diskette 3by5 Master Diskette contains

the following files:

3BY5.EXE      The main 3by5 program.  It is a machine language

program and has no overlay routines.  IT MAY

BE REMOVED FROM THE DISK DRIVE AFTER 3by5 IS

LOADED INTO MEMORY.

ADDRESS.3X5   Sample file

REF.3X5       Sample file

PROGRAMS.3X5  Sample file

NOFIELDS.3X5  Sample file

FORMS.3x5     Sample file with print forms

ADDRESS.MAP   Index file

REF.MAP       Index file

PROGRAMS.MAP  Index file

NOFIELDS.MAP  Index file

README        If the file is present, it contains information about

3by5 that has been  released  since  your documentation

was printed.

CONFIG.SYS    This configuration file instructs DOS to reserve

space  for  up  to  16  open  files  at a time.  You

may edit it as you need.

8

GETTING STARTED

INSTALLING 3by5

MAKING A BACK-UP COPY OF THE 3by5 MASTER DISKETTE

1. Place a DOS system diskette into drive A and turn

on the computer.

2.  If asked, enter the date and time.

3.  Place a new or blank diskette in drive B.

4.  Type [format b: /s] <Enter>.

This will format the diskette in drive B and add

the DOS operating system to the diskette.  Make

certain that the diskette in drive B is new or

has no files that you want to save.

5.  Remove the DOS system diskette from drive A

and insert the 3by5  Master  Diskette  in  drive  A.

Leave the newly formatted diskette in drive B.

6.  Type [copy a:*.* b:] <Enter>.  This will copy

all the 3by5 files onto your newly formatted

diskette in drive B.  Use this diskette as your

working copy of the 3by5 Master Diskette.

7. Once you have printed ALL the documentation, type

[del b:chap*.doc] <Enter>.  This will erase the

documentation files from your working copy.  THIS

STEP IS NECESSARY TO GIVE YOU ENOUGH ROOM ON THE

DISKETTE FOR THE TRAINING LESSONS.

MAKING A WORKING COPY OF THE 3by5 PROGRAM

The  only  files  you  need  to  run  the 3by5 program are 3BY5.EXE and

CONFIG.SYS.  The purpose of the CONFIG.SYS file is  to instruct  DOS to

provide space  in memory  for up  to five  source files  and five index

files.  You may copy them onto a diskette  or hard  disk.  For diskette

based systems,  the simplest method is to format a DOS system diskette.

Then place your backup copy of the 3by5 Master Diskette in drive A, the

newly  formatted  diskette  in  drive B, and type: [copy a:3by5.exe b:]

<Enter> and then type [copy a:config.sys b:] <Enter>.

IF YOU NEED TO OPEN MORE THAN TWO 3by5 FILES AT A TIME, IT IS IMPORTANT

THAT YOU  BOOT THE  SYSTEM WITH A CONFIG.SYS FILE THAT SETS THE MAXIMUM

NUMBER OF OPEN FILES TO 16.

9

3by5

LABELING AND STORING 3by5 DISKETTES

When the installation  process  is  complete,  remove  the  3by5 Master

Diskette from  drive A  and store  it in  a safe  location.  It will be

your master backup copy in the event of a disaster.

Label the new diskette with the following:

3by5 Version 3.0  (C)  1986,  Softshell Corp.

All subsequent copies of  the  3by5  programs  should  also  be labeled

with the 3by5 title and the Softshell copyright notice.

If  you  have  a  copy  of  the  3by5  Starter Kit, you may copy it and

distribute copies to your friends.

3by5, 3by5 Plus, 3by5 Legal, 3by5  Reference Manager,  and 3by5 Corres-

pondence  Manager  may  not  be  distributed to other parties except by

Softshell Corp. or its designated dealers.

10