A BASIC INTRODUCTION TO KEGGING YOUR HOMEBREW

by Tom Lyons 76474,2350

Some of the comments I make in this file are just my opinions,

and other folks may have different experiences and better ideas.

If you are thinking about getting into a keg system for your

homebrew, check around and ask questions.  There are a lot of

friendly homebrewers and shop owners who are willing to help.

I started kegging my homebrew in the summer of 1990.  I have

found it to be far, far easier than bottling for several reasons.

I only have to clean one container, rather than 48.  I can carry

a keg to a party and don't have to worry about collecting bottles

afterwards.  I enjoy sediment-free beer (and my friends do,

too!).  I can use my kegging system to carbonate my beer if I

want, rather than priming with sugar, and get better control over

the amount of carbonation.

Of course, there are some trade-offs to be made as well.  There

are certainly some costs involved in purchasing the equipment

that you don't have when you bottle.  I have been able to keep my

costs down by some scrounging and a little good luck.  Another

problem is in sharing your beer with others.  It's not as easy to

carry a keg as a six pack!  There is a method that can be used to

fill bottles from your keg without losing carbonation, though,

that works fine for filling an occasional bottle for a friend or

for sending off to a competition.  It calls for another piece of

equipment called a Counterpressure Bottle Filler which I will

explain later.

The minimum equipment required to begin kegging consists of the

keg itself, a carbon dioxide (CO2) bottle with regulator, plastic

tubing, proper fittings for attaching the tubing to the keg, and

a tap of some kind.  Also you will need a few small band clamps

or something similar for securing the tubing to the fittings.

Advanced equipment can include additional kegs, a better tap,

lines and fittings for hooking up multiple kegs, etc.

The first consideration in kegging is refrigerator space.  Most

of us don't have a spare fridge laying around and our spouses

frown on taking over the one in the kitchen.  Mine does, anyway.

I searched the classified ads in the newspaper for a while before

I found my solution.  I bought a used "Brewmeister" beer

dispenser for $100.  Not only did I get a half-height

refrigerator with a built-in tap and drain, but it also came with

a CO2 bottle and regulator and various plastic tubing and

cleaning equipment.  Tough to beat at that price.

Used CO2 bottles and regulators can be found.  Try calling

beverage dispenser companies that sell soda equipment.  Also try

fire extinguisher companies.  If you buy used equipment, make

sure it has a current inspection date stamped on it.  CO2 bottles

must be tested ever four years or so, depending on your local

laws, and you won't be able to get it refilled if it is out of

date.  You can get your bottle refilled (and tested if necessary)

at larger fire extinguisher companies.  Regulators come with

either one or two gauges.  One gauge will show pressure output, a

second gauge if present will show pressure inside the CO2 bottle

itself.  The second gauge will tell you how much CO2 is in the

bottle, which is nice to know but not absolutely necessary.

The best source of kegs is the soda-dispensing kegs that Coke and

Pepsi use.  Check some restaurants in your area, you may find

someone who will let you have a keg or two for the price of the

deposit they pay on them.  These types of kegs are also available

from many homebrew supply shops.  New kegs are fairly expensive,

the used soda kegs can be purchased for around $25.  They will

often have syrup remaining in them which must be cleaned out

before use.  Also, a good idea is to replace the rubber o-ring on

the keg opening when buying a used keg.  Some soda kegs come with

pressure relief valves on them, some do not.

Coke and Pepsi use different types of fittings on their kegs.

The Coke kegs use a pin-lock fitting, and Pepsi uses ball-lock.

I am only familiar with pin-lock.  Homebrew supply shops often

carry fittings for both types.

There are two fittings on the top of the kegs, one for the CO2 to

come in and the other for the beer to go out.  If you unscrew the

input fitting, you will find a very short stainless steel tube

under it, which just sticks down into the very top of the keg.

Under the outflow fitting, however, is a long stainless steel

tube that goes all the way to the bottom.  These are set up to

allow the gas to flow into the top of the keg, which pushes the

liquid up from the bottom and out.  Since a "keg-conditioned"

beer will have yeast sediment on the bottom, the first thing to

come out when you open the tap will be sediment.  Some people cut

off the bottom 1" of the outflow tube to prevent this, but I

don't agree with doing that.  If you go ahead and expel the

sediment, you won't have to worry about it any more.  If you

leave it in the keg, every time you move the keg the sediment

will get stirred up.  Go ahead and leave the tube long, in my

opinion, and just get rid of the gunk with the first glass or two

that you pour.

Since the keg and most of its parts are stainless steel, some

special consideration must be given to cleaning and sanitizing.

There is a problem with using chlorine bleach solutions, as I

understand the bleach is corrosive to stainless steel.  This is

true if it is left to soak for long periods, and short contact

should not be much of a problem, but why tempt the fates, right?

There is a product called B-Brite that will clean and sanitize

stainless steel without any corrosive action.  It is available

at, of course, most homebrew supply shops.

The simplest type of tap is the little black plastic "party tap"

that is used on many commercial keg tapping systems.  It is

simply attached to the end of the beer line.  Other types of taps

can be really fancy, but must be mounted on some surface.  One

option is to drill a hole through your fridge and mount the tap

in that hole.  You should rig a catch-basin and drain of some

type for a tap like that.

Lastly, locate some plastic tubing for connecting the CO2 bottle

to the keg, and for connecting the keg to the tap.  The CO2 line

should be rated for pressurized gas (although we are dealing with

fairly low pressures) and the tap line can just be clear tubing

as used for syphoning.  Secure each connection with a band clamp

or those little plastic self-locking clamps.

When you have your keg system set up, test it by putting some

plain water in your keg and pressurizing the system.  Open the

tap to expel the water.  Check each connection for leaks,

including the fittings on the keg itself.

LET'S KEG SOME BEER!

The process for brewing beer to be kegged is no different than

for bottling, until time to prime and fill.  First, sanitize the

keg.  Unscrew the fittings, remove the lid and o-ring, and remove

the input and outflow tubes.  Be careful not to lose the input

tube, it is very small.  Also take care not to lose the little o-

rings on the top of the input and outflow tubes.  Fill the keg

with plain water and mix in 1 tablespoon of B-Brite per gallon.

Drop in all the parts you have removed from the keg to sanitize

them as well.  Let the solution soak for an hour or so, then

drain and rinse the keg.  You can sanitize the keg well in

advance of brewing, just reassemble it and pressurize it slightly

with CO2 to keep it sealed.

When ready to fill the keg, put your priming sugar solution

directly into the keg (you do not need to use a priming bucket).

Use less primer when you keg than when you bottle, about 1/2 cup

sugar or less per 5 gallons.  With the lid of the keg open, turn

on your CO2 regulator slightly for a moment to put some CO2 into

the keg.  The carbon dioxide is heavier than air and will settle

to the bottom of the keg.  As you then syphon your beer into the

bottom of the keg, it will be covered by a layer of CO2 which

will help greatly in preventing oxidation of your beer during

syphoning.  When your keg is full, replace the top and then once

again open your regulator slightly.  Pressurize the keg only very

slightly.  This will help seal the top, and will allow you to

check for air leaks in your fittings.

Once the beer has had time to carbonate, pop the keg into your

fridge and then give it a try.  You will draw off some sediment

in the first glass or two, which you may discard if you wish.

You may get a little additional sediment later as well, but no

big deal.

A great deal has been discussed about the proper pressure for

dispensing beer.  I think around 5-7 psi is fine.  You don't want

to gas it up too high.  Experiment with what works best for you.

At least as much has been said about proper serving temperature.

You know what you like, so act accordingly.

An alternative to priming for carbonation is to "force prime"

your beer by injecting CO2 directly.  This method works very

well, and if done correctly will allow you to control carbonation

very precisely.  The best thing about force priming, in my humble

opinion, is that there will be little or no sediment in your beer

at all.  There are tables and graphs that will show the amount of

pressure you should use at different temperatures, but trial and

error worked well for me.  Basically, get the beer cold and pump

up the CO2 to about 20 lbs or so.  Lay the keg on its side and

gently rock it back and forth a couple of hundred times.  You

will hear the CO2 continue to flow into the keg as it is being

absorbed into the beer.  Put the keg back in the fridge for a day

or two with the pressure turned back down.  Release pressure in

the keg back to normal and give it a try.  If the beer is

overcarbonated, release more pressure and let it settle for a

while.  If undercarbonated, just pump it up again.

COUNTERPRESSURE BOTTLE FILLERS

A problem I mentioned earlier about kegs was that you can't

easily fill bottles to send to others.  You can't just stick a

bottle under the tap, fill it and then cap it.  You will lose all

carbonation, and probably oxidize the beer too.  The problem is

that when the beer is under pressure inside the keg, the CO2

stays in the beer.  When the pressure is released (when the beer

goes through the tap into the outside air) the CO2 forms bubbles

and escapes.  A perfect solution would be to have a pressurized

room, so the carbon dioxide would remain in the beer while being

poured into the bottle and capped.  Alas, this is not a perfect

world, and building a pressurized room is beyond my scope of

abilities and resources.

A workable solution is to pressurize the bottle, fill it with

beer, and then release the pressure very gradually.  Once

unpressurized, the cap can be applied and the beer will hold most

of its carbonation.  This really does work quite well.  It

requires the use of a special piece of equipment called, of

course, a counterpressure bottle filler.  I'm not going to be

able to give plans for one in this file, but a very good one was

detailed in a recent issue of Zymurgy.  I built it and have been

using it fairly regularly.  To use it, you should first get your

beer as cold as you can.  You fill the beer bottle with CO2 until

it is the same pressure as the keg.  You then open a line from

the keg to the bottle.  Since the pressures are equal, no beer

will flow.  Start slowly releasing the pressure in the bottle,

though, and the flow will begin.  Slow is the key.  The more

gradual the change in pressure on the beer, the less carbonation

will be lost.  When the bottle is full of beer, you shut the beer

line off and continue to slowly release pressure.  When all

pressure is off, remove the bottle filler and cap the bottle.

KEGS AT PARIES

This is a real chance to experience the full benefit of kegging

your beer.  Pack up your keg, CO2 bottle, lines and tap, find a

tub that the keg will sit in, pick up a couple of bags of ice,

and head out.  You don't need to have the whole keg immersed in

ice, but try to get at least the bottom half in ice and maybe

drape a towel over the top half.  A plastic 5-gallon bucket (I

know you got one of them) works well.  Show folks how to operate

the tap (open it up all the way instead of just cracking it open)

and let them have at it.  They will love it.  Of course, a

responsible homebrewer encourages his friends to enjoy in

moderation if they are going to drive.  If they ain't driving,

though, then ignore that last statement!