���������������������������������ͻ

���������������������������������͹

�͹       F O N T E D I T       �͹

���������������������������������͹

���������������������������������ͼ

modified for the OKIDATA 93  by Neil J. Rubenking

FONTEDIT started out as a listing in one of the major magazines (sorry, I

can't remember which).  It was designed for an Epson, but I modified it for

the OKIDATA 93 (should work also with OKIDATA 92, and maybe 84?, 82?).  At the

same time I added in another magazine program to get a "lookup" of the ROM dot

definition of the first 128 characters, hoping to make it possible to redefine

without starting from scratch.  Since the printer will not properly print a

a pattern that includes two horizontally adjacent dots ( �� is no good!) the

ROM lookups aren't as much of a help as they might be.

FONTEDIT creates another BASIC program which, when run, loads up to 96

characters in the OKIDATA, replacing your choice of any 96 regular characters.

The created program also turns on the DLCG (Down Loadable Character Generator)

font.  This font stays on until either the printer is turned off or you send

the code that selects the regular font.  (Note that some Word Processing

programs automatically clear the printer of codes when they start.)  The DLCG

font stays loaded until the printer is turned off or cleared, so you can go

back and forth between the DLCG font and the regular font.

You can append more redefinitions (up to the limit of 96) to an existing

font file, but you must know the line number the new items should start on.

If you load and list the font program, you must not SAVE it without using the

ASCII option (SAVE"B:NEWCHARS.BAS",A).  If you SAVE it without that option,

you will not be able to append to it.

To turn DLCG font off:  LPRINT CHR$(27);"0";

To turn DLGC on again:  LPRINT CHR$(27);"2";

When you TEST a new character on the printer, the redefinition goes into

the DLCG and stays unless (1) you redefine the same character another way,

(2) it gets "pushed out" by having more that 96 characters defined, or (3) the

printer is turned off or cleared.  If you have done a lot of TESTing, you may

want to just turn the printer off and on again before running the font file

create.

The program NEWCHARS.BAS on this disk redefines the small alphabetic

characters to new and bizarre forms--try running it and then TYPEing this

document to the printer!  The characters "c" through "m" are now line drawing

characters.  In order to actually use them to draw lines, you will have to set

the line spacing to 1/12".

To reset spacing to 12 lines/inch:  LPRINT CHR$(27);"%9";CHR$(12);

To set spacing back to 6 per inch:  LPRINT CHR$(27);"6";

Neil J. Rubenking  (13 September 1984)