EMUL87 -- An 8087 Emulator for Turbo Pascal

EMUL87 is a terminate but stay resident program that emulates the functionality

of the 8087 math coprocessor for Turbo Pascal programs. It requires an 80286

or 80386 processor to operate. Using the EMUL87 programs allows the fastest

possible floating point math on machines with the math coprocessor, while

still allowing program operation on most common hardware (AT, no 287). You

can use inline 8087 instructions, without worrying!

The only major limitation is that older PCs and XTs (8088, 8086 and 186 based

machines) will HAVE to aquire a math coprocessor to run your software.

A more minor limitation is that the Floating Point Emulator is based on

Borlands Turbo C floating point emulator, and the emulation is NOT COMPLETE.

This means that EMUL87 cannot be used as a generic 87 eliminator. The missing

instructions are FINCSTP, FDECSTP, FFREE and floating stack wraparound.

The only program that I have had a problem with is the D86 debugger (part of

the A86 assembler package; everything works, but the 87 register display

does not come up properly).

Also, EMUL87 will not "fool" any program that is deliberately looking for

a 80287 chip. The '87 code will have to be generic 8087 to be successfully

emulated.

I tried wrapping the emulator into a Turbo Pascal 4 Unit, but with segment

naming problems, etc, it was going to be far too difficult to accomplish

in any meaningful time frame, so I settled for keeping a TSR implementation.

There are three programs in this package:

EMUL87      The 87 emulator itself. If the processor is not a 286 or

386, it will refuse to go resident. If a 287 or 387 is

present, it will refuse to go resident. If the word

"force" is put on the command line, it will override

the 287/387 detection and force EMUL87 resident. EMUL87

takes 14K of memory.

EMON        Turn EMUL87 on (if it is in memory)

EMOFF       Turn EMUL87 off (if it is in memory)

Typical use of the EMUL87 package is in a batch file:

EMUL87                    -- load the emulator

EMON                      -- enable it

...myapplication...       -- run the 87 code

EMOFF                     -- turn the emulator off

There is currently no way to remove EMUL87 from memory, nor am I likely to

provide one. It does contain a check to prevent from being multiply loaded.

Hints:

The force option is useful to test programs with 87 emulation, even if

you have the chip. Its easier than removing the chip!

EMUL87 works with Turbo Pascal 3 (8087 version), Turbo Pascal 4 with

{$N+} compilation and Turbo C. The popular TCDEBUG program benefits

because it is incapable of de-assembling emulator opcodes!

Bugs:

Watch out for Ctrl-Alt-Del! If the emulator is enabled, ALL floating

point instructions are going to be trapped. Ctrl-Alt-Del Warm Start

routine DOES NOT clear the instruction trapping! In order to be safe,

EMOFF should be put into your AUTOEXEC.BAT file, BEFORE any programs

are actually run.

If a program (like the popular ARC program) tries to detect the

presence of an '87 chip, and EMUL87 is not loaded to catch the instruction,

your machine WILL end up in hyperspace.