����������� The Soft Font Tutorial

Copyright 1991 by Elfring Soft Fonts

All rights reserved.

INTRODUCTION

Both laser printers and the DeskJet have become quite popular of late.

What's the reason for this surge in popularity?  A laser printer is faster

and much quieter than a dot matrix printer, although they cost a great

deal more. So why buy one? The majority of these printers are purchased

because of the wide variety of fonts they support.

Unfortunately, it isn't until you've owned the printer for awhile that you

discover just how hard it can be to actually use those fonts.  A fair num-

ber of people never do learn how to use different fonts with their

printers.

The major difficulty with fonts is finding a complete source of informa-

tion about them.  To use fonts with your laser printer you need: some

background information on typography, definitions of laser printer ter-

minology, explanations of fonts and how they are named, and details of the

printer control language, PCL, used by the HP LaserJet and compatible

printers.

There are three different kinds of fonts available for HP LaserJet and

compatible printers, or the DeskJet.  These are: internal fonts, cartridge

fonts, and soft fonts.  Once you start printing with these fonts there are

no real differences between them.  However, getting to the point where you

can actually print something with these fonts takes some doing. Note that

DeskJet soft fonts are not compatible with LaserJet fonts.

There are some disadvantages to soft fonts.  They take up a considerable

amount of disk space.  They also must be sent to your laser printer before

they can be used.  Finally, soft fonts are volatile- when you turn off the

power to your laser printer they vanish.  We feel, however, that the ad-

vantages listed previously greatly outweigh the disadvantages listed here.

Hence the need for this tutorial.

Hopefully, this tutorial will provide you with the information you need to

take control of your laser printer. Armed with this information you should

have no trouble selecting and using soft fonts in your printer.

One final note about the DeskJet printer. The DeskJet is a true line

printer, not a page printer. While it can handle soft fonts, it requires

an add on memory cartridge to do so. The DeskJet also has other limita-

tions, most of which are based on the fact that it is a line printer.

1

LASER PRINTING

Before going further into the details of soft fonts it's necessary to ex-

plain some of the basics of laser printing and its terminology.  If you

are new to laser printing you need to have a thorough understanding of of

what a laser printer is and how it works before progressing on to soft

fonts.  The first thing to do is to define a number of new terms as-

sociated with laser printers.

The type of printer that most people are familiar with is a typewriter.

With a typewriter, you strike a key and a character is printed.  A dot

matrix printer is very similar in nature to a typewriter.  You instruct it

to print a string of characters on one line and the dot matrix printer

prints each one sequentially.  With both typewriters and dot matrix

printers vertical spacing is usually accomplished by sending a carriage

return to the printer.

A laser printer operates on a completely different manner.  Laser printers

build and print an entire page at a time.  An invisible cursor is used to

build this page. The cursor tells the laser printer where to put each

character on the page.  This cursor can be moved automatically by software

in the laser printer, or it can be addressed with software on your PC.

To use a laser printer you must send an entire page of text, followed by a

command to print it.  (If you send more text than will fit on a single

page, the laser printer will interpret that as a command to print the

first page.) The interesting thing to note here is that the text that is

sent to the printer does not have to appear in any specific order.  You

can print at the bottom of the page, move to the top, and then go back to

the middle of the page.  As long as you don't go past the bottom of the

page, or send a "print page" command (Control-L) the laser printer will

not appear to do anything.  It will simply sit passively and receive your

text.

A dot matrix printer normally prints from left to right and from top to

bottom on a page.  (The more advanced dot matrix printer prints bi-

directionally, but still must advance from the top to the bottom of a

page.) A laser printer has no restraints on what direction it prints.  So,

it then becomes possible to print in several different ways, or orienta-

tions, on a sheet of paper.  Thus a new set of terms are needed to define

the orientation of text on a page.

A page may be printed in either portrait or landscape modes.  In portrait

mode the paper is positioned just as it would be in a typewriter.  The 8.5

inch dimension of the paper runs horizontally and the 11 inch dimension of

the paper runs vertically.  Text appears to run from left to right and

from top to bottom on the page.

In landscape mode the paper is positioned as you would place an envelope

in a typewriter.  The 11 inch dimension of the paper runs horizontally and

the 8.5 inch dimension of the paper runs vertically.  Text still appears

to run from left to right and from top to bottom on the page, but the page

has been rotated 90 degrees.

2

����������Ŀ  ��������������������������Ŀ

�          �  �                          �

�          �  �       landscape          �

� portrait �  �                          �

�          �  �                          �

�          �  �          mode            �

�          �  �                          �

�   mode   �  ����������������������������

�          �

�          �

������������

Note that the DeskJet Plus and the DeskJet 500 printers can perform

limited landscape printing using fixed width cartridge of soft fonts. The

original DeskJet can only perform landscape printing with the addition of

a special cartridge. None of these printers can print proportionally

spaced fonts in landscape orientation.

LASERJET & DESKJET BASICS

There are ten different models of LaserJet printers, and three different

DeskJet printers. Each model has different font handling capabilities and

comes with a different set of fonts. There are actually three different

kinds of fonts for either the LaserJet or the DeskJet printer and we

should define these before going further. Note that DeskJet and LaserJet

fonts are not interchangeable.

INTERNAL FONTS are stored inside your laser printer in Read-Only-Memory.

Each LaserJet model has a different set of internal fonts. Most compatible

printers offer different sets of internal fonts.

CARTRIDGE FONTS are Read-Only-Memory fonts that can be added or removed

from a laser printer. They come in a small box, or cartridge, that can be

plugged into your printer. Functionally, there is no difference between

internal and cartridge fonts once the cartridge is plugged into your

printer.

SOFT FONT is a term used to describe laser printer fonts that are sold on

a floppy disk. These fonts must be sent to a laser printer, downloaded,

before they can be used. They are stored in RAM memory in your laser

printer. See the following sections for a much more detailed explanation

of soft fonts.

LASERJET RAM MEMORY

All LaserJet printers require a certain amount of RAM memory in order to

run. Determining exactly how much memory you will need for your printer

has never been an easy task. The basic LaserJet II is equipped with 512K

of RAM, and a LaserJet III has 1 megabyte. Of this amount 295K or 807K,

respectively, is available for your use. All soft fonts are stored in this

3

memory and all graphic images must also be stored here before they are

printed.

A soft font requires about as much storage space in your laser printer as

it does in a disk file. Actually, there is a complicated formula that

determines exactly how much memory is used. However, it is easier to ap-

proximate it this way. A typical 10 to 12 point soft font requires 12K to

15K bytes of storage. More space is required for fonts that include ex-

tended character sets. In general, the larger the soft font size the more

disk space it requires.

So, if you are not printing any graphic images that 295K of free RAM space

can hold quite a few soft fonts. The average user does not need any extra

memory in this case.

If you plan on printing graphics you will need to set aside memory space

for them. A LaserJet can print to within 1/4 inch of each border. This

gives you an 8 x 10.5 inch area, or 84 square inches. At a resolution of

300 dots per inch you need 7,560,000 bits to hold this image. This is

equivalent to 945,000 bytes of memory. Adding a 1 megabyte memory board to

you LaserJet gives you a total of 1,048,576 bytes plus the original 295K,

or 1,343,576 bytes.

Thus, by adding a 1 megabyte memory card you can print a full page of

graphics and still have 398,576 bytes of RAM left for soft font storage.

This is probably all the memory you will ever need.

There are a number of PostScript add-on cartridges for the LaserJet line

of printers. In general, these cartridges need a large amount of addi-

tional memory before they will work. You usually need to add a minimum of

2 megabytes of memory to use a PostScript cartridge. You may have to add

as much as 4 megabytes.

DESKJET RAM MEMORY

All DeskJet and DeskJet Plus printers come equipped with 0K of soft font

RAM memory. Soft fonts will not work unless you purchase an add-on memory

cartridge. Each add-on cartridge for the DeskJet gives you 128K of addi-

tional RAM memory. A DeskJet will accept two of these cartridges giving

you a maximum of 256K of memory for soft fonts. The DeskJet Plus add-on

RAM cartridges contain 256K of RAM. This printer also has two slots so a

Plus can have up to 512K of RAM.

Note that the only use for RAM memory in either DeskJet printer is for

soft font storage. Additional memory will not speed the printer up, be

used as a printer buffer, or let you do landscape printing.

Determining exactly how much memory you will need for your printer has

never been an easy task. The DeskJet further complicates this by offering

algorithmic bolding and half sizing of soft fonts. Thus a 12 point medium

weight font also works as a 6 point medium weight, a 12 point bold, and a

6 point bold font!

4

A soft font requires about as much storage space in your DeskJet as it

does in a disk file. Actually, there is a complicated formula that deter-

mines exactly how much memory is used. However, it is easier to ap-

proximate it this way. A typical 10 to 12 point soft font requires 14K to

17K bytes of storage. More space is required for fonts that include ex-

tended character sets. In general, the larger the soft font size the more

disk space it requires. A typical DeskJet soft font is larger than the

corresponding LaserJet font.

Since the DeskJet is a line printer, graphic images do not take up any

memory space since they are not stored in the printer. Graphic printing is

slower than printing with internal or soft fonts. Adding memory to the

printer does not effect the print speed at all.

SOFT FONTS

All LaserJet and DeskJet fonts are bit-mapped. The laser in your laser

printer marks individual dots to print on a photo-sensitive drum. These

dots combine to print a letter or draw a line. The LaserJet series of

printers have a resolution of 300 dots per inch. So everything the printer

does needs to eventually be turned into a series of dots or bits.

The DeskJet uses a 50 dot ink jet, with a horizontal resolution of 600

dots per inch and a vertical resolution of 300 dots per inch. No two

horizontal dots can ever be printed side by side, so for all practical

purposes the DeskJet's resolution is the same as the LaserJets.

Before we can intelligently discuss soft fonts we need to define a few

typographic terms that will be coming up repeatedly.  These definitions

will help you understand how bit-mapped fonts are designed and created.

Three terms in particular are very important.  These are typeface, font,

and character.

A TYPEFACE is a particular design or description of what an entire al-

phabet should look like.  A typeface is independent of the size, height,

and weight, thickness, of the letters in it.  All right, for the really

detailed person we'll admit that there may be several typefaces that

describe a single alphabet on a typesetting machine.  This is not,

however, usually the case with laser printers.

A FONT is an implementation of a typeface in a single size and weight.  A

font is something tangible.  You can buy, rent, or lease a font and use it

to produce printed documents.  By contrast, a typeface is only a design,

and thus intangible.  Each font contains a collection of characters that

follow the design of a particular typeface.

Finally, a CHARACTER is a single letter or symbol within either a typeface

or font.  Once again, a character in a font is tangible and can be used to

print something.  A character in a typeface is simply a design.

Just what is a soft font?  A soft font is simply an implementation of a

typeface in a particular size and weight that is saved as a disk file.  A

5

soft font is particular to the laser printer it will be used on, not the

computer system used to store and transmit it.  Soft fonts designed for

the HP LaserJet will not work with a Xerox 9700 laser printer, even though

both printers operate in the same way at the same resolution.  On the

other hand, a soft font for the HP LaserJet can be stored and used from an

IBM PC or an Apple Macintosh.

A soft font is a collection of binary information that describes how to

print all the characters in a typeface.  The soft font also contains addi-

tional information about horizontal spacing, underlining, kerning, and

copyrights.  An HP LaserJet prints at a resolution of 300 dots per inch.

So each character definition in a soft font must tell your laser printer

where to put each successive dot in that character.

So . . .  a soft font is a collection of dot patterns for a laser printer.

The exact format or organization of this information varies among dif-

ferent brands of laser printers.  Each point size or weight requires a

different soft font disk file.

Note that all fonts for HP laser printers are bit mapped.  The font file

stores a specific bit value for each dot that will appear on the laser

printer.  Contrast this to a PostScript printer where outlines are used to

generate fonts.  (Please remember that even PostScript printers eventually

turn their font information into a pattern of individual dots.) In

general, quality 300 dpi bit mapped fonts will look better than those

produced from outlines, such as PostScript.

There are some inherent problems with the 300 dot per inch resolution used

on most laser printers.  This affects the quality of the soft fonts used

on them.  The 300 dpi resolution is at the limits of human vision.  Almost

anyone can "see" one three-hundreths of an inch.  When you create a soft

font from an outline format it must be translated into a bit-mapped for-

mat.  Some jagged edges will always result from this process.  A hand

edited font can correct those jagged edges.  Thus, most soft fonts

produced from outlines do not look as good as a hand edited font.

PostScript fonts contain "hints" that help avoid some of these problems.

Outline soft font generators for the LaserJet typically don't have this

advantage.  However, even the best PostScript font can not compare with a

hand edited, bit mapped font.  Hand edited fonts custom shape each charac-

ter so the fonts look as good as possible.  In addition, hand edited fonts

are typically dithered or feathered.  A dithered font gives much the same

effect as printing with double the normal resolution.  (The HP Letter

Gothic font is a good example of what can be done with this process.)

USING SOFT FONTS

Using soft fonts is not as simple as it may seem. You can't just copy a

font to your printer and print with it in your word processor. It can even

be difficult to use a font from the MS-DOS command line! Just what is in-

volved in using a font? This section should answer your general questions

about using fonts.

6

Before you can print with a soft font there are several things you must

do. First, that font must be sent to your printer. The process of sending

a soft font to a printer is called downloading. Downloading a font is

*not* the same thing as simply copying the font to your printer. The

printer must be told that this is a soft font and given information on how

to store it.

Next, you have to select the font. Just downloading a font to your printer

does not make it active (an active font is the one you are currently

printing with). A printer can hold numerous fonts inside its memory, but

can only print with a single font at one time. To select a font (making a

font active) in your printer you need to send a special control string to

the printer telling it how to locate the font.

Finally, most (but not all) soft fonts are proportionally spaced. This

means every character in the font occupies a different amount of space

(has a different width). Your application program needs to have a method

for dealing with proportionally spaced fonts or you won't get good

results.

If you are working from MS-DOS or from within an application program that

does not know anything about laser printers, the best way to deal with the

problems mentioned above is to use a simple control program to send the

font to the printer. ESF's DOWNLOAD or TSR Download programs will send a

font to your printer and select it. This will let you print in a single

soft font. You *won't* be able to justify or center text in a proportion-

ally spaced font with this method.

If, you want to print multiple soft fonts on a single page or justify text

in a proportionally spaced font you need an application program that is

"laser-smart". Laser-smart programs include word processors like WordPer-

fect, MS Word, MS Works, PC Write, & WordStar. Desktop publishing programs

like Ventura and PageMaker are also laser-smart. Finally, just about any

application program that runs under Windows 3.X is laser-smart.

All laser-smart programs require something called a "printer driver". The

printer driver tells the application program how to use and control

specific soft fonts. A printer driver is just a database of information

about fonts. This database will tell the application program all it needs

to know to use those fonts.

Generally, a printer driver has four types of information in it. The

printer driver will always have a name for the font that you will use to

select that font. For example a Roman font might have an entry in the

database like: Roman 12 point. This is the name you will see when you go

to select this font in your application program.

A printer driver may have the name and location of the actual soft font

file. Application programs like WordPerfect and MS Word automatically

download soft fonts for you. To do this they need the name of the soft

font file itself, so they can locate and send it to the printer.

7

A printer driver will always have the font selection string needed to ac-

tivate a font in your printer. In addition, the driver also has to have a

table that specifies the width of each character in a proportionally

spaced font, or a width of all characters in a fixed width font.

Some application programs will make their own printer drivers, based on

the soft fonts you own. WordStar, PC Write, and Windows are examples of

these types of application programs. Other programs like WordPerfect, MS

Word, & MS Works require someone else to build printer drivers for them.

It is important to remember that *no* laser-smart program can use soft

fonts without a printer driver. If you are using ESF's LaserJet or DeskJet

soft font packages with WordPerfect or MS Word/Works you *must* install

our matching printer drivers before you can use these soft fonts. If you

use something like WordStar, PC Write, or Windows, you *must* go through

the steps necessary to build a printer driver, yourself, for each of those

products.

For more information on soft fonts contact: Elfring Soft Fonts, PO Box 61,

Wasco, IL  60183  USA.  Voice: 708-377-3520  Fax: 708-377-6402

8