UTILITIES                                     MICHAEL MEFFORD

Vol 9 No. 2

BATCHMAN

This issue's utility, BATCHMAN, is designed both to give your batch files

significantly more power and to add many of the refinements we all wish DOS

provided.  You can check the DOS version, available memory, or display type

and then branch accordingly.  You can create colorful menus that branch on a

user keypress.  And, among its many other capabilities, BATCHMAN will let you

adjust the keyboard typematic rate and tame the grating DOS beep or turn it

into a tune.

BATCHMAN is actually a collection of 48 batch file enhancement utilities, all

rolled into one 6K program.  This provides more than convenience:  if

BATCHMAN's utilities were stored separately, at a minimum one cluster each,

they'd take up about 100K of disk space.

ERRORLEVELS

The key to BATCHMAN's operations is the ERRORLEVEL code, a number that programs

can report to DOS when they terminate and that can be acted upon from a batch

file.  I'll abbreviate ERRORLEVEL as EL for the remainder of this

documentation.

An EL can have a value between 0 and 255.  Each of the BATCHMAN commands

returns information as an EL.  An explanation of the syntax of each of the

BATCHMAN commands and the ELs they return will be found below.

The syntax for BATCHMAN is:

BATCHMAN [command] [arguments] [/R]

Entering BATCHMAN without any parameters (or with an invalid parameter) will

bring up a multi-screen display that lists all the BATCHMAN commands.  The

optional arguments are parameters for any given command.  If you add the

optional /R switch, BATCHMAN will display the EL returned from the command.

The Report option will be most helpful when you are designing and debugging

your batch files.  Be sure to place BATCHMAN in a directory included on the DOS

PATH so your batch files can find it.

THE BATCHMAN COMMANDS

I will be using the following conventions throughout the article:  the commands

will be listed first, followed by any parameters they might accept, followed by

the EL the command will return.  I'll put the EL in curley brackets so no one

will think it's part of the command syntax and try to type it in. For example:

CLS [nn]  {EL=0}

If it is entered without the optional nn color argument, the command:

BATCHMAN CLS

will clear the screen with whatever color it finds at the current cursor

position.  This provides the most flexibility when you plan to distribute a

BATCHMAN batch file to other systems because it retains the user's custom

color scheme.

If you wish to control the CLS color, just add the color argument nn, where

nn is a decimal or hexadecimal value derived from the table in Figure 1.

Hexadecimal numbers need a suffix of "h" to distinguish them from decimal.

Selecting a color in hex is easier than decimal because each of the 16 possible

values for each of the background and foreground colors fits into a single

digit:  the first digit is background, the second foreground.

To clear the screen so that subsequent DOS output will use blue letters on a

light gray background, for example, you would enter:

BATCHMAN CLS 71h

The first number (7) is the background color, and the second (1) is the

foreground color.  To do the same using a decimal number requires a little

math: the background color is multiplied by 16 and added to the foreground.

In the above example, the decimal equivalent would be (7 * 16) + 7 = 113,

so the equivalent command in decimal would be:

BATCHMAN CLS 113

Like its DOS counterpart, BATCHMAN's CLS also homes the cursor to the top left

corner.  Unlike the DOS CLS, however, BATCHMAN's will clear the whole screen

properly in video modes such as the EGA/VGA 43/50 line modes.

CECHO [C] [nn,]string   {EL=0}

CECHO is similar to the DOS ECHO command except you can choose the color of the

echoed string.  The nn color value is entered in the same manner as with CLS,

and again, if you don't enter a color value, BATCHMAN will use the currently

specified color.  To echo a "Zowie!" to the display in an eye-catching blinking

yellow on red, you would enter:

BATCHMAN CECHO CEh,Zowie!

The C (hex) value for the background color normally displays as orange for a

foreground color.  When used as a background color, however, it displays as the

blinking low intensity counterpart, red.  Keep in mind that any high intensity

color selected for the background will blink in its corresponding low intensity

color value.

The optional C parameter shown in CECHO syntax (don't confuse it with the hex

C color value in the immediately-preceding example) stands for "no carriage

returns."  Normally, both CECHO and the DOS ECHO move to the next line after

echoing a string.  At times this is not desirable.  For example, you might

wish to echo two strings, with different colors, on the same line.  With

BATCHMAN you can suppress the carriage return by adding a solitary C as the

first argument.  For example, to display a red on yellow blinking "Zowie!"

followed by a normal red on yellow "Batchman!" on the same line your batch file

would consist of:

ECHO OFF

BATCHMAN CECHO C CEh,Zowie!

BATCHMAN CECHO 4Eh,Batchman!

You can suppress the carriage return to prevent the display from scrolling when

you echo to the last line of the display.  This feature makes full screen menus

feasible, whereas echoing to the last line with the DOS ECHO command causes the

screen to scroll, spoiling any previous display layout.

SETLOOP  [n]  {EL=0}

.

.

DECLOOP  {EL=SETLOOP-1}

You use the SETLOOP and DECLOOP commands when you want to repeat a set of batch

file commands.  SETLOOP sets a loop counter to the n argument, which can be a

decimal number between 0 and 255.  Once the counter is set, the BATCHMAN

DECLOOP command will decrement the counter each time it is executed and return

the new value of the loop counter as an EL.  The short batch file:

ECHO OFF

BATCHMAN SETLOOP 10

:HERE

BATCHMAN CECHO ZONK!

BATCHMAN DECLOOP

IF ERRORLEVEL 1 GOTO HERE

will echo "ZONK!" to the display 10 times before terminating.  The batch

processor will branch to the HERE label as long as EL is 1 or greater.

Programmers may wonder where the loop counter is stored while other commands

are executing.  For the SETLOOP and PUSHPATH commands (PUSHPATH/POPPATH will be

discussed below) BATCHMAN creates a small TSR of approximately 500 bytes.  The

loop counter uses one byte for its storage; the balance is reserved for path

strings.  When the loop counter becomes zero, the BATCHMAN TSR data area is

returned to the system memory pool (unless PUSHPATH is also using the data

area).  Note that since the loop counter is only one byte, you can not do loop

nesting.

If you exit a batch file before SETLOOP has decremented to zero, the TSR data

area will not be released.  You should construct your batch files so that any

early exit from a loop will execute a SETLOOP 0 to force a recovery of the TSR

data area.  BATCHMAN is smart enough not only to find its previous TSR data,

but will prevent the creation of another if you try to enter two SETLOOP

commands.

QFORMAT [d:] [N]

{EL=0 if successful; EL=1 if not.}

QFORMAT is designed to wipe out all the files and subdirectories on a floppy

disk at a single stroke.  To keep you from accidentally trashing your hard

disk, the command will only accept A: or B: as drive arguments.  All other

drive requests are ignored, and you can omit the drive argument only if A: or

B: is your current default drive.

As a further precaution, QFORMAT warns you that all data will be lost and

requires that you press Y and Enter to confirm.  Any other keypress will abort

the process.  If you don't want BATCHMAN to pause and ask for your approval,

however, you can add the optional N argument to the command.  The N stands for

No Ask.  Use this option with caution!

QFORMAT will work only on diskettes that have already been formatted by DOS.

The DOS FORMAT command puts vital information about the media type, number of

clusters per FAT, and so forth, in the boot sector of the disk.  BATCHMAN's

QFORMAT command uses this information to find and place zeros in the FAT and

root directory sectors of the disk.

The reason why QFORMAT can be so fast is that it doesn't have to do the same

work as the DOS FORMAT command.  FORMAT not only does a low level format, but

also writes over the entire data area of the disk.  It's the low-level-and-data

-format process that takes forever.  A low level format really needs be done

only once, and QFORMAT blithely skips that step.  QFORMAT is sophisticated

enough to preserve bad sector information, however.

PUSHPATH

{EL=0 if successful; EL=1 if not}

.

.

POPPATH

{EL=0 if successful; EL=1 if not}

PUSHPATH saves the current drive and directory, which can then later be

restored with POPPATH.  Typically, a batch file that executes an application

includes a DOS CD command (often a change of drive as well) move operations to

the place where the application resides.  After the application terminates,

it's usually desirable to return to the original path.

By way of example, to run Lotus 1-2-3 on drive D: and then return to the

default path afterwards, your batch file would look something like this:

BATCHMAN PUSHPATH

D:

CD \123

123

BATCHMAN POPPATH

PUSHPATH saves the current path in a TSR data area, just as the SETLOOP

command does.  While only one byte is reserved for SETLOOP's loop counter,

however, BATCHMAN's TSR data area allows stacking up several paths with

PUSHPATH.  Subsequent POPPATH commands will retrieve the paths in a LIFO

(Last In First Out) stack basis.

Suppose, for instance, that you're  currently in your directory and you issue

the following commands:

BATCHMAN PUSHPATH

CD \DOS

BATCHMAN PUSHPATH

CD \PCMAG

BATCHMAN POPPATH

BATCHMAN POPPATH

In this sequence, the first POPPATH puts you back into the \DOS subdirectory,

and the last POPPATH restores you to the root directory from which you started.

The number of path layers that can be stored will vary with the length of each

path pushed onto the stack.  BATCHMAN's TSR area reserves 408 bytes for paths,

which allows for 6 levels of nested paths if each path is the maximum 65

characters allowed by DOS.  Shorter path names will allow nesting greater than

6 levels.  If necessary, you can change the storage size by modifying the

DIR_COUNT equate in the assembly listing.

BATCHMAN returns an EL of 1 if it finds the stack too full for another

PUSHPATH or if the stack is already empty with a POPPATH.  Otherwise, the EL

returned is zero.  When the last path is popped off the stack, the TSR data

area is discarded and returned to the DOS memory pool.

ANSI  {EL=0 if ANSI found;

EL=1 if not found}

This BATCHMAN command is designed simply to determine whether ANSI.SYS is or

is not installed.  If the EL is 1 you can then branch around any ANSI.SYS

escape sequences contained in your batch files.  BATCHMAN is smart enough to

detect not only ANSI.SYS, but my own ANSI.COM alternative

(see our January 31, 1989, issue), but it will not find other ANSI.SYS

emulators, such as DesqView's DV-ANSI.

BEEP [m,n[;m,n]...]   {EL=0}

The BATCHMAN BEEP command gives you a great deal of flexibility in controlling

the frequency and length of the sound DOS emits in response to a Ctrl-G

character.  The m,n beep arguments are decimal numbers and are used in pairs,

separated by a comma.  The m value represents the frequency (in Hz.), and n is

the number of 1/18th second increments that comprise the duration.

The frequencies of various musical notes can be found in the table in Figure 2.

The command for a middle C beep of a 1/2 second, for example, would be:

BATCHMAN BEEP 262,9

By separating the argument pairs with semicolons, you can produce a series of

tones with the same BEEP command.  You might, for example, find it interesting

to try:

BATCHMAN BEEP 392,3;523,3;659,3;

784,3;10,3;659,3;784,12

The lowest valid frequency is 19, a very low buzzing sound.  You can use a

value less than 19 to create pauses between tones:  no sound will be emitted,

but the duration argument is executed, effectively producing a programmable

delay between notes.

If you omit all arguments, BEEP defaults to 1046,1--a C note for 1/18 of a

second.  The DOS Ctrl-G beep is a frequency of 886 that lasts for one second.

Musically, a frequency of 886 is an out-of-tune A, which probably explains why

it grates on so many ears.

WAITTIL hh:mm[ss]

{EL=0 if successful; EL=1 if aborted}

WAITTIL provides a convenient way to pause until a certain time of day.  The

hh:mm:[ss] argument is the time in hours, minutes, and optionally seconds.

BATCHMAN will wait until the specified time arrives, but you can manually

abort the wait by entering any keystroke.  A typical application for WAITTIL

would be to execute an electronic bulletin board communications program that

would automatically download messages late at night when rates are lower.

WAITFOR [mm:]ss

{EL=0 if successful; EL=1 if aborted}

WAITFOR simply pauses for a specified number of minutes (mm) and seconds (ss).

You can omit the minute part of the argument if you desire.  As with WAITTIL,

the  WAITFOR delay can be over-ridden by pressing a key.

CURSORTYPE [m,n]

You can use CURSORTYPE either to create a cursor size that suits your taste or

to restore the normal underline cursor after an application alters it.

Entering CURSORTYPE without any arguments will default to the standard

underline cursor.  To create a custom cursor, you supply the m,n arguments as

the start and stop line values.  These values can be either decimal or hex;

for hex input, add an "h" suffix to the number.

The top of the character box is logical line zero; successively-numbered lines

increment downward.  The valid scan line values used for m and n are 0-7 for a

CGA, 0-13 for a monochrome or EGA, and 0-15 for a VGA.  The default underline

cursor start/stop lines are 6,7 for a CGA, 11,12 for a monochrome or EGA, and

13,14 for a VGA.  To set an EGA solid block cursor, for example, enter:

BATCHMAN CURSORTYPE 0,13

Note that the EGA BIOS on some video boards will attempt to override your

selections.  BATCHMAN temporarily turns off this BIOS EGA emulation logic for

all video boards when the cursor type is set, and this seems to work for most

systems.  However, if entering CURSORTYPE without arguments does not give you

the normal default underline cursor, you can explicitly request an underline

cursor that fits your display.  You may need to do this for video modes other

than the normal 25 line modes.  A "wrap around" cursor, in which the stop line

has a lesser value than the start line, is supported by the hardware of only

some video systems.

BREAK  {EL=0 if OFF; EL=1 if ON}

The BATCHMAN BREAK command returns the current state of DOS's Ctrl-Break

checking.  You can change the state of BREAK with the DOS command of the same

name.

DRIVEEXIST d:

{EL=1 if exist; EL=0 if not}

DIREXIST directory

{EL=1 if exist; EL=0 if not}

These two BATCHMAN commands are used to return an EL of 1 to your batch files

if the specified variable exists.

ISVOL [d:]volume

{EL=1 if exist; EL=0 if not}

Similarly, this command returns an EL of 1 if a particular disk volume exists

on the named drive.

YEAR    {EL=year from 1980

(0-199), where 0=1980}

MONTH   {EL=(1-12)}

DAY     {EL=(1-31)}

WEEKDAY {EL=(0-6), where

Sun=0; Sat=6}

HOUR    {EL=(0-23)}

MINUTE  {EL=(0-59)}

SECOND  {EL=(0-59)}

This group of BATCHMAN commands returns the system time and date. Note that

you can obtain a relatively fine resolution of the time.

VIDEOMODE  {EL=(0-19)}

The EL returned by the VIDEOMODE command allows you to determine which of the

possible screen modes listed in Figure 3 is currently being used by your

display.

ROWS  {EL=display rows}

COLS  {EL=display columns}

These obvious but very handy commands return the current number of rows or

columns displayed.

SETCURSOR m,n    {EL=0}

By supplying values for m (the row) and n (the column) you can use SETCURSOR

to place your cursor at the specified screen location.  By using SETCURSOR in

concert with CECHO you can precisely control the location of your messages.

For example, to display "Gotham City" in blue in the middle of the screen, you

would create the following batch file:

ECHO OFF

BATCHMAN SETCURSOR 12,35

BATCHMAN CECHO 1,Gotham City

Be sure to include the ECHO OFF command, both so that DOS won't spoil your

display by echoing the commands, and so that the cursor won't return to the

beginning of the next line for the CECHO command.

A related use of SETCURSOR is to hide the cursor.  You can do this by setting

the cursor to display on line 26, which is off screen.  Be sure to issue a

second SETCURSOR command that puts the cursor back on screen once the hidden

cursor is no longer needed, or you won't be able to see what you type!  If you

forget,  don't panic.  Just press Esc, blindly type CLS and press Enter, and

things will return to normal with the cursor homed to the top of the display.

E43V50

{EL=0 if successful; EL=1 if not}

This BATCHMAN command loads the 8x8 BIOS font for an EGA or VGA monitor.  This

changes the number of lines displayed to 43 (EGA) or 50 (VGA}.  You can use the

DOS MODE command to return to the normal 25-line display.

PRTSC  {EL=0}

This command will execute a printer screen dump exactly as if you had pressed

Shift-PrtSc on an 83-key keyboard or the dedicated Print Screen key on 101-key

keyboard.

COMPARE string1 string2

{EL=0 if match; EL=1 if no match}

The BATCHMAN COMPARE makes a case insensitive comparison between two strings.

For most purposes you'll probably want to use COMPARE instead of the case

sensitive DOS batch command:

IF string1==string2

With the DOS IF string comparison a non-case sensitive match requires two tests

--one for uppercase and one for lowercase.  Even this does not guarantee a

match if the string is a mixture of upper and lower case, as might occur if a

batch file were looking for a person's name, for example.  COMPARE solves this

case problem.  Note that the BATCHMAN COMPARE does not need the double equal

signs the DOS IF command uses.

CANCOPY filespec [d:]

{EL=0 if room to copy; EL=1 if not}

Before you copy a file or a number of files to another drive (especially a

floppy disk drive), you can use CANCOPY to see if there is enough room for all

the specified files.  The filespec you supply can use the DOS ? and *

wildcards.  If you don't specify the target drive, the default drive is

checked.  Thus, for example, to check whether the collection of PC Magazine

utilities you keep in a directory called PCMAG on drive C: will fit onto a

floppy, you would enter:

BATCHMAN CANCOPY C:\PCMAG\*.* A:

BATCHMAN returns an EL of 0 if there is room; otherwise a 1 would be returned.

If you enter this from the DOS command line, you'll want to include the /R

option to see the EL returned on screen.

WARMBOOT

COLDBOOT

As their names imply, WARMBOOT and COLDBOOT reboot your system.  WARMBOOT does

the same thing as pressing Ctrl-Alt-Del; COLDBOOT is like hitting the big red

switch.  A COLDBOOT does a memory check (among other things) that WARMBOOT

does not, and it takes a lot more time.  Of course, be forewarned that with

either of these commands all unsaved data in RAM memory will be lost.

Note that WARMBOOT and COLDBOOT may not work on some non-compatible systems.

SHIFT ALT | CTRL

{EL=1 if depressed; EL=0 if not}

This command returns the state of either the Alt or Ctrl key.  For example,

BATCHMAN CTRL

returns an EL of 1 if the Ctrl key is currently depressed.  You can use the

SHIFT command along with the BATCHMAN GETKEY command without arguments to

detect alternate key presses.  (See GETKEY, below, for an example.)

NUMLOCK [ON | OFF]  {EL=0}

CAPSLOCK [ON | OFF]  {EL=0}

SCROLLOCK [ON | OFF]  {EL=0}

These three self-explanatory commands toggle the current shift state of the

NumLock, CapsLock or ScrollLock keys when used without the ON or OFF options.

They explicitly turn the three keys on or off if the optional arguments are

supplied.

Thus, for example:

BATCHMAN NUMLOCK

will toggle the NumLock state on if it was off or off if it was on.  Entering

BATCHMAN NUMLOCK OFF

will turn the NumLock state off regardless of whether it is currently on or

off.  This latter would be a useful line in your AUTOEXEC.BAT if you use the

numeric keypad for cursor navigation instead of number entry.  Note that on

some keyboards the LED indicator lights may not faithfully track changes in

shift status.

RENDIR old new {EL=0 if successful; EL=1 if not}

Long-time users of our utilities will recognize this command, as it performs

the same function as the standalone RENDIR utility published in our April 15,

1986 issue.  The version presented here will rename a directory, but only on

systems using DOS 3.0 or later.  To use it simply enter the path of the old

directory and the new name.

For example, to change the name of a temporary directory named TMP on the C:

drive to a permanent name, say, ROBIN, you would enter:

BATCHMAN RENDIR C:\TMP C:\ROBIN

Be sure to include the complete path for both the old and new directory names

if the directory is other than the default.

ROMDATE  {EL=0}

ROMDATE displays the eight bytes of the BIOS date at address FFFF:0005 of the

ROM BIOS.  Usually, the display will be something like 10/07/87, but some

clone BIOSs do not have a date.  If no ASCII numbers are found, BATCHMAN

displays nothing.

GETKEY ['string' n]  {EL=scan code if no ['string' n] list; else

EL=position in list.}

There are probably hundreds of versions of the batch file GETKEY program.

They return the scan code of the next key you press as an EL. If you enter the

BATCHMAN GETKEY command without the optional (bracketed) arguments, it will do

the same thing.

Entered without arguments, GETKEY can also be used with the BATCHMAN SHIFT

command to detect Alt- or Ctrl- key combinations.  The scan code for the "Q"

key is 16.  To check for an Alt-Q keypress (perhaps as a Quit key command),

you would use the following batch file:

ECHO OFF

:GETKEY

BATCHMAN GETKEY

IF NOT ERRORLEVEL 16 GOTO GETKEY

BATCHMAN SHIFT ALT

IF NOT ERRORLEVEL 1 GOTO GETKEY

ECHO Alt-Q was pressed.

GETKEY's optional key list provides still another kind of functionality with

its optional key list.  The information you need may not be the scan code of a

given key, but rather its position in a string.  If you supply the optional

'string' argument to GETKEY, BATCHMAN will wait either until one of those keys

listed has been pressed or until you break out of the command with Ctrl-Break

or Ctrl-C.  An example may helpful here.  With the command:

BATCHMAN GETKEY 'yn'

BATCHMAN will wait until either the "Y" or the "N" key or one of the break keys

mentioned above is detected.   All other keypresses are ignored, and if the

break- out keys are pressed, the EL returned will be 255.  GETKEY is not case

sensitive, so if either "Y" or "y" is pressed, the EL will be 1, since the "y"

is the first character in the string of valid keypresses.  For "N", an EL of 2

will return.

The string must be enclosed in a pair either of single or double quotes.  If

you want the single quote included as a valid keypress, enclose it in double

quotes and vice versa for double quotes, as in the following example:

BATCHMAN GETKEY '"'

You can also include the function keys (even the F11 and F12 keys of the

extended keyboard) in the GETKEY list.  The function key numbers are listed

without quotes, (they are designated by the n in the command syntax above), and

may or may not be further identified with a leading "F."  Multiple function

keys are delimited by either a space, comma or semicolon.  To add the F1 and F2

as valid keys to the "yn" string example above, you would enter either:

BATCHMAN GETKEY "yn" F1 F2

or just

BATCHMAN GETKEY "yn" 1 2

If F1 is now pressed, the returned EL will be 3, since F1 is the third entry

in the list.

DOSVER {EL=(major*32)+minor}

DOSVER returns an EL representation of the DOS version in use.  DOS version

numbers have two parts, the Major version (the number to the left of the

decimal point) and the Minor version (the number to the right).  For DOS

version 3.10, then, 3 is the major and the 10 is the minor.  Since the largest

EL number is 255, the DOS version number does not fit conveniently into an EL.

For DOS 3.10, for example, if the dot is discarded the remaining 310 is too

large for an EL.

Of course, BATCHMAN could be content to return only the major number,

truncating anything after the dot, or even the first digit after the dot.  But

that would be less than sufficient for some applications.  So instead, the

DOSVER command compresses the version number by multiplying the major number by

32 and adding it to the minor number.  With DOS 3.30 the result will be (3*32)+

30=126; with DOS 4.00 it would be 128.  If there had been a DOS version 3.32

we would have a problem of course, since this would also return 128.

Historically speaking however, we are safe here.

MAINMEM n | R  {EL=0 if enough}

EXPMEM  n | R  {EL=0 if enough} EXTMEM  n | R {EL=0 if enough}

The arguments for these three memory commands can be either a decimal number,

n, or an R.  The decimal argument is used to check whether the desired amount

of memory (in KB) is free.  If BATCHMAN finds that there is enough free memory

to meet the request, the EL will be 0.  Thus, if an application needs 512K

bytes of the 640K maximum possible with DOS, you would enter:

BATCHMAN MAINMEM 512

The EL will return 0 if there are at least 512K bytes free; otherwise, a 1.

Your batch file can then branch to an error message if there is not enough

memory to run the application.  Note that these memory commands do not

allocate the memory.  They only check if it is free.

The alternative R argument stands for Report and will display the current free

memory of the type requested on the screen.  To see how much free expanded

memory you have, for example, enter:

BATCHMAN EXPMEM R

The report option of the three types of memory will go nicely with BATCHMAN's

ROMDATE command in a system information batch file.

DISPLAY  {EL=display type}

The DISPLAY command will report on the type of display your system has

installed.  The values returned by DISPLAY as EL codes are:

1=MDA

2=CGA

4=EGA color

5=EGA mono

6=PGS

7=VGA mono

8=VGA color

11=MCGA mono

12=MCGA color

CPU20 {EL=CPU type}

Similarly, the CPU command returns an EL that indicates the microprocessor

used by your machine.  The specific EL numbers are:  1=8086/8088; 2=80186;

3=80286; and 4=80386.

WINDOW m,n,w,h[,c,b]  {EL=0 if

successful; EL=1 if not}

The powerful WINDOW command displays screen boxes.  It provides several

parameters and options.  The first two decimal numbers (m,n) are the row and

column of the top left hand corner of the window.  The next two decimal

numbers (w,h) are the desired width and height of the window (including the

border characters.)

The smallest size that WINDOW allows is 2 characters in width and height.

The optional fifth parameter, c, can be either a hexadecimal or decimal number

for the color of the window.  If no color argument is supplied, the current

color of the screen is used by default.  The last parameter, b, sets the type

of border used to surround the window.  If no parameter is found, spaces are

used.  If you enter a minus sign for b, a single-line box character will be

drawn.  An equals sign for b will produce a the double-line box character.

In all cases, the center of the window is cleared with spaces of the chosen

color.  For example, the command

BATCHMAN WINDOW 1,1,80,25,17h,=

will create a blue window with white double line box characters that will fill

the entire screen, making a nice menu frame.  Text could then be added with

the BATCHMAN SETCURSOR and CECHO commands.

TYPEMATIC [m,n | N] {EL=0 if

valid parameters; EL=1 if not}

m=typematic rate (0 - 31);

larger m=faster rate

n=initial delay  (0 - 3);

larger n=longer delay

N=normal: m=20; n=1

default: m=25; n=0

TYPEMATIC works only with keyboards that support adjustable typematic

(automatic key repeating) rates at the BIOS level.  That includes most

machines, starting with the AT, but not for all clones.

The first optional decimal parameter, m, is a number between 0 and 31 and

represents the desired typematic rate.  The larger the m value, the faster

your keyboard will repeat keystrokes.  The second parameter, n, has a range of

0-3, and sets the initial delay.  This delay is the length of time you must

hold down a key before it begins repeating.  The larger the delay value the

longer the delay.  The delay parameters are in increments of 1/4 second, with

0=1/4 and 3=1 second.  The typematic parameters are divisors and cannot be

easily be translated into fractions of a second, however, and so should be

thought of only as a gradient scale.

If you don't enter any parameters, TYPEMATIC defaults to what I consider ideal,

a repeat rate of 25 and a delay of 0.  The 25 translates into about 17

characters per second with an initial delay of 1/4 second.  You can restore

the typematic 2rate to the normal hardware defaults (11 characters per second

with a 1/2 second delay) by using the N parameter.  Just enter:

BATCHMAN TYPEMATIC N

and the normal values of 20, 1 will be used.  Once you try adding a little

zoom to your keyboard, however, I don't think you'll ever go back to the

hardware defaults.

####################################################################################