This is a repost of my original file uploaded to PLINK on April 21, 1989.

I realized that there was a bit of information left out which made the

description of this file somewhat inaccurate in that I didn't really

mention the "performance" increases realized with a turbo-board.

Since I only own an '020 not an '030 board, I can only state reported

figures about performance. But, I have included Ronin's (written by John

Hinckly of RONIN and with their permission) CPU_SPEED test program which

graphically illustrates the overall CPU speed of your system. This is an

excellent utility for seeing just how much a task slows down your system

while running. This program is basically the SEIVE as stated in BYTE mag-

azine with the exception that a 256K instead of an 8K cache is used.

On a stock Amiga, running the CPU_SPEED test will show the CPU speed at

a value of 1. With a plain '020, that will change to 1.8-1.9. With a

plain '030, the speed picks up to about 2.01. Not much, considering the

expense. But add a math chip and you will get quite decent results in

heavy computational programs like Sculpt-3D.

Now, add 32-bit memory and this is where you start to pick it up a bit!

CBM's '020 board clocks at a best of 3.8-3.9 with it's memory. The Ronin

board clocks at 5.0. Now were talking much faster screens, DIR listings

and the like.

So drop in an '030 piggyback module and the rating goes to 6 on the

scale. Remember, now we have a 256 command cache verus a 20 command

cache on the '020. Plus, with the '030s built-in Memory Management Unit we

can now map the KickStart Routines to 32-bit memory. This means MUCH

faster sytem operation. Having never seen one operating, I can only

speculate...

Up to now, we're still running at 14 MHz. What happens to the rating by

going to the new 25-28 MHz boards?

The rating goes off the scale to somewhere in the neighborhood of 12-15.

Makes you kinda drool, doesn't it????????

The rest of the file is the same as was posted earlier. Start saving your

coins!!!

Jay Kubena

***************************************************************************

THE FOLLOWING IS A TEXT FILE COMPARING THE PRICE AND PERFORMANCE OF THE

RONIN 68030 BOARD VRS THE GREAT VALLEY 68030 BOARD.

*************************************************************************

The following information was given to me by personnel at Ronin and GVP.

I am posting this information so as to better inform Amiga users

concerning expansion products for their AMIGAS. For the record, I own

the Ronin '020 Hurricane board and 32-bit 4-meg RAM board and the GVP SCSI

controller with 2-meg on-board RAM.

Both companies offer excellent products. The only discouragement with

the Ronin board is an inherent (due to the board design) problem with DMA

contention (manifested by slow harddrive throughput) when using such DMA

products as Commodore's 2090A harddrive controller. However, this same

design does make the Ronin board the fastest of the turbo-boards, and the

problem disappears if a non-DMA controller such as GVP's or Cltd' is used.

The GVP controller I bought from GO-AMIGO did have a problem with a very

tight fit in my A-2000 slots, the green wave solder insulation peeling

from the back, and the DB-25 connector at the rear of the controller being

incorrectly positioned to the edge of the cut-out so that the harddrive

connector couldn't mate all the way down onto the GVP connector. GVP is

exchanging the board with no hassle and with apology.

***************************************************************************

RONIN 68030 28 MHZ                             G.V.P. 68030 25 MHZ  *

* (BOARD ONLY) *                            (BOARD WITH 25 MHZ '030 CPU)

INCLUDES ON-BOARD AUTO-                      * 16 MHZ '030 BOARD IS AVAIL.

BOOTING SCSI CONTROLLER                      LIST PRICE    -   $849.00

* INCLUDES NON-SCSI/NON ST-506

Has two sockets for future                     HDDRIVE INTERFACE CALLED AN

software enhancements                          AT-INTERFACE, PROPRIETARY !!

* Has two sockets for future

software enhancements

( BOTH BOARDS SOCKETED FOR 881' OR 882' MATH CHIP )

Clock speed   -   28 mhz *                    Clock speed   -    25 mhz

Board runs synchronous                        Board runs asynchronous

Not recommended with DMA controllers          Works with DMA controllers

except Micro-Botics HardFrame

LIST PRICE   -   $780.00                      LIST PRICE    -   $999.00

---------------------------------------------------------------------------

* The 33 mhz '030 chip                       * 25 MHz 68030 chip included

lists at $720.00. A 25 mhz                    with board purchase.

'030 chip might be clockable

at 28 mhz saving about $150.00

PRICE SO FAR.... $1500.00                     PRICE SO FAR........ $ 999.00

---------------------------------------------------------------------------

25 MHZ 68882 math chip *                      25 MHZ 68882 math chip if

LIST PRICE...... $ 620.00                     bought with board   $ 500.00

PRICE SO FAR.... $2120.00                     PRICE SO FAR........ $1499.00

---------------------------------------------------------------------------

* A 20 mhz 68881 math

chip ($265 LIST) can be

used saving $335.00.

---------------------------------------------------------------------------

RONIN 4-MEG RAM BOARD *                        G.V.P. 8-MEG RAM BOARD

(BOARD ONLY) **                                  (BOARD ONLY)

LIST PRICE    -   $500.00 *                    LIST PRICE  - ... N.A. .....

---------------------------------------------------------------------------

RAM-BOARD WITH 4 MEGS OF RAM *                 RAM BOARD WITH 4 MEGS OF RAM

Chips used  - 32 maximum                       Chips used   -  4

Config.  -  256Kx4                             Config.  - 1 meg SIMMS

Speed   -  100 nanos.                          Speed -80-nanos. Nibble mode

1-wait state, 0-wait may                       0-wait state

be possible with 60-nanos.

RAM not auto-booting                           RAM auto-boots

RAM TOTAL PRICE - $1860.00                     RAM TOTAL PRICE  -  $1995.00

---------------------------------------------------------------------------

* Memory can be added                          * 4 more Megs of RAM can be

in 1-meg increments.                            installed onto board for

LIST PRICE  -  $340/MEG                         a total of 8 megs

* I have found these                           * Nibble mode SIMMS are the

chips for $22.00 a chip which                    same ones found in the

equals $176.00 a meg                             NEXT computer.

* 16-megs possible with

1-megx4 chips. Price N/A

---------------------------------------------------------------------------

TOTAL PRICE FOR A 28 MHZ '030                 TOTAL PRICE FOR A 25 MHZ '030

BOARD WITH 25 MHZ '882 AND 4-                 BOARD WITH 25 MHZ '882 AND 4-

MEGS OF 32-BIT RAM   (list)                   MEGS OF 32-BIT RAM   (list)

-> $3980.00 <-                                  -> 3494.00 <-

AVAILABILITY                                    AVAILABILITY

------------                                    ------------

Mid-May 1989                                    Shipping now

***************************************************************************

Obviously, the total bottom line for either board can be greatly influenced

by differing the speed of the '030, the type of math chip used, and by

starting out with a lesser amount of RAM. You will need at least two megs of

32-bit RAM when using an '030 as the KickStart routines are re-mapped to

this fast memory, which then REALLY starts speeding up the entire system.

You could get a Ronin board running an '030 at 28 MHz with 20 MHz 68882

math chip, and 2-megs of 32-bit memory for about $3000.00 by buying your

own RAM chips. For another $500.00 (list), however, you can get the GVP

board with another 2 megs of RAM.

Remember, the "bottom line" quoted above is List Price. User end price

should be quite a bit less, especially if you don't mind shopping around

for your own chips!

---------------------------------------------------------------------------

Does having a turbo-board make that much differnce?        In a word,

YOU BET IT DOES!

Why do you think the MS-DOS world is all going ga-gah over the 386 CPU?

You have to experience the difference a turbo-board makes in multi-tasking.

Once you've turbo'ed, you'll never go back, why, I even TYPE faster now!

Hope you've found this information useful. Please feel free to comment on

this file.

Jay Kubena

April 21, 1989

PLINK ID# Jay-Kub