Amiga 500 - to - PC style Hard Disk Interface

A Demonstration Software/Hardware Construction Project

Products ByNery

14 Chambers Court

Mansfield, Ma. 02048

Copyright 1990, Products ByNery

Contents

Chapter 1  INTRODUCTION                               1

1.1  Background  . . . . . . . . . . . . . . . . . 1

1.2  Shareware Plea  . . . . . . . . . . . . . . . 2

1.3  Performance . . . . . . . . . . . . . . . . . 2

1.4  Distribution Contents . . . . . . . . . . . . 3

1.5  DISCLAIMER  . . . . . . . . . . . . . . . . . 3

Chapter 2  CIRCUIT ANALYSIS                           4

Chapter 3  CONSTRUCTION                               6

3.1  Wiring List . . . . . . . . . . . . . . . .   7

Chapter 4  CHECKOUT                                  11

Chapter 5  CONTROLLER/DISK DRIVE SELECTION           12

Chapter 6  INSTALLATION                              13

6.1  Hardware  . . . . . . . . . . . . . . . . .  13

6.2  Testing and Low-Level Formatting  . . . . .  14

6.3  Software Installation . . . . . . . . . . .  14

Chapter 7  REGISTRATION                              16

i

Chapter 1

INTRODUCTION

Are you fed up with the high cost of Amiga hard disks?

Do  you  wonder  what you get when you purchase an interface to a

pc-disk?

Do you want a low cost alternative?

Well, here's your answer to all of the above questions  and more.

This project will allow you to construct a LOW COST  adapter that

allows the interface of a PC-style hard disk to  the  Amiga  500.

The adapter itself can be built for less than $20.00 and consists

of  only  3  integrated circuits that cost less than $1.00 total.

The majority of the cost is for the two edge-board connectors.

What this means is that with a little shopping you should be able

to  add  a  10Meg disk for about $150.00, or $180  with  a  power

supply.    That's about the same as  the  price  of  an  external

floppy.

Section 1.1  Background

This project started about the time a Canadian Company introduced

a PC hard disk interface for the Amiga 500. As memory serves, for

over $150.00 you could get:

1. A schematic.

2. A device driver.

3. Some photos.

4. A bare PC board.

5. Instructions on where to BUY the parts and construct the

interface.

This treatment incensed me.   I will never fault the Capitalistic

system,  BUT, to be taken advantage of  was  certainly  over  and

above  the normal rules. At that time, I  decided  to  design  an

interface and write a driver.  The  following files make  up that

decision.

Before going straight to the design and implementing, please read

the  following.

- 1 -

Section 1.2  Shareware (sorta) Plea

This  project is SHAREWARE.  The intent, as I see it, is to share

the  development cost so  that future improvements can be offered

as well as enhancements. BOTH OF WHICH WILL BE  COMING  TO  THOSE

THAT REGISTER.

At first, and maybe still  if enough request it when registering,

I  was  going  to offer a  circuit  board  to  entice  people  to

register.  The board is lazed out and I am currently running with

it.    After  researching the costs, I found it was going to cost

another $1000.00 to produce the first lot.  Since I  already have

invested about $1500, I decided not to offer a board.

Then why should you register?  Easy, the driver supplied  is only

a  partial  implementation.   It will support any drive having at

least 4 heads and 306 cylinders.  This translates to about 10 Meg

of disk.

If you register, you get:

1. A configurable driver, which provides:

* Both RLL and MFM controller support.

* Programmable drive size (tracks and heads).

* Support for 2 drives of different or same sizes.

2. User friendly installation/maintenance program.

3. A positive transparency to allow PC board manufacture.

4. A complete manual including instructions on how to make

the PC board.

The registration packages is completed and currently available.

For   the  sake  of   full   disclosure,    here  are  the  known

deficiencies with the current revision of the  REGISTERED driver,

rev 2.0.

1. All errors reported as 0 length transfers.

2. Drive does not AutoBoot.

3. All IO requests, except for immediate operations, are

queued.  This is a performance issue only and transparent

to the user.

Section 1.3  Performance

The results of this project do not produce a screamer, based upon

the claims made by other  manufacturers,  however  compared  to a

floppy disk, I don't think you'll be disappointed.

The performance figures were supplied by the program:

DiskPerformance - V3.0 - 03/21/89

- 2 -

The  test  was  run on the  last  partition  of  a  33  Megabyte,

Micropolis Disk controlled by a  Western  Digital  RLL  hard disk

controller, model 27X and  an  interleave  of  10.  The partition

size was 15 Megabytes and was empty.

Following are the results of the test:

File create/delete: create 16 files/sec, delete 40 files/sec

Directory scan:           102 entries/sec

Seek/read test:            83 seek/reads per second

r/w speed: buf  1024 bytes, rd 33608 byte/sec, wr  31968 byte/sec

r/w speed: buf  8192 bytes, rd 62415 byte/sec, wr  62415 byte/sec

r/w speed: buf 32768 bytes, rd 65536 byte/sec, wr  62415 byte/sec

Section 1.4  Distribution Contents

The following files are included in the evaluation package:

* Manual.Doc         - This file.

* Schematic.part1    - IFF of first half of the schematic

for the hardware interface.

* Schematic.part2    - IFF of second half of the schematic

for the hardware interface.

* Layout.pic         - IFF of suggested Construction Layout.

* prototype.pic      - IFF of my prototype interface.

* ByNeryDemo.device  - the device driver.

* install.ByNeryDemo - program to test interface and

low-level format the drive.

Section 1.5  DISCLAIMER

*****************************************************************

*                            DISCLAIMER                         *

*                                                               *

*   NO REPRESENTATIONS OR WARRANTIES ARE MADE WITH RESPECT TO   *

*   THE ACCURACY, RELIABILITY, PERFORMANCE OR OPERATION OF THIS *

*   DOCUMENTATION OR OF THE ByNeryDemo INTERFACE ITSELF.        *

*                                                               *

*   DUE TO THE NATURE OF THIS PROJECT, NEITHER THE AUTHOR NOR   *

*   Products ByNery CAN BE HELD LIABLE FOR DAMAGES, REAL OR     *

*   IMPLIED, RESULTING FROM THE CONSTRUCTION, INSTALLATION OR   *

*   RELIABILITY OF THIS PROJECT.                                *

*                                                               *

*                                                               *

*****************************************************************

- 3 -

Chapter 2

CIRCUIT ANALYSIS

The Amiga 500, as  well  as all Amiga family members, utilize the

Motorola  680x0  family  of  microprocessors.    The   Hard  Disk

controller to  which this project interfaces was designed for the

INTEL 808x family of microprocessors.

The Disk Controller requires the following signals:

1. Data Lines D0-D7.

2. Address Lines A0-A19.

3. *IOR  - When low an I/O read operation is in progress.

4. *IOW  - When low an I/O write operation is in progress.

5. RESET - When high the disk controller is reset.

The Amiga expansion bus provides the following signals:

1. Data Lines D0-D7.  Note, only 8 bit access is used here.

2. Address Lines A1-A23.  Note, there is no A0.  This is due

to the fact that the Amiga is a true 16 bit machine.

3. R/*W - High, read. Low, write.

4. *LDS - When low, low memory byte is being accessed.

5. *RESET - When low, the Amiga 500 is being reset.

6. *AS - When low, a valid addres is present.

As you can  see,  the  required  lines  don't  exactly  match the

signals provided by the Amiga.  This project converts the Amiga's

signals to those expected by the controller.

The major difference lies in the memory mapped I/O  used  by  the

Amiga  versus  the  direct I/O porting used  by  the  controller.

Conversion of the Amiga  to  a  compatible  form, requires that a

memory space, in the Amiga, be dedicated to the controller.

By  utilizing the top eight address lines, A16  to  A23,  as  the

input  to  an  eight-bit comparator, it is possible to locate the

controller in any 32k memory boundary.

For this project I selected EF0000  as  the base address.  IC1, a

74LS688, in  the schematic provides this comparison.  The address

strobe, *AS, is used  to  indicate to the comparator that a valid

address  is  present,  thereby  enabling the comparator's output.

When  address  is  enabled  and  the  upper  eight address  lines

represent EF, a low is  output by the comparator.

The output  of  the  comparator  is  fed  to two, three input NOR

gates.  A   second input to the NOR gates is *LDS, the lower data

- 4 -

strobe.  The third  input,  of  each  NOR gate is either from the

R/*W line directly or inverted.

The  NOR  gate  with the direct connection to R/*W, when inverted

maps directly to the controller's *IOW signal.  The NOR gate with

the  inverted  connection   to R/*W, when  inverted  becomes  the

controller's *IOR signal.

The only other  signal  modification required is the inversion of

the Amiga's reset line.

To accommodate  the differences in the address lines, the Amiga's

addresses will be shifted down by one.  Therefore, Amiga  A1 will

be  connected to the controller's A0.   This  will  be  continued

until  the Amiga A15 to the controller's A14.    What  about  the

controller's other address lines?   Easy, they are just connected

to ground.

Lastly, the controller's *MEMR, *MEMW, *DACK3 lines, features not

used in this interface, are all  tied  to 5 volts (ie. held high)

to prevent them from interfering.

- 5 -

Chapter 3

CONSTRUCTION

My prototype board was  assembled  using  a combination of point-

to-point soldering  and  a  wire  pencil.    The wire pencil is a

modification to  the wire wrapping technique which allows the use

of standard, ie. not wire-wrap,   sockets  to be used.  The catch

is that soldering must be used to finalize all connections.   The

wire used has a special insulator that melts to allow  soldering.

Unfortunately,  it is easy to melt  other  wires  insulation,  as

opposed  to  just  the one you meant, thereby causing a potential

short.  This product is manufactured by VECTOR Corporation.

Interface Construction Parts:

Qty 1, 74LS688 (IC1) - eight bit comparator.

Qty 1, 74LS27  (IC2) - triple, three input NOR gates.

Qty 1, 74LS04  (IC3) - hex inverter.

Qty 1, 86 location on .1 inch center, edge board connector

Qty 1, 62 location on .1 inch center, edge board connector

Qty 1, perf board with solder pads on .1 inch centers.

Qty 1, 20 pin dip socket.

Qty 2, 14 pin dip socket.

Qty 3, 0.1 Microfarad Capacitors.

The two edge board connectors are available from:

Digi-Key Corporation

701 Brooks Ave. South

P.O. Box 677

Thief River Falls, MN 56701-0677

According to their Jan-Feb 1990 Catalog, No. 901, they are:

62 pin connector: part number C5-31, price $2.44 quantity 1

86 pin connector: part number C5-43, price $3.22 quantity 1

These are SOLDER-TAIL GOLD connectors.  Although you will require

wire  wrap  connectors,  for at least the 86 pin connector, these

can be made  by  utilizing Vector Corporations wire-wrap posts to

extend the pin length.

Wire  wrap  connectors  are available, but it will require you to

search  for  them.   I found them at two, local to me, electronic

retail stores but they cost about double for tin connectors.

- 6 -

Following IFF, Layout.pic:

-  glue the sockets into position making sure the pin 1 tabs

are as indicated (square pad on layout).

-  Install capacitors as indicated in Layout.pic.

-  Connect one lead of Capacitor C1 to pin 20 of Socket for

IC1.

-  Connect other lead of Capacitor C1 to pin 10 of Socket

for IC1.

-  Connect one lead of Capacitor C2 to pin 14 of Socket for

IC2.

-  Connect other lead of Capacitor C2 to pin 7 of Socket for

IC2.

-  Connect other lead of Capacitor C3 to pin 7 of Socket for

IC3.

-  Connect other lead of Capacitor C3 to pin 7 of Socket for

IC3.

-  Install the 86 pin edge card connector as shown.

-  Install the 62 pin edge card connector as shown.

Section 3.1  Wiring List

Connect the following wiring list:

FROM         SIGNAL              TO                   SIGNAL

=======       ======         ===================       ======

AMIGA 1       Ground            AMIGA 2                Ground

AMIGA 3

AMIGA 4

IC1, pin 7

IC1, pin 10

IC2, pin 7

IC2, pin 9

IC2, pin 10

IC2, pin 11

IC3, pin 1

IC3, pin 7

IC3, pin 11

Controller B1

Controller B10

Controller B31

Controller A11

Controller A12

Controller A13

Controller A14

Controller A15

Controller A16

- 7 -

FROM         SIGNAL              TO                   SIGNAL

=======       ======         ===================       ======

AMIGA 5       5 Volts           AMIGA 6                5 Volts

IC1, pin 20

IC1, pin 3

IC1, pin 5

IC1, pin 9

IC1, pin 12

IC1, pin 14

IC1, pin 16

IC1, pin 18

IC2, pin 14

IC3, pin 14

Controller B3

Controller B11

Controller B12

Controller B15

Controller B29

AMIGA 10      +12 Volts         Controller B9          +12 Volts

AMIGA 75        D0              Controller A9          D0

AMIGA 77        D1              Controller A8          D1

AMIGA 79        D2              Controller A7          D2

AMIGA 81        D3              Controller A6          D3

AMIGA 83        D4              Controller A5          D4

AMIGA 86        D5              Controller A4          D5

AMIGA 84        D6              Controller A3          D6

AMIGA 82        D7              Controller A2          D7

AMIGA 29        A1              Controller 31          A0

AMIGA 27        A2              Controller 30          A1

AMIGA 26        A3              Controller 29          A2

AMIGA 24        A4              Controller 28          A3

AMIGA 21        A5              Controller 27          A4

AMIGA 23        A6              Controller 26          A5

AMIGA 28        A7              Controller 25          A6

AMIGA 30        A8              Controller 24          A7

AMIGA 32        A9              Controller 23          A8

AMIGA 34        A10             Controller 22          A9

AMIGA 36        A11             Controller 21          A10

AMIGA 38        A12             Controller 20          A11

AMIGA 39        A13             Controller 19          A12

AMIGA 41        A14             Controller 18          A13

AMIGA 43        A15             Controller 17          A14

- 8 -

FROM         SIGNAL              TO                   SIGNAL

=======       ======         ===================       ======

AMIGA 74        *AS             IC1, pin 1

AMIGA 45        A16             IC1, pin 2

AMIGA 52        A18             IC1, pin 4

AMIGA 56        A20             IC1, pin 6

AMIGA 58        A21             IC1, pin 8

AMIGA 59        A23             IC1, pin 11

AMIGA 57        A22             IC1, pin 13

AMIGA 54        A19             IC1, pin 15

AMIGA 47        A17             IC1, pin 17

AMIGA 70        *LDS            IC2, pin 2

IC2, pin 3

AMIGA 68        R/*W            IC2, pin 4

IC3, pin 3

AMIGA 53        *Reset          IC3, pin 9

IC3, pin 12                     Controller B14          *IOR

IC3, pin 6                      Controller B13          *IOW

IC3, pin 8                      Controller B2           RESET

IC3, pin 4                      IC2, pin 13

IC1, pin 19                     IC2, pin 1

IC2, pin 5

IC2, pin 12                     IC3, pin 13

IC2, pin 6                      IC3, pin 5

It  took me about 3 hours to wire wrap (pencil) my prototype.  It

actually  took  about  8 hours to make the PC board,  but  it  is

neater.  Both worked the same, but the PC board version  did work

the first time, while the wire wrap took many hours of debugging.

- 9 -

Definition of the PC controller bus.  To Determine front to back,

hold  the disk controller in front of you  with  the  edge  board

connector pointed  down and the components  on  the  right  side.

In this position, the front  of  the controller is  the  part  of

the  controller  closest  to  you,  the  rear  is farthest away.

Rear of Controller

Ground ---   B1    A1  --- *I/O Channel Check (nu)

Reset ---   B2    A2  ---  Data bit 7

+5 Volts ---   B3    A3  ---  Data bit 6

IRQ2 (nu) ---   B4    A4  ---  Data bit 5

-5 Volts (nu) ---   B5    A5  ---  Data bit 4

DRQ2 (nu) ---   B6    A6  ---  Data bit 3

-12 Volts (nu) ---   B7    A7  ---  Data bit 2

(nu) ---   B8    A8  ---  Data bit 1

+12 Volts ---   B9    A9  ---  Data bit 0

Ground ---  B10    A10 ---  I/O Channel Ready (nu)

*Memory Write ---  B11    A11 ---  Address Enable    (nu)

*Memory Read ---  B12    A12 ---  Address bit 19

*I/O Write ---  B13    A13 ---  Address bit 18

*I/O Read ---  B14    A14 ---  Address bit 17

*DACK3 (nu) ---  B15    A15 ---  Address bit 16

DRQ3 (nu) ---  B16    A16 ---  Address bit 15

*DACK1 (nu) ---  B17    A17 ---  Address bit 14

DRQ1 (nu) ---  B18    A18 ---  Address bit 13

*DACK0 (nu) ---  B19    A19 ---  Address bit 12

CLOCK (nu) ---  B20    A20 ---  Address bit 11

IRQ7 (nu) ---  B21    A21 ---  Address bit 10

IRQ6 (nu) ---  B22    A22 ---  Address bit 9

IRQ5 (nu) ---  B23    A23 ---  Address bit 8

IRQ4 (nu) ---  B24    A24 ---  Address bit 7

IRQ3 (nu) ---  B25    A25 ---  Address bit 6

*DACK2 (nu) ---  B26    A26 ---  Address bit 5

Terminal Count (nu) ---  B27    A27 ---  Address bit 4

Addr Latch Enb (nu) ---  B28    A28 ---  Address bit 3

+5 Volts ---  B29    A29 ---  Address bit 2

Oscillator (nu) ---  B30    A30 ---  Address bit 1

Ground ---  B31    A31 ---  Address bit 0

FRONT OF CONTROLLER

Please note that the component side  of  the  controller  is  the

A-side of  the connector and the B side is the solder side.  (nu)

indicates this is not actively used by the controller.

- 10 -

Chapter 4

CHECKOUT

****************** THIS IS IMPORTANT ******************

******************* DON'T SKIP THIS *******************

With  an  ohmmeter,  verify  ALL  connections.    Pay  particular

attention to the ground, 5 volt and 12 volt connections.

With the ohmmeter verify  that  there  are NO connections between

the 5 Volt lines and ground, the 12 Volt lines and ground, the 12

Volt  line  and  all  controller  connections  (except   the  one

expected) and lastly the 12 Volt line and the 5 Volt line.

*****************************************************************

***   PLEASE MAKE SURE YOU PERFORM THE PREVIOUSLY OUTLINED    ***

***   CHECKOUT!!!!  I DIDN'T AND IT COST ME 3 DISK CONTROLLER ***

***   BOARDS!!!                                               ***

*****************************************************************

- 11 -

Chapter 5

CONTROLLER/DISK DRIVE SELECTION

At the current time, this interface has only been tested with:

WESTERN DIGITAL's 27X and XT-GEN2 series, PC-XT 8-bit bus

disk controllers.

The  27X  series  is an RLL controller while the XT-GEN2 is a MFM

controller.   What does this mean to you?  Well, the 27X, for $10

more, will  provide approximately 50% more disk capacity than the

XT-GEN2.

With the demonstration version of this project, both will provide

the same  capacity,  10  Megabytes.    Once  registered, however,

you'll  be  able   to   take  advantage  of  the  the  additional

capability.

Why then would you use the XT-GEN?  Supposedly, it should be used

with  a  certified RLL drive.  Currently  I'm  using  it  with  a

Micropolis  33  Meg  non-certified  RLL  drive and yet to have  a

reported error, and  it  gives  me  53 Meg of space. Friends have

reported similar  success.  But, if you believe the manufactures,

I'm risking a massive failure.  Since I backup regularly,  I take

the risk for the additional storage.

While  it   is  anticipated  that  other  PC-XT  8-bit  bus  disk

controllers  should  work,  I  have  as yet tested any and  don't

expect to.  I will, however, make available information regarding

other controllers if you all will keep me informed. If you happen

to try one,  it  may work.  If it does let me know and I'll share

it with others.

- 12 -

Chapter 6

INSTALLATION

Section 6.1  Hardware

Install the adapter card on  the Amiga's expansion port.  Install

the  hard  disk  controller,  making  sure  that  it is  properly

oriented.    Refer  to the layout.iff picture  to  determine  the

connector numbering and verify that it  matches your layout.

The hard disk controller requires  two ribbon cables, in the case

of  the    specified Western Digital 27X series controller, a 34-

conductor control cable should be connected to connector J1 and a

20-conductor data cable should be connected to J2.

Since there are different  hardware  configurations  for  the 27X

series,  if controller configuration jumpers are available,  they

should be set as follows:

1. BIOS ROM Disabled (sorry but the Amiga can't use a PC

BIOS).

2. Primary Addressing Enabled (Card address 320 through 323

hex).

3. Neither IRQ2 or IRQ5 are used (any selection acceptable).

4. Primary Controller (I don't think this matters and mainly

relates to the BIOS ROM which is disabled).

5. Firmware translation disabled (I don't think this matters

either as it probably relates to the ROM too).

Now that you're confused, I used mine as  it  came  from  the box

with no problems.  But if  you  have problems, you might try this

setup.

Now  attach  the  other  ends  of  the  ribbon  cables  to  their

appropriate  connectors on  the  disk drive.  Attach a disk power

supply to the disk drive.

As far as power is concerned, my standard Amiga power  supply was

able  to    power  my  500,  512k  expansion  module and the disk

controller.  The disk had to  be  powered separately.  I ended up

adapting a PC-clone power  supply  to allow me to power my entire

system, including the Amiga.  I even purchased a clone PC cabinet

to hold my external (self-built) 3.5 inch drive and my hard disk.

This project should be appearing in the Public Domain shortly, if

not yet.

- 13 -

Congratulations!  The entire hardware installation is complete.

Section 6.2  Testing and Low-Level Formatting

From the CLI, enter install.ByNeryDemo.  Hard Disks require to be

low-level formatted.  This is  not to be confused with AmigaDos's

format.  This  operation prepares the disk to be  compatible with

the  controller.    To  perform this operation select format from

the list.  The requested  parameter, Interleave, is the number of

sectors  that  must  be  skipped  between  consecutive  reads are

performed.  My testing indicates  an  interleave  of  10-12 (RLL)

yields the best performance.  At this time, request 12  and press

the carriage return.

At  this point the drive light should light.  It's time for a cup

of  coffee.  This  operation can take up to 10 minutes.  Once the

Drive light goes out, the operation is complete.

Now, from the menu of install.ByNeryDemo, try reading and writing

some sectors.  For  the  public  domain  version,  the acceptable

ranges are:

HEADS:     0 to 3

Cylinders: 0 to 305

Sectors:   1 to 16

After playing with it for a  while,  you are ready to install the

driver  add  a  mountlist entry.  Move on to the next chapter for

this information.

Section 6.3  Software Installation

1. Make a copy of your boot disk and put the original away!

2. Copy the file, pcif.device, in the Devs drawer of the new

boot disk.

3. CD to the new disk's Devs directory.

4. Using your favorite editor:

a) Edit the mountlist file

b) Go to the end of the file

- 14 -

c) Add the following (make sure that  when  entering  "dh0:

Device =" line, that Capitalization of ByNeryDemo.device

is exactly as shown.

/*

** pcif.device - hard disk definition.

*/

dh0:     Device = ByNeryDemo.device

Unit = 0

Flags = 0

Surfaces = 4

BlocksPerTrack = 17

Reserved = 2

Interleave = 0

LowCyl = 0 ; HighCyl = 305

Buffers = 30

BufMemType = 0

#

d) write the file and exit the editor.

5. After write completes, reboot your Amiga (Ctrl-Amiga-Amiga).

6. Open a cli after boot completes.

7. Type "mount dh0:"

8. Type "system/format drive dh0: name AnyName QUICK

You have now completed the installation of the disk.  Try copying

files to it  and looking at it with dir.  You can now modify your

S:Startup-Sequence file  to  automatically  mount  it  by  adding

"mount dh0:" in the startup file.

In  case your wondering, yes you can  modify  the  mountlist  and

utilize the drive with the FFS (Fast File System).  And  you will

notice a significant performance improvement.  Just don't forget,

after modifying the mountlist you must perform step 8 above again

(format it).  You can also play with the low-level format  to try

to  optimize  the  performance of the  disk,  using  any  of  the

available disk performance programs.

From this point on, your on your own.  You can  use  this  as any

disk  would be used.  If your interested  in  the  full  featured

driver/setup program, please see the next chapter and register.

If you wish to contact me, I am available on Compuserve, ID:

76662,3651

Please do not request ready made units, I currently do  not  plan

on manufacturing them, at this time.  Also please understand that

my main support will be to those registered users.

- 15 -

Chapter 7

REGISTRATION

Please PRINT clearly.

NAME:     _______________________________________________

STREET:   _______________________________________________

CITY:     ______________________________ STATE: _________

COUNTRY:  __________________

ZIP CODE: ______

Requested Information Questionnaire:

What Controller Did You Use? _____________________________

Did you have problems?       ____YES     ____NO

Would you purchase a PC board if available?

____YES     ____NO

Would a cost of $25 be excessive?

____YES     ____NO

Comments?_________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

Enclosed is $30.00 to register my usage of Product ByNery's

Amiga 500 - to - IBM style Hard Disk Interface

Please   send   my   registration  kit  and  add   me   to   your

support/notification list.

Send this form, and a check made payable to Products ByNery, to:

Products ByNery

14 Chambers Court

Mansfield, Ma  02048

- 16 -