cMIDI Function Library Description

==================================

The cMIDI function library is a set of over 50 C functions designed

for controlling MIDI equipment via an IBM PC equipped with a Roland

MPU-401 MIDI interface.  The cMIDI function library is versatile, yet

simple to use.  For example, the nine MIDI applications in this

demonstration were written using the cMIDI function library.  Each of

these applications required less than a page of source code.  We

invite you to examine this source code (included on this

demonstration diskette).

The cMIDI function can control up to 255 track pointers.  A track

pointer is analogous to a multitrack recorder's play/record head.

However, instead of just a volume control, each track pointer has a

set of controls that allow you to adjust note transposition, time

delay, MIDI channel, and note on and off velocities.  The cMIDI

function library can also read and write tracks of MIDI data in

several different file formats (including Standard MIDI Files).

The cMIDI function library supports the small, medium, compact, and

large memory models and can be executed within the Microsoft QuickC

environment.  The library's manual is over 100 pages long and

includes 17 sample applications.

Below is a short description of each of the 52 functions in the

cMIDI function library:

1)  "CmidiClose" removes the cMIDI interrupt service routine.

2)  "CmidiOpen" installs the cMIDI interrupt service routine.

3)  "ErrorDump" displays the run-time errors detected by the cMIDI

function library.

4)  "ErrorPop" gets the last run-time error logged by the cMIDI function

library.

5)  "ErrorPush" logs a run-time error number.

6)  "ErrorSetFatal" defines which run-time errors halt execution of the

application.

7)  "ErrorSetNonfatal" defines which run-time errors do not halt execution

of the application.

8)  "MpuRead" reads a byte of data (e.g., tempo) maintained by the

MPU-401.

9)  "MpuReset" resets the MPU-401.

10) "MpuSet" sets a byte of data (e.g., acceptable MIDI channels)

maintained by the MPU-401.

11) "MpuSwitch" toggles a switch (e.g., metronome on/off) maintained by

the MPU-401.

12) "MpuUartModeRx" receives data from the MPU-401 while in UART mode.

13) "MpuUartModeTx" transmits data to the MPU-401 while in UART mode.

14) "QueueMidiChan" enables/disables the receiving of channel MIDI

messages.

15) "QueueMidiSysCom" enables/disables the receiving of system common MIDI

messages.

16) "QueueMidiSysEx" enables/disables the receiving of system exclusive

MIDI messages.

17) "RxMidiChan" retrieves a channel MIDI message from the receive

buffer.

18) "RxMidiSysCom" retrieves a system common MIDI message from the receive

buffer.

19) "RxMidiSysEx" retrieves a system exclusive MIDI message from the

receive buffer.

20) "TxMidiChan" transmits a channel MIDI message.

21) "TxMidiSysCom" transmits a system common MIDI message.

22) "TxMidiSysEx" transmits a system exclusive MIDI message.

23) "TrackLength" determines the size of a track.

24) "TrackPlay" starts track pointers playing.

25) "TrackPlayAndRecord" starts track pointers playing and recording.

26) "TrackReadCmidiData" reads a track from a cMIDI data file.

27) "TrackReadCmidiText" reads a track from a cMIDI text file.

28) "TrackReadMidiFile" reads a track from a Standard MIDI file.

29) "TrackReadSeqPlus" reads a track from a Sequencer Plus MKIII file.

30) "TrackRecord" starts track pointer recording.

31) "TrackStop" stops track pointers playing or recording.

32) "TrackTempo" specifies the record and play tempo.

33) "TrackWriteCmidiData" writes a track to a cMIDI data file.

34) "TrackWriteCmidiText" writes a track to a cMIDI text file.

35) "TrackWriteMidiFile" writes a track to a Standard MIDI file.

36) "TrackWriteSeqPlus" writes a track to a Sequencer Plus MKIII file.

37) "TrackPtrChannel" specifies the MIDI channel on which a track

pointer's data is played.

38) "TrackPtrCopy" makes a copy of a track pointer.

39) "TrackPtrDec" decrements a track pointer to the previous message in

its track.

40) "TrackPtrDelay" delays the data played by a track pointer.

41) "TrackPtrInc" increments a track pointer to the next message in its

track.

42) "TrackPtrMute" mutes the notes played by a track pointer.

43) "TrackPtrPlayListDelete" removes a track pointer from the list of

track pointers to be played.

44) "TrackPtrPlayListInsert" adds a track pointer to the list of track

pointers to be played.

45) "TrackPtrRead" reads the message from the location pointed to by a

track pointer.

46) "TrackPtrRegister" associates a handle with a track pointer.

47) "TrackPtrReset" moves a track pointer to the beginning of its track.

48) "TrackPtrTranspose" transposes the pitch of note messages played by a

track pointer.

49) "TrackPtrUnmute" unmutes the notes played by a track pointer.

50) "TrackPtrVelocityOff" adjusts the velocity of note off messages played

by a track pointer.

51) "TrackPtrVelocityOn" adjusts the velocity of note on messages played

by a track pointer.

52) "TrackPtrWrite" writes a message to the location pointed to by a track

pointer.

Hardware/Software Requirements

==============================

1) an IBM PC/XT/AT (or compatible)

2) a Roland MPU-401 (or compatible) MIDI interface

3) Dos 2.1 (or higher)

4) Microsoft's C or QuickC, or Borland's TurboC compiler

Authors

=======

Greg Sepesi (MS Electrical Engineering, Syracuse University)

Rex Lau (MS Computer Science, Michigan State University)

cMIDI

P.O. Box 4903

East Lansing, MI  48823

(517) 337-2569

Price (until 01 Nov 88)

=======================

cMIDI Function Library                  $79 (30 day satisfaction guarantee)

cMIDI Function Library Source Code	$79