/*

Turbo C Direct Access Interface

by	Joel Lucsy

Vroom-Diggy-Diggy Software, (c) 1991

Defined Macros:

SBOK		Return value indicating success from reset_dsp.

Functions Defined For Direct Access:

int reset_dsp(void)

-Resets the DSP. Do this before calling any direct access routines.

-Returns SBOK if able to reset.

int read_data(void)

-Returns data from D-A converter.

void write_data(void)

-Writes data to the A-D converter.

-Speaker_on must be called first in order to hear the sound.

void speaker_on(void)

-Turns on the speaker.

void speaker_off(void)

-Turns off the speaker.

void asmdelay(int delay)

-A delay for timing output, more usefull than Turbo C's delay.

** Sound Blaster is a copyright of Creative Labs, Inc. **

*/