C-Module for Sound Card Detection

Function documentation of Creative Sound Library Module

1. Introduction

Author          :  T S Lee

Version         :  1.00

Last Updated    :  22 Sep 1989

This version uses a more reliable detection scheme. Please

replace all older verions with it.

The  library  functions provided is for Microsoft  C  version

5.00 and above. All the memory models, SMALL, COMPACT, MEDIUM

and  LARGE  are  available. These functions  are  grouped  in

library files named in following conversion:

DETECTx.LIB

where  "x"  is the first character of the  respective  memory

model.

The default segment and group name defined in Microsoft C for

each memory model is used to ensure compatibility. Any global

variable  memtioned in following section must be declared  in

the default data segment '_DATA'.

The  FM feature of the Sound Blaster Card may be an  optional

item in some OEM versions. So we suggest that if you want  to

play  FM  music on the Sound Blaster Card, separate  FM  chip

detection is required.

The Sound Blaster Card is compatible with the C/MS card (Game

Blaster), but it uses a different card detection scheme. Hence,

to maintain upward compatibility in your software, we strongly

suggest that you call this library routine for card detection.

2. Function Documentation

Function    :  unint _ct_card_here()

Description :  Check   for  the  availibity  of   the   various

music/sound  features  provided by  Creative.

The  check is based on the I/O address  stored  in

the global variable ct_io_addx which is defined in

the application program.

Entry       :  none

Return      :  Music/Sound feature available

High byte is alway zero now

Low byte right most 3 bits indicate as follow

bit 0 : on for C/MS music available

bit 1 : on for FM music available

bit 2 : on for Creative voice (DSP) available

The type of Creative Music/Sound Card available on

the  system  can  be determined  with  the  return

value.

Game  Blaster Card returns 1.

Sound Blaster Card without FM music  returns 5 ,

with the FM music, returns 7.

Note        -  The I/O address used for the music/sound card must

be   stored   in  ct_io_addx  before   the   call.

(Application can set this variable to the  factory

defualt value, 220 Hex). There are 6 sets of I/O

addresses : 210H, 220H, 230H, 240H, 250H and 260H.

-  A  zero return value may due to wrong I/O  address

used.