Notes for RTG users

===================

Jurassic  Pack is optimized for a 640x256 display and looks

slightly  strange  in other modes.  It has been tested on a

Cybervision 64/3D under Picasso96.  If you want to create a

640x256 mode for your Picasso96 based display, it is really

very easy.

It  should also be possible to create such a mode for CGFX,

but  as  I  don't use it, the details are not included here

(anyone?)

Although Jurassic Pack will work happily with either AGA or

RTG,  on  slower CPUs it is a lot faster to use AGA.  On my

030/25, page rendering is noticeably much faster under AGA.

An 040 or 060 should in theory be able to make up for this,

as the speed loss is the CPU doing the work of the blitter.

Note  that  the  taller  the screen is, the slower Jurassic

Pack

will be.  1280x1024 is painfully slow on the 030/25, but it

all looks too small anyway :)

Disclaimer

==========

You can break your  monitor by  playing with Picasso96Mode.

You follow the actions described in this document  ENTIRELY

AT YOUR  OWN RISK!  This worked  100% for  the author,  and

should work fine with all MODERN SVGA monitors.  The author

takes  no responsibility  for any  consequences of  the use

or misuse of this information.

By  following these  instructions,  you are agreeing  to be

bound by the above statement.

How to go about creating a 640x256 mode

=======================================

1. Load up Picasso96Mode

2. Make sure the correct GFX card is highlighted

3. Click on the "New Item" icon,  and drag&drop it over the

resolutions (it should say "New Resolution")

4. Go to the width and height boxes and change it to  640 x

256. Give it a new name if you like.

5. Click on the  "New Item" icon, and drag&drop it over the

modes  (it should say "New ModeInfo")

6. Click on  "Edit" and use the  cursors to centralize  the

new mode (use your monitor controls also)

7. If the mode is  flickery, try upping the clock speed. If

the screen goes black and/or your monitor emits  a  high

pitched  whine, PRESS ESC IMMEDIATELY!

8. Click Save, then reboot

9. Configure Jurassic Pack to use your new mode

Easy huh?

Any questions: mrtickle@amiga4k.ndo.co.uk