PDATA DOCUMENTATION

~~~~~~~~~~~~~~~~~~~

Release version 3.51 Final, January 1995

(Yes, the FINAL release version of PData EVER!)

Copyright � 1993-1995 Deadbeat/The Sharks

(C64 end, optimized cable schematics, documentation)

Copyright � 1993 Hude

(Amiga end)

PLEASE NOTICE THAT THE AMIGA EXECUTABLE FILE IS NOW CALLED "XDATA"!

(100% compatible with PData, read "FUTURE" for more information...)

1. Introduction and features

PData is an utility for transfering both binary and text files between

the C64/C128 and the Amiga. It's about 25 times faster than a null modem

cable (The maximum BPS rate on a standard C64 is 2400), and it even

leaves your Amiga's serial port free for other applications.

You can transfer up to 255 files both directions in multitransfer mode.

PData is easy to use and it won't let you down like most others. ;-)

2. Distribution and disclaimer

This archive may be freely distributed as long as all files are included

intact. PD libraries should only ask a nominal fee for copying it. PData

may not be used for commercial purposes without a written permission from

the author(s). When applying for your permission, you should include a

reasonable contribution.

The author(s) can't be held liable for any damage directly or indirectly

caused by the use or misuse of this manual and/or program it describes.

The latest version of PData can always be found from The Sharks' boards.

Internet users will most likely find the latest version from nic.funet.fi

at C64 utilities directory (I think it was at /pub/cbm/c64/utilities/).

If you're an IRC addict, you might find it convenient to /DCC CHAT with

DemoBot on #C-64... Besides the latest version of PData (XData from now

on, read "FUTURE" for more information), you can leech loads of classic

C64 demos... (Ain't that nice?)

3. Requirements

The C64 end of PData will work in every C64/C128. The Amiga end of PData

will work in every model, but Kickstart v37.175 (v2.04) or above is

required for utilizing every function. A special cable between the C64

and the Amiga is required. It can be built by following the schematic

in the next chapter. The following things are required for the cable:

- One card edge connector (C64 user port)

- One male D-25 connector (Amiga parallel port)

- Suitable lenght of flat cable (Width 12 or more)

- A soldering iron and some solder

Additionally, you'll need the following files for your Amiga...

(I guess this shouldn't be too big problem as these are system files!)

- Devs/Parallel.device

- L/Port-handler

- Libs/Asl.library

- Libs/Locale.library

4. PData cable schematics

C64 user port              Amiga parallel port

Pin      Description       Description      Pin

B        FLAG*             STROBE           1

C        PB0               DATA 0           2

D        PB1               DATA 1           3

E        PB2               DATA 2           4

F        PB3               DATA 3           5

H        PB4               DATA 4           6

J        PB5               DATA 5           7

K        PB6               DATA 6           8

L        PB7               DATA 7           9

M        PA2               BUSY             11

8        PC*               ACK*             10

N        GND               GND              17

5. Using PData C64

Using PData C64 is fairly easy. You'll first be prompted for your work

drive. Any device between 8 and 11 will do. Then you can select whether to

handle your files as binary or text. If you select binary, the files will

be transfered without any alterations. Selecting text mode will cause all

files be translated from PETSCII to ASCII and vice versa. The next step is

to decide whether to send or receive files. At this point, you can press

"," to access SEQ file handler, using which you can print out or view

sequential files. Once you've made your decision, you can press F1 to

toggle between normal/zip mode. If zip mode is on, PData will add those

1! 2! 3! and 4! appearing in front of each zipped file automatically and

you only need to worry about the fourteen last chars. If you start

receiving in single mode, you'll be prompted whether to process files as

PRG or SEQ. If a file exists, you'll be given a chance to regret. You can

use "$" (Default) to see directory, "@" for DOS commands and "?" to see

free blocks. Pressing "=" at all times (Except in multi transfer menu)

will take you one step "backwards" in the menu. In zip mode you can

decide which files to overwrite individually. Pressing F3 will take you

to multi transfer menu. Press F1 to send files, F3 to receive files, F5

for directory and F7 for main menu. After pressing F1, you can use Y/N to

select files. Pressing return will quit or start transfer, depending if

you selected any files or not. After pressing F3, you can enter target

filenames. Leave one blank line to start transfer. Multi transfer can

handle 255 files both directions. It handles only binary PRG files and

never overwrites it's target files. At ALL times, pressing restore will

clear the channel, initialize vectors and restart program. If you're

in doubt of something, just try it. There's no way the program could crash.

6. Using XData Amiga

xdata <<s/r/?> <path><filename>>

(s = send / r = receive / ? = help)

Example : xdata r dh1:dl/c64_rules.txt

Pressing CTRL+C will HOPEFULLY abort transfer. Use it in combination with

RESTORE to gain maximum effect. If you have a multifacecard (MFC) in your

Amiga, remember you must use the XData.CFG in S: directory to re-route PAR:

into PIT:. Edit the file so that it reads PIT:<cr>. If you're not a hard

drive user, I suggest you receive the files into RAM: if you don't have

lots of spare time to waste (See missing features to see why).

7. General tips

Configuring your regular directory tool for PData makes your life much

easier. Once you've made the first transfer after turning your computers

on, you can ignore the C64's on-screen prompts about setting the other end

in send/receive mode. This is because once the port-handler has been

activated, PData can't "lose" anything. When transfering the PDATA.C64 to

the C64, you MUST use the filename "PDATA 3.5-SHARKS".

To transfer the file, try a null modem cable. Schematics are supplied below.

You'll need a RS-232 adapter for the C64 and a suitable terminal program

for both ends. If, however, you should experience problems in doing so,

e-mail the address kemu@zombie.oulu.fi and beg for help! :)

(It's Mysdee/Canus*Patrol's e-mail address, not mine, so... He just

volunteered to help all the people in need... ;)

(You could of course e-mail my very self, but I rarely read my e-mail!)

8. Null modem cable schematics

Pin     Description       Description       Pin

2       TXD               RXD               3

3       RXD               TXD               2

4       RTS               DCD               8

6       DSR               DTR               20

7       GND               GND               7

8       DCD               RTS               4

20      DTR               DSR               6

9. Bugs and missing features

C64 end:   - None known, although there has been some reports.

(Mostly with multifiletransfers...)

Amiga end: - Sometimes refuses to work if a filename ends with "+".

- Refuses to work if a filename contains any spaces.

- Receive buffer missing.

(... the list is endless, you wouldn't have the time!)

PData doesn't send a header containing a filename. This is NOT a bug.

Until a PC version of PData appears, it will stay like this. I didn't want

a filename header to be installed in the first place, because we need to

determine a common format first. We'll probably end up in the old 8+3

filename format because of the great (Hmmm...?) MS-DOS. PData can be used

to transfer data between two C64's or two Amiga's with minor cable

modifications. Hardware wizards may figure this one out on their own,

full cable schematics will be released later on. If we manage with the

PC end of PData, the holy triangle is complete and PData may become the

global binary data transfer format... :-)

10. Contact address

Alias:           Deadbeat/The Sharks

Snailmail:       Timo R�nkk�

Linjakuja 3

FIN-74120 Iisalmi

Finland

E-mail:          deadbeat@freenet.hut.fi

If you got an access to Internet Relay Chat (IRC), you might occasionally

spot me on the channels #C-64, #Amiga and #Amigascne.

11. Future

This special edition of PData is meant to be on the Assembly '94 CD-ROM, so

I've finally decided to share my views about the future with the public.

PDATA IS DEAD. LONG LIVE XDATA. After careful consideration and lots of

time, I've finally decided to make some major changes. The changes are so

major that I've decided to change the name aswell. Expect the initial

release of XData soon... In fact, I've had the C64 end finished for months

now, but finding a qualified Amiga coder has been the most difficult thing.

Hude kinda like lost interest towards this project, and he even managed to

lose the source codes somewhere. Anyways, there'll be lots of new features.

The entire C64 end has been reprogrammed from scratch. Finally there should

be a PC version aswell, so no need to weep. There's been some radical ideas

for future versions - like a TwinExpress with GUI, but much better and more

advanced. Data transfer rates will be *MUCH* higher, etc... Wait & see...

(Even ideas beyond your wildest dreams EVENTUALLY will be fulfilled, such

as support for JiffyDOS (All thanks to Harlekin/FairLight for lending me

his neat PIA cartridge) and possibly a software-based floppyspeeder!)

12. Boards

- MASTER & SERVANT

+49-(0)y�U-kN�W! / +49-(0)p�i-vA73!

THE SHARKS World Headquarters (C64)

QUARTEX German Headquarters (Amiga, SNES)

REBELS German Headquarters (PC)

- DEEP THOUGHT

+358-(9)77-n3wN�M�E�$s��n / +358-(9)77-wAiTfخTh3m

THE SHARKS European Headquarters (C64)

DIGITAL European Headquarters (Amiga)

>>> OFFLINE UNTIL FURTHER NOTICE <<<