MIDITALK

Copyright 1987 Rich Hickey

May be distributed freely but NOT sold, provided this

copyright notice remains intact.  All other rights reserved.

Miditalk is an interactive utility that let's you 'converse'

with your MIDI equipment from your Atari ST.  If you're just getting

into MIDI it is a great way to see what's really travelling over the

MIDI lines. If you are a programmer working in a compiled language

like C or Pascal, you know how frustrating it can be to re-compile

and link every time you try to get a new MIDI message to work.

Miditalk displays all incoming MIDI data on screen.  It also

enables you to type in data and have it sent over the MIDI port.

Miditalk also let's you create your own MIDI-macros and save and

load data dumps.

MIDI Basics

What actually goes on over MIDI ?  Well, the first concept

to grasp is that no 'sound' travels via MIDI.  As with most computer

interfaces, numeric data is the stuff of MIDI.  Musical instrument

manufacturers got together and decided on a set of codes and a

protocol for their transmission.  The keeper of the code is the

IMA (International MIDI Association), and for $35 they will send

you the detailed MIDI Specification.  If you plan on doing any

serious MIDI work I recommend it.  If you're just tinkering, or

you can't wait for the spec to arrive - have no fear.  Miditalk

does most of the work for you.

(Note: All values in this program and article will be notated

in hexadecimal unless otherwise indicated.  If you are unfamiliar

with hex consult any introductory level computer text for an

explanation.)

Let's look at a typical MIDI message.  If you press and

release middle C on your keyboard it will transmit something like

this:

90 3c 40    80 3c 00

How do we decipher this ?  The MIDI spec tells us that all

bytes in the range of 80 to ff (all of which have their leftmost

bit set to 1) are 'Status' bytes.  Status bytes can tell us anything

from 'A note was turned on' to 'The mod wheel was moved'.  These

status bytes are usually followed by one or more 'Data' bytes, which

tell us what note was played and how hard, or how far the mod wheel

was spun.  Data bytes range from 00 to 7f. (Never greater).

In the message above, 90 and 80 are status bytes.  90 is the

code for 'Note On', 80 for 'Note Off'.  Both commands are followed

by two data bytes; the note number and the velocity (how hard the

note was struck).  When you try this with Miditalk you'll see something

like this:

NON01 3c 40  NOF01 3c 00

You might get this:

NON01 3c 40  NON01 3c 00

But you only turned one note on ?!  Some synths turn off a note

with a 'Note On' command with a velocity value of 00.  The beauty of

Miditalk is being able to see how different synths implement the

'standard'.  Miditalk checks for incoming status bytes, translates

them into an abbreviation and displays them in reverse text.  Some

status bytes (like 'Note On') have the MIDI channel number on which

they were sent incorporated in them. MIDI has 16 channels so that

many synths can communicate over the same line.  Each synth responds

only to messages sent on it's channel.  The channel #, in decimal,

will be displayed in reverse following the abbreviation as in the

above case.  NON01 represents one byte, not two.  All data bytes

are displayed in normal text.  Remember, all values are hex.

Using Miditalk

When you first boot up Miditalk (medium or high rez recommended)

you will be prompted for the capture buffer size.  This buffer will

be used to capture data dumps etc.  10K should be large enough for

most synth dumps.

Next you will be asked to specify a library to load.  DEFAULT.LIB

contains macros for all of the status bytes.  If you hit return

without an entry the load will be cancelled and you will start with

Miditalk's small, built-in library.

Finally, you will be asked if you want to 'channelize' your data.

If your synth is set to a MIDI channel other than one, you will want

to send data on that channel.  Indicate yes and specify a channel.

If you choose not to channelize, your data will be sent on channel 1.

O.K. - play some notes, move the pitch bend, change patches

and watch the show !

The program is Miditalk, not Midilisten, and the ability to

transmit is what it's all about.  To go into the Talk/Command mode

just hit the spacebar.  You can now type in your command/message.

The parser expects the following format:

Status and data bytes are two digit hex values using lower

case letters.  (b0 21 7a etc.)

Each byte is separated by a space.

Macros are all Upper case letters.  Macros are separated from

one other by a space.

Macros and hex values can be combined on a Talk line.  for

example, to turn on middle C:

Go into Talk/Command mode (space bar)

Type       NON 3c 50      Hit return.

NOF 3c 00  will turn it off.  After each command/message you are

returned to the monitor mode to see what response, if any , there

was from the synth.  Note that NON and NOF are not followed by

the MIDI channel.  This is automatically handled by the channelization

feature.  You are probably wondering what macros are available

and what they represent.  Go into the Talk/Command mode and type:

SHOWLIB   (return)

This will display the contents of the currently loaded library.

SHOWLIB is a command.  Commands are built into the program,

not the library.  They are always available.  Like macros, they are

all caps and must be separated by a space.  Most do not cause anything

to be sent over the MIDI line.  The available commands are:

CHAN    - to change MIDI channels or channelization status.

SHOWLIB - displays library.

SAVELIB - saves library to disk

LOADLIB - loads library from disk

SENSING - toggles active sensing filter

BUFON   - turns on capture buffer

BUFOF   - turns off capture buffer

BUFRST  - resets capture buffer for new data

SAVEBUF - saves capture buffer to disk

LOADBUF - loads capture buffer from disk

TRANSMIT  sends capture buffer data over MIDI port

RPT     - repeats previous Talk/Command line

CLR     - clears MIDI input backlog

MACRO   - create or redefine a macro

Note that commands are executed as they are encountered by

the parser while outgoing data is not sent until the entire line

is parsed.  Therefore they will be executed before any data is

sent, even if they are in the middle of the line.

Macros

As you can see from looking at the library, macros can stand

for one byte or a whole string.  Macros can also contain other

macros.  By 'nesting' macros you can create very powerful, easy to

use tools.  To create a macro use the MACRO command.  You'll be

prompted for a name.  If you enter a name that already exists you

will have an opportunity to redefine it.  Next, simply enter the

macro in the same format as any Talk line.  Voila! Your macro is

now in the library (try SHOWLIB).  Remember to save the library if

you've added something worthwhile to it.  You can create custom

libraries for each synth if you like.  Each library can contain up

to 100 macros.  Theoretically, macros may contain commands, but

any commands will be carried out while you are defining the macro.

This is due to the macro validation process.  If you can deal with

this, fine, otherwise keep commands out of macros.

The Capture Buffer

At some point in your experimentation you will probably meet

up with 'System Exclusive'.  Most synths and drum machines have

some way of 'dumping' their patch data or sequencer data via MIDI.

Unfortunately, there is no standard way to do this.The MIDI spec

provides the System Exclusive status byte, after which each

manufacturer can utilize their own system for dumps or editing or

anything not covered by the spec.  The exclusive message is

terminated by the End Of eXclusive status byte.  Each synth will

have a different system exclusive implementation.  If the system

exclusive info is not in your owners manual request it from the

manufacturer.  Note that this is different information than what

is included in the 'MIDI implementation chart' included with

virtually every piece of MIDI gear.

Many synths provide a straight dump, no 'handshaking' or

checksum is required.  If this is the case you may want to direct

the dump into the capture buffer.

BUFRST CLR BUFON  (your dump request codes here)

should do the trick.  Turn the buffer off after you receive the

dump (BUFOF).  You can now save it to disk, with Miditalk acting as

a backup/librarian utility.  The save function only saves as much

data as was recieved, not the entire size of the buffer.  To send

the data back to the synth simply load it and TRANSMIT it.  Not

all synths will accept the data in the same format as they

transmit it, so you'll have to experiment.  The Ensonic ESQ-1

sends and receives a single patch dump just fine.  The dump

request is in the library, and my moog-style patch (ESQMOOG2.BUF)

is on the disk.

Whether you are a MIDI novice or expert, Miditalk should

prove a valuable tool in the often obscure but never

insurmountable MIDI universe.  P.S.  let's see your Miditalk

libraries in the Public Domain!

Table 1               MIDI Status Bytes

Status              Macro     # of data bytes expected   Channel Msge

------              -----     ------------------------   ------------

Note On             NON       2 - Note # & Velocity         Y

Note Off            NOF       2 -  "         "              Y

Poly After Touch    PLYAFT    2 - Note # & Pressure         Y

Control Change      CNT       2 - Controller # & Value      Y

Program Change      PRG       1 - Program #                 Y

Channel Aft.Touch   CHAFT     1 - Pressure                  Y

Pitch Bend          BEND      2 - Bend LSB & MSB            Y

System Exclusive    SYSX      Variable                      N

MIDI Time Code      MTC       1                             N

Song Position Ptr.  SPP       2                             N

Song Select         SONG      1 - Song #                    N

Tune Request        TUNE      None                          N

End of Exclusive    EOX       None                          N

MIDI Clock          CLCK      None                          N

Start               STRT      None                          N

Continue            CONT      None                          N

Stop                STOP      None                          N

Active Sensing      SENS      None                          N

System Reset        RESET     None                          N

********************* Important Notice ! **************************

I have uploaded this program and ask no money for its use.

It is the result of many hours of labor on my part. I make only

one request :

Please send in your name and address for my mailing list!

I am currently compiling a database of ST/MIDI users. In addition

to your name and address you can list your current MIDI equipment

and software.  Also list any software or utilities you would like

to see developed.  Comments on MIDITALK are welcome.  This

information will be used by myself and my company, SAVANT AUDIO

only.  As a result you will receive information on the MIDI

software I develop.

You can contact me (Rich Hickey) via MIDI-NET

2 nodes of which are:

ENIAC BBS (212) 751-2347

MIDWEST MIDI BBS (405) 733-3102

or on the ST/AMIGA MIDI BBS (619) 452-7535

or on DATELINE BBS (718) 648-0947

or drop me a line

Rich Hickey

2140 Bellmore Ave.

Bellmore, NY 11710

USA

or give me a call(voice) (516) 826-6336

This list is important to me, so if the software is useful to you

- respond!

Have fun..........Rich