CHRFLP Font Flipper

Bill Aycock [76703,4061]

CHRFLP is an ML subroutine for flipping

between several character sets (fonts)

at regular intervals. You can use as

many fonts as you want, as long as you

have enough room -- each font takes

up 1024 bytes (that's four pages).

The fonts are flipped regularly by a

short interrupt routine called by one

of the software timers -- you control

the speed. Your BASIC program will

continue to run while the character

sets change.

The sample program is pretty basic,

but it shows what you can do. Before

the routine can be used, it must be

placed in the infamous Page Six. (Note

that the assembler code is included so

you can move the routine elsewhere if

desired.)

You also need to set up your alternate

fonts. The sample program reserves

enough space under RAMTOP for four

fonts, then loads them from disk. Note:

the font loader here is S-L-O-W! but

it's easy to understand (and it was

really quick to write! :-)

Remember when you're setting up your

fonts that they MUST START ON AN EVEN

PAGE BOUNDARY! The standard ROM font

starts on page 224 (address $E000).

To make things easier to understand, I

set up two variables as constants.

SETUP has the value 1545, and is the

address to call the setup routine.

CHRFLP has the value 1577; this is the

entry point for the main routine.

Before you start flipping fonts, do a

USR(SETUP). After that, the routine is

controlled with USR calls to CHRFLP.

The format for the USR call is:

X=USR(CHRFLP,sp,set1,set2[,set3...])

where:

CHRFLP is the address of the ML

control routine.

sp is the speed to flip fonts - this

can be from 1 to 255, with 1

being the fastest.

set1 is the HIGH BYTE (MSB) of the

address of your first font.

set2 is the HIGH BYTE (MSB) of the

address of your second font.

set3 is for the third font...

You can use as many fonts as you like,

within reason. Just add the high bytes

of their addresses to the list. If you

want to include the normal ROM font,

just include the number 224 in the

list.

When you've finished, just call the

routine without any parameters at all:

X=USR(CHRFLP)

and the flipping will stop. After you

do this, you can switch to whichever

font you want by POKEing its MSB into

the character set base register:

POKE 756,224

will restore the standard font, but

you can replace the 224 with the MSB

of another font if you wish.

That's the basics. Study the sample

program, try it out, and if you have

any questions be sure to leave me a

message!

Bill Aycock

################################################################################################################