DOS WIZARD

Copyright 1983 by Gary C. Crider

INTRODUCTION

DOS WIZARD was written to

accomplish several tasks that no

other disk utility seemed to address.

Some of these are listed below.  Some

knowledge of DOS file structures is

assumed. There are many good

tutorials on DOS available (see

BIBLIOGRAPHY).  IT IS IMPORTANT THAT

YOU UNDERSTAND WHAT YOU ARE DOING

WHEN YOU USE DOS WIZARD.  It will be

one of your handiest utilities, but

also one of the most potentially

dangerous if you don't understand it.

PLEASE READ THIS DOCUMENTATION

THOROUGHLY!

1. RECOVERING THE SECTORS THAT

JUST SEEM TO VANISH FROM DOS DISKS.

When a directory listing is produced,

the sum of all file sectors plus the

remaining free sectors should total

707.  You have probably noticed that

this is not always true.  The primary

cause for this is pressing SYSTEM

RESET while a file is open for

output.  This leaves the directory

entry marked open and the file and

its associated sectors are forever

lost to DOS.

Prior to DOS WIZ the only way

to reclaim the sectors was to copy

all good files to another disk,

reformat the bad disk and selectively

copy each good file back to the

reformatted diskette.

DOS WIZ will read all files

while noting the sectors used by the

files. Any sector not referenced by a

file entry will be freed into the

available sectors pool and all open

file directory entries will be

released for reuse.

2. You probably have received

the notorious ERROR 164 or ERROR 144

at one time or another when trying to

read a disk file. Sometimes you can't

even delete a file because DOS wants

a valid file chain before he will

delete.

All sectors used by a DOS file

are chained together, that is, a

sector of data has a pointer to the

next sector belonging to that file.

Also on each sector is a reference to

the file which owns it. ERROR 164

indicates that a sector was pointed

to by a prior sector, but the two

sectors do not reference the same

file.  This means that somehow

another file used a sector belonging

to the file in question.  ERROR 144

generally indicates that a sector has

for some reason become unreadable.

This can also be the indicated by

some other error codes.

In either case, the integrity

of the file chain is lost and DOS

will not be able to read the file.

Usually there is nothing that can be

done because a binary load file or a

tokenized BASIC file that is not

complete will not be usable in any

fashion.  However, recovering a

portion of a data file can be very

useful at times. It can prevent a lot

of re-entry of data.  DOS WIZ allows

recovery of any file up to the point

where the chain is broken.  This then

becomes a valid DOS file and can once

again be accessed.

3.  A bad sector on a disk is

often a big headache even when your

file is backed up.  It means that all

good files must be copied and the

disk reformatted and the good files

copied back. Only then can you

restore your file from the backup.

DOS WIZ allows another approach

that is very often successful.  You

can analyze the disk to get the

location of the bad sector and it's

associated file name.  You can then

write zeros to that sector and the

odds are very good that you will then

be able to read it again. If you

can't you haven't lost anything since

the sector was unreadable.  You may

then recover the file up to the point

of the zero sector and then delete

the file. Now you can recover the

sectors used by the file and then

rewrite your backup file to the disk.

It sounds complicated, but is really

quite simple and much easier than the

alternative discussed earlier.

4. Occasionally a file gets

deleted by mistake.  If the sectors

used by that file have not been

overwritten, DOS WIZ will allow easy

recovery of the file.

5. Sometimes you end up with two

or more files of the same name.  If

you delete or rename one with DOS,

all files with that name are deleted

or renamed. DOS WIZ lets you delete

or rename by file number.  There are

never duplicate file numbers.

6. DOS WIZARD provides a

translation of the sector bit map

from the VTOC that is easily

readable.  It allows you to "hide"

sectors from use by DOS by marking

them as allocated.  You can also free

allocated sectors.

7. DOS WIZARD's directory

listing lists ALL entries including

deleted and open files.  It allows

you to perform many DOS functions on

those files by simply referencing the

file number.  This does however

eliminate the use of wild cards.  The

directory listing also tells you at a

glance whether any sectors are

unaccounted for.

8. DOS WIZARD will allow you to

flag sectors as allocated and

effectively hide them from DOS so

that they will be reserved for your

use.

9. DOS WIZARD can be used to

clear unused sectors of data. This

will speed up the disk duplication

process when FASTDUP (Copyright 1982

by Gary C. Crider) is used.

10. DOS WIZARD will display

sectors in hex and ASCII on the same

screen. It also gives much valuable

information about that sector such as

the file to which it belongs.

11. DOS WIZARD will list all

extents by sector range which are

occupied by a given file.  An extent

is a set of contiguous sectors.

PRINTING THE DOSWIZ DOCUMENTATION

Running the DOSWIZ.PRT program

using BASIC will print the

DOSWIZ.DOC file in 1.75 width letters

on a STAR (tm) GEMINI series printer.

It may work on other printers as

well.

Another way to print it would be

to use the COPY feature of your DOS

or DUP.SYS utility, specifying P: as

the output file name.  It can also be

printed from most word processors.

The actual GEMINI control characters

for page skipping are imbedded in the

text.  If you have a different brand

of printer, you may want to change

them.  They should work okay on the

EPSON (tm) printers.

A  DISKETTE ANALYSIS

This MENU option performs a

two-stage analysis of the contents of

a DOS format diskette. Either stage

may be interrupted by pressing the

ESC key, but any statistics will only

be valid up to the point where the

interrupt occurred.

Stage 1 reads all DOS files

which are not marked as deleted or

open and verifies that the chain is

intact. All chain errors are reported

with the sector number where the

violation occurred and the file name

for the chain being validated. A

chain error is reported if:

a) The file number in the sector

does not reflect the file being

validated;

b) The next sector pointer does

not point to a valid sector number;

c) The sector is not marked as

allocated in the VTOC;

d) A read error occurs while

trying to read the sector;

e) The byte count in the sector

is less than 1 or greater than 125.

Stage 1 reports the following

counts:

1. VALID FILES - the number of

files which are accessible by DOS and

contain no chain errors.

2. DELETED FILES - the number of

directory entries flagged as deleted.

3. OPEN FILES - the number of

directory entries flagged as open.

4. BAD CHAINS - the number of

files which were found to have

invalid chains.

5. SECTORS ALLOCATED - the

number of sectors which are marked as

allocated in the Volume Table Of

Contents (VTOC).

6. FREE SECTORS - the number of

sectors marked as not allocated in

the VTOC.

7. DEAD SECTORS - the number of

sectors allocated less the number of

sectors validated while verifying

chains.  These sectors cannot be

referenced by DOS.

Stage 2 reads every sector on

the diskette and reports the count of

sectors containing no data (ZERO

SECTORS) and the count of sectors

which could not be read (BAD

SECTORS).

Stage 2 will list by sector

number the first 64 bad sectors

encountered.  If there are more than

64 and you need to know where they

are, consider using FASTDUP.

C  CHANGE DISKETTES

This function must be performed

before processing a different

diskette since the VTOC and directory

are read into core only once by DOS

WIZARD.

D  DISPLAY DIRECTORY

This MENU function lists all

directory entries currently in use or

formerly used on this diskette.  It

also gives access to many functions

which may be performed on a file.

The directory listing provides

the following for each file:

a) FILE#- This is the number of

the directory entry.  They are

numbered from 0-63;

b) NAME- This is the file name

contained in this directory entry

Files which are recognizable and

accessible by DOS are in REVERSE

VIDEO.  These would appear in a DOS

directory listing;

c) START- The starting sector

number for this file;

d) COUNT- The number of

sectors used by this file;

e) STATUS- Will contain "D" if

the file has been marked deleted, "O"

if it is marked as open, and "L" if

the file is locked.

The display trailer contains the

number of free sectors and the number

of sectors accounted for which should

equal 707. If this number is other

than 707, there are dead sectors on

the disk. However there could still

be dead sectors even if there are 707

sectors accounted for. DISKETTE

ANALYSIS can be used to reveal this

condition.

DISPLAY DIRECTORY allows the

following functions to be performed:

a) UNLOCK - unlock file by

specifying file number.

b) LOCK - lock file by

specifying file number.

c) DELETE - delete file by file

number.  This will delete the entry

and free all sectors up to

encountering any chain error.  This

can be used when DOS delete will not

work due to errors.  If a file will

not delete with DOS WIZ, do a RECOVER

FILE (R) against the file and then

delete it.

d) RENAME - rename a file by

specifying a name for a file number.

e) LIST FILE EXTENTS - obtain a

list of all areas of contiguous

sectors occupied by a file.

f) RECOVER FILE - recover a

deleted file or a portion of a file

with a broken chain.  If the file is

an open file, an extender will be

used of Xnn where nn is the file

number. This prevents duplicate file

names due to having recreated a file

that was open and thought to be lost.

File recovery makes a valid file out

of the portion of the file that can

be read up to the point of an error

being encountered, thus to DOS it is

now a good file but it may still be

useless to you.  You can salvage

portions of a data (LIST format) file

but BINARY LOAD AND CSAVEd FILES

CANNOT BE USED AFTER RECOVERY except

by the DELETE function of DOS WIZ or

DOS.

M  SECTOR MAP

This MENU function displays a

translation of the sector bit map

from the VTOC.  Each sector from

0-719 is represented by either a

period (free sector) or an asterisk

(allocated sector). Since sector 0

does not exist, it is always shown as

allocated.  Sector 720 is not

recognized by DOS and is not shown on

the map.

To obtain the sector number of a

position on the map, add the numbers

from the scales on the left and top

of the screen.

There are three functions

available from this screen:

a) ALLOCATE SECTOR(S) - specify

starting and ending sector numbers

(inclusive).  These sectors will be

marked as allocated and will be

unavailable to DOS.  If the ending

sector is not specified, it will

default to the starting sector so

only one sector will be allocated.

b) FREE SECTOR(S) - same as

above except sectors are marked as

free.  These sectors become available

for use by DOS when new sectors are

needed.

c) RECLAIM LOST SECTORS - This

is the powerful recovery utility

discussed in the INTRODUCTION.  It

chases all non-deleted and unopened

file chains and remembers the good

sectors. Any sectors that are

allocated but not on a valid chain

are zeroed and released for use by

DOS.  If a chain error is

encountered, a message is displayed

indicating the sector and file. A

file recovery should be done on that

file (see DISPLAY DIRECTORY) and then

redo the reclaim function.  It is not

always necessary, but can't hurt

anything.

S  DISPLAY SECTOR(S)

This MENU function is used to

read a given sector and display it.

The left portion of the screen

displays hex representation of the

sector data while the right portion

displays most of the pertinent ASCII

characters represented by the sector

data.  The bottom of the screen has

valuable information about the

sector.  Information is displayed

only when pertinent to the sector

being displayed.

a) SECTOR: This is the sector

number of the sector being displayed.

b) NEXT SECTOR: This is the next

sector on the chain.  It is only

displayed when it is determined that

the current sector is indeed on a

chain.

c) BYTES: This is the number of

bytes used by DOS in this sector.

d) The lower left corner is used

to display the file name to which

this sector belongs and whether that

file is open, closed, deleted, or

locked.

There are several options

available from this screen.  If you

press Z, 128 binary zeros will be

written in the sector.  If the sector

is allocated, it can be freed by

pressing F.  If it is free, it can be

allocated by pressing A.  Pressing L

will read the next sector on the

chain, while RETURN will read the

next sequential sector.

If the sector is the last sector

on a chain then message END OF CHAIN

will appear in the trailer in place

of NEXT SECTOR.

Z  ZERO UNUSED SECTORS.

This MENU function writes a

sector of binary zeros to all sectors

which are marked free in the VTOC.

This is beneficial if you use my

FASTDUP program to duplicate disks.

Since FASTDUP does not store or write

any sectors which contain all zeros,

getting rid of unused data can speed

up the duplication process.  The

delete functions of DUP.SYS and DOS

WIZARD do not zero sectors as they

delete the file.  This is good since

mistakenly deleted files can

sometimes be recovered with DOS WIZ.

CAUTION--If you have sectors marked

as allocated but they are not on a

file chain (to protect them from use

by DOS), this function will not zero

those sectors, but the RECLAIM

function of the SECTOR MAP will.

Know what you are doing!

GENERAL INFORMATION

DOS WIZARD will work in a 24K+

ATARI 400 or 800 (ROM A or B).  It is

doubtful that it will work on the new

XL machines.  DO NOT USE ON DOUBLE

DENSITY disks or OS/A+ Version 4

files.  It will work on the ATARI 810

DOS Version 2 files and OS/A+ Version

2 files.  All 810 compatable drives

should work fine with DOS WIZARD.

This program has undergone

extensive testing, but the

flexibility and versatility of the

program makes complete testing

difficult.  Please contact me at the

address/phone number below if you

discover errors.  Please have good

documentation of the problem so that

it can be recreated.  Also please

contact me with ideas for future

enhancements.

DOS WIZARD was written for my

own use and for my friends in the

North Texas Atari Computer Team of

Fort Worth, Texas.  Please do not

sell this product for profit.  Make

as many copies as you like, but GIVE

them to your friends.

Send all correspondence to:

GARY C. CRIDER

1105 BURGESS COURT

ARLINGTON, TEXAS 76015

(817) 467-1114

BIBLIOGRAPHY

The following publications

(among others) have good explanations

of DOS file formats and disk struc-

tures:

DE RE ATARI by Chris Crawford, et al.

INSIDE ATARI DOS by Bill Wilkinson of

Optimized Systems Software, Inc..

Published by COMPUTE! Books of

Greensboro, North Carolina.

If all this went by faster than you

could read it, list it again and use

CTRL-1 to stop the display wherever

you want and then CTRL-1 again will

resume listing.

OTHER PRODUCTS BY GARY CRIDER:

FASTDUP- Sector-by sector disk

duplicator.

DISASSEMBLER 6502- Reassembleable

disassemblies from disk boot

files, binary load files,

selected sectors or memory to

disk, printer and/or screen.

THE SHADOW- You have to see it to

appreciate it.  Full buffer

copying of multiple files.

Crosses densities even on a

single drive system.  Many nice

and convenient features.  You

may never use DOS copy again.