LINKLINE:   (LINKLINE.EXE)

Written by Patrick Bass

(Originally published in the Aug.  '86 ANTIC.

Enhanced ST version is brand new.)

LinkLine is a PAIR of programs which let you quickly transfer files

between information between your 8-bit and your ST computers.

LinkLine will quickly and reliably transfer programs, text files,

picture files -- any type of file shorter than 33K (about 270 5-1/4"

disk sectors).  It's easy and it's fast (4800 baud, about sixteen times

the speed of an Atari 1030 modem)!

HOW TO GET LINKLINE

Use XMODEM to download *both* LINKLINE programs.  Use a modem

program for your 8-bit computer to download the 8-bit side of LinkLine,

and download the ST side with an ST modem program.

USING LINKLINE

You need:

A - 8-bit Atari computer

B - An Atari 850 Interface Module

with a modem cable (RS-232 to D-9.)

C - Null modem connector

D - Gender changer (if required)

E - Any Atari ST computer

| A |=>| B |=>| C |=>| D |=>| E |

Sample porting configuration

WALK-THROUGH DEMO

1) Rename your ST version of LinkLine to "LINKLINE.ACC". Rename the

8-bit version to "AUTORUN.SYS".

2) Turn both computers off.  Connect the two computers together by

plugging the ST end of the Null Modem cable into the 25 Pin RS-232

connector on the back of the ST, and the 8-bit Atari end into Port 1 of

the 850 Interface.  Now insert your LinkLine disks, and turn both

computers on again.

3) At the ST, point to the "Desk" item and click on the menu choice

called "LINKLINE".

4) To transfer a file from the 8-bit Atari into the ST, choose the

LOAD A FILE option and type in the name of the file you want to

transfer.  LINKLINE will read the file off the 8-bit disk, and copy it

to an ST disk.

5) To transfer a file from the ST into the Atari 8-bit, choose the

SAVE A FILE option and type in the name of the ST file to transfer.

LinkLine will read the file in and then prompt you for the name to call

it on the Atari 8-bit disk. When you type in the destination filename

and click on OK, LinkLine will copy the desired file from the ST onto

the 8-bit disk.

ADVANCED USE

LinkLine will copy any file under (roughly) 33K long. It cannot

tell the difference between a picture file and a text file.  Atari 8-bit

word processors use a special character, number #155, to mark where

lines of text end. Atari ST computers use a standard carriage

return/line feed at the end of each line.  Just before LinkLine

transfers a file, it will ask you if you wish to convert Atari 8-bit

End-Of-Line (#155) characters to Atari ST CR/LF pairs.

If you are transferring a *text* file, use the EOL and CR/LF

translations.

Some ST word processors place a CR/LF pair at the end of each line

onscreen, with a double CR/LF pair between paragraphs.  Most Atari 8-bit

word processors want their EOL characters only between paragraphs.  To

account for this, LinkLine will ask if you desire any "single CR/LFs"

stripped out of the file.  Again, if you are transferring a *text* file

from the ST into the Atari 8-bit, and aren't satisfied with the results,

try stripping all single CR/LFs.

That's it.  You're done.

!ZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZ