WEFAX

Weather Satellite Facsimile

by CHARLES JACKSON

Antic Program Editor

Tune in a weather satellite and downlink a few pictures

from space!  A standard shortwave radio plus this issue's

WEFAX Decoder software and WEFAX Interface board are all that

your Atari needs for receiving and displaying Weather

Facsimile pictures from the satellites of the US and other

nations, as well as facsimile photos from newswire services.

This Weather Facsimile system is the most ambitious

project ever published by Antic.  It required more than six

months of intensive research, programming and hardware

tinkering.  But the result is the most versatile and

friendliest weather satellite software available for the

Atari 8-bit and ST computers.

The reason this project turned out to be so demanding is

that its success depends on many factors -- including

software, hardware and the forces of nature Especially

important are what type of radio and antenna you use, and the

area you live in.

You need a shortwave radio capable of receiving SSB

(single side-band) signals.  Inexpensive "all-band" portables

with telescoping antennas will not suit our purposes.  We

developed these programs using a Radio Shack DX-302 attached

to a 25-foot longwire antenna.  But for good results, you

should use the best receiver and outdoor shortwave antenna

you can afford.

Also, even if you type in the program perfectly and

build the interface circuit without a hitch, you could be out

of luck if you live in an area with poor shortwave reception.

WEFAX signals are clearest in suburban and rural areas.  If

you live in the center of a large city, you may have trouble

receiving a clean WEFAX picture.

PROJECT ELEMENTS

Before you can use the Decoder programs, you'll have to

build the WEFAX Interface described in this issue.  The

interface is a simple circuit that can be built for under

$20.

WEFAX.EXE is for 8-bit machines, WEFAX.TOS is for the

ST. Copy WEFAX.EXE to another

disk and rename it AUTORUN.SYS, then follow the instructions

in the accompanying articles.

Listings 2 and 3, FAX.M65 and FAXA.M65, contain the

MAC/65 source code for WEFAX.EXE.  You do not need to type in

these listings to use the WEFAX program.

Listing 4, WESIM.BAS, is a BASIC program that simulates

a WEFAX signal.  If you've never heard a WEFAX signal, this

program will create one for you.  Listing 5, WETST.BAS, is a

BASIC program that helps you test your WEFAX Interface

circuit.

The ST version of WEFAX Decoder is WEFAX.TOS and you'll

find the article explaining it in this issue's ST Resource

section.

THANK YOU!

Finally, Antic would like to thank all the people who

helped us with this project, including:  Dr.  Ralph Taggart,

Dr.  Marty Goodman, CompuServe Vice-President Sandy Trevor,

Vic Moore, Jim Grubs, Chris Elmquist and Wayne Day.  Special

thanks to the staff of the San Francisco Area Office of the

National Oceanic and Atmospheric Administration, including

William Pettyplace and Jan Null.

SUGGESTED READING

If you want to learn more about Weather Satellite

transmissions, here are some of the best sources to get you

started:

The New Weather Satellite Handbook, by Dr.  Ralph

Taggart.  This is the definitive text on WEFAX.  Formerly out

of print, an updated edition of this popular book is

available from the author for $12.50.