Here is the DOCs and UUE T: handler for the Xm301.  I couldn't get

it to work with Kermit65.  If someone does, please let me know!

ATARI[R] XM301 MODEM[TM]

TECHNICAL SOFTWARE SPECIFICATIONS

This manual contains the reference

specifications for the software

interfaces used by applications

programs and the Operating System to

control the XM301 Modem.

Contents

Section 1: Description of the XM301

Modem

Interface With Other Products

Family of Products

Reference Documents

Section 2: Logical Requirements for

the XM301 Modem

Man/Machine Interface

The Six Standard Input/Output

Command Interfaces

OPEN

CLOSE

INPUT

OUTPUT

STATUS

Parameters Passed as Data Strings

Error Handling

Buffer Use

Handler/Operating System Interface

SECTION 1

DESCRIPTION OF THE XM301 MODEM

The XM301 Modem is a stand-alone

peripheral which connects to the

ATARI 400[TM]/800[TM], and the

XL[TM]/XE[TM] series with the SIO

interface by means of the serial bus.

The modem connects to the telephone

line by means of the RJ-11C jack.

The modem control software resides in

the internal ROM of the 8048 single

chip microprocessor.  A separate

software handler will be disk-based.

Interface With Other Products

The XM301 Modem is an input-output

peripheral, and performs no actions

without instructions from an

applications program.

To use the XM301 Modem from a

high-level language, the following

types of commands are required:

OPEN

CLOSE

GET (or equivalent READ, INPUT)

PUT (or equivalent PRINT, WRITE)

POKE

PEEK

Family Of Products

The XM301 Modem performs

telecommunications functions in

conjunction with applications

designed specifically for use with

the XM301.  It is interface

compatible with the Telelink II[TM]

application.  The modem is capable of

accepting any commands issued by the

Telelink II handler without

performing in an irregular fashion or

returning the "illegal command" error

code.

Reference Documents

ATARI Home Computer System Technical

Reference Notes:

Operating System User's Manual

Operating System Source Listing and

Hardware Manual

The Technical Reference Notes are

available from ATARI Customer

Relations, P.O. Box 61657, Sunnyvale,

CA  94088.  Cost:  $29.95 plus $2.50

for shipping.  California residents

should add appropriate sales tax.

Write "Order for Technical Reference

Notes" on the outside of the envelope

to speed up the order process.

SECTION 2

LOGICAL REQUIREMENTS FOR THE XM301

MODEM

The handler is provided by an

AUTORUN.SYS disk file.

The modem controller, an 8048H

microprocessor, is accessed by the

handler through the serial I/O

channel.

The handler uses RAM for input and

output buffers, variables for

communicating with the user, and

internal variables.  Internal

variables are in low memory, with one

variable (CMCMD) permanently

allocated in the Operating System

database for use by this handler.

User communication variables are the

system IOCBs, the zero page IOCB, and

the command flag CMCMD.  IOCB use is

described in the ATARI Home Computer

System Technical Reference Notes, and

in Section 2, Handler/Operating

System Interface.  CMCMD use is

described in Section 2, Parameters

Passed As Data Strings.

The XM301 handler allocates an input

and output buffer where data can be

entered between the application and

the handler.  The output is 64 bytes

long; the input buffer is 256 bytes

long.  There are 2 bytes used for

buffer control variables by the

handler.  Applications use of buffer

control bytes is described in Section

2, Buffer Use.  The remainder of the

buffer area is reserved for the

buffers.  The use of the buffer area

is decided upon by the handler.  The

application program has no defined

way of directly accessing data in

this area.

Man/Machine Interface

It is the responsibility of the mass

storage media device to contain and

load the XM301 handler along with the

required applications.

The Six Standard Input/Output Command

Interfaces

The applications program accesses the

modem handler by  calling on CIO,

which in turn, calls the handler.

There are six I/O calls.

(1)  OPEN

An OPEN command must be performed

before any other modem operations are

allowed.  Any of the IOCBs may be

used.  Aux1 specifies the I/O

direction:  bit 2 for input; bit 3

for output; both for bidirectional.

The directionality applies only to

communications data; initialization

parameters are passed in either

direction after any OPEN command.

Aux2 is ignored by the handler.

The device name is "T".  The rest of

the filename is ignored.

The command flag (CMCMD) is set to

zero by the handler during OPEN.

(2)  CLOSE

The CLOSE command terminates the I/O

procedure and releases the IOCB.  Any

data waiting in the output buffer is

sent first.  The buffer area is then

relinquished and any input data

remaining in the buffer is lost.

When the buffer area is empty, the

phone is placed on hook.

[System.Reset] produces similar

results to the CLOSE command except

that the output buffer is not sent.

Refer to Section 2, Handler/Operating

System Interface, for details.

The modem operation may be suspended

without closing the IOCB, allowing

the serial bus to continue

communicating with other devices.

This operation is described under the

appropriate commands in Section 2,

Parameters Passed As Data Strings.

(3)  INPUT

Any INPUT or GET operation through

CIO will result in the modem handler

being asked to provide data for the

user.

If no data is available, the handler

waits for a character.  There is no

time-out when waiting for received

data.  This waiting is terminated

when either the [Break] or

[System.Reset] key is pressed.  The

waiting is also terminated if the

carrier is lost. Error 136 is

returned when the carrier is lost.

If data has been received, the

Handler stores the characters in the

input buffer as the data is received

(up to the buffer's capacity).  The

CIO request for data is then

satisfied from this FIFO buffer.

(See Section 2, Buffer Use.)

The modem is double-buffered, so the

handler has one character time to

input each character received.

Communications data is processed for

parity and for translation from ASCII

to ATASCII.  These operations are

described under the appropriate

commands in Section 2, Parameters

Passed As Data Strings.

(4)  OUTPUT

Any PRINT or PUT operation to CIO

will result in one or more bytes for

the handler to hand to the modem.

This data is interpreted either as

set-up commands and parameters, or as

communications data, as controlled by

the command flag byte CMCMD.  (See

Section 2, Parameters Passed As Data

Strings.)

Communications data is processed for

translation from ATASCII to ASCII,

and then from parity.  These

operations are described under the

appropriate commands in Section 2,

Parameters Passed As Data Strings.

Output bytes are received by the

handler and placed into the output

buffer as fast as the bytes come from

CIO.  If the FIFO buffer fills, the

handler loops other character from

CIO until space becomes available in

the buffer.  There is no time-out

when the handler is waiting for

buffer space.  The looping is broken

when either the [Break] or

[System.Reset] key is pressed.  A

loss of carrier also breaks the loop,

and returns Error 136.

Any time the buffer contains data,

the modem continues to send until the

buffer is emptied.

(5)  STATUS (Combines with PEEK)

The STATUS command causes the handler

to place four bytes of status

information in the Operating System

status area DVSTAT through DVSTAT+3

(02EA - 02ED).  These four bytes

contain error and status information

from both the modem and from the

handler.  The application program can

then PEEK at these four bytes.

The four bytes contain the following

bit-encoded data:

DVSTAT+0:  Communications Error Flags

The internal flags corresponding to

the status bits in this byte are

reset upon STATUS request.

Bit 7: Framing error encountered on

received data.

Bit 6:  Byte-level overrun

encountered.  The modem lost one or

more characters due to slow interrupt

handling.  This is caused either by

slow reading of data by the

application, or by an incorrectly

programmed main CPU.

Bit 5:  Parity error encountered on

received data.

Bit 4:  Received data buffer

overflow. The received data buffer

overflowed because the applications

program did not input data from the

buffer in time.  If the received

buffer has not been supplied, only

error bit 6 is set.

Bit 3:  Zero  (Reserved for

expansion).

Bit 2:  Zero  (Reserved for

expansion).

Bit 1:  Zero  (Reserved for

expansion).

Bit 0:  Illegal command (recognized

by handler).  This error applies only

to the most recent command.  This

error is cleared upon acceptance of a

legal command.

DVSTAT+1:  Modem  Status Flags

Bit 7:  Carrier detect.  1 when the

carrier is present.

Bit 6:  Zero  (Reserved for

expansion).

Bit 5:  Loop Test enabled.  1 when

Analog Loopback Test is active.

Bit 4:  Originate/Answer.  Zero =

originate, 1 = answer.

Bit 3:  Auto-Answer.  1 = enabled, 0

= disabled.

Bit 2:  DTMF dialing.  1 for DTMF

dialing.

0 for pulse

dialing.

Bit 1:  Zero  (Reserved for

expansion)

Bit 0:  Off hook.  1 when the phone

is off-hook.

0 when the phone

is on-hook.

DVSTAT+2:  All bits set to zero.

(Reserved for expansion).

DVSTAT+3:  All bits set to zero.

(Reserved for expansion).

(6)  SPECIAL (XIO)

This call is not supported by the

handler.  The handler returns Error

92 (hex), 146 (decimal), Function Not

Supported.

Parameters Passed As Data Strings

Control functions (i.e., setting

translation and parity handling,

sending a break signal, and dialing

the telephone) are handled by passing

the commands and parameters to the

handler as the output and input data.

There are two types of commands:

Those with parameters and those

without.  Commands start with a

two-byte escape saequence output to

the modem handler.  This escape

sequence identifies the start of the

command and specifies the command.

If the command containsparameter

bytes, they are sent immediately

following the escape sequence.

In normal operation, the CIO output

deals only with communications data.

The handler checks the Operating

System RAM area command flag byte

CMCMD as it receives each output from

CIO.  (CMCMD is fixed in the

Operating System database at location

0007.)  If the flag is zero, the

handler treats the byte as

communications data.  If the flag is

nonzero, the handler treats the byte

as a command.

When the handler is not processing a

command, output bytes sent with CMCMD

nonzero are checked for a command and

are not transmitted.  The handler

starts processing a command when:  It

is not processing a previous command;

CMCMD is nonzero; or the output data

byte received is ESC (1B hex, 27

decimal).  Once a command sequence

starts, the handler finishes the

sequence regardless of the state of

CMCMD.  The number of output and

input bytes is constant for any given

command