THE HAPPY KILLER

BLITZ COPIER

////////////////

/  BLITZ v1.50 /

////////////////

Are  you  ready for a $22.00 copier that will copy as good,  or  in  most

cases better,  than a $150.00  cart?  Yes??  Well then, Blitz is

for  you.   Blitz is a revolutionary new back-up system for the Atari  ST

computer.   Using  a  special  cable (that you  can  make  yourself)  and

software included in this arc,  it will back up your software at a  speed

and power unheard of before.

If  you know a little about wiring and how to solder,  that's  the  most

that this copier will cost you.   If you have to have someone make one up

for you,  it might cost you $30-40 (although some people have been  known

to pay up to $50 for a pre-made cable).

That's  right,  all  that  is needed is a cable  (and  two  drives).   No

internal  wiring to be done in the  computer.   No  special,  over-priced

cartridges.  In a nutshell, all you do is cut an Atari drive cable in two

and then attach a third plug that goes into your printer port.  What this

cable does, is copy from drive one, out through the printer port to drive

2.  (Copying is ALWAYS from drive 1 to drive 2.)

It reads from drive 1 and writes to drive 2 at the same time!   So in the

time it takes ACopy,  ProCopy,  or Happy to just read a disk into memory,

this program has read AND written out the same disk in one pass.   And it

does  not care if the disk is protected or not,  it copies just  as  fast

either  way  (this  way  you can make a legal backup  copy  of  all  your

protected software).   Plus,  it seems to copy a lot of stuff that ACopy,

ProCopy AND Happy won't copy!!!

BLITZ operates the disk drives at a very low level. When backing up disks

it  should go through an entire disk without a pause.  If it  does  pause

(for   more  than  a  second  or  so)  then  it  may  have  got  out   of

synchronization.  In this case,  press ESCAPE to abort the copy and start

it going again.   So the best thing to do is watch it copy and count down

the tracks.   If it stops for even a second,  then you can forget it  and

start  again,  as it has gotten out of sync and the copy will  not  work.

But  for a program as fast and as cheap as this,  you can afford to  take

the time to count down the tracks.

One  other thing.   There are very few programs that this  program  won't

copy  IF you're drives are in alignment and up to proper speed.   If  the

copy  that  you make does not work properly,  try  changing  your  drives

around (if possible) or try it on someone else's computer and drives.

BLITZ comes as a program or a desk accessory.  To run it as a program you

will need "BLITZ.PRG" and "BLITZ.RSC" in your working folder.   To run it

as  an accessory,  you will need "BLITZ.ACC" and "BLITZ.RSC" on the  root

directory of your boot-up disk.

If  you are unsure as to whether a disk is single sided or  double  sided

copy  it  double  sided (the main difference will be that  it  will  take

longer).

BLITZ will quite happily appear to write to a write protected  disk.   Do

not  worry here - If a disk is write protected then there is NO WAY  that

BLITZ can write to it (unless your disk drive is  faulty).  However,  you

must be careful that when you DO want to write to a disk that the disk is

not write protected - because BLITZ will not notice.

IMPORTANT  NOTE:  You  will not be able to use your external drive  in  a

normal way with the BLITZ lead plugged in.

MAKING YOUR OWN BLITZ CABLE

(See BLITZ.PC3 or BLITZ.IMG for cable layout!)

Here's all you'll need........

1 Atari drive cable.           Cost = about $20 for a six foot one.

1 24-pin printer plug (male).  Cost = $1.00

1 Printer plug hood.           Cost = $1.00

....That's it!  Just print out the BLITZ.PC3 or BLITZ.IMG file and you'll

see how to hook up the printer plug into the drive cables.

I suggest that you obtain an Atari drive cable and cut into two parts  to

obtain plugs one and two.

Plug 3 is a 25-pin male D connector, available at any Radio Shack Store.

On machines with one internal drive and one external drive,  plug P1 into

the computer drive-out port.   Plug P2 into the 2nd drive's drive-in port

and plug P3 into the computer printer port.

On  machines with two external drives,  connect drive one in  the  normal

manner.   Plug P1 into the drive 1's drive-out port.   Plug P2 into drive

2's drive-in port and P3 into the computer printer port.

You might consider which setup you have before you cut the cable.

With this setup, the computer will not recognize drive 2 unless the BLITZ

software is running.   So if you leave your machine setup with the  BLITZ

cables  connected,  you will lose the use of drive 2 for  everything  but

copying.

Switch boxes can be setup to allow you to leave the setup intact.

I'm  sorry  but I can't tell you the color code on the  Atari  cables  as

every one is different.

/////////////////

/  B_TURBO v1.9 /

/////////////////

You will by now have seen that B_TURBO has a New Menu, you can now set the

tracks for Blitz to start its coping and there has been added the AUTO that

lets Blits read for its self what type of Disk it is going to copy.

//////////////

/ BUG REPORT /

//////////////

There seems to be a Bug in the AUTO part of B_TURBO v1.9, that is some times

when you run it, it does not Read the Disk right, its no Big thing all you

have to do is Just ReBoot and it will work Okay!

//////////////////

/  T_BLITZ v2.5  /

//////////////////

The Bug in v1.9 has been Fixed in this New Version of T_BLITZ. This version

is by Plexus Systems Limited and is (c) by same, there are a lot of different

versions of BLITZ in the ST market as of this Date, but they all seem to be

on the same line as to how they work, copying at a low level through the

Printer Port. Even though this New Version has the same byte size as v1.9

it has been Fixed. So don't let the Size of the Program fool you.

If any one finds any other Bug please let us know!

/////////////

/ Amendment /

/////////////

This is an Amendment for Turbo Blitz v2.5, if your File Date is not 05-10-90

or higher then your copy is not the one with the AUTO Bug Fixed. Sorry

but when you have as many files as we do to Upload one can grab the wrong

file.

The RSC file Dated 5-10-90 or higher has been redone also,

1.) If you use T_BLITZ v2.5 on a Atari SC1224 you will find that it now

has Color.

2.) If you use T_BLITZ v2.5 on a Atari SM124 you will also see a New Dialog

Screen.

^

EOF

"""