Roland D10 Dump utility v1.0

����������������������������

Files

�����

This archive should contain the following files.

- DUMP1_0.PRG (Roland D10 Dump program v1.0)

- ENGLISH.DOC (This document file)

- README.1ST

- 13 Sound-files (12 *.SND + DEFAULT.ALL)

Public Domain

�������������

This program is public domain. This means that you can make as

many copies of it as You see fit, and freely distribute these to

friends, Bulletin Board Systems etc. Please note that You can't

charge anything for the program(except a reasonable charge to

cover distribution expenses). You're not allowed to alter the

program or the accompanying document-file, and You must always

distribute these files together.

Disclaimer

����������

This program is supplied in the belief that it operates as specified,

and the author shall not be liable in any circumstance whatsoever for

any direct or indirect loss or damage to property incurred or suffered

by the person using this software, or any other person, as a result of using

this software.

Revision History

����������������

This is version 1.0(as of 92/04/26).

Using the Program

�����������������

The program only runs in high resolution mode, but this will

probably change when the next version hits the streets. I didn't

have enough time to learn how to use resource-files in conjunction

with GFA Basic, so I made my own graphical user interface, and

this is screen-resolution dependent. Sorry! If You don't have

access to a mono monitor You could always try one of the many

mono emulators on the PD-market, but I'm not sure if this will

work because the dumping of data from the D10 to disk is time

critical, and most screen emulators have a tendency to devour

CPU time. Because of this there is a probability of losing data.

There's no harm in trying, though, as the program will inform You

about any loss of data. If the mono emulator is capable of

setting the amount of CPU time that it uses, then please set this

as low as possible.

The available menus(with options) are:  * Atari (Atari logotype)

- About DUMP1_0

* Dump

- D10  to Disk

- Disk to D10

- Quit

* Help

- Dumping

- Filetypes

About DUMP1_0: Displays version number etc.

D10 to Disk:    Initiates data transfer from D10 to disk.

An alert box will pop up and ask for the

filetype to be used. Next You will see a

GEM fileselector where You can enter the

name of Your dump-file to be, or abort by

pressing the Cancel button. There's no need

to give a file-suffix as this will be auto-

matically appended by the program, and even

if You do, the program still will be

appending the correct suffix(according to

chosen filetype). This is for Your con-

venience, so that You'll be able to keep

some kind of order among Your files.

Having entered a filename and pressed OK,

the transfer will start immediately. You

won't need to push any buttons on the D10.

The dumped data will then be saved under the

filename You gave in the fileselector.

This option can also be reached from the

keyboard via ALT-S(Hold down Alternate

key and press S). S as in Synthesizer or

Save.

Disk to D10:    The other way around! It operates the same

way as D10 to Disk(above). When in the file-

selector, just pick the desired file and

press OK-button. This will dump data in

file to the D10. Note that there's no check

to see if the filename given has the same

suffix as the filetype selected, but the

fileselector will open up with *.filetype

(e.g. *.SND if .SND is the selected file-

type). This option can also be reached from

the keyboard via ALT-D. D as in Disk or Dump.

(Clarification to the above): The program will

quite happily dump any MIDI Exclusive file

that You have around for Your D10. In order

for this to work You must first rename Your

file from *.XXX to *.SND(or *.RHY or *.ALL),

and then select the appropriate filetype to

the same suffix as You gave Your old file.

Example: You have a MIDI Excl. file for Your

D10 that contains the factory settings, and

this file is named DEFAULT.D10. Just rename it

to DEFAULT.SND, and when in DUMP1_0 select

Disk to D10 and .SND as the filetype.

Quit:           Quite self-explanatory! You will be able to

take back if You made this choice by accident.

This option can also be reached via ALT-Q.

Q as in ... hmm? Quadragenarian maybe?

Dumping:        This displays a brief helptext about the

dumping procedure.

Filetypes:      This displays a brief helptext about the

filetypes used by the program, and their

meaning.

Filetypes

���������

The filetypes(i.e. suffixes) used are: .SND, .RHY and .ALL.

They indicate the kind of data to be processed.

.SND: Sound data only, i.e. Timbres, Patches and user

defined Tones(i01-64).

.RHY: Rhythm data only, i.e. User Rhythm Patterns(P51-88),

Rhythm Track and Rhythm Setup.

.ALL: Sound and Rhythm data as above plus Master Tuning,

Pan(Part 1-8), Reverb Type/Time/Level(Multi Timbral

mode), and Level(Part 1-8, Rhythm Part).

If in doubt, please consult Your Roland D10 manual, as this

program transfers the same data as the Data Transfer function on

the D10.

Other Roland D-series Synthesizers

����������������������������������

If I own a D20, can I use this program? Frankly, I don't know.

This depends on a whole lot of things, the memory map of the D20

compared to that of the D10, The Model ID#(used in MIDI Exclusive

messages, etc. The D20-owners maybe aren't that very interested in

this program as they can save their sounds to disk via the built-in

disk drive, but then there's always nice to be able to swap

sounds with D10-owners. If You do decide to try this program with a

D20, You should first check if the D20's Model ID# corresponds to

that of the D10. If so, I think there is a fairly large chance that it

will work due to the small differences between the models. The D10's

Model ID# is 16HEX(=22 Decimal). As for the D110 I don't think that

it will work, because I seem to remember reading something about different

addresses compared to the D10. The D50 is definetely ruled out, and as

for the D5, I really can't say. It would be easy to incorporate

functions for these(except for the D50), but in order for me to

do that I need information about address spaces etc. Maybe I will

try to obtain this info before the next release, who knows?

Your own D10-dumps

������������������

If you've made some great sounds, rhythm patterns etc. Yourself,

why not dump them and upload to Your local BBS or the atari

archive (terminator).

Bugs!?

������

I have tested, and used, this program for a while now and it

seems to work just fine(they always do, don't they!). If You

should discover a bug, or some other kind of annoying feature

please let me know, and I will try to fix it. In this version

there is one thing I'm not very content with. GEM alert-boxes

that appear when a TOS-error is generated(e.g. when You're trying

to save a file and the disk is write-protected). I spent a great

deal of time trying to direct these errors to my own error

handler(as suggested in the GFA manual), and while this worked

with simulated errors, it was another story with the real ones!

Therefore if You should happen to encounter an operating system

alert-box, never pick the Abort option as this most certainly

will crash the program. The Retry option usually works. I've run

into this problem when I've tried to save a dump to disk, and the

disk was write-protected. The Retry option worked just fine!

Just before releasing this program I discovered another peculiarity. This

time about the handling of the System Area(Master Tuning, Pan etc.).

For some strange reason the D10 won't write this packet into its

internal memory upon receiving it. It simply writes it into

temporary memory. I've checked to see if the dumps You get from

DUMP1_0 differs in any way from the D10's dumps(using Data

Transfer/One Way transfer) concerning this area, but I found no

diffs. Maybe one must issue some kind of write command to make

the changes permanent. What does this mean for me, the user?

Let's take a brief example! Your'e going to dump a *.ALL-file

to Your D10(i.e. Sounds, Rhythm & System Area). Your D10 is on

and in Multi Timbral mode. You start DUMP1_0.PRG on Your Atari,

and dump the file. When checking under the TUNE/FUNCTION menu on

the D10 You can see that the new data for Master Tuning etc. are

there(provided that You knew what was there before, of course!),

but as soon as You switch over to Performance mode and then back

to Multi Timbral mode You will notice that the old values are

restored. This DOES NOT apply to Sound & Rhythm data, only to the

System Area! Meaning that the new Sound & Rhythm data will be

retained even if You switch off the power and then switch it

back on again.

Just one more thing! This is not a bug really, but it could easily be

mistaken for one. Here it goes! If You manipulate the fileselector to

show *.*(all files) when You try to dump a file from Disk to D10, and

then select a file with a different file-suffix than that You gave as

filetype(in the alert-box), the program will tell You that the selected

file does not exist! But You saw it! It was there, wasn't it? You're right,

it was there(and still is). It's just that the program takes the filename

given in the fileselector, appends the suffix, and then checks to see if

it exists. That's why!

Updates

�������

Here is a brief summary of what I hope to be able to include in

the next version of this program.

- Better GEM support by use of resource file (all screen

resolutions supported, and correct error handling)

- Support for other Roland D-series synthesizers

- Options to dump single Patches/Timbres etc.

- Librarian

- Support for other data formats(e.g. from tone editors)

- Correct handling of the System Area

- Improved file handling(see note under Bugs!?)

Suggestions

�����������

As for the program in it's present state, it is nothing more than a

smart dump utility(You don't have to press any buttons on Your synthesizer),

but it is also a lot dumber than an ordinary dump utility(it's not generic,

but limited to a specified synthesizer), so suggestions are welcome.

If You have a great idea for an improvment, or a feature that You think

ought to be included, You can either write me a letter or send an E-mail.

Nils Ekholm

Amanuensv. 1/106

104 05 Stockholm

Sweden

E-mail: ekholmATelixir.e.kth.se

The AT between ekholm and elixir is the commercial AT-sign

(an a with a "ring" around it). Here it is "@"! If You can't

see it then try a wordprocessor or an editor to read this file.

Bye!