A SYNC SIGNAL SWITCH BOX FOR THE MIDI HOME STUDIO.

Files in this .lzh file include..........

SYNC_SWT.DOC  What you are reading.

SYNC_SWT.PI2  Med. rez (degas) wiring diagram.

SYNC_SWT.PI3  High rez (degas) wiring diagram.

Note...  The  PI3  and PI2 files must be viewed with degas, degas

elite or some kind of degas picture viewing program. One public domain

program that works well is DCSHW11.ARC file# 14572.

INTRODUCTION.......

This  is  a  handy  MIDI  sync signal switcher for home recording

studios  with  multi  track  tape  decks.  If  you've got a 4,8, or 16

channel  setup, and you hate to repatch RCA cables every time you want

to  use  a  sync  box,  you  too  can  benefit  from  this  simple and

inexpensive  project.  You  will  never have to repatch the RCA cables

from  the  in  and  out on your sync box, or unplug them from the tape

deck  input  and  output  if your not using the sync box. And the same

goes for your mixer input and output.

PARTS..............

The  parts  are  easy to come by. you'll need (6) RCA panel mount

female  jacks, (1) project box, (just about any size will do). (2 feet

at  the  most) of 20 gauge hookup wire, (1) 3 position wafer switch. A

wafer  switch unfortunately can't be bought at radio shack. However it

can  be  found  at  any electronic supply house or just about any well

stocked  music store. You see, this switch is also the pickup selector

for  a  Fender Stratacaster guitar. (In the degas drawing is a section

called  "signal path". I originally did this to further understand how

the switch actually worked and to help in the design. I left it in the

drawing  as an aid to those who might know of another switch that will

do the same job.) And if you want to install the optional test switch,

you'll need (1) normally open momentary push button switch. You should

be able to set yourself up with everything for around $10.

CONSTRUCTION.......

The hardest part of constructing this project is cutting the slot

for the wafer switch. It requires a slot (1 inch long) and (1/16 of an

inch  wide).  I've  included  a  section in the drawing called "switch

slot".  This  shows  the  exact measurements of the slot. There's many

ways  to  cut this, more then I shall discuss. But here are three. The

best  and  easiest  way  is if you have access to a bridgeport milling

machine  and  1/16"  bit,  like  I  did.   But  a  DREMEL tool with an

adjustable  fence  and a tiny carving bit is a more realistic approach

to  the  job. And if a DREMEL tool is beyond your means. Alot of 1/16"

holes  in  a  line and some time with a small rat tail file will serve

the  purpose. The RCA jacks require drilling 1/4 inch holes. I suggest

you  label the switch, RECORD - BYPASS - PLAY  for convenience. And it

wouldn't  hurt  to  label  the  jacks  either.  There's  not much more

explanation of the assembly necessary. The degas drawings say it all.

SETUP PATCHING.....

This  part might get a little confusing because there are so many

different  systems  and setups. I'll try to make it plain as possible.

Just for example, lets say you have an 8 channel deck and an 8 channel

mixer setup. And you always put your sync signal on track#8. Jack#1 is

labeled  TO  MIXER.  It  goes to channel #8 line or tape input on your

mixer.  Jack#2  is  labeled  FROM DECK. Connect a cord from channel #8

output  on your tape deck to this jack. Jack#3 is labeled TO DECK. You

want  to  connect a cord from here to the channel #8 input on you tape

deck.  Jack#4  is labeled FROM MIXER. A cord from channel #8 output on

your  mixer  goes  here.  Jack#5  is  labeled TO SYNC IN and jack#6 is

labeled  FROM  SYNC OUT these are self explanatory. This box will work

with  many different setups. If you have a 4 channel setup just change

the #8 to #4 etc.

OPERATION..........

The  operation  of  this  box  is pretty simple. When you wish to

record  the  sync  signal  on the tape, place the switch in the RECORD

position.  If  you  want your recorded signal to trigger your sequence

program  or sequencer, place the switch in the PLAY position. And when

you want to return to normal operation with no sync, set the switch in

the  BYPASS  position.  If  you've  installed  the  test  switch, when

pressed,  you can hear the sync tone. The tone is sent to the mixer. I

installed  the  test  switch  to  troubleshoot  the  system.  It's not

necessary for operation.

OTHER STUFF........

As  simple  as  this box is, and if you were able to buy it, you'd

probably  pay  somewhere  in  the  neighborhood of $40 to $50 or more.

Sickening, isn't it? Well, the point I'm getting at is, this box is my

own  invention, and if it makes your MIDI setup more efficient, my job

is complete. But still any contributions would be greatly appreciated,

no matter how small.

There's  not much more to say, except I hope you'll be doing more

music and less repatching.

Another D.I.Y. project from Flying Monkey Studios.

Mark Bombard

74 Loveland Rd.

North Granby, Ct. 06060

(203) 653-3134