#06601030305800

#F0110030

#9[...................................................]001

#�The#Z8530#SCC#driver#for#the#Atari#ST#and#PC#Clones#�

By#R.E.#Janssen,#PE1CHL.

A#driver#has#been#written#to#support#a#number#of#Z8530#SCC#chips#

connected#to#an#Atari#ST#computer.##The#driver#is#configurable#

for#different#hardware#configurations,#and#an#example#schematic#

supporting#up#to#7#SCC's#(14#channels)#has#been#included.

The#driver#now#also#runs#on#PC#Clones,#although#the#interrupt#

handling#capabilities#of#this#type#of#computer#limits#the#maximum#

speed#and#number#of#channels#to#a#lower#value#than#on#the#Atari.#

(depending#on#the#speed#of#your#machine)

Each#channel#may#be#indepently#programmed#to#support:

#####-#An#asynchronous#SLIP#line

#####-#An#asynchronous#interface#to#a#KISS#TNC

#####-#Direct#AX.25#(HDLC)

The#software#has#been#written#in#such#a#way#that#a#port#to#

another#environment#where#Z8530#SCC's#are#available#should#be#

relatively#straightforward.##Addresses#of#the#chips#are#

completely#configurable,#and#most#probably#only#the#first-level#

interrupt#handler#(written#in#assembly#language)#has#to#be#

adapted#to#the#specific#configuration#of#the#interrupt#

acknowledge#logic#in#the#system.

The#PC#version#supports#a#few#special#types#of#interface#board,#

that#use#specific#IO#ports#to#control#certain#on-board#features#

like#MODEMs#and#interrupt#configuration.

#�Initialization#and#attachment#of#the#channels#�

To#use#the#driver,#2#steps#must#be#performed:

#####1.#Global#initialization#of#the#driver

#####2.#Attachment#of#each#channel,#specifying#mode#and#options

The#global#initialization#is#needed#to#reset#all#SCCs#and#to#

install#a#common#interrupt#handler.##Also,#the#hardware#addresses#

of#the#chips#are#defined#in#this#step.##In#the#second#step,#each#

channel#is#set#up#for#the#intended#use.

#�#�1.#Initialization#�

Initialization#is#performed#using#the#"attach#scc#init"#command,#

using#the#following#syntax:

#####attach#scc#<number#of#chips>#init#<base#address>

##########<spacing#between#chips>#<offset#to#channel#A#ctrl>

##########<offset#to#channel#B#ctrl>#<offset#from#ctrl#to#data>

##########<address#of#INTACK#latch>|0#<interrupt#number>

##########p<PCLK#frequency>|r<RTxC#frequency>

##########[<special#interface#type>#<interface#parameter>]

To#initialize#a#board#with#2#SCCs#using#my#schematic,#the#

following#command#would#be#used:

#####attach scc 2 init fffd00 8 3 7 -2 fffd3f 3 p4915200

This#specifies#interrupt#number#3,#a#normally#unused#interrupt#in#

520ST#and#1040ST#machines.##In#the#newer#Mega-ST#this#interrupt#

is#used#for#the#Blitter.##In#this#case,#it#is#possible#to#use#the#

"ring#indicator"#input#of#the#RS232#interface#as#an#interrupt#

source.#It#has#interrupt#number#14.

The#INTACK#latch#(write,#then#read#to#get#the#interrupt#vector)#

is#located#at#0xfffd3f.##It#is#also#possible#to#specify#0#for#

this#address.##In#that#case,#INTACK#is#not#used#and#the#driver#

finds#the#interrupting#chip#by#polling#each#RR3A#register.##This#

allows#a#more#simple#hardware#design,#especially#for#PCs.

The#crystal#clock#is#specified#as#4.9152#MHz.##Other#frequencies#

can#be#used,#and#this#parameter#should#be#adjusted#accordingly.

The#clock#used#for#baudrate#generation#is#connected#to#PCLK.#It#

would#also#be#possible#to#supply#a#baudrate#clock#to#RTxC,#and#a#

clock#of#a#different#frequency#to#PCLK#(like#the#processor#

clock).##This#option#is#selected#by#prefixing#the#clock#frequency#

with#an#"r"#instead#of#a#"p".

The#"special#interface#type"#and#"interface#parameter"#fields#can#

be#used#to#specify#some#special#type#of#interface#board#to#be#

used.##This#requires#modification#to#the#driver#software.

Currently,#the#following#special#types#are#supported#on#the#PC:

#####-#type#1:#EAGLE

#####-#type#2:#PC-100

#####-#type#4:#PRIMUS

-#type#8:#DRSI#PC*Packet

The#PC-100#and#the#PRIMUS#accept#an#interface#parameter,#that#is#

used#to#control#the#on-board#MODEM.##It#must#be#specified#as#a#

hexadecimal#value#to#be#sent#to#the#output#port#on#the#board.

#�#�Possible#initialisation#commands#are:

EAGLE:####attach scc 1 init 2e8 8 2 0 1 0 2 p3686400#1

PC-100:###attach scc 1 init 2e8#16 6 4 1 0 2 p4915200#2#22

PRIMUS:###attach scc 1 init 2e8 4 2 0 1 0 2 r2457600#4#02

DRSI:#####attach scc 1 init 300 16 2 0 1 0 2 p4915200#8

When#using#an#AT-type#machine#(with#two#8259#interrupt#

controllers),#and#using#the#"interrupt#2"#line#on#the#BUS,#

specify#interrupt#number#9#instead#of#2.##This#better#reflects#

the#actual#configuration.##The#BIOS#will#attempt#the#redirection#

of#interrupt#vector##9#to##2,#but#it#does#not#completely#do#the#

job.

The#SCC#driver#uses#the#"buffers"#feature#that#pre-allocates#a#

number#of#receiver#buffers#to#be#used#by#the#drivers#at#interrupt#

time.##Be#sure#to#issue#a#"buffers#<n>"#command#before#you#attach#

any#of#the#SCC's#channels#(see#below).##The#<n>#parameter#of#the#

buffers#command#specifies#the#number#of#buffers#to#pre-allocate,#

this#is#a#function#of#the#number#of#channels#you#will#attach,#the#

baudrate#of#the#channels,#the#activity#on#each#of#these#channels,#

and#the#time#the#computer#can#spend#without#re-allocating#new#

buffers.##A#good#number#of#receiver#buffers#to#try#is#32.#If#you#

see#a#non-zero#number#in#the#"Space"#column#in#the#SCCSTAT#output#

(see#below)#you#can#increase#this#number.##It#specifies#the#

number#of#128-byte#buffers#that#are#pre-allocated#for#the#

receiver#buffers#of#all#channels#attached.#So#in#this#case,#

4Kbytes#are#allocated.

It#is#also#possible#to#set#the#size#of#the#buffers#to#be#

allocated,#using#the#2nd#and#3rd#argument#of#the#"buffers"#

command.##These#set#a#minimal#and#maximal#size#of#each#buffer.##

The#PC#version#will#only#allocate#buffers#of#the#maximum#size#

(and#ignores#the#second#argument),#but#the#Atari#ST#versions#

minimizes#the#thrashing#of#the#memory#freelist#by#allocating#

buffers#between#the#minimal#and#maximal#size,#that#can#be#carved#

out#of#the#freelist#without#further#splitting#blocks#of#memory.

Example:#buffers#64#10#128

#�2.#Attach#commands#�

When#the#SCC#driver#has#been#initialized,#you#can#attach#the#

channels.##This#is#done#using#one#of#3#forms#of#the#"attach#scc"#

command:

#####attach#scc#<channel>#slip#<mtu>#<baudrate>

attach#scc#<channel>#kiss#<mtu>#<baudrate>#<callsign>

attach#scc#<channel>#ax25#<mtu>#<baudrate>#<callsign>

These#will#be#refferred#to#as#"slip",#"kiss"#and#"ax25"#mode.

#�The#<channel>#parameter#specifies#which#half#of#which#SCC#will#be#

programmed,#as#follows:

#####channel#0:#"A"#side#of#first#SCC

#####channel#1:#"B"#side#of#first#SCC

channel#2:#"A"#side#of#second#SCC

channel#3:#"B"#side#of#second#SCC

etcetera.###The#channel#number#ranges#from#0#to#(2#*#nchips)#-#1,#

where#nchips#is#the#number#of#chips#specified#in#the

"attach#scc#init"#command.

###�SLIP#mode#�

This#can#be#used#for#a#link#to#another#machine,#when#the#channel#

has#an#RS232#interface.#Example:

#####attach#scc#0#slip#sl0#256#9600

##This#attaches#the#"A"#side#of#the#first#SCC#as#interface#"sl0",#

with#an#MTU#of#256#and#an#initial#baudrate#of#9600.##The#baudrate#

can#be#changed#later#by:

#####param#sl0#<baudrate>

Entering#just#"param#sl0"#will#display#the#current#baudrate.

#�KISS#mode#�

KISS#mode#can#be#used#to#talk#to#a#KISS#TNC.##It#can#also#be#used#

on#an#asynchronous#link#to#another#computer#running#NET.

The#difference#with#SLIP#is#that#the#interface#will#be#AX.25#

type,#and#can#therefore#be#used#for#AX.25#connections.##A#

callsign#must#be#given#as#a#parameter,#this#will#be#used#for#IP#

and#NET/ROM#purposes#on#the#interface.#Example:

#####attach scc 2 kiss 430 256 4800 pe1chl-7

In#the#example#the#"A"#side#of#the#second#SCC#will#be#attached#as#

interface#"430"#using#an#MTU#of#256,#a#baudrate#of#4800#and#the#

callsign#PE1CHL-7.

The#command#"param#430#<paramnum>#<decimal#value>"#can#be#used#to#

set#the#parameters#of#the#KISS#TNC.##It#is#not#possible#to#

display#the#current#settings.

#�AX.25#mode#�

This#is#probably#the#most#interesting#mode#to#use#with#this#

interface.##It#allows#AX.25#communication#without#a#TNC.##Only#a#

MODEM#is#needed.#Example:

#�#####attach scc 2 ax25 430 256 1200 pe1chl-7

The#parameters#have#the#same#meaning#as#in#KISS#mode.#In#fact,#

the#AX.25#mode#is#emulating#an#extended#KISS#TNC,#so#the#same#

commands#can#be#used#to#set#the#parameters#of#the#interface#(see#

below).

To#be#able#to#run#fullduplex#using#an#SCC#in#AX.25#mode,#an#

external#divider#must#be#available,#that#divides#the#baudrate#

generator#clock#available#on#the#TRxC#pin#by#32,#and#puts#the#

resulting#signal#on#the#RTxC#pin#of#the#same#channel#of#the#SCC.##

Such#a#divider#is#not#necessary#for#normal#CSMA#packet#radio#

operation,#but#interrupt#overhead#is#slightly#reduced#if#you#

still#install#it.##If#you#have#installed#the#divider,#prefix#the#

baudrate#with#a#letter#"d",#as#in:

#####attach scc 2 ax25 430 256 d1200 pe1chl-7

Note#that#when#using#the#RTxC#input#as#the#baudrate#generator#

clock#source,#you#will#have#to#feed#the#exact#rate#of#the#

transmitter#to#TRxC#to#be#able#to#use#fullduplex.#For#this#

reason,#it#is#better#to#use#PCLK#as#the#clocksource#whenever#

possible.

The#DRSI#PC*Packet#adapter#uses#the#Z8536#CIO#on#the#board#as#a#

divider,#so#you#can#(and#should)#specify#the#"d"#option#for#that#

board.

Another#option#is#to#use#external#clocking,#supplied#by#the#

MODEM.##This#is#specified#using#the#keyword#"ext",#as#in:

#####attach scc 2 ax25 430 256 ext pe1chl-7

The#receive#clock#will#be#taken#from#the#RTxC#pin,#the#transmit#

clock#from#the#TRxC#pin.##Watch#the#phase#relationship#between#

the#clock#and#the#data!

The#setting#of#parameters#of#the#emulated#KISS#TNC#is#done#in#the#

same#way#as#in#the#KISS#mode#of#the#SCC#driver:

param#430#<paramnum>#<decimal#value>

In#AX.25#mode#however,#it#is#also#possible#to#display#the#

current#settings#using#"param#430".##This#will#display#a#line#

like:#speed=1200#div=y#group=000#1=36#2=50#3=30#4=3#5=0#6=0#7=50#

######8=7#9=3#10=30.

The#"clocktick"#unit#mentioned#in#the#descriptions#below#depends#

on#the#computer#type.##On#the#Atari#ST,#each#clocktick#is#10ms.##

On#the#PC#Clones,#a#clocktick#is#usually#55ms,#but#the#actual#

value#is#printed#during#execution#of#the#"attach#scc#init".##

Consider#that#the#exact#time#of#each#delay#may#be#up#to#1#

clocktick#time#less#than#the#specified#time.##It#is#therefore#

unwise#to#specify#a#value#less#than#2#for#a#delay#(except#when#

the#special#case#0#is#selected).

#�

The#parameters#have#the#following#meaning:

1:###The#delay#(in#units#of#clocktick)#after#keying#of#the#

#####transmitter,#until#the#first#byte#is#sent.#This#is#usually#

#####called#"TXDELAY"#in#a#TNC.##When#0#is#specified,#the#driver#

#####will#just#wait#until#the#CTS#signal#is#asserted.#This#

#####assumes#the#presence#of#a#timer#or#other#circuitry#in#the#

#####MODEM#and/or#transmitter,#that#asserts#CTS#when#the#trans#

#####mitter#is#ready#for#data.

#####The#default#value#of#this#parameter#is#360ms.

2:###This#is#the#probability#that#the#transmitter#will#be#keyed#

#####when#the#channel#is#found#to#be#free.##It#is#a#value#from#0#

#####to#255,#and#the#probability#is#(value+1)/256.##The#value#

#####should#be#somewhere#near#20-60,#and#should#be#lowered#when#

#####the#channel#is#used#more#heavily.

#####The#default#value#is#25#(10%#persistence).

##3:###This#is#the#time#between#samples#of#the#channel.#It#is#

#####expressed#in#units#of#clocktick.##About#100-300#ms#seems#to#

#####be#a#good#value.

#####This#parameter#should#never#be#0.#The#channel#will#not#

#####transmit#when#0#is#used!

#####The#default#value#is#160ms.

##4:###The#time#the#transmitter#will#remain#keyed#after#the#last#

#####byte#of#a#packet#has#been#transferred#to#the#SCC.#This#is#

#####necessary#because#the#CRC#and#a#flag#still#have#to#leave#the#

#####SCC#before#the#transmitter#is#keyed#down.#The#value#depends#

#####on#the#baudrate#selected.##A#few#character#times#should#be#

#####sufficient,#e.g.#30ms#at#1200#baud.

#####The#value#of#this#parameter#is#in#clocktick#units,#the#

#####default#is#30ms#(or#at#least#2#clock#ticks).

##5:###The#full-duplex#mode#switch.#This#can#be#one#of#the#folowing#

#####values:

#####0:###The#interface#will#operate#in#CSMA#mode#(the#normal#

##########half-duplex#packet#radio#operation)

#####1:###Fullduplex#mode,#i.e.#the#transmitter#will#be#keyed#at#

##########any#time,#without#checking#the#received#carrier.##It#

##########will#be#unkeyed#when#there#are#no#packets#to#be#sent.

#####2:###Like#1,#but#the#transmitter#will#remain#keyed,#also#

##########when#there#are#no#packets#to#be#sent.##Flags#will#be#

##########sent#in#that#case,#until#a#timeout#(parameter#10)#

##########occurs.

The#default#value#is#0,#CSMA#operation.

6:###Control#of#the#DTR#output#of#the#SCC.##DTR#will#be#set#when#

#####this#value#is#nonzero,#it#will#be#reset#when#the#value#is#0.##

#####After#initialization#DTR#will#be#set#by#default.##The#DTR#

#####output#can#be#used#as#a#general-purpose#output,#e.g.#to#

#####change#between#2#transceiver#frequencies#or#output#power#

#####levels.

#�#####The#default#value#of#this#parameter#is#1,#DTR#SET.

7:###The#initial#waittime#before#any#transmit#attempt,#after#the#

#####frame#has#been#queued#for#transmit.##This#is#the#length#of#

#####the#first#slot#in#CSMA#mode.##In#fullduplex#modes#it#can#be#

#####set#to#1#for#maximum#performance,#or#to#a#higher#value#to#

#####give#NET#a#chance#to#combine#several#packets#in#a#single#

#####transmission.

#####The#value#of#this#parameter#is#in#clocktick#units.#It#should#

#####never#be#set#to#0.#(The#channel#will#not#transmit#in#that#

#####case)

#####The#default#value#is#500ms.

8:###The#maximal#time#the#transmitter#will#be#keyed#to#send#

#####packets,#in#seconds.##This#can#be#useful#on#busy#CSMA#

#####channels,#to#avoid#"getting#a#bad#reputation"#when#you#are#

#####generating#a#lot#of#traffic.##After#the#specified#time#has#

#####elapsed,#no#new#frame#will#be#started.#Instead,#the#trans#

#####mitter#will#be#switched#off#for#a#specified#time#(parameter#

#####9),#and#then#the#selected#algorithm#for#keyup#will#be#

#####started#again.

#####The#value#0#will#disable#this#feature,#and#allow#infinite#

#####transmission#time.

#####The#default#value#is#7#seconds.

9:###This#is#the#time#the#transmitter#will#be#switched#off#when#

#####the#maximum#transmission#time#is#exceeded.##This#parameter#

#####is#in#seconds,#and#should#never#be#0.

#####The#default#value#is#3#seconds.

#!10:##In#CSMA#mode,#this#parameter#specifies#the#maximum#time#the#

#####transmitter#will#wait#for#the#channel#to#become#clear,#

#####expressed#in#units#of#one#second.##When#this#time#has#

#####elapsed#without#an#opportunity#to#transmit,#the#transmitter#

#####will#be#keyed.##This#is#a#safeguard#against#situations#where#

#####the#squelch#refuses#to#close,#or#a#weak#carrier#is#present#

#####on#the#frequency,#to#prevent#an#indefinite#pileup#of#frames#

#####to#transmit#(with#a#possible#danger#of#memory#overflow).##Of#

#####course,#the#value#should#be#set#sufficiently#high#to#prevent#

#####accidental#keyups#in#periods#of#high#channel#load,#e.g.#60#

#####seconds.##There#is#a#compile-time#option#in#the#driver#to#

#####throw#away#frames#in#this#case.

#####In#fullduplex#2#mode,#this#parameter#specifies#the#maximum#

#####idle#time,#in#seconds.##When#no#frames#have#been#sent#for#

#####this#time,#the#transmitter#will#be#keyed#down.##A#value#of#0#

#####will#disable#this#feature,#i.e.#the#transmitter#will#be#

#####keyed#indefinitely.

#####The#default#value#of#this#parameter#is#120#seconds.

#�#�Transmitter#Groups#�

It#is#possible#to#build#special#radio#equipment#to#use#more#than#

one#frequency#on#the#same#band,#e.g.#using#several#receivers#and#

only#one#transmitter#that#can#be#switched#between#frequencies.##

Also,#you#can#connect#several#radios#that#are#active#on#the#same#

band.##In#these#cases,#it#is#not#possible,#or#not#a#good#idea,#to#

transmit#on#more#than#one#frequency.##The#SCC#driver#provides#a#

method#to#lock#transmitters#on#different#interfaces,#using#the#

"param#<interface>#group#<x>"#command.##This#will#only#work#when#

you#are#using#CSMA#mode#(parameter##5#=#0).

The#number#<x>#must#be#0#if#you#want#no#group#restrictions,#and#

can#be#computed#as#follows#to#create#restricted#groups:

<x>#is#the#sum#of#some#HEX#numbers:

#####200##This#transmitter#will#only#be#keyed#when#all#other#

##########transmitters#in#the#group#are#off.

#####100##This#transmitter#will#only#be#keyed#when#the#carrier#

##########detect#of#all#other#interfaces#in#the#group#is#off.

#####0xx##A#byte#that#can#be#used#to#define#different#groups.##

##########Interfaces#are#in#the#same#group,#when#the#logical#AND#

##########between#their#xx#values#is#nonzero.

Examples:

When#2#interfaces#use#group=201,#their#transmitters#will#never#be#

keyed#at#the#same#time.

When#2#interfaces#use#group=101,#the#transmitters#will#only#key#

when#both#channels#are#clear#at#the#same#time.##When#group=301,#

the#transmitters#will#not#be#keyed#at#the#same#time,#and#only#if#

both#channels#are#clear.

###�SCCSTAT#command#�

Once#the#SCC#driver#has#been#initialized,#some#statistic#

information#can#be#shown#using#the#sccstat#command.#The#output#of#

this#command#shows#one#line#of#information#per#attached#channel.

##Example:

Ch#Iface##Sent##Rcvd#Error#Space#Overr#Rxints#Txints#Exints#Spints

#0#144######88###152###200#####0#####0##10013###4488####905####235

#1#430#######6####70#####0#####0#####0###1915#####29######0######0

##The#info#shown#is:

#####-#channel#number#of#the#attach#command

#####-#name#of#the#interface

#####-#number#of#frames#queued#for#transmission

#####-#number#of#frames#received#correctly

#####-#number#of#receive#errors#(CRC,#ABORT)

#####-#number#of#times#the#receiver#buffer#pool#was#found#empty

#####-#number#of#receiver#overruns#and#transmitter#underruns

#####-#number#of#receiver#interrupts

#�#####-#number#of#transmitter#interrupts

#####-#number#of#receiver#special#condition#interrupts

#####-#number#of#external/status#interrupts

It#is#normal#that#a#SLIP#or#KISS#channel#shows#no#errors,#and#no#

special#condition#or#external/status#interrupts,#while#an#AX25#

channel#has#lots#of#these.

An#overrun#is#abnormal#for#all#operating#modes.##If#lots#of#these#

occur,#the#product#of#baudrate#and#number#of#interfaces#is#too#

high#for#the#processing#power#of#your#computer.

If#"Space"#errors#occur,#specify#a#higher#number#of#buffers#in#

the#"buffers"#command.##It#is,#however,#normal#if#these#errors#

occur#when#you#start#a#shell,#or#when#you#pause#the#output#of#any#

command#using#CTRL-S.##This#is#because#the#processing#and#

allocation#of#buffers#stops#in#these#cases,#while#receiver#input#

keeps#coming#in#under#interrupt#control.

When#you#see#only#transmitted#frames,#the#number#of#transmitter#

interrupts#is#1,#and#all#other#counters#are#0,#the#SCC#is#not#

generating#interrupts#to#the#computer.##The#single#transmitter#

interrupt#is#a#"simulated"#interrupt#that#should#start#the#

transmission#(but#apparently#doesn't).