STRUCTURE OF KA9Q NET (PE1CHL VERSION)

This picture clarifies the relation between the different protocol components

present in the PE1CHL version of NET.

Transfer of information is only possible crossing horizontal lines in this

picture.  E.G: Information can be transferred from TELNET to TCP, but not from

TELNET to NET/ROM level 7.

+----------------------------------------------------------------------------+

|                                                                            |

|                        U S E R   I N T E R F A C E                         |

|                                                                            |

+------------------+-------------------+--------+-----+------+------+--------+

|                  |                   |        |     |      |      |        |

| TNC, MHEARD,     |   NET/ROM level 7 | TELNET | FTP | SMTP | RCMD | REMOTE |

|                  |                   |        |     |      |      |        |

| NETDIGI, BRIDGE, | +-----------------+--------+-----+------+------+--------+

|                  | |                 |                            |        |

| TNC2 EMULATOR    | | NET/ROM level 4 |             TCP            |  UDP   |

|                  | |                 |                            |        |

|                  | +-----------------+----------------------------+--------+

|                  | |                 |                                     |

|                  | |                 |             IP / ICMP               |

|                  | | NET/ROM level 3 |                                     |

|                  | |                 +----+---------------------+          |

|                  | |                      |                     |          |

+------------------+-+----------------------+----+                |          |

|                                                |      ARP       |          |

|   AX.25 Level 2 Version 1 / 2  (LAPB / UI)     |                |          |

|                                                |                |          |

+--------------------------+---------------------+----------------+----------+

|                          |                     |                |          |

| Direct HDLC (SCC Driver) |  KISS (MBBIOS/X00)  |    Ethernet    |   SLIP   |

|                          |                     |                |          |

+--------------------------+---------------------+----------------+----------+

|                         |                      |           |

| /|                      | /|                   |           | /|

|/ |                      |/ |                   |           |/ |

|                         |                   |              |

v                         v                   |              v

MODEM                      TNC             //=======//    Other Sys

TNC, MHEARD, NETDIGI, BRIDGE and TNC2 EMULATOR are the AX.25 servers.

NET/ROM level 7 is the NET/ROM command interpreter.  It is both an AX.25 and

a NET/ROM level 4 server.

TELNET, FTP, SMTP, RCMD and FINGER (not shown) are the TCP/IP servers.

REMOTE is a UDP/IP server.

NET/ROM level 4 is the transport protocol of the NET/ROM stack.  It manages

connections between NET/ROM endnodes.

TCP is the transport protocol of the Internet Suite.  It manages connections

between endnodes in the TCP/IP world.

UDP is the datagram protocol of the Internet Suite.  It is comparable to the UI

frame transmissions of AX.25.

NET/ROM level 3 is the network protocol of the NET/ROM stack.  It manages the

routing of packets between NET/ROM endnodes, relying on a table built from

information received in NODES transmissions.

IP/ICMP is the network protocol of the Internet Suite.  This implementation

routes packets based on a fixed routing table.

ARP manages the mapping of Internet addresses to the addresses of the

underlying network.  In the case of NET these can be amateur callsigns or

Ethernet hardware addresses.

AX.25 Level 2 manages all packet traffic on the amateur packet radio network.

it supports both virtual circuit (LAPB) and datagram (UI) modes.

Virtual circuits can be used directly by AX.25 servers, by NET/ROM level 3, and

by IP.  Datagrams are only used by IP.

The drivers for HDLC, KISS, Ethernet and SLIP manage the physical lines

connected to the system.  The HDLC driver provides AX.25 packets directly,

making only a MODEM necessary for amateur radio packet communication.

The KISS driver provides the same information encoded in KISS frames, which

will be sent to a TNC (in asynchronous form) for conversion to HDLC.

The Ethernet and SLIP drivers provide ways to connect systems for IP traffic

using either an Ethernet LAN or an asynchronous connection.

Rob Janssen, PE1CHL