Storm VT100/VT102 terminal emulation

Features:

Graphics character set (line drawing)

Double-width, double-height characters

Text effects include bold, underline, inverse. (Blinking effect displayed

as 'light' characters).

Detailed specifications.

Note: Columns and rows are numbered starting at row 1, column 1 in the upper

left corner.

Note: In the following table, 'Pn' indicates a numeric parameter. Numeric

parameters are simply Ascii numbers that are used to modify the effect of

an escape sequence. e.g. specify how many columns the cursor is to move.

If multiple numeric parameters are sent, they are separated by a semicolon.

The absence of a parameter means a default value is assumed.

Scrolling regions: Under software control, a host computer can set up a

region of lines as a 'scrolling region'. Control characters and escape

sequences that normally cause the entire screen to scroll up or down will

cause only the area of the scrolling regions to scroll up or down instead.

Character(s)    Effect

5       ENQ. Terminal replies with Answerback string. To add an

answerback string, you need to edit the STORM.INI file to

add an entry in [VT100] of the form:

Answerback="string"

You could also use the Basic command SET

e.g. SET "STORM","VT100","Answerback","string"

The answerback string is limited to 80 characters.

Note that answerback is disabled if the CompuServe BPLUS.TRF

file transfer module is loaded because ENQ is required for

B-PLUS file transfers.

7       Bell. Causes a bell sound, if it's enabled by your system

setup. This is controlled by the control panel usually.

8       Backspace. Moves the cursor to the previous column. No action

taken if the cursor is already in column 1. Backspaces are

non-destructive in VT100, though that can be overridden by

selecting "BS Erases" in the generic Terminal setup dialog.

9        Tabs to the next tab stop. Default tab stops are every 8 columns,

starting at column 9. Tab stops can be set and cleared by the

host computer by sending the appropriate escape sequences.

10       Line Feed. Causes the cursor to move to the next line. If the

cursor is already at the bottom of the screen or scrolling

region, scrolls the screen upwards. 'New Line' mode, if set,

causes the cursor to move to column 1 when a line feed occurs,

simulating the effect of a carriage return.

11

12        Generate line feeds.

13      Carriage return. Causes the cursor to move to column 1 on the

current line. A separate setting in the generic Terminal

setup dialog can add a line feed to all carriage returns

received, though that is not really a part of the VT100

emulation.

14      Shift Out. Selects 'G1' character set. Specific

escape sequences (described later) allow you to specify what

character set is used as the G1 character set.

15      Shift In. Selects 'G0' character set. G0 character set can

be altered by escape sequences (described later)

27 (ESC)The escape character introduces an escape sequence, which is

used to control many terminal features.

ESC [   CSI or Control String Introducer. Consists of the ESC

(Ascii 27) followed by a left square bracket. Indicates the

start of a category of escape sequences.

ESC D   Index. Moves the cursor to the next line, scrolling the screen

up if necessary.

ESC E   Next Line. Equivalent in effect to a Carriage Return + Line

Feed combination. Moves the cursor to the start of the next

line, scrolling the screen up if necessary.

ESC M   Reverse Index. Moves the cursor to the previous line, scrolling

the screen down if necessary.

ESC H   Sets a tab stop at the current cursor location.

ESC 7   Saves the currrent cursor location, character set, and text

effects for later restoring via the cursor restore sequence.

ESC 8   Restores a cursor state saved by a previous ESC 7 sequence.

ESC =   Sets Application Keypad mode. Causes the numeric keypad on the

ST to send special escape sequences that can be interpreted

by a host program.

ESC >   Resets keypad to normal numeric mode.

ESC c   Resets the terminal to default settings. i.e. whatever settings

you selected in the setup dialog box.

ESC [ Pn @  Inserts the number of characters specified by the numeric

parameter at the current cursor location. Characters at the

end of the line are lost.

ESC [ Pn A  Moves the cursor up by the number of rows specified by the

numeric parameter. The cursor will not move above the top of

the screen or the top line of a scrolling region.

ESC [ Pn B  Moves the cursor down by the number of rows specified by the

numeric parameter. The cursor will not move above the bottom of

the screen or the top line of a scrolling region.

ESC [ Pn C  Moves the cursor right by the number of columns specified by

the numeric parameter. Will not move past the end of the line.

ESC [ Pn D  Moves the cursor right by the number of columns specified by

the numeric parameter. Will not move past the end of the line.

ESC [ Pn E  Moves the cursor down, as with ESC [ Pn B, but the cursor is

always repositioned to column 1.

ESC [ Pn F  Moves the cursor up, as with ESC [ Pn A, but the cursor is

always repositioned to column 1.

ESC [ Pn G  Cursor Horizontal Absolute. Moves the cursor to the column

specified by the numeric parameter.

ESC [ Pn;Pn H  Position Cursor. Moves the cursor to the specified row and

ESC [ Pn;Pn f  column. The first number is the row, the second number is the

column. Default if a parameter is absent is row/column 1.

When relative origin mode is in effect (discussed later) then

the cursor row is calculated relative to the top of the

scrolling region.

ESC [ 0 J    Clears from the current cursor location to the end of the

ESC [ J      screen. Does not move the cursor.

ESC [ 1 J    Clears from the start of the screen to the current cursor

location. Does not move the cursor.

ESC [ 2 J    Clears the entire screen. Does not move the cursor.

ESC [ K      Clears from the current cursor location to the end of the line.

ESC [ 0 K    Does not move the cursor.

ESC [ 1 K    Clears from the start of the line to the cursor location. Does

not move the cursor.

ESC [ 2 K    Clears the entire line that the cursor is on. Does not move the

cursor.

ESC [ Pn L   Insert Line. Inserts Pn blank lines at the current cursor

location and scrolls the rest of the screen (or scrolling

region) down.

ESC [ Pn M   Delete Line. Deletes Pn lines at the current cursor location

and scrolls the rest of the screen (or scrolling region) up.

ESC [ Pn P   Delete Character. Deletes Pn characters at the current cursor

location and shifts the line to fill in the space. Blank

characters are placed at the end of the line when characters

are deleted.

ESC [ Pn X   Erase Character. Erases Pn characters at the current cursor

location.

ESC [ c      Report Device Attributes. Responds by transmitting an escape

ESC [ 0 c    sequence identifying the terminal. The default response is

"ESC[?6;2c", which is a VT102 response. This reponse can be

changed by inserting a line in the [VT100] section of the

STORM.INI file with the heading ID="(Replacement ID)".

e.g. ID="\33[1;2c" would change the reponse to a VT100 type

response.

ESC [ g      Clear Column Tab. Clears the tab stop (if any) at the cursor

ESC [ 0g     location.

ESC [ 3 g    Clear All Column Tabs. Clears all tab stops.

ESC [ Pn h   Sets various terminal modes detailed below. Note that more

than one mode can be set if multiple parameters are sent.

ESC [ 4 h    Set Insert mode. Subsequent characters are inserted at the

current cursor location and the rest of the line is scrolled

over to. Characters at the end of the line are lost.

ESC [ 12 h   Set Local Echo. Turns half duplex on.

ESC [ 20 h   Set Newline mode. Causes received Line Feed characters

(Ascii 10) to also move the cursor to the start of the line.

ESC [ Pn l   Resets the various terminal modes detailed above. Reverses the

effect of the corresponding ESC [ Pn h escape sequence. Note:

'l' is a lowercase L, not the number one '1'.

ESC [ 4 l    Set Replace mode. Subsequent characters overwrite.

ESC [ 12 l   Resets local echo. Turns full duplex on.

ESC [ 20 l   Resets Newline mode.

ESC [ Pn m   Sets/resets various character attributes used in subsequent

terminal output. Multiple parameters allow the setting of

more than one attribute in the same escape sequence.

ESC [ m

ESC [ 0 m    No attributes. Turns all attributes off.

ESC [ 1 m    Set Bold attribute.

ESC [ 4 m    Set Underlined attribute.

ESC [ 5 m    Set Blinking attribute. Instead of the characters actually

blinking, this attribute is indicated by printing 'light'

characters.

ESC [ 7 m    Set Reversed attribute. Subsequent characters are displayed in

reverse video.

ESC [ 22 m   Turn Bold attribute off.

ESC [ 24 m   Turn Underlined attribute off.

ESC [ 25 m   Turn Blinking attribute off.

ESC [ 27 m   Turn Reversed attribute off.

ESC [ 5 n    Report Terminal Status. Emulator responds by sending ESC [ 0 n

ESC [ 6 n    Report Cursor Location. Emulator responds by sending an

escape sequence reporting the current cursor location.

ESC [ Pn;Pn r  Set Scrolling region. The first parameter is the top of the

scrolling region, the second is the bottom. Also causes the

cursor to move to location 1,1 (either top of the scrolling

region or top of the screen, depending on origin mode).

ESC [r       Resets the scrolling region to full screen.

ESC [0;0r

ESC ( 0      Sets the G0 character set as the Graphics character set.

ESC ( 1      Sets the G0 character set as the Alternate ROM character set.

This is not currently supported by the emulator, mostly to

save memory. However, it could easily be added.

ESC ( 2      Sets the G0 character set as the Alternate ROM graphics set.

This is also not currently supported by the emulator.

ESC ( A      Sets the G0 character set as the UK character set. The only

change is that '#' is replaced by the UK Pound Sterling

symbol.

ESC ( B      Sets the G0 character set as the ASCII character set. This is

the default character set.

ESC ) 0      Perform the same functions as the corresponding ESC ) escape

ESC ) 1      sequences, but affect the G1 character set instead.

ESC ) 2

ESC ) A

ESC ) B

ESC # 5      Set character width single on current line.

ESC # 6      Set double character width on current line.

ESC # 3      Set double-width/double-height character display on current

line. Displays the top half of each character as double-height

and double width. Used in conjunction with ESC # 4 to display

a line of double-height, double-width characters.

ESC # 4      Set double-width/double-height character display on current

line. Displays the bottom half of each character as

double-height and double-width.

ESC # :      Sets double-height character display on current line. Displays

the top half of each character double-height;

ESC # ;      Sets double-height character display on current line. Displays

the bottom half of each character double-height;

ESC [ ? Pn h Sets various extended terminal modes described below. Multiple

modes can be set if more than one parameter is sent.

ESC [ ? 1 h  Set cursor key Application mode. Causes the cursor keys to

send escape sequences which can be interpreted by a host

computer.

ESC [ ? 3 h  Sets 132 column mode. Currently not supported.

ESC [ ? 4 h  Sets Smooth Scroll. Not implemented.

ESC [ ? 5 h  Sets Reverse Screen. Toggles output to white characters on a

black background. Note that Storm uses white and black by

default. However you can set different colors for fore and

back colors by inserting entries in the [Terminal] section

of the STORM.INI file. The default entries are

Fore=1

Back=0

which set foreground to black, background to white. The digits

are GEM color indexes as detailed in the following table:

The number of colors supported depends on video mode. The

actual color that appears depends on how you have setup the

color palette. Setting reverse screen simply swaps foreground

and background colors.

This feature has not been testing on the Falcon or TT.

Index    Color

===============

0    White

1    Black

2    Red

3    Green

4    Blue

5    Cyan

6    Yellow

7    Magenta

8    White

9    Black

10    Light Red

11    Light Green

12    Light Blue

13    Light Cyan

14    Light Yellow

15    Light Magenta

16 or greater is device dependant.

ESC [ ? 6 h  Set Relative Origin Mode. Cursor positioning commands in

Relative Origin mode are relative to the top of the scrolling

region (if any) rather than the top of the screen.

ESC [ ? 7 h  Set Line Wrap. When line wrap is set, typing a character at

the end of a row will cause the cursor to move to the start

of the next line.

ESC [ ? Pn l Resets verious extended terminal modes. Has opposite effect

of corresponding Esc [ ? Pn h setting.

ESC [ ? 1 l  Resets cursor keys to normal mode.

ESC [ ? 2 l  Set VT52 Mode. This will cause the emulator to switch to

emulating a VT52 terminal. To return from VT52, use the

sequence ESC < from within VT52 emulation.

ESC [ ? 3 l  Set 80 column mode. Since the emulator doesn't yet support

132 column mode, this has no effect.

ESC [ ? 4 l  Set Jump Scroll. Since smooth scroll is not supported, this

has no effect.

ESC [ ? 5 l  Set Normal Screen. Restores normal foreground and background

colors.

ESC [ ? 6 l  Reset Origin Mode. Restores absolute cursor origin mode.

ESC [ ? 7 l  Reset Line Wrap. When entering characters, the cursor will

not automatically wrap to the next line.

ESC [ ? 15 n Report Local Printer Status. No printer support at present.

The emulator responds with "ESC ? 13 n" to indicate that

no printer is present.

Other VT100 Info

The VT100 default settings are saved in the STORM.INI file in a section

labelled [VT100]. The entry Reset=00000000000 contains the settings. Each

digit is a VT100 setting. Most settings are either 0 or 1. A few have

larger ranges The settings are (in order)

Digit = 1            Digit = 0

=================================================

Keypad Application Mode    Keypad Numeric Mode

Cursor Application Mode    Normal Cursor Mode

Relative Origin Mode    Absolute Origin Mode

Insert Mode            Overwrite Mode

New Line Mode Set        New Line Mode Reset

-----------------

G0 (can range from 0 to 3)

G1 (can range from 0 to 3)

Current Selected Set. 0 or 1 at present, indicating G0 (0) or G1 (1). In

future this could change to 0 to 3 if support for four character sets is

added.

-----------------

Line Wrap On        Line Wrap Off

Destructive Backspace    Non-Destructive Backspace

ReverseVideo        Normal Video

Note that this includes extra settings not available from the setup dialog.

When the ESC c sequence is received and the terminal is reset, these values

are used in resetting the terminal. This is useful if you want a particular

setup to be retained when the terminal is reset. e.g. keypad application mode

to remain set.

I should note that Line Wrap, Destructive Backspace, and Reverse Video are

actually part of the general terminal setup. They are included here in order

to be able to override the general terminal settings when resetting the

VT100 terminal. When you use the VT100 setup dialog, the current state of

these three settings are automatically saved in the VT100 Reset setting.

Note: Setting the VT100 New Line mode alters the general terminal settings to

set Add Linefeeds Inbound in addition to the VT100 New Line setting. If you

have New Line selected in VT100, but Add Linefeeds Inbound is not selected

in the general terminal setup, then loading VT100 will automatically set

Add Linefeeds Inbound.